3GPP TSG RAN WG1 #118							  R1-2406447

Maastricht, NL, August 19th – 23rd, 2024

Agenda Item:	9.11.2

Source: 	LG Electronics

Title: 	Discussion on support of (e)RedCap UEs with NR-NTN operating in FR1-NTN bands

Document for:	Discussion and decision

- Introduction

In RAN#104, the Rel-19 NTN WI objectives for the support of HD-FDD (e)RedCap UE with NR NTN operating in FR1-NTN bands were revised as follows [1]:

5. Support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands [RAN4, RAN1] - For full-duplex and half duplex FDD RedCap and eRedCap UEs, define the RF and RRM requirements [RAN4] - For HD-FDD RedCap UEs and eRedCap UEs, specify enhancements for mitigating issues caused by TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB [RAN1] - Enhancements are targeted for the following HD collisions cases (cases 3 and 4): - Semi-statically configured DL reception collides with semi-statically configured UL transmission - Dynamically scheduled DL reception collides with dynamic scheduled UL transmission - Note: enhancements for other HD collision cases are not targeted. Enhancements for HD collision cases 3 and 4 that may improve other HD collision cases are not precluded. - Note: potential work in RAN2, if any, starts after RAN1 has decided on the solution. - Notes for this objective: - GNSS (Global Navigation Satellite Systems) capabilities and simultaneous GNSS and NR-NTN operation is supported in RedCap/eRedCap UE.

Moreover, the following observation(s) and conclusion(s) were made the support of HD-FDD (e)RedCap UE with NR NTN operating in FR1-NTN bands in RAN1#117 meeting [2]:

Conclusion For Rel-19 HD-FDD RedCap/eRedCap UE in NTN, the issues caused by TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB should be mitigated for collision cases 3 and 4. - Note: further discussion on other cases is not precluded Conclusion For collision cases 1, 2, 5 and 6, the existing priority rules can be reused for a HD-FDD (e)RedCap UE in NTN. Observation TA reporting is beneficial to mitigate the TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB for HD-FDD RedCap/eRedCap UE in NTN from RAN1 perspective. - Note: complexity, power consumption and signaling overhead impact of TA reporting for (e)redcap UEs was not investigated in this work item

In this contribution, we discuss potential enhancements for half-duplex collision casesto support (e)RedCap UEs with NR NTN in FR1.

- Discussion

  - HD collision cases 3 and 4

    - TA report enhancement

In order to determine whether there will be critical RAN1 issues when supporting HD-FDD (e)RedCap UE with NR NTN in FR1, a quantitative study on the level of TA misalignment between gNB and UE should be conducted first. In the RAN1 #116bis meeting [3], potential issues for DL/UL collision rules of HD-FDD UE were discussed under the assumption that TA mismatch between gNB and UE is relatively large. However, there was no clear consensus on the amount of TA mismatch between gNB and UE. For example, in NR NTN, the TA mismatch may occur if gNB does not receive any TA report or if the existing TA report is outdated or if the TA report granularity is not fine enough. If UE does not perform a TA report at all, gNB cannot set some key scheduling variables (e.g., Kcell,offset, KUE,offset), so the scenario (e.g., no TA report) is not considered as a feasible scenario.

Observation #1: When UE performs TA reporting in NR NTN, TA misalignment may occur mainly due to outdated TA reporting and/or coarse TA report granularity.

Meanwhile, the difference due to outdated TA reporting may occur when UE location changes, and may occur in proportion to the RTT difference depending on UE location within the cell (e.g., difference between the minimum TA and the maximum TA). For example, assuming LEO 600km, beam size of 50km and the target elevation angle of 30 degrees, the difference between the shortest RTT (minimum TA) and the longest RTT (maximum TA) is within about 300us, which can correspond to about 4~5 OFDM symbols with 15kHz SCS. Considering that the TA report granularity in NTN is 1ms (e.g., 14 OFDM symbols with 15kHz SCS), for the LEO example, the main cause of TA mismatch can be TA report granularity rather than outdated TA report. Therefore, to reduce the TA mismatch, enhancement of TA report granularity can be considered.

Observation #2: For LEO 600km, beam size of 50km and the target elevation angle of 30 degrees, the difference between the minimum TA and the maximum TA can be less than TA report granularity (e.g., 1ms).

Proposal #1 For HD collision cases 3 and 4 in Rel-19 NR NTN, support TA reporting enhancement(s) including the follow(s):

- Support of finer TA report granularity

Figure 1. Example of TA misalignment within a beam/cell

    - UE behavior enhancement

In RAN1 #116 meeting [3], an issue was raised where gNB was unable to avoid the error cases (e.g., Case 3 and Case 4) due to TA misalignment. Therefore, it can be considered to define new UE behaviors (e.g., DL/UL priority rule) for HD collision cases 3 and 4. However, it should be noted that defining new DL/UL priority rule is not a solution for the resource inefficiency issue due to TA misalignment. This is because whether or not resource utilization is efficient depends entirely on the scheduling of gNB but the occurrence of HD collision case 3 or 4 cannot be determined in gNB due to TA misalignment. For example, even if DL/UL priority rules are defined for HD collision cases 3 or 4 at UE side, gNB may conservatively schedule only one transmission, either DL or UL since it cannot guarantee whether HD collision case 3 or 4 will occur between adjacent DL and UL transmissions at the UE side. Therefore, if the mismatch between the UE and the gNB for the DL/UL collision decision is not resolved, the scheduling decision of the gNB may still be the same as when the DL/UL priority rule is not determined for HD collision case 3 or 4. Therefore, it is necessary to discuss on how to have the same understanding of potential DL/UL collision resources between gNB and UE under TA misalignment. If gNB and UE do not have the same understanding of potential DL/UL collision resources, unintended transmission and/or reception may occur and system performance may degrade.

In NR NTN where TA misalignment exists, a method of utilizing TA report-based guard time (GT) can be considered as a way to support that gNB and UE have the same understanding of potential DL/UL collision resource(s). For example, if a DL resource and a UL resource exist within GT, gNB and UE may regard these resources as potential DL/UL collision resources. The value of GT can be set sufficiently large to absorb TA misalignment (e.g., difference between actual TA and reported TA) between gNB and UE. However, even if the GT is introduced, if the UL TA values assumed by gNB and UE are different, the understanding of the potential DL/UL collision resource(s) may still be different. To solve this problem, gNB and UE can determine the GT-based DL/UL collision resource(s) under the same UL TA assumption. For example, gNB and UE can assume the common TA or the TA value in the latest TA report as UL TA, and then determine potential DL/UL collision resource(s) based on GT (e.g., DL/UL resources exist within GT under given UL TA assumption). Meanwhile, when the GT-based DL/UL collision determination is introduced, exception handling for cases where the actual TA misalignment is larger than the GT should be considered. As an example, when the GT is no longer valid, UE can perform TA report to gNB to reconfigure the value of GT.

Observation #3: If TA assumption is different between UE and gNB due to TA misalignment, it can lead to mismatch between the UE and the gNB regarding the occurrence of HD collision case 3 or 4, which may cause inefficient resource scheduling.

Proposal #2: For HD collision cases 3 and 4 in Rel-19 NR NTN, support UE behavior enhancement(s) including the follow(s):

- Introduction of DL/UL priority rule

- Alignment for DL/UL collision decision

Additionally, if HD collision case 3 and 4 are not treated error cases in Rel-19 NR NTN, there can be more specification works to handle the combinations of multiple HD collision cases. For example, for a series of transmissions, a case may occur where a dynamic DL resource and semi-static UL resource collide, and then the semi-static UL resource collides with another semi-static DL resource (e.g., maxed case of case 1 and case 3). In the above case, it is necessary to discuss which collision will be resolved first, which can complicate UE operation and cause many specifications impacts. For example, it can be considered to resolve collisions between semi-static resources first and to resolve collisions with dynamic resources second.

Observation #4: If new DL/UL priority rule(s) are defined in HD collision case 3 and case 4, multiple DL/UL collision cases can be mixed and the relevant UE behavior may be complicated.

Proposal #3: If new DL/UL priority rule(s) are defined for HD collision case 3 and 4 in Rel-19 NR NTN, study how to handle the combination of multiple HD collision cases.

Figure 2. Example of mixed DL/UL collision cases

Therefore, if case 3 and/or case 4 is not treated as an error case(s) as in before and UE behavior is defined, RAN1 should study enhanced DL/UL collision handling rule(s) that can be applied in various case(s) including the above mixed DL/UL collision case(s). For example, UE may determine the existence of colliding DL resources based on UL resources and then apply DL/UL collision handling rule(s) if at least one colliding DL resource exists. UE processing time can be considered when determining the colliding DL resources. For example, only DL resources scheduled and/or activated before the UE processing time from the start of the UL resource can be considered for DL/UL collision for the potential UL cancellation. When there are multiple UL resources that need to be checked for DL/UL collisions, the UE can determine the colliding DL resources starting from the UL resource with the earliest transmission time and apply DL/UL collision handling rules in order.

  - Other HD collision cases

    - UL transmission colliding with SSB

When the colliding DL reception is SSB (e.g., HD collision case 5), the available slot count determined by UE and gNB may be different, which may result in UE transmitting UL signal to resources not intended by gNB, causing resource waste and/or interference. This is mainly due to the misalignment of DL/UL collision decision between UE and gNB. If the UE behavior enhancement for HD collision case 3 and 4 is supported, the same solution can be applied to resolve HD collision case 5. For example, if DL/UL collision decision are aligned by the UE behavior enhancement, then the UE and gNB can count available slots on the non-collisional UL slots.

Proposal #4: For UL transmission colliding with SSB in Rel-19 NR NTN, the UE behavior enhancement for HD collision cases 3 and 4 can be applied to align the DL/UL collision decision between UE and gNB, and available slot can be counted based on the non-collisional UL slots.

    - Actual TDW determination

In RAN1 #116 meeting [1], an issue was raised that the event decision rule for actual TDW determination in TN may not appliable for NTN due to TA misalignment. For example, if there is TA misalignment between gNB and UE, gNB cannot know the exact UL resource(s) which is overlapped with DL reception, and therefore gNB and UE can have different understanding on the actual TDW. If gNB performs DM-RS bundling that includes a time interval other than the actual TDW of UE, channel estimation (CE) and reception performance for the corresponding UL transmission may significantly deteriorate. One possible solution is for gNB and UE to have the same understanding of potential DL/UL collision resource(s) as discussed in section 2.1.2. Then, gNB can avoid the potential DL/UL collision by scheduling. Even if a collision occurs, gNB and UE can exclude the potential colliding UL resource(s) from the actual TDW. It means that UE may not guarantee the phase continuity and power consistency in the potential DL/UL collision resource(s). Whether to allow UL transmission in the potential DL/UL collision resource(s) can be further discussed.

Proposal #5: For actual TDW determination in Rel-19 NR NTN, the UE behavior enhancement for HD collision cases 3 and 4 can be applied to align the DL/UL collision decision between UE and gNB, and actual TDW can be determined based on the non-collisional UL slots.

- Conclusions

In this contribution, we discussed whether or how to change half-duplex collision rules to support (e)RedCap UEs with NR NTN in FR1. Based on the above discussion, our observations and proposals are given as follows:

Observation #1: When UE performs TA reporting in NR NTN, TA misalignment may occur mainly due to outdated TA reporting and/or coarse TA report granularity.

Observation #2: For LEO 600km, beam size of 50km and the target elevation angle of 30 degrees, the difference between the minimum TA and the maximum TA can be less than TA report granularity (e.g., 1ms).

Observation #3: If TA assumption is different between UE and gNB due to TA misalignment, it can lead to mismatch between the UE and the gNB regarding the occurrence of HD collision case 3 or 4, which may cause inefficient resource scheduling.

Observation #4: If new DL/UL priority rule(s) are defined in HD collision case 3 and case 4, multiple DL/UL collision cases can be mixed and the relevant UE behavior may be complicated.

Proposal #1 For HD collision cases 3 and 4 in Rel-19 NR NTN, support TA reporting enhancement(s) including the follow(s):

- Support of finer TA report granularity

Proposal #2: For HD collision cases 3 and 4 in Rel-19 NR NTN, support UE behavior enhancement(s) including the follow(s):

- Introduction of DL/UL priority rule

- Alignment for DL/UL collision decision

Proposal #3: If new DL/UL priority rule(s) are defined for HD collision case 3 and 4 in Rel-19 NR NTN, study how to handle the combination of multiple HD collision cases.

Proposal #4: For UL transmission colliding with SSB in Rel-19 NR NTN, the UE behavior enhancement for HD collision cases 3 and 4 can be applied to align the DL/UL collision decision between UE and gNB, and available slot can be counted based on the non-collisional UL slots.

Proposal #5: For actual TDW determination in Rel-19 NR NTN, the UE behavior enhancement for HD collision cases 3 and 4 can be applied to align the DL/UL collision decision between UE and gNB, and actual TDW can be determined based on the non-collisional UL slots.

Reference

[1] RP-241667, “Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3,” Thales, CATT, 3GPP TSG RAN Meeting #104, Shanghai, China, June 17-20, 2024.

[2] RAN1 Chair’s Notes, RAN1 #117.

[3] RAN1 Chair’s Notes, RAN1 #116bis.

[4] 3GPP TS 38.213: “NR; Physical layer procedure for control”.

[5] 3GPP TS 38.321: “NR; Medium Access Control (MAC) protocol specification”.