3GPP TSG RAN WG1 #118											      R1-2406549

Maastricht, NL, August 19th – 23rd, 2024

Agenda item:	9.2.3

Source:	NEC

Title:              	Discussion on 3-antenna-port codebook-based transmissions

Document for:	Discussion and Decision

- Introduction

The enhancement for objective agreed for uplink with 3Tx in Rel-19 has almost been completed.

4. Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource Note: UL full power transmission mode 1 and 2 are not supported.

During discussion of WID of Rel-19, the scope for 3Tx enhancement is only focused on non-coherent codebook design due to the limited TU. While the work has been finished in advance, to utilize the TU more efficiently, we prefer further updating the objective, before the RANP meeting, we can have some preliminary discussion on the potential scopes.

- Discussion

Considering the UE antenna structure for 3Tx, there are two typical structures: one is 3 antennas with linear array, and another one is a cross-polarized two antennas and a single-polarized antenna, which was also agreed in RAN1#116 [1].

Agreement For performance evaluation of 3TX UE, consider following reference configurations, - A linear array (1D) of single-polarized antenna configuration with a spacing of 0.5λ, - For example: ¦←0.5λ→¦←0.5λ→¦ - A configuration of a cross-polarized and a single-polarized antennas, - For example: ×←0.5λ → ⁄

Considering the cross-polarization two antennas, it’s possible to be coherent, and the single-polarized antenna can be either coherent or non-coherent with the cross-polarized two antennas. To achieve better performance, at least partial coherent codebook should be supported.

Proposal1: At least support to design partial coherent codebook for 3Tx.

And it was discussed whether to support antenna switching with 3T6R, we think it can be introduced, and two 3-port SRS resources can be configured.

Proposal2: Open to introduce 3T6R with two 3-port SRS resources.

- Conclusion

In this contribution, we provided our views on uplink 3Tx enhancement, and we have the following proposals:

Proposal1: At least support to design partial coherent codebook for 3Tx.

Proposal2: Open to introduce 3T6R with two 3-port SRS resources.

- References

- Chair’s notes, RAN1#116, Athens, Greece, February 26th – March 1st, 2024