3GPP TSG RAN WG1 #118	R1-2406739

Maastricht, NL, August 19th – 23rd, 2024

Agenda item:	9.11.2

Source: 	ETRI

Title:	Discussion on HD UEs with NR NTN

Document for:	Discussion/Decision

Introduction

This contribution presents ETRI’s views on RedCap and eRedCap UEs with NR NTN operating in FR1-NTN bands based on the agreements/observations in Table 1.

Table 1. Agreements on support of NTN HD UEs [1], [2], and [3].

Agreement Study at least the following scenarios for (e)RedCap HD-FDD UEs for NTN: - Whether existing handling rules for the following cases should be reused or updated when taking into account TA mismatch between actual TA used by UE and assumed TA at the gNB based on available TA report: - Case 1: Dynamically scheduled DL reception collides with semi-statically configured UL transmission - Case 2: Semi-statically configured DL reception collides with dynamically scheduled UL transmission - Case 3: Semi-statically configured DL reception collides with semi-statically configured UL transmission - Case 4: Dynamically scheduled DL reception collides with dynamic scheduled UL transmission - Case 5: Configured SSB collides with dynamically scheduled or configured UL transmission - Case 6: Dynamic or semi-static DL collides with valid RO - Case 7: Collision due to direction switching - At least the following potential issues can be further considered for (e)RedCap HD-FDD UEs - Error cases in case 3 and case 4 - SIB19 reception collides with UL transmission - Slot counting for UL repetition transmission colliding with SSB reception - Invalid symbol determination for PUSCH repetition type B - Actual TDW determination due to the collision between DL reception and UL transmission with DMRS bundling - CPU occupation due to omitted DL reception or UL transmission Note: Both GSO and Non-GSO should be considered. RAN1 observation #1 To avoid the occurrence of error cases 3 and 4 through network scheduling, there are less resources available for a scheduled HD-FDD RedCap/eRedCap UE in NTN compared to TN when there is TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB. RAN1 observation #2 For collision cases 1, 2, 5 and 6, when there is TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB, there might be less resources available for the scheduled HD-FDD RedCap/eRedCap UE in NTN compared to TN if gNB attempts to avoid the collision or there is a loss of DL/UL transmissions due to collision. RAN1 observation #3 When there is TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB, there may be a BLER performance degradation for the reception of UL transmissions at the gNB for the scheduled HD-FDD RedCap/eRedCap UE in NTN compared to TN if gNB does not attempt to avoid the collision at least in the following cases: - UL transmission with repetitions due to different available slot counting at UE and gNB when colliding with SSB reception - PUSCH repetition type B due to different invalid symbol determination at gNB and UE when colliding with DL transmissions - UL transmission with DMRS bundling due to the different actual TDW determination at gNB and UE when colliding with DL transmissions Note: the above cases happen at least with one of collision cases 1, 2, 5, 6, and 7. Conclusion For Rel-19 HD-FDD RedCap/eRedCap UE in NTN, the issues caused by TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB should be mitigated for collision cases 3 and 4. - Note: further discussion on other cases is not precluded Conclusion For collision cases 1, 2, 5 and 6, the existing priority rules can be reused for a HD-FDD (e)RedCap UE in NTN. Observation TA reporting is beneficial to mitigate the TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB for HD-FDD RedCap/eRedCap UE in NTN from RAN1 perspective. - Note: complexity, power consumption and signaling overhead impact of TA reporting for (e)redcap UEs was not investigated in this work item

Discussion

Table 2 shows the revised WID objectives on HD FDD UE from RAN#104 meeting. As in Table 2, RAN1 will identify solution(s) to resolve issues caused by TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB, especially for case 3 and 4. In this regard, we discuss the following three perspectives:

- UE-specific TA report

- Collision handling for case 3 and 4

- RACH-less HO for NTN HD UE

Table 2. Revised NR NTN phase 3 WID objectives on HD FDD UE [4].

Support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands [RAN4, RAN1] - For full-duplex and half duplex FDD RedCap and eRedCap UEs, define the RF and RRM requirements [RAN4] - For HD-FDD RedCap UEs and eRedCap UEs, specify enhancements for mitigating issues caused by TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB [RAN1] - Enhancements are targeted for the following HD collisions cases (cases 3 and 4): - Semi-statically configured DL reception collides with semi-statically configured UL transmission - Dynamically scheduled DL reception collides with dynamic scheduled UL transmission - Note: enhancements for other HD collision cases are not targeted. Enhancements for HD collision cases 3 and 4 that may improve other HD collision cases are not precluded. - Note: potential work in RAN2, if any, starts after RAN1 has decided on the solution. - Notes for this objective: - GNSS (Global Navigation Satellite Systems) capabilities and simultaneous GNSS and NR-NTN operation is supported in RedCap/eRedCap UE.

UE-specific TA report

As in Table 3, a UE can report the least integer number of slots for the timing advance using subcarrier spacing of 15 kHz (i.e., with 1 msec reporting granularity). Therefore, when the actual length of uplink slot or downlink slot is less than 1 msec, then the network may have a certain level of ambiguity on the time reference for the half-duplex (HD) prioritization rule given that the reported TA value may represent multiple uplink slots rather than a single fixed slot.

Table 3. Timing advance report MAC CE in TS 38.321 [5]

#### 6.1.3.56 Timing Advance Report MAC CE The Timing Advance Report MAC CE is identified by MAC subheader with LCID as specified in Table 6.2.1-2. It has a fixed size and consists of two octets defined as follows (Figure 6.1.3.56-1): - R: Reserved bit, set to 0; - Timing Advance: In FR1 except for ATG, the Timing Advance field indicates the least integer number of slots for NTN or symbols for ATG, using subcarrier spacing of 15 kHz for NTN and either 15 kHz or 30 kHz for ATG, greater than or equal to the Timing Advance value (see TS 38.211 [8], clause 4.3.1). For ATG, the Timing Advance field indicates the least integer number of symbols greater than or equal to the Timing Advance value (see TS 38.211 [8], clause 4.3.1). The symbol duration is based on the subcarriers spacing the UE is currently configured with. In this release of the specification, only 15 kHz and 30 kHz SCS are applicable and only values 1 … 56 are used. The length of the field is 14 bits. Figure 6.1.3.56-1: Timing Advance Report MAC CE

Regarding the agreed RAN1 observation #1, #2, and #3 in Table 1, we can see that all the potential issues start from the existence of “TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB”. Therefore, enhancements on TA report should be considered as a candidate of general solution for all the identified issues. More specifically, we think the potential enhancements on TA report may include the following topics, which may require some RAN1 discussions:

- Required TA report granularities per NTN payload type, SCS, etc.

Proposal 1. RAN1 to consider introducing finer granularity than 1 msec for TA report MAC CE.

- FFS, required TA report granularities per NTN payload type, SCS, etc.

Collision handling for case 3 and 4

As a potential solution of collision handling for case 3 and 4, simple and fixed rules such as “higher priority on UL transmission than DL reception” or “higher priority on DL reception than UL transmission” can be considered. However, given that the exact value of TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB may not be identified as a single value of a certain time instance (e.g., as a specific slot) due to various reasons including ambiguities on the UE/satellite locations, different SCSs for DL and UL, lack/delay of TA report, etc., an NTN gNB needs to consider multiple pairs of potential DL-UL collisions rather than a clearly defined single pair of DL-UL collision as in TN.

Taking the above into account, we think RAN1 can consider to introduce a new higher layer parameter that can indicate priority of channels/signals associated with the parameter. For case 3 and 4, DL(UL) channel/signal, which is associated with the priority flag, has higher priority than UL(DL) channel/signal without priority flag. The collisions between channels/signals with same priority (e.g., priority flags for all colliding channels/signals, or no priority flags for all colliding channels/signals) can be considered as error cases, as it was in Rel-18. Priority flag can provide gNB a flexible collision handling rules for various collision cases at the cost of limited specification efforts. Figure 1 shows several examples of collision handling for case 3 and 4 based on the priority flag.

Figure 1. Examples of collision handling for case 3 and 4 based on the priority flag.

Proposal 2. For collision case 3 and 4, higher layer parameter(s) comprising priority flag for channels/signals with higher priority than the others can be introduced.

- UE transmits/receives channels/signals configured with the priority flag.

- UE does not expect overlapping between channels/signals with same priority (i.e., “collision between channels/signals without priority flag” or “collision between channels/signals with priority flag” is an error case).

For the cases where multiple pairs of potential DL-UL collisions can be assumed in gNB due to ambiguities on the UE/satellite locations, different SCSs for DL and UL, lack/delay of TA report, etc., it would be worth to introduce an additional higher layer parameter indicating the (range of) possible TA mismatch. Based on this parameter, gNB and UE can have the same understanding on the collision handling behaviour during the configured time duration.

Proposal 3. gNB can configure NTN HD UE with higher layer parameter(s) indicating the (range of) possible TA mismatch.

RACH-less HO for NTN HD UE

From Rel-18 RACH-less HO was introduced for mobile IAB and NTN, then now became a generic feature considering the benefit. For NTN HD FDD UEs, however, the potential impact from RACH-less HO may need to be studied further. After the RACH-less HO in NTN, UE assumes NTA by the configured targetNTA, e.g., 1) same as source in case of intra-satellite HO, or 2) as 0 in other cases. See Table 4 for more details.

Table 4. targetNTA configuration for RACH-less HO [6].

RACH-LessHO-r18 ::= SEQUENCE { targetNTA-r18 ENUMERATED {zero, source} OPTIONAL, -- Need N beamIndication-r18 CHOICE { tci-StateID-r18 TCI-StateId, ssb-Index-r18 SSB-Index } OPTIONAL, -- Need N ... }

When the UE is configured with “zero” for satellite switch, for example, the exact time of UL resynchronization is up to UE implementation, as shown in Table 5. Therefore, at least a certain level of clarification on UE behaviour during such ambiguous duration is required from our understanding. For instance, before receiving the first TA report after satellite switch, gNB and UE may assume the worst case of TA mismatch. In other words, before receiving the first TA report after satellite switch, gNB and UE may consider both the possible minimum and the maximum values of NTA,adjUE to determine the collision cases.

Table 5. Several notes on UE uplink synchronization [6].

NOTE: UE should attempt to re-acquire SIB19 after satellite switch with resynchronization. The exact time is left to UE implementation. NOTE: The exact time when UL synchronisation is obtained (after SIB19 is acquired) is left to UE implementation, which can be from the subframe indicated by epochTime and optionally before the subframe indicated by epochTime.

Proposal 4. RAN1 to study potential issues on satellite switch of NTN HD UE.

- E.g., collision handling after RACH-less HO.

Conclusion

In this contribution, ETRI’s views on NTN HD UEs were shown and the following observations and proposals were made:

Proposal 1. RAN1 to consider introducing finer granularity than 1 msec for TA report MAC CE.

- FFS, required TA report granularities per NTN payload type, SCS, etc.

Proposal 2. For collision case 3 and 4, higher layer parameter(s) comprising priority flag for channels/signals with higher priority than the others can be introduced.

- UE transmits/receives channels/signals configured with the priority flag.

- UE does not expect overlapping between channels/signals with same priority (i.e., “collision between channels/signals without priority flag” or “collision between channels/signals with priority flag” is an error case).

Proposal 3. gNB can configure NTN HD UE with higher layer parameter(s) indicating the (range of) possible TA mismatch.

Proposal 4. RAN1 to study potential issues on satellite switch of NTN HD UE, e.g., collision handling after RACH-less HO.

References

- 3GPP RAN1#116 Chair notes

- 3GPP RAN1#116-bis Chair notes

- 3GPP RAN1#117 Chair notes

- RP-241667, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3, Thales, CATT

- 3GPP TS 38.321 V18.2.0

- 3GPP TS 38.331 V18.2.0