3GPP TSG RAN WG1 Meeting #118                                   	R1-2406744

Maastricht, NL, 19 Aug – 23 Aug, 2024

Source:           BUPT, Spark NZ Ltd

Title:              Discussion on channel model validation of TR38.901 for 7-24 GHz

Agenda item:    	9.8.1

Document for:  	Discussion and Decision

- Introduction

In RAN1 #117, a summary on channel modelling validation for 7-24 GHz was approved [1]:

Agreement - Further study whether the following parameters for existing deployment scenarios is necessary to be updated: Delay spread, pathloss, penetration loss, AoD/AoA/ZoA/Zod spreads, ZoD offset, Angle distribution characteristics (e.g. exponential, Gaussian, Laplacian distributions), XPR, number of clusters, number of rays per cluster, LSP correlations type (e.g. site-specific or all correlated), UE antenna modeling parameters, K factor (mean, variance) - Study of updates to other parameters are not precluded and subject to further study. Agreement - Further study whether/how to model intra-cluster K factor to the TR38.901 models, such as power re-normalization among intra-cluster rays of a cluster so that first intra-cluster ray has K times more average power compared to rest of the intra-cluster rays. - FFS: whether same or different intra-cluster K factor is applied for each clusters - FFS: which applicable deployment scenarios

We rename the intra-cluster K-factor as the intra-cluster power factor to distinguish it from the Ricean K-factor.

In this contribution, we provide our views on the validation and the details for Rel-19 channel modeling enhancements for 7-24 GHz. Key considerations include the pathloss, shadow fading, O-to-I penetration loss, and the modeling of Intra-cluster power factor.

- Pathloss, shadow fading, and penetration modelling

2.1 Channel measurement system setup and scenarios

2.1.1 Pathloss and shadow fading channel measurements

The measurement campaign was carried out in a typical UMa scenario, and the scenario is illustrated in Figure 1. The buildings in the scenario range from 12 to 60 meters in height, and there is also a significant amount of vegetation present, with vegetation heights ranging approximately from 10 to 20 meters. The measurement routes and Tx positions are also marked in Figure 1, with the 3D distance between the Tx and Rx ranging from 70.7 to 304.6 m. Four Tx antenna positions were selected based on the location of the measurement routes.

Figure 1 The measured UMa scenario.

At each measurement point, a total of 11 frequency points were measured: 7, 8, 9, 10.2, 11.2, 11.8, 12.4, 13, 13.6, and 14.8 GHz. The specific configuration of the channel sounder is summarized in Table 1.

Table 1: Measurement settings in the UMa scenario.

| Parameters | Value/Type |
| --- | --- |
| Carrier Frequency | 10.2, 11.2, 11.8, 12.4, 13.0, 13.6, 14.2, 14.8 GHz |
| Bandwidth | 400 MHz |
| Antenna Type | TX: Double-ridged horn antenna RX: Biconical antenna |
| Antenna Height | TX: 62.5 m, RX: 1.85 m |

2.1.2 O2I penetration loss measurements

The measurements for O2I penetration loss were made by placing the antennas corresponding to the frequency bands at each end of the material and connecting the transceiver antennas using a vector network analyser (VNA). The penetration loss was obtained as the difference between the received power levels of the unobstructed path (LOS) and the received power level with the material obstructing the path between TX and RX (NLOS). The unobstructed path (LOS) was first measured without placing the materials, as shown in the left figure of Figure 2. Then the measured material was placed in the middle position of the two antennas and the path loss was measured when it was obstructed by different materials (NLOS), as shown in the right figure of Figure 2. The parameters of different materials used are shown in Table 2.

Figure 2: The penetration loss measurement scenario

Table 2: Summary of Different Materials Settings

| Material | Width(cm) | Height(cm) | Thickness(cm) |
| --- | --- | --- | --- |
| Wooden Board 1 | 130 | 130 | 0.6 |
| Wooden Board 2 |  |  | 1.0 |
| Wooden Board 3 |  |  | 1.4 |
| Glass 1 | 120 | 120 | 0.8 |
| Glass 2 | 120 | 120 | 0.8 |
| Concrete | 80 | 80 | 4 |

Among them, wooden boards 1 and 2 are pressed boards, wooden board 3 is solid board, the glass 1 and 2 used in this measurement are common ordinary double-layer glass and frosted glass, respectively.

2.2 Results analysis

2.2.1 Path loss and shadow fading

The parameters of the CI model are shown in Table 3. From the frequency perspective, there is no significant frequency dependency observed in PLE across all scenarios. However, shadow fading exhibits clear frequency dependency in both LoS and NLoS scenarios, with an increasing trend as the frequency rises.

Table 3: The path loss results of measurement and 3GPP model in the UMa scenario.

| Frequency (GHz) | LoS |  | NLoS |  |
| --- | --- | --- | --- | --- |
|  | PLE | σSF | PLE | σSF |
| 7 | 2.28 | 2.73 | 3.05 | 3.77 |
| 8 | 2.31 | 2.94 | 3.05 | 3.44 |
| 9 | 2.26 | 2.83 | 3.02 | 3.53 |
| 10.2 | 2.15 | 2.84 | 3.04 | 5.59 |
| 11.2 | 2.14 | 3.39 | 3.04 | 4.72 |
| 11.8 | 2.21 | 4.25 | 3.1 | 4.93 |
| 12.4 | 2.16 | 3.85 | 3.1 | 5.31 |
| 13 | 2.21 | 3.77 | 3.2 | 5.42 |
| 13.6 | 2.18 | 3.71 | 3.06 | 5.47 |
| 14.2 | 2.22 | 4.21 | 3.12 | 5.58 |
| 14.9 | 2.21 | 4.67 | 3.09 | 5.12 |
| 3GPP | 2.2 | 4 | 3.9 | 6 |

By comparing the measured results with the path loss models in the 3GPP TR38.901 UMa scenario, it is observed that the PLE in LoS scenarios is similar to the PLE in the 3GPP TR38.901, whereas in NLoS scenarios, the PLE at each frequency point is lower than the PLE in the 3GPP TR38.901 (3.9).

As mentioned earlier, we observed potential frequency dependence in the results of SF, which is not considered in the 3GPP TR38.901. The 3GPP TR38.901 instead establishes the SF as a value that varies only with the scenario (4 dB for LoS and 6 dB for NLoS). Therefore, we use a logarithmic model to describe the relationship between frequency and SF, i.e.,

σSF=a∙log10(1+fc)−b.

Figure 3 shows the fitting results. In the LoS scenario, a = 6.01 and b = -2.96, while in the NLOS scenario, a = 7.2 and b = -3.

Figure 3: The values of σSF and the fitted model curve.

Observation 1：The measurement results in the UMa scenario show that the shadow fading σSF exhibit frequency dependence, increasing with frequency.

Proposal 1：RAN1 needs to further model the frequency dependence of shadow fading under different scenarios.

2.2.2 O2I penetration loss

The data obtained from the measurement were processed to obtain the value of penetration loss, which was plotted as a linear function for fitting, as shown in Figures 4, 5 and 6. The relevant results were summarized in Table 4.

Figure 4: Comparison of wood penetration loss measurement and fitting with 3GPP model

Figure 5: Comparison of glass penetration loss measurement and fitting with 3GPP model

Figure 6: Comparison of concrete penetration loss measurement and fitting with 3GPP model

Table 4: The final measurement results of three different materials.

| Material |  | Slope | Intercept | Difference (Average of 3GPP and fitted model differences) |
| --- | --- | --- | --- | --- |
| Glass | Standard multi-pane glass in 3GPP | 0.20 | 1.20 | - |
|  | Glass 1 | 0.30 | 2.30 | 0.66 |
|  | Glass 2 | -0.06 | 3.94 | -1.30 |
| Concrete | Concrete in standard | 4.00 | 5.00 | - |
|  | Concrete | 0.95 | 9.83 | 25.67 |
| Wood | Wood in standard | 0.12 | 4.85 | - |
|  | Wooden board 1 | 0.23 | 1.75 | 2.00 |
|  | Wooden board 2 | 0.23 | 1.52 | 2.23 |
|  | Wooden board 3 | 0.07 | 3.55 | 1.80 |

According to the results, the penetration loss values of the materials generally exhibit a linear relationship, increasing with frequency and thickness. In order to analyse the differences between the models obtained from the measurements and the TR 38.901 material penetration loss model, we checked the applicability using the average of 3GPP and fitted model differences. For wood, the differences are mainly centred around 2 dB, suggests that the 3GPP model is slightly higher compared to the fitted model, but the difference is not significant. Meanwhile, the difference for concrete reaches about 26 dB. According to Figure 5, it can be seen that the glass measurements do not exactly obey a linear distribution, indicating a large gap between the TR 38.901 model and the measured data. The similar phenomenon is observed in literature [4].

The TR 38.901 material penetration loss model for wood closely match with the measured penetration loss value, the overall data shows low dispersion and the differences are small. On the other hand, there is a significant discrepancy between the TR 38.901 material penetration loss model for standard multi-pane glass and concrete, indicating that it has the greatest prediction error for these two materials and requires modifications to improve its prediction accuracy.

Observation 2: The maximum difference between the measurement result of wood penetration loss and 3GPP is not more than 3 dB, while the concrete model has an error about 40 dB. Glass measurements do not exactly follow a linear distribution.

Proposal 2: RAN1 needs to further validate the O2I penetration loss considering materials of different thicknesses, especially concrete materials.

- Intra-cluster power factor model

3.1 Channel measurement system setup and scenarios

As Rel-19 considers the existing scenario, we carried out the channel measurement work in outdoor UMa scenario. In the 7-24 GHz frequency band, 13 GHz is selected as the typical frequency point. The uniform planar array (UPA) (128 array elements) is placed on the transmitting (TX) end. The omni-directional array (ODA) with 64 array elements is placed on the receiving (RX) end. At the Tx end, the vector signal generator (R&S SMW 200A) is used to generate a Pseudo Noise (PN) sequence with a central frequency of 13 GHz and a bandwidth of 400 MHz. The spectrum analyzer (R&S FSW 43) is utilized to receive the signals at the Rx side. The channel sounder configuration is illustrated in Figure 7 and Table 5.

Figure 7: The channel measurement system.

Table 5: Channel measurement configuration

| Parameters | Value/Type |
| --- | --- |
| Scenario | UMa |
| Carrier frequency | 13 GHz |
| Chip length | 511 |
| Symbol rate | 200 Msym/s |
| Bandwidth | 400 MHz |
| Tx | UPA (128 elements) |
| Rx | ODA (64 elements) |
| Tx antenna element gain | 4.5 dBi |
| Rx antenna element gain | 5.0 dBi |
| Tx antenna height in UMa | 27.5 m |
| Rx antenna height in UMa | 1.8 m |
| Polarization | ±45° |

The TX is placed on the roof of the teaching experiment complex building on the Shahe Campus of Beijing University of Posts and Telecommunications, with a height of 27.5 m (higher than the roof). The RX is placed on the ground and the measurement is moved along the routes shown in Figure 8. The interval distance between adjacent Rx points is 20 m. Route 1, 2, 2 ' and 3' are LOS environments.

| (a) | (b) |
| --- | --- |

Figure 8: The channel measurement campaign in the UMa scenario (a) measurement routes (b) measurement environments.

3.2 Results analysis

3.2.1 Cluster number

The number of clusters in LOS and NLOS scenarios is studied at 13 GHz. The measured cluster number is estimated using SAGE algorithm and clustered using KPowerMeans algorithm. We conducted simulation experiments based on the 3GPP channel model framework. We simulate the number of clusters using the parameters in the large-scale parameter table (Table 7.5-6) of 3GPP TR 38.901, and use a threshold (-25dB) to cut off low-power clusters.

Figure 9 shows the cluster power distribution of one of the samples. Table 6 provides the average number of clusters for 100 samples. From the Figure 9 and Table 6, it is found that in LOS scenarios, increasing the power of LOS clusters due to the K factor results in the -25 dB threshold cutting off more clusters. In the NLOS scenario, the -25 dB threshold hardly cuts off clusters. The CDF of cluster number is illustrated in Figure 10, which demonstrates the cluster number obtained by measurement is less than the values in 3GPP TR 38.901.

- (b)

Figure 9: The distribution of cluster power in the UMa scenario. (a) LOS. (b) NLOS.

(a)                                                                           (b)

Figure 10: The CDF of cluster number in the UMa scenario. (a) LOS. (b) NLOS.

Table 6 Cluster number

|  |  | Cluster number | Cluster number with -25 dB |
| --- | --- | --- | --- |
| LOS | Simulated results based on 3GPP | 12 | 7 |
|  | Measured results | 9 | \ |
| NLOS | Simulated results based on 3GPP | 20 | 19 |
|  | Measured results | 14 | \ |

Observation 3: The measured cluster number is smaller than the 3GPP model, and there is still a gap after interception using the threshold of -25 dB.

Proposal 3: The number of clusters in TR 389.01 needs modification.

3.2.2 Intra-cluster power factor

We rename the intra-cluster K-factor (ICK) as the intra-cluster power factor (ICP) to distinguish it from the Ricean K-factor. The ICP is utilized to address the issue that the 3GPP channel model does not accurately characterize the sparsity [2]. The measured MPCs are clustered for joint delay, angle, and power using the KPowermeans algorithm [3] in the UMa scenario at 13 GHz to verify the distribution of ray power. The clustering results indicate a similar pattern to the indoor scenario, that is, there is a dominant ray within each cluster which contains most of the cluster power [2]. Figure 11 illustrates an example of the power distribution within a cluster in both LoS and NLoS conditions. The first cluster contains the LoS component, which has strong power. It is evident that the ray power in NLoS condition is smaller. However, there are also dominant rays (specular reflection and first-order diffraction) with strong power in the NLoS condition. According to the clustering results, it is found that different clusters have different ICP values due to varying dominant ray propagation mechanisms, leading to different power ratios.

- (b)

(c)                           (d)

Figure 11. The ray power distribution within the cluster at 13 GHz. (a) 1st cluster in LoS. (b) 2nd cluster in LoS. (c) 1st cluster in NLoS. (d) 2nd cluster in NLoS.

Therefore, the ICP model is proposed. Furthermore, the simulation experiments are conducted to verify the performance of the model. The measurements show that for the most part the cluster with smaller delay have stronger dominant ray, while the power of the dominant ray in the cluster with larger delay is closer to the remain weak rays, as illustrated in Figure 9.

We calculate the difference of cluster delay to observe the ICP variation. The cluster delay is normalized as expressed

τn=τn'−min⁡τn'

where τn' represents the true value of the cluster delay, τn represents the normalized cluster delay. It is found that the ICP value is related to the cluster delay and DS by fitting the results of channel snapshots from multiple measurement positions, as illustrated in Figure 12. The DS indicates the dispersion degree of the MPC delay. Since the DS is impressed by power, the larger DS represents that the strong paths are more dispersed. The ICP model is expressed as

ICP=b∙10a∙τnDS

Table 6 provides the reference coefficients of the ICP model in UMa scenarios at mid-band. b represents the ICP of the cluster with the minimum delay. In the LoS scenarios, the cluster with the minimum delay contains the LoS path which has the strongest power. It is found that b in LoS scenarios is larger than NLoS scenarios, which means the ICP of the LoS cluster is larger than remain clusters.

(a)                                         (b)

Figure 12: ICP vs. the ratio of the normalized Cluster Delay with delay spread. (a) In LoS scenario. (b) In NLoS scenario.

Table 6: Coefficients of the ICP model

| Scenarios | a | b |
| --- | --- | --- |
| LOS | -0.04 | 2.8 |
| NLOS | -0.04 | 1.7 |

The ICP uses delay spread and normalized cluster delay (obtained by Step 5) to change the ray power in the channel coefficients 7.5-22. The channel coefficients with ICP are given by:

Hu,s,nNLOSt=ICP∙PnICP+1Frx,u,θθn,1,ZOA,ϕn,1,AOAFrx,u,ϕθn,1,ZOA,ϕn,1,AOATexpjΦn,1θθκn,1−1expjΦn,1θϕκn,1−1expjΦn,1ϕθexpjΦn,1ϕϕ

Ftx,s,θθn,1,ZOD,ϕn,1,AODFtx,s,ϕθn,1,ZOD,ϕn,1,AODexpj2πrrx,n,1T∙drx,uλ0expj2πrtx,n,1T∙dtx,sλ0expj2πrrx,n,1T∙vtλ0

+PnICP+1M−1m=2MFrx,u,θθn,m,ZOA,ϕn,m,AOAFrx,u,ϕθn,m,ZOA,ϕn,m,AOATexpjΦn,mθθκn,m−1expjΦn,mθϕκn,m−1expjΦn,mϕθexpjΦn,mϕϕ

Ftx,s,θθn,m,ZOD,ϕn,m,AODFtx,s,ϕθn,m,ZOD,ϕn,m,AODexpj2πrrx,n,mT∙drx,uλ0expj2πrtx,n,mT∙dtx,sλ0expj2πrrx,n,mT∙vtλ0(7.5-22)

The performance of ICP is verified in UMa scenarios using the Gini index. We simulate the channel sparsity based on the 3GPP model framework and compare the results from measurement, 3GPP model, and ICP model, as illustrated in Figure 13.

(a)                                                                  (b)

Figure 13. The CDF of Gini index generated by measured channels, by the 3GPP channel model, and by the 3GPP model with proposed intra-cluster power allocation model (modified model). (a) LoS scenario. (b) NLoS scenario.

The yellow lines represent the results at 6 GHz, and the blue lines represent the results at 13 GHz. Similarly, the results of MIMO channel measurements in both LoS and NLoS conditions in the UMa scenario show that the channels at 13 GHz is sparser than at 6 GHz because the Gini index is closer to 1 at 13 GHz. It is found that the Gini index in NLoS condition is smaller than that in LoS condition because the power of the LoS components contains much power. The LoS component is the strongest dominant ray leading to an increase in the Gini index.

The dashed lines in Figure 11 show that the Gini index obtained from the 3GPP channel model is generally smaller than the measurement results. Additionally, the Gini index obtained from the 3GPP channel model cannot reflect the sparsity differences between frequencies, especially in NLoS condition.

The dotted lines in Figure 11 show that the Gini index obtained by the modified 3GPP channel model with ICP are closer to the measurement results. These results indicate that the modified 3GPP channel model accurately characterizes the sparsity of mid-band MIMO channels in the UMa scenario. The performance is also proved in indoor scenarios [2].

Observation 4: The value of ICP is different in each cluster, and it is related to the cluster delay and delay spread.

Observation 5: The ICP model has been proved to be applicable to indoor [2] and UMa scenarios. When ICP is introduced into the 3GPP model, the level of sparsity is closer to the measured results.

Proposal 4: To distinguish the intra-cluster K-factor (ICK) from Ricean K-factor, ICK is renamed as the intra-cluster power factor (ICP).

- Conclusion

In this contribution, we provide our views on the validation and the details for Rel-19 channel modeling enhancements for 7-24 GHz. For example: the pathloss, shadow fading, O-to-I penetration loss, , number of clusters, and the modeling of intra-cluster power factor. The observation and proposals are as follows:

Observation 1：The measurement results in the UMa scenario show that the shadow fading σSF exhibit frequency dependence, increasing with frequency.

Observation 2: The maximum difference between the measurement result of wood penetration loss and 3GPP is not more than 3 dB, while the concrete model has an error about 40 dB. Glass measurements do not exactly follow a linear distribution.

Observation 3: The measured cluster number is smaller than the 3GPP model, and there is still a gap after interception using the threshold of -25 dB.

Observation 4: The value of ICP is different in each cluster, and it is related to the cluster delay and delay spread.

Observation 5: The ICP model has been proved to be applicable to indoor [2] and UMa scenarios. When ICP is introduced into the 3GPP model, the level of sparsity is closer to the measured results.

Proposal 1：RAN1 needs to further model the frequency dependence of shadow fading under different scenarios.

Proposal 2: RAN1 needs to further validate the O2I penetration loss considering materials of different thicknesses, especially concrete materials.

Proposal 3: The number of clusters in TR 389.01 needs modification.

Proposal 4: To distinguish the intra-cluster K-factor (ICK) from Ricean K-factor, ICK is renamed as the intra-cluster power factor (ICP).

References

[1] R1-2403630, Summary #4 of discussions for Rel-19 7-24 GHz Channel Modeling Validation, RAN WG1 Meeting#117, 20th – 24th May, 2024.

[2] X. Liu et al., "Channel Sparsity Variation and Model-Based Analysis on 6, 26, and 105 GHz Measurements," in IEEE Transactions on Vehicular Technology, vol. 73, no. 7, pp. 9387-9397, July 2024.

[3] L. Rokach, “A survey of Clustering Algorithms,” in Data Mining & Knowledge Discovery Handbook, vol. 16, no. 3, pp. 269-298, 2009.

[4] Y. C. Lee, et al., “Impact of Window Penetration Loss on Building Entry Loss From 3.5 to 24 GHz,” in IEEE Access, vol. 9, pp. 138571-138579, 2021.