3GPP TSG-RAN WG1 Meeting #118	R1-2406780

Maastricht, The Netherlands, August 19th – 23rd, 2024

Source:	MediaTek Inc.

Title:	Discussion on IoT-NTN uplink capacity and throughput enhancement

Agenda item:	9.11.4

Document for:	Discussion

# Introduction

  - Study then specify, if beneficial, enhancements to enable multiplexing of multiple UEs (e.g. up to the min of 4 and the maximum allowed by the existing UL and DL signalling) in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH [RAN1, RAN2]

    - Multi-tone support for 15 kHz SCS should also be considered

Note: Impact of impairment shall be taken into account

  - Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an EDT transaction [RAN2]:

    - Msg3 transmission without msg1/RAR

    - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete

In RAN#103 [1], a revised WID on IoT NTN enhancements was endorsed for Release 19. This contribution aims to discuss Uplink Capacity/Throughput Enhancement as shown below. We’ll outline enhancements for IoT-NTN uplink capacity and throughput, focusing on code optimization, single-tone prioritization, NPUSCH performance, OCC mapping, and UE pairing.

# OCC for NPUSCH

- For 3.75kHz single-tone OCC for NPUSCH format 1, RAN1 supports either symbol-level OCC or slot-level OCC. Other OCC schemes are not pursued.

- For 15kHz single-tone OCC for NPUSCH format 1, RAN1 supports either symbol-level OCC or slot-level OCC. Other OCC schemes are not pursued.

- The Rel-17 guard period locations and length for NB-IoT 3.75kHz UL slot are preserved when OCC is applied to NPUSCH format 1.

The following agreements were made in RAN1#117:

- OCC multiplexing is not supported between a UE using NPUSCH format 1 with 3.75kHz SCS and another UE using NPUSCH format 1 with 15kHz SCS.

- For OCC of NPUSCH format 1, RAN1 will not consider multiplexing more than 4 UEs.

The following agreements were made in RAN1#116bis:

A ST 15 kHz is specified in TS 38.211 Table 10.1.2.3-1 to be NslotsUL=16 with a slot duration 0.5 ms and ST 3.75 kHz kHz is NslotsUL=16 with a slot duration 2 ms. The time duration of OCC applied to repetitions of NPUSCH Format 1 is 16 ms for OCC length 2 and 32 ms for OCC length 4 with ST 15 kHz, and 64 ms for OCC length 2 and 128 ms for OCC length 4 with ST 3.75 kHz

Phase rotation over time duration X ms as summarize in Table 1can be calculated as

- Δφ = 2π (rad) × timing drift (us/s) × X (ms) × channel bandwidth (Hz) /2

- Δφ = 2π (rad) × CFO × X (ms) × Fc/2

Where max time drift = 80 ppm and max CFO = ±0.1 ppm

Observation 1: The maximum phase rotation due to timing drift or CFO may allow inter-slot OCC with NPUSCH Format 1 with OCC length 2 or 4 with ST 15 kHz, and OCC length 2 with ST 3.75 kHz.

| Max ΔΘ [deg] @ Timing drift 80 ppm |  | Max ΔΘ [deg] @ Max CFO ±0.1 ppm @ 2 GHz |  |
| --- | --- | --- | --- |
| ST 15 kHz | ST 3.75 kHz | ST 15 kHz | ST 3.75 kHz |
| 1.728 deg (OCC=2) | 1.728 (OCC=2) | 92 deg (OCC=2) | 368 deg (OCC=2) |
| 3.456 deg (OCC=4) | 3.456 (OCC=4) | 184 deg (OCC=4) | 1472 deg (OCC=4) |

Table 1: Max phase rotation due to time drift and CFO  with OCC schemes

Slot-level OCC:

In slot-level OCC, a TB is mapped to NRU resource units  NscRU NslotUL NsymbUL are repeated K times, where NRU, NscRU , NslotUL , NsymbUL are specified in TS 36.211 Table 10.1.2.3-1. Figure 1 shows an example with OCC length 2.

Observation 2: Slot-level OCC NPUSCH Format 1 can re-use legacy resource allocation in time domain for block-wise spreading of DFT-s-OFDM symbols y(n) with OCC sequence wi(m) of length m=NSFNPUSCH  based on the following equation:

zl,mMscRUNsymbUL+n=wimyn

- n=0..NscRUNsymbUL-1, l=0..NRUNslotUL-1,  m=0..NSFPUSCH-1

where NRU, NscRU , NslotUL , NsymbUL are specified in TS 36.211 Table 10.1.2.3-1

Observation 3: Slot-level OCC with NPUSCH Format 1 can use RV cycling applied every NRU resource units NscRU NslotUL NsymbUL

Proposal 1: RAN1 to support slot-level OCC PUSCH with OCC length 2 or 4 for ST 15 kHz and 3.75 kHz

Figure 1: Slot-level OCC wih OCC length = 2

Symbol-level OCC:

In symbol-level OCC, a TB is mapped to NRU resource units  NscRU NslotUL NsymbUL are repeated K times, where NRU, NscRU , NslotUL , NsymbUL are specified in TS 36.211 Table 10.1.2.3-1. Figure 1 shows an example with OCC length 2.

Observation 4: Symbol-level OCC NPUSCH Format 1 can re-use legacy resource allocation in time domain for block-wise spreading of DFT-s-OFDM symbols y(n) with OCC sequence wi(m) of length m=NSFNPUSCH  based on the following equation:

zl,mMscRU+n=wimyn			Equation (2)

- n=0..NscRU-1, l=0..NRUNslotULNsymbUL-1,  m=0..NSFPUSCH-1

where NRU, NscRU , NslotUL , NsymbUL are specified in TS 36.211 Table 10.1.2.3-1

Observation 5: Symbol-level OCC with NPUSCH Format 1 can use RV cycling applied every NRU resource units NscRU NslotUL NsymbUL

Figure 2: Symbol-level OCC wih OCC length = 2

# OCC for NPRACH

- Inter-repetition OCC for NPRACH is not studied further in RAN1.

- The study of OCC for NPRACH does not consider NPRACH format 2.

The following agreements were made in RAN1#117, RAN1#116bis

- At least the following NPRACH OCC schemes are considered by RAN1 for study:

- Intra-symbol group OCC

- Inter-symbol group(s) OCC

- Inter-repetition OCC

The following agreements were made in RAN1#116

Figure 1 shows the NPRACH Format #0 and #1.

Figure #1 NPRACH Format #0 and #1

| Preamble Format | CP length, TCP | TSEQ | SCS | Length |
| --- | --- | --- | --- | --- |
| 0 | 66.67 us (2048.Ts) | 1333.3 us (5*8192.Ts) | 3.75 kHz | 5.6 ms |
| 1 | 266.67 us (8192.Ts) | 1333.3 us (5*8192.Ts) | 3.75 kHz | 6.4 ms |

The following should be considered in the NPRACH study:

- CP insertion for intra symbol group OCC

- Maintaining phase continuity and Timing Advance estimation with Frequency Hopping for inter-symbol OCC

  - Option 1: OCC mapped across preamble symbol group within 3.75 kHz FO jump

  - Option 2: OCC mapped across preamble symbol groups within 22.5 kHz FO jump

Observation 6: Inter-symbol group OCC for NPRACH Format 0 or 1:

- With OCC length 2: OCC mapped across preamble symbol group within 3.75 kHz FO jump

- With OCC length 4: OCC mapped across preamble symbol group within 3.75 kHz and 22.5 kHz FO jump

Observation 7: OCC for NPRACH does not apply to Rel-19 enhanced EDT procedure since Msg1 and Msg2 of legacy EDT procedure are skipped.

# DM RS for OCC

The following agreements were made in RAN1#117 and RAN1#116bis:

For the time-domain DMRS pattern (including blanked DMRS, if any):

- For 15kHz single-tone, RAN1 strives to reuse the Rel-17 DMRS pattern

- For 3.75kHz single-tone, RAN1 studies

  - Rel-17 DMRS pattern

  - A new DMRS pattern

- The DMRS overhead (including blanked DMRS, if any) for OCC is the same as for Rel-17

The Rel-17 guard period locations and length for NB-IoT 3.75kHz UL slot are preserved when OCC is applied to NPUSCH format 1.

For single-tone DMRS when OCC is applied to NPUSCH format 1, RAN1 considers at least the following for further study:

- TDM of DMRS. The time domain locations of DMRS for different UEs are different. No OCC is applied for the DMRS of different UEs.

  - FFS: Detailed mapping

- CDM of DMRS. The time domain locations of DMRS for different UEs are the same. Different OCCs are applied for the DMRS of different UEs.

  - FFS: Detailed mapping

- Other schemes are not precluded, including combinations of the above

For Time-domain OCC slot-level the DM RS symbols are within each of the NPUSCH slots in NPUSCH Format 1 repetition unit. This presents the advantage that no enhancement of DM RS is needed.

DM RS for each multiplexed UE needs to differentiated at the eNB receiver for channel estimation when OCC is used.

In CDM of DM RS, OCC is applied to consecutive DM RS symbols.  For example, with OCC length 2 and slot-level OCC, OCC sequence [+1 +1] for UE#1 is applied to DM RS symbols in 1st slot 2nd slot; OCC sequence [+1 -1] for UE#2 is applied to DM RS symbols in 1st slot and 2nd slot.

TDM with R17 DM RS patterns with reduced number of DM RS symbols per multiplexed UE. For example, with OCC length 2 and slot-level OCC, UE#1 uses DM RS in every other slot starting with 1st slot; UE#1 uses DM RS in every other slot starting with 2nd slot.

Observation 8: R17 DM RS patterns for 3.75kHz single tone can be re-used

- CDM with OCCs applied to R17 DM RS patterns for single tone 3.75 kHz.

- TDM with R17 DM RS patterns with reduced number of DM RS symbols per multiplexed UE.

Both CDM DM RS and TDM DM RS may have impact on channel estimation.

DM RS symbols are within each of the NPUSCH slots in NPUSCH Format 1 repetition unit for Time-domain OCC slot-level. R17 DM RS patterns for 3.75kHz single-tone can be re-used without impact on specifications and complexity.

Proposal 2: RAN1 can study need for new DM RS for ST 3.75 kHz.

# Conclusion

In this contribution, we have the following observations and proposals:

OCC for NPUSCH:

Observation 1: The maximum phase rotation due to timing drift or CFO may allow inter-slot OCC with NPUSCH Format 1 with OCC length 2 or 4 with ST 15 kHz, and OCC length 2 with ST 3.75 kHz.

Observation 2: Slot-level OCC NPUSCH Format 1 can re-use legacy resource allocation in time domain for block-wise spreading of DFT-s-OFDM symbols y(n) with OCC sequence wi(m) of length m=NSFNPUSCH  based on the following equation:

zl,mMscRUNsymbUL+n=wimyn

- n=0..NscRUNsymbUL-1, l=0..NRUNslotUL-1,  m=0..NSFPUSCH-1

where NRU, NscRU , NslotUL , NsymbUL are specified in TS 36.211 Table 10.1.2.3-1

Observation 3: Slot-level OCC with NPUSCH Format 1 can use RV cycling applied every NRU resource units NscRU NslotUL NsymbUL

Proposal 1: RAN1 to support slot-level OCC PUSCH with OCC length 2 or 4 for ST 15 kHz and 3.75 kHz

Observation 4: Symbol-level OCC NPUSCH Format 1 can re-use legacy resource allocation in time domain for block-wise spreading of DFT-s-OFDM symbols y(n) with OCC sequence wi(m) of length m=NSFNPUSCH  based on the following equation:

zl,mMscRU+n=wimyn			Equation (2)

- n=0..NscRU-1, l=0..NRUNslotULNsymbUL-1,  m=0..NSFPUSCH-1

where NRU, NscRU , NslotUL , NsymbUL are specified in TS 36.211 Table 10.1.2.3-1

Observation 5: Symbol-level OCC with NPUSCH Format 1 can use RV cycling applied every NRU resource units NscRU NslotUL NsymbUL

OCC for NPRACH:

Observation 6: Inter-symbol group OCC for NPRACH Format 0 or 1:

- With OCC length 2: OCC mapped across preamble symbol group within 3.75 kHz FO jump

- With OCC length 4: OCC mapped across preamble symbol group within 3.75 kHz and 22.5 kHz FO jump

Observation 7: OCC for NPRACH does not apply to Rel-19 enhanced EDT procedure since Msg1 and Msg2 of legacy EDT procedure are skipped.

DM RS for OCC:

Observation 8: R17 DM RS patterns for 3.75kHz single tone can be re-used

- CDM with OCCs applied to R17 DM RS patterns for single tone 3.75 kHz.

- TDM with R17 DM RS patterns with reduced number of DM RS symbols per multiplexed UE.

Proposal 2: RAN1 can study need for new DM RS for ST 3.75 kHz.

# Appendix

The following Tables are copied below from the specifications of NB-IoT for convenience:

- TS 36.213 Table 16.5.1.2-2: Transport block size (TBS)

- TS 36.213 Table 16.5.1.1-2: Number of resource units () for NPUSCH

- TS 36.213 Table 16.5.1.1-3: Number of repetitions () for NPUSCH

- TS 36.211 Table 10.1.2.3-1: Supported combinations of , , and  for frame structure type 1

TS 36.211 Table 10.1.2.3-1: Supported combinations of , , and  for frame structure type 1.

| NPUSCH format |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | 3.75 kHz | 1 | 16 | 7 |
|  | 15 kHz | 1 | 16 |  |
|  |  | 3 | 8 |  |
|  |  | 6 | 4 |  |
|  |  | 12 | 2 |  |
| 2 | 3.75 kHz | 1 | 4 |  |
|  | 15 kHz | 1 | 4 |  |

TS 36.213 Table 16.5.1.1-2: Number of resource units () for NPUSCH.

| 0 | 1 |
| --- | --- |
| 1 | 2 |
| 2 | 3 |
| 3 | 4 |
| 4 | 5 |
| 5 | 6 |
| 6 | 8 |
| 7 | 10 |

TS 36.213 Table 16.5.1.1-3: Number of repetitions () for NPUSCH.

| 0 | 1 |
| --- | --- |
| 1 | 2 |
| 2 | 4 |
| 3 | 8 |
| 4 | 16 |
| 5 | 32 |
| 6 | 64 |
| 7 | 128 |

TS 36.213 Table 16.5.1.2-2: Transport block size (TBS) table for NPUSCH.

|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 16 | 32 | 56 | 88 | 120 | 152 | 208 | 256 |
| 1 | 24 | 56 | 88 | 144 | 176 | 208 | 256 | 344 |
| 2 | 32 | 72 | 144 | 176 | 208 | 256 | 328 | 424 |
| 3 | 40 | 104 | 176 | 208 | 256 | 328 | 440 | 568 |
| 4 | 56 | 120 | 208 | 256 | 328 | 408 | 552 | 680 |
| 5 | 72 | 144 | 224 | 328 | 424 | 504 | 680 | 872 |
| 6 | 88 | 176 | 256 | 392 | 504 | 600 | 808 | 1000 |
| 7 | 104 | 224 | 328 | 472 | 584 | 712 | 1000 | 1224 |
| 8 | 120 | 256 | 392 | 536 | 680 | 808 | 1096 | 1384 |
| 9 | 136 | 296 | 456 | 616 | 776 | 936 | 1256 | 1544 |
| 10 | 144 | 328 | 504 | 680 | 872 | 1000 | 1384 | 1736 |
| 11 | 176 | 376 | 584 | 776 | 1000 | 1192 | 1608 | 2024 |
| 12 | 208 | 440 | 680 | 1000 | 1128 | 1352 | 1800 | 2280 |
| 13 | 224 | 488 | 744 | 1032 | 1256 | 1544 | 2024 | 2536 |

# References

- RP-234775, Moderator (Thales), New WID: Non-Terrestrial Networks (NTN) for NR Phase 3, RAN#103, Maastricht, The Netherlands, March 18-22, 2024

- R1-2405494, Moderator (Sony), FL Summary #2 for IoT-NTN , RAN1#117, Fukuoka, Japan, 20th-24th May 2024, 2024