3GPP TSG RAN WG1 #118		        R1-2406839

Maastricht, NL, August 19th – 23rd, 2024

Agenda Item:	9.4.1.2

Source:	Apple

Title:	On device architecture for AIoT

Document for:	Discussion/Decision

# Introduction

In this contribution, we provide our views on the remaining aspects on the device architectures relevant for device type1, device type 2a and device type 2b.

# Discussion

## Device 1

One key aspect for device type 1 is the feasibility of small frequency shift in the order few 10s of kHz to allow for multiplexing in frequency domain. In our view, such small frequency shift could be possible based on baseband processing. Specifically, line coding schemes with different rates could be utilized for achieving the desired small frequency shift. For example, in UHF RFID, Miller coding with different values of M allows to achieve different frequency shifts, as illustrated in Figure 1 [3]. Based on this observation, it should be feasible to achieve small frequency shift within the constraints of device type 1

Figure 1: Illustration of small frequency shift with Miller coding with different values of M

Proposal 1: For device type 1, small frequency shift in order of few 10s of kHz is achieved by baseband processing, e.g. by line coding and no dedicated block is studied for small frequency shift

Another aspect discussed is related to the sequence detector. In the currently agreed device architecture, there is no inclusion of not any block for power detection / sequence detection. In our view, sequence detection based on correlation of sequence is not feasible for such low power devices. Therefore, no additional block for sequence detection should be considered for device type 1. For power detection, we can further discuss if any additional block is needed, or RF-ED receiver blocks currently agreed are sufficient or not.

Proposal 2: For device type 1, sequence correlation-based detection should not be further considered for device architecture

Furthermore, power conversion efficiency is also considered for further discussion in the device architecture. In [4], a comparison is provided in terms of activation threshold and peak PCE corresponding to different CMOS technologies, circuit design and operating frequency. Below table from [4] provides a good summary for the achievable activation threshold.

Table 1: Comparison of different RF energy harvesters

Another good survey of RF energy harvesting circuits is discussed in [5]. In the table below from [5], for different topologies of rectifier circuit and different diodes components, peak PCE is shown. In addition, [7] also provides an analysis in terms of PCE corresponding to different levels of input powers for different circuit topologies, as shown in Figure 3.

Table 2: Comparison of different RF energy harvesters

Figure 2: Comparison of PCE for different circuit topologies

Based on the literature survey in [4] and [5], we can make following observations and proposal:

Observation 1: Peak power conversion efficiency can be achieved in the range of 75%-85% for typically high SNR regime

Observation 2: Power conversion efficiency of less than 40% is achievable in the range of achievable activation threshold of -20dBm to -25 dBm

One another key aspect for consideration is the impact of waveform on device architecture. Currently, three options are considered including OOK, PSK and FSK. In [6], a survey of low-power transceivers is provided. In Figure 3 from [6], a comparison of power consumption of the low-power receivers using OOK and FSK/PSK is provided. For OOK based receiver, power consumption as low as ~0.1µW is achievable. For FSK/PSK based receiver, the power consumption targets need to be higher, e.g. more than 10µW. Based on this observation, it seems unlikely to accommodate FSK/PSK blocks for device type 1 and OOK should be the baseline consideration

Furthermore, in [6], for different low-power receivers, a comparison of power consumption and corresponding sensitivity is shown, as illustrated in Figure 4 (a) and a comparison of power consumption and corresponding gross data rate is shown, as illustrated in Figure 4 (b).

Figure 3: Comparison of power consumption for OOK and FSK/PSK based receivers

- Power consumption vs Sensitivity

(b) Power consumption vs Gross Data Rate

Figure 4: Comparison of power consumption for low-power receivers

Observation 3: For device type 1, OOK waveform can be accommodated in the constraints of device architecture based on its power consumption requirements

Observation 4: For device type 1, FSK/PSK waveform may not be able to be accommodated in the constraints of device architecture based on their power consumption requirements

## Device 2a

In RAN1#117, following agreements have been made related to device type 2a in [2]

Observation

Reflection amplifier with following characteristics could be considered for device 2a.

- Direction of amplification

  - Uni-directional reflection amplifier (baseline) can amplify backscattered signal in D2R which can improve D2R link budget.

  - Bi-directional amplifier can amplify both signal in R2D and backscatter signal in D2R at least when R2D and D2R are in the same spectrum.

  - Bi-directional amplifier has higher complexity, higher noise figure, and reduced isolation between tx and rx path.

- Amplification gain ranges from 10 to 20dB.

- Power consumption of reflection amplifier is in the range of a tens of uW to 100s of uW.

- Reflection amplifier can operate in FDD frequency bands.

- Reflection amplifier bandwidth can support 10s of MHz.

- Note: reflection amplifier can get unstable when the input power exceeds a certain value, which may be frequency-dependent.

Observation

For large frequency shift:

- Large frequency shift can be used in shifting reflected signal in tens of MHz, e.g., from FDD DL to FDD UL frequency or vice versa.

- Large frequency shift consumes 10s of uW to 100s of uW.

- Large frequency shift is not feasible for device 1.

- Large frequency shift requires a clock for IF generation which is accurate enough to avoid large guard band and interference to adjacent channels/bands.

- Large frequency shift requires image suppression and may require harmonics suppression

  - Note: details of image suppression and harmonics suppression are not discussed in RAN1

- FFS: whether large frequency shift is necessary and feasible for device 2a

Regarding large frequency shift for device 2a, although, based on the observations, it seems feasible to implement them under the device constraints, however, the necessity may depend on the spectrum that can be used for CW transmission and D2R backscattering with device 2a.

Observation 5: For device 2a, the need for large frequency shift may depend on the regulatory requirements for the spectrum that can be used for CW2D and D2R transmissions

Regarding the aspect of reflection amplifier, we are currently considering two candidates including unidirectional and bidirectional amplifier. Unidirectional amplifier, primarily used for D2R (device-to-reader) signal amplification, is simpler, consumes less power, and provides higher amplification gain (typically 10-25 dB) compared to its bidirectional counterpart. In contrast, a bidirectional amplifier can amplify signals in both R2D (reader-to-device) and D2R directions. However, it introduces higher complexity, increased noise figure, and reduced isolation between transmission and reception paths, leading to generally lower amplification gain and higher power consumption. Therefore, unidirectional amplifiers are preferred for their efficiency and stability, while bidirectional amplifiers, despite their broader functionality, are more prone to stability issues and are considered more complex

Observation 6: Unidirectional amplifier is simpler, consumes less power, and provides higher amplification gain (typically 10-25 dB) compared to its bidirectional counterpart

Observation 7: Bidirectional amplifier introduces higher complexity, increased noise figure, and reduced isolation between transmission and reception paths, leading to generally lower amplification gain and higher power consumption

Proposal 3: For device 2a, consider unidirectional amplifier as the baseline

Regarding sequence detection, similar observation as device type 1 is made, i.e. sequence detection based on correlation of sequence is not feasible for device type 2a. Therefore, no additional block for sequence detection should be considered for device type 2a. For power detection, we can further discuss if any additional block is needed, or RF-ED receiver blocks currently agreed are sufficient or not.

Proposal 4: For device type 2a, sequence correlation-based detection should not be further considered for device architecture

For device type 2a, OOK and FSK/PSK could be feasible within the constraints of device type 2a. For OOK based receiver, power consumption as low as ~0.1µW is achievable. For FSK/PSK based receiver, the power consumption targets need to be higher, e.g. more than 10µW.

Observation 8: For device type 2a, OOK and FSK/PSK waveform can be accommodated in the constraints of device architecture based on its power consumption requirements

## Device type 2b

Like device type 1 and device type 2a, due to considerations for harmonized design, following proposals can be made for device type 2b as well:

Proposal 5: For device type 2b, sequence correlation-based detection should not be further considered for device architecture

Observation 9: For device type 2b, OOK and FSK/PSK waveform can be accommodated in the constraints of device architecture based on its power consumption requirements

## Clock Functions

In RAN1#117, following agreement related to clock functionalities in the device have been made in [2]:

Agreement

The following template is used for capturing descriptions on clock/LO.

|  | Purpose | Applicable device types | Clock speed | Power consumption | [Initial clock Accuracy] | [Accuracy after clock sync] | [Clock drift] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Purpose #1 of the clock |  |  |  |  |  |  |  |
| Purpose #N of the clock |  |  |  |  |  |  |  |
| Etc |  |  |  |  |  |  |  |

Essentially, there was discussion on different functionalities required for the clock for different devices. However, there was no agreement on the explicit purpose and related aspects. We agreed only on a template for further inputs/discussions. In our view, at least following clock purposes can be considered for each device types, as shown in Table 3. Values is square brackets, such as clock for frequency shift can be further discussed, depending on other discussions.

Table 3: Clock Purposes for devices

| Clock # | Purpose | Applicable device types | Clock speed | Power consumption | Initial clock Accuracy (i.e. before clock sync) | Accuracy after clock sync |
| --- | --- | --- | --- | --- | --- | --- |
| Clock 1 | Sampling for preamble detection | Device 1, Device 2a, Device 2b | At least 10s of kHz | << 1uW | 10^4 ~ 10^5 ppm | [< 10^4]ppm |
| Clock 2 | [Frequency shift] | Device 2a | At least 1.92 MHz | < 10s uW | 10^4 ~ 10^5 ppm | [< 10^4]ppm |
| Clock 3 | Clock for generating carrier frequency | Device 2b | Same as carrier frequency | < 100 uW | 10^3~ 10^5 ppm | [10^2]ppm |

Proposal 6: For ambient IoT device types, consider at least the following clock purposes and required parameters/constraints

| Clock # | Purpose | Applicable device types | Clock speed | Power consumption | Initial clock Accuracy (i.e. before clock sync) | Accuracy after clock sync |
| --- | --- | --- | --- | --- | --- | --- |
| Clock 1 | Sampling for preamble detection | Device 1, Device 2a, Device 2b | At least 10s of kHz | << 1uW | 10^4 ~ 10^5 ppm | [< 10^4]ppm |
| Clock 2 | [Frequency shift] | Device 2a | At least 1.92 MHz | < 10s uW | 10^4 ~ 10^5 ppm | [< 10^4]ppm |
| Clock 3 | Clock for generating carrier frequency | Device 2b | Same as carrier frequency | < 100 uW | 10^3~ 10^5 ppm | [10^2]ppm |

# Conclusion

In this contribution, we made the following observations and proposals on device architecture for ambient IoT devices:

Device 1

Observation 1: Peak power conversion efficiency can be achieved in the range of 75%-85% for typically high SNR regime

Observation 2: Power conversion efficiency of less than 40% is achievable in the range of achievable activation threshold of -20dBm to -25 dBm

Observation 3: For device type 1, OOK waveform can be accommodated in the constraints of device architecture based on its power consumption requirements

Observation 4: For device type 1, FSK/PSK waveform may not be able to be accommodated in the constraints of device architecture based on their power consumption requirements

Proposal 1: For device type 1, small frequency shift in order of few 10s of kHz is achieved by baseband processing, e.g. by line coding and no dedicated block is studied for small frequency shift

Proposal 2: For device type 1, sequence correlation-based detection should not be further considered for device architecture

Device 2a

Observation 5: For device 2a, the need for large frequency shift may depend on the regulatory requirements for the spectrum that can be used for CW2D and D2R transmissions

Observation 6: Unidirectional amplifier is simpler, consumes less power, and provides higher amplification gain (typically 10-25 dB) compared to its bidirectional counterpart

Observation 7: Bidirectional amplifier introduces higher complexity, increased noise figure, and reduced isolation between transmission and reception paths, leading to generally lower amplification gain and higher power consumption

Observation 8: For device type 2a, OOK and FSK/PSK waveform can be accommodated in the constraints of device architecture based on its power consumption requirements

Proposal 3: For device 2a, consider unidirectional amplifier as the baseline

Proposal 4: For device type 2a, sequence correlation-based detection should not be further considered for device architecture

Device 2b

Proposal 5: For device type 2b, sequence correlation-based detection should not be further considered for device architecture

Observation 9: For device type 2b, OOK and FSK/PSK waveform can be accommodated in the constraints of device architecture based on its power consumption requirements

Clock Functions

Proposal 6: For ambient IoT device types, consider at least the following clock purposes and required parameters/constraints

| Clock # | Purpose | Applicable device types | Clock speed | Power consumption | Initial clock Accuracy (i.e. before clock sync) | Accuracy after clock sync |
| --- | --- | --- | --- | --- | --- | --- |
| Clock 1 | Sampling for preamble detection | Device 1, Device 2a, Device 2b | At least 10s of kHz | << 1uW | 10^4 ~ 10^5 ppm | [< 10^4]ppm |
| Clock 2 | [Frequency shift] | Device 2a | At least 1.92 MHz | < 10s uW | 10^4 ~ 10^5 ppm | [< 10^4]ppm |
| Clock 3 | Clock for generating carrier frequency | Device 2b | Same as carrier frequency | < 100 uW | 10^3~ 10^5 ppm | [10^2]ppm |

# References

- 3GPP RAN1#116bis, “Chairman Notes”

- 3GPP RAN1#117, “Chairman Notes”

- EPC™ Radio-Frequency Identity Protocols Generation-2 UHF RFID Standard, Specification for RFID Air Interface Protocol for Communications at 860 MHz – 960 MHz, Release 2.1, Ratified, Jul 2018

- D. A. Loku Galappaththige, F. Rezaei, C. Tellambura and S. Herath, "Link Budget Analysis for Backscatter-Based Passive IoT," in IEEE Access, vol. 10, pp. 128890-128922, 2022

- C. Achille Fumtchum, Pierre Tsafack, Florin Hutu, Guillaume Villemaud, & Emmanuel Tanyi. (2021), “A Survey of RF Energy Harvesting Circuits”, International Journal of Innovative Technology and Exploring Engineering (IJITEE), 10(7), 99–106

- J. Blanckenstein, J. Klaue and H. Karl, "A Survey of Low-Power Transceivers and Their Applications," in IEEE Circuits and Systems Magazine, vol. 15, no. 3, pp. 6-17