3GPP TSG-RAN WG1 Meeting #118	R1-2407146

Maastricht, Netherlands, August 19th – August 23rd, 2024

Agenda Item:	9.9.1

Source:	Ericsson

Title:	Measurement related enhancements for LTM

Document for:	Discussion, Decision

Introduction

- Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1]

  - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM

  - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1]

    - RAN1 and RAN2 to progress independently on the event triggered measurements objectives of their respective MIMO and Mobility enhancement WIs. Review progress at RAN#105 to see if any modification of objectives is required to avoid/manage any overlap in the work

  - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management, and/or other necessary physical layer operations on candidate cells before LTM [RAN1]

The WID for the Rel. 19 “NR mobility enhancements Phase 4” [1] states the following objective:

In this contribution we present proposals for discussion on this topic.

Discussion

The justification for event triggered L1 measurement reporting in the WID [1] points out the reduced uplink signaling overhead that can be achieved compared to periodic reporting. Other benefits with LTM such as pre-synchronization for short interruption time should remain. It is important that the events are well designed and configured so that the network does not miss out on important information that are needed to make timely and well targeted mobility decisions. Event-triggered measurement reporting will be discussed in section 2.1.

For the support for CSI-RS measurements for LTM procedures, the justification of the WID mentions that it “can be expected to enable greater throughput on the target cell directly after cell switch”. Physical layer operations based on CSI-RS will be discussed in section 2.2.

## Event triggered L1 measurement reporting

### Requirements

It is important to have a common understanding of the requirements for the feature so that important aspects are not overseen, but also to avoid complex optimizations on aspects that turn out not important for the functionality.

- Low overhead: The main objective of event triggered L1 reporting is according to the WID to reduce the uplink measurement reporting overhead compared to the periodic or semi-persistent reporting that is supported by Release 18 LTM, which causes large uplink signaling that is for the most cases not needed or used in any way. If the UE only sends measurement reports when there is a need to trigger mobility actions, then the reporting overhead will be dramatically reduced. The size of each report is another aspect, but the potential for overhead reductions by minimizing the report size is very small in comparison to the reductions that can be obtained by reducing the number of reports sent.

- Reliability: In contrast to Rel. 18 LTM measurement reporting, with event triggered measurement reporting the NW does not know that a report is coming, and it cannot determine if a report was missed. If a report is missed, then the UE will fail to convey the information that the event has occurred. Clearly, this will lead to a performance impairment.

- Flexibility: Compared to periodic reporting that enables the network to observe continuously the changes and variations in radio conditions experienced by the UE, with event triggered measurement reporting each measurement report must include all the information that the network needs to take proper actions. Measurement reports will be triggered based on different criteria, thresholds, triggering resources etc., all this must be conveyed to the network, requiring a flexible report format.

- Measurement report delay: Too long delay can in the worst-case lead to that a cell switch cannot be completed in time before the UE lose connection to the serving cell. However, the need to minimize delays should not be exaggerated: We need to understand the timescale that the feature is operating on. For example, SSBs are normally broadcast with 20ms periodicity. If filtering is applied to reduce the impact of noise and fast fading, that comes with filtering delay that is a couple of SSB cycles long. If time-to-trigger mechanisms are used (as has been found beneficial for L3 mobility), that adds yet another delay. If in the end the feature operates with delays in the order of 100ms or more, then optimizations that saves a just a fraction of that at the expense of other requirements or increased complexity won’t be meaningful.

The WID contains two parts that concerns RAN1: event-driven reporting and measurements on CSI-RS. In our view, it is natural to also support the combination: event-driven reporting based on CSI-RS, and we note the following RAN2 agreement:

Support the beam config of both SSB and CSI-RS in L1 measurement resource configuration in LTM config. Working assumption: Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5.

We thus observe

Event triggered L1 measurement reporting is supported both for SSB and CSI-RS.

### Events

- Event triggered L1 measurement should be designed for the following LTM purposes:

- Select the candidate beam/cell to trigger early synchronization.

- Select the target beam/cell and trigger LTM cell switch procedure.

- For event triggered L1 measurement, use of beam level measurement result for event evaluation is baseline. FFS for the cell level measurement.

- Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events.

- Event LTM2: Beam of serving cell becomes worse than absolute threshold;

- Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell;

- Event LTM4: Beam of candidate cell becomes better than absolute threshold;

- Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

FFS on what beam(s) of the serving cell and neighboring cell is used for event evaluation.

FFS on the need of Event LTM1.

In RAN2#126 the following was agreed:

To minimize the measurement reporting overhead, events should only be triggered when needed. Ideally, every time an event is triggered, the measurement report that is provided to the network will lead to actions for early synchronization or LTM cell switch, according to RAN2#126 agreement 1 (see above). The optimal configuration of the events depends largely on the network deployment, hence it is important that the event conditions are network configured and not left to UE implementation. This allows the network to control the UL reporting overhead and at the same time ensure that changes in radio conditions that requires mobility actions are observed by the network.

As leading working group, it is natural that RAN2 drives the discussion on event definitions. RAN1 can however contribute with inputs regarding the FFS on what beam(s) of the serving cell and neighboring cell is used for event evaluation (see agreement 3 above):

Regarding the serving cell, the intentions with the events are to evaluate if the radio conditions are good or bad in the serving cell (LTM2) or can be improved by mobility to another cell (LTM3, LTM5). In the serving cell the UE experience the channel of the indicated TCI state, consequently the serving cell evaluation should be based on the indicated TCI state. In the Rel. 19 MIMO WI, RAN1 has agreed two options on how the beam quality is derived from the indicated TCI state:

- The beam quality is based on the QCL RS in the indicated TCI state

- The beam quality is based on the SSB that is QCLed with the QCL RS in the indicated TCI state

The NW configures the UE to use either option.

We propose to have the corresponding options for Event-triggered LTM:

The indicated TCI state should be used for evaluation of events on the beam of the serving cell. The RSRP measurements can be based on either
1) the QCL RS in the indicated TCI state, or
2) the SSB that is QCLed with the QCL RS in the indicated TCI state.
The NW configures the UE to use either option.

For candidate cells, we assume that the RS of each candidate beam is provided to the UE explicitly, and for the events that involve beams of candidate cells (LTM3, LTM4 and LTM5), the beam quality is evaluated by measurements on the configured RS. With the current formulation for LTM3, LTM4 and LTM5, the reasonable interpretation is that the event is triggered if any candidate beam is better than beam of the serving cell. With this interpretation and Proposal 1, the agreed events can be clarified:

- Event LTM2: Beam corresponding to the indicated TCI state becomes worse than absolute threshold;

- Event LTM3: Any beam of candidate cell becomes amount of offset better than the beam corresponding to the indicated TCI state;

- Event LTM4: Any beam of candidate cell becomes better than absolute threshold;

- Event LTM5: Beam corresponding to the indicated TCI state becomes worse than absolute threshold1 AND any beam of candidate cell becomes better than another absolute threshold2.

We realize that RAN2 is discussing the support of additional events.

### Filtering

- RAN2 assumes filtering of the L1 measure results is needed. It’s up to RAN1 whether the specified L1 filtering is needed or ok to leave it to UE implementation.

- For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. FFS on the need of measurement reporting once leaving condition is met.

In RAN2#126 the following was agreed:

The risk to trigger events based on noisy measurement samples or fast fading characteristics can be reduced by filtering the RSRP measurements. It follows that filtering can reduce the risk for ping-pong handover and spurious TCI state activation/deactivation etc.

In this contribution we use the following terminology:

- UE implementation filter: Internal layer 1 filtering of RSRP measurement which is up to UE implementation, see TS 38.300 Sec. 9.2.4.

- Network configurable filter: A filter that is configured by the network.

The L3 filtering is one example of a filter with parameters that can be configured by the network and tuned for performance, but a network configurable filter could also be used in e.g. the MAC layer. The current L3 filtering includes a first order IIR filter with a tuneable filter coefficient that is a forgetting factor: a large value means that only the most recent measurements have a significant weight, while older measurements are quickly “forgotten”. Hence, with a large filter coefficient, the filter quickly adapts but is sensitive to noisy samples. A low filter coefficient can on the other hand reduce the impact of noise at the expense of slower adaptation.

There are several arguments for including a network configurable filter in addition to a UE implementation filter:

- The quality of measurements differs between UEs. A network configurable filter can provide a consistent quality of the RSRP measurements used for event evaluation.

- The network can tune the parameters for different deployments to achieve optimal performance with respect to for example handover success rate or other KPIs.

- Time-to-trigger and hysteresis parameters for the events can be configured jointly with the filter parameters. The different parameters interact and are therefore best configured jointly.

A network configurable filter can ensure consistent quality of the RSRP measurements used for event evaluation.

Different network deployments and network configurations require different filter settings for optimal performance.

The network can configure time-to-trigger and hysteresis parameters for the events jointly with filter parameters for optimal performance.

To illustrate the effect of filtering on the measurement process, we have performed system simulations for an urban macro scenario, where the LTM3 event has been used. We have included a UE implementation filter and investigated the effect of including a network configurable filter, which operates on the output of that UE implementation filter. We have studied the following quantities:

Number of times the LTM3 event was triggered

Number of times the LTM switch was finished

Number of ping-pong handovers. A ping-pong handover is a handover where the UE stays in the new cell for less than a second before going back to the source.

We have used a first order IIR filter as a NW configurable filter:

Fn=1-a×Fn-1+a×Mn 					(1)

where Fk is the output of the NW-configurable filter at time instant k, Mk is the output of the UE-implementation filter at time instant k, and a is a configurable filter coefficient. More simulation parameters are collected in the appendix.

The results are depicted in Figure 1 for pedestrian speed and in Figure 2 for vehicular speed.

Figure 1: Results for pedestrian speed. Note that the filtering significantly reduces the number of ping-pong handovers.

Figure 2: Results for vehicular speed. Note that the filtering significantly reduces the number of ping-pong handovers.

The results in Figure 1 and Figure 2 show the benefits of the NW-configurable filtering. In particular, there is a reduction in the number of ping-pong handovers:

With NW-configurable filtering, the number of ping-pong handovers is reduced.

Based on the discussion and these results, we propose to introduce a NW-configurable filter for Event-triggered LTM after the UE implementation filter. The output from the UE implementation filter is the input to the NW-configurable filter. As a starting point for further discussions, we propose that it should be a first order IIR filter like the one used for L3 mobility since it is simple to configure, intuitive and has worked well for L3 mobility. Compared to L3 filtering, the LTM filter coefficient can be tuned differently if it is desirable to achieve for example faster dynamics, but the fundamental structure of the filter can be the same.

Introduce a network configurable filter. Event conditions should be evaluated on RSRP measurements that are filtered with the network configurable filter.

Use a first order IIR network configurable filter like the one used for L3 filtering.

### Reporting

#### MAC CE or UCI

The question whether MAC CE or UCI should be used to send event-triggered L1 measurement reports is being discussed in RAN2. As leading WG, RAN2 should decide about that:

The decision to use MAC CE or UCI to convey measurement reports is made by RAN2.

Below we elaborate on a few RAN1 related aspects of the question, but a more detailed analysis and proposals are presented in the joint RAN2 contribution Error! Reference source not found..

- The main objective of this agenda point is to reduce the reporting overhead. The bulk of the overhead reduction comes from reducing the number of times that measurement reports are sent. Optimizations with regard to the size of each measurement report are insignificant in comparison, then it is much more important to have a flexible reporting format that can capture different events, conditions and measurement quantities that can help the network to make correct mobility actions. MAC CE is the more flexible alternative.

- Reliability is guaranteed for MAC CE by Hybrid ARQ. To achieve reliability with UCI, solutions with retransmissions would have to be developed, which could lead to higher reporting overhead.

- Reporting delay: According to our analysis in [2], MAC CE is as fast as UCI in most practical scenarios. UCI is only faster with configured grant and low periodicity, but that is inefficient from a resource point of view. Regardless, the reporting delay is unlikely to be an issue because the timescale that the feature is operating on is significantly longer, due to e.g. SSB periodicity, filtering delays and more.

In summary, we observe

Given the requirements on reliability, flexibility, report overhead and delay, our analysis is that MAC CE is the best option.

#### Report content

The current LTM measurement report is a natural starting point for the discussion on the report content although adaptations for event-triggered reports need to be done. To begin with, it must be clear to the network what was the event that triggered the reporting. It appears reasonable that all configured events are associated with an event ID and that the measurement report includes the ID of the triggering event.

Event triggered LTM measurement report should include an identifier of the event and conditions that triggered the report.

Below we propose additional report content for each event type that has been agreed in RAN2.

LTM2: From the fact that the event has triggered, the network can conclude that the RSRP of the serving beam is below the configured threshold. It is however beneficial for the network to know the exact RSRP of the serving beam to understand the urgency and how fast the conditions are degrading:

For LTM2 the measurement report should include the RSRP of the serving beam.

LTM3: This event is triggered by conditions on two different resources: the candidate beam and the serving beam. The candidate beam identity is the target of a mobility action (presync or cell switch) and need to be included in the report. It can be beneficial to also include the serving beam RSRP to give the network more detailed understanding of the situation:

For LTM3 the measurement report should include an identifier of the candidate beam that triggered the event, the RSRP of the triggering beam, and the serving beam RSRP.

LTM4: This event is triggered when a candidate beam becomes better than an absolute threshold. The candidate beam identifier needs to be included in the report since it is the mobility target.

For LTM4 the measurement report should include an identifier of the candidate beam that triggered the event, and the corresponding RSRP.

LTM5: This event is triggered by conditions on two different resources: the serving beam which is worse than a first absolute threshold and a candidate beam that is better than a second absolute threshold. It is the combination of LTM2 and LTM4 and for the reasons already mentioned for those events, we think that the serving beam RSRP, an identifier of the candidate beam that triggered the event, and the RSRP of the triggering beam should be included in the measurement report when LTM5 is triggered.

For LTM5 the measurement report should include the serving beam RSRP, an identifier of the candidate beam that triggered the event, and the corresponding RSRP.

The above proposals are summarized in Table 1.

| Event | Measurement report content |
| --- | --- |
| LTM2 | Serving beam RSRP |
| LTM3 | Serving beam RSRP + triggering beam (identifier, RSRP) |
| LTM4 | Triggering beam (identifier, RSRP) |
| LTM5 | Serving beam RSRP + triggering beam (identifier, RSRP) |

Table 1: Mandatory content of the event-driven measurement reports for the agreed events.

For all the mentioned measurement reports it should also be possible to include measurements on additional candidate beams, like in the Rel-18 measurement report format.

For all the defined LTM events it should be possible to include measurement on additional candidate beams that did not trigger the event in the measurement report, and the corresponding RSRP values.

Release 18 LTM measurement reporting is either periodic, semi-persistent on PUCCH or PUSCH or aperiodic. With event triggered measurement reporting only one measurement report is strictly needed to be sent by the UE to the NW to inform about the occurrence of the event and the relevant RSRP measurements. With L3 event triggered measurement reporting there is an option to provide a limited number of measurement reports over a time-period when an event has occurred by setting the RRC parameter reportAmount > 1. In certain deployments where the problem with reporting overhead is less critical, a corresponding functionality for event-triggered LTM measurement reports can be considered. With multiple reports received over a period of time, the network can observe if the reported RSRP measurements are stable or changing quickly.

The decision to introduce this type of reporting is up to RAN2, but it is important that whatever functionality RAN1 agrees can handle the functionality.

### UE autonomous TCI state activation

In Rel-18 LTM, the NW activates candidate TCI states based on the content of measurement reports, before the actual LTM cell-switch is executed. This is done by sending the Candidate Cell TCI States Activation/Deactivation MAC CE to the UE. With the Rel-18 LTM reports, this independent activation step is reasonable, since all the reported candidates are not necessarily suitable for any immediate subsequent procedure, i.e., candidate TCI state activation, followed by a cell-switch command.

However, the situation is different for event triggered reports: the event triggered reporting should be designed and configured so that most measurement reports do lead to an updated TCI state (RAN2#126 Agreement 1). In fact, it is quite likely that a measurement target that triggered an event will be activated very soon. In this situation, performing an independent activation of candidate TCI states, is in many cases unnecessary.

For event triggered LTM reporting, separate MAC CE signalling for TCI state activation can be avoided by configuring the UE to autonomously activate candidate TCI states that correspond to measurements targets included by the UE in an event triggered report. This will reduce the overhead as well as the latency, since the candidate TCI state activation delay is to a large extent avoided.

UE autonomous TCI state activation can reduce the overhead and the latency compared to separate TCI State Activation signaling from the network.

Hence, we propose:

After sending an event-triggered LTM measurement report, the UE will activate the candidate TCI states that are associated with the reference signals in the measurement report.

For example, if the UE reports an SSB, the UE activates any TCI state that includes a reference signal that has the reported SSB as QCL source.

For these candidate TCI states, there would be no additional delays, e.g., for an LTM cell switch, or for an early RACH.

The UE can only activate a limited number of candidate TCI states. Requiring that the UE activates all the reported TCI states may be too demanding. Instead, the UE could include the activation status for each reported measurement target:

For each measurement target, the UE includes a flag that indicates if the corresponding candidate TCI states are activated.

### Requesting UL resources for transmission of the report

The UE may include a MAC CE measurement report in any PUSCH allocation, provided that the UL resource is large enough. The PUSCH resource is granted to the UE using one of the following methods:

- Periodic UL resources are provided using configured grant Type I PUSCH by RRC configuration.

- Semi-persistent UL resources are provided using configured grant Type II, using a combination of RRC configuration and MAC CE activation.

- Dynamic UL resources are allocated to the UE in DCI. The UE requests these UL resources using scheduling request (SR) or RACH.

With a MAC CE measurement report, these allocation methods are natively supported:

All the legacy UL resource allocation methods are natively supported for LTM event-driven reporting over MAC CE.

In our view, there is little need to support any additional UL allocation methods:

Do not introduce any additional UL allocation methods for LTM event-driven reporting.

## Physical layer operations based on CSI-RS

- Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management, and/or other necessary physical layer operations on candidate cells before LTM [RAN1]

The WID for the Rel. 19 “NR mobility enhancements Phase 4” [1] includes the following objective:

### CSI-RS measurements for LTM

During the LTM standardization in Rel-18, measurements on CSI-RS were extensively discussed, but the specification was down-prioritized. Now when the basic support is in place, adding CSI-RS as a measurement target seems straightforward, and therefore we propose

Support LTM L1-RSRP measurement reporting for CSI-RS.

It would seem natural if the LTM CSI-RS measurements could be reported in the same way as the LTM SSB measurements. The LTM SSB measurement report includes an SSBRI and RSRP, where the SSBRI is the index into a list configured in RRC:

To align how the CSI-RS measurements are reported, there is a need to also align the measurement configuration. Hence, we propose

Specify the CSI-RS for LTM measurements in the LTM-CSI-ResourceConfig.

During the Rel-18 discussion, there was a discussion on what “type” of CSI-RS would be relevant for LTM measurements: CSI-RS for beam management or CSI-RS for mobility. The only difference between these types of CSI-RS is how they are configured to the UE: the actual transmission is the same. To be precise,

CSI-RS for beam management is a CSI-RS resource set with repetition off. The CSI-RS resources (NZP-CSI-RS-Resource) in the set are all single-port or dual-port.

CSI-RS for mobility is basically a list of single-port CSI-RS resources. Those CSI-RS resources are not configured using the NZP-CSI-RS-Resource: the information that defines the CSI-RS resource is provided using another structure.

- When looking at these options, it seems that the CSI-RS for LTM measurements will use neither of these configurations: we cannot use an NZP-CSI-RS-ResourceSet, since all the NZP-CSI-RS-Resources are associated with the same cell, and the configuration of CSI-RS for mobility includes additional parameters, e.g., associatedSSB and cellId, that do not blend well with the LTM configuration,  Hence, we propose to define a new configuration for the purpose of LTM measurement reporting based on CSI-RS:

Define a new configuration for the list of CSI-RS resources used for LTM measurements.

- The list should be specified so that the measurement format defined for LTM in TS 38.212, Table 6.3.1.1.2-8C, can be used.

- If Proposal 14, Proposal 15, and Proposal 16 are agreed, what remains is to specify the RRC signaling.

### Other procedures based on CSI-RS

Link adaptation is an important tool to improve the spectral efficiency in NR. Based on link quality reports, the NW adjusts

modulation and coding scheme for the PDSCH, and

aggregation level for PDCCH

to reduce the resource consumption while still maintaining adequate error rates. In legacy LTM, the UE starts estimating the link quality of the target after the LTM execution. This leads to a delay before accurate link quality reports reach the NW, and the link adaptation can be performed.

To solve this issue, we propose to make it possible for the UE to measure the link quality for candidate cells before the LTM execution:

Support measurements on a candidate cell before LTM cell switch to support link adaptation immediately after the cell switch.

Based on these link quality reports, the NW can then perform link adaptation for PDSCH and PDCCH directly after the LTM execution.

The CSI measurements would have to be sent to the candidate cell. There are essentially two ways of doing this:

The UE reports the CSI to the serving cell, which then forwards the CSI to the relevant candidate cell, which uses the CSI after the cell switch.

The UE reports the CSI in one of the first transmissions to the candidate cell. Preferably, the UE would include the measurements in the same UL transmission that includes the RRC reconfiguration complete.

These two procedures are illustrated in Figure 3.

Figure 3: Procedures for CSI measurements on a candidate cell before LTM cell switch.

Both options have pros and cons: with the first method, the NW will be able to use the CSI a little earlier at the cost of additional NW signaling. With the second method, the UE can perform CSI measurements later since there is no need to reserve time to report the measurement to the candidate cell, and the CSI report is sent only once. Also, the second approach is more generally applicable. We propose that RAN1 studies the two approaches:

RAN1 studies methods to convey the CSI measurements to the candidate.

To perform link adaptation, the NW needs a CQI value, which is one part of CSI. In NR, there are many additional parameters that describe CSI, and the natural question is what parameters would be relevant to report also for a candidate cell. Since this would be used only during a short time, only the most important parameters should be reported. For example, supporting it would be difficult to utilize TDCP reporting for a candidate cell. Here we propose to limit this to Type I codebook reporting, and only report CRI, CQI, PMI and RI:

Support reporting of CRI, CQI, PMI and RI for a Type I codebook for a candidate cell before or after LTM cell switch.

We thus exclude the Type II codebook, due to complexity and reporting overhead. We also exclude CRI reporting for the same reason.

In legacy beam management, there is support for the P3 procedure. In the P3 procedure, the NW transmits a CSI-RS resource set with repetition on, meaning that the UE can assume that the CSI-RS resources in the CSI-RS resource set is transmitted using the same beam. The UE can use this CSI-RS resource set to quickly evaluate the quality with different Rx beams. Unfortunately, the overhead on the NW side is large, since multiple CSI-RS resources must be transmitted in the same beam. Therefore, P3 is more suitable with aperiodic CSI-RS, where only a few Tx beams are probed:

The P3 procedure is more suitable with aperiodic CSI-RS.

However, for LTM it is difficult to use aperiodic reference signals due to the transport NW delay: only periodic reference signals are currently supported, for good reasons. Based on this, we propose

Do not support the P3 procedure, i.e., measurements on a CSI-RS resource set with repetition on, before the LTM cell switch.

We also note that the UE can perform the P3 procedure across consecutive instances of a periodic CSI-RS resource when timeRestrictionForChannelMeasurements is not configured.

# Conclusion

In the previous sections we made the following observations:

Observation 1	Event triggered L1 measurement reporting is supported both for SSB and CSI-RS.

Observation 2	A network configurable filter can ensure consistent quality of the RSRP measurements used for event evaluation.

Observation 3	Different network deployments and network configurations require different filter settings for optimal performance.

Observation 4	The network can configure time-to-trigger and hysteresis parameters for the events jointly with filter parameters for optimal performance.

Observation 5	Given the requirements on reliability, flexibility, report overhead and delay, our analysis is that MAC CE is the best option.

Observation 6	UE autonomous TCI state activation can reduce the overhead and the latency compared to separate TCI State Activation signaling from the network.

Observation 7	All the legacy UL resource allocation methods are natively supported for LTM event-driven reporting over MAC CE.

Observation 8	The P3 procedure is more suitable with aperiodic CSI-RS.

Based on the discussion in the previous sections we propose the following:

Proposal 1	The indicated TCI state should be used for evaluation of events on the beam of the serving cell. The RSRP measurements can be based on either 1) the QCL RS in the indicated TCI state, or 2) the SSB that is QCLed with the QCL RS in the indicated TCI state. The NW configures the UE to use either option.

Proposal 2	Introduce a network configurable filter. Event conditions should be evaluated on RSRP measurements that are filtered with the network configurable filter.

Proposal 3	Use a first order IIR network configurable filter like the one used for L3 filtering.

Proposal 4	The decision to use MAC CE or UCI to convey measurement reports is made by RAN2.

Proposal 5	Event triggered LTM measurement report should include an identifier of the event and conditions that triggered the report.

Proposal 6	For LTM2 the measurement report should include the RSRP of the serving beam.

Proposal 7	For LTM3 the measurement report should include an identifier of the candidate beam that triggered the event, the RSRP of the triggering beam, and the serving beam RSRP.

Proposal 8	For LTM4 the measurement report should include an identifier of the candidate beam that triggered the event, and the corresponding RSRP.

Proposal 9	For LTM5 the measurement report should include the serving beam RSRP, an identifier of the candidate beam that triggered the event, and the corresponding RSRP.

Proposal 10	For all the defined LTM events it should be possible to include measurement on additional candidate beams that did not trigger the event in the measurement report, and the corresponding RSRP values.

Proposal 11	After sending an event-triggered LTM measurement report, the UE will activate the candidate TCI states that are associated with the reference signals in the measurement report.

Proposal 12	For each measurement target, the UE includes a flag that indicates if the corresponding candidate TCI states are activated.

Proposal 13	Do not introduce any additional UL allocation methods for LTM event-driven reporting.

Proposal 14	Support LTM L1-RSRP measurement reporting for CSI-RS.

Proposal 15	Specify the CSI-RS for LTM measurements in the LTM-CSI-ResourceConfig.

Proposal 16	Define a new configuration for the list of CSI-RS resources used for LTM measurements.

Proposal 17	Support measurements on a candidate cell before LTM cell switch to support link adaptation immediately after the cell switch.

Proposal 18	RAN1 studies methods to convey the CSI measurements to the candidate.

Proposal 19	Support reporting of CRI, CQI, PMI and RI for a Type I codebook for a candidate cell before or after LTM cell switch.

Proposal 20	Do not support the P3 procedure, i.e., measurements on a CSI-RS resource set with repetition on, before the LTM cell switch.

# References

3GPP RP-234036, New WID: NR mobility enhancements Phase 4, 3GPP TSG RAN Meeting #102, December 2024.

R1-240982, Enhancements for UE-initiated beam management, Ericsson, RAN1#116bis, April 2024.

R2-2406921, Discussion on which layer reports the event triggered L1 measurements, Ericsson, Honor, Huawei, InterDigital Inc., MediaTek Inc., NEC, Nokia, NTT DOCOMO, Oppo, T-Mobile USA, Vivo, ZTE corporation, RAN2#127, Maastricht, The Netherlands, August 2024

- Appendix: simulation parameters

| Simulation parameter​ | Setting |  |
| --- | --- | --- |
| Evaluation scenario | Dense urban macro, macro layer only |  |
| Carrier frequency | 3.5 GHz ​ |  |
| Bandwidth | 20MHz |  |
| Number of users | 20 |  |
| Mobility model | Straight motion with random change in directions |  |
| UE speed | 3 km/h | 50 km/h |
| UE-implementation filter coefficient | 0.33 |  |
| NW-configurable filter coefficient (a in (1)) | 0 (baseline) | 0.87 |
| Event configuration​ | LTM3 event with 160 ms time-to-trigger, Best beam in the cell is evaluated​ |  |
| SSB interval | 20ms |  |
| SSB configuration​ | 1 SSB | 4 SSBs |
| Measurement Error Model | Gaussian Error models, +/-2dB Relative Error​ |  |