3GPP TSG- RAN WG1 Meeting #118	R1-2407245

# Maastricht, Netherlands, August 19 - 23, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [Draft] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR |  | rev |  | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on PSFCH Power Control |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (Huawei), Huawei, Hisilicon, NEC |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-08-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - When sl-TransmissionStructureForPSFCH = 'commonInterlace', for power limited case, a UE first determines the largest value K satisfying PPSFCH,one+10log10NPSFCH,oneK≤PCMAX , where PCMAX is determined according to [8-1, TS 38.101-1] for transmission of all PSFCHs in i=1KMi, and NPSFCH,oneK=max1,i=1KMi⋅NPSFCH,oneinterlace2+NPSFCH,oneinterlace1,K⋅10(-PPSFCH,offset/10) . Then, the UE finally choose NTx,PSFCH≥max1,i=1KMi PSFCH transmission with power PPSFCH,ki without exceeding the PCMAX. Therefore, the finally transmitted number of PSFCH is NTx,PSFCH, which might be larger than max1,i=1KMi. However, when determining the final power PPSFCH,ki on each PSFCH transmission k, current spec in TS38.213 only considers common PRBs among max1,i=1KMi PSFCH transmissions, instead of NTx,PSFCH PSFCH transmissions, which may result in total power of NTx,PSFCH exceeding PCMAX for simultaneous transmission among multiple RB sets. Take the example below: - Assume M1=2, and a UE determines the largest value K=1 satisfying power limit PCMAX. Assume these two PSFCH transmission are located in RB set 1, and the number of common PRBs for the max1,i=1KMi=2 PSFCH transmissions is 10. Assume the UE finally choose NTx,PSFCH=3 . - Then, in TS38.213, when determining the final power PPSFCH,ki on each PSFCH transmission k in power limited cases, only 10 common PRBs among max1,i=1KMi=2 are counted. Actually, the total number of common PRBs to be transmitted are 20 PRBs among NTx,PSFCH PSFCH transmissions. As a result, the total power on NTx,PSFCH=3 PSFCH transmissions may exceed PCMAX. - When dl-P0-PSFCH is not provided and sl-TransmissionStructureForPSFCH = 'commonInterlace', current spec in TS38.213 does not state that the number of common PRBs are calculated among NTx,PSFCH PSFCH transmissions. However, the relevant parameters NPSFCH,oneinterlace1 and NPSFCH,one,kinterlace1 in the equation for determining PPSFCH,ki are not clearly defined. - Paragraph level is incorrect |  |  |  |  |  |  |  |  |
| Summary of change: |  | - When sl-TransmissionStructureForPSFCH = 'commonInterlace', for power limited case, when determining the final power PPSFCH,ki on each PSFCH transmission k, the number of common PRBs is calculated among NTx,PSFCH PSFCH transmissions, instead of max1,i=1KMi PSFCH transmissions. - When dl-P0-PSFCH is not provided and sl-TransmissionStructureForPSFCH = 'commonInterlace', the number of common PRBs are calculated among NTx,PSFCH PSFCH transmissions, and the relevant parameters NPSFCH,oneinterlace1 and NPSFCH,one,kinterlace1 are clearly defined. - Revise the paragraph level |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - When sl-TransmissionStructureForPSFCH = 'commonInterlace', for power limited case, the total power of NTx,PSFCH PSFCH transmissions may exceed PCMAX. - When dl-P0-PSFCH is not provided and sl-TransmissionStructureForPSFCH = 'commonInterlace', the specification on calculating PPSFCH,ki is not clear. - The paragraph level logic is confused |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Impact Analysis: No backward compatible issue is expected from the CR. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 16.2.3	PSFCH

A UE with Nsch,Tx,PSFCH scheduled PSFCH transmissions for HARQ-ACK information and conflict information, and capable of transmitting a maximum of Nmax,PSFCH PSFCHs, determines a number NTx,PSFCH of simultaneous PSFCH transmissions and a power PPSFCH,k(i) for a PSFCH transmission k, 1≤k≤NTx,PSFCH, on all the resource pools in PSFCH transmission occasion i on active SL BWP b of carrier f as

-	if dl-P0-PSFCH is provided,

PPSFCH,one=PO,PSFCH+10log102μ+αPSFCH⋅PL [dBm]

where

-	PPSFCH,one is applicable for

-	the PRB of the PSFCH transmission for operation without shared spectrum channel access,

-	each PRB in the interlace of the PSFCH transmission for operation with shared spectrum channel access and sl-TransmissionStructureForPSFCH = ''dedicatedInterlace'',

-	each PRB in the subset of PRBs in the second interlace of the PSFCH transmission for operation with shared spectrum channel access and sl-TransmissionStructureForPSFCH = 'commonInterlace'

-	PO,PSFCH is a value of dl-P0-PSFCH-r17, if using the parameter is supported by the UE and the parameter is provided; else dl-P0-PSFCH-r16 if provided

-	αPSFCH is a value of dl-Alpha-PSFCH, if provided; else, αPFSCH=1

-	PL=PLb,f,c(qd) when the active SL BWP is on a serving cell c, as described in clause 7.1.1 except that

-	the RS resource is the one the UE uses for determining a power of a PUSCH transmission scheduled by a DCI format 0_0 in serving cell c when the UE is configured to monitor PDCCH for detection of DCI format 0_0 in serving cell c

-	the RS resource is the one corresponding to the SS/PBCH block the UE uses to obtain MIB when the UE is not configured to monitor PDCCH for detection of DCI format 0_0 in serving cell c

-	for operation with shared spectrum channel access and sl-TransmissionStructureForPSFCH = ‘commonInterlace’, PPSFCH,k(i) includes the power on PRBs in both the first and second interlaces and, for more than one PSFCH transmissions from the UE, the power on any PRB in the first interlace is not accumulated among the more than one PSFCH transmissions within a same RB set and is same as the power PPSFCH,PRB,first,k(i) on the PRB in the first interlace for PSFCH transmission k.

-	if Nsch,Tx,PSFCH≤Nmax,PSFCH

-	if PPSFCH,one+10log10NPSFCH,one≤PCMAX, where PCMAX is determined for Nsch,Tx,PSFCH PSFCH transmissions according to [8-1, TS 38.101-1] and

-	NPSFCH,one=Nsch,Tx,PSFCH for operation without shared spectrum channel access

-	NTx,PSFCH=Nsch,Tx,PSFCH and PPSFCH,k(i)=PPSFCH,one [dBm]

-	NPSFCH,one=k=1Nsch,Tx,PSFCHNPSFCH,one,kinterlace for operation with shared spectrum channel access and sl-TransmissionStructureForPSFCH = 'dedicatedInterlace', where NPSFCH,one,kinterlace is the number of PRBs in the interlace for PSFCH transmission k

-	NTx,PSFCH=Nsch,Tx,PSFCH and PPSFCH,ki=PPSFCH,one+10log10NPSFCH,one,kinterlace [dBm], where the power on one PRB in the interlace for PSFCH transmission is PPSFCH,PRB,k(i)=PPSFCH,one

-	NPSFCH,one=Nsch,Tx,PSFCH⋅NPSFCH,oneinterlace2+NPSFCH,oneinterlace1⋅10(-PPSFCH,offset/10) for operation with shared spectrum channel access and sl-TransmissionStructureForPSFCH = 'commonInterlace', where NPSFCH,oneinterlace2 is provided by sl-NumDedicatedPRBs-ForPSFCH, PPSFCH,offset is provided by sl-PSFCH-PowerOffset, and NPSFCH,oneinterlace1 is the number of PRBs in the first interlace for all Nsch,Tx,PSFCH PSFCH transmissions after excluding PRBs for PSFCH transmissions as described in Clause 16.3.0

-	NTx,PSFCH=Nsch,Tx,PSFCH and PPSFCH,ki=PPSFCH,one+10log10NPSFCH,oneinterlace2+NPSFCH,one,kinterlace1⋅10(-PPSFCH,offset/10) [dBm], where NPSFCH,one,kinterlace1 is the number of PRBs in the first interlace within the same RB set of PSFCH transmission k after excluding PRBs for PSFCH transmissions as described in Clause 16.3.0, and the power on one PRB in the first interlace for PSFCH transmission is PPSFCH,PRB,first,k(i)=PPSFCH,PRB,second,ki-PPSFCH,offset and the power on one PRB in the subset of PRBs in the second interlace for PSFCH transmission is PPSFCH,PRB,second,k(i)=PPSFCH,one, where PPSFCH,offset is provided by sl-PSFCH-PowerOffset

-	else

-	UE autonomously determines NTx,PSFCH PSFCH transmissions first with ascending order of corresponding priority field values as described in clause 16.2.4.2 over the PSFCH transmissions with HARQ-ACK information, if any, and then with ascending order of priority value over the PSFCH transmissions with conflict information, if any, such that NTx,PSFCH≥max1,i=1KMi where Mi, for 1≤i≤ 8, is a number of PSFCHs with priority value i for PSFCH with HARQ-ACK information and Mi, for i>8, is a number of PSFCHs with priority value i-8 for PSFCH with conflict information and K is defined as

-	the largest value satisfying PPSFCH,one+10log10NPSFCH,oneK≤PCMAX where PCMAX is determined according to [8-1, TS 38.101-1] for transmission of all PSFCHs in i=1KMi, if any

-	NPSFCH,oneK=max1,i=1KMi for operation without shared spectrum channel access

-	NPSFCH,oneK=k=1max1,i=1KMiNPSFCH,one,kinterlace for operation with shared spectrum channel access and sl-TransmissionStructureForPSFCH = 'dedicatedInterlace'

-	NPSFCH,oneK=max1,i=1KMi⋅NPSFCH,oneinterlace2+NPSFCH,oneinterlace1,K⋅10(-PPSFCH,offset/10) for operation with shared spectrum channel access and sl-TransmissionStructureForPSFCH = 'commonInterlace', where NPSFCH,oneinterlace1,K is the number of PRBs in the first interlace for the max1,i=1KMi PSFCH transmissions after excluding PRBs for PSFCH transmissions as described in Clause 16.3.0

-	zero, otherwise

and

-	PPSFCH,k(i)=minPCMAX-10log10(NTx,PSFCH),PPSFCH,one [dBm] for operation without shared spectrum channel access

-	PPSFCH,ki=minPCMAX-10log10k=1NTx,PSFCHNPSFCH,one,kinterlace,PPSFCH,one+10log10NPSFCH,one,kinterlace [dBm] for operation with shared spectrum channel access and sl-TransmissionStructureForPSFCH = 'dedicatedInterlace', where the power on one PRB in the interlace for PSFCH transmission is PPSFCH,PRB,k(i)=minPCMAX-10log10k=1NTx,PSFCHNPSFCH,one,kinterlace,PPSFCH,one

-	PPSFCH,ki=minPCMAX-10log10NTx,PSFCH⋅NPSFCH,oneinterlace2+NPSFCH,oneinterlace1,K⋅10-PPSFCH,offset10 ,PPSFCH,one+10log10NPSFCH,oneinterlace2+NPSFCH,one,kinterlace1⋅10-PPSFCH,offset/10 [dBm] for operation with shared spectrum channel access and sl-TransmissionStructureForPSFCH = 'commonInterlace', where  NPSFCH,oneinterlace1,K is the number of PRBs in the first interlace for the NTx,PSFCH PSFCH transmissions after excluding PRBs for PSFCH transmissions as described in Clause 16.3.0, NPSFCH,one,kinterlace1 is the number of PRBs in the first interlace for PSFCH transmission(s) among the NTx,PSFCHmax1,i=1KMi PSFCH transmissions which are within the same RB set of PSFCH transmission k after excluding PRBs for PSFCH transmissions as described in Clause 16.3.0, and the power on one PRB in the first interlace for PSFCH transmission is PPSFCH,PRB,first,k(i)=PPSFCH,PRB,second,ki-PPSFCH,offset and the power on one PRB in the subset of PRBs in the second interlace for PSFCH transmission is PPSFCH,PRB,second,k(i)=minPCMAX-10log10NTx,PSFCH⋅NPSFCH,oneinterlace2+NPSFCH,oneinterlace1,K⋅10-PPSFCH,offset/10 ,PPSFCH,one, where PPSFCH,offset is provided by sl-PSFCH-PowerOffset

where PCMAX	is defined in [8-1, TS 38.101-1] and is determined for the NTx,PSFCH PSFCH transmissions

-	else

-	the UE autonomously selects Nmax,PSFCH PSFCH transmissions with ascending order of corresponding priority field values as described in clause 16.2.4.2

-	if PPSFCH,one+10log10NPSFCH,one,max≤PCMAX, where PCMAX is determined for the Nmax,PSFCH PSFCH transmissions according to [8-1, TS 38.101-1]

-	NPSFCH,one,max=Nmax,PSFCH [dBm] for operation without shared spectrum channel access

-	NTx,PSFCH=Nmax,PSFCH and PPSFCH,k(i)=PPSFCH,one [dBm]

-	NPSFCH,one,max=k=1Nmax,PSFCHNPSFCH,one,kinterlace for operation with shared spectrum channel access and sl-TransmissionStructureForPSFCH = 'dedicatedInterlace', where NPSFCH,one,kinterlace is the number of PRBs in the interlace for the PSFCH transmission k

-	NTx,PSFCH=Nmax,PSFCH and PPSFCH,ki=PPSFCH,one+10log10NPSFCH,one,kinterlace [dBm], where the power on one PRB in the interlace for PSFCH transmission is PPSFCH,PRB,k(i)=PPSFCH,one

-	NPSFCH,one,max=Nmax,PSFCH⋅NPSFCH,oneinterlace2+NPSFCH,oneinterlace1⋅10(-PPSFCH,offset/10) for operation with shared spectrum channel access and sl-TransmissionStructureForPSFCH = 'commonInterlace', where NPSFCH,oneinterlace2 is provided by sl-NumDedicatedPRBs-ForPSFCH, PPSFCH,offset is provided by sl-PSFCH-PowerOffset, and NPSFCH,oneinterlace1 is the number of PRBs in the first interlace for all Nmax,PSFCH PSFCH transmissions after excluding PRBs for PSFCH transmissions as described in Clause 16.3.0

-	NTx,PSFCH=Nmax,PSFCH and PPSFCH,ki=PPSFCH,one+10log10NPSFCH,oneinterlace2+NPSFCH,one,kinterlace1⋅10(-PPSFCH,offset/10) [dBm], where NPSFCH,one,kinterlace1 is the number of PRBs in the first interlace for PSFCH transmission(s) among all Nmax,PSFCH PSFCH transmissions within the same RB set of PSFCH transmission k after excluding PRBs for PSFCH transmissions as described in Clause 16.3.0, and the power on one PRB in the first interlace for PSFCH transmission is PPSFCH,PRB,first,k(i)=PPSFCH,PRB,second,ki-PPSFCH,offset and the power on one PRB in the subset of PRBs in the second interlace for PSFCH transmission is PPSFCH,PRB,second,k(i)=PPSFCH,one, where PPSFCH,offset is provided by sl-PSFCH-PowerOffset

-	else

-	the UE autonomously selects NTx,PSFCH PSFCH transmissions in ascending order of corresponding priority field values as described in clause 16.2.4.2 over the PSFCH transmissions with HARQ-ACK information, if any, and then with ascending order of priority value over the PSFCH transmissions with conflict information, if any, such that NTx,PSFCH≥max1,i=1KMi where Mi, 1≤i≤8, is a number of PSFCHs with priority value i for PSFCH with HARQ-ACK information and Mi, i>8, is a number of PSFCHs with priority value i-8 for PSFCH with conflict information and K is defined as

-	the largest value satisfying PPSFCH,one+10log10NPSFCH,oneK≤PCMAX where PCMAX is determined according to [8-1, TS 38.101-1] for transmission of all PSFCHs in i=1KMi, if any

-	NPSFCH,oneK=max1,i=1KMi for operation without shared spectrum channel access

-	NPSFCH,oneK=k=1max1,i=1KMiNPSFCH,one,kinterlace for operation with shared spectrum channel access and sl-TransmissionStructureForPSFCH = 'dedicatedInterlace'

-	NPSFCH,oneK=max1,i=1KMi⋅NPSFCH,oneinterlace2+NPSFCH,oneinterlace1,K⋅10(-PPSFCH,offset/10) for operation with shared spectrum channel access and sl-TransmissionStructureForPSFCH = 'commonInterlace', where NPSFCH,oneinterlace1,K is the number of PRBs in the first interlace for the max1,i=1KMi PSFCH transmissions after excluding PRBs for PSFCH transmissions as described in Clause 16.3.0

-	zero, otherwise

and

-	PPSFCH,k(i)=minPCMAX-10log10(NTx,PSFCH),PPSFCH,one [dBm] for operation without shared spectrum channel access

-	PPSFCH,k(i)=minPCMAX-10log10(k=1NTx,PSFCHNPSFCH,one,kinterlace),PPSFCH,one+10log10NPSFCH,one,kinterlace [dBm] for operation with shared spectrum channel access and sl-TransmissionStructureForPSFCH= 'dedicatedInterlace', where the power on one PRB in the interlace for PSFCH transmission is PPSFCH,PRB,k(i)=minPCMAX-10log10k=1NTx,PSFCHNPSFCH,one,kinterlace,PPSFCH,one

-	PPSFCH,k(i)=minPCMAX-10log10(NTx,PSFCH⋅NPSFCH,oneinterlace2+NPSFCH,oneinterlace1,K⋅10(-PPSFCH,offset/10)),PPSFCH,one+ 10log10NPSFCH,oneinterlace2+NPSFCH,one,kinterlace1⋅10(-PPSFCH,offset/10) [dBm] for operation with shared spectrum channel access and sl-TransmissionStructureForPSFCH = 'commonInterlace', where NPSFCH,oneinterlace1,K is the number of PRBs in the first interlace for the NTx,PSFCH PSFCH transmissions after excluding PRBs for PSFCH transmissions as described in Clause 16.3.0, NPSFCH,one,kinterlace1 is the number of PRBs in the first interlace for PSFCH transmission(s) among the  NTx,PSFCHmax1,i=1KMi PSFCH transmissions which are within the same RB set of PSFCH transmission k after excluding PRBs for PSFCH transmissions as described in Clause 16.3.0, and the power on one PRB in the first interlace for PSFCH transmission is PPSFCH,PRB,first,k(i)=PPSFCH,PRB,second,ki-PPSFCH,offset and the power on one PRB in the subset of PRBs in the second interlace for PSFCH transmission is PPSFCH,PRB,second,k(i)=minPCMAX-10log10NTx,PSFCH⋅NPSFCH,oneinterlace2+NPSFCH,oneinterlace1,K⋅10-PPSFCH,offset/10 ,PPSFCH,one, where PPSFCH,offset is provided by sl-PSFCH-PowerOffset

where PCMAX is determined for the  NTx,PSFCH simultaneous PSFCH transmissions according to [8-1, TS 38.101-1]

-	else

-	PPSFCH,k(i)=PCMAX-10log10(NTx,PSFCH) [dBm] for operation without shared spectrum channel access

-	PPSFCH,k(i)=PCMAX-10log10(k=1NTx,PSFCHNPSFCH,one,kinterlace)+10log10NPSFCH,one,kinterlace [dBm] for operation with shared spectrum channel access and sl-TransmissionStructureForPSFCH = 'dedicatedInterlace', where the power on one PRB in the interlace for PSFCH transmission is PPSFCH,PRB,k(i)=PCMAX-10log10k=1NTx,PSFCHNPSFCH,one,kinterlace

-	PPSFCH,k(i)=PCMAX-10log10(NTx,PSFCH⋅NPSFCH,oneinterlace2+NPSFCH,oneinterlace1⋅10(-PPSFCH,offset/10))+ 10log10NPSFCH,oneinterlace2+NPSFCH,one,kinterlace1⋅10(-PPSFCH,offset/10) [dBm] for operation with shared spectrum channel access and sl-TransmissionStructureForPSFCH = 'commonInterlace', where NPSFCH,oneinterlace1 is the number of PRBs in the first interlace for the NTx,PSFCH PSFCH transmissions after excluding PRBs for PSFCH transmissions as described in Clause 16.3.0, NPSFCH,one,kinterlace1 is the number of PRBs in the first interlace for PSFCH transmission(s) among the NTx,PSFCH PSFCH transmissions which are within the same RB set of PSFCH transmission k after excluding PRBs for PSFCH transmissions as described in Clause 16.3.0, the power on one PRB in the first interlace for PSFCH transmission is PPSFCH,PRB,first,k(i)=PPSFCH,PRB,second,ki-PPSFCH,offset and the power on one PRB in the subset of PRBs in the second interlace for PSFCH transmission is PPSFCH,PRB,second,k(i)=PCMAX-10log10NTx,PSFCH⋅NPSFCH,oneinterlace2+NPSFCH,oneinterlace1⋅10-PPSFCH,offset/10, where PPSFCH,offset is provided by sl-PSFCH-PowerOffset

-	for operation with shared spectrum channel access and sl-TransmissionStructureForPSFCH = ‘commonInterlace’, PPSFCH,k(i) includes the power on PRBs in both the first and second interlaces and, for more than one PSFCH transmissions from the UE, the power on any PRB in the first interlace is not accumulated among the more than one PSFCH transmissions within a same RB set and is same as the power PPSFCH,PRB,first,k(i) on the PRB in the first interlace for PSFCH transmission k.

where the UE autonomously determines NTx,PSFCH PSFCH transmissions with ascending order of corresponding priority field values as described in clause 16.2.4.2 over the PSFCH transmissions with HARQ-ACK information, if any, and then with ascending order of priority value over the PSFCH transmissions with conflict information, if any, such that NTx,PSFCH≥1 and where PCMAX is determined for the NTx,PSFCH PSFCH transmissions according to [8-1, TS 38.101-1].

For resource pools configured with PSFCH resources overlapping in time, the UE either expects not to be provided with dl-P0-PSFCH or dl-Alpha-PSFCH in any of the resource pools, or expects to be provided with the same values of dl-P0-PSFCH and the same values of dl-Alpha-PSFCH for all the resource pools.