3GPP TSG RAN WG1 #118                                                                 R1-2407312

Maastricht, NL, August 19th – 23rd, 2024

Agenda Item:	9.4.2.4

Source:	Moderator (Spreadtrum Communications)

Title:	FL summary#1 on CW waveform characteristics for A-IoT

Document for:	Discussion and decision

Introduction

This feature lead summary (FLS) concerns the Rel-19 study item (SI) on solutions for Ambient IoT (Internet of Things) in NR [1]. The detailed objectives can be seen in appendix (section 9.2). This Rel-19 WGs level study item was preceded by a Rel-18 RAN study item [2, 3] and a Rel-19 SA1 study item [4, 5].

The final CW (carrier wave) FLS in last RAN1 meeting can be found in [6], and the discussion in RAN#103 and RAN#104 can be found in [7] and [8]. The achieved agreements in previous meetings are attached in section 9.3. The agreements achieved in this meeting will be added in section 9.4.

The Contact Information for 9.4.2.4 is attached in section 9.1, please feel free to update the contact info, if any.

This document summarizes contributions [9]-[40] submitted to agenda item 9.4.2.4, i.e., discussion on the characteristics of carrier-wave waveform for a carrier wave provided externally to the Ambient IoT device. The section arrangement and numbering in this document based on the submitted contributions, the issues that included in the previous agreements or have been discussed extensively are placed at the front. According to the workplan, this is the last SI meeting for CW part, we will focus on essential issues first.

If the summary missed or misunderstood companies’ views, FL apologizes for that, and companies can feel free to contact with FL or correct the position/views in the summary.

The issues in this document are tagged and color coded with High Priority or Medium Priority or Low priority. The issues that were in the focus of this round of the discussion are tagged FL1.

Follow the naming convention in this example:

- .. characteristics for A-IoT -v000.docx

- .. characteristics for A-IoT -v001-CompanyA.docx

- .. characteristics for A-IoT -v002-CompanyA-CompanyB.docx

- .. characteristics for A-IoT -v003-CompanyB-CompanyC.docx

There is NO need to send an info email to the reflector to inform that you have uploaded a new version.

CW waveform

Characteristics of CW waveforms

In RAN1#116bis, it was agreed to study single-tone unmodulated sinusoid waveform and multiple unmodulated single-tone waveform (section 9.3). For these waveforms, several characteristics was considered for the study.

Agreement Study at least the following characteristics of unmodulated single-tone and multiple unmodulated single-tone CW waveforms for backscattering: - For D2R - Reception performance - Spectrum utilization of backscattered signal corresponding to the CW waveforms - CW interference suppression at D2R receiver - Including complexity and CW cancellation capability value/range (if any) - For scenarios ’A1’, ’A2’ and ’B’ - Relative complexity of CW generation

For multiple unmodulated single-tone waveform, it was agreed that two unmodulated single-tones as a starting point. Section 2.1.1 to 2.1.4 discuss the above characteristics for single-tone unmodulated sinusoid waveform and two unmodulated single-tone waveform.

D2R reception performance [Open]

In RAN1#117, the following observation was achieved.

Observation For D2R reception performance, - Compared to single-tone unmodulated sinusoid waveform without frequency hopping, two unmodulated single-tones waveform provides [X Y] dB frequency diversity gain in a fading channel, at least depending on the gap between the two tones and the channel’s coherence bandwidth. - Note: The total transmission power is assumed the same for single-tone unmodulated sinusoid waveform without frequency hopping and two unmodulated single-tones waveform. - Note: For two unmodulated single-tones waveform, assume the two tones are transmitted from the same CW node.

In this meeting, contributions [10], [11], [12], [15], [14], [16], [21], [22], [23], [25], [26], [32], [36], continue discussed D2R reception performance for single-tone unmodulated sinusoid waveform and two unmodulated single-tones waveform. For the frequency diversity gain (i.e., [X Y]), the following contributions provided simulation results. The performance gain and corresponding parameters are captured below.

Table 2.1.1-1 Quantitative analysis for D2R reception performance (Single-tone vs. Two single-tones)

| Source | BLER | Channel | Gain | Part of parameters or conditions* |
| --- | --- | --- | --- | --- |
| [11] | 1% | TDL-A | 0, 5-7 dB | 1 Rx, OOK, 96 bits, TDL-A (30 and 150 ns), Manchester, gap{180 kHz, 2.16 MHz, and 5 MHz} |
| [14] | 1% | TDL-A | {0.2dB, 3dB} | 1 Rx, OOK, 16bits, TDL-A(30 ns ), Manchester, gap{180KHz, 2.16MHz} |
| [21] | 1% | TDL-A | [X=1 Y=8] dB | 1 Rx, OOK, 96 bits+16 bits CRC, TDL-A(30 ns), Miller-8, gap {180kHz, 1.08MHz, 2.16MHz, 5MHz} |
| [23] | 1% | TDL-A | 4 dB or 3dB | 1 Rx, OOK, 20 bits + 6 bits CRC, 1Tx, TDL-A 30ns, 1/2 code rate - Manchester, gap{360kHz, 3MHz, 5.01 MHz} |
| [32] | 1% | TDL-A | [0, 5.5] dB | 1Rx, BPSK, 96 bits, TDL-A 30ns, gap{75KHz, 180KHz, 900KHz, 3.3MHz, 3.6MHz, 4.2MHz} |
| [36] | 1% | TDL-A | [~0, 4, 8]dB | 1Rx, BPSK, 128bits, TDL-A (30 and 150 ns), gap{180kHz, 2.16MHz, 10MHz}, 1Tx CW transmission antenna |
| [12] | 1% | TDL-D | 0.7 dB | 1 Rx, BPSK, 96 bits, TDL-D(30 ns), Manchester, gap{10MHz}, 105 SFO. |
| [16] | 1% | TDL-C | 6 dB | 1Rx, OOK, 16bits + 8bit CRC, TDL-C(30ns), 3 km/h, 1Rx, gap{7MHz} |
| [11] | 10% | TDL-A | 0, 1-3 dB | 1 Rx, OOK, 96 bits, TDL-A (30 and 150 ns), Manchester, gap{180 kHz, 2.16 MHz, and 5 MHz} |
| [26] | 10% | TDL-A | 3dB | 1Rx, OOK, 96 bits+16 bits CRC, TDL-A (30 and 150 ns), Manchester, gap{5MHz }. |
| [12] | 10% | TDL-D | -0.2 dB | 1 Rx, BPSK, 96 bits, TDL-D/TDL-A(30 ns), Manchester, gap{10MHz}, 105 SFO. |
| [16] | 10% | TDL-C | 2dB | 1Rx, OOK, 16bits + 8bit CRC, TDL-C(30ns), 3 km/h, 1Rx, gap{7MHz} |
| *Note: Parameters here are for reference, details can be found in corresponding contributions. |  |  |  |  |

In addition, contribution [12] provided performance simulation for a reader with 2 Rx antennas in TDL-A channel, and 0.1 / 4.5 dB gain at 10% / 1% BLER were observed. Contribution [36] provided performance simulation for a CW transmitter with 2 Tx antennas in TDL-A channel, and observed that two-tone without frequency hopping on 2-tx ants is slightly better (1dB) than single-tone w/ Tx switching but w/o FH on 1-tx ant and much better (7dB) than 1-tx single tone w/o FH.

In addition to quantitative analysis for D2R reception performance, some contributions continued to share qualitative analysis. Contribution [10] observed that if more scatterers are present between the reader and the device, Single-tone  based communication cannot be feasible, Two unmodulated single-tones Robustness against fading. However, if the total transmitted power is fixed, coverage may be sacrificed due to power distribution among the CW tones transmitted. Contribution [22] generally repeated the agreed observations, that two unmodulated single-tones waveform provides frequency diversity gain, at least depending on the gap between the two tones and the channel’s coherence bandwidth. Contribution [25] indicated that frequency selective gain of two unmodulated single-tones compared with single-tone is limited due to the large coherence bandwidth in the typical A-IoT deployment scenarios. Lower transmission power of each single-tone of two single-tones has an impact on the reception performance of two unmodulated single-tones. FL understand that coverage can be discussed in 9.4.1.1, if needed, while the other quantitative analysis can be reflected by quantitative analysis, no additional observations are needed.

Based on the above and the tables in appendix 10.1, there can be the following observations:

- For a reader with 1Rx and for a CW transmitter with 1Tx, two unmodulated single-tones waveform provides {0, 8}dB/{-0.2, 3}dB frequency diversity gain at 1%/10% BLER target in a fading channel (including TDL-A/D/C), at least depending on the gap between the two tones and the channel’s coherence bandwidth.

  - In a TDL-A or C fading channel with 30ns delay spread

    - For the gap is between {75KHz, 900KHz}, 6 sources observed {0, 1.5}dB frequency diversity gain at 1% BLER target, and 3 sources observed no frequency diversity gain at 10% BLER target.

    - For the gap is between {1.08MHz, 4.2MHz}, 6 sources observed {3, 5.8}dB frequency diversity gain at 1% BLER targe, and 3 sources observed {0.4, 2.5}dB frequency diversity gain at 10% BLER target.

    - For the gap is between {5MHz, 10MHz}, 5 sources observed {5, 8}dB frequency diversity gain at 1% BLER target, and 5 sources observed {1.3, 4}dB frequency diversity gain at 10% BLER target.

  - In a TDL-D fading channel with 30ns delay spread

    - For the gap is 10MHz, 1 sources observed 0.7 dB@1%BLER and -0.2dB@10%BLER frequency diversity gain.

  - In a TDL-A fading channel with 150ns delay spread

    - For the gap is 180Khz, 2 sources observed {1, 3}dB frequency diversity gain at 1% BLER target, and 2 sources observed {0, 2.5}dB frequency diversity gain at 10% BLER target.

    - For the gap is 2.16MHz, 2 sources observed {7, 8}dB frequency diversity gain at 1% BLER target, and 2 sources observed {2.5, 5.5}dB frequency diversity gain at 10% BLER target.

    - For the gap is 5MHz, 2 sources observed {7, 8}dB frequency diversity gain at 1% BLER target, and 2 sources observed {2.5, 3}dB frequency diversity gain at 10% BLER target.

- For a reader with 2Rx, 1 source observed that the spatial diversity gain obtained through the 2 Rx antenna compensates the loss of frequency diversity for the unmodulated single-tone carrier-wave, and 0.1 or 4.5 dB gain at 10% or 1% BLER can be achieved.

- For a CW transmitter with 2Tx, 1 source observed two-tone transmitted on 2-Tx provides slightly higher (1dB) performance gain than single-tone with Tx switching but without frequency hopping on 1-Tx ant, and provides much higher (7dB) performance gain than 1-tx single tone without frequency hopping.

Therefore, the following observation is considered.

FL1 High Priority proposal 2.1.1-1a: Update the agreed observation for D2R reception performance in RAN1#117 as follows:

Observation

For D2R reception performance,

- Compared to single-tone unmodulated sinusoid waveform without frequency hopping, two unmodulated single-tones waveform provides [0, 8] dB or [-0.2, 3]dB frequency diversity gain at 1% or 10% BLER target in a fading channel for a 1Rx reader and a 1Tx CW transmitter, at least depending on the gap between the two tones and the channel’s coherence bandwidth.

  - In a TDL-A or C fading channel with 30ns delay spread

    - For the gap between [75KHz, 900KHz], the frequency diversity gains at 1% BLER target observed by 6 sources are within {0, 1.5}dB, and the frequency diversity gains at 10% BLER target observed by 3 sources are almost 0dB.

    - For the gap between {1.08MHz, 4.2MHz}, the frequency diversity gains at 1% BLER target observed by 6 sources are within {3, 5.8}dB, and the frequency diversity gains at 10% BLER target observed by 3 sources are within {0.4, 2.5}dB.

    - For the gap between {5MHz, 10MHz}, the frequency diversity gains at 1% BLER target observed by 5 sources are within {5, 8}dB, and the frequency diversity gains at 10% BLER target observed by 5 sources are within {1.3, 4}dB.

  - In a TDL-D fading channel with 30ns delay spread

    - For 10MHz gap, 1 sources (R1-2405855) observed 0.7 dB@1%BLER and -0.2dB@10%BLER frequency diversity gain. (Note: loss due to the power split in TDL-D)

  - In a TDL-A fading channel with 150ns delay spread

    - For the gap is 180Khz, the frequency diversity gains at 1% BLER target observed by 2 sources are within {1, 3}dB, and the frequency diversity gains at 10% BLER target observed by 2 sources are within {0, 2.5}dB.

    - For the gap is 2.16MHz, the frequency diversity gains at 1% BLER target observed by 2 sources are within {7, 8}dB, and the frequency diversity gains at 10% BLER target observed by 2 sources are within {2.5, 5.5}dB.

    - For the gap is 5MHz, the frequency diversity gains at 1% BLER target observed by 2 sources are within {7, 8}dB, and the frequency diversity gains at 10% BLER target observed by 2 sources are within {2.5, 3}dB.

  - Note: The total transmission power is assumed the same for single-tone unmodulated sinusoid waveform without frequency hopping and two unmodulated single-tones waveform.

  - Note: For two unmodulated single-tones waveform, assume the two tones are transmitted from the same CW node.

| Company | Y/N | Comments |
| --- | --- | --- |
| OPPO | Yes |  |
| vivo | Yes |  |
| Huawei, HiSilicon | Yes | Since this is capturing the different evaluation results provided by companies, we are generally fine with capturing this observation in the TR. |
| Xiaomi | Yes | We are fine with the proposal. However, the current simulation assumptions are still not convergent, we propose to further align the simulation assumptions on the channel, frequency gap between two tones, and TX/RX number, with this alignment, it is better to compare the result among companies and determine the frequency gap between the two tones. |
| LGE | Yes |  |
| Lenovo | Yes |  |
| Samsung | Yes | We are generally fine with this proposal. |
| CEWiT | Yes | Fine with the proposal |
| Ericsson | Y |  |
| FL | FL1 High Priority proposal 2.1.1-1a will be further checked during Tuesday offline. |  |

Regarding the observations for 2Rx and 2Tx, as there is only one company for each part, an independent observation is proposed to further check companies’ view.

FL1 Medium Priority Observation 2.1.1-2a: For D2R reception performance,

- For a reader with 2Rx, one source observed that the spatial diversity gain obtained through the 2Rx antenna compensates the loss of frequency diversity for the unmodulated single-tone carrier-wave, and 0.1 / 4.5 dB gain at 10% / 1% BLER can be achieved for two unmodulated single-tones waveform compared to single-tone unmodulated sinusoid waveform without frequency hopping .

- For a CW transmitter with 2Tx, one source observed two-tone transmitted on 2Tx provides slightly higher(1dB) performance gain than single-tone with Tx switching but without frequency hopping on 1Tx, and provides much higher (7dB) performance gain than 1Tx single-tone unmodulated sinusoid waveform without frequency hopping.

| Company | Y/N | Comments |
| --- | --- | --- |

D2R spectrum utilization

In last meeting, D2R spectrum utilization was discussed [6]. The latest FL proposal is as follows

FL5 High Priority Proposed Observation 2.1.2-1e: For spectrum utilization of backscattered signal corresponding to the CW waveforms, - Compared to single-tone unmodulated sinusoid waveform without frequency hopping, two unmodulated single-tones waveform: - Consumes up to 2 times the spectrum resources for D2R transmission, depending to the gap between the two tones. 2 times the spectrum resources are needed to achieve lager frequency diversity gain. - Note: For two unmodulated single-tones waveform, assume the two tones are transmitted from the same CW node.

In this meeting, contributions [9], [10], [11], [12], [14], [16], [19], [21], [22], [25], [26], [33], [35], [36], [38] continue discussed D2R spectrum utilization for single-tone unmodulated sinusoid waveform and two unmodulated single-tone waveform.

Required spectrum resources/BW [Open]

Contributions [9], [12], [14], [16], [19], [22], [25], [26], [33], [35], [36], [38] shared views for required spectrum resources/BW for D2R, and summarized as follows:

Table 2.1.2.1-1 Views for required spectrum resources/BW for D2R

|  | Single-tone unmodulated sinusoid waveform vs.Two unmodulated single-tones |
| --- | --- |
| General (2 sources) | - Two single-tone utilize more D2R spectrum resources than the single-tone. [9], [38] |
| Two times required resources (10 sources) | - The backscattered signal corresponding to the external carrier-wave with the two unmodulated single-tone waveform utilizes 2 times the frequency resources when compared to a single-tone waveform ([12], [14], [16], [19], [22], [25], [26], [33], [35], [36]) , depending on the gap between the two tones ([25], [33]). In addition, large guard band is needed between the D2R backscattered based on each tone in case of parallel two-tone [36]. |
| More than two times the required resources (3 sources) | - Consumes at least 2 times the spectrum resources for D2R transmission, depending on the gap between the two tones. [25] - two unmodulated single-tones will consume at least two times the bandwidth of baseband signal, which will lead to lower spectrum efficiency. [26] - If the nonlinear factor of the reflection amplifier cannot be ignored, two tones will produce intermodulation components during the backscattering on the tag side, which may occupy more resources (larger than two times).[14] |

For the required spectrum resources/BW, almost all the contributions observed that compared to single-tone unmodulated sinusoid waveform without frequency hopping, two unmodulated single-tones waveform consumes 2 times the spectrum resources for D2R transmission. Some contributions indicated that more than two times the spectrum resources are needed.

According to the discussion in the last RAN1 meeting, some views mentioned that, if the gap between two tones is small (corresponding to less frequency diversity gain), the D2R transmission may be overlapped, then the required spectrum resources may be smaller than 2 times.

Therefore, the following observation can be considered.

FL1 High Priority Observation 2.1.2.1-1a: For the required spectrum resources/BW of backscattered signal corresponding to the CW waveforms,

- Compared to single-tone unmodulated sinusoid waveform without frequency hopping, two unmodulated single-tones waveform consumes up to 2 times or more the spectrum resources for D2R transmission.

  - If the gap between the two tones is large enough to achieve larger frequency diversity gain, 2 times the spectrum resources are needed.

  - If intermodulation components exist, more than 2 times resources may needed.

  - Note: For two unmodulated single-tones waveform, assume the two tones are transmitted from the same CW node.

| Company | Y/N | Comments |
| --- | --- | --- |
| OPPO | Yes with comments | In our understanding, more than 2 times resources may be needed mainly due to the potential guard band for intermodulation components. Thus, we propose to change the second sub-bullet as “More than 2 times resources may be needed due to the potential guard band at least for intermodulation components” |
| TCL | Y | In our understanding, how to enhance IMD to reduce to the impact of intermodulation components is a implementation problem at reader. If the gap between two-tone is large, the 3rd intermodulation could be filtered by device’s antenna or matching network. In addition, limited by the receiving sensitivity, if IMD is enhanced by other method (like DPD) at reader side, 3rd intermodulation will not be received by device because of the pathloss. Thus, we think intermodulation components could be ignored for D2R spectrum resource discussion. |
| vivo |  | For the intermodulation bullet, the inter modulation always exists, but if the power of intermodulation is lower than the emission requirements (e.g., spurious emissions), no additional resource needed. Besides, whether additional intermodulation suppression is needed depends on transmission power, IP3 value of PA, gap between the two tones, parameter for existing RF filters, and RF requirements we concerned. More analysis is needed to justify this issue. |
| Huawei, HiSilicon | Yes | This observation mentions twice the spectrum resources being consumed by the two single-tones waveform, but what is required is that the frequency gap between each of these two tones need to be no smaller than the transmission bandwidth of the corresponding D2R transmission. The impact of the harmonics in the backscattered signal has to be additionally considered, which can require larger frequency gap. - Compared to single-tone unmodulated sinusoid waveform without frequency hopping, two unmodulated single-tones waveform consumes up to 2 times or more the spectrum resources for D2R transmission requires that the frequency gap between the two tones have to be no smaller than the transmission bandwidth of the corresponding D2R transmission. For the intermodulation components, it is fine to capture that more than 2 may be needed, but it would also have an impact on the CW generation complexity since it may generate out-of-band intermodulation interference, which requires additional interference suppression to meet the out-of-band emission requirements. |
| Xiaomi |  | We think the sub bullet is enough and clear, but the first sub sub-bullet is not clear and bring confusion e.g., how to define that the gap between the two tones is large enough, and how to define larger frequency diversity gain, we propose to make it clear or delete it. |
| Samsung |  | For the second bullet, we believe that the impact of intermodulation is more of an implementation issue. If we want to address this in this observation, the assumptions under which such a situation could occur (for example, when intermodulation that is not filtered out by the device's matching network is present) also need to be provided. |
| Ericsson | N | In our view, this proposal is about the bandwidth of wanted signal, i.e., the total bandwidth of each sideband of backscattered transmission. Note that the total bandwidth covers multiple non-contiguous segments. As long as there is no overlapping between sidebands of two tones, the total bandwidth of backscattered transmission of a two-tone CW is two times of that of single-tone CW. This is independent from small or large frequency gap between two tones to achieve frequency diversity gain. Intermodulation component exists with two tones, but it is not considered as the bandwidth of wanted signal. |
| FL | Based on the feedback above, it seems views for intermodulation is not converged for now, Samsung and TCL think that this would be an implementation issue, vivo points out that if the power of intermodulation is lower than the emission requirements (e.g., spurious emissions), no additional resource needed. Ericsson understand that this proposal is about the bandwidth of wanted signal, then intermodulation component is not need considered. FL thinks that Ericsson’s understanding is a good suggestion for move forward. The following version can be considered. FL1 High Priority Observation 2.1.2.1-1b: For the D2R transmission BW corresponding to the CW waveforms, - Compared to single-tone unmodulated sinusoid waveform without frequency hopping, two unmodulated single-tones waveform - requires 2 times the frequency domain spectrum resources for D2R transmission, if the frequency gap between each of these two tones is no smaller than the transmission bandwidth of the corresponding D2R transmission. - Note: For two unmodulated single-tones waveform, assume the two tones are transmitted from the same CW node. |  |

Spectrum efficiency [Open]

Contributions [9], [10], [11], [19], [21], [25], [26], [36], [38] shared views for spectrum efficiency for D2R, and summarized as follows:

Table 2.1.2.2-1 Views for spectrum efficiency for D2R

|  | Single-tone unmodulated sinusoid waveform vs.Two unmodulated single-tones |
| --- | --- |
| Two-tones lead to lower spectrum efficiency (6 sources) | - Two unmodulated single-tones will consume two times the bandwidth of baseband signal, which will lead to lower spectrum efficiency. [19] - D2R based on two tone CW achieves better performance at expense of lower spectrum efficiency. [21] - Assuming same total system BW, the two-tone without FH has less spectrum efficiency than single-tone without FH due to more restriction on D2R transmission BW. [36] - Spectrum efficiency would decrease with the increasing frequency gap size between two single-tones due to other signal(s) should not be transmitted between two tones [25] - At the same target reception performance, two -tones leads to lower spectrum efficiency but a better SNR working point compared to single-tone waveform without hopping.[26] - Two single-tone waveforms utilize more D2R spectrum resources and reduce the available D2R system BW for frequency-domain multiplexing in the D2R transmission. [9] |
| Two-tones lead to higher spectrum utilization (1 source) | - Overall power per frequency contribution from the backscattered transmission of a dual-tone carrier wave might be almost similar to that of single-tone carrier wave.[11] - Using a multi-tone CW can improve spectrum utilization compared with a single-tone, when the latter leads to a large portion of the spectrum remaining underutilized for the cases of 1) transmission bandwidth of a device << allocated system bandwidth and 2) a small number of FDMed devices.[11] |

For spectrum efficiency, most contributions observed that compared to single-tone unmodulated sinusoid waveform without frequency hopping, two unmodulated single-tones waveform will lead to lower spectrum efficiency, while one contribution observed that for the cases of 1) transmission bandwidth of a device << allocated system bandwidth and 2) a small number of FDMed devices, multi-tone CW can improve spectrum utilization. FL understand that different company has different understandings for D2R bandwidth, so FL suggests considering it from two perspectives.

Based on the above and discussions in last RAN1 meeting, the following observations can be considered.

FL1 High Priority Observation 2.1.2.2-1a: For D2R spectrum efficiency corresponding to the CW waveforms,

- From D2R occupied bandwidth perspective, compared to single-tone unmodulated sinusoid waveform without frequency hopping, two unmodulated single-tones waveform will lead to lower spectrum efficiency.

  - Note: For two unmodulated single-tones waveform, assume the two tones are transmitted from the same CW node.

| Company | Y/N | Comments |
| --- | --- | --- |
| OPPO | Yes |  |
| TCL | Yes | Okay |
| Huawei, HiSilicon | No | CW does not carry data, thus does not have a spectral efficiency defined: efficiency is necessarily a ratio of two quantities. We suppose this refers to the backscattered signal from the CW, and this should be reflected more exactly. Otherwise, parts such as the NOTE are rather confusing, e.g. perhaps the NOTE is irrelevant once we are talking about D2R link. |
| Xiaomi | Y | We are fine with the observation. |
| Samsung | Y |  |
| CEWiT | Yes |  |
| Ericsson | N | In our view, section 2.1.2.1 discusses the required frequency-domain resources for the backscattered transmission, which corresponds to FDMA, frequency resources allocated for the wanted signal. Regardless of 1-tone CW or 2-tone CW, the frequency resources not used for backscattered transmission are not counted, for example, those between two images of a tone. We discuss spectrum efficiency by considering the unwanted emission, like harmonics. We take into account the total of spectrum used for backscattered transmission and that polluted by the harmonics with power above a threshold in the comparison. However, some observations in Table 2.1.2.2-1 are still about the bandwidth of wanted signal, the same as section 2.1.2.1, and some others seem to consider the frequency span from the lowest frequency resource to the highest one, including the frequency resources not used by backscattered transmission. One underlying assumption is that the frequency resources in the span can't be used for other devices or other purposes. Could FL please clarify what the spectrum efficiency in 2.1.2.2 is? Is it about the spectrum of both wanted signal and unwanted signal or the spectrum span from the lowest to the highest frequency resources? For the latter, is it reasonable to assume the resources within the span not allocated for the backscattered transmission of a device cannot be used by other devices? Namely, no FDMA is assumed in the comparison. |

FL1 Medium Priority Observation 2.1.2.2-2a: For D2R spectrum efficiency corresponding to the CW waveforms,

- From D2R system bandwidth perspective, compared to single-tone unmodulated sinusoid waveform without frequency hopping, two unmodulated single-tones waveform can improve spectrum utilization for the cases of 1) transmission bandwidth of a device << allocated system bandwidth and 2) a small number of FDMed devices

  - Note: For two unmodulated single-tones waveform, assume the two tones are transmitted from the same CW node.

| Company | Y/N | Comments |
| --- | --- | --- |
| TCL | Y | For case 2, it is confuse why we should emphases a small number of FDMed devices. If two-tone as CW to be used for D2R FDMed, we think spectrum utilization may not be improved because of the impact of harmonic/2SB/3rd intermodulation. |
| Huawei, HiSilicon |  | We are not sure about the motivation for this observation and would like more clarity on how two single-tones waveform can improve spectral efficiency if the transmission bandwidth of a device << allocated system bandwidth. |
| Xiaomi | Y |  |
| Ericsson | Y | The observation provides a view from the perspective of multiple devices. |

CW interference suppression at D2R receiver

Interference suppression complexity comparison for CW waveforms at D2R receiver [Open]

In last meeting, interference suppression comparison for different CW waveforms at D2R receiver was discussed [6]. One observation was achieved.

Observation For CW interference suppression at D2R receiver, - Compared to single-tone unmodulated sinusoid waveform without frequency hopping, two unmodulated single-tones waveform: - Requires additional complexity if RF interference cancellation is used at least with CW waveform reconstruction. - Note: RF interference cancellation is needed when the received CW interference power exceeds the blocking threshold of the receiver - Note: For two unmodulated single-tones waveform, assume the two tones are transmitted from the same CW node.

In this meeting, contributions [10], [12], [14], [25], [26], [32], [36], [38] continue shared views for Interference suppression comparison. Contributions [10][36] generally think that two-tone CW has more complicated interference suppression than single-tone. Contribution [14], [25], [26], [32], [38] indicated that two tones waveform requires additional complexity on narrow band-pass filtering.

Table 2.1.3.1-1 Views for interference suppression complexity comparison

|  | Single-tone unmodulated sinusoid waveform vs.Two unmodulated single-tones |
| --- | --- |
| General (2 sources) | - Two-tone CW has more complicated interference suppression than single-tone[10][36] - the interference suppression is required per tone independently [36] |
| Band-pass filtering impacts (4 sources) | - Two-tone CW requires additional complexity on narrow BPF[14] [25] [26] [32] - especially for DSB D2R transmission [14] - The smaller bandwidth with the BPF is needed [25] - narrow band BPF for separating each single-tone[32] |
| Challenging to reconstruct (2 sources) | - Additional complexity on RF interference cancellation, e.g., reconstruct the CW waveform for two tones[32][38] |

In addition, contribution [12] indicated that the increased hardware and software complexity for the support of the multiple unmodulated single-tone waveform carrier-wave could be too high. [12] proposed to add a note for the agreed observation that “ Note: The two unmodulated single-tones waveform can be too complicated to be supported by an intermediate UE in D2T2.”

Based on the above, it seems majority observed that two unmodulated single-tones requires additional complexity for narrow band bandpass filtering. For RFIC/reconstruct, FL understand that this observation is included in the agreed observation. Therefore, the following observation is considered.

FL1 High Priority Observation 2.1.3.1-1a: For interference suppression complexity for different CW waveforms at D2R receiver,

- Compared to single-tone unmodulated sinusoid waveform without frequency hopping, two unmodulated single-tones:

  - Requires additional complexity for narrow band-pass filtering, as two tones lead to smaller pass-band bandwidth and smaller guard band for each RF band-pass filter, both of which increase the implementation complexity.

  - Note: Two unmodulated single-tones waveform would be too complicated to be supported by an intermediate UE in D2T2.

  - Note: For two unmodulated single-tones waveform, assume the two tones are transmitted from the same CW node.

| Company | Y/N | Comments |
| --- | --- | --- |
| OPPO | Yes |  |
| TCL | Y | Good proposal. One question is whether narrow band-pass filtering is only for RF or not? |
| Qualcomm | Comment | We prefer to delete the 1st note. Whether it is feasible to use two tones in T1 or T2 needs separate discussion. For 1st subbullet, it is too specific and different implementation may be used for CW-IC at reader size. It’s better to capture more general description that two tones require interference suppression per tone separately in parallel. |
| vivo |  | The observation of using narrow band-pass filters is generally true. However, it seems further details assuming bandpass filter as one example for interference cancellation in implementation. We are also fine if there is no further observation on interference suppression for comparison between single tone CW and 2 tone CW, in addition to the observations we made last meeting. |
| Huawei, HiSilicon | Yes | What is the relevance to this observation of the NOTE? It seems that note should be its own agreement (or is already since Fukuoka) and only adds noise by repeating it in each place. |
| Xiaomi | N | We do not support the last second note. In the D2T2, if the CW node is outside, only the CLI exists at the intermediate UE, but the CLI is weaker than self-interference and can be simply suppressed by the antenna and physical isolation. So we make the following revision with the blue part: FL1 High Priority Observation 2.1.3.1-1a: For interference suppression complexity for different CW waveforms at D2R receiver, - Compared to single-tone unmodulated sinusoid waveform without frequency hopping, two unmodulated single-tones: - Requires additional complexity for narrow band-pass filtering, as two tones lead to smaller pass-band bandwidth and smaller guard band for each RF band-pass filter, both of which increase the implementation complexity. - Note: Two unmodulated single-tones waveform would be too complicated to be supported by an intermediate UE in D2T2. - Note: For two unmodulated single-tones waveform, assume the two tones are transmitted from the same CW node. |
| Samsung |  | Rather than giving a very specific example like narrow band-pass filtering, it would be more appropriate to explain that additional complexity may be required when individual cancellation is applied to each of the two tones. We also prefer to delete the first note. |
| Ericsson | N | The observation includes two aspects. - Two-tone CW requires two bandpass filters, while only one filter is needed for 1-tone CW. - With the same system bandwidth for both single tone and two tones, backscattered transmission of a two-tone CW may require smaller passband bandwidth and smaller guard band. If a larger system bandwidth is allocated D2R transmission based on two-tone CW in order to better leverage frequency diversity gain, the observation of smaller passband BW and smaller guard band may not hold. Regarding Note1, it is not clear to use which of the two aspects is too complicated. Is an intermediate UE unable to support two bandpass filters or unable to support a filter with a small passband bandwidth? |
| FL | Based on the feedback, the following version is considered for Tuesday offline discussion. FL1 High Priority Observation 2.1.3.1-1b: For interference suppression complexity for different CW waveforms at D2R receiver, - Compared to single-tone unmodulated sinusoid waveform without frequency hopping, two unmodulated single-tones: - Requires two RF narrow-band bandpass filters. when individual cancellation is applied to each of the two tones - Note: For two unmodulated single-tones waveform, assume the two tones are transmitted from the same CW node. |  |

Interference suppression capability [Open]

Contributions [10], [11], [12], [14], [15], [16], [19], [21], [22], [23], [25], [26], [30], [33], [34], [36] , [40] discussed interference suppression and capability.

Contribution [10] propose to study if mitigation procedures/techniques based on existing tools in RAN1 specification can be used. Contribution [22] and [40] prefer to study interference management/mitigation techniques should be studied for AIoT devices. Contribution [22] thinks that at least for cross-link interference, normative work for interference suppression solution may needed.

Some contribution proposed some interference suppression methods, and the views are summarized below.

Table 2.1.3.2-1 Interference suppression for different interference types at D2R receiver

| Interference type | Interference suppression methods |
| --- | --- |
| Self-interference | - Spatial domain cancellation [19] [23][25] [26] [34] [40] - antenna isolation [19] [23] [26] [34] [40], additional circulator[34] [40] or directional coupler [26] [40], duplexer[23], Physical shielding or cross-polarization [25] - Analog cancellation [19] [25] [26] - RF-IC suppression [19] [26] [34], BB filtering [26] [34], BB/IF self-interference cancellation [26] , BPF [23] - Digital cancellation [19] [23][25] [26] - channel estimation[25], high pass filter [26] [34], and/or spatial signalling processing [26] - Frequency shift for PDRCH at device, if supported [26] [40] [18], [25], [27], |
| Cross-link interference | - Spatial domain cancellation [19] [23][25] [26] [40] - antenna separation/ isolation [19][25] [26], antenna design [26], circulator [34] - Analog cancellation [19] [26] [34] [40] - BB filtering [26], BB/IF interference cancellation [26] - Digital cancellation [19] [26] [40] - high pass filter [26] [34], digital BB interference cancellation [26] - Design for channel estimation and frequency & timing offset estimation [25][15][33] - Frequency shift for PDRCH at device, if supported [26] [40] [18], [25], [27] |

For whether frequency shift should be supported, contribution [19] further pointed out that the frequency shift operation may introduce mirror interference which causes a degradation of R2D reception performance. Contribution [22] thinks that it can be assumed that A-IoT devices do not support large frequency shift as the baseline for study. Contribution [23] indicated that at least large frequency shift on device 2a and device 2b shall be supported, while contribution [25] thinks that frequency shifting introduced by FSK modulation and Miller-M code is beneficial to avoid the self-interference.

From the above, at least spatial domain, analog domain, digital domain, and frequency shift can be potential methods for interference suppression. For these method, some contributions further discussed  interference suppression capability.

Contribution [11] thinks that different values for CW interference cancellation can be considered for: 1)D1T1 and D2T2; 2) scenarios A2 (monostatic) and A1/B (bistatic), 3) single-tone or multi-tone CW, 4) frequency shift applied by the device. The received backscattered signal level should be taken into account in the discussion CW signal suppression/cancellation. Contribution [34] propose to study CW cancellation capability at least from spatial isolation, RF interference cancelation and BB interference cancelation. Contribution [30] thinks that companies should provide receiver structures for CW interference cancellation capability and cancellation schemes for the coverage evaluation. Send an LS to RAN4 requesting CW cancelation capability at D2R reader side.

The reported CW interference suppression capabilities are listed below.

Table 2.1.3.2-2 CW interference suppression capability for A2 (self-interference)

| Source | Part1 spatial domain | Part2 analog domain | Part3 Digital Domian | Others | Total capability |
| --- | --- | --- | --- | --- | --- |
| For D1T1 |  |  |  |  |  |
| [11] R1-2405829 | 30dB |  |  |  |  |
| [12] R1-2405855 | ~40 dB | 20 dB | 80 dB |  | 140 dB |
| [15] R1-2405970 | 77dB | 45dB | 10dB |  | 132dB |
| [21] R1-2406189 | 30 dB | 50 dB | No |  | 146 dB |
| [36] R1-2407036 | 60-80dB | 0dB | 60dB |  | 120~140dB |
| For D2T2 |  |  |  |  |  |
| [12] R1-2405855 |  |  |  |  | 130dB |
| [21] R1-2406189 | 15 dB | 50 dB | No |  | 121 dB |
| [15] R1-2405970 | 30dB | 45dB | 10dB |  | 85dB |
| [36] R1-2407036 | 30-80dB isolation |  | 60dB |  | 90-140dB |
| General |  |  |  |  |  |
| [14] R1-2405915 | 30-50 dB | 42-60dB | 43-60dB |  | 110-155dB |
| [16] R1-2405992 | 10-30dB | 20-30dB | Up to 80dB |  |  |
| [25] R1-2406375 | 40dB | 30dB | 50dB | 20dB(Frequency shifting) |  |
| [26] R1-2406408 | 40-60 dB, | 10-30 dB | Up to 80 dB |  | 130-150 dB |
| Note: Please refer to the source(contribution) for specific numerical analysis and calculation methods |  |  |  |  |  |

Table 2.1.3.2-3 CW interference suppression capability for A1/B (cross-link interference)

| Source | Part1 spatial domain | Part2 analog domain | Part3 Digital Domian | Others | Total capability |
| --- | --- | --- | --- | --- | --- |
| For D1T1 |  |  |  |  |  |
| [12] R1-2405855 | ~60 dB @ 25 m ~70 dB @ 70 m | 20 dB | 80 dB |  | 150-160dB dB |
| [15] R1-2405970 | 77dB | 45dB | 10dB |  | 132dB |
| [21] R1-2406189 | 20dB+ 61 dB@20m | No | No |  | 149dB |
| [36] R1-2407036 | 60dB+10dB | No RF-IC | 60dB |  | 120-130 dB |
| [36] R1-2407036 | 55dB@10m, 60dB@20m | No RF-IC | 60dB |  | 115-120dB |
| For D2T2 |  |  |  |  |  |
| [12] R1-2405855 |  |  |  |  | 140-150dB |
| [15] R1-2405970 | 30dB | 45dB | 10dB |  | 85dB |
| [21] R1-2406189 | 10 dB+65dB@20m | No | No |  | 151dB |
| [36] R1-2407036 | 54dB | No RF-IC | 60dB |  | 114dB |
| [36] R1-2407036 | 49dB@10m, 54dB@20m | No RF-IC | 60dB |  | 109-114dB |
| General |  |  |  |  |  |
| [14] R1-2405915 | 58dB@20m | Less(10-20dB)/no | 43-60dB |  | 120-138dB |
| [16] R1-2405992 | 60dB @10m | less/no RF-IC | Up to 80dB |  |  |
| [25] R1-2406375 | 40dB | 30dB | 50dB | 20dB(Frequency shifting) |  |
| [26] R1-2406408 | 50-60 dB@10-20 m | No RF-IC or 10-30 dB | Up to 80 dB |  | 130-150 dB |
| Note: Please refer to the source(contribution) for specific numerical analysis and calculation methods |  |  |  |  |  |

In addition to the above, contribution [11] observed that since the self-interference generated inside the hardware or in the reactive near field is not modelled in 9.4.1.1 link budget assumptions, if the self-interference cancellation capability discussed in 9.4.2.4 is used directly in 9.4.1.1, simulation results for scenario A2 would be overestimated by ignoring some inherent self-interference. Contribution [11] propose to study the impact of self-interference generated inside the hardware or in the reactive near field and think that 3% transmission power, namely transmission power – 15dB, can be a starting point. The value by deducting the impact from self-interference cancellation capability can be used in link budge calculation in 9.4.1.1.

FL1 High Priority proposal 2.1.3.2-1a: For CW interference suppression capability, capture Table 2.1.3.2-2 and Table 2.1.3.2-3 in the TR.

| Company | Y/N | Comments |
| --- | --- | --- |
| OPPO | Yes |  |
| TCL | Y | Agree with this proposal. |
| Qualcomm | Comment | It seems a large value range based on companies’ input. If to be captured in TR, need to emphasize that it is RAN1 observation and the feasibility needs to be further discussed in RAN4. |
| vivo | Y with comment | Fine to capture the tables, but it should be noted that these tables/values are used for evaluation purpose for AI 9411. |
| Huawei, HiSilicon | No | The SID does not task RAN1 to evaluate the amount of interference suppression achieved by whatever technique. Our task is to find the CW waveform characteristics. |
| Xiaomi | comment | For cross-link interference and self-interference in the D2T2, the Analog cancellation suppression is complicated for the UE, because it requires to generate a carrier-wave which has same amplitude value and the opposite phase value with the actual carrier-wave. So it is preferred to only apply the spatial domain and digital domain suppression on the UE. |
| LGE |  | Similar comment as Qualcomm/Huawei. Regarding the value/amount of this CW interference suppression, RAN1’s observation/task needs to be clarified. |
| Samsung |  | We are fine with the table. But does this table apply to all types of CW? Or do different candidates have different tables? Also, we are concerned that capturing these values in the TR might imply that they are feasible. Therefore, it would be better to clarify that these values are only assumptions for evaluation and do not include any discussion on feasibility, which needs RAN4’s study |
| Ericsson |  | In mono-static scenario A2, some self-interference is generated in the transmitter and leaked into the receiver inside the physical hardware, rather than through the air from Tx antenna to Rx antenna. This is called return loss in RFID, which is about 3% of transmission power. The return loss doesn’t exist in bi-static scenario A1. When the interference cancellation value is used in link budge calculation, the cancellation capability value for scenario A2 can’t be used directly as it is for scenario A1 and instead should be used with a penalty to compensate the return loss. As to the cancellation capability values, we think 80dB of digital domain cancellation is possible for D1T1, regardless of scenario A1 or A2. Therefore, the total cancellation capability for D1T1 scenario A2 is 110dB. The total capability for D1T1 scenario A1 is 80dB + penetration loss (subject to pathloss model and 3D CW2R distance). For D2T2, given the complexity and size restrictions of an intermediate UE, a value 10~20dB smaller than those of D1T1 can be considered. |

Relative complexity of CW generation [Open]

In last meeting, relative complexity of CW generation was discussed [6]. One observation was achieved.

Observation For relative complexity of CW generation - Compared to single-tone unmodulated sinusoid waveform without frequency hopping, two unmodulated single-tones waveform: - Leads to higher PAPR of the generated CW, which impacts the implementation of the power amplifier in the CW node. - Note: For two unmodulated single-tones waveform, assume the two tones are transmitted from the same CW node.

In this meeting, contributions [10], [12], [14], [15], [19], [25], [26], [32], [35] continue compared relative complexity of CW generation for single-tone unmodulated sinusoid waveform and two unmodulated single-tones.

Contributions [10], [14], [19], [25], [26], [36] mentioned that the complexity difference is marginal or very small between the components that generate one and two unmodulated sinusoid waveforms.

Contribution [25] and [32] observed that two unmodulated single-tones increase the signal PAPR by 3dB and the increased complexity of CW generation depends on the implementation.

Contribution [12] and [15] observed that two unmodulated single-tones leads to additional out-of-band interference suppression to deal with the intermodulation interference generated by the two unmodulated single-tones. [15] further propose to discuss the feasibility of spectrum mask for CW node to avoid interference in RAN 1 and RAN4.

Contribution [18] prefers to study in-band baseband generation of carrier wave reusing OFDM transmitter

Based on the above, before clarifying the complexity difference of CW generation is marginal, it may need to discuss whether the impacts of intermodulation interference issue are essential or not. For 3dB PAPR, if each tone of 2-tone CW is transmitted with half power of that of single-tone CW, it does not seem like this value anymore. Companies can share views for this in comments part. Therefore, the following observation is considered first.

FL1 high Priority observation 2.1.4-1a: For relative complexity of CW generation,

- Compared to single-tone unmodulated sinusoid waveform without frequency hopping, two unmodulated single-tones waveform:

  - Leads to additional out-of-band interference suppression complexity to deal with the intermodulation interference generated by the two unmodulated single-tones.

  - Note: For two unmodulated single-tones waveform, assume the two tones are transmitted from the same CW node.

| Company | Y/N | Comments |
| --- | --- | --- |
| OPPO | Yes |  |
| TCL | Y | In our understanding, 3rd intermodulation interference is the main impact for relative complexity of CW generation. We prefer consider this issue in in-band and guard-band configuration, because it will generate interference to coexistence systems or impact the setting of AIoT guard-band. Thus, we suggest adding some limited in this proposal: FL1 high Priority observation 2.1.4-1a: For relative complexity of CW generation, - Compared to single-tone unmodulated sinusoid waveform without frequency hopping, two unmodulated single-tones waveform: - Leads to additional out-of-band interference suppression complexity to deal with the intermodulation interference generated by the two unmodulated single-tones for in-band or guard-band configuration. Note: For two unmodulated single-tones waveform, assume the two tones are transmitted from the same CW node. |
| Qualcomm | Comment | Whether to have out-of-band interference needs more discussion. For example, if freq shift of 3rd, 5th, ...is not used, it may not be a big issue to consider. |
| vivo | N | As our comment for Observation 2.1.2.1-1a, for impact of intermodulation interference, more justification is needed. |
| Huawei, HiSilicon | Yes | Again, the note should just be in one place, and may not be relevant to this observation. |
| Xiaomi | N | We think if the suitable frequency gap between the two tones is applied, the intermodulation interference issue can be solved. |
| Samsung | N | We think that out-of-band emission highly depends on the gap between the two tones. |
| Ericsson |  | We suggest split the observation into the presence of IM product for two-tone CW and whether it will incur additional suppression complexity. Whether IM product will necessarily require higher complexity of interference suppression depend on many factors. In the following aspects, a two-tone CW doesn’t require additional out-of-band interference suppression complexity compared with single-tone CW. - A two tone signal has a lower PAPR than an OFDM signal. Meaning that, a node that is capable of generating OFDM signals can handle generating a two-tone signal without creating much OOB interference in a simpler way than the OFDM signals case. Proper selection of PAs and filters at the CW node can limit the effect of OOB intermodulation products. - Taking into account that in a two-tone signal, each tone has half of the power as that of a single tone signal, the complexity difference is marginal or very small between the components that generate one and two unmodulated sinusoid waveforms. |

Gap between tones for two-tone CW waveform [Open]

Gap between tones for two-tone CW waveform was discussed, the latest FL proposal was as follows[6]

FL5 High Priority Proposal 2.2.2-1c On the gap size between the two unmodulated single-tones, at least the following aspects need to be jointly considered for further study: - Coherence bandwidth / delay spread - D2R transmission bandwidth from the backscattering of one or multiple devices - [D2R system bandwidth] - Frequency shift(s) for one or more devices - Harmonics impacts - [RF filter bandwidth of the device]

In this meeting, contributions [11], [12], [14], [15], [19], [22], [23], [25], [29], [31], [32], [34], [38] discussed or mentioned the gap between tones. The views are as follows:

- Views for gap determination factors

  - Larger than the coherence bandwidth (related to delay spread, to achieve frequency diversity gain): [11] [12][14] [15] [22] ][25] [31] [32] [38] [21]

  - D2R transmission bandwidth from the backscattering of one or multiple devices [12] [14] [15] [22] [31] [32] [25] [21]

  - Smaller than system BW or BW allocated for AIoT. [21][25]

  - Harmonics impacts [12][15]

  - Frequency shift(s) for one or more devices [22] [29] [21] [25]

  - D2R data rate (i.e., BW) and/or device type [29]

- Proposed values for the gap

  - 1.6 MHz [34]

  - 3MHz [23]

  - 5MHz [19][23]

Based on the above, it seems the gap values are quite divergent. However, most companies observed some factors for gap determination, e.g., the gap between tones should be larger than coherence bandwidth, which is related to the delay spread. Therefore, the following proposal can be considered.

FL1 High Priority observation 2.2-1a: The gap size of two unmodulated single-tones at least related to the following aspects:

- Coherence bandwidth/delay spread

- D2R transmission bandwidth from the backscattering of one or multiple devices

- BW allocated for A-IoT

- Harmonics impacts

- Frequency shift(s) for one or more devices

- D2R data rate and/or device type

| Company | Y/N | Comments |
| --- | --- | --- |
| OPPO | Yes with comments | It seems that the D2R transmission BW is related to the D2R data rate, it should be merged into one bullet. |
| TCL | Y | Except for proposed aspects, we think interfermodulaton components will impact the gap size of two unmodulated single-tones. For example, as shown figure blow, the 3rd intermodulation frequency of 2w1-w2 and 2w2-w1 is related with w1 and w2. Thus, in our understanding, we suggest that adding one subbullet to explain the impact of intermodulation component. FL1 High Priority observation 2.2-1a: The gap size of two unmodulated single-tones at least related to the following aspects: - Coherence bandwidth/delay spread - D2R transmission bandwidth from the backscattering of one or multiple devices - BW allocated for A-IoT - Harmonics impacts - Intermodulation components impacts - Frequency shift(s) for one or more devices - D2R data rate and/or device type |
| Qualcomm |  | Generally ok, but need to clarify the BW allocated for A-IoT is R2D or D2R or both? |
| Huawei, HiSilicon |  | We agree that the intermodulation aspect needs to be added to the list of aspects to be considered when determining the gap between the two single-tones. |
| Xiaomi |  | For the last sub bullet, for the harmonized design, the gap size shall be the same for the different device types, so we make the following revision with the blue part: FL1 High Priority observation 2.2-1a: The gap size of two unmodulated single-tones at least related to the following aspects: - Coherence bandwidth/delay spread - D2R transmission bandwidth from the backscattering of one or multiple devices - BW allocated for A-IoT - Harmonics impacts - Frequency shift(s) for one or more devices - D2R data rate and/or device type |
| LGE | Yes with comment | Similar comment as OPPO/QC. The A-IoT BW seems necessary to be clarified as D2R BW, and it could be merged with D2R data rate. |
| CEWiT |  | Fine with the observation |
| Ericsson |  | Agree in principle. The intention of the gap between two tones may be to leverage frequency diversity gain, where the following aspects should be taken into account. Maybe we can clarify it in the main sentence. |
| FL | Based on the feedback, the following version is considered for Tuesday offline discussion. FL1 High Priority observation 2.2-1b: For the gap between two tones to leverage frequency diversity gain, the following aspects should be taken into account: - Coherence bandwidth/delay spread - D2R transmission bandwidth from the backscattering of one or multiple devices - BW allocated for A-IoT - Harmonics and/or intermodulation impacts - Frequency shift(s) for one or more devices |  |

Variants for waveforms

Single-tone waveform with frequency hopping (FH) [Open]

Contributions [10], [11], [12], [15], [16], [18], [19], [21], [27], [28], [31], [32], [35] discussed single-tone waveform with frequency hopping. The views are summarized below:

- General support of single-tone with FH

  - Contributions [15][16][18] prefer to consider frequency hopping for single-tone unmodulated sinusoid waveform to improve frequency diversity gain and avoid channel fading.

- Characteristics analyzation for single-tone with FH

  - Contributions [16][26][33][35][36] analyzed characteristics (i.e. characteristics discussed in section 2.1) for single-tone waveform with frequency hopping.

- How to achieve gain for single-tone with FH

  - Contributions [11], [16], [21], [35] discussed or mentioned that to exploit the benefits of frequency diversity, repetition on bit-level or packet-level is required. Contribution [11] observed that single-tone CW with 2 frequency hops can achieves similar detection performance as 2-tone CW. Contribution [35] further identified that by using lower coding rate with interleaving, single-tone with FH can achieve frequency diversity gain. Contribution [19] indicated that the minimum gap between two hops should be 5.3MHz

- Hopping pattern for single-tone with FH

  - Contributions [10] [11] [32] propose to further study the hopping periodicity, pattern, whether/how frequency hopping pattern and/or frequency location of each hopped single-tone is recognized by the reader receiving the corresponding D2R transmission in the case of D1T1–A1/B and D2T2–A1/B.

- Not support the study of single-tone with FH

  - Contribution [12] pointed out that the unknown timing offset of each D2R transmission makes it hard to align the boundary of each hop of the external carrier-wave with the corresponding segment of the D2R transmission. For the RF interference cancellation at the D2R receiver, the frequency hopping of external carrier-wave requires the re-detection of the parameters of the received carrier-wave interference. Contribution [12] proposed that the unmodulated single-tone waveform with frequency hopping is not studied in Rel-19.

  - Contribution [28] prefer that single-tone unmodulated sinusoid waveform with frequency hopping is not supported at the first release, and [28] pointed out that if interleaver after channel coding is not supported for PDRCH transmission, the diversity gain due to frequency hopping may be much less than two unmodulated single-tones. Contribution [31] thinks that unless a significant gain in reception performance is confirmed compared to the complexity of the terminal and the entire network for frequency hopping management, CW waveform based on frequency hopping of single tone would be studied as a low priority

Based on the above, it seems the majority companies prefer to study single-tone waveform with FH, and some of them provided detailed analysis for single-tone waveform with FH. As there is no agreement to study single-tone waveform with FH before, and this might be the last meeting for the CW part. FL would like to check whether there is consensus on capturing the observations and/or comparisons of single-tone without FH and single-tone with FH in the TR first, if so, we can try to discuss the aspects what we can capture in the TR.

Therefore, the following proposal is considered.

FL1 High Priority Proposal 2.3.1-1a: Observations and/or comparisons of single-tone unmodulated sinusoid waveform without frequency hopping and single-tone unmodulated sinusoid waveform with frequency hopping can be captured in the TR.

| Company | Y/N | Comments |
| --- | --- | --- |
| OPPO | Yes |  |
| TCL | Y | Fine. |
| Qualcomm | Yes and comment | In addition, we think two-tone with freq hopping combined with 2 tx antenna transmission should also be captured. Based on our simulation [36], two-tone with FH on 2-tx ant can achieve 1 or 2dB better than two-tone without FH and 8 or 12dB gain@BLER=1% than single-tone without FH with 2.14MHz or 10MHz freq gap in TDL-A 30ns DS. |
| vivo |  | Fine to have observations, 3rd and 5th bullet summarized by FL in this section can be starting point. |
| Huawei, HiSilicon | No | We have concerns in including the option with frequency hopping. There is the issue of a guard band being required by the D2R transmissions between each of these frequency hops, resulting in the repetitions being transmitted in each of the backscatter D2R to be discontinuous, which also results in a latency increase and overhead depending on how frequently the guard bands are required. Another issue is that the D2R receiver has to complete the detection of the single tone CW in the first frequency before hopping to the second frequency, without which RF interference cancellation cannot work appropriately. These aspects require excessive study for the return on the time it would take. RAN1 identified already sufficient waveforms. |
| Xiaomi |  | We prefer that single-tone unmodulated sinusoid waveform with frequency hopping is not studied in R19 AIOT, because it is difficult to accurately align the hopping timing between the CW node and AIOT device if CW node is responsible for the frequency hopping. |
| LGE |  | Similar view as Huawei/Xiaomi. The frequency hopping requiring the data repetition or low code rate, would increase time domain overhead and also require longer device’s on-time for D2R/R2D reception/transmission. |
| Ericsson | Y | We are fine with capturing the comparisons/observations of single tone with FH in the TR. |

Two-tone waveforms from different node. [Open]

Contributions [11], [21], [33], [35], [36] shared views for multi-tone waveforms from different nodes. The views are summarized below:

- Propose to study

  - Contributions [33][35][36] prefer to study multi-tone waveforms from different node. These contributions analyzed characteristics (i.e. characteristics discussed in section 2.1) for multi-tone waveforms from different nodes, and observed several benefits, e.g., two tones can be transmitted from different nodes with no power splitting,  two-tone from different CW nodes have no additional complexity of CW generation compared to single tone.

- Negative views

  - Contribution [11] shared negative views as they observed that regarding two tones of a carrier wave transmitted separately from different CW nodes, a reader is required to distinguish between devices which backscatter the two single-tone CW from two CW nodes and those which backscatter the single-tone CW from one CW node. Control and synchronization of multiple CW nodes simultaneously would be required.

  - Contribution [21] thinks that two single tones CW from different nodes is a deployment choice, and no additional spec impact is expected. Contribution [21] propose to deprioritize the cases two tones are transmitted from the different CW nodes for two unmodulated single-tones waveform.

In the last RAN1 meeting, few companies shared negative views in their feedback. Considering that this issue has not been widely discussed (according to the contributions this meeting and the feedback last meeting), FL would like to collect more views for this case: Do multi-tone waveforms from different nodes need to be studied, and whether the relevant observations need to be captured in the TR?

FL1 Medium priority question 2.3.2-1a: Whether two-tone waveforms from different node is an essential case and needs to be studied in R19 SI?

| Company | Y/N | Comments |
| --- | --- | --- |
| OPPO |  | We think it can be deprioritized in R19 SI. |
| Huawei, Hisilicon | No |  |
| Xiaomi | N | The system interference of AIOT will largely increase if introducing two CW node. |
| LGE | N |  |
| Apple | Yes | The discussion related to the two factors: - D2R transmission bandwidth from the backscattering of one or multiple devices - BW allocated for A-IoT Compared to two un-modulated tones from the same CW nodes, the two un-modulated tones from two CW nodes enjoy: - Larger transmission power (3dB higher) - Larger spatial diversity (do not require large BW separation larger than coherent BW). - Simple CW generation (low PAPR) - The inference cancellation is similar to two tones from the same CW nodes - Due to spatial-seperation, D2R transmission can be FDMed based on proximity of different CW nodes Compared to single un-modulated tones without frequency hopping, the two un-modulated tones from two CW nodes enjoy: - Larger transmission power (3dB higher) - Larger spatial diversity (do not require large BW separation larger than coherent BW). - Larger frequency diversity - Due to spatial-separation, D2R transmission can be FDMed based on proximity of different CW nodes When single tone CW per CW node is used, unless 3GPP specification specifies only one channel/carrier for D2R transmission, the multi-tone transmission with multiple CW nodes is formed naturally as AIoT backscattering operation cannot filter out one CW and only modulate the other CW. |
| Ericsson |  | We have provided our views toward the issue in our contribution. We are open to study it. |

CW transmission (without large frequency shift)

For the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering, Six CW transmission cases were agreed in the last meeting RAN1#116.

Agreement For the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering, and for topology 1, the following cases for CW transmission are studied. - Case 1-1: CW is transmitted from inside the topology, transmitted in DL spectrum - Case 1-2: CW is transmitted from inside the topology, transmitted in UL spectrum - Case 1-4: CW is transmitted from outside the topology, transmitted in UL spectrum Agreement For the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering, and for topology 2, the following cases for CW transmission are studied. - Case 2-2: CW is transmitted from inside the topology (i.e., intermediate UE), transmitted in UL spectrum - Case 2-3: CW is transmitted from outside the topology, transmitted in DL spectrum - Case 2-4: CW is transmitted from outside the topology, transmitted in UL spectrum

The agreed cases in RAN1#116 can be illustrated by the following figure.

| Case 1-1 | Case 1-2 | Case 1-4 |  |
| --- | --- | --- | --- |

Figure 3.1-1 CW transmission for topo 1

| Case 2-2 | Case 2-3 | Case 2-4 |  |
| --- | --- | --- | --- |

Figure 3.1-2 CW transmission for topo 2

Contributions [9], [11], [12], [14], [15], [16], [17], [18], [19], [21], [22], [23], [25], [26], [27], [31], [33], [34], [35], [36], [38], [39], [40] continue shared views for those CW transmission cases.

Contributions [21], [23], [26], [40] shared views for CW interference types at D2R receiver for CW transmission cases/scenarios, some of the views are summarized below. Contribution [23] further indicated for scenarios “B” and ‘A1’, the CW interference type at D2R receiver is cross-link interference, and cross-link interference indicates the interference between the CW and the D2R/R2D signal. For scenario “A2”, the CW interference type at D2R receiver is self-interference, and self-interference interference indicates the interference between the CW and the D2R signal.

According to the online discussion in last meeting, the CW interference type at D2R receiver is obvious this observation is not helpful that much, it is not necessary to have independent observation. The observation for CW interference type is combined in the views for CW transmission cases.

CW transmission cases for topo 1 [Open]

Views from contributions for the topology 1 CW cases are summarized below.

Table 3.1-1 views for CW transmission cases for topology 1

| CW Transmission case | Advantages | Disadvantages |
| --- | --- | --- |
| Case 1-1: inside/DL | - CW transmission by BS in DL spectrum is in-line with the existing spectrum usage.[12][16] [31][36] - NO needs for gNB to support full-duplex capability in DL(for A1) [21] [35][40][36] - Cross-link interference handling(for A1) [21] [26] [40] - Higher CW power[33][36] - Low power D2R backscattered transmissions would not interfere with other R2D transmissions in the DL spectrum.[12] - Beam nulling gain may be possible between BS/reader tx and rx to increase spatial separation for A1[36] | - FDD gNB needs to support full-duplex capability (for A2) [11][12][14][16][21][23][31][33][35][40][36][27] [34] - Ambient IoT BS requires self-interference suppression capability(for A2) [11][16][21][33][38] [26] [27] [32] [35] [40] - Additional hardware is required at gNB to receive D2R transmissions in DL spectrum. [11][12][14] [16][21][35][40][36] - D2R transmission in DL spectrum is not in line with current spectrum usage [17][22][34][31][33] [35] [38] - Spatial isolation is only possible for A1. [12] - Additional NR interference [33] - Separate BS/reader for tx/CW and BS/reader for rx require coordination, Estimation of CW interference outside the D2R receiver may be needed for A1 [36] - dense deployment of gNBs[11] |
| Case 1-2: inside/UL | - No changes required at BS to receive D2R transmissions in UL spectrum. [12][16][21][31][33] - NO needs for gNB to support full-duplex capability in UL(for A1) [21][35][40][36] - Cross-link interference handling(for A1) [21] [26] [40] - No regulation limitation for gNB to transmit CW in UL band.[26] - D2R reception in UL spectrum is in-line with the existing spectrum usage [36] - Beam nulling gain may be possible between BS/reader tx and rx to increase spatial separation for A1[36] | - FDD gNB needs to support full-duplex capability(for A2) [11][12][14][15][17][21][31] [33][35][40][36] [27] - Additional hardware is required at gNB to transmit CW in UL spectrum. [11][12] [14][16][21][33][34] [35][40][36] - CW transmission by BS in UL spectrum is not in-line with the existing spectrum usage. [12][14][15][22] [31][33][34][35][38][36] [23] - Ambient IoT BS requires self-interference suppression capability [16][21][33] [26] [27] [32] [35] [40] - Dense deployment of gNBs, less cost-efficient [11][33] - Spatial isolation is only possible for ‘A1’. [12] - Separate BS/reader for tx/CW and BS/reader for rx require coordination, Estimation of CW interference outside the D2R receiver may be needed for A1 [36] - lower CW power may be permitted in the UL spectrum than that of DL.[36] |
| Case 1-4: outside/UL | - D2R transmissions by device in UL spectrum is in-line with existing spectrum regulations. [11][12][14][22][31][33][34] [35][36] [38] - Full duplex capability is not required[14] [21] [31] [35] [40] [36] - Cross-link interference handling [16] [21] [26] [27] [35] [40] - No changes required at BS to receive D2R transmissions in UL spectrum. [12][14][16] [33][35] - Spatial isolation is possible, reducing the received interference power. [12] [33] | - Separate CW and BS/reader for tx/rx require coordination. [36][38] - Estimation of CW interference outside the D2R receiver may be needed. [36][33] - Ambient IoT BS requires to handle co-channel interference from CW when spatial isolation between Ambient IoT BS and CW node is not enough [16] - lower CW power may be permitted in the UL spectrum than that of DL. [36] |

In the above table, the views for regulation are divergent, e.g., contributions [25] [26] point out that no regulation limitation for gNB to transmit CW in UL band. While some other contributions understand that CW transmission by BS in UL spectrum is not allowed as per existing spectrum regulations. Contribution [26] thinks that the regulation violation issue should be considered for case 1-1, case 1-2, and case 2-3. Contribution [18] proposes to study whether reduced gNB transmit power (e.g., maximum 23/26 dBm as UE) could satisfy the regulation, while contribution [36] prefers to FFS whether to capture the aspects on CW/D2R transmission satisfying the spectrum regulation rules. FL’s suggestion is that, RAN1 can identify whether the spectrum used for CW/D2R transmission is in-line with the usage of current spectrum, and RAN1 does not discuss whether the case is allowed by the regulation or not.

Contribution [19] discussed D2R reception performance, spectrum utilization and other possible issues for case 1-1/1-2/1-4. FL understand that these characteristics are for different CW waveforms, not need to compare for different CW cases.

Contribution [38] thinks that for support a case 1-1, the device needs a large frequency shifter. FL understand that this is related to 9.4.1.2.

Based on the above, the following observation can be considered:

FL1 High Priority proposal 3.1-1a: For the cases that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering, and for topology 1, capture the following table in the TR.

| CW Transmission case | Advantages | Disadvantages |
| --- | --- | --- |
| Case 1-1: inside/DL | - CW transmission by BS in DL spectrum is in-line with the existing spectrum usage.(4 sources) - NO needs for BS to support full-duplex capability in DL(for A1) (4 sources) - Cross-link interference handling(for A1) (3 sources) - Higher CW power (2 sources) - Low power D2R backscattered transmissions would not interfere with other R2D transmissions in the DL spectrum. (1 source) - Beam nulling gain may be possible between BS/reader tx and rx to increase spatial separation for A1 (1 source) | - FDD BS needs to support full-duplex capability (for A2) (13 sources) - BS requires self-interference suppression capability(for A2) (10 sources) - Additional hardware is required at BS to receive D2R transmissions in DL spectrum. (8 sources) - D2R transmission in DL spectrum is not in line with current spectrum usage (7 sources) - Spatial isolation is only possible for A1. (1 source) - Additional NR interference (1 source) - Separate BS/reader for tx/CW and BS/reader for rx require coordination, Estimation of CW interference outside the D2R receiver may be needed for A1 (1 source) - dense deployment of BSs (1 source) |
| Case 1-2: inside/UL | - No changes required at BS to receive D2R transmissions in UL spectrum. (5 sources) - NO needs for BS to support full-duplex capability in UL(for A1) (4 sources) - Cross-link interference handling(for A1) (3 sources) - No regulation limitation for BS to transmit CW in UL band. (1 source) - D2R reception in UL spectrum is in-line with the existing spectrum usage (1 source) - Beam nulling gain may be possible between BS/reader tx and rx to increase spatial separation for A1(1 source) | - FDD BS needs to support full-duplex capability(for A2) (12 sources) - Additional hardware is required at BS to transmit CW in UL spectrum. (10 sources) - CW transmission by BS in UL spectrum is not in-line with the existing spectrum usage. (11 sources) - BS requires self-interference suppression capability (8 sources) - Dense deployment of BSs, less cost-efficient (2 sources) - Spatial isolation is only possible for ‘A1’. (1 source) - Separate BS/reader for tx/CW and BS/reader for rx require coordination, Estimation of CW interference outside the D2R receiver may be needed for A1 (1 source) - Lower CW power may be permitted in the UL spectrum than that of DL. (1 source) |
| Case 1-4: outside/UL | - D2R transmissions by device in UL spectrum is in-line with existing spectrum usage. (10 sources) - Full duplex capability is not required (6 sources) - Cross-link interference handling at D2R receiver side (6 sources) - No changes required at BS to receive D2R transmissions in UL spectrum. (5sources) - Spatial isolation is possible, reducing the received interference power. (2 sources) | - Separate CW and BS/reader for tx/rx require coordination. (2 sources) - Estimation of CW interference outside the D2R receiver may be needed. (2 sources) - BS requires to handle co-channel interference from CW when spatial isolation between Ambient IoT BS and CW node is not enough (1 source) - Lower CW power may be permitted in the UL spectrum than that of DL. (1 source) |

| Company | Y/N | Comments |
| --- | --- | --- |
| OPPO | Comments | - For case 1-1, we propose to remove “Low power D2R backscattered transmissions would not interfere with other R2D transmissions in the DL spectrum. (1 source)” by considering the near-far effect and no frequency selectivity at R2D receiver. - For case 1-2, we disagree with “No regulation limitation for BS to transmit CW in UL band. (1 source)”. In our view, the transmission of BS on UL spectrum will lead to TDD operation on FDD band, which violates the spectrum regulation. - For case 1-4, we propose to remove “BS requires to handle co-channel interference from CW when spatial isolation between Ambient IoT BS and CW node is not enough (1 source)” because it also exists in case 1-1 and 1-2 as well. |
| TCL | Y | Okay |
| Qualcomm |  | Several concerns: - ‘Spatial isolation is only possible for A1. (1 source)’ and ‘ BS requires self-interference suppression capability (8 sources)’ have duplicated meaning as full duplex? - “No changes required at BS to receive D2R transmissions in UL spectrum. (5sources)” is not accurate. D2R reception by reader is anyway new function for BS or UE. - “No regulation limitation for BS to transmit CW in UL band. (1 source)” is not clear yet. The CW transmission restriction is not clear yet, which may need further discussion in RAN4. - Also, any potential impact related with NR coexistence may need to wait for RAN4 discussion. |
| vivo |  | We have following comments - Not clear the meaning of ‘Additional NR interference, if there is co-existence issue, we can let RAN4 to make the observation.’, it seems co-existence issue, and suggest to left RAN4 to make these observations. - For ‘Low power D2R backscattered transmissions would not interfere with other R2D transmissions in the DL spectrum. ’, it seems an intra- AIoT co-existence issue, but we are not sure the issue of D2R interfering R2D is valid at the first place. Besides, even if this is true, it is not considered as a disadvantage, but seems no need to be captured as advantage. - For the advantages, the last bullet (beam nulling for A1) for Case 1-1: inside/DL and Case 1-2: inside/UL, can be replace by the same description for case 1-4 outside/UL, ‘Spatial isolation is possible, reducing the received interference power.’, which seems more inclusive. Besides, it seems there are some duplicate bullets, for example - full duplex capability and self-interference cancellation for disadvantages seem talking about the same thing, prefer to merge. |
| Huawei, HiSilicon | Comments | For the heading, we prefer to call it “Pros” and “Cons” since advantages/disadvantages would be relative. For case 1-1 - It would be incorrect to say that the CW transmission by the BS in the DL spectrum is in-line with existing spectrum usage since we do not have any regulations for spectrum usage for A-IoT. Similar comment for the D2R transmission in the DL spectrum. - It is not clear what is meant by cross-link interference handling and why it is an advantage. - Regarding the additional hardware that the BS requires to receive in the DL spectrum, having such an implementation at the BS would not be too difficult because it would require only a few filters to be changed from the RF chain for UL. Most of the important components can support both DL and UL spectrum. Hence, we do not consider it a disadvantage. - It is not clear on what exactly is meant by coordination being required between the CW node and the reader receiving the D2R transmission. Any such coordination is surely beyond the scope of the SID since it would involve CW control mechanisms. Estimation of CW interference outside the D2R receiver is also an aspect that should not be discussed here in RAN1, and should not be listed as a disadvantage. For case 1-2 - Similar comment as case 1-1 regarding the cross-link interference handling. - Similar comment as case 1-1 for the UL spectrum not being in-line with existing spectrum usage. - We do not agree with the advantage of “No regulation limitation for BS to transmit CW in UL band” since there are no A-IoT spectrum regulations yet, and moreover, current regulations do not permit the BS to transmit in the UL band. - For the low CW power permitted in the UL spectrum, since this is an external CW node, outside 3GPP, we should not make presumptions on its regulation in RAN1. For case 1-4 - Similar comment as case 1-1 for the usage of the UL spectrum. - Similar comment as case 1-1 for the cross-link interference. - We do not understand the motivation for the disadvantage on “BS requires to handle co-channel interference from CW when spatial isolation between Ambient IoT BS and CW node is not enough”. This would be based entirely on the deployment and is not a disadvantage for this particular case. - For the low CW power permitted in the UL spectrum, since this is an external CW node, outside 3GPP, we should not make presumptions on its regulation in RAN1. |
| Ericsson |  | Regarding advantages of Case 1-1, we have a question abou the following one. In our view, R2D spectrum is not considered in all the cases. The reason that D2R would not interfere R2D is because they are TDMed. • Low power D2R backscattered transmissions would not interfere with other R2D transmissions in the DL spectrum. (1 source) Regarding disadvantages of Case 1-1, aren't the first two of the following common to every case? Ix • Spatial isolation is only possible for A1. (1 source) • Additional NR interference (1 source) The following one appears in the disadvantages of case 1-1 and case 1-2. We can add scenario A1 and A2 for clarification. • dense deployment of BSs (1 source) Regarding ‘Lower CW power may be permitted in the UL spectrum than that of DL. (1 source)’, shall we change ‘may be permitted’ to ‘is assumed’? |

CW transmission cases for topo 2 [Open]

Views from contributions for the topology 2 cases are summarized below.

Table 3.2-1 views for CW transmission cases for topology 2

| CW Transmission case | Advantages | Disadvantages |
| --- | --- | --- |
| Case 2-2: inside/UL | - CW transmission by intermediate UE in UL spectrum is in-line with the existing spectrum usage.[12][14][16] [31] [33] [35] [36] - Cross-link interference handling(for A1) [21] [26] [35] [40] - NO needs for UE to support and operate in full-duplex mode (for A1) [21] [35] [40] [36] [33] - D2R transmissions by device in UL spectrum is in-line with the existing spectrum usage.[12] | - Intermediate UE needs to support full duplex(for A2) [11][14][26] [21] [22] [23] [31] [33] [35] [40] [36] [27] [34] - Intermediate UE requires self-interference suppression capability(for A2) [16][21][38] [23] [26] [27] [32] [33] [35] [40] - Additional hardware is required at UE to receive D2R in UL spectrum [11][14][16] [22][21] [33] [35] [38] [40] [32] - Spatial isolation is only possible for ‘A1’. [12] - Additional TDM coordination of NR UL transmission, D2R receiving and R2D [33] - lower CW power may be permitted in the UL spectrum than that of DL. [36] - Separate UE/reader for tx/CW and UE/reader for rx require coordination. Estimation of CW interference outside the D2R receiver may be needed for A1. [36] |
| Case 2-3: outside/DL | - UE does not need to support higher capability, e.g., reception in DL, full duplex capability [14][16] [21] [23] [31] [33] [35] [40] [36] - Cross-link interference handling[16] [21] [23] [26] [35] [40] - Spatial isolation is possible, reducing the received interference power.[12] - higher CW power may be permitted in the DL spectrum than that of UL. [36 | - D2R transmission in DL spectrum is not in line with current spectrum usage [14][15] [36] [22] [31] [33] [35] - Separate CW and UE/reader for tx/rx require coordination. [36] [38] - With high transmission power, can cause interference to neighbor gNBs.[27][33] - Increased complexity of the intermediate UE since it would be required to support reception on the DL spectrum and UL spectrum, to support the CW being transmitted from inside the topology.[12] - Additional TDM coordination of NR DL transmission, CW transmission/D2R receiving and R2D transmission is needed [33] - Estimation of CW interference outside the D2R receiver may be needed. [36] |
| Case 2-4: outside/UL | - D2R in UL spectrum is in-line with the existing spectrum usage. [12] [14] [22] [31] [33] [35] [36] - Full-duplex capability is not needed. [21][36] [23][31] [40][35] - Cross-link interference handling[21] [23] [26] [27] [35] [40] [16] [23] - Do not require additional hardware for CW transmission when UE is used as CW node. [33] - Spatial isolation is possible, reducing the received interference power. [12] | - Additional hardware is required at UE to receive D2R in UL spectrum. [11][14] [16][21] [31] [33] [35] [40] [32] - Separate CW and UE/reader for tx/rx require coordination[36] [34] [38] - full duplex capability may be needed if an intermediate UE simultaneously receives D2R in UL band and transmits to gNB in the same UL band. [11] - Additional TDM coordination of NR UL transmission, CW transmission/D2R receiving and R2D transmission is needed. [33] - lower CW power may be permitted in the UL spectrum than that of DL. [36] - Estimation of CW interference outside the D2R receiver may be needed. [36] |

In addition, contribution [17] proposes to FFS on the support of Case 2-2 (CW is transmitted from inside the topology in the UL spectrum), taking into account the inter-UE coordination latency in Scenario D2T2-A1 and UE implementation complexity in Scenario D2T2-A2. Contribution [33] thinks that for topology 2, to enable efficient inference mitigation and cancellation, procedure for TDM schedule intermediate UE’s Uu link transmission and CW/R2D transmission and D2R receiving should be studied. Contribution [38] prefer to define UE behavior for receiving D2R transmission in UL spectrum. FL thinks that these issues can be considered in CW node control or coexistence part.

Therefore, the following proposal is considered.

FL1 High Priority proposal 3.2-1a: For the cases that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering, and for topology 2, capture the following table in the TR.

| CW Transmission case | Advantages | Disadvantages |
| --- | --- | --- |
| Case 2-2: inside/UL | - CW transmission by intermediate UE in UL spectrum is in-line with the existing spectrum usage.(7 sources) - Cross-link interference handling at D2R receiver side(for A1) (4 sources) - NO needs for UE to support and operate in full-duplex mode (for A1) (5 sources) - D2R transmissions by device in UL spectrum is in-line with the existing spectrum usage. (1 source) | - Intermediate UE needs to support full duplex(for A2) (13 sources) - Intermediate UE requires self-interference suppression capability(for A2) (10 sources) - Additional hardware is required at UE to receive D2R in UL spectrum (10 sources) - Spatial isolation is only possible for ‘A1’. (1 source) - Additional TDM coordination of NR UL transmission, D2R receiving and R2D (1 source) - Lower CW power may be permitted in the UL spectrum than that of DL. (1 source) - Separate UE/reader for tx/CW and UE/reader for rx require coordination. Estimation of CW interference outside the D2R receiver may be needed for A1. (1 source) |
| Case 2-3: outside/DL | - UE does not need to support higher capability, e.g., reception in DL, full duplex capability (9 sources) - Cross-link interference handling at D2R receiver side (6 sources) - Spatial isolation is possible, reducing the received interference power. (1 source) - higher CW power may be permitted in the DL spectrum than that of UL. (1 source) | - D2R transmission in DL spectrum is not in line with current spectrum usage (7 sources) - Separate CW and UE/reader for tx/rx require coordination. (2 sources) - With high transmission power, can cause interference to neighbor BSs. (2 sources) - Increased complexity of the intermediate UE since it would be required to support reception on the DL spectrum and UL spectrum, to support the CW being transmitted from inside the topology. (1 sources) - Additional TDM coordination of NR DL transmission, CW transmission/D2R receiving and R2D transmission is needed (1 sources) - Estimation of CW interference outside the D2R receiver may be needed. (1 sources) |
| Case 2-4: outside/UL | - D2R in UL spectrum is in-line with the existing spectrum usage. (7 sources) - Full-duplex capability is not needed. (6 sources) - Cross-link interference handling at D2R receiver side (8 sources) - Do not require additional hardware for CW transmission when UE is used as CW node. (1 source) - Spatial isolation is possible, reducing the received interference power. (1 source) | - Additional hardware is required at UE to receive D2R in UL spectrum. (9 sources) - Separate CW and UE/reader for tx/rx require coordination (3 sources) - Full duplex capability may be needed if an intermediate UE simultaneously receives D2R in UL band and transmits to BS in the same UL band. (1 source) - Additional TDM coordination of NR UL transmission, CW transmission/D2R receiving and R2D transmission is needed. (1 source) - Lower CW power may be permitted in the UL spectrum than that of DL. (1 source) - Estimation of CW interference outside the D2R receiver may be needed. (1 source) |

| Company | Y/N | Comments |
| --- | --- | --- |
| OPPO | Comments | For case 2-4, “Full duplex capability may be needed if an intermediate UE simultaneously receives D2R in UL band and transmits to BS in the same UL band. (1 source)” and “Additional TDM coordination of NR UL transmission, CW transmission/D2R receiving and R2D transmission is needed. (1 source)” should be merged. They are talking about the same issue in our view. |
| TCL | Y | Okay |
| Qualcomm |  | Several concerns: - ‘Intermediate UE requires self-interference suppression capability(for A2) (10 sources) and ‘Spatial isolation is only possible for ‘A1’. (1 source)’ have duplicated meaning as full duplex? - “Additional hardware is required at UE to receive D2R in UL spectrum ” may not be only for UL spectrum. D2R reception by reader is anyway new function for BS or UE. - “Do not require additional hardware for CW transmission when UE is used as CW node. (1 source)” may not only for 2-4. The CW transmission is anyway new function for BS or UE. - Also, any potential impact related with NR coexistence may need to wait for RAN4 discussion. It’s too early to capture ‘ TDM coordination’. |
| vivo |  | - The disadvantage of ‘Additional TDM coordination of NR DL transmission, CW transmission/D2R receiving and R2D transmission is needed’, is provided for all three cases, hence not necessary for comparison between cases. - For disadvantages for Case 2-2: inside/UL, ‘Intermediate UE needs to support full duplex’, suggest to revise to ‘Intermediate UE needs to support full duplex in UL’, since FDD UE is by default full duplex between DL and UL spectrum. Besides, some bullets seems duplicated as we comment for CW cases for D1T1. |
| Huawei, HiSilicon | Comments | For the heading, similar to the previous proposal, we prefer to call it “Pros” and “Cons”. For case 2-2 - It is not clear what is meant by cross-link interference handling at the receiver and why it is an advantage. - As mentioned previously, we do not have any regulations for spectrum usage for A-IoT devices, and hence saying it is in-line with existing spectrum usage is incorrect. - For the additional hardware requirement to receive in UL by the iUE, we do not think this is a serious disadvantage since it has already been implemented in RFID readers. - We do not agree with the TDM coordination between NR UL and D2R receiving and R2D. This is a RAN4 issue and cannot be regarded as a disadvantage. - Same comment as in case 1-1 for coordination being required between the CW node and the reader receiving the D2R transmission. For case 2-3 - Similar comment as case 2-2 for cross-link interference. - Similar comment as case 2-2 for the DL spectrum not being in-line with current spectrum usage, since no such regulations have been determined for A-IoT yet. For case 2-4 - Similar comment as case 2-2 for cross link interference. - Similar comment as case 2-2 for the UL spectrum. - For the full duplex capability, we do not expect the D2R UL and NR UL in the same band. Moreover, since such a scenario does not exist currently, it should not be considered as a disadvantage. - Similar to the comment in 1-4, the low CW power is also not a limiting factor since this is an external CW node that would not have such low power CW TX restrictions. Moreover, it would be speculative to state this since we do not have any regulations in place for the CW TX power. |
| Xiaomi | comment | For the case 2-3, except the advantages in the table, there does not exist the self-interference. R2D still occurs on the UL spectrum, D2R occurs on the DL spectrum, so the spectrum of D2R reception and R2D transmission on the UE is the same with legacy UE information transmission/reception spectrum, which has small impact on the UE. For the listed disadvantages, we have comment on the third and the fourth sub bullet. For the third sub bullet, the power of CW will be controlled, and coexistence issue is evaluated in RAN4, so it will not exist the case that CW transmission with high transmission power causes interference to neighbor BSs. For the fourth sub bullet, for the case 2-3, intermediate UE only receives the D2R on the downlink and DL NR signal on the downlink, it does not require UE to support reception on the DL spectrum and UL spectrum, so the fourth sub bullet has technical error and shall be deleted. So based on above analysis, we make the following revision with the blue part: Case 2-3: outside/DL; - UE does not need to support higher capability, e.g., reception in DL, full duplex capability (9 sources) - Cross-link interference handling at D2R receiver side (6 sources) - Spatial isolation is possible, reducing the received interference power. (1 source) - higher CW power may be permitted in the DL spectrum than that of UL. (1 source) - does not exist the self-interference - the spectrum of D2R reception and R2D transmission on the UE is the same with legacy UE information transmission/reception spectrum, which has small impact on the UE; - D2R transmission in DL spectrum is not in line with current spectrum usage (7 sources) - Separate CW and UE/reader for tx/rx require coordination. (2 sources) - With high transmission power, can cause interference to neighbor BSs. (2 sources) - Increased complexity of the intermediate UE since it would be required to support reception on the DL spectrum and UL spectrum, to support the CW being transmitted from inside the topology. (1 sources) - Additional TDM coordination of NR DL transmission, CW transmission/D2R receiving and R2D transmission is needed (1 sources) - Estimation of CW interference outside the D2R receiver may be needed. (1 sources). |

CW characteristics which need control [Open]

The issue on whether CW node can be controlled was discussed in RAN#103, the following agreements was achieved in RAN#103.

RAN#103 agreement: - Regarding the objective in the SID: Study necessary characteristics of carrier-wave waveform for a carrier wave provided externally to the Ambient IoT device, including for interference handling at Ambient IoT UL receiver, and at NR basestation. - This objective allows studying CW waveform characteristics which would need control of the CW node(s), e.g. waveform characteristics that impact interference such as when CW is transmitted or not transmitted, power, bandwidth, spectrum, etc. - No SID revision is necessary

According to the agreements and the discussion in RANP [7], the SI can consider “characteristics which would need control” and does not introduce the study of how to do that control.

In last RAN1 meeting, CW characteristics which need control were discussed, the latest proposal was as follows.

FL5 High Priority Proposal 4-1a: From RAN1 perspective, at least the following CW characteristics would need control: - Timing (e.g., when CW is transmitted or not transmitted, Time resources) - Transmission Power - Frequency resources - FFS other CW characteristics - FFS what kind of control is needed - Note: How to control (including signaling design) is not studied in R19 SI phase.

In this meeting, contributions [9], [10], [12], [14], [15], [16], [17], [18], [19], [21], [22], [23], [25], [26], [29], [30], [31], [35], [36], [38], [39], [40] continue discussed this issue. For the characteristics that would need control, the views from contributions are summarized below

Table 4-1: views for CW characteristics which would need control

| CW characteristics would need control | Contributions |
| --- | --- |
| Timing (e.g., when CW is transmitted or not transmitted, Time resources) | [9], [10], [12], [14], [15], [16], [17], [18], [19], [22], [23], [30], [31], [35], [36], [38], [39], [40] |
| Transmission Power | [9], [10], [12], [14], [15], [16], [17], [18], [19], [22], [31], [35], [36], [37], [39], [40] |
| Frequency resources | [10], [14], [15], [16], [17], [18], [19], [22], [30], [31], [35], [36], [37], [38], [39], [40] |
| Spectrum | [10], [12], [19] |
| Frequency hopping patterns | [10], [19] |
| Number of tones | [15], [29], [39] |

In addition, the following issues were discussed in the contributions.

- What aspects for each domain can be control

  - Contribution[12] proposes to consider dynamic CW on-off and semi-static CW on-off for time domain characteristics control, and consider interference and CW transmitter capability for CW power control.

  - Contribution [23] proposes to consider on-demand, periodically, always-on mode for timing domain, and consider PSD, maximum/minimum transmission power for power domain, and consider transmission bandwidth for frequency domain.

  - Contribution  [30] thinks that time duration of CW, period of CW should be considered for time domain; one or multiple center frequencies of CW, Gap between tones, frequency hopping pattern of CW should be considered for frequency domain

  - Contribution [22] points out that starting and ending position of CW transmission, Guard period due to large SFO of AIOT devices, OFDM symbol/slot boundary alignment with NR system should be studied for the timing of carrier wave. Besides, contribution [22] propose to study close-loop power control mechanism for power control of CW, and thinks that CW transmission should be TDMed with R2D transmission at least in D2T2.

  - Contribution [18] [39] mentioned that it should be possible to turn the CW node on and off. Contribution [18] further suggests to study the impact of following aspects on the power control of the carrier wave transmission: Amplification gain and Activation threshold

- Signaling/association related:

  - Contribution [8] doubts that whether the control of a CW node which is a UE by a reader (which is a gNB), would incur any signaling or data is unclear.

  - Contribution [18] prefers to study the association between carrier wave node and ambient IoT device, e.g., serving carrier wave node determination for the ambient IoT device. activation/deactivation of the carrier wave node. Study the mechanisms to support the power control of the carrier wave transmission.

  - Contribution [38] wants to study the transmission of control signal from the reader to the CWN, and consider time resources, frequency resources, beamforming related information.

- Which CW node need control:

  - Contribution [16] thinks that when CW is inside topology, study necessary CW control, details can be up to WI. [30] How to control CW node inside topology (e.g., control signaling) should studied, Contribution [21] proposed that for the case that the CW node is outside topologies (both for topology 1 and 2), the control of the CW node can be achieved by gNB implementation. Contribution [21] proposed that not support UE to be the CW node if the UE does not participate in AIoT D2R/R2D transmission. [26] From RAN1 perspective, CW characteristics including transmission power and resource allocation can be up to implementation.

- Others

  - Contribution [14] proposes to capture in the TR the aspects related to the control of the CW node(s) which impact interferences.

  - Contribution [10] observed that depending on the RRC state and the capability of the UE participating as CW node for AIoT operation, the scheduled NR traffic can either be performed concurrently or completed before CW transmission, respectively.

Based on the above and considering this is the last RAN1 meeting for CW, RAN1 can capture in the TR the aspects related to the control of the CW node(s) which impact interferences. Detailed solution or signaling can be discussed and specified in normative phase, if needed.

FL1 High Priority Proposal 4-1a: From RAN1 perspective, at least the following CW characteristics that impact interference would need control:

  - Timing (e.g., when CW is transmitted or not transmitted, Time resources)

  - Transmission Power

  - Frequency resources

  - Spectrum

  - Frequency hopping patterns (if supported)

  - Number of tones

Note: How to control (including signaling design) is not studied in R19 SI phase.

| Company | Y/N | Comments |
| --- | --- | --- |
| OPPO | Comments | We agree with this proposal except for the Note. We think the control of CW characteristics can be studied in R19 SI phase and some initial observations can be achieved (e.g., dynamic control or semi-static control). |
| TCL | Y | Good proposal |
| Huawei, HiSilicon | Comments | We do not see the need to include spectrum and number of tones here. Spectrum is not “controlled”, rather it is licensed outside of 3GPP and deployed by an MNO or spectrum owner. The number of tones is not clear if it requires any control, or can be chosen by the CW node implementation. Frequency hopping patterns can be removed unless the study is expanded to add it. |
| Xiaomi | Y |  |
| LGE | comment | Similar view with OPPO/Huawei. We can update the proposal to only include common part among the companies as the following. FL1 High Priority Proposal 4-1a: From RAN1 perspective, at least the following CW characteristics that impact interference would need control: - Timing (e.g., when CW is transmitted or not transmitted, Time resources) - Transmission Power - Frequency resources - Spectrum - Frequency hopping patterns (if supported) - Number of tones Note: How to control (including signaling design) is not studied in R19 SI phase. |
| Lenovo | Comments | We share the same view as OPPO, and some initial study on the control of CW characteristics could be done in SI phase. |
| CEWiT | Comment | We agree with OPPO’s comment, study on control of CW characteristics can be discussed in R19 SI phase. |
| Ericsson |  | Maybe it is good to clarify that a reader is aware of these aspects, if they are not under the control of the reader. |
| FL | @Ericsson, FL’s understanding is that this may depending on the deployment or implementation. The following version is considered for Tuesday offline FL1 High Priority Observation 4-1b: From RAN1 perspective, at least the following CW characteristics that impact interference would need control: - Timing (e.g., when CW is transmitted or not transmitted, Time resources) - Transmission Power - Frequency resources - Spectrum - Frequency hopping patterns (if supported) - Number of tones Note: How to control (including signaling design) is not studied in R19 SI phase. |  |

CW interference at base station [Open]

Several contributions discussed CW interference at base station.

Contributions [22], [30], [32] propose to study CW interference at NR base station (e.g., at Uu UL reception at base station). In addition, contribution [22] suggests to study potential solutions to eliminate the interference of: 1) interference between NR UL transmission and D2R/CW transmission for topo 1 CW case 1-4 and topo 2 CW case 2-4; 2) Interference between NR UL transmission and R2D transmission in case 2-4 of Topology 2. Contribution [30] thinks that for CW interference in legacy NR system, study TDM and FDM between the legacy NR and AIoT systems are studied as a starting point.

Contribution [16] and [26] analyzed CW interference at base station, e.g., NR gNB UL reception may suffer interference from CW for case1-2/1-4.

Contribution [19] propose to study potential interference types at base station for topology 1 and topology 2, e.g., for topo1, the NR UL signal and the IoT signal (D2R) are received at the same time by gNB if one device and one gNB are considered. Contribution [25] propose to study interference between NR UL signal and R2D/D2R A-IoT transmissions in UL spectrum. Contribution [18] propose to study the required frequency guard band to reduce the interference between CW and UE UL data, and suggest to configure a separate component carrier dedicated for CW transmission to the ambient IoT device.

However, [16] indicated that the study on CW interference at base station is tasked by RAN4. Contribution [36] also mentioned that RAN1/4 will both discuss the co-existence scenarios. The coexistence evaluation study, e.g., co-channel coexistence/adjacent channel coexistence, may be studied in RAN4. The interference handling schemes if needed may be studied in RAN1 later. RAN1 can study the intra-AIoT interferences and discuss interference handling schemes if needed.

FL observation：Few contributions propose to study CW interference at NR base station and corresponding solutions to eliminate the interference. Some companies propose to study potential interference types at base station, while the interference is caused by R2D/D2R, rather than CW. One contribution propose to study the required frequency guard band to reduce the interference between CW and UE UL data, which may related to RAN4. Two contributions pointed out that CW interference at base station or co-channel/adjacent channel coexistence is tasked by RAN4.

As far as FL knows, RAN4 is now discussing interference cases and evaluating interference level at gNB side. In RAN1, it seems there is no necessary to study interference type, whether solution is needed may related to the RAN4’s outcome. Therefore, it seems there is no need to discuss these parts in RAN1 at current stage, unless RAN4 asks RAN1 to involve.

Even though, FL would like to give the following two options to check companies’ views.

FL1 Low Priority Question 5-1a: which of the following option is preferred for the study of CW interference at base station

- Option 1: Considering RAN4 is now discussing interference cases and evaluating interference level at gNB side, no need to discuss co-existence related in RAN1 at current stage, unless RAN4 asks RAN1 to involve.

- Option 2: Parallelized with RAN4 co-existence study, RAN1 can study interference handling schemes.

| Company | Option | Comments |
| --- | --- | --- |
| OPPO | Option 2 | The study for CW interference handling at base station is explicitly included in the objective of RAN1. Study necessary characteristics of carrier-wave waveform for a carrier wave provided externally to the Ambient IoT device, including for interference handling at Ambient IoT UL receiver, and at NR basestation. |
| Huawei, HiSilicon | Option 1 | Agree with the FL that RAN1 does not need to work on an aspect that is already being worked on by RAN4. Companies must show responsible management of workload when wanting unnecessary things in RAN1. |
| xiaomi | Option1 | It is preferred that this interference is up to the RAN4’s evaluation and design. |
| LGE | Option1 |  |
| Lenovo | Option 1 |  |

Others [closed]

CW interference at legacy NR UE side

Some contribution discussed CW interference at NR UE side.

Contribution [10] thinks that sufficient guard band should be used for AIoT transmissions to avoid any interference caused by the AIoT signals to legacy NR UEs. Contribution [22] thinks that CW interference to DL reception at UE should be studied in RAN1. Contribution [26] propose to study UE-device CLI. Contribution [40] analyzed effects on Legacy NR UE case by case

Contribution [23] indicated that for interference suppression between NR signal and AIoT signal, it is preferred to wait the RAN4’s evaluation and design. Contribution [36] thinks that RAN1/4 will both discuss the co-existence scenarios. The coexistence evaluation study, e.g., co-channel coexistence/adjacent channel coexistence, may be studied in RAN4. The interference handling schemes if needed may be studied in RAN1 later. RAN1 can study the intra-AIoT interferences and discuss interference handling schemes if needed.

According to the SID scope, CW interference at NR UE side is a kind of co-existence issue, and RAN4 is now studying co-existence including evaluation and interference scenarios. Therefore, FL thinks that there is no need to discuss these parts in RAN1, unless RAN4 ask RAN1 to involve.

FL: No discussion for CW interference at NR UE side in this meeting.

NR interference to A-IoT system

Some contribution discussed NR interference to A-IoT system.

Contribution [24] discussed interference from NR for deployment scenarios considered in RAN4, and observed that 1) for D1T1 option 1-1, a small guard between NR and AIoT in frequency domain could be enough to protect AIoT signal from the potential interference of NR signal. 2) for D1T1 option 1-2, a small guard between NR and AIoT in frequency domain could be enough to protect AIoT signal from the potential interference of NR DL signal. 3) for D1T1 option 1-2 with R2D/D2R on FDD UL, a large guard between AIoT and NR in frequency domain is necessary. 4) for D1T1 option 2, FDM or TDM between NR and AIoT is needed.

Contribution [21] [23] [36] thinks that it is preferred to wait the RAN4’s evaluation and design for this part.

Similar to that of section 6.1, NR interference to A-IoT system is a kind of co-existence issue. According to the SID scope, RAN4 is now studying co-existence including evaluation and interference scenarios. Therefore, FL thinks that there is no need to discuss these parts in RAN1, unless RAN4 ask RAN1 to involve.

FL1: No discussion for NR interference to A-IoT system in this meeting.

Down selection for CW cases

Several contributions [11] [15] [17] [18] [22] [23] [31] [32] [33] [34] continue share views for prioritize for CW cases without large frequency shift.

Down-selection of CW transmission cases was discussed in the previous meeting, but no consensus was reached.

FL observed that all the CW transmission cases have been inputted into the simulation section and RAN4, whether to down-select CW cases for evaluation or coexistence studies should be discussed in simulation section(9.4.1.1) or RAN4. Otherwise, additional LS may needed to inform such a modification to RAN4. This may affect RAN4’s work plan and progress.

The CW discussions in this section are focused on the interference type and the pros/cons, the observed workload is not that heavy. In order to avoid further redundant discussion and save time, down-selection discussion is not suggested. Whether the CW cases are suitable for deployment can be discussed in later phase.

Therefore, FL suggests to close the discussion of this issue.

FL: No further down-selection of CW transmission cases is pursued in A.I 9.4.2.4 in SI phase.

CW transmission (with large frequency shift)

In this meeting, several contributions ( [11], [14], [16], [23], [32], [34]) continue discussed CW transmission with frequency shift capability (e.g., CW transmitted in DL band, and D2R transmitted in UL band).

The proposed CW transmission cases with large frequency shift are summarize below.

Table 6.4-1 Priority views for CW cases with large frequency shift

| Cases for topo1 | Supporters | Cases for topo2 | Supporters |
| --- | --- | --- | --- |
| Inside CW in DL, D2R in UL | [11][14] [23] [32] [34] | Inside CW in DL, D2R in UL | [34] |
| Inside CW in UL, D2R in DL |  | Inside CW in UL, D2R in DL | [32] |
| Outside CW in DL, D2R in UL | [23] [32] [34] | Outside CW in DL, D2R in UL | [11] [34] |
| Outside CW in UL, D2R in DL |  | Outside CW in UL, D2R in DL | [23] [32] |

Contributions [14][35] shared detailed analysis for the above cases.  Contribution [30] thinks that all 8 cases should be studied.

In addition to the above, serval contributions [27] shared negative views for CW cases with large frequency shift. Contribution [11] indicated that a drawback common for the cases without a large frequency shift between CW transmission and backscattered transmission is the direct interference from CW node to gNB. Contribution [18] proposes to deprioritize D2R backscattering is transmitted in the different carrier as CW for D2R backscattering, until 9.4.1.2 confirms the support of large frequency shift for device type 1. This contribution also observed that the power consumption is at least in order of 100 µW, if the tag support frequency shift from DL spectrum to UL spectrum.

RAN 1 observed that large frequency shift is not feasible for device 1, and whether large frequency shift is necessary and feasible for device 2a needs to be FFS.

Observation For large frequency shift: - Large frequency shift can be used in shifting reflected signal in tens of MHz, e.g., from FDD DL to FDD UL frequency or vice versa. - Large frequency shift consumes 10s of uW to 100s of uW. - Large frequency shift is not feasible for device 1. - Large frequency shift requires a clock for IF generation which is accurate enough to avoid large guard band and interference to adjacent channels/bands. - Large frequency shift requires image suppression and may require harmonics suppression - Note: details of image suppression and harmonics suppression are not discussed in RAN1 - FFS: whether large frequency shift is necessary and feasible for device 2a

Therefore, until 9.4.1.2 confirm its capability for backscattering device, the discussion is not pursued.

FL:  Until 9.4.1.2 confirm its capability for backscattering device, the discussion on CW cases with large frequency shift is not pursued.

Waveform recommendations

Contribution [12] thinks that the unmodulated single-tone waveform is regarded as the baseline for the external carrier-wave. Contribution [17] thinks that both single-tone and two-tone unmodulated waveform for CW transmission can be prioritized for A-IoT system design. Contribution [11] proposed that unmodulated two-tones waveform with frequency gap comparable to the channel coherence bandwidth is a suitable candidate for CW, unless prohibited by limiting factors (e.g., available bandwidth for AIoT devices, interference mitigation techniques).

Contribution [15] mentioned that two-tone as CW is only considered for device 1 to get frequency diversity gain.

FL: This can be determined based on all observations at the end of SI.。

Miscellaneous

Contribution[10] proposed that existing NR/LTE OFDM waveforms must be used as baseline for study, where considering new waveforms, if adequately justified by practical evidence, is also not precluded, since multi-carrier/tone waveforms who display high PAPR are better suited for the RF energy harvester, which in turn could charge the device more quickly

FL: RAN1 already agreed to study single-tone and two-tone. In addition, according to RAN’s guidance, study of design of energy harvesting signal/waveform is out of SI scope in Rel-19.

Contribution [10] thinks that RAN1 to identify the aspects and metrics that should drive the selection of the carrier-wave signal type and study the impact that a certain waveform selection has on the main L1 aspects considered in this SI.

FL: the aspects and metrics are discussed in section 2.1.

Contribution [29] mentioned that study multi-tone CW for the purpose of energy harvesting. Contribution [36] propose to study the resource allocation of CW for EH, e.g., time resource allocation, frequency resource allocation and power allocation

FL:  according to RAN’s guidance, CW design for energy harvesting is not discussed in SI

Contribution [13] thinks that the CWT can transmit two steered beams of CW waveforms at frequencies f1 and f2 simultaneously to communicate with two A-IoT devices placed side-by-side. The tags backscatter to incident CW signals at the respective frequencies to transmit the UL data through PDRCH signal. Thus, multiple devices can be served simultaneously without interference while reducing the overall latency.

FL: this is a kind of implementation?

Contribution [12] points out that it is feasible for the same Ambient IoT device to support backscattering on both the downlink and uplink spectrum, with modest impact on device power consumption and complexity.

Contribution [15] propose to consider how to calculate guardband of AIoT systems for in-band and guardband configuration based on AIoT transmission BW, SFO, SCS, the number of chip during one NR symbol.

[29] Consider to support multiplexing between multiple devices (e.g. between device type 1/2a and 2b or between different frequency shift (i.e., F-shift) capabilities) as the following: 1)FDMA by assigning different set/range of F-shift values for D2R (backscattering) transmission, 2) TDMA by assigning different D2R-to-R2D timing and/or R2D-to-D2R timing

Contribution [15] wants to clarify the D2R frequency spectrum in given cases according to CW transmission spectrum.

Contribution [25] thinks that inter-device interference cancelation should be studied in following cases, 1) Devices communicate with the same reader. 2) Devices communicate with different readers.

Contribution [34] prefer to support frequency shift of the backscattered signal using subcarrier modulation. AIoT interface specifies interference measurement and sounding procedures at the reader to facilitate allocation of resources to different readers.

FL:  the above issues should be discussed in other section or RAN4?

Contribution  [28] thinks that read could determine to use one single-tone unmodulated sinusoid waveform or two unmodulated single-tones waveform by itself. Unified device backscatter transmission is expected regardless of CW waveform, i.e., no special implementation to support different CW waveform if both CW waveform types are supported.

Contribution [29] prefer to consider same topology/deployment/spectrum (in terms of R2D/CW/D2R transmissions) for all device types 1/2a/2b.

Contribution [21] observed that compared to single tone CW, there is no additional power consumption and complexity at AIoT device side by adopting multiple single-tone CW. [26] The impact of power consumption at A-IoT device side is minimal because CW waveform can be transparent to A-IoT device.

Contribution [15] propose to discuss the feasibility of SFBC (Space-frequency block coding) at transmitter side and Rx multi-antenna to compensate the loss of frequency diversity.

Contribution [37] suggested to establish a straightforward and clear communication method between the read and CW node with a harmonized scheme in all 6 cases for ambient IoT scenarios, which also involves signaling about the switching of single-tone and multi-tone waveform.

Contribution [18] thinks that the topology is transparent to the A-IoT devices, and the A-IoT devices support the backscatter communication in both topologies.

Contribution [26] thinks that spectrum used for CW, backscatter and R2D transmission should be considered together.

Contribution [26] proposed that CW, backscatter and R2D transmission in same spectrum (e.g. case 1-2 (a)) to reduce A-IoT device complexity, interference to NR UE.

Contribution [27] suggests to consider D2R channelization in terms of arranging/configuring the set of tones for composition of multi-tone CW (as well as for FH-based single-tone CW) and associated set of F-shift values for multi-device multiplexing (FDMA) of D2R (backscattering) transmission

Contribution [29] studied time delay for forwarding, timing adjustment, collision handling between R1 and R2 and so on for different deployment cases listed in 9.4.1.1(e.g., D1T1-A1, D1T1-A2)

Contribution [20] proposes to study what CW characteristics (used by CW nodes in the system) would need to be known by type 2b devices in order to mimic the backscatter signals, to harmonize the three devices.

Contribution [36] prefers to study the impact of CW2D and D2D interference in FDD-DL at R2D receiver. whether/how to consider the interference of the FDMed D2R at D2R receiver if supported.

Contribution [11] thinks RAN1 to further study the different aspects affecting the selection of the CW signal, i.e., the bandwidth allocated to the A-IoT devices, multiple access, channel conditions, band deployment (in-band, out-of-band, guard band), backscatter modulation/detection assumptions, and interference mitigation techniques.

FL: For the above, as these issues mentioned by very few companies, they will not be discussed with high priority in this meeting.

Proposals for online

Tuesday

FL1 High Priority proposal 2.1.1-1a: Update the agreed observation for D2R reception performance in RAN1#117 as follows:

Observation

For D2R reception performance,

- Compared to single-tone unmodulated sinusoid waveform without frequency hopping, two unmodulated single-tones waveform provides [0, 8] dB or [-0.2, 3]dB frequency diversity gain at 1% or 10% BLER target in a fading channel for a 1Rx reader and a 1Tx CW transmitter, at least depending on the gap between the two tones and the channel’s coherence bandwidth.

  - In a TDL-A or C fading channel with 30ns delay spread

    - For the gap between [75KHz, 900KHz], the frequency diversity gains at 1% BLER target observed by 6 sources are within [0, 1.5]dB, and the frequency diversity gains at 10% BLER target observed by 3 sources are almost 0dB.

    - For the gap between [1.08MHz, 4.2MHz], the frequency diversity gains at 1% BLER target observed by 6 sources are within [3, 5.8]dB, and the frequency diversity gains at 10% BLER target observed by 3 sources are within [0.4, 2.5]dB.

    - For the gap between [5MHz, 10MHz], the frequency diversity gains at 1% BLER target observed by 5 sources are within [5, 8]dB, and the frequency diversity gains at 10% BLER target observed by 5 sources are within [1.3, 4]dB.

  - In a TDL-D fading channel with 30ns delay spread

    - For 10MHz gap, 1 sources (R1-2405855) observed 0.7 dB@1%BLER and -0.2dB@10%BLER frequency diversity gain. (Note: loss due to the power split in TDL-D)

  - In a TDL-A fading channel with 150ns delay spread

    - For the gap is 180Khz, the frequency diversity gains at 1% BLER target observed by 2 sources are within [1, 3]dB, and the frequency diversity gains at 10% BLER target observed by 2 sources are within [0, 2.5]dB.

    - For the gap is 2.16MHz, the frequency diversity gains at 1% BLER target observed by 2 sources are within [7, 8]dB, and the frequency diversity gains at 10% BLER target observed by 2 sources are within [2.5, 5.5]dB.

    - For the gap is 5MHz, the frequency diversity gains at 1% BLER target observed by 2 sources are within [7, 8]dB, and the frequency diversity gains at 10% BLER target observed by 2 sources are within [2.5, 3]dB.

  - Note: The total transmission power is assumed the same for single-tone unmodulated sinusoid waveform without frequency hopping and two unmodulated single-tones waveform.

  - Note: For two unmodulated single-tones waveform, assume the two tones are transmitted from the same CW node.

FL1 High Priority Observation 2.1.2.1-1b: For the D2R transmission BW corresponding to the CW waveforms,

- Compared to single-tone unmodulated sinusoid waveform without frequency hopping, two unmodulated single-tones waveform

  - Requires 2 times the frequency domain resources for D2R transmission, if the frequency gap between each of these two tones is no smaller than the transmission bandwidth of the corresponding D2R transmission.

  - Note: For two unmodulated single-tones waveform, assume the two tones are transmitted from the same CW node.

FL1 High Priority Observation 2.1.3.1-1b: For interference suppression complexity for different CW waveforms at D2R receiver,

- Compared to single-tone unmodulated sinusoid waveform without frequency hopping, two unmodulated single-tones:

  - Requires two RF narrow-band bandpass filters when individual cancellation is applied to each of the two tones.

  - Note: For two unmodulated single-tones waveform, assume the two tones are transmitted from the same CW node

FL1 High Priority Observation 4-1b: From RAN1 perspective, at least the following CW characteristics that impact interference would need control:

  - Timing (e.g., when CW is transmitted or not transmitted, Time resources)

  - Transmission Power

  - Frequency resources

FL1 High Priority observation 2.2-1b: For the gap between two tones to leverage frequency diversity gain, the following aspects should be taken into account:

- Coherence bandwidth/delay spread

- D2R transmission bandwidth from the backscattering of one or multiple devices

- BW allocated for A-IoT

- Harmonics and/or intermodulation impacts

- Frequency shift(s) for one or more devices

Proposals for offline

Tuesday

FL1 High Priority proposal 2.1.1-1a: Update the agreed observation for D2R reception performance in RAN1#117 as follows:

Observation

For D2R reception performance,

- Compared to single-tone unmodulated sinusoid waveform without frequency hopping, two unmodulated single-tones waveform provides {0, 8} dB or {-0.2, 3}dB frequency diversity gain at 1% or 10% BLER target in a fading channel for a 1Rx reader and a 1Tx CW transmitter, at least depending on the gap between the two tones and the channel’s coherence bandwidth.

  - In a TDL-A or C fading channel with 30ns delay spread

    - For the gap between {75KHz, 900KHz}, the frequency diversity gains at 1% BLER target observed by 6 sources are within {0, 1.5}dB, and the frequency diversity gains at 10% BLER target observed by 3 sources are almost 0dB.

    - For the gap between {1.08MHz, 4.2MHz}, the frequency diversity gains at 1% BLER target observed by 6 sources are within {3, 5.8}dB, and the frequency diversity gains at 10% BLER target observed by 3 sources are within {0.4, 2.5}dB.

    - For the gap between {5MHz, 10MHz}, the frequency diversity gains at 1% BLER target observed by 5 sources are within {5, 8}dB, and the frequency diversity gains at 10% BLER target observed by 5 sources are within {1.3, 4}dB.

  - In a TDL-D fading channel with 30ns delay spread

    - For 10MHz gap, 1 sources observed 0.7 dB@1%BLER and -0.2dB@10%BLER frequency diversity gain.

  - In a TDL-A fading channel with 150ns delay spread

    - For the gap is 180Khz, the frequency diversity gains at 1% BLER target observed by 2 sources are within {1, 3}dB, and the frequency diversity gains at 10% BLER target observed by 2 sources are within {0, 2.5}dB.

    - For the gap is 2.16MHz, the frequency diversity gains at 1% BLER target observed by 2 sources are within {7, 8}dB, and the frequency diversity gains at 10% BLER target observed by 2 sources are within {2.5, 5.5}dB.

    - For the gap is 5MHz, the frequency diversity gains at 1% BLER target observed by 2 sources are within {7, 8}dB, and the frequency diversity gains at 10% BLER target observed by 2 sources are within {2.5, 3}dB.

  - Note: The total transmission power is assumed the same for single-tone unmodulated sinusoid waveform without frequency hopping and two unmodulated single-tones waveform.

  - Note: For two unmodulated single-tones waveform, assume the two tones are transmitted from the same CW node.

FL1 High Priority Observation 2.1.2.1-1b: For the required spectrum resources/BW of backscattered signal corresponding to the CW waveforms,

- Compared to single-tone unmodulated sinusoid waveform without frequency hopping, two unmodulated single-tones waveform

  - Consumes 2 times the spectrum resources for D2R transmission, if the frequency gap between each of these two tones is no smaller than the transmission bandwidth of the corresponding D2R transmission.

  - Note: For two unmodulated single-tones waveform, assume the two tones are transmitted from the same CW node.

FL1 High Priority Observation 2.1.3.1-1b: For interference suppression complexity for different CW waveforms at D2R receiver,

- Compared to single-tone unmodulated sinusoid waveform without frequency hopping, two unmodulated single-tones:

  - Requires two RF narrow-band bandpass filters when individual cancellation is applied to each of the two tones.

  - Note: For two unmodulated single-tones waveform, assume the two tones are transmitted from the same CW node.

FL1 High Priority observation 2.2-1b: For the gap between two tones to leverage frequency diversity gain, the following aspects should be taken into account:

- Coherence bandwidth/delay spread

- D2R transmission bandwidth from the backscattering of one or multiple devices

- BW allocated for A-IoT

- Harmonics and/or intermodulation impacts

- Frequency shift(s) for one or more devices

FL1 High Priority Observation 4-1b: From RAN1 perspective, at least the following CW characteristics that impact interference would need control:

  - Timing (e.g., when CW is transmitted or not transmitted, Time resources)

  - Transmission Power

  - Frequency resources

Reference

|  | RP-240826 | Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR, RAN#103, Maastricht, Netherlands. | RAN WGs SID (Revised SID) |
| --- | --- | --- | --- |
|  | RP-223396 | Study on Ambient IoT (Internet of Things) in RAN | RAN SID |
|  | TR 38.848 | Study on Ambient IoT (Internet of Things) in RAN | RAN TR |
|  | S1-220118 | Study on Energy Harvesting enabled Communication Services in 5GS. | SA1 SID |
|  | TR 22.840 | Study on Ambient power-enabled Internet of Things | SA1 TR |
|  | R1-2405729 | Final FL summary on CW waveform characteristics for A-IoT, RAN1#117, Fukuoka City, Japan, May 20th – 24th, 2024 | Moderator (Spreadtrum) |
|  | RP-240854 | Moderator's summary on R19 Ambient IoT, RAN#103, Maastricht, Netherlands. | Moderator (Huawei) |
|  | RP-241688 | Moderator's summary for offline discussion about Ambient IoT, RAN#104, Shanghai, China, June 17-20, 2024. | Moderator (RAN1 Vice-Chair) |
|  | R1-2405805 | Discussion on External Carrier Waveform Characteristics for Rel-19 Ambient IoT devices | FUTUREWEI |
|  | R1-2405823 | Waveform characteristics of carrier-wave provided externally to the Ambient IoT device | Nokia |
|  | R1-2405829 | Waveform characteristics of carrier wave provided externally to the Ambient IoT device | Ericsson |
|  | R1-2405855 | On external carrier wave for backscattering based Ambient IoT device | Huawei, HiSilicon |
|  | R1-2405967 | Discussion on CW waveform characteristics of A-IoT | Tejas Networks Limited |
|  | R1-2405915 | Discussion on waveform characteristics of external carrier-wave for Ambient IoT | Spreadtrum Communications |
|  | R1-2405970 | Discussion on waveform characteristics of external carrier-wave for Ambient IoT | TCL |
|  | R1-2405992 | Discussion on waveform characteristics of carrier-wave provided externally to the Ambient IoT device | CMCC |
|  | R1-2406013 | Discussions on waveform characteristics of carrier-wave for A-IoT | Intel Corporation |
|  | R1-2406073 | Discussion on external carrier wave for Ambient IoT | Lenovo |
|  | R1-2406094 | Discussion on waveform characteristics of carrier-wave provided externally to the Ambient IoT device | China Telecom |
|  | R1-2406144 | Waveform characteristics of carrier-wave provided externally to the Ambient IoT device | Semtech Neuchatel SA |
|  | R1-2406189 | Discussion on CW waveform and interference handling at AIoT UL receiver | vivo |
|  | R1-2406245 | Discussion on Waveform characteristics of carrier-wave provided externally to the A-IoT device | OPPO |
|  | R1-2406291 | Discussion on waveform characteristics of carrier-wave | Xiaomi |
|  | R1-2406318 | Analyses on interference between AIoT and NR | Fujitsu |
|  | R1-2406375 | Discussion on the waveform characteristics of carrier-wave for the Ambient IoT device | CATT |
|  | R1-2406408 | Discussion on carrier wave for Ambient IoT | ZTE Corporation, Sanechips |
|  | R1-2406430 | Discussion on waveform characteristics of carrier-wave for Ambient IoT device | Panasonic |
|  | R1-2406560 | Discussion on the waveform of carrier-wave for the Ambient IoT | NEC |
|  | R1-2406607 | Considerations on carrier-wave transmission for Ambient IoT | LG Electronics |
|  | R1-2406657 | Considerations for Waveform characteristics of carrier-wave | Samsung |
|  | R1-2406731 | Discussion on carrier-wave for Ambient IoT | ETRI |
|  | R1-2406776 | Waveform characteristics of carrier-wave provided externally to the Ambient IoT device | MediaTek Inc. |
|  | R1-2406843 | On carrier waveform and interference handling for AIoT | Apple |
|  | R1-2406893 | On the carrier-wave for Ambient IoT | InterDigital, Inc. |
|  | R1-2406937 | Study on waveform characteristics of carrier-wave for Ambient IoT | NTT DOCOMO, INC. |
|  | R1-2407036 | Waveform characteristics of carrier-wave provided externally to the Ambient IoT device | Qualcomm Incorporated |
|  | R1-2407072 | Discussion on waveform characteristics of carrier-wave provided externally to the Ambient IoT device | China Unicom |
|  | R1-2407091 | Discussion on Waveform characteristics of carrier-wave provided externally to the Ambient IoT device | CEWiT |
|  | R1-2407124 | Controllable Carrier Wave Characteristics | Sony |
|  | R1-2407128 | Discussion on Carrier wave related aspects for AIoT | IIT Kanpur, Indian Institute of Tech (M) |

Appendix

D2R performance gain for different gap at different BLER and delay spread

Table 10.1-1 Performance gain vs. gap @1% BLER+TDL-A/D/C 30ns+1R1T

| Gap | Gain (compared to single-tone) | Sources |
| --- | --- | --- |
| 75KHz | 0 dB(TDL-A) | [32] |
| 180KHz | 0 dB(TDL-A) | [11] |
|  | 0 dB (TDL-A) | [14] |
|  | 1 dB* (TDL-A) | [21] |
|  | 0 dB(TDL-A) | [32] |
|  | 0 dB* (TDL-A) | [36] |
| 360KHz | 0 dB (TDL-A) | [23] |
| 900KHz | ~1.5 dB(TDL-A) | [32] |
| 1.08MHz | ~5 dB* (TDL-A) | [21] |
| 2.16MHz | ~5 dB(TDL-A) | [11] |
|  | ~3 dB(TDL-A) | [14] |
|  | ~5.8 dB* (TDL-A) | [21] |
|  | 4 dB (TDL-A) | [36] |
| 3MHz | 5.5 dB* (TDL-A) | [23] |
| 3.3MHz | ~5.5 dB(TDL-A) | [32] |
| 3.6MHz | ~5.5 dB(TDL-A) | [32] |
| 4.2MHz | ~5.5 dB(TDL-A) | [32] |
| 5/5.01MHz | 6dB(TDL-A) | [11] |
|  | 9 dB (TDL-A) | [21] |
|  | 7 dB* (TDL-A) | [23] |
| 7MHz | 6 dB (TDL-C) | [16] |
| 10MHz | 0.7 dB (TDL-D) | [12] |
|  | 8 dB (TDL-A) | [36], |
| Note: The * indicates that the numerical value is read from the simulation graph |  |  |

Table 10.1-2 Performance gain vs. gap @1% BLER+TDL-A 150ns+1R1T

| Gap | Gain (compared to single-tone) | Sources |
| --- | --- | --- |
| 180KHz | ~1dB (TDL-A) | [11] |
|  | 3 dB (TDL-A) | [36] |
| 2.16MHz | ~7dB (TDL-A) | [11] |
|  | 8 dB[1T] (TDL-A) | [36] |
| 5 MHz | ~7dB (TDL-A) | [11] |
|  | 8 dB (TDL-A) | [26] |
| Note: The * indicates that the numerical value is read from the simulation graph |  |  |

Table 10.1-3 Performance gain vs. gap @10% BLER @TDL-A/C 30ns+1R1T

| Gap | Gain (compared to single-tone) | Sources |
| --- | --- | --- |
| 180KHz | 0dB (TDL-A) | [11] |
|  | 0 dB* (TDL-A) | [36] |
| 360kHz | 0 dB* (TDL-A) | [23] |
| 2.16MHz | ~0.8 dB* (TDL-A) | [11] |
|  | ~0.4 dB* (TDL-A) | [23] |
|  | ~2.5 dB* [1T] (TDL-A) | [36] |
| 3MHz | ~1.7 dB* (TDL-A) | [23] |
| 5/5.01MHz | ~1.3dB* (TDL-A) | [11] |
|  | ~2 dB* (TDL-A) | [21] |
|  | ~3.2 dB* (TDL-A) | [23] |
| 7MHz | 16: 2dB* (TDL-C) | [16] |
| 10MHz | -0.2 dB* (TDL-D) | [12] |
|  | ~4 dB*[1T] (TDL-A) | [36] |
| Note: The * indicates that the numerical value is read from the simulation graph |  |  |

Table 10.1-4 Performance gain vs. gap @10% BLER +TDL-A 150ns+1R1T

| Gap | Gain (compared to single-tone) | Sources |
| --- | --- | --- |
| 180KHz | 0dB* (TDL-A) | [11] |
|  | ~2.5 dB* (TDL-A) | [36] |
| 2.16MHz | ~2.5dB(TDL-A) | [11] |
|  | ~5.5 dB* (TDL-A) | [36] |
| 5/5.01MHz | ~2.5dB*(TDL-A) | [11] |
|  | 3 dB* (TDL-A) | [26] |
| Note: The * indicates that the numerical value is read from the simulation graph |  |  |

Table 10.1-5 Performance gain vs. gap

| Channel and Delay spread | Gap or Gap range | Gain or gain range | Target BLER | Source number |
| --- | --- | --- | --- | --- |
| TDL-A or C 30ns | [75KHz, 900KHz] | [0, 1.5] | 1% | 6 |
|  |  | 0dB | 10% | 3 |
|  | [1.08MHz, 4.2MHz] | [3, 5.8] | 1% | 6 |
|  |  | [0.4, 2.5] | 10% | 3 |
|  | [5MHz, 10MHz] | [5, 8] | 1% | 5 |
|  |  | [1.3, 4] | 10% | 5 |
| TDL-D 30ns | 10MHz | 0.7 dB | 1% | 1 |
|  |  | -0.2dB | 10% | 1 |
| TDL-A 150n | 180Khz | [1, 3] | 1% | 2 |
|  |  | [0, 2.5] | 10% | 2 |
|  | 2.16MHz | [7, 8] | 1% | 2 |
|  |  | [2.5, 5.5] | 10% | 2 |
|  | 5MHz | [7, 8] | 1% | 2 |
|  |  | [2.5, 3] | 10% | 2 |

Contact info for 9.4.2.4

Based on feedback in RAN1#116. Please feel free to update it, if any.

| Company | Point(s) of contact | Email address(es) |
| --- | --- | --- |
| NTT Docomo | Weiqi Sun | sunwq@docomolabs-beijing.com.cn |
| FUTUREWEI | Brian Classon | brian@classonconsulting.com |
| China Telecom | Yi Gu | guy6@chinatelecom.cn |
| xiaomi | Yajun Zhu Fu ting Wensu Zhao | zhuyajun@xiaomi.com futing@xiaomi.com zhaowensu@xiaomi.com |
| OPPO | Teng Ma | mateng1@oppo.com |
| CEWiT | Deepak PM Vishakha Singh | deepakpm@cewit.org.in vish@cewit.org.in |
| TCL | Rongling Jian | rongling.jian@tcl.com |
| Samsung | Hyemin Choe | hams.choe@samsung.com |
| Nokia/NSB | Ganesh Venkatraman | Ganesh.venkatraman@nokia.com |
| Spreadtrum | Mimi Chen | mimi.chen@unisoc.com |
| CATT | Fang-Chen Cheng Ren Da | fcc@catt.cn renda@catt.cn |
| SONY | Martin Beale | martin.beale@sony.com |

SID(Revised in RAN#103)

- 4 Objective ### 4.1 Objective of SI or Core part WI or Testing part WI This study targets a further assessment at RAN WG-level of Ambient IoT, a new 3GPP IoT technology, suitable for deployment in a 3GPP system, which relies on ultra-low complexity devices with ultra-low power consumption for the very-low end IoT applications. The study shall provide clear differentiation, i.e. addressing use cases and scenarios that cannot otherwise be fulfilled based on existing 3GPP LPWA IoT technology e.g. NB-IoT including with reduced peak Tx power. General Scope The definitions provided in TR 38.848 are taken into this SI, and the following are the exclusive general scope: - The overall objective shall be to study a harmonized air interface design with minimized differences (where necessary) for Ambient IoT to enable the following devices: - ~1 µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, neither DL nor UL amplification in the device. The device’s UL transmission is backscattered on a carrier wave provided externally. - ≤ a few hundred µW peak power consumption1, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, DL and/or UL amplification in the device. The device’s UL transmission may be generated internally by the device, or be backscattered on a carrier wave provided externally. - X is to be decided in WGs. - Coverage design target: Maximum distance of 10-50 m with device indoors as per TR 38.848: “…a range that WGs can sub-select within”. - For Topologies 1 & 2 (UE as intermediate node under NW control) per TR 38.848, with no RRC states, no mobility (i.e. at least no cell selection/re-selection -like function), no HARQ, no ARQ. NOTE 1: It is to be understood that “≤ a few hundred µW” means WGs are not tasked with setting a particular value, and that it will be for WG discussions to determine if a presented design with corresponding power consumption satisfies the “≤ a few hundred µW” requirement. - Deployment Scenarios with the following characteristics, referenced to the tables in Clause 4.2.2 of TR 38.848: - Deployment scenario 1 with Topology 1 - Basestation and coexistence characteristics: Micro-cell, co-site - Deployment scenario 2 with Topology 2 and UE as intermediate node, under network control - Basestation and coexistence characteristics: Macro-cell, co-site - The location of intermediate node is indoor - FR1 licensed spectrum in FDD. - Spectrum deployment in-band to NR, in guard-band to LTE/NR, in standalone band(s). - Traffic types DO-DTT, DT, with focus on rUC1 (indoor inventory) and rUC4 (indoor command). - From RAN#104, the study will assess whether the harmonized air interface design (per bullet ‘A’ above) can address the DO-A (Device-originated autonomous) use case, only to identify which part(s) of the harmonized air interface design (per bullet ‘A’ above) is/are not sufficient for the DO-A use case. Transmission from Ambient IoT device (including backscattering when used) can occur at least in UL spectrum. The following objectives are set, within the General Scope: - Evaluation assumptions - Conclude at least the following aspects of design targets left to WGs in Clause 5 (RAN design targets) of TR 38.848 [RAN1]. - Clause 5.3: Applicable maximum distance target values(s) - Clause 5.6: Refine the definition of latency suitable for use in RAN WGs - Clause 5.8: 2D distribution of devices - Define necessary further evaluation assumptions of deployment scenarios for coverage and coexistence evaluations [RAN1, RAN4] - Identify basic blocks/components of possible Ambient IoT device architectures, taking into account state of the art implementations of low-power low-complexity devices which meet the RAN design target for power consumption and complexity. [RAN1] - Define link budget calculation for coverage, including whether/how to model carrier wave from node(s) inside or outside the connectivity topology. NOTE: Assessment performance of the design targets is within the study of feasibility and necessity of proposals in the following objectives, e.g. by inspection of reference implementations in the field, simulations, analytically. NOTE: strive to minimize evaluation cases in RAN1. - Study necessary and feasible solutions for Ambient IoT as prescribed in the General Scope, including decisions on which functions, procedures, etc. are needed and not needed, and ensuring at least the required functionalities in Section 6.2 of TR 38.848. Study of positioning in Rel-19 is RAN3-led, limited to functionalities which would have no, or minimal, specification impact (note: this does not imply any decision relating to WI creation). Study the feasibility and required functionalities for proximity determination, which is the determination of whether BS or intermediate UE and ambient IoT device are near each other or not (coordination with SA3 is required for privacy aspects). - RAN1-led: For the Ambient IoT DL and UL: - Frame structure, synchronization and timing, random access - Numerologies, bandwidths, and multiple access - Waveforms and modulations - Channel coding - Downlink channel/signal aspects - Uplink channel/signal aspects - Scheduling and timing relationships - Study necessary characteristics of carrier-wave waveform for a carrier wave provided externally to the Ambient IoT device, including for interference handling at Ambient IoT UL receiver, and at NR basestation. For Topology 2, no difference in physical layer design from Topology 1. - RAN2-led: - Study and decide which functions are needed for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission, and study those functions. For example: - Paging - Random access - Data transmission, including necessary radio resource control aspects, respecting the limitation in the General Scope - Interactions with upper layers For functionalities not listed above, they are studied only if found essential. - RAN3-led: - Identify necessary impacts on signaling and procedures for CN-RAN interface, to enable: - Paging - Device context management - Data transport - Identify RAN architecture aspects, including whether support for split architecture is necessary. - Identify potential solutions for locating an Ambient IoT device with no specification impact, e.g. reusing existing user location report, or minimal specification impact to convey location information to core network. - RAN4-led: - Coexistence study of Ambient IoT and NR/LTE. - RF requirements study for Ambient IoT: - Ambient IoT BS transmission and reception - Ambient IoT Device, as per the General Scope, transmission and reception - Intermediate node (UE), as per the General Scope, transmission and reception RAN2 and RAN3 are expected to identify RAN-CN functional split in coordination with SA2. Note: This study shall target for an IoT segment well below the existing 3GPP IoT technologies, e.g. NB-IoT, eMTC, RedCap, etc. The study shall not aim to replace existing 3GPP LPWA technologies.

Previous agreements

RAN1#116

Agreement For R19 A-IoT study item, at least single-tone unmodulated sinusoid waveform is a candidate waveform for carrier wave for D2R backscattering. Agreement For R19 A-IoT study item, multi-tone waveforms for carrier wave for D2R backscattering can be studied. Agreement For the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering, and for topology 1, the following cases for CW transmission are studied. - Case 1-1: CW is transmitted from inside the topology, transmitted in DL spectrum. - Case 1-2: CW is transmitted from inside the topology, transmitted in UL spectrum. - Case 1-4: CW is transmitted from outside the topology, transmitted in UL spectrum. Agreement For the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering, and for topology 2, the following cases for CW transmission are studied. - Case 2-2: CW is transmitted from inside the topology (i.e., intermediate UE), transmitted in UL spectrum. - Case 2-3: CW is transmitted from outside the topology, transmitted in DL spectrum. - Case 2-4: CW is transmitted from outside the topology, transmitted in UL spectrum.

RAN#103

RAN#103 agreement: - Regarding the objective in the SID: Study necessary characteristics of carrier-wave waveform for a carrier wave provided externally to the Ambient IoT device, including for interference handling at Ambient IoT UL receiver, and at NR basestation. - This objective allows studying CW waveform characteristics which would need control of the CW node(s), e.g. waveform characteristics that impact interference such as when CW is transmitted or not transmitted, power, bandwidth, spectrum, etc. - No SID revision is necessary RAN#103 agreement: - Confirm that study of design of energy harvesting signal/waveform is out of SI scope in Rel-19 - The potential impact of energy harvesting on device availability for transmission and reception procedures can be considered for the study [RAN2, RAN1] - Duration of one device’s unavailability due to charging by energy harvesting can be assumed up to several tens of seconds - Note: this value can be revisited in future RAN plenary meetings, if necessary - TR 38.848 clause 5.6 statement on latency remains the case with respect to a single device, i.e.: “NOTE: The time for charging the Ambient IoT device storage (if present) is not included in the latency defined above. Time for energy harvesting, charging, etc. is regarded as an implementation issue only.” - No SID revision is necessary RAN#103 agreement: - RAN design targets for user experienced data rate, maximum message size, and moving speed of device: those can be used as assumptions in coverage evaluations, i.e. the coverage evaluations are done under the conditions that meet those targets. - Evaluations of RAN design targets for latency and connection/device density are allowed by the Rel-19 SID and observations on those evaluations can be captured in the TR38.769 - Note: this is as per the SID: “NOTE: Assessment performance of the design targets is within the study of feasibility and necessity of proposals in the following objectives, e.g. by inspection of reference implementations in the field, simulations, analytically.”

RAN1#116bis

Agreement For CW waveform for D2R backscattering, multiple unmodulated single-tone is studied compared to single-tone in R19 SI. - Two unmodulated single-tones as a starting point - FFS: Other number of tones - FFS: how large gap is needed between tones Agreement For CW waveform for D2R backscattering, contiguous multi-tone OFDM signal is not studied in R19 SI. Agreement Study at least the following characteristics of unmodulated single-tone and multiple unmodulated single-tone CW waveforms for backscattering: - For D2R - Reception performance - Spectrum utilization of backscattered signal corresponding to the CW waveforms - CW interference suppression at D2R receiver - Including complexity and CW cancellation capability value/range (if any) - For scenarios ’A1’, ’A2’ and ’B’ - Relative complexity of CW generation

RAN1#117

Agreement For the study of characteristics of CW waveforms, the following table is adopted as a template for capturing observations. Note 1: Further row(s) can be added, if other CW waveform characteristic is agreed. CW waveform characteristics; Observations and/or comparisons of single-tone unmodulated sinusoid waveform without frequency hopping and two unmodulated single-tones waveform for backscattering; … (if any). D2R reception performance. Spectrum utilization of backscattered signal corresponding to the CW waveforms. CW interference suppression at D2R receiver. Relative complexity of CW generation. Note: For two unmodulated single-tones waveform, the two tones are transmitted from the same CW node.. Observation For D2R reception performance, - Compared to single-tone unmodulated sinusoid waveform without frequency hopping, two unmodulated single-tones waveform provides [X Y] dB frequency diversity gain in a fading channel, at least depending on the gap between the two tones and the channel’s coherence bandwidth. - Note: The total transmission power is assumed the same for single-tone unmodulated sinusoid waveform without frequency hopping and two unmodulated single-tones waveform. - Note: For two unmodulated single-tones waveform, assume the two tones are transmitted from the same CW node. Observation For CW interference suppression at D2R receiver, - Compared to single-tone unmodulated sinusoid waveform without frequency hopping, two unmodulated single-tones waveform: - Requires additional complexity if RF interference cancellation is used at least with CW waveform reconstruction. - Note: RF interference cancellation is needed when the received CW interference power exceeds the blocking threshold of the receiver - Note: For two unmodulated single-tones waveform, assume the two tones are transmitted from the same CW node. Agreement For multiple unmodulated single-tone transmitted by one CW node, other number of tones (i.e. >2) is deprioritized. - Note: other number of tones (i.e. >2) is studied only when obvious gains are provided. Observation For relative complexity of CW generation - Compared to single-tone unmodulated sinusoid waveform without frequency hopping, two unmodulated single-tones waveform: - Leads to higher PAPR of the generated CW, which impacts the implementation of the power amplifier in the CW node. - Note: For two unmodulated single-tones waveform, assume the two tones are transmitted from the same CW node.

Agreements in RAN1#118

To be added.