3GPP TSG-RAN WG1 Meeting # 118	R1-2407462

# Maastricht, Netherlands, August 19 - 23, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 38.213 | CR |  | rev |  | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the application time of cell DRX for NR NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (Huawei) |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_enh-Core |  |  |  |  |  | Date: |  |  | 2024-08-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In current specification, a PDCCH in DCI format 2_9 received in slot m of a first serving cell can be used to activate and deactivate cell DTX and/or cell DRX for a second serving cell, and the application time is in slot m+d of the second serving cell, where d is the number of slots in 3 ms. However, the above application time of the cell DRX activation/deactivation would have a problem in NR NTN due to the large timing advance. Secondly, cell DRX and /or cell DTX are operated on a single cell for NR NTN, and the current description in the specification are applicable to multiple cell scenario. To avoid the confusion that may mislead people understand NTN is working with carrier aggregation, it is not proper to directly modify the original paragraph for supporting cell DRX and/or cell DTX operation in NR NTN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new paragraphs to specify the application time of activation and deactivation for cell DTX and cell DRX for the operation when cellSpecificKoffset is provided. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The application time of cell DTX and cell DRX activation/ deactivation when cellSpecificKoffset is provided is not correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***Unchanged parts are omitted***

11.5	Adaptation of cell operation

When a UE not provided with cellSpecificKoffset receives in slot m on the active DL BWP of a first serving cell a PDCCH providing DCI format 2_9 that indicates a change in activation or deactivation of a current cell DTX operation or cell DRX operation for a second serving cell, the UE operates on the second serving cell according to the indicated cell DTX operation or cell DRX operation starting from a slot on the active DL BWP or on the active UL BWP of the second serving cell, respectively, that is not before the beginning of the slot m+d on the active DL BWP of the first serving cell where d is a number of slots for the SCS of the active DL BWP of the first serving cell in Table 11.5-1.

When a UE provided with cellSpecificKoffset receives in slot m on the active DL BWP of a serving cell a PDCCH providing DCI format 2_9 that indicates a change in activation or deactivation of a current cell DTX operation for the serving cell, the UE operates on the serving cell according to the indicated cell DTX operation starting from a slot on the active DL BWP that is not before the beginning of the slot m+d on the active DL BWP of the serving cell, where d is a number of slots for the SCS of the active DL BWP of the serving cell in Table 11.5-1,  is the SCS configuration of the active DL BWP of the serving cell.

When a UE provided with cellSpecificKoffset receives in slot m on the active DL BWP of a serving cell a PDCCH providing DCI format 2_9 that indicates a change in activation or deactivation of cell DRX operation for the serving cell, the UE operates on the serving cell according to the indicated cell DRX operation starting from a slot on the active UL BWP of the serving cell. The slot on the active UL BWP of the serving cell is not before the beginning of the slot m+d+Kcell,offset∙2μ on the active DL BWP of the serving cell at the uplink time synchronization reference point, where d is a number of slots for the SCS of the active DL BWP of the serving cell in Table 11.5-1,  is the SCS configuration of the active DL BWP of the serving cell, and Kcell,offset is provided by cellSpecificKoffset.

***Unchanged parts are omitted***