3GPP TSG RAN WG1 #118							R1-2407199

Maastricht, NL, August 19th – 23rd, 2024

Source:		Ad-Hoc Chair (NTT DOCOMO)

Title:			Session Notes for 8.2.1

Agenda Item:		8.2.1

Document for: 	Endorsement

- 8.2.1	UE features for other Rel-18 work items (Topics A)

Including UE features for sidelink, MC enhancements, eRedCap, XR, CovEnh, dedicated spectrum less than 5MHz, eDSS, endorsed TEI proposals, and MBS (triggered by R1-2401941).

R1-2407198	Summary of discussion on UE features for MC enhancements	Moderator (NTT DOCOMO, INC.)

Agreement:

Update FG49-12/12a as below.

- Update the name of FG 49-12 to include “inter-cell” beam management

- Clarify the DL assignment condition in Component 1 of both FGs 49-12/12a as follows

  - “a)	MAC-CE+DCI-based TCI state indication (use of DCI formats 1_3 with DL assignment for at least one serving cell in a scheduledCellListDCI-1-3 to provide indicated unified TCI state(s) for the CC(s) in the scheduledCellListDCI-1-3)”

Agreement:

Introduce following FG for default QCL assumption for multi-cell scheduling.

- FG name

  - Default QCL assumption for multi-cell scheduling by DCI format 1_3

- Component

  - Indicates whether the UE can be configured with enabledDefaultBeamFormultiCellScheduling for default QCL assumption for multi-cell scheduling by DCI format 1_3 for same/different numerologies

    - Candidate values are {different only, both}

      - When "both" is reported, the UE supports this feature for same SCS and for different SCS combination(s) (low-to-high, high-to-low or both) reported for 49-1b

- Prerequisite

  - At least one of {49-1, 49-1b}

- Type

  - Per BC

- Note

  - None

- Mandatory or optional

  - Optional with capability signaling

Agreement:

Introduce following FG that indicates support of BWP switch by DCI format 0_3/1_3.

- FG name

  - Support of BWP switch indication by DCI format 0_3/1_3

- Component

  - Support of BWP switch indication by DCI format 0_3/1_3

- Prerequisite

  - At least one of {49-1, 49-1b, 49-2, 49-2b} for the BC

  - At least one of {6-2, 6-3, 6-4} for at least one band of the BC

- Type

  - Per BC

- Note

  - none

- Mandatory or optional

  - Optional with capability signaling

R1-2405932	Discussion on UE feature for topics A	ZTE Corporation, Sanechips

R1-2406225	Discussion on UE features for multi-carrier enhancement	OPPO

R1-2406635	UE features for other Rel-18 work items (Topics A)	Samsung

R1-2406980	UE features for other Rel-18 work items (Topics A)	Huawei, HiSilicon

R1-2407161	Rel-18 UE features topics set A	Ericsson