3GPP TSG-RAN WG1 Meeting #118	Tdoc R1- 2407751

Maastricht, Netherlands, 19th – 23th August 2024

Agenda Item:	9.7.2

Source:	Tejas Networks

Title:	Discussion on ISAC Channel Modeling

Document for:	Discussion, Decision

# Discussion

The objectives of the study item “Channel modeling for integrated sensing and communication (ISAC) in 5G-NR” [1], listed in Rel-19, and endorsed in RAN1#103, are as follows:

The focus of the study is to define channel modelling aspects to support object detection and/or tracking (as per the SA1 meaning in TS 22.137). The study should aim at a common modelling framework capable of detecting and/or tracking the following example objects and to enable them to be distinguished from unintended objects:

- UAVs

- Humans indoors and outdoors

- Automotive vehicles (at least outdoors)

- Automated guided vehicles (e.g. in indoor factories)

- Objects creating hazards on roads/railways, with a minimum size dependent on frequency

All six sensing modes should be considered (i.e. TRP-TRP bistatic, TRP monostatic, TRP-UE bistatic, UE-TRP bistatic, UE-UE bistatic, UE monostatic).

For the above use cases, sensing modes and frequencies:

- Identify details of the deployment scenarios corresponding to the above use cases.

In RAN1#118, the following agreements were made where a few of the details related to the modeling of the signal propagation paths between the Tx, target, and the RX were left as FFS [4]. This contribution provides our view on some of these topics.

RAN1#118 Agreement If RCS related coefficient of a scattering point is included in small scale, the RCS related coefficients are separately determined for different pairs of incident/scattered ray(s) at the scattering point. Agreement For radio propagation Case 1, for modelling the target channel of a target with single scattering point, - To model a direct path, a single LOS ray from Tx to target and a single LOS ray from target to Rx are generated - AoA/ZoA of the direct path at Rx, AoD/ZoD of the direct path at target are generated at least based on the 3D location of target and Rx in the global coordinate system - AoD/ZoD of the direct path at Tx, AoA/ZoA of the direct path at target are generated at least based on the 3D location of Tx and target in the global coordinate system - The Delay of the direct path = (d3D_tx_target + d3D_target_rx)/c - The Doppler of the direct path is generated by spherical unit vectors by AoD/ZoD at Tx, by spherical unit vectors by AoA/ZoA at Rx, and velocity of Tx, target and Rx - The power of the direct path is generated as the product of the power of the LOS ray from Tx to target, the power of the LOS ray from target to Rx, and the effect of RCS - FFS initial phase - FFS how to model RCS, polarization of target - FFS number of direct paths - FFS on detailed modelling of indirect path(s) - FFS applicability of direct path generation to each scattering point when the target is modelled as multiple scattering points Agreement For the target channel of a target with single scattering point, when stochastic cluster is used to model an indirect path in the target channel, - An indirect path in small scale is modelled by concatenation of path(s) from Tx to target and from target to Rx, i.e., Option 1 in the agreement of RAN1 #117 - AoD/ZoD/AoA/ZoA from Tx to target or from target to Rx are - generated for a LOS ray at least based on the 3D location of Tx/Rx and target in the global coordinate system - stochastically generated for a NLOS ray using section 7, 38.901 as starting point - Delay is sum of delay of LOS/NLOS ray from Tx to target and the LOS/NLOS ray from target to Rx - Doppler is generated by spherical unit vector by AoD/ZoD at Tx and velocity of Tx, by spherical unit vector by AoA/ZoA at Rx and velocity of Rx, and by spherical unit vectors by AoA/ZoA/AoD/ZoD at target and velocity of target - FFS The mobility of stochastic clutter - The power of the indirect path is generated as the product of the power of the LOS/NLOS ray from Tx to target, the power of the LOS/NLOS ray from target to Rx and the effect of RCS - FFS initial phase - FFS how to model effect of RCS at target - FFS whether/how to model polarization at target - FFS How to reduce complexity - FFS applicability of the indirect path generation to each/a single scattering point when the target is modelled as multiple scattering points Working assumption The RCS related coefficient of a scattering point can be modelled with two components, i.e., linear value RCS = A*B - A first RCS component A (m2) is included in large scale - FFS the first RCS component is deterministic or stochastic - FFS The first component is dependent on incident and scattered directions at target - A second RCS component B (unit ratio) is included in small scale - FFS The second component is dependent on incident and scattered directions at target - FFS the second RCS component is deterministic or stochastic or combination - Note: RCS component A or B can be disabled by setting its linear value to 1 - Whether to disable a component can be discussed per target type - FFS how to determine A and B for each target - FFS whether/how to model polarization impact at target - FFS whether/how to normalize power accounting for target channel and background channel Agreement The impact of a scattering point of the target in the target channel is modelled by a scalar RCS value RCS times a complex-valued 2x2 polarization matrix CPMsp, i.e., RCS∙CPMsp - FFS whether CPMsp is angular/ray-dependent or independent - FFS whether polarization matrix CPMsp is modelled assuming specular reflection or random coefficient for diffraction or scattering - FFS whether polarization matrix CPMsp is explicitly modelled or merged with other polarization matrixes from Tx to target and/or from target to Rx Agreement For modeling stochastic cluster in background channel, in order to define the background channel for TRP-UE and UE-TRP bistatic sensing mode, - The large scale and small scale parameters defined in TR 38.901, TR 37.885, TR 36.777 are used as start point In order to define the background channel for TRP-TRP and UE-UE bistatic sensing mode, - RAN1 to study how to model background channel - Option 1: The large scale and small scale parameters defined in TR 38.901, TR 38.858, 37.885, 38.859 are used as start point - Option 2: New channel model based on measurement results or ray-tracing model validated by experimental results FFS whether/how to do power normalization between target channel and background channel Agreement In order to define the background channel for TRP and UE mono-static sensing mode, - RAN1 to study how to model the background channel - Option 1: randomly drop at least one virtual Rx, and then the background channel is generated based on the channel generated as TR 38.901 between the real Tx and each virtual Rx for a scenario - FFS EO is modelled in the background channel - Option 2: New channel model based on measurement results or ray-tracing model validated by experimental results or radar literatures - FFS EO is modelled in the background channel - Option 3: the locations of clusters in the target channel are deterministically generated, then the background channel is generated using the clusters with the determined locations. - Other options are not precluded Agreement When EO type-2 is modelled, specular reflection is considered to model EO type-2 using section 7.6.8 of TR 38.901 as reference - As starting point, the effect of type-2 EO (i.e., in the path node1-EO-node2) is modelled as bR∥specular00-R⊥specular, b is a scaling factor (e.g., c equals to d3DdGR relative to LOS ray in section 7.6.8 in TR 38.901) - FFS any update to b,R∥specular,R⊥specular - FFS any update taking EO orientation into account - FFS any changes based on section 7.6.8 of TR 38.901 if EO type-2 has finite size - FFS whether diffraction and scattering can be considered in addition to specular reflection - EO type-2 is an optional modelling component if supported in a sensing scenario - FFS which deployment scenario(s) EO type-2 will apply

# 2.Generation of Channel Coefficients

Agreement For radio propagation Case 1, for modelling the target channel of a target with single scattering point, - To model a direct path, a single LOS ray from Tx to target and a single LOS ray from target to Rx are generated - AoA/ZoA of the direct path at Rx, AoD/ZoD of the direct path at target are generated at least based on the 3D location of target and Rx in the global coordinate system - AoD/ZoD of the direct path at Tx, AoA/ZoA of the direct path at target are generated at least based on the 3D location of Tx and target in the global coordinate system - The Delay of the direct path = (d3D_tx_target + d3D_target_rx)/c - The Doppler of the direct path is generated by spherical unit vectors by AoD/ZoD at Tx, by spherical unit vectors by AoA/ZoA at Rx, and velocity of Tx, target and Rx - The power of the direct path is generated as the product of the power of the LOS ray from Tx to target, the power of the LOS ray from target to Rx, and the effect of RCS - FFS initial phase - FFS how to model RCS, polarization of target - FFS number of direct paths - FFS on detailed modelling of indirect path(s) - FFS applicability of direct path generation to each scattering point when the target is modelled as multiple scattering points.

In RAN#117, the question of whether to account for initial phase generation when modeling the target was left for further study. This topic remained open in RAN#118, with no further discussion. Typically, random initial phase generation is introduced, as outlined in TR 38.901, to capture the phase shifts caused by the interaction of signals with different clusters, including effects beyond random cross-polarization. It is well known that electromagnetic waves undergo phase changes when they encounter an object, influenced by the object's geometry, material, and boundary conditions [11]. This phenomenon applies to sensing targets as well, making it essential to account for the initial phases for accurate target modeling. Additionally, to simplify the process, we propose using a single initial phase for each polarization, regardless of the number of scattering points of a target. This approach reduces complexity while still providing a reasonable level of accuracy for modeling.

One method for generating initial random phases for a target involves empirically determining the mean and variance for a specific type of target. These phases can then be generated from a normal distribution with the corresponding mean and variance. This approach requires experimental data collection for a particular target, such as a UAV at various 3D locations, with different orientations and environmental conditions. The collected data can then be approximated by a Gaussian distribution, enabling the calculation of the corresponding mean and variance for random phase generation.

Observation 1: It is well known that electromagnetic waves undergo phase changes when they encounter an object, influenced by the object's geometry, material, and boundary conditions. Therefore, it is necessary to model the initial phase for the sensing targets as well

Observation 2: In order to simplify the process, we propose using a single initial phase for each polarization, regardless of the number of scattering points of a target. This approach reduces complexity while still providing a reasonable level of accuracy for modeling.

Proposal 1: We propose to model the initial phases for the sensing targets

Proposal 2: We propose to consider a single initial phase for each polarization irrespective of the number of scattering points of the target.

Proposal 3: To generate the initial phase, we propose to randomly sample from a Gaussian distribution for each polarization where the mean and the variance are found experimentally.

Proposal 4: We propose to have a target specific distribution parameterized by the mean and the variance.

Agreement The impact of a scattering point of the target in the target channel is modelled by a scalar RCS value RCS times a complex-valued 2x2 polarization matrix CPMsp, i.e., RCS∙CPMsp - FFS whether CPMsp is angular/ray-dependent or independent - FFS whether polarization matrix CPMsp is modelled assuming specular reflection or random coefficient for diffraction or scattering - FFS whether polarization matrix CPMsp is explicitly modelled or merged with other polarization matrixes from Tx to target and/or from target to Rx

As discussed earlier, since the polarization matrix is a characteristic of the sensing object, it should be captured by the radar cross section (RCS). To simplify the model, assuming the target is relatively small compared to the distance between the transmitter (Tx) and receiver (Rx), it is reasonable to treat the cross-polarization matrix (CPMsp) as independent of the rays. Additionally, to better extract the specific characteristics of the sensing target, we recommend explicitly modeling the target's polarization matrix. This approach allows for more precise characterization of the target's polarization behavior in the sensing process.

Observation 3: In order to assist in extracting the characteristics of the sensing target, it is better to explicitly model the polarization matrix of a target.

Observation 4: To keep it simple it is sufficient to keep the CPMsp as ray independent

Proposal 5: We propose to explicitly model the polarization matrix of a target.

Proposal 6: We propose ray independent CPMsp modelling

# 3. Background Channel modelling

Agreement For modeling stochastic cluster in background channel, in order to define the background channel for TRP-UE and UE-TRP bistatic sensing mode, - The large scale and small scale parameters defined in TR 38.901, TR 37.885, TR 36.777 are used as start point In order to define the background channel for TRP-TRP and UE-UE bistatic sensing mode, - RAN1 to study how to model background channel - Option 1: The large scale and small scale parameters defined in TR 38.901, TR 38.858, 37.885, 38.859 are used as start point - Option 2: New channel model based on measurement results or ray-tracing model validated by experimental results FFS whether/how to do power normalization between target channel and background channel

In ISAC, the sensing channel model consists of two components, as agreed in RAN#116: the target-related component, which carries information about the target's position and dynamics, and the background component, which represents the rest of the sensing channel. The background channel in the bistatic sensing mode, both the TRP-TRP and UE-UE, exists independently of the sensing target, meaning it remains present even in the absence of a target. When no sensing target is present, the background channel model should be same as the standard channel models, such as those outlined in 3GPP TR 38.901. Given this, it is reasonable to model the background channel in a manner consistent with TR 38.901.

Additionally, we propose using the large-scale parameters (LSP) and small-scale parameters (SSP) as defined in TR 38.901, TR 38.858, TR 37.885, and TR 38.859. This approach not only minimizes complexity but also serves as a reliable base solution. Therefore, we suggest adopting option 1 to model the background channel.

Furthermore, when modeling both the target and background channels separately, and after determining the power levels of each path, we can, without loss of generality, normalize the power across all paths. Since a signal transmitted from the TRP is subjected to the path losses across both channels, it is typical to keep the losses within the 0-1 range. These signal power levels are eventually multiplied by a scalar that corresponds to the power level at the TRP, ensuring the right power distribution across the channels.

Observation 5: The background channel exists independently of the sensing target, meaning it remains present even in the absence of a target. When no target is detected, the background channel model should be same as the standard channel models, such as those outlined in 3GPP TR 38.901.

Observation 6: With both the target and background channels modelled separately, we can, without loss of generality, normalize the power across all paths after determining the power levels of each path. Since a signal transmitted from the TRP is subjected to the path losses across both channels, it is typical to keep the losses within the 0-1 range and eventually multiplied by a scalar that corresponds to the power level at the TRP

Proposal 7: We propose to use the Option 1 for modelling the background channel.

Agreement In order to define the background channel for TRP and UE mono-static sensing mode, - RAN1 to study how to model the background channel - Option 1: randomly drop at least one virtual Rx, and then the background channel is generated based on the channel generated as TR 38.901 between the real Tx and each virtual Rx for a scenario - FFS EO is modelled in the background channel - Option 2: New channel model based on measurement results or ray-tracing model validated by experimental results or radar literatures - FFS EO is modelled in the background channel - Option 3: the locations of clusters in the target channel are deterministically generated, then the background channel is generated using the clusters with the determined locations. - Other options are not precluded

# Proposal 8: We propose to normalize the power levels across the paths in both the target and the background channel.

In integrated sensing and communication (ISAC), "monostatic" refers to a system where the transmitter and receiver are collocated, while "bistatic" refers to a system where the transmitter and receiver are separated by a certain distance. In monostatic configurations, due to the proximity of the transmitter and receiver or a duplex transceiver the receiver typically experiences high self-interference.

To accurately model such a channel, randomly placing a virtual receiver (Rx) without considering this proximity would fail to capture the significant self-interference. To simplify the model while capturing the key characteristics, we propose placing the virtual Rx at the same location as the transmitter (Tx), or to avoid implementation artifacts, consider an infinitesimally small 3D distance (ε) from the transmitter.

Moreover, with regard to environmental objects (EOs), which can either resemble sensing targets (Type 1) or represent large, static objects that are not meant to be sensed, we propose classifying them as part of the background channel. This approach aligns with the agreed modeling structure, where the overall channel is divided into two components: the background channel and the target channel.

Observation 7: To accurately model monostatic channel, randomly placing a virtual receiver (Rx) without considering the proximity of Tx and Rx would fail to capture the significant self-interference

Observation 8: With respect to the agreed modeling structure, where the overall channel is divided into two components: the background channel and the target channel, EO should be considered as part of the background channel

Proposal 9: We propose Option 1 but with a modification that the virtual Rx should be placed close to the Tx

Proposal 10: We propose to model EO as part of the background channel.

# 4. Physical Object Modelling

Agreement For radio propagation Case 1, for modelling the target channel of a target with single scattering point, - To model a direct path, a single LOS ray from Tx to target and a single LOS ray from target to Rx are generated - AoA/ZoA of the direct path at Rx, AoD/ZoD of the direct path at target are generated at least based on the 3D location of target and Rx in the global coordinate system - AoD/ZoD of the direct path at Tx, AoA/ZoA of the direct path at target are generated at least based on the 3D location of Tx and target in the global coordinate system - The Delay of the direct path = (d3D_tx_target + d3D_target_rx)/c - The Doppler of the direct path is generated by spherical unit vectors by AoD/ZoD at Tx, by spherical unit vectors by AoA/ZoA at Rx, and velocity of Tx, target and Rx - The power of the direct path is generated as the product of the power of the LOS ray from Tx to target, the power of the LOS ray from target to Rx, and the effect of RCS - FFS initial phase - FFS how to model RCS, polarization of target - FFS number of direct paths - FFS on detailed modelling of indirect path(s) - FFS applicability of direct path generation to each scattering point when the target is modelled as multiple scattering points.

The RADAR cross-section is traditionally defined as "the ratio of the intensity of the radar signal returned by the target to the intensity of the radar signal transmitted towards the target, adjusted for factors like distance and antenna characteristics", which seems limited for usage in ISAC scenario where, unlike RADAR scenarios, the communication channel is multipath, and the location of the transmitter and receiver is random. Hence, the definition of the RCS needs to be broadened for ISAC scenarios. The RCS can be defined as "the ratio of the intensity of the radar signal scattered by the target to the intensity of the radar signal coming from a particular incident angle", which will make it a function of the orientation of the target, incident angle, scattering angle, size of the target, shape of the target, and distance between the target and the transmitter. The modelling of RCS can be simplified by splitting it into two parts,

- The total power reflected, which is the function of the Tx to target separation, and the size of the target,

- The 3D scattering characteristic of the target (like the antenna's radiation pattern) is a function of the shape/geometry of the target, incident angle, scattering angle, and orientation of the target.

σ(dtx-target,l,h,w, otarget, θin,ϑin,θsc,ϑsc)= σ0dtx-target,l,h,w.aθin,ϑin,θsc,ϑsc,otarget

where dtx-target is the distance between the transmitter and the target, l,h,w denote the size/shape of the target, otarget denotes the orientation of the target, θin,ϑin denotes the incident (elevation, azimuth) angles, and θsc,ϑsc denotes the scatter (elevation, azimuth) angles. The σ0dtx-target,l,h,w captures the component of RCS impacted by the Tx to target separation and size of the target and aθin,ϑin,θsc,ϑsc,otarget denotes the 3D scatter pattern of the RCS which is a function of shape, orientation, incident angle and scatter angle. The latter is normalized with respect to distance and size, similar to the radiation pattern of antennas.

This will allow the generalization of the modelling of the RCS of the targets. In the future, if new targets are introduced as sensing targets, then only the 3D scattering characteristics of the new targets will be agreed upon, with insignificant changes to the rest of the channel model.

Observation 9: The target's RCS can be modelled similarly to cluster modelling in 38.901.

Proposal 11: The definition of the RCS of the target should be extended from the classic Radar literature definition to suit the ISAC needs [9].

The models for the RCS of humans, cyclists, vehicles, UAVs, UGVs, and hazards can be designed based on the data collected. To reduce the complexity of channel modelling, the RCS models can be approximated using mathematical equations.

## 4.1 RCS modelling of UAVs

The definition of RCS and the way to model it was discussed elaborately in many contributions to RAN#117. However, there were issues left for further discussion and not agreed on. In this section, we put forth our views on the way to model the RCS of a target.

A significant amount of research conducted both in academia and defense has focused on understanding the RCS characteristics of different categories of commercial UAVs, particularly the HH-plane and VV-plane characteristics. The RCS characteristics of the quadcopter, hexacopter, and octocopter UAVs are shown in Figure 1 [5], and they can be approximated using an omnidirectional scattering pattern. The results compare the simulated and measured RCS patterns with simulations performed based on the CAD model of the DJI Phantom-2/3/4 drones. The measurements and simulation results indicate that the average RCS value for these categories of drones is approximately -10 dBsm. Additionally, Fig-2 shows that the variance in the RCS value decreases with the number of wings on the drone. It starts to behave more like an omnidirectional scatterer as the number of wings increases on the x-copter class UAVs.

Observation 10: The RCS characteristics of the quadcopter, hexacopter and octocopter UAVs can be approximated using omnidirectional scattering patterns.

Observation 11: The measurements and simulation results indicate that the average RCS value for these categories of drones is approximately -10 dBsm. Additionally, Fig-2 shows that the variance in the RCS value decreases with the number of wings on the drone. It starts to behave more like an omnidirectional scatterer as the number of wings increases on the x-copter class UAVs.

Figure 1: Comparison of the complex quadcopter simulated RCS against the measured VV Phantom 2 RCS (left) and the measured HH Parrot RCS (right).

Proposal 12: At 9 GHz, μRCS= -10 dBsm,  σRCS= f(nwings) for x-copter class UAVs, these parameters should be computed for other frequencies as well.

Figure 2: The measured RCS CDFs for the quadcopters. The UAVs are the Phantom 2 (top left), Parrot (top right), Phantom 3 (bottom left) and Phantom 4 (bottom right) [5].

Figure 3: Euclid UAV RCS results for θ = 85°, φ = 0°–360° and (top) f= 8.7GHz, (middle) f=9.1GHz, (bottom) f=9.5GHz [6].

The second category of UAVs is fixed-wing or X-wing UAVs; for example, the family of Euclid UAVs that have RCS focused towards broadside and endfire direction. The RCS for such UAVs can be approximated using sinusoid functions with perturbation as a function of shape, incident and scattering angles, as shown in Fig-3 [6]. The details of the modelling are shown in Table 1. Furthermore, the mean value of RCS decreases as the frequency increases. Based on the measurements available, RCS can also be approximated using the model proposed for the antenna radiation pattern in TR 38.901.

Observation 12: The Flying Wing type UAVs have RCS focused toward broadside and endfire directions.

Observation 13: The mean value of RCS decreases with increase in the frequency.

Proposal 13: The RCS for Flying Wing type UAVs can be approximated using sinusoid functions with perturbation as a function of shape, incident and scattering angles.

Proposal 14: Based on the measurements available, RCS can also be approximated using the model proposed for antenna radiation pattern in TR 38.901.

Table 1: RCS models for different categories of UAVs.

| UAV type | RCS Model |
| --- | --- |
| x-copter UAVs (Quadcopter, hexacopter and octacopter UAVs such as Phontam-2/4/6) | μRCS= -10 dBsm, σRCS= f(dtx-target,l,h,w,nwings); where nwings denotes the number on the UAV, f = 9 GHz. |
| Flying Wing type UAVs such as Euclid UAV | μRCS= μ0θin,ϑin,θsc,ϑsc,f+αf.cos⁡(2ϑin), σRCS= f(dtx-target,l,h,w, ∅,θ, d); where nwings denotes the number on the UAV, θ=85°. |

The RCS should be modeled similarly for other categories of UAVs as well.

## 4.2 RCS modelling of Automotive and AGVs

For modelling the RCS of vehicles, the shape of the vehicles can be approximated to a rectangular box, and the body can be considered metallic [7]. The RCS of the automotive vehicles is also dominant in broadside and endfire direction, as shown in Figure 4 [7]. The RCS scattering model for vehicles can be approximated using the antenna radiation model stated in TR 38.901 based on the RCS measurements available.

Observation 14: For modelling the RCS for automotive, the shape of the vehicles can be approximated to a rectangular box, and the body can be considered as metallic.

Observation 15: The RCS of the automotive/vehicles is also dominant in broadside and endfire direction.

Figure 4: (a) Car like model, (b) Sedan car model meshed with 7962 facets. (c) RCS of car like for θ = 90 at frequency 72-110 GHz [7].

Proposal 15: The RCS scattering model for vehicle can be approximated using the antenna radiation model stated in TR 38.901 based on the RCS measurements available.

# 6. Conclusions

In this contribution, we make the following observations regarding the modeling of ISAC channel.

Observation 1: It is well known that electromagnetic waves undergo phase changes when they encounter an object, influenced by the object's geometry, material, and boundary conditions. Therefore, it is necessary to model the initial phase for the sensing targets as well

Observation 2: In order to simplify the process, we propose using a single initial phase for each polarization, regardless of the number of scattering points of a target. This approach reduces complexity while still providing a reasonable level of accuracy for modeling.

Observation 3: In order to assist in extracting the characteristics of the sensing target, it is better to explicitly model the polarization matrix of a target.

Observation 4: To keep it simple it is sufficient to keep the CPMsp as ray independent

Observation 5: The background channel exists independently of the sensing target, meaning it remains present even in the absence of a target. When no target is detected, the background channel model should be same as the standard channel models, such as those outlined in 3GPP TR 38.901.

Observation 6: With both the target and background channels modelled separately, we can, without loss of generality, normalize the power across all paths after determining the power levels of each path. Since a signal transmitted from the TRP is subjected to the path losses across both channels, it is typical to keep the losses within the 0-1 range and eventually multiplied by a scalar that corresponds to the power level at the TRP

Observation 7: To accurately model monostatic channel, randomly placing a virtual receiver (Rx) without considering the proximity of Tx and Rx would fail to capture the significant self-interference

Observation 8: With respect to the agreed modeling structure, where the overall channel is divided into two components: the background channel and the target channel, EO should be considered as part of the background channel

Observation 9: The target's RCS can be modelled similarly to cluster modelling in 38.901.

Observation 10: The RCS characteristics of the quadcopter, hexacopter and octocopter UAVs can be approximated using omnidirectional scattering patterns.

Observation 11: The measurements and simulation results indicate that the average RCS value for these categories of drones is approximately -10 dBsm. Additionally, Fig-2 shows that the variance in the RCS value decreases with the number of wings on the drone. It starts to behave more like an omnidirectional scatterer as the number of wings increases on the x-copter class UAVs.

Observation 12: The Flying Wing type UAVs have RCS focused toward broadside and endfire directions.

Observation 13: The mean value of RCS decreases with increase in the frequency.

Observation 14: For modelling the RCS for automotive, the shape of the vehicles can be approximated to a rectangular box, and the body can be considered as metallic.

Observation 15: The RCS of the automotive/vehicles is also dominant in broadside and endfire direction.

Additionally, the following proposals were made based on our understanding of the use cases and scenarios of ISAC channel modelling.

Proposal 1: We propose to model the initial phases for the sensing targets

Proposal 2: We propose to consider a single initial phase for each polarization irrespective of the number of scattering points of the target.

Proposal 3: To generate the initial phase, we propose to randomly sample from a Gaussian distribution for each polarization where the mean and the variance are found experimentally.

Proposal 4: We propose to have a target specific distribution parameterized by the mean and the variance.

Proposal 5: We propose to explicitly model the polarization matrix of a target.

Proposal 6: We propose ray independent CPMsp modelling

Proposal 7: We propose to use the Option 1 for modelling the background channel.

# Proposal 8: We propose to normalize the power levels across the paths in both the target and the background channel.

Proposal 9: We propose Option 1 but with a modification that the virtual Rx should be placed close to the Tx

Proposal 10: We propose to model EO as part of the background channel.

Proposal 11: The definition of the RCS of the target should be extended from the classic Radar literature definition to suit the ISAC needs [9].

Proposal 12: At 9 GHz, μRCS= -10 dBsm,  σRCS= f(nwings) for x-copter class UAVs, these parameters should be computed for other frequencies as well.

Proposal 13: The RCS for Flying Wing type UAVs can be approximated using sinusoid functions with perturbation as a function of shape, incident and scattering angles.

Proposal 14: Based on the measurements available, RCS can also be approximated using the model proposed for antenna radiation pattern in TR 38.901.

Proposal 15: The RCS scattering model for vehicle can be approximated using the antenna radiation model stated in TR 38.901 based on the RCS measurements available.

# 7. References

RP-240799, Revised SID: Study on channel modelling for Integrated Sensing And Communication (ISAC) for NR, Xiaomi, AT&T, 3GPP TSG RAN Meeting #103, March 2024

R1-2403995, Discussion on ISAC channel modeling, Nokia, 3GPP TSG RAN WG1 #117, Fukuoka, Japan, 20 – 24 May, 2024.

TR 38.901, Study on channel model for frequencies from 0.5 to 100 GHz, v17.0.0.

Chairman’s Notes RAN1#118, Maastricht, NL, August 19th – 23rd, 2024.

Speirs, P. J., et al. "High-detail simulations of consumer-grade UAV RCS signatures, and comparisons against measurements." STO-MP-MSG-SET-183. (2021).

Kapoulas, Ioannis K., et al. "Small fixed-wing UAV radar cross-section signature investigation and detection and classification of distance estimation using realistic parameters of a commercial anti-drone system." Drones 7.1 (2023): 39.

Kamel, Emna Bel, Alain Peden, and Patrice Pajusco. "RCS modeling and measurements for automotive radar applications in the W band." 2017 11th European Conference on Antennas and Propagation (EUCAP). IEEE, 2017.

Dogaru, Traian, Lam Nguyẽ̂n, and Calvin Le. Computer models of the human body signature for sensing through the wall radar applications. Triangle Park: Army Research Laboratory, 2007.

Knott, E., J. Shaeffer, and M. Tuley. "Radar Cross Section 2nd edn (Boston, MA: Artech House)." (1993).

Dogaru, Traian, Lam Nguyẽ̂n, and Calvin Le. Computer models of the human body signature for sensing through the wall radar applications. Triangle Park: Army Research Laboratory, 2007.

Russell, Daniel A. "Reflection of Waves from Boundaries". Graduate Program in Acoustics. Pennsylvania State University. Retrieved 2021-05-12.