3GPP TSG RAN WG1 #118bis			                                  R1-2408255

Hefei, China, October 14th – 18th, 2024

Source:	Sharp

Title:	Discussion on LP-WUS operation in CONNECTED modes

Agenda Item:	9.6.3

Document for:	Discussion and decision

Introduction

In RAN#102, WI on Low-power wake-up signal(LP-WUS) and receiver for NR was approved for UE power saving[1]. The objectives of the WI include specifying procedures to allow UE MR PDCCH monitoring triggered by LP-WUS and activation and deactivation procedures of LP-WUS monitoring. In RAN1#118, some issues on LP-WUS monitoring are discussed and agreed for further study. This contribution discusses further considerations on LP-WUS monitoring for UE in CONNECTED mode.

Discussion

In RAN1#118, the LP-WUS procedures for triggering PDCCH monitoring were discussed and converged to two options, and options for combining them with C-DRX were selected for further study. In RAN2#126, it is assumed that legacy DCP and Option 1-1 are not configured simultaneously for a UE.

Agreement For option 1-2 of LP-WUS CONNECTED mode operation, the followings are assumed from RAN1 perspective. - LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - UE is configured with legacy C-DRX configurations as Rel-18 - UE is expected to be configured with LP-WUS monitoring configuration (periodicity and offset can be different from those from C-DRX configuration) - FFS potential restriction for LP-WUS configuration in relation with C-DRX configuration - LP-WUS triggers the start of a timer during which UE monitors PDCCH - FFS the timer is existing timer and/or new timer - UE PDCCH monitoring behaviors related to other legacy DRX timers are not affected - drx-InactivityTimer, drx-RetransmissionTimerDL, drx-RetransmissionTimerUL, drx-HARQ-RTT-TimerDL, drx-HARQ-RTT-TimerUL - No impact on RRM/RLM/BFD measurement requirements is assumed - For periodic CSI/L1-RSRP reporting, UE can be configured with one of the following (same as Rel-16 DCP and option 1-1) - Periodic CSI/L1-RSRP is not reported if UE is not indicated to wake-up - Periodic CSI/L1-RSRP is periodically reported regardless if UE is indicated to wake-up or not - UE PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS Working Assumption From RAN1 perspective, for RRC CONNECTED mode, PDCCH monitoring is triggered by LP-WUS with C-DRX configuration - Support Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer. - Support Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - FFS whether/how to support both Option 1-1 and Option 1-2 simultaneously configured for the same UE Note: Above can be revisited considering RAN2 decisions.

According to the working assumption, both options will be supported. Option 1-1 can provide a wake-up indication in LP-WUS, replacing a similar function in DCP. Since LP-WUS receiver can tolerate greater synchronization shifts and lower power consumption than DCP, UEs can achieve more sleep time with LP-WUS signal monitoring, resulting in greater power savings. Additionally, LP-WUS monitoring in Option 1-1 operates outside of C-DRX timer, requiring less specification effort to adapt to C-DRX procedures.

In contrast, with Option 1-2, UE PDCCH monitoring is not triggered by the legacy C-DRX cycle and drx-OnDurationTimer while the UE is monitoring LP-WUS. More effort in RAN2 will be needed to adopt the new procedure for synchronizing the LP-WUS monitoring cycle and the DRX cycle.

Another question is whether these two options can be configured simultaneously. We do not see significant power-saving benefits from this combination. Furthermore, if the combination is allowed, further clarification is needed regarding how the UE should handle overlapping durations of the two options. For instance, does “outside of active time” include the duration of the OnDurationTimer for C-DRX triggered by Option 1-1?

Proposal 1: Does not support the simultaneous configuration of both options for one UE.

On the content of LP-WUS for UE in connected mode, bitmap or codepoint will be selected.

Agreement Regarding the LP-WUS information to trigger PDCCH monitoring of RRC connected UEs (for non-CA case), select at least one from the following - Option 1: A bitmap with each bit corresponding to [one or more] UEs - Option 3: A codepoint value corresponding to [one or more] UEs - FFS details for extension of option 1 and/or 3 when UE is configured with CA

Regarding the selection between the two options for connected UEs, it differs from the selection for idle UEs, where code points offer a better balance of overhead and paging probability. For connected UEs, there is a greater need for independent indications for each UE. In this context, using a bitmap has more advantages than relying on code point values.

Proposal 2: Support bitmap with each bit corresponding to one UE for UE in connected mode for non-CA case.

It is agreed that LP-WUS can be supported for the UE configured with CA in connected mode. and the detail for the indication needs further discussion.

Agreement Study whether/how LP-WUS works when UE is configured with CA in RRC CONNECTED mode - FFS: The cell(s) where PDCCH monitoring triggered by a LP-WUS is applicable - Option 1: one or more serving cells based on gNB indication/configuration - Option 2: all activated serving cells - Note: other options are not precluded Agreement LP-WUS is at least supported for the case where a UE is configured with CA in RRC CONNECTED mode - FFS: DC

firstly, similar to R16 DCP, the LP-WUS for connected UE configured with CA can be provided only in Pcell to carry indication for all SCells.

Furthermore, LP-WUS can employ a more flexible method to trigger PDCCH monitoring for serving cells ,for example, it can trigger PDCCH monitoring according to traffic model for each Scells respectively. Option 1 can be applied as providing indications for one or more serving cells based on gNB indications/configurations in LP-WUS. Typically, Option 2 can be implemented as a specific case of Option 1.

Proposal 3: Support to provide only LP-WUS in the Pcell to carry indication for all SCells in CA cases.

Proposal 4: Support Option 1, which can include indications for one or more serving cells seperately with an LP-WUS.

To enable fast SCell activation when carrier aggregation (CA) is configured, the dormancy indication in the DCI is used to trigger dormant BWP switching. Similar to the function of DCP, it can be discussed whether the dormancy indication should also be included in LP-WUS. Notably, the dormancy indication for UEs is included in DCI format 2_6 for DCP. Since RAN2 assumes that DCP and LP-WUS for Option 1 will not be configured simultaneously, the function of dormancy indication should also be considered in the LP-WUS design.

In addition to DCI format 2_6, dormant BWP switching can also be triggered by indications in DCI formats 0_1 and 1_1 for data scheduling. Therefore, including the dormancy indication in LP-WUS may not be obligatory.

Observation 1: DCI format 0_1/1_1 can also provide the function of dormancy indication. whether including dormancy indication in LP-WUS needs further study.

Proposal 5: It is not necessary to include dormancy indication in LP-WUS.

Regarding UE reporting for delay requirements, two alternatives are considered.

Agreement Select one of the following alternatives in RAN1#118bis Alt 1: For RRC CONNECTED mode, UE reports one value for each SCS from X candidate values for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting. - FFS: X - FFS: definition of reported value Alt2: For RRC CONNECTED mode, UE reports multiple values for each SCS from X candidate values for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting. - FFS: X - FFS: definition of reported value - Different minimum time gaps correspond to different sleep states - Companies are encouraged to details on how the reported values are to be used by the network Agreement For LP-WUS monitoring in RRC CONNECTED mode, SSB and/or CSI-RS can be the QCL source of LP-WUS - FFS applicable QCL type(s)

The only difference between Alternative 1 and Alternative 2 is the number of UEs reported for different sleep states. Alternative 2 may provide the gNB with greater flexibility to group UEs into a single LP-WUS, allowing UEs to adapt their sleep states according to the gNB’s configuration.

Proposal 6: Support Alternative 2, which allows UEs to report multiple values for each SCS.

Conclusion

In this contribution, we have the following observations and proposals:

Observation 1: DCI format 0_1/1_1 can also provide the function of dormancy indication. whether including dormancy indication in LP-WUS needs further study.

Proposal 1: Does not support the simultaneous configuration of both options for one UE.

Proposal 2: Support bitmap with each bit corresponding to one UE for UE in connected mode for non-CA case.

Proposal 3: Support to provide only LP-WUS in the Pcell to carry indication for all SCells in CA cases.

Proposal 4: Support Option 1, which can include indications for one or more serving cells seperately with an LP-WUS.

Proposal 5: It is not necessary to include dormancy indication in LP-WUS.

Proposal 6: Support Alternative 2, which allows UEs to report multiple values for each SCS.

References

[1] RP-234056, New WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)

[2] Chair notes RAN1#118