3GPP TSG RAN WG1 #119					       	R1-24010663

Orlando, US, November 18th – 22nd, 2024

Source:		Ad-Hoc Chair (NTT DOCOMO)

Title:			Session Notes for 8.2

Agenda Item:		8.2

Document for: 	Endorsement

- 8.2	Rel-18 UE Features

The maximum number of contribution per company/organization/university is limited to 1 per agenda item unless stated otherwise.

[119-R18-UE_features] Email discussion on Rel-18 UE features – Hiroki (DOCOMO)

- To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc

R1-2410661	Summary of discussion on Rel-18 NR UE features	Moderator (NTT DOCOMO, INC.)

### NR_MIMO_evo_DL_UL

Agreement:

Introduce following FG for UE STxMP processing capability for codebook.

- FG name

  - UE STxMP processing capability for codebook

- Component

  - 1. Require additional timeline to process multiple TBs for codebook multi-DCI based STx2P PUSCH+PUSCH for DG+DG

  - Note. This FG can be applied for CG+DG also if UE can support those FG.

- Prerequisite

  - 40-6-3a

- Consequence if not supported

  - UE should process multiple TBs within legacy timeline

- Type

  - Per FSPC

- Need of FR1/FR2 differentiation

  - FR2 only

- Note

  - candidate values:

  - UE reports candidate value independently for each SCS in unit of symbols

  - For 15kHz SCS: {1,2,4}

  - For 30kHz SCS: {1,2,4,8}

  - For 60kHz SCS: {2,4,8,16}

  - For 120kHz SCS: {4,8,16,32}

  - For 480kHz SCS: {16,32,64,128}

  - For 960kHz SCS: {32,64,128,256}

  - Note: SCS above is the minimum between SCS of the scheduling DCI and SCS of the scheduled PUSCH.

- Mandatory or optional

  - Optional with capability signaling

Agreement:

Introduce following FG for UE STxMP processing capability for non-codebook.

- FG name

  - UE STxMP processing capability for non-codebook

- Component

  - 1. Require additional timeline to process multiple TBs for noncodebook multi-DCI based STx2P PUSCH+PUSCH for DG+DG

  - Note. This FG can be applied for CG+DG also if UE can support those FG.

- Prerequisite

  - 40-6-3b

- Consequence if not supported

  - UE should process multiple TBs within legacy timeline

- Type

  - Per FSPC

- Need of FR1/FR2 differentiation

  - FR2 only

- Note

  - candidate values:

  - UE reports candidate value independently for each SCS in unit of symbols

  - For 15kHz SCS: {1,2,4}

  - For 30kHz SCS: {1,2,4,8}

  - For 60kHz SCS: {2,4,8,16}

  - For 120kHz SCS: {4,8,16,32}

  - For 480kHz SCS: {16,32,64,128}

  - For 960kHz SCS: {32,64,128,256}

  - Note: SCS above is the minimum between SCS of the scheduling DCI and SCS of the scheduled PUSCH.

- Mandatory or optional

  - Optional with capability signaling

### NR_Mob_enh2

Agreement:

Add following note for FG45-1a.

- Note: the SSBs of candidate cell(s) to be measured can be on any band where the UE supports LTM

### NR_pos_enh2

Agreement:

Reply to RAN2 as below.

- From RAN1 perspective, to make the additional SL-TDOA/SL-TOA measurement feature work,

  - following changes are recommended. It is up to RAN2 for final decision.

    - SL-TOA-MeasElementPerARP-ID-Rx ::= SEQUENCE (SIZE(1..416)) OF SL-TOA-MeasElement

    - SL-TDOA-MeasElementPerARP-ID-Rx ::= SEQUENCE (SIZE(1..416)) OF SL-TDOA-MeasElement

  - The signaling of requesting additional measurement is also needed to support above recommendation.

FFS: other reply LS contents

R1-2409392	Rel-18 UE Features	Huawei, HiSilicon

R1-2409579	UE features for Rel-18 work items	Samsung

R1-2410340	Rel-18 UE Features	Ericsson

R1-2410465	UE features for Rel-18	Qualcomm Incorporated