3GPP TSG RAN WG1 #120			R1- 2500167

Athens, Greece, February 17th – 21st, 2025

Agenda item:	9.3.2

Source: 	Spreadtrum, UNISOC

Title: 	Discussion on SBFD random access operation

Document for:	Discussion and decision

Introduction

In RAN#104, the revised WI on evolution of NR duplex operation was approved [1].

- Specify SBFD operation to support random access in SBFD symbols by UEs in RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode [RAN1, RAN2]

In this contribution, we discuss some technical issues related to random access in subband non-overlapping full duplex (SBFD), including both of RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode.

Discussion

RACH configuration Option 1

PRACH power control parameters

It was agreed to support separate power control parameter of preambleReceivedTargetPower for PRACH in SBFD symbols and non-SBFD symbols in RAN1#119.

Agreement RAN#119 For RACH configuration Option 1, for support of separate power control parameters for PRACH transmission in additional-ROs and legacy-ROs, support separate preambleReceivedTargetPower for additional-ROs. - FFS: powerRampingStep, preamble maximum output power PCMAX, preambleTransMax, powerRampingStepHighPriority

According to the FFS point, powerRampingStep can be separately configured. New powerRampingStep can provide different power boosting step for additional-ROs considering the different interference. It can assign bigger value to do faster power ramping.

For preamble maximum output power PCMAX, it needs more clarification. It seems it refers to PCMAX,f,c in the following. Then this configured maximum output power PCMAX,f,c is set to within a range [PCMAX_L,f,c , PCMAX_H,f,c]. So it is not a configured value, but a value decided by the following some aspects. It definitely cannot be provided as a separate parameter for additional-ROs.

38213 - PCMAX,f,c(i) is the UE configured maximum output power defined in [8-1, TS 38.101-1], [8-2, TS 38.101-2], [8-3, TS 38.101-3] and [8-5, TS 38.101-5] for carrier f of cell c within transmission occasion i,

38.101-1 6.2.4 Configured transmitted power The UE is allowed to set its configured maximum output power PCMAX,f,c for carrier f of serving cell c in each slot. The configured maximum output power PCMAX,f,c is set within the following bounds: PCMAX_L,f,c ≤ PCMAX,f,c ≤ PCMAX_H,f,c with PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c, (PPowerClass – ΔPPowerClass) – MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c + ∆TC,c + ∆TRxSRS, P-MPRc) } PCMAX_H,f,c = MIN {PEMAX,c, PPowerClass – ΔPPowerClass } where PEMAX,c is the value given by either the p-Max IE or the field additionalPmax of the NR-NS-PmaxList IE, whichever is applicable according to TS 38.331[7]; PPowerClass is the maximum UE power specified in Table 6.2.1-1 without taking into account the tolerance specified in the Table 6.2.1-1; …

preambleTransMax can be separately configured, too. Because RAN2 agreed that after certain (configured) number of times of RACH attempt in SBFD RACH occasions, UE is allowed to switch to legacy RACH occasions. So the preambleTransMax of additional-ROs is the maximum number of preamble transmission in additional ROs before it switches to legacy ROs. The parameter of additional ROs has different meaning from the legacy parameter.

- In addition to legacy power control parameters, two new parameters preambleTransMax and powerRampingStep for additional-ROs can be introduced.

RACH configuration Option 2

RACH configuration table

Agreement RAN#119 For RACH configuration Option 2, and for interpretation of the parameter prach-ConfigurationIndex provided by the additional RACH configuration, down-select from the following options: - Alt-1: introduce a subframe/slot offset (similar as prach-ConfigurationSOffset-IAB) to determine the subframe/slot number for ROs in SBFD symbols for FR1/FR2 - FFS: details of the subframe/slot offset - Alt-2: introduce a new parameter indicating from an entry from a pre-defined/configured table to derive additional subframe/slot numbers. - Details are FFS. - Alt-3: introduce a bitmap indicating or puncturing subframe(s)/slot(s) for additional ROs in SBFD symbols. - Details are FFS. - Alt-4: no enhancement

For this open issue, our view is not to introduce new parameters to determine the slot number for ROs in SBFD symbols, including the following reasons. PRACH configuration period can be 10/20/40/80/160ms, and SBFD period is the same as TDD-UL-DL pattern period or sum of the two TDD-UL-DL pattern periods configured by dl-UL-TransmissionPeriodicity in TDD-UL-DL-ConfigCommon, the candidate values are much smaller than PRACH period. It is signalling wasting to configure RO time domain location by bitmap, since PRACH configuration period is large. Another alternative is to configure time offset. However, the time offset is not a complete method. Because there can be several RO locations in a period, so how to determine the RO position still need more considerations, such as the every RO or each RO configured by RACH table index.

- For RACH configuration Option 2, using existing random access configurations table for unpaired spectrum in FR1 and FR2, without new parameter(s) to determine the slot number for additional ROs.

SSB-RO mapping rules

The case that additional ROs overlap with legacy ROs has been discussed for several meetings.

Agreement RAN#119 For RACH configuration Option 2, the association period and association pattern period of additional-ROs are determined separately from legacy-ROs. - The legacy rules for determining association period and association pattern period are reused in principle for additional-ROs. Agreement RAN#119 For RACH configuration Option 2, for the case that additional ROs overlap with legacy ROs, down-select from the following alternatives: - Alt-1: do not optimize this case - Alt-2: the additional-ROs are considered invalid when overlapping with legacy-ROs

For the FFS point, no additional operation is necessary to handle the overlapping between legacy RO and additional-ROs. Because the overlapping of ROs which are configured by different RACH configurations are legacy issues, and no special handling is applied. Second, it was agreed that the additional-ROs in non-SBFD symbols configured by additional RACH configuration are invalid for SBFD-aware UE. It can reduce the collision possibilities. So no further considerations are needed for the overlapping with legacy-ROs and additional-ROs.

- For RACH configuration Option 2, for the case that additional ROs overlap with legacy ROs, do not optimize this case.

PREAMBLE_RECEIVED_TARGET_POWER

When after certain (configured) number of times of RACH attempt in SBFD RACH occasions, UE switch to legacy RACH occasions. In this case, the PRACH target reception power can be similar as 2-step RACH fallback to 4-step RACH. So we have the following proposal for PREAMBLE_RECEIVED_TARGET_POWER. It can be further used for the power control of MSG3 PUSCH and MSG4 PUCCH.

- When after certain (configured) number of times of RACH attempt in SBFD RACH occasions, UE switch to legacy RACH occasions, the target power is:

- Power ramping counter should continue increasing after additional RO switches to legacy RO.

- The power ramping step size follows the legacy RO configuration after switching

- After switching, the target power level follow the 4-step RACH formula plus an offset, where the offset is

  - OFFSET = (N -1) × (PREAMBLE_POWER_RAMPING_STEP(additional-RO)  -  PREAMBLE_POWER_RAMPING_STEP(legacy-RO))

    - N is the power ramping counter when switching back to legacy RO

PRACH repetition

PRACH repetition is supportive for additional ROs, and it will not across additionals-ROs and legacy-ROs. Details of PRACH repetitions with RACH configuration Option 1 and 2 need more discussion.

Agreement RAN#119 Confirm the following working assumption Working Assumption For a PRACH transmission with Npreamblerep preamble repetition within additional-ROs, support reusing current specification rules for the determination of the set of Npreamblerep valid additional-ROs and the time period of the set(s) of Npreamblerep valid additional-ROs. - the time period of the set(s) of Npreamblerep valid additional-ROs is determined separately from the time period for legacy-ROs

RO group of additional-ROs configured by RACH configuration Option 1 with Alt 1-1

Agreement RAN#119 For RACH configuration Option 1, support separate configuration of rsrp-ThresholdMsg1-RepetitionNum2/4/8 for PRACH transmission with preamble repetitions within additional-ROs and PRACH transmission with preamble repetitions within legacy-ROs. - Same msg1-RepetitionNum is used for PRACH transmission with preamble repetitions within additional-ROs and PRACH transmission with preamble repetitions within legacy-ROs.

From the agreements/conclusions until now, RACH configuration Option 1 with Alt 1-1 only has different parameters for additional-RO, time and frequency resources for additional-ROs and legacy-ROs are determined by legacy parameters. From this point of view, all parameters related with RO resources do not have separate configuration, including time offset of RO group and repetition numbers. For additional-ROs configured by RACH configuration Option 1 with Alt 1-1, repetition number/time offset of RO group consisted with additional-ROs can reuse the legacy parameters RepetitionNum/msg1-RepetitionTimeOffsetROGroup-r18, without new configurations.

RO group of additional-ROs configured by RACH configuration Option 2

For additional-ROs configured by RACH configuration Option 2, separate parameters of repetition number/time offset/RSRP threshold of RO group can be configured, including time offset of RO groups.

Agreement RAN#119 For RACH configuration Option 2, support separate configuration of rsrp-ThresholdMsg1-RepetitionNum2/4/8 and msg1-RepetitionNum for PRACH transmission with preamble repetitions within additional-ROs and PRACH transmission with preamble repetitions within legacy-ROs. - msg1-RepetitionNum is configured in RACH-ConfigCommon in current specification, i.e., for RACH configuration Option 2, msg1-RepetitionNum can already be separately configured for legacy-ROs and additional-ROs.

- For additional-ROs configured by RACH configuration Option 1 with Alt 1-1,

- Time offset of RO group for additional-ROs can reuse the legacy parameters msg1-RepetitionTimeOffsetROGroup-r18, without new configurations.

- For RACH configuration Option 2, support separate configuration of msg1-RepetitionTimeOffsetROGroup-r18 for PRACH transmission with preamble repetitions within additional-ROs and PRACH transmission with preamble repetitions within legacy-ROs.

2-step RACH

It can support 2-step RACH for random access in SBFD symbols by UEs in RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode. First, 2-step RACH is a part of random access procedure. It would be good to have this feature to have a complete random access function. Second, RO of MSGA in 2-step RACH can share a common design as 4-step RACH. Only PO configuraition/validation/association are specific design. Thus, the specification impacts are affordable. Last, either 4-step RA or 2-step RA can be configured as contention free. If 2-step RACH is not supported for SBFD, a UE can only be configured with 4-step CFRA, although it in a good RSRP condition and be capable of 2-step RACH.

In order to have full random access procedure, we support 2-step RACH for random access in SBFD symbols for random access in SBFD symbols.

- It can support 2-step RACH for random access in SBFD symbols by UEs in RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode.

Msg3 enhancement

Power control

There was an initial agreement for MSG3 PUSCH power control. In this section, we give more considerations for MSG3 PUSCH power control.

Agreement RAN#119 Study how to determine the Msg3 PUSCH power control in SBFD symbols and non-SBFD symbols based on at least the following parameters. - preambleReceivedTargetPower, msg3-DeltaPreamble/deltaPreamble, TPC Command in RAR

38.213: PPUSCH,b,f,ci,j,qd,l=minPCMAX,f,ciPO_PUSCH,b,f,cj+10log102μ∙MRB,b,f,cPUSCHi+αb,f,cj∙PLb,f,cqd+∆TF,b,f,ci+fb,f,ci,l [dBm] where, … - PO_PUSCH,b,f,c(j) is a parameter composed of the sum of a component PO_NOMINAL,PUSCH,f,c(j) and a component PO_UE_PUSCH,b,f,c(j) where j∈0,1,…,J-1. - If a UE established dedicated RRC connection using a Type-1 random access procedure, as described in clause 8, and is not provided P0-PUSCH-AlphaSet or for a PUSCH (re)transmission corresponding to a RAR UL grant as described in clause 8.3, j=0, PO_UE_PUSCH,b,f,c0=0, and PO_NOMINAL,PUSCH,f,c0=PO_PRE+∆PREAMBLE,Msg3, where PO_PRE is provided by preambleReceivedTargetPower [11, TS 38.321] and ΔPREAMBLE_Msg3 is provided by msg3-DeltaPreamble or deltaPreamble, or ∆PREAMBLE,Msg3=0 dB if msg3-DeltaPreamble and deltaPreamble are not provided, for carrier f of serving cell c … - For αb,f,cj - For j=0, - if PO_NOMINAL_PUSCH,f,c(0)=PO_PRE+ΔMsgA_PUSCH and msgA-Alpha is provided, αb,f,c(0) is the value of msgA-Alpha - elseif PO_NOMINAL_PUSCH,f,c(0)=PO_PRE+ΔPREAMBLE_Msg3 or msgA-Alpha is not provided, and msg3-Alpha is provided, αb,f,c(0) is the value of msg3-Alpha - else, αb,f,c0=1 … - If the UE receives a random access response message in response to a PRACH transmission or a MsgA transmission on active UL BWP b of carrier f of serving cell c as described in clause 8 - fb,f,c0,l=∆Prampup,b,f,c+δmsg2,b,f,c, where l=0 and - δmsg2,b,f,c is a TPC command value indicated in a random access response grant of the random access response message corresponding to a PRACH transmission according to Type-1 random access procedure, or in a random access response grant of the random access response message corresponding to a MsgA transmission according to Type-2 random access procedure with RAR message(s) for fallbackRAR, on active UL BWP b of carrier f of serving cell c, and - and ∆Prampup_requested,b,f,c is provided by higher layers and corresponds to the total power ramp-up requested by higher layers [11, TS 38.321] for carrier f in the serving cell c, MRB,b,f,cPUSCH(0) is the bandwidth of the PUSCH resource assignment expressed in number of resource blocks for the first PUSCH transmission on active UL BWP b of carrier f of serving cell c, and ∆TF,b,f,c0 is the power adjustment of first PUSCH transmission on active UL BWP b of carrier f of serving cell c. …

If legacy RO is used for PRACH, the MSG3 can only be in non-SBFD symbols rather than in SBFD symbol. Since gNB cannot know whether the UE can or cannot support SBFD operation if legacy RO is used. Then PO_PRE and ΔPREAMBLE_Msg3 use the parameters of the legacy RO respectively.

- If legacy RO is used for PRACH, PO_PRE and ΔPREAMBLE_Msg3 of MSG3 PUSCH use the parameters of the legacy RO, preambleReceivedTargetPower and msg3-DeltaPreamble/deltaPreamble.

On the other hand, if additional RO is used, the MSG3 can be in either non-SBFD symbol or SBFD symbols. In this case, how to decide its PO_PRE and ΔPREAMBLE_Msg3 is still open, as shown case1/2 in the following table.

- Case 1: Additional RO for PRACH and MSG3 in non-SBFD symbols

- Case 2: Additional RO for PRACH and MSG3 in SBFD symbol.

Table  1:  MSG3 located in SBFD/non-SBFD symbols

| MSG1/MSG3 | Legacy RO | Additional RO |
| --- | --- | --- |
| MSG3 in non-SBFD symbol | preambleReceivedTargetPower msg3-DeltaPreamble/deltaPreamble | Case 1? |
| MSG3 in SBFD symbols | NA | Case 2? |

The  PO_PRE parameter of MSG3 PUSCH can be preambleReceivedTargetPower of additional-RO for both case 1 and case 2. Since their preambles are transmitted in additional-ROs. However, ΔPREAMBLE_Msg3 needs more discussion, whether one common offset or two separate offsets should be introduced. Our suggestion is to two different values can be configured for ΔPREAMBLE_Msg3, due to it can have separate power control for MSG3 in non-SBFD or SBFD symbol, based on single power of preamble.

- If additional-RO is used for PRACH, PO_PRE and ΔPREAMBLE_Msg3 of MSG3 PUSCH in SBFD symbols or non-SBFD symbol use two separate parameters of msg3-DeltaPreamble/deltaPreamble, new ones for SBFD symbol, the legacy ones for non-SBFD symbols.

For TPC command, it can reuse the legacy rule. It is still carried in RAR grant corresponding to a PRACH transmission either in SBFD symbol or non-SBFD symbol. No enhancement is needed.

Besides the three parameters in the agreement, the msg3-Alpha can be separate configured for SBFD or non-SBFD symbols. Legacy parameter is used for MSG3 in non-SBFD symbols, and one new can be provided for MSG3 PUSCH in SBFD symbol. Because separate power control of PUSCH was agreed in 9.3.1 that Alpha and P0 are introduced for SBFD symbol for PUSCH/PUCCH/SRS.

- New msg3-Alpha is introduced for SBFD symbols for MSG3 PUSCH.

MSG 3 transmission

Msg3 PUSCH resource allocation

For frequency resource allocation of MSG3, there are two cases (Case 1 and Case 2) in the spec related with initial UL BWP and active UL BWP.

38213, 8.3 PUSCH scheduled by RAR UL grant If useInterlacePUCCH-PUSCH is not provided by BWP-UplinkCommon and BWP-UplinkDedicated, for determining the frequency domain resource allocation for the PUSCH transmission within the active UL BWP - if the active UL BWP and the initial UL BWP have same SCS and same CP length and the active UL BWP includes all RBs of the initial UL BWP, or the active UL BWP is the initial UL BWP, the initial UL BWP is used - else, the RB numbering starts from the first RB of the active UL BWP and the maximum number of RBs for frequency domain resource allocation equals the number of RBs in the initial UL BWP

Similarly, MSG3 transmission in SBFD symbols should also consider the two cases. Moreover, Figure X provide an example for Case 1 and Case 2 separately.

- For Case 1 (if the active UL BWP and the initial UL BWP have same SCS and same CP length and the active UL BWP includes all RBs of the initial UL BWP, or the active UL BWP is the initial UL BWP), there is no issue for frequency resource allocation, legacy FDRA indication can be resued.

- For Case 2 (Else), UL usable PRBs are determined as intersection between cell-specific UL subband and the UL BWP in SBFD symbols, then the UL usable PRBs for MSG3 start from the first RB of UL usable PRBs in the active UL BWP. However, the schedulable PRBs starts from the first RB of the active UL BWP and the maximum number of RBs for frequency domain resource allocation equals the number of RBs in the initial UL BWP  for case 2. Then it is possible that UL usable RBs are not schedulable.

The following proposal was suggested in [2]. Alt 1 is preferred. Because alt 2 is unnecessary changed the bits length in FDRA field. It does not matter a larger range can be scheduled but will not allocated.

Updated proposal 3-1-2a: For determining the frequency domain resource allocation for Msg3 PUSCH transmission in SBFD symbols, consider the following alternatives: - Alt 1: the RB numbering starts from the first RB of the UL usable PRBs for Msg3 PUSCH transmission and the maximum number of RBs for frequency domain resource allocation equals the number of RBs in the initial UL BWP. - Alt 2: the RB numbering starts from the first RB of the UL usable PRBs for Msg3 PUSCH transmission and the maximum number of RBs for frequency domain resource allocation equals the min {size of UL usable PRBs, number of RBs in the initial UL BWP}.

Figure 2: FDRA for MSG3, Case 2

- For determining the frequency domain resource allocation for Msg3 PUSCH transmission in SBFD symbols, the RB numbering starts from the first RB of the UL usable PRBs and the maximum number of RBs for frequency domain resource allocation equals the number of RBs in the initial UL BWP

MSG 3 transmission with repetition

For MSG3 repetition with Configuration 1, the valid symbol type is determined based on the symbol type of the first transmission.

Agreement RAN#119 For an SBFD aware UE, if addition RO is selected for Msg1 transmission, only Configuration 1 is supported for Msg3 repetition Agreement RAN#119 For Msg3 PUSCH repetition Configuration 1, the valid symbol type is determined based on the symbol type of the first transmission occasion. - UE does not expect the first transmission occasion includes different symbol types (i.e., SBFD symbols and non-SBFD symbols) - FFS: Available slot counting on SBFD symbols

38213: For unpaired spectrum operation, the UE determines the NPUSCHrepeat slots as the first NPUSCHrepeat slots starting from slot n+k2+∆+2μ∙Kcell,offset where a repetition of the PUSCH transmission does not include a symbol indicated as downlink by tdd-UL-DL-ConfigurationCommon or indicated as a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst, and where Kcell,offset is provided by cellSpecificKoffset; otherwise, if not provided, Kcell,offset=0.

For SBFD-aware UE, based on the valid symbol type MSG3 PUSCH, UE postpones transmissions in the invalid symbol type. Available slot counting of MSG3 PUSCH repetition is always on by default, as same as legacy MSG3 PUSCH repetition always enable available slot counting.

- For MSG3 repetition with SBFD symbols, available slot counts only slots with SBFD symbols; for MSG3 repetition with non-SBFD symbols, available slot counts only slots with non-SBFD symbols

Conclusion

In this contribution, we made the following proposals.\

- In addition to legacy power control parameters, two new parameters preambleTransMax and powerRampingStep for additional-ROs can be introduced.

- For RACH configuration Option 2, using existing random access configurations table for unpaired spectrum in FR1 and FR2, without new parameter(s) to determine the slot number for additional ROs.

- For RACH configuration Option 2, for the case that additional ROs overlap with legacy ROs, do not optimize this case.

- When after certain (configured) number of times of RACH attempt in SBFD RACH occasions, UE switch to legacy RACH occasions, the target power is:

- Power ramping counter should continue increasing after additional RO switches to legacy RO.

- The power ramping step size follows the legacy RO configuration after switching

- After switching, the target power level follow the 4-step RACH formula plus an offset, where the offset is

  - OFFSET = (N -1) × (PREAMBLE_POWER_RAMPING_STEP(additional-RO)  -  PREAMBLE_POWER_RAMPING_STEP(legacy-RO))

    - N is the power ramping counter when switching back to legacy RO

- For additional-ROs configured by RACH configuration Option 1 with Alt 1-1,

- Time offset of RO group for additional-ROs can reuse the legacy parameters msg1-RepetitionTimeOffsetROGroup-r18, without new configurations.

- For RACH configuration Option 2, support separate configuration of msg1-RepetitionTimeOffsetROGroup-r18 for PRACH transmission with preamble repetitions within additional-ROs and PRACH transmission with preamble repetitions within legacy-ROs.

- It can support 2-step RACH for random access in SBFD symbols by UEs in RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode.

- If legacy RO is used for PRACH, PO_PRE and ΔPREAMBLE_Msg3 of MSG3 PUSCH use the parameters of the legacy RO, preambleReceivedTargetPower and msg3-DeltaPreamble/deltaPreamble.

- If additional-RO is used for PRACH, PO_PRE and ΔPREAMBLE_Msg3 of MSG3 PUSCH in SBFD symbols or non-SBFD symbol use two separate parameters of msg3-DeltaPreamble/deltaPreamble, new ones for SBFD symbol, the legacy ones for non-SBFD symbols.

- New msg3-Alpha is introduced for SBFD symbols for MSG3 PUSCH.

- For determining the frequency domain resource allocation for Msg3 PUSCH transmission in SBFD symbols, the RB numbering starts from the first RB of the UL usable PRBs and the maximum number of RBs for frequency domain resource allocation equals the number of RBs in the initial UL BWP

- For MSG3 repetition with SBFD symbols, available slot counts only slots with SBFD symbols; for MSG3 repetition with non-SBFD symbols, available slot counts only slots with non-SBFD symbols

References

- RP-2401614	Revised WI on evolution of NR duplex operation

- R1-2410676	Summary#3 on SBFD random access operation	Moderator (CMCC)