3GPP TSG RAN WG1 #120	                                       	 R1-2500180

Athens, Greece, February 17th – 21st, 2025

Agenda Item:     9.9.1

Source:	Spreadtrum, UNISOC

Title:	             Discussion on measurements related enhancements for LTM

Document for:	Discussion and decision

Introduction

The revised work item for Rel-19 NR mobility enhancements Phase 4  in RAN#105 [1] includes the objectives as follows. In this contribution, we will share our views on the issues for measurements related enhancements for purpose of supporting LTM, which include the event triggered L1 measurement reporting and CSI-RS measurements for LTM procedures.

- Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support - Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1] - Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1] NOTE: RAN1 WG to decide on whether to support CSI report before or during the LTM cell switch, as part of the CSI acquisition procedure. - Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint

Discussion on CSI-RS measurement for LTM

Agreement - Explicit configuration of CSI-RS resource(s) for candidate cell(s) for L1-measurement is supported Agreement For gNB scheduled reporting and event triggered reporting - At least periodic CSI-RS is supported for L1-RSRP measurement for candidate cell - FFS: aperiodic and semi-persistent CSI-RS - At least CSI-RS for beam management is supported for L1-RSRP measurement for candidate cell - FFS: CSI-RS for mobility Agreement From RAN1 perspective, there is no restriction with regards to the frequency location of CSI-RS used for L1-measurement Conclusion There is no consensus in RAN1 on the support L1-SINR measurement based on CSI-RS for candidate cells Working Assumption In addition to periodic CSI-RS, semi-persistent CSI-RS is supported for candidate cell L1-RSRP measurement for gNB scheduled reporting from RAN1 perspective - Send an LS to RAN3 (CC RAN2) to ask for the feasibility of specifying the signalling for coordination between serving cell and candidate cell(s) on the transmission of semi-persistent CSI-RS(s) and any other potential issues (e.g. RAN3 workload). Support of semi-persistent CSI-RS is subject to UE capability. Final LS in R1-2409283. Agreement The agreement “Rel-18 LTM CSI reporting framework is the baseline for CSI-RS based L1-measurement report by gNB scheduled measurement reporting” made in RAN#118 is further clarified for L1-RSRP as follows: - UCI format defined in Table 6.3.1.1.2-8C of TS38.212 can be used by replacing SSBRI with CRI. - Whether the L1-RSRP(s) of serving cell is always included is configurable (in line with Rel-18) - The quantization method defined in clause 5.2.1.4.3 of TS38.214 and bit width defined in Table 6.3.1.1.2-6 of TS38.212 can be used - No L1 specified filtering for time and spatial domain is introduced - No enhancement on how to report L cells x M beams - Periodic reporting on PUCCH is supported - FFS: semi-persistent reporting on PUCCH/PUSCH, and aperiodic reporting on PUSCH Agreement For CSI-RS based L1-measurement report by gNB scheduled measurement reporting, semi-persistent reporting on PUCCH/PUSCH and aperiodic reporting on PUSCH are supported. Agreement For the identification of the serving cell RS for event evaluation, If the RS(s) for candidate cell(s) are CSI-RS configured in a CSI-RS resource set configured with repetition, QCL RS of the indicated TCI-state is used for the serving cell; otherwise, SSB QCLed with QCL RS of the indicated TCI-state is used for the serving cell. - UE does not expect the following configuration: - CSI-RS resource in the indicated TCI state of serving cell is NOT configured in a CSI-RS resource set configured with repetition, and - CSI-RS is configured as measurement resource for the candidate cell(s). Agreement CSI-RS for BM as the referenceSignal with QCL-TypeD is supported for an LTM TCI state, where TRS is configured as referenceSignal with QCL-TypeA Conclusion - No consensus to support CSI-RS for mobility for L1 measurement in Rel-19 LTM - Note: From the actual gNB transmission viewpoint, CSI-RS for mobility and CSI-RS for BM may be the same

In Rel-18, LTM was introduced to improve handover latency and interruption time compared to Layer 3 based mobility. To further enhance LTM procedure, the measurement related enhancements will be discussed in Rel-19, which include the event triggered L1 measurement reporting and CSI-RS based measurements. Only SSB based L1 measurement is supported in Rel-18 LTM procedure and limit throughput on the target cell immediately after cell switch due to coarse beam measurement. CSI-RS based measurement can achieve more accurate beam alignment than SSB and the time-frequency resource configuration for CSI-RS is more flexible.

For the time domain behavior of CSI-RS, we think that long-term L1 measurement for cell level are necessary to obtain more accurate measurement results for candidate cells. Then the gNB can decide to configure UE reporting in a periodic/semi-persistent/aperiodic manner. In RAN1#118b meeting, the working assumption was made to support SP-CSI-RS for candidate cell L1-RSRP measurement and an LS was sent to RAN3 to ask the feasibility. In this RAN1 meeting, the reply from RAN3 is received, where RAN3 confirmed it is feasible to specify the signaling for coordination between serving cell and candidate cell(s) on the transmission of semi-persistent CSI-RS(s), and RAN3 will continue to work on the details. Based on the RAN3 reply and the potential benefit to avoid some unnecessary measurement, we can accept to confirm the working assumption. As for AP-CSI-RS, considering the more frequent coordination between serving cell and candidate cells and complex spec design, e.g. triggering signaling design and relaxation for measurement latency, there is no clear benefit to introduce AP-CSI-RS for candidate cells.

Proposal 1:

- In Rel-19 LTM for gNB scheduled reporting, for time domain property of CSI-RS,

  - for SP-CSI-RS, confirm the following working assumption

Working Assumption In addition to periodic CSI-RS, semi-persistent CSI-RS is supported for candidate cell L1-RSRP measurement for gNB scheduled reporting from RAN1 perspective Support of semi-persistent CSI-RS is subject to UE capability.

  - do not support AP-CSI-RS for beam management.

From the perspective of flexibility and backward compatibility of L1 measurement configuration for candidate cells, in Rel-18 SSB configuration and candidate cell configuration are in the LTM-Config IE. For SSB configuration, SSB resource set including SSB resource list and corresponding candidate cell list is configured for one or more LTM candidate configuration.

In Rel-19, CSI-RS for LTM measurements can be explicitly configured under serving cell or separately from serving cell like SSB. Considering compatibility with Rel-18 signaling framework, the similar configuration design as SSB can be used for CSI-RS. For example, CSI-RS resource is configured under candidate cell configuration, and the CSI-RS resource set including CSI-RS resource list is introduced under Rel-19 LTM CSI resource configuration, where the corresponding candidate cell for every CSI-RS resource is configured in LTM candidate cell list.

In TS 38.214, for a CSI-RS resource for beam management, the QCL-TypeD RS can be a TRS, a CSI-RS for beam management or an SSB. In Rel-19 LTM measurements, this QCL chain should be maintained. Regardless of the type of the QCL source RS, the root SSB for the CSI-RS resource or root SSB for the QCL source RS of the CSI-RS resource can be from a candidate cell. That is to say, the candidate cell corresponding to CSI-RS resource can be derived based on the correspondence between root SSB and the candidate cell. In addition, the configuration similar to CSI-RS for mobility can also be used to determine the corresponding candidate cell, e.g. associated SSB from candidate cell is configured under the CSI-RS resource. Anyway, the detailed signaling design for CSI-RS configuration can be left to RAN2.

Proposal 2:

- Support CSI-RS configuration similar to SSB, i.e. CSI-RS resource set including CSI-RS resource list is configured under Rel-19 LTM CSI resource configuration.

Proposal 3:

- The corresponding relationship between the candidate cell and the CSI-RS resource can be determined based on the correspondence between the candidate cell and root SSB for the CSI-RS resource or root SSB for QCL source RS of the CSI-RS resource.

Discussion on event triggered L1 measurement reporting

Agreement - SSB based L1-RSRP measurements is supported for event triggered reporting - CSI-RS based L1-RSRP measurements is supported for event triggered reporting - FFS: CSI-RS based L1-SINR measurements is supported for event triggered reporting Agreement For gNB scheduled reporting and event triggered reporting - At least periodic CSI-RS is supported for L1-RSRP measurement for candidate cell - FFS: aperiodic and semi-persistent CSI-RS - At least CSI-RS for beam management is supported for L1-RSRP measurement for candidate cell - FFS: CSI-RS for mobility Agreement - For the identification of the serving cell RS for event evaluation, - At least the following options are further studied in RAN1, where different options could apply to different LTM event - Option. 1: Derived from QCL (type-D) RS(s) of the indicated joint/DL TCI state for the serving cell - Option. 2: Derived from QCL RS(s) or SSB QCLed with the QCL RS of the indicated joint/DL TCI state for the serving cell - QCL RS or SSB is configured by the network - Option. 3: Measurement RS(s) is/are explicitly configured - Option. 4: Derived from QCL RSs of activated TCI states with the best quality, or SSB which is QCLed with the QCL RSs of activated TCI states with the best quality. - Option 6: Derived from QCL RSs of activated TCI states, or SSB which is QCLed with the QCL RSs of activated TCI states - The RSs of the candidate cell(s) for event evaluation are explicitly configure - Note: Companies are encouraged to take into account the RAN2 agreement (i.e current beam rather than best beam) for their further study. Agreement The serving cell RS for event evaluation is at least derived from QCL RS or SSB QCLed with the QCL RS of the indicated joint/DL TCI state for the serving cell - QCL RS above is the RS w.r.t. QCL-TypeD when the indicated joint/DL TCI state is configured with two QCL RSs - FFS: Details on determination of QCL RS or SSB QCLed with QCL RS Note: This does not imply the support of mTRP scenarios

For event triggered LTM reporting, some LTM events were already defined based on beam specific quality of serving cell and candidate cells, i.e. LTM2/3/4/5. The QCL RS or SSB QCLed with the QCL RS of the indicated TCI state for serving cell is used as the serving cell RS for event evaluation. According to RAN2 agreements, either CSI-RS or SSB could be configured as candidate beam, the measurement RS of the serving cell beam is determined based on the candidate beam to ensure same RS type. A MAC CE is used as the report container to carry the triggered event information, IDs and quantities of up to N beams (N is configurable by NW, NW controls if the beam(s) not satisfying the event could be reported), the information and quantity of current beam based on NW configuration.

For the aspects of time domain behavior of CSI-RS, the activation/deactivation mechanism of LTM events and evaluation needs to be considered, otherwise the gNB does not know when the SP-CSI-RS should be activated. In addition, AP-CSI-RS is not conducive to continuous monitoring of the LTM event. So it seems the P-CSI-RS is sufficient.

In Rel-18 the unified TCI framework was used for LTM beam indication mechanism and the Rel-15/16 TCI framework was precluded. We think the principle for TCI framework in Rel-18 can be reused in Rel-19 LTM.

Proposal 4:

- Do not support SP-CSI-RS and AP-CSI-RS for beam management for event evaluation.

Proposal 5:

- Only support unified TCI framework for beam indication mechanism in Rel-19 LTM.

Discussion on CSI acquisition for candidate cell(s)

Agreement The following alternatives are further studied: - Alt-1: CSI-RS measurement and CSI reporting operations are performed before reception of LTM Cell Switch Command (CSC) MAC CE. - The report is sent to the serving cell and transferred to the candidate/target cell(s) - Alt-2: CSI-RS measurement can start before reception of LTM CSC MAC CE and CSI reporting operation is performed after reception of LTM CSC MAC CE. - The report is sent directly to target cell - Alt-3: CSI-RS measurement and CSI reporting operations are performed after reception of LTM CSC MAC CE. - The report is sent directly to target cell Companies are requested to provide the details of exact report timing and triggering mechanism in the next meeting Working Assumption As baseline, CSI-RS measurement and CSI reporting operations are performed after reception of LTM CSC MAC CE. - The report is sent directly to target cell - Introduce UE capability for CSI-RS measurement can start before reception of LTM CSC MAC CE - Other than UE capability, strive for no additional spec impact compared to the baseline (only one triggering mechanism will be specified)

In RAN#105 meeting, the work item for Rel-19 NR mobility enhancements Phase 4 was revised which include CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch. The motivation for CSI acquisition is to obtain CSI of target cell and improve throughput on the target cell immediately after cell switch.

In RAN1#118b meeting, there were three alternatives to be studied for CSI acquisition for candidate cell(s). For Alt-1, the CSI measurement and reporting before reception of LTM CSC not only helps improve throughput on the target cell immediately after cell switch, but also assists the gNB in combining LTM beam reporting to determine the target cell. The issues of both Alt-1 and Alt-3 are reflected in Alt-2, e.g. more UE capability to measure multiple candidate cells, outdated CSI, reporting timeline, etc. In RAN1#119 meeting, the working assumption was made, i.e. the Alt-3 is the baseline for CSI acquisition. Although the complex spec design is needed, for progress on the CSI acquisition, we can accept Alt-3 and confirm the working assumption.

Proposal 6:

- Support confirming the working assumption as follows

Working Assumption

As baseline, CSI-RS measurement and CSI reporting operations are performed after reception of LTM CSC MAC CE.

- The report is sent directly to target cell

- Introduce UE capability for CSI-RS measurement can start before reception of LTM CSC MAC CE

  - Other than UE capability, strive for no additional spec impact compared to the baseline (only one triggering mechanism will be specified)

To distinguish LTM CSI report from LTM beam report, RRC parameter reportQuantity needs to be used in LTM-CSI-ReportConfig to indicate beam report or CSI report.

Proposal 7:

- RRC parameter reportQuantity in LTM-CSI-ReportConfig is used to indicate LTM beam report or LTM CSI report.

Regarding CSI acquisition during the LTM cell switch, the most direct way is to trigger CSI-RS measurement and CSI reporting using LTM CSC MAC CE. In addition, the RAR UL grant for RACH-based LTM can be also considered. For early CSI acquisition and interruption time reduction, a field can be introduced in LTM CSC MAC CE to trigger the measurement and reporting, where a pre-configured CSI report configuration can be indicated. At least the AP-CSI reporting can be supported to associate with a CSI-RS resource set including aperiodic CSI-RS. It is unnecessary to associate with periodic CSI-RS and semi-persistent CSI-RS to avoid the spec design on the CSI-RS configuration or activation and to reduce the UE complexity and measurement overhead. Regarding the CSI-RS beam, since the TCI state is indicated by LTM CSC MAC CE, the indicated TCI state for target cell can be applied to the CSI-RS for measurement. Regarding the uplink resource carrying CSI report in target cell, if the UL resource scheduled by target cell after cell switching is used to carry CSI report, the reporting latency is increased. For early CSI reporting reception at the target cell, the CSI reporting can be included on the Msg3 for RACH-based LTM or the first UL channel to the target cell for RACH-less LTM.

Proposal 8:

- Regarding CSI acquisition after reception of LTM CSC MAC CE, support triggering CSI-RS measurement and CSI reporting by LTM CSC MAC CE.

  - MAC CE format design up to RAN2.

Proposal 9:

- At least support AP-CSI reporting associated with the aperiodic CSI-RS triggered by LTM CSC MAC CE.

Proposal 10:

- The indicated TCI state for target cell in LTM CSC MAC CE is applied for CSI-RS transmission.

Proposal 11:

- The CSI reporting can be included on the Msg3 for RACH-based LTM or the first UL channel for RACH-less LTM.

For CSI-RS measurement before reception of LTM CSC MAC CE, the power assumption and complexity at UE are increased to estimate the channel and report PMI/CQI/RI, especially for multiple candidate cells measurement. Even if only the CSI of the target cell needs to be measured in Alt-3, the measurement overhead may be still significant for the UE with continuous motion and frequent cell switching. From the perspective of UE complexity reduction and power saving, some configuration for CSI acquisition can be limited. For example, only Type I codebook is allowed to be configured, the number of CSI-RS ports per CSI-RS resource shall not exceed 32, the number of candidate cells to be measured t not exceeds N (e.g. 1 or 2) for CSI-RS measurement before reception of LTM CSC MAC CE, etc.

Proposal 12:

- For further UE complexity reduction and power saving, some configuration for CSI acquisition should be limited, e.g.

  - only Type I codebook is configured,

  - the number of CSI-RS ports per CSI-RS resource not exceeds 32,

  - the number of candidate cells to be measured not exceeds N (e.g. 1 or 2) for CSI-RS measurement before reception of LTM CSC MAC CE.

Conclusion

In this contribution, we provide our proposals on Rel-19 NR mobility enhancements Phase 4:

Proposal 1:

- In Rel-19 LTM for gNB scheduled reporting, for time domain property of CSI-RS,

  - for SP-CSI-RS, confirm the following working assumption

Working Assumption In addition to periodic CSI-RS, semi-persistent CSI-RS is supported for candidate cell L1-RSRP measurement for gNB scheduled reporting from RAN1 perspective Support of semi-persistent CSI-RS is subject to UE capability.

  - do not support AP-CSI-RS for beam management.

Proposal 2:

- Support CSI-RS configuration similar to SSB, i.e. CSI-RS resource set including CSI-RS resource list is configured under Rel-19 LTM CSI resource configuration.

Proposal 3:

- The corresponding relationship between the candidate cell and the CSI-RS resource can be determined based on the correspondence between the candidate cell and root SSB for the CSI-RS resource or root SSB for QCL source RS of the CSI-RS resource.

Proposal 4:

- Do not support SP-CSI-RS and AP-CSI-RS for beam management for event evaluation.

Proposal 5:

- Only support unified TCI framework for beam indication mechanism in Rel-19 LTM.

Proposal 6:

- Support confirming the working assumption as follows

Working Assumption

As baseline, CSI-RS measurement and CSI reporting operations are performed after reception of LTM CSC MAC CE.

- The report is sent directly to target cell

- Introduce UE capability for CSI-RS measurement can start before reception of LTM CSC MAC CE

  - Other than UE capability, strive for no additional spec impact compared to the baseline (only one triggering mechanism will be specified)

Proposal 7:

- RRC parameter reportQuantity in LTM-CSI-ReportConfig is used to indicate LTM beam report or LTM CSI report.

Proposal 8:

- Regarding CSI acquisition after reception of LTM CSC MAC CE, support triggering CSI-RS measurement and CSI reporting by LTM CSC MAC CE.

  - MAC CE format design up to RAN2.

Proposal 9:

- At least support AP-CSI reporting associated with the aperiodic CSI-RS triggered by LTM CSC MAC CE.

Proposal 10:

- The indicated TCI state for target cell in LTM CSC MAC CE is applied for CSI-RS transmission.

Proposal 11:

- The CSI reporting can be included on the Msg3 for RACH-based LTM or the first UL channel for RACH-less LTM.

Proposal 12:

- For further UE complexity reduction and power saving, some configuration for CSI acquisition should be limited, e.g.

  - only Type I codebook is configured,

  - the number of CSI-RS ports per CSI-RS resource not exceeds 32,

  - the number of candidate cells to be measured not exceeds N (e.g. 1 or 2) for CSI-RS measurement before reception of LTM CSC MAC CE .

References

[1] RP-242356, Revised Work Item: NR mobility enhancements Phase 4.