3GPP TSG RAN WG1 #120	R1-2500293

Athens, Greece, February 17th–February 21th, 2025

Source: 	CMCC

Title:	Discussion on adaptation of common signal/channel transmissions

Agenda item:	9.5.3

Document for:	Discussion/Decision

- Introduction

In RAN#102 meeting, a new WID on enhancements of network energy saving has been approved [1], one of the objectives is to adapt the common signal/channel transmission, the scope of this objective has been discussed and revised in RAN#104 [2], the revised objective is as following,

3. Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown

In this contribution, we give our considerations on SSB adaptation in time domain, PRACH adaptation in time domain and Paging adaptation, and give our proposals.

- Discussion on SSB adaptation in time domain

For discussion on SSB adaptation, the following agreements are agreed during RAN1#119 [3] and RAN1#118bis [4],

Agreement in RAN1#118bis: Reply to Q1(What is the relation in terms of time location before and after SSB adaptation?): - RAN1 agreed that at least SSB burst periodicity is adapted. - There are no RAN1 agreements to adapt the time location of the SSB burst other than the periodicity but RAN1 is still discussing other options. Agreement in RAN1#118bis: Reply to Q2(What is the relation in terms of frequency location before and after SSB adaptation?): - The frequency location is same before and after SSB adaptation. Agreement in RAN1#118bis: Reply to Q3(What is the spatial relation before and after SSB adaptation?): - There is no change to spatial relation (in terms of QCL assumption) for the same SSB index before and after SSB adaptation. - At least the case where there is no change to actually transmitted SSBs within a burst before and after SSB adaptation is supported. Further update to be made based on RAN1#119 progress if any. Conclusion in RAN1#119: There is no RAN1 consensus to support SSB adaptation in time domain for Rel-19 NES-capable UE’s PCell (connected mode) Conclusion in RAN1#118bis: There is no consensus on the support of adaptation of SSB for idle mode UEs in Rel-19 Agreement in RAN1#118bis: For Cell DTX extension to SSBs not on sync-raster for connected mode UEs, select Option 3, i.e. Cell DTX does not impact UE assumption on SSB transmissions (i.e. legacy behavior). - No spec impact

In this section, we discuss the applicable scenario for SSB adaptation in time domain.

During RAN1#116bis-119 meeting, companies have a heated discussion with the specific scenario for SSB adaptation. Despite some progress, the issue on applicable scenario has not been fully addressed yet. Thus, we discuss from some general concepts first.

The first issue is whether SSB adaptation can only applicable for NCD-SSB/SSB that is not a CD-SSB while CD-SSB is always transmitted in a normal periodicity. To solve this, we perform an evaluation on NES gain for this mechanism. The settings for baseline and SSB adaptation mode are shown as below:

- Baseline:

- Both CD-SSB and corresponding SIB1 are transmitted per 20ms

- PRACH is transmitted periodically with configuration index 5 and the frequency bandwidth is 12 RB.

- NCD-SSB is transmitted per 20 ms, NCD-SSB is FDM transmitted with CD-SSB.

- NCD-SSB adaptation mode:

- Both CD-SSB and corresponding SIB1 are transmitted per 20ms

- PRACH is transmitted periodically with configuration index 5 and the frequency bandwidth is 12 RB.

- NCD-SSB is transmitted per 160 ms, NCD-SSB is FDM transmitted with CD-SSB.

- Note that the assumption for cell load is empty load, and other parameters come from SLS parameter settings in [6]. Figure 1 and Figure 2 show an example for CD-SSB/SIB1/PRACH transmission pattern in TDD and FDD structure, respectively.

Figure 1. An example for CD-SSB/SIB1/PRACH transmission pattern for baseline in TDD structure

Figure 2. An example for CD-SSB/SIB1/PRACH transmission pattern for baseline in FDD structure

For NES gain on NCD-SSB adaptation, Table 1 shows the evaluation result for different BS type in [6]. It can be observed that only 0.7%~2.7% NES gain can be obtained for Cell A in various BS types and TDD/FDD structures. Since the benefit is relatively limited, we do not support SSB adaptation is only applicable for NCD-SSB/not CD-SSB while CD-SSB is always transmitted in normal periodicity.

Table 1. NES gain on NCD-SSB adaptation for different BS type in [6]

| BS category and structure | Cat 1 BS, TDD | Cat 1 BS, FDD | Cat 2 BS, TDD | Cat 2 BS, FDD |
| --- | --- | --- | --- | --- |
| NES gain (%) | 0.7% | 1.6% | 0.7% | 2.7% |

Observation 1: For SSB adaptation that is only applicable for NCD-SSB while CD-SSB is always transmitted in normal periodicity, 0.7%~2.7% NES gain can be obtained in various BS types and TDD/FDD structures.

- Note that the assumption for cell load is empty load.

Proposal 1: Do not support SSB adaptation that is only applicable for NCD-SSB/not CD-SSB while CD-SSB is always transmitted in normal periodicity.

Another issue worth noting is that although the current discussion makes a distinction on cell properties for R19 NES-capable UE, the corresponding cell may also be considered as a PCell, SCell or camped cell for another UE. Therefore, since it is already agreed that CD-SSB adaptation (i.e. the periodicity of CD-SSB is larger than 20ms) on PCell is not supported, only the following scenario can be applicable for SSB adaptation:

- Scenario 1: SSB adaptation for a cell that only have NCD-SSB/not CD-SSB (e.g., SSB not on sync raster)

- Note that the cell can only be a SCell for both R19 NES-capable UE and other UE.

Proposal 2: Support RAN1 further discuss SSB adaptation under the following scenario:

- SSB adaptation for the cell that only have NCD-SSB/not CD-SSB (e.g. SSB not on sync raster)

- Note that the cell can only be a SCell for both R19 NES-capable UE and other UE.

Furthermore, we discuss the specific mechanism for SSB adaptation on SCell. Since SCell can be activated/deactivated when needed, the SSB adaptation scheme should be considered together with these procedures. Meanwhile, the current discussion in AI 9.5.1 is focus on SSB transmission enhancement on SCell. Therefore, we suggest to discuss the design of specific mechanism under AI 9.5.1.

Proposal 3: For SSB adaptation on SCell, support to discuss the design of specific mechanism under AI 9.5.1.

- Discussion on PRACH adaption in time domain

For PRACH adaptation, the following agreements are agreed during RAN1#118-119 meeting [3-5].

Agreement in RAN1#119: At least msg1-FrequencyStart can be configured separately for the additional PRACH resources at least for 4-step RACH. Agreement in RAN1#119: For DCI-based adaptation for additional PRACH resources, select only from the following alternatives: - Alt 1: DCI-based adaptation to indicate whether the additional PRACH resources provided by semi-static signalling are available or not - [DCI payload size = 1 bit] - FFS: A single PRACH mask provided by semi-static signalling is used to identify the subset of the additional PRACH resources - FFS: details Agreement in RAN1#119: Confirm the following working assumption from RAN1#118bis. Working Assumption For DCI-based adaptation for additional PRACH resources, at least DCI format 1_0 can carry the adaptation indication for UEs in idle/inactive and connected mode. - P-RNTI is used Agreement in RAN1#118bis: For adaptation of PRACH in time-domain, for determining the additional PRACH resources in time-domain, - For Alt 1 (same PRACH configuration index for additional and legacy PRACH resources), select one or more of the following additional mechanism(s), - Opt 1-2a: up to N1 additional value(s) of (x, y) - FFS: value of N1(>=1) - Opt 1-2b: up to N2 additional value(s) of y - FFS: value of N2 (>=1) - Opt 1-3: Muting/masking ROs - Opt 1-4: - up to N4_1 additional timing offset(s) at frame-level - up to N4_2 additional timing offset(s) at slot-level - FFS: values of N4_1(>=1) and N4_2(>=1) - Opt 1-5: No additional mechanism is selected. - Note: x and y refer to the parameters from the random-access configuration tables (from 38.211) - For Alt 2 (different PRACH configuration index for additional and legacy PRACH resources), select one or more of the following additional mechanism(s), - Opt 1-2a: up to N1 additional value(s) of (x, y) - FFS: value of N1(>=1) - Opt 1-2b: up to N2 additional value(s) of y - FFS: value of N2 (>=1) - Opt 1-3: Muting/masking ROs - Opt 1-4: - up to N4_1 additional timing offset(s) at frame-level - up to N4_2 additional timing offset(s) at slot-level - FFS: values of N4_1(>=1) and N4_2(>=1) - Opt 1-5: No additional mechanism is selected. - Note: x and y refer to the parameters from the random-access configuration tables (from 38.211) Agreement in RAN1#118: Extend the RAN1#117 agreement on SSB-RO mapping rule for additional PRACH resources to Case 1 - Case 1: no time-domain overlap between the additional PRACH resources for NES-capable UEs and the PRACH resources for legacy UEs RAN1#117 Agreement At least for the case where legacy ROs and additional ROs overlap in neither time nor frequency domain, for adaptation of PRACH in time-domain, the SSB-RO mapping rule for additional PRACH resources follows the legacy SSB-RO mapping rule. - Mapping SS/PBCH block indexes to valid additional PRACH occasions provided by semi-static signalling follows the legacy mapping order for preamble/time resource/frequency/PRACH slot indexes. - Note: This mapping is not impacted by time domain PRACH adaptation - Validation rules for the additional PRACH resources follow the legacy validation rules for PRACH resources configured for legacy UEs. Agreement in RAN1#118: For SSB-RO mapping rule for additional PRACH resources for Case 2. - Extend the RAN1#117 and RAN1#118 agreements on SSB-RO mapping

In this section, we discuss the specific mechanism and overlapping issue for PRACH adaptation.

  - Discussion on specific mechanism for PRACH adaptation

In RAN1#118bis meeting, it is agreed that the additional PRACH resource can be configured based on the same or different PRACH configuration index, and several potential options are raised for down-selection. In this subsection, we discuss the specific mechanism for PRACH adaptation. The options and our preference are as follows:

- Option 1: The PRACH configuration index for additional PRACH resource is the PRACH configuration index for the legacy resource. Under this option, the following solutions can be considered:

- Option 1.1: Introducing a time offset/scaling factor (e.g. periodicity), or adjusting the parameter (x, y) under the existing PRACH configuration index.

- Option 1.2: Introduce a muting pattern/enhanced cell DRX pattern (for RRC_CONNECTED UE) under the existing PRACH configuration index.

- Regarding the two options, option 1.2 needs gNB to configure both additional PRACH configuration and muting pattern/enhanced cell DRX pattern configuration. Compared with option 1.1, this introduce more RRC signaling overhead without obvious benefits on NES gain. Thus, we slightly prefer option 1.1.

- Option 2: The PRACH configuration index for additional PRACH resource is a new PRACH configuration index. Under this option, the following options can be considered:

- Option 2.1: Directly enable/disable the corresponding PRACH configuration index.

- Option 2.2: Introduce a muting pattern/enhanced cell DRX pattern (for RRC_CONNECTED UE) under the new PRACH configuration index.

- Regarding the two options, option 2.2 needs gNB to configure both additional PRACH configuration and muting pattern/enhanced cell DRX pattern configuration. Compared with option 2.1, this introduce more RRC signaling overhead without obvious benefits on NES gain. Thus, we slightly prefer option 2.1.

Proposal 4: For specific mechanism on PRACH adaptation, support the following options,

- Option 1: The additional PRACH resource is configured via introducing the time offset, scaling factor, or adjusting the parameter (x, y) under an existing PRACH configuration index.

- This refers to Opt 1-2a, Opt 1-2b and Opt 1-4 for Alt1 in RAN1#118bis agreement

- Option 2: The additional PRACH resource is configured via the new PRACH configuration index. No additional muting pattern/enhanced cell DRX pattern is needed under this option.

- This refers to Opt 1-5 for Alt2 in RAN1#118bis agreement

  - Discussion on overlapping issue for PRACH adaptation

In RAN1#118 meeting, the possible overlapping cases between legacy PRACH resource and additional PRACH resource are discussed, and the case that additional PRACH resource and legacy PRACH resource overlap in both time domain and frequency domain still need some further consideration. In this subsection, we discuss the solution on overlapping issue under this case. The options and our preference are as follows:

- Option 1: The overlapping issue can be solved via gNB implementation.

- For the case that additional PRACH resource is configured via introducing the time offset or adjusting the parameter (x, y) under an existing PRACH configuration index (subcases under option 1.1 in subsection 3.1), the feasibility of this solution can be ensured.

- For the case that additional PRACH resource is configured via introducing the scaling factor under an existing PRACH configuration index (subcase under option 1.1 in subsection 3.1), or via a new PRACH configuration index (option 2.1 in subsection 3.1), consider the current design of PRACH configuration time pattern in TS 38.211, the flexibility for gNB is relatively limited.

- Option 2: The overlapping issue can be solved via extra enhancement.

- This is useful for the case that additional PRACH resource is configured via introducing the scaling factor under an existing PRACH configuration index, or via a new PRACH configuration index.

- Regarding the specific solution, the following options can be considered:

- Option 2.1: Enhancing the definition of PRACH configuration. In current NR system, a configuration for PRACH transmission is determined by the configuration rule in TS 38.211. For the additional PRACH resource, the definition for its configuration may also be determined by the legacy PRACH resource. E.g., if one PRACH configuration index m is configured for legacy PRACH resource and another PRACH configuration index n is configured for determining additional PRACH resource, the PRACH configuration of additional PRACH resource is the difference set of parameters under PRACH configuration index m and parameters under PRACH configuration index n.

- Option 2.2: Enhancing the validation rule of PRACH occasion. In current NR system, a valid RO is determined by both frame structure and SSB transmission pattern. For the additional PRACH resource, the valid RO may also be determined by the legacy PRACH resource. E.g., the RO in additional PRACH resource that is overlapped with legacy RO resource can be treated as invalid.

Proposal 5: When additional PRACH resource and legacy PRACH resource overlap in both time domain and frequency domain, support the following options to solve the overlapping issue:

- Option 1: Up to gNB implementation.

- This is useful for the case that additional PRACH resource is configured via introducing the time offset or adjusting the parameter (x, y) under an existing PRACH configuration index.

- Option 2: Via extra enhancement.

- Option 2.1: Enhancing the definition of PRACH configuration.

- Option 2.2: Enhancing the validation rule of PRACH occasion.

- This is useful for the case that additional PRACH resource is configured via introducing the scaling factor under an existing PRACH configuration index, or via a new PRACH configuration index.

- Discussion on Paging adaptation

For Paging adaptation, the following agreements are agreed during RAN2#125bis-128 meeting [7-9].

Agreements on paging adaptation in RAN2#125bis: - For adaptation of paging occasions in time domain, RAN2 to study a) bundle paging frames and b) extend the values of N to have increased interval between PFs (e.g. T/64, T/128 ...) and compensating decrease in number of PFs by increasing POs per PF. Agreements on paging adaptation in RAN2#127bis: - Select option-b as baseline for R19 NES paging enhancement. - R2 should aim at signaling overhead minimization (e.g., Ns=8 and FFS for other larger values) - Allowing legacy and R19 UEs to co-ex in the same PF/PO is possible, based on NW configuration. - Legacy UE is not barred. - Rel-19 UEs only monitor the PO(s) according to Rel-19 paging configuration. Agreements on paging adaptation in RAN2#128: - Introduce value for Ns=8. FFS on 16. - Introduce value for N= T/32. FFS on T/64, T/128, T/256.

In this section, we discuss the potential RAN1 impact for paging adaptation.

Based on the above agreements, RAN2 has agreed to adapt PF density per I-DRX cycle and PO number per PF together to achieve paging adaptation. From our understanding, the indication method for adaptation on above parameters, i.e., whether the existing procedure (e.g. SI update and RRC re-configuration) is enough or enhancement is needed, should to be considered and discussed in RAN1.

Furthermore, during the previous discussion on SSB adaptation and PRACH adaptation, companies reach a consensus that the adaptation should be triggered by gNB instead of UE. From this point of view, instead of designing separate indication signaling for each common signal, it is recommended to have a unified design, which can obviously reduce the overhead for indication signaling.

Proposal 6: For Paging adaptation, support RAN1 to discuss:

- Whether the existing procedure (e.g., SI update and RRC re-configuration) on indication method is enough or enhancement is needed.

- Whether a unified design is needed for indication on common signal adaptation.

- Conclusion

In this contribution, we discussed the adaption of common signal/channels, include SSB adaptation in time domain, PRACH adaptation in time domain and Paging adaptation, and the following proposals and observation are made:

Observation 1: For SSB adaptation that is only applicable for NCD-SSB while CD-SSB is always transmitted in normal periodicity, 0.7%~2.7% NES gain can be obtained in various BS types and TDD/FDD structures.

•	Note that the assumption for cell load is empty load.

Proposal 1: Do not support SSB adaptation that is only applicable for NCD-SSB/not CD-SSB while CD-SSB is always transmitted in normal periodicity.

Proposal 2: Support RAN1 further discuss SSB adaptation under the following scenario:

- SSB adaptation for the cell that only have NCD-SSB/not CD-SSB (e.g. SSB not on sync raster)

- Note that the cell can only be a SCell for both R19 NES-capable UE and other UE.

Proposal 3: For SSB adaptation on SCell, support to discuss the design of specific mechanism under AI 9.5.1.

Proposal 4: For specific mechanism on PRACH adaptation, support the following options,

- Option 1: The additional PRACH resource is configured via introducing the time offset, scaling factor, or adjusting the parameter (x, y) under an existing PRACH configuration index.

- This refers to Opt 1-2a, Opt 1-2b and Opt 1-4 for Alt1 in RAN1#118bis agreement

- Option 2: The additional PRACH resource is configured via the new PRACH configuration index. No additional muting pattern/enhanced cell DRX pattern is needed under this option.

- This refers to Opt 1-5 for Alt2 in RAN1#118bis agreement

Proposal 5: When additional PRACH resource and legacy PRACH resource overlap in both time domain and frequency domain, support the following options to solve the overlapping issue:

- Option 1: Up to gNB implementation.

- This is useful for the case that additional PRACH resource is configured via introducing the time offset or adjusting the parameter (x, y) under an existing PRACH configuration index.

- Option 2: Via extra enhancement.

- Option 2.1: Enhancing the definition of PRACH configuration.

- Option 2.2: Enhancing the validation rule of PRACH occasion.

- This is useful for the case that additional PRACH resource is configured via introducing the scaling factor under an existing PRACH configuration index, or via a new PRACH configuration index.

Proposal 6: For Paging adaptation, support RAN1 to discuss:

- Whether the existing procedure (e.g., SI update and RRC re-configuration) on indication method is enough or enhancement is needed.

- Whether a unified design is needed for indication on common signal adaptation.

- Reference

- RP-234065, New WID: Enhancements of network energy savings for NR.

- RP-241650, Revised WID: Enhancements of network energy savings for NR.

- R1-2410888, Moderator (Ericsson), Final summary of AI 9.5.3 for R19 NES

- R1-2409278, Moderator (Ericsson), Final summary of AI 9.5.3 for R19 NES

- R1-2407552, Moderator (Ericsson), Final summary of AI 9.5.3 for R19 NES

- 3GPP TR 38.864, Study on network energy savings for NR.

- R2-2403731, Vice Chairman (Samsung), Report from session on V2X/SL, R19 NES and MOB

- R2-2407571, Vice Chairman (Samsung), Report from session on V2X/SL, R19 NES and MOB

- R2-2410911, Vice Chairman (Samsung), Report from session on V2X/SL, R19 NES and MOB