3GPP TSG RAN WG1 #120	R1-2500294

Athens, Greece, February 17th – 21st, 2025

Source: 	CMCC

Title:	Discussion on LP-WUS and LP-SS design

Agenda item:	9.6.1

Document for:	Discussion & Decision

- Introduction

As the WID in RP-234056 [1], the objectives of LP-WUS and LP-SS design are as the following：

- To specify an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes (RAN1, RAN4)

  - Specify OOK (OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK symbol

    - The LP-WUS design shall ensure that for IDLE/INACTIVE operation, the same information is delivered irrespective of LP-WUR type. The OFDM sequence can carry information.

  - At least duty-cycled monitoring of LP-WUS is supported

- For IDLE/INACTIVE modes

  - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4)

    - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI.

    - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS.

    - Y will be decided within WI. 320ms is the start point.

In this contribution, the design of LP-WUS and LP-SS will be discussed.

- Discussion

  - Overlaid OFDM sequence

In last RAN1 meeting, the following agreements were agreed for the OFDM sequence design. Two remaining issues need further down-selection.

Agreement For the overlaid OFDM sequence in time domain, the sequence is generated based on , - M=1, is given by the largest prime number such that,is the overlaid OFDM sequence length. - The base overlaid sequence is generated by extension of , - With CS(s) applied to the base overlaid OFDM sequence if any:denotes the potential cyclic shift (s) ,, - M=2, 4, is down-selected from the following: - Alt1: is given by the largest prime number such that,is the overlaid OFDM sequence length. - The base overlaid sequenceis generated by extension of , - With CS(s) applied to the base overlaid OFDM sequence if any:denotes the potential cyclic shift (s) , - Note it doesn’t preclude any pulse shaping scheme if any. - Alt2:is given by the largest prime number such that, is the overlaid OFDM sequence length. - The base overlaid sequenceis generated by inserting zeros before and/or after . The total number of zeros is. For example,, - With CS(s) applied to the base overlaid OFDM sequence if any: denotes the potential cyclic shift (s) For example,, - where, - FFS the value of G, with G>0. - FFS value(s) of root q and/or CS for generating the candidate sequences if applied - FFS on whether changes are needed to handle intra-/inter-cell interference - Note: the overlaid OFDM sequence in time domain is based on potential modification to ZC sequence as listed above. - Above overrides any previous RAN1 agreement / working assumption Agreement At least for M>1, for the overlaid OFDM sequence in time domain, down-select the sequence length LZC from the following: - Alt1:, with - FFS how to map the generated overlaid OFDM sequence to the frequency domain - Alt2: - Note: X is the number of RBs of LP-WUS/LP-SS bandwidth (blanked guard RBs are not included) Agreement CS(s) and/or root(s) used for overlaid OFDM sequence in the time domain are derived from RRC signalling: - FFS: the set of values of CS and root for configuration - FFS: details of the RRC signaling

The first FFS issue is the largest prime numberfor M=2,4. Alt 1 is the same as M=1 case and also similar to current NR ZC sequence generation. Alt 2 inserts zeros in the sequence which can tolerate larger timing error than Alt 1 but considering the residual time error of OFDM-based LP-WUR is much smaller than OOK-based LP-WUR, the benefit is marginal. In addition, Alt 2 also needs more standard work to determine the value G. Thus, we think Alt 1 can be supported for a common design between different M values.

Proposal 1: For M=2, 4, support Alt1:is given by the largest prime number such that,is the overlaid OFDM sequence length.

Another FFS is the overlaid OFDM sequence length. According to previous working assumption, overlaid ZC sequence is generated in time domain and through DFT/LS operation then IFFT operation to results in a frequency domain sequence. Thus the determine of ZC sequence length also impacts on the DFT size as well as truncation operation. For Alt 1, the , k equals the DFT size. Because the , some REs in the LP-WUS/LP-SS bandwidth will be blank. Taking X=11 PRB as example, k=128 and number of blank REs are 4. For Alt 2,  which equals the LP-WUS/LP-SS bandwidth, but DFT size is limited to 2n or 2n1*3n2*5n3. Also taking the example of X=11 PRB, DFT size can be 144, which causes 12bits truncation operation. From the performance perspective, Alt 1 can be better than Alt 2. Regarding how to map the generated overlaid OFDM sequence to the frequency domain, i.e., the position of blank REs, it can be configured by the gNB considering the location of LP-WUS/LP-SS bandwidth within the whole system carrier.

Proposal 2: At least for M>1, for the overlaid OFDM sequence in time domain, support Alt 1: , with as the sequence length. Whether the blank REs are located at the lowest REs or highest REs within the LP-WUS/LP-SS bandwidth can be configured by gNB.

Option 2-2 was agreed as for method of OFDM sequence carrying information, i.e., the overlaid OFDM sequence(s) carry all information bits of LP-WUS, one FFS is the detailed solution.

Agreement Update the existing agreement as shown below: In case of overlaid OFDM sequence carrying information, support option 2: - Option 2: One sequence is selected from multiple candidates overlaid OFDM sequences on each OOK ‘ON’ symbol, and OFDM-based LP-WUR obtain LP-WUS information at least by overlaid OFDM sequence(s). Consider the following two sub-options for potential down-selection. - Option 2-1: The overlaid OFDM sequence(s) carry part of information bits of LP-WUS. OFDM-based LP-WUR can obtain the whole information bits by OFDM sequence(s) and location of the OFDM sequence(s)/OOK ‘ON’ symbols. - Option 2-2: The overlaid OFDM sequence(s) carry all information bits of LP-WUS. OFDM-based LP-WUR can obtain the whole information bits by the overlaid OFDM sequence(s) - FFS the how the information bits are carried by the overlaid OFDM sequence(s) Note: the overlaid OFDM sequence in each OOK ‘ON’ symbol can be different according to information bits to be carried by the overlaid OFDM sequence within the LP-WUS.

Generally, there are two alternatives:

- Alt 1: One sequence carries 1 bit information: two overlaid OFDM sequences are predefined which represents value ‘1’ and value ‘0’ respectively, each ‘ON’ symbol carries one sequence from value ‘1’ and value ‘0’.

- Alt 2: One sequence carries n>1 bits information: 2n overlaid OFDM sequences are predefined and ‘ON’ symbol carries one sequence from 2n overlaid OFDM sequences.

  - Alt 2-1: without block level repetition

  - Alt 2-2: with block level repetition

Comparison between two alternatives, the sequence detection performance of Alt 1 is better than Alt 2 in case of same sequence length. Regarding Alt 2, one remaining issue is whether all ‘ON’ chips should carry the overlaid sequences. Taking 8 bits LP-WUS information and n=2 as example. Solution Alt 2-1 is only 4 ON chips overlaying the OFDM sequences and another solution Alt 2-2 is all 8 ON chips overlaying the OFDM sequences with repetition number equals 2. Considering the performance and complexity, Alt 1 is simplest.

Regarding the mapping order, it can be in ascending order or descending order. We don’t observe benefit to support descending order, thus, ascending order can be considered.

Proposal 3: For overlaid OFDM sequence, one overlaid sequence carries 1 bit information with ascending order mapping on each ‘ON’ symbol.

  - OOK Waveform generation

As the working assumption time-domain sequence was supported as the generation method of overlaid OFDM sequence for OOK-4 and OOK-1 as special case of OOK-4. In a consequence, the OOK waveform generation should also be in time domain before DFT/LS/IFFT processing. Regarding the multiplexing between legacy NR signal and LP-WUS/LP-SS, we think it should be done before IFFT without additional hardware requirement.

Proposal 4: Support to specify time domain signal before DFT/LS/IFFT processing for LP-WUS/LP-SS waveform generation for both OOK-4 and OOK-1.

Proposal 5: The multiplexing between legacy NR signal and LP-WUS/LP-SS should be before IFFT.

  - Carried information indication

For future meetings: Companies are encouraged to consider the following for future discussions Regarding the LP-WUS information to trigger PDCCH monitoring of RRC connected UEs, - Option 1: A bitmap with each bit corresponding to [one or more] UEs - 1A No CRC - Per bit coding, e.g., one bit is mapped to one of 2 codeword/sequences - The maximum number of codewords targeting per bit position in the bitmap for a UE is 1 - FFS coded bit length - Single MO - 1B No CRC - Bitmap-level coding, e.g., one bitmap of L bits is mapped to one of 2L codeword/sequences - The maximum number of codewords/sequences targeting for a UE is 2L-1 - Single MO - 1C With CRC - FFS CRC length - Single MO - Option 2: A codepoint value corresponding to [one or more] UEs - 2A only 1-1 mapping from codepoint to UE - The maximum number of codepoints targeting per MO for a UE is 1 - FFS the maximum number of MOs to be detected by a UE - 2B 1-to-1, 1-to-all mapping from codepoint to UEs - The maximum number of codepoints targeting per MO for a UE is up to 2 - FFS the maximum number of MOs to be detected by a UE - 2C 1-to-X mapping from codepoint to UEs, - The maximum number of codepoints targeting per MO for a UE is up to - FFS the maximum number of MOs to be detected by a UE - Note: L is the number of UEs associated monitoring the same WUS

In last meeting, some alternatives are listed for further discussion on how to carry the PDCCH triggering indication for RRC_CONNECTED state. Generally, there are two series including bitmap based and codepoint values based. Compared with bitmap based solutions, codepoint values based solutions will cause more detection complexity and power consumption due to LP-WUR needs monitoring multiple MOs. In addition, the specification effort is also higher, e.g., the association between codepoint values and UE indices/groups. Thus, we think bitmap based solutions like R16 DCP design can be considered for LP-WUS in RRC_CONNECTED state. In comparison among Option 1 series, Option 1A or Option 1C are preferred with less work load. In addition, the LP-SS sequence can be reused for the Option 1A.

Proposal 6: Regarding the LP-WUS information to trigger PDCCH monitoring of RRC connected UEs, support one of the following two options:

- Option 1: A bitmap with each bit corresponding to [one or more] UEs

  - 1A No CRC

    - Per bit coding, e.g., one bit is mapped to one of 2 codeword/sequences

    - The maximum number of codewords targeting per bit position in the bitmap for a UE is 1

    - Single MO

  - 1C With CRC

    - Single MO

  - Structure design

Agreement For the M value for LP-WUS and LP-SS, down-select one alternative from the following in RAN1#120: - Alt1: The M values for LP-WUS and LP-SS are always same. - Alt2: The M values for LP-WUS and LP-SS can be configured to be same or different. M value for LP-WUS cannot be larger than that of LP-SS. - Alt3: The M values for LP-WUS and LP-SS can be configured to be same or different.

The first issue is M value for LP-WUS and LP-SS. From our perspective, it is better to provide gNB some flexibility to configure different M values for LP-WUS and LP-SS. Considering the UE complexity and tolerated timing error, Alt 2 is good compromise among these three alternatives.

Proposal 7: For the M value for LP-WUS and LP-SS, support Alt2: The M values for LP-WUS and LP-SS can be configured to be same or different. M value for LP-WUS cannot be larger than that of LP-SS.

Agreement For frequency error evaluation purpose, the following two options for residual time error are considered: - Option 1: The maximum frequency error (Fe) of oscillator is assumed, companies report Fe value and the applied LP-WUR type. - Option 2: The residual frequency error (Fr) after frequency error correction by LR or after assistance from MR is assumed, companies report Fr value, how to achieve it and the applied LP-WUR type. Agreement For OOK-based LP-WUR, the LP-WUS design assumes the following: - As starting point, the time error is based on the 5-10ppm residual frequency error after frequency error correction without considering impact of drift. - The frequency error: up to company report. - Note: 5-10ppm assumes frequency error correction is performed, e.g., RTC calibration and/or MR assistance - Note: Companies can assume other values outside of 5-10ppm as long as the values are justified Agreement As a starting point, for both time error and frequency error, the overlaid OFDM sequence design of LP-WUS assumes that the residual frequency error is 0.1-5ppm for OFDM-based LP-WUR after frequency error correction without considering impact of drift. - Note: Companies can use any value within the above range with justification on the value (including impact on power consumption) Agreement Regarding whether to support additional sync signal to LP-SS, down-selection from the following: - Option 1A: No additional sync signal, LP-SS periodicity =320ms - Additionally support at least one of [80ms,160ms] - FFS: whether different values can be applicable for different M values - Option 1B: No additional sync signal, LP-SS periodicity =320ms - No additional support of other periodicities - Option 2: With additional sync signal, LP-SS periodicity =320ms - FFS additionally support other LP-SS periodicities - FFS details on additional sync signal - FFS additional sync signal is configurable and/or conditionally present

During previous meetings, how to assume the time error/frequency error and whether to support additional sync signal were discussed, and companies also proposed some methods for frequency error correction. In this section, we will analyze the residual time error under different assumptions:

- If residual frequency error is assumed after frequency error correction. Take the transition region (TR38.869 section 6.2.3 model for time-frequency impairment) model as an example, i.e., Te= ΔT+ Tr = Fr*T ±0.5 * F’ *T2 + Tr, assuming T=80ms/160ms/320ms, Tr=~ 0 (remaining residual time error after each LP-SS time synchronization is ideal), F’=0.1ppm/s, the time errors for OOK-based LP-WUR are:

| Time error | T=80ms | T=160ms | T=320ms |
| --- | --- | --- | --- |
| Fr=5ppm | 0.40032us | 0.80128us | 1.60512us |
| Fr=10ppm | 0.80032us | 1.60128us | 3.20512us |

- If maximum frequency error is assumed, which is suitable for LP-WUR without the implementation of frequency error correction. Take the linear region model as an example, i.e., ΔT = ±Fe * T, the time errors are:

| Time error | T=80ms | T=160ms | T=320ms |
| --- | --- | --- | --- |
| Fe=20ppm | 1.6us | 3.2us | 6.4us |

From the above analysis we can see that if maximum frequency error (Fe) of oscillator is assumed, the residual timing error is so large which is far beyond the upbound the maximum timing error OOK waveform can tolerate. Even assuming the residual frequency error (Fr) can be corrected by LR or assistance from MR, the residual timing error is still large enough in some case, e.g., T=320ms which brings difficulty for OOK-4 M=2/4 detection. Therefore, Option 1B (No additional sync signal, LP-SS periodicity =320ms) should be dropped. Regarding the comparison between Option 1A and Option 2, we prefer to specify the additional sync signal without introducing other LP-SS periodicities. For LP-WUS operation in IDLE/INACTIVE mode, the codepoint based information indication is agreed, that is gNB will not transmit LP-WUS if no UE needs be waken up. Thus, the signalling overhead in Option 2 is smaller than Option 1A which needs the periodic LP-SS transmissions. To reduce the specification effort, the additional sync signal can reuse the sequence design of LP-SS.

Proposal 8: Support additional sync signal with LP-SS periodicity =320ms.

- Without additionally support other LP-SS periodicities.

- The LP-SS sequence can be reused as the additional sync signal.

Another issue is how to meet the requirement of LP-WUS/LP-SS, including FAR/MDR and coverage.

Agreement For evaluation of LP-WUS and LP-SS design to achieve coverage of PUSCH for message3 from RAN1 perspective, at least the following SNR values should be considered: - SNR=-3dB for NF of LR = NF of MR+ 8dB - SNR= -0.5dB for NF of LR = NF of MR+ 5dB (OOK-LR) - SNR=2dB for NF of LR = NF of MR+ 2dB (OFDM-LR) Note 1: The NF of MR is assumed as 7dB Agreement Down-select among the following alternatives for LP-WUS information for OOK to meet target requirements: - adding CRC - channel coding other than Manchester coding, including rate matching if any - mapped to a binary sequence/codeword, include truncation if any - repetition, including rate matching if any - Note: combination of options above is not precluded - Note: combination of Manchester coding with above is assumed FFS how LP-WUS is transmitted within a MO/MOs

FAR/MDR

Generally, there are two structures of physical layer design of LP-WUS signal, including encoding bits based and sequence based solutions.

- For encoding bits based LP-WUS structure, CRC part is necessary to determine whether the LP-WUS is absent or present depends on the CRC pass or not.

- For sequence based LP-WUS structure, the LP-WUS information sequence can be divided into two parts, including sequence 1 and sequence 2. The sequence 1 is used for absent or present detection by one pre-defined sequence, which indicates whether this is a LP-WUS or not. The sequence 2 is used to carry the waking up information. The reason why to design two sequence parts is sequence based LP-WUS design does not have the check module for the correction. In order to decrease the miss detection rate and false alarm rate, sequence 1 is used to check whether it is an LP-WUS, and then investigate the MDR of sequence 2.

Proposal 9: The following solutions can be considered for LP-WUS design to meet the FAR/MDR requirement:

- Solution 1: encoding bits + CRC

- Solution 2: sequence 1(absence or not) + sequence 2(waking up information)

Coverage

To determine the resource used for LP-WUS for meeting the target coverage requirement, we provide some evaluation results with the assumption of 8 bits payload+8 bits CRC to meet the target SNR = -0.5dB. The detailed simulation assumption can be found in Annex. In the simulation, we assume two time/frequency error cases, which the Case 1 is with frequency error correction, i.e., residual frequency error (Fr) is assumed, and Case 2 is without frequency error correction, i.e., maximum frequency error (Fe) is assumed. In addition, two repetition methods are also simulated, including method 1: bit-level repetition and method 2: block-level repetition.

- Case 1: with frequency error correction

In this case, residual frequency error Fr is assumed as 10ppm, and T=320ms, F’= 0.1 ppm/s, the timing error ΔT is 3.2us, and frequency error Fd is 10 ppm.

| ΔT =3.2us, Fd=10 ppm | OOK-1 |  | OOK-4 M=2 |  |
| --- | --- | --- | --- | --- |
| Repetition method | bit level repetition * 2 | block level repetition*2 | bit level repetition*4 | block level repetition |
| BLER@-0.5dB SNR | 0.006 | 0.0035 | 0.005 | NA |

Note: NA means the BLER result can not be smaller than 1% using this method

- Case 2: without frequency error correction

In this case, residual frequency error Fr is assumed as 20ppm, and T=320ms, F’= 0.1 ppm/s, the timing error ΔT is 6.4us, and frequency error equals Fd is 50 ppm.

| ΔT=6.4us, Fd=50 ppm | OOK-1 |  | OOK-4 M=2 |  |
| --- | --- | --- | --- | --- |
| Repetition method | bit level repetition * 2 | block level repetition*3 | bit level repetition*4 | block level repetition |
| BLER@-0.5dB SNR | 0.0065 | 0.0055 | 0.006 | NA |

Note: NA means the BLER result can not be smaller than 1% using this method

For OOK-4 with M=4, the 3.2us or 6.4us is difficult to tolerate, thus preamble is necessary as the discussion above. We also provide some simulation results for OOK-4 M=4 assuming the timing estimation error after the LP-WUS preamble part is 1us.

|  | OOK-4 M=4, ΔT=1us, Fd=10 ppm |  | OOK-4 M=4, ΔT=1us, Fd=50 ppm |  |
| --- | --- | --- | --- | --- |
| Repetition method | bit level repetition * 6 | block level repetition*12 | bit level repetition*6 | block level repetition*12 |
| BLER@-0.5dB SNR | 0.006 | 0.01 | 0.0065 | 0.006 |

From the simulation results, we can see that repetition is necessary for LP-WUS OOK waveform to achieve the target SNR with the same coverage of PUSCH for message3 and the bit level repetition provides better performance than block level repetition.

Proposal 10: Bit-level or block-level repetition is necessary for OOK-1 and OOK-4 to achieve the same coverage as Msg3.

  - Time and frequency resource

Agreement For RRC idle/inactive state, for the case where associated CD-SSB and initial DL BWP have the same SCS at least when the associated CD-SSB and LP-WUS are on the same carrier - The single SCS for LP-WUS/LP-SS is same as the associated CD-SSB - FFS: Case where associated CD-SSB and initial DL BWP have different SCSs - FFS: Which initial BWP for RedCap

Regarding the SCS for LP-WUS/LP-SS, there are two FFS need to be resolved. The first one is case where associated CD-SSB and initial DL BWP have different SCSs, considering to reduce the complicity of multiplexing between LP-WUS/LP-SS and other NR signals and channels within initial DL BWP, it is preferred that the single SCS for LP-WUS/LP-SS is same as the initial DL BWP.

The second one is the initial BWP for RedCap. The separate initial DL BWP for Redcap must include CD-SSB if the separate initial DL BWP is used for paging. Thus if separate initial DL BWP is configured for Redcap, the separate initial BWP is used to determine the SCS, otherwise, the common initial DL BWP is used.

Proposal 11: For RRC idle/inactive state, for the case where associated CD-SSB and initial DL BWP have different SCSs, the single SCS for LP-WUS/LP-SS is same as the initial DL BWP.

Proposal 12: For Redcap, if separate initial DL BWP is configured,

- for the case where associated CD-SSB and separate initial DL BWP have the same SCS, the single SCS for LP-WUS/LP-SS is same as the associated CD-SSB within separate initial DL BWP.

- for the case where associated CD-SSB and separate initial DL BWP have different SCSs, the single SCS for LP-WUS/LP-SS is same as the separate initial DL BWP.

if separate initial DL BWP is not configured, the SCS for LP-WUS/LP-SS for ReCap is the same as non-RedCap.

Another issue is the frequency location of LP-WUS, whether the frequency location is flexible or restricted needs discussion.

- For LP-WUS location within a carrier, the advantage of the LP-WUS bandwidth at a certain location is a lower requirement for the hardware and easier to implement than flexible location. The disadvantage of bandwidth restriction is it will cause the scheduling restrictions when multiplexing with other NR signals.

- If the LP-WUS is configurable within carrier/BWP in a band, another issue that needs discussion is whether the LP-WUS should always be confined within the initial DL BWP. Initial DL BWP is located in a limited BW and fixed location. LP-WUS may occupy 5MHz BW and different UE/group may be located in different frequency resources. Hence restricting LP-WUS to be confined within the initial DL BWP is not flexible. Moreover, the LR can receive signals independently of MR. Therefore, whether the LP-WUS bandwidth is inside or outside of DL BWP can be considered.

Proposal 13: Support flexible configuration of LP-WUS frequency location. Both inside and outside initial DL BWP can be considered.

- Conclusions

In this contribution, we discussed the LP-WUS and LP-SS design, and the following proposals were made.

Proposal 1: For M=2, 4, support Alt1:is given by the largest prime number such that,is the overlaid OFDM sequence length.

Proposal 2: At least for M>1, for the overlaid OFDM sequence in time domain, support Alt 1: , with as the sequence length. Whether the blank REs are located at the lowest REs or highest REs within the LP-WUS/LP-SS bandwidth can be configured by gNB.

Proposal 3: For overlaid OFDM sequence, one overlaid sequence carries 1 bit information with ascending order mapping on each ‘ON’ symbol.

Proposal 4: Support to specify time domain signal before DFT/LS/IFFT processing for LP-WUS/LP-SS waveform generation for both OOK-4 and OOK-1.

Proposal 5: The multiplexing between legacy NR signal and LP-WUS/LP-SS should be before IFFT.

Proposal 6: Regarding the LP-WUS information to trigger PDCCH monitoring of RRC connected UEs, support one of the following two options:

- Option 1: A bitmap with each bit corresponding to [one or more] UEs

  - 1A No CRC

    - Per bit coding, e.g., one bit is mapped to one of 2 codeword/sequences

    - The maximum number of codewords targeting per bit position in the bitmap for a UE is 1

    - Single MO

  - 1C With CRC

    - Single MO

Proposal 7: For the M value for LP-WUS and LP-SS, support Alt2: The M values for LP-WUS and LP-SS can be configured to be same or different. M value for LP-WUS cannot be larger than that of LP-SS.

Proposal 8: Support additional sync signal with LP-SS periodicity =320ms.

- Without additionally support other LP-SS periodicities.

- The LP-SS sequence can be reused as the additional sync signal.

Proposal 9: The following solutions can be considered for LP-WUS design to meet the FAR/MDR requirement:

- Solution 1: encoding bits + CRC

- Solution 2: sequence 1(absence or not) + sequence 2(waking up information)

Proposal 10: Bit-level or block-level repetition is necessary for OOK-1 and OOK-4 to achieve the same coverage as Msg3.

Proposal 11: For RRC idle/inactive state, for the case where associated CD-SSB and initial DL BWP have different SCSs, the single SCS for LP-WUS/LP-SS is same as the initial DL BWP.

Proposal 12: For Redcap, if separate initial DL BWP is configured,

- for the case where associated CD-SSB and separate initial DL BWP have the same SCS, the single SCS for LP-WUS/LP-SS is same as the associated CD-SSB within separate initial DL BWP.

- for the case where associated CD-SSB and separate initial DL BWP have different SCSs, the single SCS for LP-WUS/LP-SS is same as the separate initial DL BWP.

if separate initial DL BWP is not configured, the SCS for LP-WUS/LP-SS for ReCap is the same as non-RedCap.

Proposal 13: Support flexible configuration of LP-WUS frequency location. Both inside and outside initial DL BWP can be considered.

- Reference

[1] RP-234056 New WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)

[2] Chair notes RAN1#118

[3] Chair notes RAN1#118bis

[4] Chair notes RAN1#119

Annex

Table 1. LLS Simulation assumption

| Parameter | Assumptions |
| --- | --- |
| Scenario | 2.6GHz dense urban |
| Signal Bandwidth | 11RB |
| Subcarrier spacing | 30kHz |
| Channel model | TDL-C 300ns |
| UE speed | 3 km/h |
| Number of RX antennas | 1 |
| Phase noise | Non |
| ADC sampling rate | 7.68 MHz |
| ADC width | 8bit |
| Coding | 1/2 Manchester encoding |