3GPP TSG RAN WG1 #120		    R1-2500435

Athens, Greece, February 17th – 21st, 2025

Source:	OPPO

Title:	Discussion on soft satellite switch with re-sync

Agenda Item:	5

Document for:	Discussion and Decision

Introduction

In last meeting, RAN2 sends an LS to RAN1and RAN4 asking questions about DL synchronization in the case of soft satellite switch [1].

# 1. Overall Description: For soft satellite switching with re-synchronization, to ensure non-colliding SSB with same PCI at the UE side, RAN2 has discussed one possible solution to introduce a temporarily shifted SSB pattern during [t-ServiceStart, t-Service]. To be specific, the target satellite transmits SSB during [t-ServiceStart, t-Service] which is not overlapping with SSB from the source satellite. At t-Service, the new serving satellite SSB pattern (e.g., SSB offset) is restored to the same as the previous source satellite. RAN2 wonders whether the pre-acquired DL synchronization started between [t-ServiceStart, t-Service] with target satellite can be assumed as maintained if the SSB pattern (e.g., SSB offset) is changed (determined by ssb-TimeOffset) at t-Service. # 2. Actions: To RAN4, RAN1 ACTION: RAN2 kindly asks RAN4 and RAN1 whether the pre-acquired DL synchronization started between [t-ServiceStart, t-Service] with target satellite can be assumed as maintained if the SSB pattern (e.g., SSB offset) is changed (determined by ssb-TimeOffset) at t-Service.

In this contribution, we provide our views on the raised questions.

Discussion

The option pointed by RAN2 is that the network set an offset between the SSB of source satellite and the SSB of target satellite at the network side. Then the network informs the UE about this offset (by ssb-TimeOffset), so that the UE can perform DL synchronization based on the shifted SSB of the target satellite within a period [t-ServiceStart, t-Service]. Here by DL synchronization we understand the process to obtain the SFN boundary, slot boundary and symbol boundary of the target satellite. Then RAN2 asks after t-Service, the network shifts back the SSB of the target satellite by the same offset amount, i.e., ssb-TimeOffset, whether the UE will lose the above mentioned DL synchronization.

From our point of view, as long as the offset value (ssb-TimeOffset) is fixed and known to the UE, it will not impact the DL synchronization. The UE obtains the DL sync within [t-ServiceStart, t-Service] and the UE will continue to track the SSB by removing the offset after t-Service.

Proposal: RAN1 replies RAN1that there is no impact on the DL synchronization by changing the SSB pattern via SSB offset, as long as the SSB offset is fixed and known to UE.

Conclusion

In this document, we provide our analysis on the LS received from RAN2, and we drawn the following proposal:

Proposal: RAN1 replies RAN1that there is no impact on the DL synchronization by changing the SSB pattern via SSB offset, as long as the SSB offset is fixed and known to UE.

Reference

[1]     R2-2410978, “LS on soft satellite switch with re-sync ”, Huawei, 3GPP RAN2#128.