3GPP TSG RAN WG1 #120		                                      R1-2500473

Athens, Greece, February 17th – 21st, 2025

Source:	OPPO

Title:	Enhancements on asymmetric DL STRP/UL MTRP scenarios

Agenda Item:	9.2.4

Document for:	Discussion

# Introduction

The WID [1] of NR MIMO evolution Phase 5 was agreed in RAN#102 meeting. The objectives related to this agenda item are listed below

- Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2

  - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP.

In this contribution, we present our view on the enhancement of asymmetric DL sTRP and UL mTRP scenario.

# Scenarios and assumptions

The deployment of heterogeneous networks (HetNets) is a strategy to improve user experience by combining macro cells with smaller cells (e.g., micro, pico, and femtocells). This setup caters to the differing needs of users and helps improve overall network coverage and capacity, particularly for uplink (UL) throughput.

This scenario provides a strategic advantage in network topology, catering to dense urban environments where uplink data traffic is significantly high. By offloading the uplink communication to dedicated UL TRPs, NW can efficiently manage the increased uplink traffic without overburdening the regular TRP, which manages both uplink and downlink. Such a deployment scenario is instrumental in improving uplink throughput and enhances network capacity and coverage.

The basic scenario of asymmetric deployment is illustrated in Figure 1. UE is engaged in downlink communication with a regular TRP capable of both downlink and uplink transmission. However, for UL communication, the UE targets a separate TRP, designated as an UL TRP.

- : the asymmetric deployment scenario in which a UE communicates with one single TRP for DL and with one or more TRPs for UL

In the WID [1], the assumptions for the enhancement in this agenda item were explicitly listed in what follows. From our point of view, those assumptions underlying this deployment are constrained to ensure focus and manageability in standardization efforts.

  - DL sTRP/UL mTRP deployment

  - Intra-band & intra DU & non-co-located mTRP

  - No definition change on cell

  - Reuse Rel-17/18 unified TCI framework

  - Applicable to both FR1 and FR2

# PL offset(s) for UL TRP

For UL TRP, since no DL RS is transmitted, UE cannot measure its path loss (PL) from the UL TRP. In legacy, the UE measures the PL from the DL/UL TRP. Hence, a PL offset (based on the PL RS from DL/UL TRP) should be introduced according to the WID.

In RAN1#118, the value range of PL offset has been determined mainly as positive values. For asymmetric multi-TRP scenario, consider the fact that the UL PL to UL TRP is normally smaller than that of DL PL to DL/UL TRP.

Agreement For the asymmetric DL sTRP/UL mTRP scenarios, the value range of PL offset includes at least [-10, 60] dB - FFS: Extending the range to lower than -10dB - Step size is 4dB

Given the current progress, it is time to present detailed formulas on how to reflect the PL offset in determining UL Tx power for PUSCH/PUCCH/SRS/PRACH toward UL TRP. Note that the unit is in dBm.

PPUSCH,b,f,c,ki,j,qd,l=minPCMAX,f,c,kiPO_PUSCH,b,f,cj+10log102μ∙MRB,b,f,cPUSCHi+αb,f,cj∙(PLb,f,cqd-PLOk)+∆TF,b,f,ci+fb,f,ci,l

PPUCCH,b,f,c,ki,qu,qd,l=minPCMAX,f,c,kiPO_PUCCH,b,f,cj+10log102μ∙MRB,b,f,cPUCCHi+PLb,f,cqd-PLOk+∆F_PUCCHF+∆TF,b,f,ci+gb,f,ci,l

PSRS,b,f,c,ki,qs,l=minPCMAX,f,ciPO_SRS,b,f,c(qs) +10log10(2μ⋅MSRS,b,f,ci)+αSRS,b,f,c(qs)⋅(PLb,f,cqd-PLOk)+hb,f,c(i)

PPRACH,b,f,ci=minPCMAX,f,ci,PPRACH,target,f,c+PLb,f,c-PLOk

where PLOk and stands for the PL offset associated with the k-th indicated UL/joint TCI state.

With the above being said, our first proposal is to consider how to implement the PL offset for UL Tx power calculation for UL channels and signals in current specification, i.e. TS 38.213.

- Support to reflect the impact of PL offset as above for Tx power calculation for PUSCH/PUCCH/SRS/PRACH in TS 38.213.

## PHR triggering condition

In TS 38.321, it specifies whether/how to trigger PHR based on events at UE side. Part of TS 38.321 on PHR is captured below for reference. The very 1st condition on PHR triggering is that the path loss has changed significantly (larger than a pre-defined threshold) since last PHR transmission.

Next, what matters is how to understand the pathloss variance in RAN2 specification. From the note under the first condition, one may find that the pathloss variance is between the pathloss measured at present time and the pathloss measured at previous time when last PHR transmitted. In short, the pathloss variance is based on path loss RS measurement.

- For one of the PHR triggering conditions, the pathloss variance is calculated based on the measurement of path loss RS.

### 5.4.6 Power Headroom Reporting <Unrelated parts are omitted> A Power Headroom Report (PHR) shall be triggered if any of the following events occur: - phr-ProhibitTimer expires or has expired and the path loss has changed more than phr-Tx-PowerFactorChange dB for at least one RS used as pathloss reference for one activated Serving Cell of any MAC entity of which the active DL BWP is not dormant BWP since the last transmission of a PHR in this MAC entity when the MAC entity has UL resources for new transmission; NOTE 1: The path loss variation for one cell assessed above is between the pathloss measured at present time on the current pathloss reference and the pathloss measured at the transmission time of the last transmission of PHR on the pathloss reference in use at that time, irrespective of whether the pathloss reference has changed in between. The current pathloss reference for this purpose does not include any pathloss reference configured using pathlossReferenceRS-Pos in TS 38.331 [5]. <Unrelated parts are omitted>

For UL TRP in the asymmetric multi-TRP scenario, there is no DL PL RS from the UL TRP. UE has to measure PL RS from DL/UL TRP and adopt a PL offset associated with an indicated TCI state on path loss.

From this sense, it is reasonable to consider the impact of PL offset along with measured path loss into PHR triggering condition. The PL offset can be configured in UL/joint TCI state, and it can be updated by MAC CE.

One example could be that for a particular UE, the PL from DL/UL TRP remains stable, but its PL offset has been updated by MAC CE with a largely different value, e.g. from -30dB to -10dB. Hence the PHR triggering condition should consider the change of pathloss and PL offset in total, rather than path loss variance by measurement.

- On PHR triggering condition, support the case when the total change of path loss (by measurement) and PL offset is more than the pre-configured threshold (e.g. phr-Tx-PowerFactorChange dB).

## PHR impacted by PL offset

In RAN1#118, PL offset is supported to be reflected in Type 1 PHR. RAN1 continued to study whether to PL offsets should be reflected in Type 3 PHR too.

Agreement For the asymmetric DL sTRP/UL mTRP scenarios, support to include PL offset in the calculation of Type 1 PHR based on actual PUSCH transmission and Type 1 PHR based on reference PUSCH Agreement Study whether to support Type 3 PHR reporting in a serving cell/BWP where the UE is configured with two separate SRS CLPC adjustment states. - Continue to study whether to support including PL offset in the calculation of Type 3 PHR.

For Type 3 PHR (for SRS) with actual and reference transmission, one may argue that in legacy UE reports Type 3 PHR when the UE is not configured for PUSCH transmission on that carrier f of serving cell c. Now for UL TRP, there is PUSCH configured. Then the question in one’s mind could be whether UE report Type 3 PHR for UL TRP for any other good reason?

From our understanding, one use case is to facilitate NW for PL offset determination. Specifically, UE sends two separate SRS resources (SRS resource #1 and SRS resource #2) to UL/DL TRP and UL TRP respectably. If the same Tx power used for SRS resource #1 and #2, then NW can determine the UL PL difference by using SRS RSRP #1 – SRS RSRP #2. However, if different Tx powers (Pc,max - PH) applied for SRS resource #1 and 2#, then NW needs to know estimate UL PL toward DL/UL TRP and UL TRP separately. In other words, UE has to report Type 3 PHR to give NW enough information to determine the PL offset between DL/UL TRP and UL TRP.

- Type 3 PHR could be helpful for NW to determine PL offset value between DL/UL TRP and UL TRP.

With the above being said, we suggest supporting Type 3 PHR along with Type 1 PHR on the same carrier f of the serving cell c. Details impacting Type 3 PHR can be listed as below for actual and reference SRS transmission, respectively.

PHtype3,b,f,ci,qs=PCMAX,f,ci - PO_SRS,b,f,c(qs) +10log10(2μ⋅MSRS,b,f,ci)+αSRS,b,f,c(qs)⋅(PLb,f,cqd-PLOk)+hb,f,c(i)[dB]

PHtype3,b,f,ci,qs=PCMAX,f,ci- PO_SRS,b,f,c(qs) +αSRS,b,f,c(qs)⋅(PLb,f,cqd-PLOk)+hb,f,c(i)[dB]

where PLOk stands for the PL offset associated with the k-th indicated UL/joint TCI state.

- For carrier f of serving cell c configured with PUSCH, support both actual and reference Type 3 PHR with PL offset considered.

# Enhancements on dynamic power control for SRS

In the asymmetric deployment scenario, UE may transmit SRS towards multi-TRP including DL/UL TRP and UL TRP. However, in legacy releases, dynamic power control for SRS was inherited from the design of single TRP, i.e. single close-loop power control state.

## SRS not configured to follow indicated UL/Joint TCI state

In RAN1#118, the following agreement was achieved to study the case when SRS resource set is not configured with TCI state.

Agreement - Study whether/how to apply PL offset for SRS resource set when the SRS resource set is not configured with TCI state - Study whether/how to apply one of the two separate SRS CLPC adjustment states on the SRS resource set when the SRS resource set is not configured with TCI state - E.g., defining i0 as the default CLPC for SRS resource set in this case. E.g,, configure one of the separate SRS CLPC adjustment states to the SRS resource set.

From our understanding, NW can avoid the case when SRS is not configured with any unified TCI state. In Rel-17, SRS can be applied with two types of unified TCI states. The first type is the indicated TCI state shared with other UL channel/signals, and the second type is dedicatedly configured for each SRS resource when the first case doesn’t hold.

More specifically, for the second case, when the RRC parameter followUnifiedTCI-StateSRS is not configured for a SRS resource set, NW can configure UL/joint TCI state for each SRS resource within the SRS resource set. Each TCI state includes a set of UL power control parameters, e.g. P0, alpha, CLI and PL RS. To determine one set of UL PC parameters for all SRS resources within the SRS resource set, the current spec (highlighted as yellow) applies the UL power control parameter associated with the SRS resource with lowest ID in the SRS resource set. For more detailed information, one may refer to the following specifications.

TS 38.213 # 7 Uplink Power control -----------------------------------------<unrelated part>---------------------------------------------- - in clause 7.3.1, if p0AlphaSetforSRS is provided, - if followUnifiedTCI-StateSRS is provided for a SRS resource set, the values of PO_UE_SRS,b,f,cqs, αSRS,b,f,cqs, and SRS power control adjustment state l are provided by p0AlphaSetforSRS associated with the indicated TCI-State or TCI-UL-State - else, if followUnifiedTCI-StateSRS is not provided for a SRS resource set and for a SRS resource from the SRS resource set, the values of PO_UE_SRS,b,f,cqs, αSRS,b,f,cqs, and SRS power control adjustment state l are provided by p0AlphaSetforSRS associated with TCI-State or TCI-UL-State of an SRS resource with lowest SRS-ResourceId in the SRS resource set and a RS index qd for obtaining a pathloss estimate for the SRS transmission is provided by pathlossReferenceRS-Id-r17 associated with or included in the TCI-State or TCI-UL-State of an SRS resource with lowest SRS-ResourceId in the SRS resource set PO_SRS,b,f,cqs is the sum of the component PO_UE_SRS,b,f,cqs and a component p0 provided by SRS-ResourceSet corresponding to the SRS resource set. -----------------------------------------<unrelated part>----------------------------------------------

In RAN1#119, four alternatives were discussed and proposed in the following agreement to be down selected.

Agreement For an SRS resource set not configured with followUnifiedTCI-StateSRS, regarding how to determine the PL offset and the CLPC adjustment state, down-select from the following Alts: - Alt1: The PL offset and CLPC adjustment state indicated by the TCI state configured to the SRS resource with lowest ID (i.e., reusing the rule specified in Rel-17) and no extra enhancement. - Alt2: The PL offset and CLPC adjustment state indicated by the TCI state configured to the SRS resource with lowest ID (i.e., reusing the rule specified in Rel-17), the SRS with lowest ID is always configured with one joint/UL TCI state, the “referenceSignal” in a UL TCI state is optional. - Alt3: - When the SRS resource with lowest ID is configured with a TCI state, The PL offset and CLPC adjustment state indicated by that TCI state are applied to those SRS resources (i.e., reusing the rule specified in Rel-17) - Otherwise, one of the two separate SRS CLPC adjustment states and one PL offset are configured in the SRS resource set, and they are applied on the SRS resources. - Alt4: - When the SRS resource with lowest ID is configured with a TCI state, The PL offset and CLPC adjustment state indicated by that TCI state are applied to those SRS resources (i.e., reusing the rule specified in Rel-17) - Otherwise, one PL offset and one CLPC adjustment state are applied on the SRS resources. - FFS how to define the PL offset and CLPC adjustment state

Following the legacy in Rel-17, when one SRS resource set is not configured with followUnifiedTCI-StateSRS, we could adopt the same rule (i.e. Alt1: applying the PL offset associated with the SRS resource with lowest ID) to determine the PL offset for all the SRS resources within the SRS resource set.

- When a SRS resource set is not configured with followUnifiedTCI-StateSRS, and for a SRS resource from the SRS resource set

- Support Alt1: Applying the PL offset associated with the UL/Joint TCI state of an SRS resource with lowest SRS-ResourceId in the SRS resource set

# Conclusion

In this section, allow us to repeat our proposals and observations.

- Support to reflect the impact of PL offset as above for Tx power calculation for PUSCH/PUCCH/SRS/PRACH in TS 38.213.

- On PHR triggering condition, support the case when the total change of path loss (by measurement) and PL offset is more than the pre-configured threshold (e.g. phr-Tx-PowerFactorChange dB).

- For carrier f of serving cell c configured with PUSCH, support both actual and reference Type 3 PHR with PL offset considered.

- When a SRS resource set is not configured with followUnifiedTCI-StateSRS, and for a SRS resource from the SRS resource set

- Support Alt1: Applying the PL offset associated with the UL/Joint TCI state of an SRS resource with lowest SRS-ResourceId in the SRS resource set

- For one of the PHR triggering conditions, the pathloss variance is calculated based on the measurement of path loss RS.

- Type 3 PHR could be helpful for NW to determine PL offset value between DL/UL TRP and UL TRP.

# References

- RP-242394, WID revision: NR MIMO Phase 5, Samsung, RAN#105, Sept. 2024