3GPP TSG RAN WG1 #120	R1-2500537

Athens, Greece, February 17th – 21st, 2025

Source:	ZTE Corporation, Sanechips

Title:	Discussion on transmission power reduction for STxMP in TS 38.213

Agenda Item:	8.1

Document for:	Discussion and Decision

- Introduction

In this contribution, we elaborate our views on the transmission power reduction for STxMP in TS 38.213.

- Discussion

According to the agreement reached in RAN1#112 meeting [1], for STxMP, the maximum Tx power when the UE determines UL Tx power for the PUSCH/PUCCH transmission occasion(s) or antenna port(s) are intended to be discussed after receiving RAN4 reply on UE power limitation for STxMP in FR2. Till now, the configured UE maximum output power for STxMP has already been captured in RAN1 and RAN4 specifications.

According to clause 7.1 and clause 7.2 in TS 38.213, in case of STxMP, where UE determines to apply both the first TCI-State or TCI-UL-State and the second TCI-State or TCI-UL-State in a PUCCH/PUSCH transmission occasion, UE should determine PUCCH/PUSCH transmission power for the first and second TCI-State or TCI-UL-State based on the UE configured maximum output power for the first and second TCI-State or TCI-UL-State, respectively. The UE configured maximum output power for each of joint/UL TCI states indicated for simultaneous transmission to multiple directions are defined in RAN4 specifications.

According to the following conditions excerpted from the LS from RAN4 regarding the configured Tx power for STxMP in FR2 [2], we have the following considerations.

RAN4 would like to confirm to RAN1 the conditions to be simultaneously met by a UE for STxMP: 1. The total EIRP over all panels cannot exceed the existing EIRP limitation. 2. The total TRP over all panels cannot exceed the existing TRP limitation. 3. The sum of per-panel EIRP power limitation for STxMP can be greater than the existing EIRP limitation. a. See condition #1, which must always be obeyed. b. In cases where the beams for each panel have low enough spatial overlap, this clause allows EIRP for each panel to individually approach the existing EIRP limitation. c. RAN4 has not completed spec. definition work for ‘per panel EIRP power limitation’. 4. No explicit ‘per-panel’ TRP limit is deemed necessary.

- Condition-1: EIRP represents the limitation of the total transmission power of the entire device. Regardless of the number of panels, the total EIRP must not exceed this limitation.

- Condition-2: TRP focuses on the total radiated power of the entire device across all directions, while EIRP represents the maximum power in a specific direction. Both EIRP and TRP serve as respective metrics for presenting total Tx power limitation.

- Condition-3: Although the sum of per-panel EIRP power limitation may exceed the existing EIRP limit, during actual uplink transmission, the total EIRP still cannot exceed the limitation. This may require sufficient spatial isolation between beams to avoid excessive total EIRP caused by simultaneous high-power transmissions.

Meanwhile, according to the following excerpted from clause 6.2K.4 in TS 38101-2, it can be observed that the above conditions have been captured in RAN4 specifications.

### 6.2K.4 Configured transmitted power for simultaneous transmission to multiple directions <Unrelated parts are omitted> The configured UE maximum output power PCMAX,f,c,k shall be set such that the corresponding measured peak EIRP PUMAX,f,c,k is within the following bounds PPowerclass + ΔPIBE – MAX(MAX(MPRf,c,k, A- MPRf,c,k,) + ΔMPRSTxMP + ΔMBP,n, P-MPRf,c,k) – MAX{T(MAX(MPRf,c,k, A- MPRf,c,k,) + ΔMPRSTxMP), T(P-MPRf,c,k)} – ΔTSTxMP ≤ PUMAX,f,c,k ≤ EIRPmax and PUMAX,f,c, the corresponding measured peak EIRP for carrier f of a serving cell c, aggregated over all indicated joint/UL TCI states in a given direction, satisfies over all directions PUMAX,f,c ≤ EIRPmax while the corresponding measured total radiated power PTMAX,f,c is bounded by PTMAX,f,c ≤ TRPmax <Unrelated parts are omitted>

Based on the above analysis, we can assume that the sum of per-panel power limitation for STxMP can be greater than the existing power limitation for a given power class while the sum of actual transmission power for STxMP cannot be greater than the existing power limitation for a given power class.

For scenarios involving two uplink carriers, carrier aggregation and LTM, transmission power reduction operation is specified in clause 7.5 in TS 38.213 if the total UE transmit power for uplink transmissions on serving cells or on a candidate cell in a respective transmission occasion would exceed the linear value of maximum output power in the transmission occasion.

Therefore, for STxMP in a single carrier of a single serving cell, the total UE transmit power for simultaneously transmitted PUSCH or PUCCH on the serving cell in a frequency range in a respective transmission occasion i can exceed the linear value of PCMAX(i) in transmission occasion i. Then, transmission power reduction should also be operated to allocate power to the PUSCH/PUCCH transmissions so that the total UE transmit power for transmissions on the serving cell is not greater than PCMAX(i) in transmission occasion i.

In light of the above, we have the following proposal and the corresponding CR can be found in [3].

Proposal 1: Support transmission power reduction for multiple panel simultaneous uplink transmission of a single carrier of a single serving cell.

- Conclusion

In this contribution, we elaborate our views on the transmission power reduction for STxMP and have the following proposal.

Proposal 1: Support transmission power reduction for multiple panel simultaneous uplink transmission of a single carrier of a single serving cell.

- References

- Chair notes RAN1#112 eom, Feb 27th - Mar 3rd, 2023, 3GPP RAN1

- R4-2306657, Reply LS on configured Tx power for STxMP in FR2, Apr, 2023, RAN4

- R1-2500536, Draft CR on transmission power reduction for STxMP in TS 38.213, ZTE