3GPP TSG-RAN WG1 #120	R1-2500560

Athens, Greece, February 17th – 21st, 2025

Agenda Item:	9.1.1

Source:	Panasonic

Title:	Discussion on specification support for beam management

Document for:	Discussion

# Introduction

In Rel. 19, a new WID on AI/ML for NR air interface has been approved in [1], wherein beam management (BM) is one of the objectives, as shown in the following. This document will discuss on how to support AI/ML-based beam management.

- Beam management - DL Tx beam prediction for both UE-sided model and NW-sided model, encompassing [RAN1/RAN2]: - Spatial-domain DL Tx beam prediction for Set A of beams based on measurement results of Set B of beams (“BM-Case1”) - Temporal DL Tx beam prediction for Set A of beams based on the historic measurement results of Set B of beams (“BM-Case2”) - Specify necessary signalling/mechanism(s) to facilitate LCM operations specific to the Beam Management use cases, if any - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE NOTE: Strive for common framework design to support both BM-Case1 and BM-Case2

In the following, we discuss detailed aspects, including model inference, consistency between training and inference, and performance monitoring for DL Tx beam prediction.

# Discussion

## Model training and model inference

### NW-side model training

For NW-side model training, it has been agreed that for BM-Case1 and BM-Case2, RAN1 supports (i) to use existing CSI framework for configuration of Set A and/or Set B as the starting point and (ii) to use differential L1-RSRP reporting with legacy quantization step and range for quantization of a reported L1-RSRP value in L1 signaling.

In RAN1#118 meeting, FL proposed to discuss content for data collection via higher layer signaling as follows [2]

FL’s proposal 5.3: For content for data collection for training for NW-sided model via higher layer signaling, for both BM-Case 1 and BM-Case2, for each per instance, support the following options: - Opt 1: L1-RSRPs measured based on one resource set (for Set A and Set B) [FFS: two sets] configured to UE - Opt 2: L1-RSRPs measured based on one resource set (for Set A) [FFS: two sets] configured to UE, and beam information of Top K measured based on another resource set (for Set B) configured to UE - Differential L1-RSRP reporting is supported with legacy quantization steps and range - FFS on whether other values for quantization steps and/or ranges can be configured by NW. - FFS on whether/how to determinate a subset of L1-RSRPs, including - Alt 1: Top M beam(s) is the beams with largest M measured values of L1-RSRPs, where M is configured by gNB - Alt 2: All beams within X dB gap to the largest measured value of L1-RSRP - FFS on whether/how to define “per instance” - FFS on whether/how to contain time stamp information for BM-Case 2

For data collection for NW-sided model training, it is sufficient that gNB configures one resource set (for Set A and Set B) to UE and UE just reports the corresponding measured L1-RSRPs (aka Option 1). It is not necessary to distinguish 2 sets because gNB understands what it configures. Moreover, we understand a main motivation of Option 2 is about signalling overhead reduction. However, it might be not an issue for training as data collection for training for NW-sided model is supposed to be reported via higher layer signaling as main bullet point. Hence, there is no need of an option for signalling overhead reduction. Furthermore, regarding quantization steps and range, we think a legacy one can be reused to support differential L1-RSRP reporting. Having said that, we support Option 1 in the following.

Proposal 1: For content of data collection for NW-sided model training via higher layer signaling, for both BM-Case 1 and BM-Case2, for each per instance, support L1-RSRPs measured based on one resource set (for Set A and Set B) configured to UE

- Differential L1-RSRP reporting is supported with legacy quantization steps and range

- FFS: Details on reporting configuration

### NW-sided model inference

Configuration of Set A and Set B

For NW-sided model inference, it was agreed to support to report more than 4 beams related information in a beam report, which is initiated by network, based on a measurement resource set. For the measurement resource set, an existing CSI framework was agreed to be used for configuration of Set B as the starting point. Similarly, the existing CSI framework was also agreed to be used for configuration of Set A as the starting point.

Agreement For NW-sided model, for inference, in a beam report initiated by network, based on one measurement resource set, support the report of more than 4 beam related information in L1 signaling. - Note: Purpose, such as above “For NW-sided model, for inference”, will not be specified in RAN 1 specifications - FFS on the report content for beam related information. - FFS on max number of reported beam related information in one report. Agreement For network-sided AI/ML model for BM-Case1 and BM-Case2, - support using existing CSI framework for configuration of Set A as the starting point - support using existing CSI framework for configuration of Set B as the starting point - Note: Purpose, such as above “For NW-sided model, for BM-Case1 and BM-Case2” and “Set A” and “Set B”, will not be specified in RAN 1 specifications

Furthermore, for periodic CSI and semi-persistent CSI, UE can measure Set B in multiple past time instances subject to separate periods based on its implementation. However, for aperiodic CSI, UE needs to know how long to measure them for each event when model inference is triggered. Hence, gNB can configure UE with a measurement window for the measurement resource set, wherein the measurement window can include a number of measurement instances. We observe that a similar mechanism has already been specified in Rel. 18 MIMO to support UE-side CSI prediction. To support at least BM-Case2, this mechanism should be considered to be extended to support NW-sided AI/ML model. Therefore, we propose the following.

Proposal 2: NW-sided model inference, support to that a measurement window can be configured with the measurement resource set.

Beam reporting

Current specification supports a group-based beam reporting for non-AI/ML model, where a UE reports measurement results of beams in one report that it can receive either with the same Rx filter or with multiple Rx filters simultaneously. For single TRP case, UE reports measurement results of L=2 beams in one report. For multi-TRP case, UE reports L groups of simultaneously received beams (1<=L<=4, configured by parameter nrofReportedRSgroup) in one report, wherein each group includes measurement results of 2 beams and each beam in each group corresponds to one TRP. Hence, for NW-sided model inference, the group-based beam reporting can be enhanced to be used to report more than 4 beam related information. Hence, we propose the following.

Proposal 3: Group-based beam reporting can be enhanced to support to report more than 4 beams in one report for NW-sided model.

Beam indication

In RAN1#116, it has been agreed that for NW-sided model and UE-sided model, beam indication is based on unified TCI state framework in Rel. 17.

Agreement For NW-sided model and for UE-sided model, beam indication is based on unified TCI state framework - FFS on whether/how potential enhancement is needed

Regarding FFS point, we think one possible enhancement is about how to support beam indication for N future time instances in BM-Case 2. To do so, we support to extend Rel. 17 TCI state activation signaling methods to activate TCI states of K predicted beams for N future time instances in BM-Case2. The following 2 options can be considered:

- Option 1: The TCI states of K predicted beams for N future time instances are included in a combined set of TCI states together with that of legacy BM.

- Option 2: The TCI states of K predicted beams for N future time instances are included in a separate set of TCI states, compared to that of legacy BM.

Proposal 4: For NW-sided model, support to extend Rel. 17 TCI state activation signaling methods to activate TCI states of K predicted beams for N future time instances in BM-Case 2. The following 2 options can be considered:

- Option 1: The TCI states of K predicted beams for N future time instances are included in a combined set of TCI states together with that of legacy BM.

- Option 2: The TCI states of K predicted beams for N future time instances are included in a separate set of TCI states, compared to that of legacy BM.

### UE-sided model inference

Configuration of measurement set

Similar to NW-side model inference, for periodic CSI and semi-persistent CSI, UE can measure Set B in multiple past time instances subject to separate periods based on its implementation. However, for aperiodic CSI, UE needs to know how long to measure them for each event when model inference is triggered. Hence, gNB can configure UE with a measurement window for the measurement resource set, wherein the measurement window can include a number of measurement instances. Hence, we propose the following for UE-sided model inference.

Proposal 5: For UE-sided model inference, support that a measurement window can be configured with the measurement resource set.

Reference time of the earliest time instance for the predicted results

In RAN1#118bis meeting, for BM-Case 2 of UE-side model, it has been discussed 3 options for determining reference time of the earliest time instance for the predicted results as follows:

Agreement(118bis) For BM-Case 2 of UE-side model, for the reference time of the earliest time instance for the predicted results, consider at least the following options for potential down-selection: - Option 1: Based on the uplink slot for the report - Option 2: Based on the CSI reference resource corresponding to the report - Option 3: Based on the latest transmission occasion of the CSI-RS/SSB resource in Set B for measurement for the report, wherein the transmission occasion is no later than the CSI reference resource

We observe that Options 1 and 2 are based on a solution that is supported for CSI prediction in doppler domain in Rel. 18 MIMO, wherein a time duration between the latest measurement and the earliest prediction time instance is based on slot-level. For example, the earliest of prediction time instance start at slot l=n+δ, where n is the time slot for the report and the slot offset δ∈{-nCSI_ref,0,1,2} is configured by a higher layer parameter. From gNB perspective, Option 1 may maximize usable time (e.g., a duration of time that predicted results are still valid) for a possible subsequent scheduling after receiving a report of predicted results. However, it may increase a complexity of UE-side model as a beam predication might need to be performed without a reference time, such as for a case when a UL slot for the report is not configured.

On the other hand, Option 3 may be more beneficial to UE-side internal processing, which could simplify UE-side model inference, but it may reduce usable time of gNB for a possible subsequent scheduling. We observe that for BM-Case 2 of UE-side model, a reference time for defining a starting point of N future time instances might be different, and it might vary depending on the UE capability for running a UE-sided model. For example, for a powerful UE capability (e.g., high-end UE), a reference time can be started from the latest transmission occasion of measurement resource set with a small offset. For a less powerful UE capability (e.g., mid-/low-end UE), a reference time can be started from the latest transmission occasion of measurement resource set with a longer offset. For a given UE-sided model, a powerful/less powerful UE might require a specific (fixed) offset to run the UE-sided model inference, wherein the specific (fixed) offset might not be dependent on a basic of slot-level. Therefore, we support Option 3 as a baseline and FFS on the offset between the latest transmission occasion of the CSI-RS/SSB resource in Set B and the first-time instance for the predicted results. Hence, we propose the following.

Proposal 6: For BM-Case 2 of UE-side model, support that a reference time of the earliest time instance for the predicted results is determined based on an offset and the latest transmission occasion of the CSI-RS/SSB resource in Set B for measurement for the report, wherein the transmission occasion is no later than the CSI reference resource (i.e., Option 3)

- FFS: Details on offset between the latest transmission occasion of the CSI-RS/SSB resource in Set B and the first-time instance for the predicted results

Quantization of a RSRP value

In RAN1#118 meeting, for quantization of a RSRP value at least for the report of inference results, it has been agreed to support differential RSRP reporting with legacy quantization step and range for L1-RSRP reporting as follows:

Agreement For UE-sided model, for the quantization of a RSRP value at least for the report of inference results, support - Support differential RSRP reporting with legacy quantization step and range for L1-RSRP reporting - For BM-Case 1, support differential RSRP report among multiple beams - For BM-Case 2, support differential RSRP report among multiple beams over multiple time instances - FFS details

Regarding FFS point for BM-Case2, there could be 2 possible options as follows:

- Option 1: Determine the largest RSRP among multiple beams over multiple time instances

  - Report the largest RSRP and differential RSRP report among multiple beams over multiple time instances

- Option 2: Determine the largest RSRP among multiple beams per time instance

  - Report the largest RSRP and differential RSRP report among multiple beams per time instance

We would like to discuss pros/cons of these 2 options and any other option further.

Whether/how to differentiate predicted beams and measured beams

Current specifications support to report the measured beam(s) only. In particular, a report configuration (i.e., CSI-ReportConfig as described in TS38.331) indicates L1-RSRP related metrics (e.g., ssb-index-RSRP, or cri-RSRP) as the report quantity. The report configuration is associated with one or several resource sets (such as a set of SSB, a set of CSI-RS or both) on which the measurements should be performed.

Moreover, to support UE-sided model inference, in the last meeting, it has been concluded to take the current CSI reporting framework as the starting point to minimize specification impact. However, the current CSI reporting framework for beam management always assumes that the beams reported by UE have been actually measured by the UE. Therefore, it is necessary to discuss how to differentiate predicted beams and measured beams if the current CSI reporting framework is reused. In addition, if NW has observed some inconsistency in the prediction result, NW should be able to request UE to report more measurement results.

Conclusion For UE sided model at least for inference, for measurement, the configuration of Set B, - Take the current CSI framework as the starting point.

We observe there are two ways to realize above as follows:

- Option 1: To introduce prediction-related metrics as the reporting quantities.

  - For example, the most obvious metric could be predicted best beam ID. If NW sees benefit for obtaining other metrics, NW should be allowed to configure those metrics. Other metrics could be predicted beam quality such as predicted L1-RSRP, or L1-SINR, predicted beam application time, etc.

- Option 2: To introduce different resource sets in a report configuration for prediction and measurement.

  - In this option, two resource sets can be configured to the UE, wherein one set is for prediction and the other set is for measurement. In case that two sets are non-overlapping (containing different beams), if the reported beam belongs to set for prediction, NW knows it is the prediction result. On the other hand, if two sets are partially overlapping is allowed, further indication in the UE reporting would be required to indicate whether the report is based on prediction or measurement.

Proposal 7: To differentiate between prediction and measurement results, the following options can be considered:

- Option 1: To introduce prediction-related metrics as the reporting quantities.

- Option 2: To introduce different resource sets in a report configuration for prediction and measurement.

Indication of the association between Set A and Set B

To improve the beam prediction performance for UE-sided model inference, the indication from NW to UE on the association/mapping of beams within Set A and beams within Set B would be required such as QCL relationship. For example, Set A consists of narrow beams and Set B consists of wide beams.

Proposal 8: Support mapping/association of beams within Set A and beams within Set B based on QCL relationship.

## Consistency between training and inference

To ensure consistency of NW-side additional conditions across training and inference for UE-sided model, it has been introduced “associated ID” to further study and discuss, as shown in below agreement [3]. NW-side additional conditions might include two types of information as follows:

- Non-proprietary information: Configuration of RS resources of Set A and/or Set B, QCL configurations for Set A and/or Set B, configuration of association between Set A and Set B (if any), number of historic measurement results of Set B, etc.

- Proprietary information: NW-side beam shaping information (aka NW-vendor-specific implementation for beam shaping), such as DL spatial Tx filter (3dB beamwidth, codebook implementation for DL Tx beam boresight direction (azimuth and elevation), etc.), RF modules, antenna patterns, etc.

Motivations of introduction of “associated ID” are (i) to represent implicitly NW-side additional conditions (or to abstract these conditions in an implicit manner) and (ii) to let UE distinguish whether it collects data based on same or different NW-side additional conditions. When a UE receives a same “associated ID”, it understands that is the same configuration of NW-side additional conditions from gNB or NW.

Agreement Further study, for the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2, where the NW-side additional condition may at least impact UE assumption on beams of Set A/Set B: - Opt. 1: Based on associated ID (Referring to AI 9.1.3.3) - FFS on what can be assumed by UE with the same associated ID across training and inference - FFS on how associated ID is introduced, e.g., within CSI framework, or outside of CSI framework - Opt. 2: Performance monitoring based - FFS details - Other options are not precluded

For Option 1, an associated ID is assigned to a configuration of NW additional conditions (e.g., the configuration includes specific values of NW additional conditions), and it can be configured within CSI framework. For Option 2, to detect NW-side additional condition blindly is almost impossible from UE complexity perspective and it increases the latency to start the inference to NW-side additional condition is required. Hence, we support Options 1.

Regarding a concept of “associated ID”, it might cause a potential risk of disclosing proprietary information for NW vendor and/or NW operator. In RAN1#117 meeting, RAN1 assumed to restrict applicability of “associated ID” at least within a cell-level [4]. It implies that a same “associated ID” in different cells may represent different NW-side additional conditions. Hence, it causes a limitation of usage of UE-sided model training and model inference within a cell. It cannot handle a case that UE-sided model training and model inference are performed at different cells.

Working Assumption Regarding the associated ID for Rel-19, the UE assumes that NW-side additional conditions with the same associated ID are consistent at least within a cell. - FFS: whether/how UE assumption can be applicable for multiple cells (including the feasibility study)

We think that the concept of “associated ID” should be applicable for multiple cells because it can reduce the number of models to be trained and deployed as follows:

- It reduces dataset categorization at UE-side where data is collected from UEs in different cells.

- UE-sided model training and inference can be used in different cells of the same associated ID.

To preserve proprietary information for applying “associated ID” for multiple cells, we propose that “associated ID” should be managed per a NW operator (or MNO). In an example of usage for multiple cells, NW vendor(s) in a NW operator can be considered. The NW operator and each of NW vendor(s) share information of NW vendor identification (e.g., NW-vendor-specific ID(s)). NW operator utilizes the information of NW vendor identification to determine a set of associated IDs, which represent NW-side additional conditions, for each of NW vendor(s). The NW operator only signals a set of associated IDs to UE(s), so that UE(s) collect(s) data for training and inference based on the set of associated IDs. The NW operator does not share UE(s) the information of NW vendor identification and how NW operator utilizes the information to determine a set of associated IDs, so that UE(s) only see the set of associated IDs. By doing so, it can preserve the proprietary information for applying “associated ID” for different cells because of the following reasons:

- NW vendor does not share with each other about the proprietary information of NW-vendor-specific implementation for beam shaping as it is up to NW operator to determine “associated IDs”.

- NW operator preserves information of which NW vendor(s) are deploying in which area/site in different cells.

Fig. 1. NW operator utilizes NW-vendor-specific ID(s) to determine a set of associated IDs. The set of associated IDs are signalled to UE(s).

Proposal 9: Support to apply concept of “associated IDs” for multiple cells for ensuring consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2.

Proposal 10: Support to determine “associated IDs” within a NW operator (or an MNO) to preserve proprietary information.

In addition, in previous RAN1#118bis meeting, feature lead proposed the following proposals related to associated IDs [5].

FL’s proposal For the case that associated ID configured in CSI framework for UE-sided model, down selection - Alt 1: Associated ID is configured per CSI report configuration - Note: this means one associated ID per Set A and Set B pair - Alt 2: Associated ID is configured separately for Set A and Set B - Note: this means the associated ID can be different for Set A and Set B. FL’s proposal For the case that associated ID configured in CSI framework for UE-sided model, - Consider the following options for down selection on where to configure the associated ID - Alt 1: CSI-ReportConfig - Alt 2: CSI-ResourceConfig - Alt 3: CSI-ResourceSet

These proposals are depending on how RAN1 down-selects one of 3 options for applicability of UE-side model inference for Q4 of LS handling in Section 2.1, wherein associated ID can be configured within CSI-ReportConfig or not. We might revisit these proposals later.

## Performance monitoring

### Performance monitoring for UE-sided model

In RAN1#117 and RAN1#118bis, it has been agreed to support NW-side performance monitoring (i.e., Option 1) and UE-assisted performance monitoring (i.e., Option 2) of Type 1 performance monitoring for BM-Case1 and BM-Case2 with a UE-side AI/ML model as follows:

Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model: - Support Type 1 performance monitoring, including the following two options: - Option 1 (NW-side performance monitoring): - UE sends a report to NW (for the calculation of performance metric at NW) - Measurement results from resource set for monitoring, e.g., L1-RSRP and/or RS index is supported as the content of the report - FFS on other contents - The report is at least configured/triggered by NW - Note: this may or may not have additional spec impact - Option 2 (UE-assisted performance monitoring): - UE calculates performance metric(s) - FFS how to report and what to report - FFS whether to trigger the report based on event(s) for Option 1 and/or Option 2 - FFS Type 2 performance monitoring Agreement For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), - At least support Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/ resources for monitoring - FFS on detail definition of the metric, including whether/how to configure or define a window for calculation - FFS: on other details including how to configure the resource set/resources for monitoring, including - E.g. whether/how to use full set of Set A for measurement. If the full set A is not configured, whether/how to define the metric - FFS other alternatives

For Option 2 (UE-assisted performance monitoring), it has been agreed to support UE to calculate Top 1/Top K beam prediction accuracy (with or without margin) for performance monitoring. However, it is not concluded how to define a metric of beam prediction accuracy. Therefore, in RAN1#119 meeting, FL proposed to introduce a beam accuracy indicator (BAI) as a performance metric for beam prediction accuracy in the following proposal [6].

FL’s proposal 2.1-1b At least for BM-Case1 and at least for BM-Case2 with single time instance in a report with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), introduce a new quantity (i.e., beam accuracy indicator, BAI) for beam prediction accuracy in the CSI report for monitoring, - BAI = Np, and the accuracy corresponds to the BAI is Np/N. - FFS: the size of CSI field associated with the BAI is log2N - Where Np as c for which, FFS the following statement holds with potentially downselection (at least for the case that full set of Set A can be measured): - Option 1 (Top-1/1): the Top-1 beam with largest measured value of the resource set(s) for monitoring is Top-1 predicted beam - Option 2 (1/Top-K): the Top-1 beam with largest measured value of L1-RSRP of the resource set(s) for monitoring is one of the Top-K predicted beams - K >1, or K is equal to the number of reported predicted beam configured by inference report configuration - Option 3a (Top-K/M): the Top-K predicted beams are among Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of the resource set(s) for monitoring - K= 1, FFS other values - M is configured - K<=M - Option 3b (Top-K/M): at least one of the Top-K predicted beams is among Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of the resource set(s) for monitoring - M is configured - Option 4 (best of Top-K/1 with margin): The beam with largest measured value of L1-RSRP of Top-K predicted beams is within a margin X dB of largested measured value of L1-RSRP of the resource set(s) for monitoring - X=0, 1, FFS on whether to support other values - K=1 or K is equal to the number of reported predicted beam configured by inference report configuration, FFS other values - Further study on whether to update some definition of the description of above options if subset of Set A is used. - Note: the above options at least with the assuming that Top-K predicted beam are included in monitoring set(s). - Other options are not precluded. Note: The above options are at least for discussion purpose, whether to merge the down selected options can be considered.

Regarding BAI, we think that

- A same definition of BAI should be applicable for both BM-Case1 and BM-Case2. We do not see any benefit to define different BAI for these 2 use cases.

- If BAI would be introduced, it should be a ratio, instead of a number as proposed in FL’s proposal 2.1-1b. This is because a size of CSI field associated with the BAI should be a fixed size due to due to a limited size of UCI, so that it can avoid any potential increase in UCI size.

- To achieve a common understanding on calculation of BAI for performance monitoring between UE and NW, it is necessary to link reported inference result(s) and monitoring instance(s) in time domain. A simple solution is to define a monitoring window such that BAI can be calculated within the monitoring window. For details on the monitoring window, it can be further discussed. For instances, the following options can be considered

  - Alt. 1: A starting point and a length of monitoring window can be configured explicitly by NW

  - Alt. 2: A length of monitoring window can be configured explicitly by NW and its starting point is implicitly determined based on a rule, e.g., it starts from a first OFDM symbol after receiving a dedicated CSI report configuration for monitoring.

  - Alt. 3: A starting point and a length of monitoring window can be implicitly determined.

Having said that, we propose the following

Proposal 11: For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), support to define a beam accuracy indicator (BAI) as a metric of beam prediction accuracy

Proposal 12: For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, support to define a monitoring window for BM-Case1 and BM-Case2 such that BAI can be calculated within the monitoring window

- FFS: Details on the monitoring window

### Performance monitoring for NW-sided model

NW monitors the performance metric(s) and makes decision(s) of model selection/activation/ deactivation/switching operation. To support NW monitors the performance metric(s), it is required UE to report the measurement results of more than 4 beams. In this case, the proposal 2 can be used.

Proposal 13: Group-based beam reporting can be enhanced to support performance monitoring for NW-sided model.

# Conclusion

In this document, we have discussed some details on AI/ML for beam management. We have the following proposals:

Proposal 1: For content of data collection for NW-sided model training via higher layer signaling, for both BM-Case 1 and BM-Case2, for each per instance, support L1-RSRPs measured based on one resource set (for Set A and Set B) configured to UE

- Differential L1-RSRP reporting is supported with legacy quantization steps and range

- FFS: Details on reporting configuration

Proposal 2: NW-sided model inference, support to that a measurement window can be configured with the measurement resource set.

Proposal 3: Group-based beam reporting can be enhanced to support to report more than 4 beams in one report for NW-sided model.

Proposal 4: For NW-sided model, support to extend Rel. 17 TCI state activation signaling methods to activate TCI states of K predicted beams for N future time instances in BM-Case 2. The following 2 options can be considered:

- Option 1: The TCI states of K predicted beams for N future time instances are included in a combined set of TCI states together with that of legacy BM.

- Option 2: The TCI states of K predicted beams for N future time instances are included in a separate set of TCI states, compared to that of legacy BM.

Proposal 5: For UE-sided model inference, support that a measurement window can be configured with the measurement resource set.

Proposal 6: For BM-Case 2 of UE-side model, support that a reference time of the earliest time instance for the predicted results is determined based on an offset and the latest transmission occasion of the CSI-RS/SSB resource in Set B for measurement for the report, wherein the transmission occasion is no later than the CSI reference resource (i.e., Option 3)

- FFS: Details on offset between the latest transmission occasion of the CSI-RS/SSB resource in Set B and the first-time instance for the predicted results

Proposal 7: To differentiate between prediction and measurement results, the following options can be considered:

- Option 1: To introduce prediction-related metrics as the reporting quantities.

- Option 2: To introduce different resource sets in a report configuration for prediction and measurement.

Proposal 8: Support mapping/association of beams within Set A and beams within Set B based on QCL relationship.

Proposal 9: Support to apply concept of “associated IDs” for multiple cells for ensuring consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2.

Proposal 10: Support to determine “associated IDs” within a NW operator (or an MNO) to preserve proprietary information.

Proposal 11: For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), support to define a beam accuracy indicator (BAI) as a metric of beam prediction accuracy

Proposal 12: For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, support to define a monitoring window for BM-Case1 and BM-Case2 such that BAI can be calculated within the monitoring window

- FFS: Details on the monitoring window

Proposal 13: Group-based beam reporting can be enhanced to support performance monitoring for NW-sided model.

# Reference

[1]. RP-234039, “New WID on AI/ ML for NR Air Interface”, 3GPP, RAN#102.

[2]. R1-2407604/RAN2-2407848, “LS on applicable functionality reporting for beam management UE-sided model”, Hefei, China, 2024.

[3]. R1-2403756, “FL summary #5 for AI/ML in management”, RAN1#116-bis, Changsha, China, 2024.

[4]. Online, “Chair notes RAN1#117”, Fukuoka, Japan, 2024.

[5]. R1-2409305, “FL summary #5 for AI/ML in management”, RAN1#118bis, Hefei, China, 2024.

[6]. R1-2410892, “FL summary #5 for AI/ML in management”, RAN1#119, Orlando, US, 2024.