3GPP TSG RAN WG1 #120	R1-2500611

Athens, Greece, February 17th – 21st, 2025

Agenda Item:	9.4.2

Source:	NEC

Title:	Physical layer design – line coding, FEC, CRC, repetition aspects

Document for:	Discussion and Decision

1	Introduction

In RAN#106 meeting, a new WID (RP-243326)[1] is proposed with RAN1 details below.

The following objectives are set, within the General Scope: - RAN1 scope: - PRDCH and PDRCH, which are the only physical channels in R2D and D2R, respectively. - R2D and D2R signal(s) - Multiplexing/multiple access in R2D is by only TDMA, and in D2R is by only TDMA and FDMA. - R2D supports only OOK-4 modulation, one solution for CP handling. D2R backscattering supports only OOK and BPSK modulations. - R2D transmission supports only the Manchester line code in TR 38.769 - D2R transmission supports: - Either the Manchester line code in TR 38.769 or no line code (one to be down-selected); and - A corresponding small frequency shift method according to the options in TR 38.769. - R2D does not support FEC. D2R supports only convolutional code with generator polynomials as per TS 36.212 (unless RAN1 decides to use other generator polynomials by RAN1#120bis). - PRDCH and PDRCH both support transmission without CRC, and with CRC as per the generator polynomials in TS 38.212 (unless RAN1 decides to use other generator polynomials by RAN1#120bis) for 6-bit CRC and 16-bit CRC. Cases to use which length of CRC, or no CRC, to be decided in RAN1. - D2R supports physical layer repetition transmission. R2D does not support physical-layer repetition transmission.

In this paper, we discuss and give our views on the aspects of line coding, FEC, CRC and repetition of ambient IoT physical channel design.

2	Discussion

2.1	Line coding

For D2R transmission, whether Manchester line coding or no line coding will be discussed. As we can see, Manchester line coding could be regarded as a XOR of bit without line coding and bit of clock that the frequency of the clock equals to the bit rate without line coding, i.e., one bit duration without line coding equals to two chips {0, 1} duration of clock. We could have a unified design that treat Manchester line coding as a small frequency shift of transmission of no line coding.

Proposal 1: Specify a unified design that treat Manchester line coding as a small frequency shift of transmission of no line coding.

2.2	Channel coding

LTE TBCC could be used as the baseline for ambient IoT’s FEC. It has two issues to be discussed when applies TBCC for ambient IoT. The first one is shift register initialization requiring at least 6 bits for transmission. In LTE, the channel coding used by TBCC is expected to be larger than 6. Otherwise, other channel coding method (e.g., Reed Muller) is used. In ambient IoT, we need to first discuss whether the number of TBS less than 6 is possible. And then how to initialize TBCC shift register if less than 6 bits transmission is allowed.

Proposal 2: In ambient IoT, we need to first discuss whether D2R transmission with number of TBS less than 6 bits is possible. And then how to initialize TBCC shift register if less than 6 bits transmission is allowed.

The second one is rate matching of TBCC. Rate matching to reduce number of encoding bits for good channel condition devices can reduce transmission latency and increase system performance. However, the rate matching mechanism in LTE is too complex for ambient IoT devices. It benefits to look for a simple and effective rate matching procedure for ambient IoT.

Proposal 3: Specify a simple and effective rate matching procedure for ambient IoT.

2.3	CRC

For data transmission, CRC attachment is effective for receiver to determine whether the reception is successful. However, in a few cases, CRC may not be needed and its overhead can be saved. For example, when device transmit Msg1 with 16-bit ID, adding 6-bit CRC is redundant. When reader responses Msg2 to each device, each device only needs to detect the information related to its ID, but they have to receive the whole Msg2 if CRC is added. Such a behaviour is energy consuming, and not friendly to energy-limited AIoT device.

Proposal 4: CRC attachment is exempted for the following transmission:

- D2R Msg1 transmission;

- R2D Msg2 response.

2.4	Repetition

Repetition is study as a solution to address the possible coverage problem for D2R transmission. In TR 38.769 [2], several repetition schemes were included, such as block-level repetition and two types of bit-level repetitions. The block level repetition is repeated in terms of blocks, while the bit level repetition is repeated in terms of bits, which are shown in Fig.1. For block-level repetition, longer time difference between the repetitions provides additional gain of time division while increases the complex of buffering longer signal, and bit-level repetition is at the other side. Hence, to provide more flexibility in balancing or adjusting between performance and complexity/device’s capability, a hybrid repetition scheme of block repetition and bit repetition can be studied.

Fig.1 Examples of repetition schemes

Proposal 5: Support study hybrid repetition scheme of bit level repetition and block level repetition as a complement to provide more flexibility on adjusting between performance and complexity.

Bit 0 and bit 1 were agreed with mapping to chip {10} and chip {01} for Manchester encoding. The same time duration for high-level power and low-level power is applied to bit 0 and bit 1, and the difference for the chips after coding is the power switching direction.  However, different time durations are applied for PIE encoding bit 0 and bit 1, which may lead to a different detection probability of the two bits. To balance the time length occupied by high-level power or low-level power of the two bits, switching the mapping relationship to each other on part of the repetitions can be considered. And to align the scheme between different coding strategies, it can be applied to Manchester also. For example, map bit 0 to chip {10} for the 1st repetition and map bit 0 to chip {01} for the 2nd repetition. Or flip the source bits after CRC attachment or FEC coding when number of repetitions larger than 1 is indicated, as shown in Fig.2 where the repetition number is 4 with Manchester. In the figures, source bits are used for repetition #0 and repletion #2, and flipping bits based on source bits are used for repetition #1 and repetition #3 with the same mapping relationship between bits and chips.

Fig.2 Examples of flipping bits with repetition

Proposal 6: Support study flipping source bits for different repetitions before mapping to chips when multiple repetitions are applied.

2.5	Small frequency shifts

In TR 38.769 [2], small frequency shift is discussed as copied below.

##### 6.1.2.7.1 Small frequency shifts For OOK and BPSK, small frequency shifts are studied: - For applying with Manchester line codes with a repetition number R ≥ 1: - Option 1: Each Manchester codeword is repeated by a codeword repetition number R, within the same time duration Tb corresponding to an information bit, where R = Tb/(2 × chip length), such that the amount of small frequency shift in Hz is R/Tb = 1/(2 × chip length). - Option 2: By multiplying the Manchester codeword with a square wave corresponding to the small frequency shift, the time duration Tb corresponding to each information bit includes R number of square wave periods, where R = Tb/(2 * chip length), such that the amount of small frequency shift in Hz is R/Tb = 1/(2 × chip length). The multiplication operation is performed as either an XOR or XNOR operation between a Manchester codeword corresponding to the information bit and the square wave for the small frequency shift. - If no D2R line code is used, by using a square-wave corresponding to the small frequency-shift, the time duration Tb corresponding to each information bit includes R number of square wave periods generated by 2R OOK chips [0, 1, 0, 1 …]/[1, 0, 1, 0 ..] or BPSK chips [-1, +1, -1, +1, …]/[+1, -1, +1, -1, …], such that the amount of small frequency shift in Hz is R/Tb.

When applying small frequency shift with R, the chip of duration is reduced to 1/R of the original chip. For example, as shown in Fig. 3, the original chips after small frequency shift are transformed to an effective chip with 1/R duration of the original chip. However, after small frequency shift operation, the tolerance of synchronization error is actually reduced. For the original chip, as long as the synchronization error is not larger than 50% of its chip duration, the synchronization error does not introduce much inter-chip interference to affect detection. But for the effective chips after small frequency shift, the synchronization error tolerance is reduced to 1/R of the original chip duration, which means more midamble is required for rectifying synchronization error.

Fig.3 Examples of chips with small frequency shift R

Proposal 7: When applying small frequency shift, the density of ambles, at least midamble, is increased by small frequency shift factor R.

The second issue of small frequency shift is to maintain frequency orthogonality of among different devices. Since the density of ambles are increased, the gap of chips Tgap between amble should be adjusted. For example, the gap of chips between two neighbouring midambles, or the gap of chips between the preamble and the first midamble should be carefully determined. If Tgap is not multiples of R, it means the chips of one information bit after small frequency shift is truncated by one amble, e.g., midamble. In frequency domain, it means that the small frequency shift of the information bit is not completed, and it would cause interference to other devices. An example is shown in Fig.4.

Fig.4 Examples of chips with small frequency shift R

Proposal 8: When applying small frequency shift, the number of effective chips after small frequency shift should be multiples of R between nearby ambles.

Similar cases are also applicable to ambles after small frequency shift. To avoid inter-device interference, ambles, including preamble (at least the timing acquisition part), midamble and postamble (if any) should be transmitted with small frequency shift. As the discussion before, the density of ambles increases after small frequency shift. However, the overhead of ambles is expected to maintain the same so that the transmissions among different device are roughly aligned. To achieve the same overhead, we suggest that the number of chips of amble should be multiples of R, and the overhead of each amble after small frequency shift is squeezed to 1/R. Take Fig.4 as an example. The example of X-amble (X=pre, mid or post) takes 12 chips, and after small frequency shift with factor R, the amble consists of 12 effective chips, and only occupies 1/R duration of the original amble.

Fig.4 Examples of X-amble after small frequency shift R

Proposal 9: To avoid increase of amble overhead, the number of chips of each amble should be multiples of R.

3	Conclusion

In this contribution, we give our views on the aspects of line coding, FEC, CRC and repetition of ambient IoT physical channel design. We propose that:

Proposal 1: Specify a unified design that treat Manchester line coding as a small frequency shift of transmission of no line coding.

Proposal 2: In ambient IoT, we need to first discuss whether D2R transmission with number of TBS less than 6 bits is possible. And then how to initialize TBCC shift register if less than 6 bits transmission is allowed.

Proposal 3: Specify a simple and effective rate matching procedure for ambient IoT.

Proposal 4: CRC attachment is exempted for the following transmission:

- D2R Msg1 transmission;

- R2D Msg2 response.

Proposal 5: Support study hybrid repetition scheme of bit level repetition and block level repetition as a complement to provide more flexibility on adjusting between performance and complexity.

Proposal 6: Support study flipping source bits for different repetitions before mapping to chips when multiple repetitions are applied.

Proposal 7: When applying small frequency shift, the density of ambles, at least midamble, is increased by small frequency shift factor R.

Proposal 8: When applying small frequency shift, the number of effective chips after small frequency shift should be multiples of R between nearby ambles.

Proposal 9: To avoid increase of amble overhead, the number of chips of each amble should be multiples of R.

References

[1]	RP-234058, New SID: Study on solutions for Ambient IoT (Internet of Things) in NR, 3GPP TSG RAN Meeting #102, December 11-15, 2023.

[2]	3GPP TR 38.769 Study on solutions for ambient IoT (Internet of Things)