3GPP TSG RAN WG1 #120									R1-2500656

Athens, Greece,  17 – 21 Feburary 2025

Agenda Item 	:	9.6.1

Source 	:	Sony

Title 	:	LP-WUS and LP-SS design

Document for 	:	Discussion and decision

# Introduction

During RAN1#119 [1], aspects related to details of LP-WUS and LP-SS design were further discussed, and the following agreements were made. Agreements related to earlier meetings can be found in the appendix.

Agreement Update the existing agreement as shown below: In case of overlaid OFDM sequence carrying information, support option 2: - Option 2: One sequence is selected from multiple candidates overlaid OFDM sequences on each OOK ‘ON’ symbol, and OFDM-based LP-WUR obtain LP-WUS information at least by overlaid OFDM sequence(s). Consider the following two sub-options for potential down-selection. - Option 2-1: The overlaid OFDM sequence(s) carry part of information bits of LP-WUS. OFDM-based LP-WUR can obtain the whole information bits by OFDM sequence(s) and location of the OFDM sequence(s)/OOK ‘ON’ symbols. - Option 2-2: The overlaid OFDM sequence(s) carry all information bits of LP-WUS. OFDM-based LP-WUR can obtain the whole information bits by the overlaid OFDM sequence(s) - FFS the how the information bits are carried by the overlaid OFDM sequence(s) Note: the overlaid OFDM sequence in each OOK ‘ON’ symbol can be different according to information bits to be carried by the overlaid OFDM sequence within the LP-WUS. Agreement For the overlaid OFDM sequence in time domain, the sequence is generated based on , - M=1, is given by the largest prime number such that,is the overlaid OFDM sequence length. - The base overlaid sequence is generated by extension of - , - With CS(s) applied to the base overlaid OFDM sequence if any:denotes the potential cyclic shift (s) - ,, - M=2, 4, is down-selected from the following: - Alt1: is given by the largest prime number such that,is the overlaid OFDM sequence length. - The base overlaid sequenceis generated by extension of - , - With CS(s) applied to the base overlaid OFDM sequence if any:denotes the potential cyclic shift (s) - , - Note it doesn’t preclude any pulse shaping scheme if any. - Alt2:is given by the largest prime number such that, is the overlaid OFDM sequence length. - The base overlaid sequenceis generated by inserting zeros before and/or after . The total number of zeros is. - For example,, - With CS(s) applied to the base overlaid OFDM sequence if any: denotes the potential cyclic shift (s) - For example,, - where, - FFS the value of G, with G>0. - FFS value(s) of root q and/or CS for generating the candidate sequences if applied - FFS on whether changes are needed to handle intra-/inter-cell interference - Note: the overlaid OFDM sequence in time domain is based on potential modification to ZC sequence as listed above. - Above overrides any previous RAN1 agreement / working assumption Agreement At least for M>1, for the overlaid OFDM sequence in time domain, down-select the sequence length LZC from the following: - Alt1:, with - FFS how to map the generated overlaid OFDM sequence to the frequency domain - Alt2: - Note: X is the number of RBs of LP-WUS/LP-SS bandwidth (blanked guard RBs are not included) Agreement CS(s) and/or root(s) used for overlaid OFDM sequence in the time domain are derived from RRC signalling: - FFS: the set of values of CS and root for configuration - FFS: details of the RRC signaling Agreement For the length L of LP-SS binary sequence, limit the selection to the following: - M=1, L= {4, 6, 8} - M=2, L= {8, 12, 16} - M=4, L= {16, 32} Agreement To compare LP-SS binary sequence performance, cross-correlation is deprioritized over sync accuracy, i.e., cross-correlation within a set of 4 sequences is further compared for different sets of sequences with similar sync accuracy. Agreement For the M value for LP-WUS and LP-SS, down-select one alternative from the following in RAN1#120: - Alt1: The M values for LP-WUS and LP-SS are always same. - Alt2: The M values for LP-WUS and LP-SS can be configured to be same or different. M value for LP-WUS cannot be larger than that of LP-SS. - Alt3: The M values for LP-WUS and LP-SS can be configured to be same or different. Agreement Regarding the maximum number of candidates overlaid sequences to carry LP-WUS information per OOK ON chip for one cell: - support maximum 4 candidates overlaid sequences for M=4 For future meetings: Companies are encouraged to consider [H][FL4] Proposal 3.3-1 in R1-2410838 Agreement Regarding whether to support additional sync signal to LP-SS, down-selection from the following: - Option 1A: No additional sync signal, LP-SS periodicity =320ms - Additionally support at least one of [80ms,160ms] - FFS: whether different values can be applicable for different M values - Option 1B: No additional sync signal, LP-SS periodicity =320ms - No additional support of other periodicities - Option 2: With additional sync signal, LP-SS periodicity =320ms - FFS additionally support other LP-SS periodicities - FFS details on additional sync signal - FFS additional sync signal is configurable and/or conditionally present For future meetings: Companies are encouraged to consider the following for future discussions Regarding the LP-WUS information to trigger PDCCH monitoring of RRC connected UEs, - Option 1: A bitmap with each bit corresponding to [one or more] UEs - 1A No CRC - Per bit coding, e.g., one bit is mapped to one of 2 codeword/sequences - The maximum number of codewords targeting per bit position in the bitmap for a UE is 1 - FFS coded bit length - Single MO - 1B No CRC - Bitmap-level coding, e.g., one bitmap of L bits is mapped to one of 2L codeword/sequences - The maximum number of codewords/sequences targeting for a UE is 2L-1 - Single MO - 1C With CRC - FFS CRC length - Single MO - Option 2: A codepoint value corresponding to [one or more] UEs - 2A only 1-1 mapping from codepoint to UE - The maximum number of codepoints targeting per MO for a UE is 1 - FFS the maximum number of MOs to be detected by a UE - 2B 1-to-1, 1-to-all mapping from codepoint to UEs - The maximum number of codepoints targeting per MO for a UE is up to 2 - FFS the maximum number of MOs to be detected by a UE - 2C 1-to-X mapping from codepoint to UEs, - The maximum number of codepoints targeting per MO for a UE is up to - FFS the maximum number of MOs to be detected by a UE - Note: L is the number of UEs associated monitoring the same WUS Agreement For RRC idle/inactive state, for the case where associated CD-SSB and initial DL BWP have the same SCS at least when the associated CD-SSB and LP-WUS are on the same carrier - The single SCS for LP-WUS/LP-SS is same as the associated CD-SSB - FFS: Case where associated CD-SSB and initial DL BWP have different SCSs - FFS: Which initial BWP for RedCap Agreement For collecting LP-SS binary sequence results - Endorse the template for collecting LP-SS binary sequence results in R1-2410925 - Email discussion for collecting LP-SS binary sequences by 10th Jan. - Please include your cross-checking results (e.g., a separate excel sheet) in the t-doc for #120.

In this document, we describe our view on LP-WUS and LP-SS designs based on the progress of the work in the previous meeting.

# LP-WUS design

## OOK-based LP-WUS

Let us start by discussing our view on OOK-1/OOK-4 waveform design.

OOK-1/OOK-4 waveform

To tailor LP-WUR/WUS scheme to the NR protocol, the OOK-based LP-WUS transmission needs to be compliant with the transmission principles of NR, i.e., based on OFDM transmission. In other words, to avoid interfering with the OFDM transmission, the LP-WUS needs to be orthogonal in the same way as OFDM sub-carriers are, despite being based on different modulation principles. Two alternative OOK-based LP-WUS transmission schemes, namely OOK-1 and OOK-4, were selected after the Rel-18 study period and during RAN1#116 it was agreed to support both approaches.

Agreement Support both OOK-1 and OOK-4 for LP-WUS. - FFS how OOK-1 and OOK-4 are specified - For OOK-4, M<=4, FFS supported values - The SCS of a CP-OFDM symbol used for LP-WUS generation can be the same as one of the SCS(s) used for other NR transmissions in the same CP-OFDM symbol - FFS different SCS.

Below we give a detailed description of OOK-4 generation, where OOK-1 can be considered as a special case where the number of bits per OFDM symbol is limited to one bit. Figure 1 shows a block diagram of the modified OFDM transmitter, including the two blocks used to create the embedded WUS, allowing OOK-4 transmission scheme. The first block (A) is a sectioning block creating sections of the target LP-WUS and is only used if the length of the LP-WUS becomes longer than one OFDM symbol. The WUS shaping block (B) then shapes each LP-WUS section so that the resulting signal as output from the OFDM modulator is as close to the target LP-WUS as possible using the designated sub-carriers. Step (C) to (D) adds CPs to maintain orthogonality in channels with time dispersion.

Figure 1 – Block diagram of modified OFDM transmitter (for OOK-4 transmission mechanism)[4]

Two alternatives can be considered when generating OOK-based LP-WUS based on the above scheme.

- alternative 1 – We assume the removal of CP section is handled at the receiver side, for instance through higher sampling rate or capability to shift the sample instant by non-integer multiple of bit length, equal to the length of the CP.

- alternative 2 - the transmitter handles the signal generation, illustrated in Figure 2. With this not only we can keep the total length of the resulting signal the same as the original LP-WUS signal, but also we can maintain a regular LP-WUS signal bit period, allowing for low-complexity reception, i.e., no need for any knowledge regarding CP length or higher sampling rate at LP-WUR.

In our simulation, see appendix, we compare results for both approaches.

Figure 2 - Illustration of input/output signals of each block in the transmitter of Fig. 1, using a simple example.

In principle, assuming the same LP-WUS transmission bandwidth, the fewer the number of bits being fitted in one OFDM symbol, the lower the bit rate (or the longer length) the OOK-based LP-WUS becomes. This means that the OOK-4 transmission scheme with higher number of bits has advantages compared to the OOK-1 waveform generation candidates or OOK-4 with M=1 in terms of system overhead, latency and power consumption. This is since OOK-4 with higher M can lead to lower system overhead, shorter delay and lower total power consumption.

Observation 1 – OOK-4 with M>1 has lower system overhead and can be detected with shorter delay using lower power LP-WUR, compared to OOK-1 or OOK-4 candidates with lower values of M.

Further details on OOK-4/OOK-1 waveform design on

- M value for OOK-4

- How to specify OOK-1 and OOK-4 waveforms

- SCS configuration for LP-WUS

are still under discussion. In the following we describe our view on them.

M value for OOK-4 – In our view, the maximum number of bits per OFDM symbol, M, needs to be decided based on LP-WUS bit rate and, preventing ISI and the tolerable time/frequency errors by the LP-WUR. The number of bits that can be accommodated in one OFDM symbol can further be calculated knowing the SCS. For a LP-WUS with bit rate Rb, the number of bits per OFDM symbol M is equal to Rb/SCS. During both study and normative work M value of 4 has been chosen as working assumption. The following was agreed during the RAN1#118.

Agreement [H][FL1] Proposal 3.1-1: Confirm the Working Assumption that OOK-4 with M=4 for 15KHz SCS is supported for LP-WUS. Agreement For OOK-4 with M >1, support M=2 & M=4 (working assumption) for LP-WUS. - M=4 for 15KHz SCS - M=4 for 30KHz SCS (working assumption) - FFS M=1 for OOK-4.

For SCS of 30kHz, this value allows for a bit rate of 120 kbps, making it resilient to TDL-C fading channel. If the OFDM transmitter is configured with 15kHz SCS, then the maximum number can become twice as high, i.e., M = 8 without any degradation in LP-WUS performance.

As discussed earlier, OOK-4, larger value of M has lower system overhead. This leads in return to both lower power consumption and shorter delay using lower power LP-WUR, compared to OOK-1 or OOK-4 with other values than the maximum. To support the full advantages of LP-WUS design we propose to support M with its highest possible value for the channel in hand.

Observation 2 - The maximum number of bits per OFDM symbol, M, needs to be calculated based on LP-WUS bit rate and according to channel delay spread, preventing ISI and the tolerable time/frequency errors by the LP-WUR.

Proposal 1 – RAN1 to support setting M value to its maximum, i.e., M=4 for 30 kHz SCS and M=8 for 15 kHz to allow for low system overhead, lower latency and less power consumption at the LP-WUR.

How to specify OOK-1 and OOK-4 – As mentioned earlier, OOK-1 can be considered as a special case where the number of bits per OFDM symbol is limited to one bit. Transmission of the signal is to a large extent  transparent to the OOK-based LP-WUR, for both alternatives described above. The only part that the LP-WUR and the transmitter needs to agree on is basically bit rate. This is since the design of last low-pass filter in the OOK-based LP-WUR architecture, before bit detection, will be influenced when changing the number of bits per OFDM symbol. For alternative 2, the UE needs to also know the CP length to be able to adjust its sampling, this is however the same for both OOK-1 and OOK-4 for the same SCS configuration.

The information related to bit rate, and therefore number of bits per OFDM symbol (M), has to be exchanged as UE capability. The gNB can easily generate the suitable bit rate based on this information. Thus, there is no need for a separate specification when generating OOK-1 and OOK-4 waveform and a unified specification can easily be used for both.

Proposal 2 – RAN1 to support to use a unified specification for both OOK-1 and OOK-4.

SCS configuration for LP-WUS –In our view using the same SCS configuration as other NR has less complexity for gNB. If the LP-WUS signal is generated based on alternative 1 described above, the SCS configuration needs to be known by the UE, so that the LP-WUR can handle removal of CP at the LP-WUR.

Observation 3 – Using the same SCS configuration as other NR has less complexity for gNB.

Proposal 3 – RAN1 to support the same SCS configuration as other NR as it has less complexity for gNB.

## OFDM-based LP-WUS

An OFDM-based LP-WUS design can have the following advantages:

- There is more capacity to carry more signalling information, e.g., for use in identifying UEs or UE groups

- The RRM measurements regime does not have to change too drastically – measurements such as RSRP and RSRQ can be carried out on the existing PSS/SSS signalling.

- An OFDM LP-WUS can more easily deal with frequency and time offsets because it has to incorporate means of detecting and correcting these since a coherent receiver is needed.

- An OFDM LP-WUR operates in WUS windows as an OFDM LP-WUS would have similar coverage to the main signal and so can easily be detected within a short time window, this is particularly when the UE is within the coverage of PDCCH.

An OFDM LP-WUR, however, consumes more power than an OOK-based LP-WUR because of the need for coherent reception and I, Q processing. The increased power consumption of the OFDM based LP-WUR can be mitigated by:

Reducing the bandwidth of the LP-WUS compared to the bandwidth of the component carrier. The lower sampling rate engenders less power consuming DSP for the LP-WUR. It is expected that while the OFDM LP-WUR power consumption reduces drastically compared to its main receiver, its power consumption can still be more than several times higher than an OOK-based LP-WUR.

Modulating the OFDM symbols of the LP-WUS with known quadrature sequences and carrying the signalling data in some property of the modulating sequences instead of using QAM modulation of the OFDM subcarriers. This allows decoding of the signalling without the need for channel estimation, de-mapping, FEC etc.

Concentrating the whole LP-WUS in a few OFDM symbols so that the LP-WUR is only ON for a short time. This is particularly beneficial as the OFDM based LP-WUS can reach the coverage of PDCCH coverage without any need for repetition, compared to the OOK-based LP-WUS.

Figure 3: Sequence carried on each OFDM symbol for OFDM-based LP-WUS

Figure 3 illustrates how the dot product of a ZC sequence S(k) and m-sequence P(k) can be used to modulate LP-WUS OFDM symbols. By choosing P(k) based on the cell ID, the LP-WUS transmitted can be made cell-specific.

The LP-WUR operates on the same bandwidth as is used by the SSBs. The LP-WUR can therefore also be used to carry out RRM measurements on legacy SSBs. In this case, the SSBs can be used as LP-SS too.

## LP-WUS structure and its content

The same LP-WUR is expected to be used to receive LP-WUS in IDLE/INACTIVE and CONNECTED mode. To specify a LP-WUS design applicable for both modes of operation, we need to understand the potential subsequent actions after LP-WUS detection.

In IDLE/INACTIVE state the LP-WUS transmitted can be used to power up the main receiver before paging. In order to avoid false wake-up of the UEs, a further sub-grouping of the UEs listening to the same paging DRX-ON can be done. In CONNECTED mode, on the other hand, the potential sub-sequent action not only can be related to the monitoring of the next DRX-ON but also whether to advance or delay the DRX-ON monitoring due to jitter in the XR applications. Again, it means different content/sequences are needed to wake-up the UE for different purposes. In the CONNECTED mode, the UE can only be addressed individually. Therefore, the LP-WUS design should at least allow for

- different content/sequences for different subsequent actions/purposes, and

- addressing of group of UE or individual UEs, depending on the mode of operation.

These two pieces of information can be included as a data field in the LP-WUS structure. Time and frequency multiplexing can further be used to further sub-group the UEs.

Additionally, to allow for fine synchronization and to prevent the NW from always-on transmission of LP-SS with short periodicity, it is beneficial that the LP-WUS also includes a preamble prior to its data information, see also section 3. Another advantage of adding such a preamble is reduced power consumption at the UE while monitoring for LP-WUS. The LP-WUR first needs to only look for the preamble and only continues to detect the data if the preamble is detected. Figure 6 shows the general signal structure used in this contribution containing sequences of length M and L bits, representing preamble and data, respectively, where data part is to indicate the subsequent action by the UE after LP-WUS detection and/or identity of (group) UE.

Figure 6 - LP-WUS structure, consisting of two fields, preamble used for synchronization and possibly cell identity purposes and data part to indicate the potential subsequent action or UE (group) identity.

As described earlier, Time and frequency multiplexing can further be used to further sub-group the UEs. By allowing to transmit the LP-WUS in smaller bandwidths, we can accommodate and address higher number of UEs in the time-frequency grid. We discuss our view on this also in our companion contributions [1] and [3].

Observation 4 - To allow for fine synchronization and to prevent the NW from always-on transmission of LP-SS with short periodicity, it is beneficial that the LP-WUS also includes a preamble prior to its data information.

Observation 5 – A use of a preamble before LP-WUS data part results in reduced power consumption at the UE while monitoring for LP-WUS. The LP-WUR first needs to only look for the preamble and it only continues to detect the data if the preamble is detected.

Proposal 4 – Support LP-WUS structure with two fields, a preamble field for synchronization and cell identification purposes and a data field for indication of subsequent actions and/or wake-up group identity, depending on state of the operation.

Inclusion of preamble in the LP-WUS structure is still under discussion and not finalized.

### LP-WUS structure and overlaid sequences

During RAN1#116bis, RAN1#117, RAN1#118bis, and RAN1#119 meetings details related to information being carried by the data part, including overlaid sequence, were discussed. Below we have listed the latest agreements made in RAN1#118bis and RAN1#119 to address details of overlaid sequence.

RAN1#118

Agreement In case of overlaid OFDM sequence carrying information, support option 2: - Option 2: One sequence is selected from multiple candidates overlaid OFDM sequences on each OOK ‘ON’ symbol, and OFDM-based LP-WUR obtain LP-WUS information at least by overlaid OFDM sequence(s). Consider the following two sub-options for potential down-selection. - Option 2-1: The overlaid OFDM sequence(s) carry part of information bits of LP-WUS. OFDM-based LP-WUR can obtain the whole information bits by OFDM sequence(s) and location of the OFDM sequence(s)/OOK ‘ON’ symbols. - Option 2-2: The overlaid OFDM sequence(s) carry all information bits of LP-WUS. OFDM-based LP-WUR can obtain the whole information bits by the overlaid OFDM sequence(s) Note: the overlaid OFDM sequence in each OOK ‘ON’ symbol can be different according to information bits to be carried by the overlaid OFDM sequence within the LP-WUS. Agreement In case of overlaid OFDM sequence not carrying information: - Option 1: Single overlaid sequence is on each OOK ‘ON’ symbol. - Note 1: multiple overlaid OFDM sequences are specified. - Note 2: gNB can configure different overlaid OFDM sequence(s) for different cells. Agreement Regarding the maximum number of candidate overlaid sequences to carry LP-WUS information per OOK ON chip for one cell, down-select from the below: - 4 - 8 - 16 - 32 - 64 for M not larger than 2 - FFS whether the same or different set of candidate overlaid sequences are used for different cells

RAN1#119

Agreement Update the existing agreement as shown below: In case of overlaid OFDM sequence carrying information, support option 2: - Option 2: One sequence is selected from multiple candidates overlaid OFDM sequences on each OOK ‘ON’ symbol, and OFDM-based LP-WUR obtain LP-WUS information at least by overlaid OFDM sequence(s). Consider the following two sub-options for potential down-selection. - Option 2-1: The overlaid OFDM sequence(s) carry part of information bits of LP-WUS. OFDM-based LP-WUR can obtain the whole information bits by OFDM sequence(s) and location of the OFDM sequence(s)/OOK ‘ON’ symbols. - Option 2-2: The overlaid OFDM sequence(s) carry all information bits of LP-WUS. OFDM-based LP-WUR can obtain the whole information bits by the overlaid OFDM sequence(s) - FFS the how the information bits are carried by the overlaid OFDM sequence(s) Note: the overlaid OFDM sequence in each OOK ‘ON’ symbol can be different according to information bits to be carried by the overlaid OFDM sequence within the LP-WUS. Agreement For the overlaid OFDM sequence in time domain, the sequence is generated based on , - M=1, is given by the largest prime number such that,is the overlaid OFDM sequence length. - The base overlaid sequence is generated by extension of - , - With CS(s) applied to the base overlaid OFDM sequence if any:denotes the potential cyclic shift (s) - ,, - M=2, 4, is down-selected from the following: - Alt1: is given by the largest prime number such that,is the overlaid OFDM sequence length. - The base overlaid sequenceis generated by extension of - , - With CS(s) applied to the base overlaid OFDM sequence if any:denotes the potential cyclic shift (s) - , - Note it doesn’t preclude any pulse shaping scheme if any. - Alt2:is given by the largest prime number such that, is the overlaid OFDM sequence length. - The base overlaid sequenceis generated by inserting zeros before and/or after . The total number of zeros is. - For example,, - With CS(s) applied to the base overlaid OFDM sequence if any: denotes the potential cyclic shift (s) - For example,, - where, - FFS the value of G, with G>0. - FFS value(s) of root q and/or CS for generating the candidate sequences if applied - FFS on whether changes are needed to handle intra-/inter-cell interference - Note: the overlaid OFDM sequence in time domain is based on potential modification to ZC sequence as listed above. - Above overrides any previous RAN1 agreement / working assumption Agreement At least for M>1, for the overlaid OFDM sequence in time domain, down-select the sequence length LZC from the following: - Alt1:, with - FFS how to map the generated overlaid OFDM sequence to the frequency domain - Alt2: - Note: X is the number of RBs of LP-WUS/LP-SS bandwidth (blanked guard RBs are not included) Agreement CS(s) and/or root(s) used for overlaid OFDM sequence in the time domain are derived from RRC signalling: - FFS: the set of values of CS and root for configuration - FFS: details of the RRC signaling Agreement Regarding the maximum number of candidates overlaid sequences to carry LP-WUS information per OOK ON chip for one cell: - support maximum 4 candidates overlaid sequences for M=4

We first analyse two possible ways of carrying the information in the overlaid LP-WUS OFDM sequences.

- Separate sequences per information word: In this method, if k bits of information are to be carried per OFDM overlaid sequence, then 2k sequences are required for all the possible information words. This method may require sequence detection for each overlaid OFDM symbol at the receiver. Such correlation-based operations incur significant complexity.

- Use one configured OFDM overlaid ZC sequence per UE and carry the information in a cyclic shift of the sequence. In this method, if k bits of information are to be carried per OFDM overlaid sequence, then 2k cyclic shifts are required for all the possible information words. This method does not require sequence detection for each overlaid OFDM symbol at the receiver and so is less complex than method 1. It does require however, configuration of the sequence to be used per UE.

Proposal 5 – RAN1 to support the use of one configured OFDM overlaid ZC sequence per UE and carry the information in a cyclic shift of the sequence.

The preamble/synchronization sequence, described earlier, can serve an additional purpose for the OFDM overlay LP-WUR receiver if a cyclic shift of a configured sequence is used to carry the information in the OFDM overlaid LP-WUS. The ZC sequence for synchronization of a particular UE can be the configured sequence that would be used to carry data through cyclic shifts of the original sequence.

Proposal 6 – RAN1 to support carrying information bits in cyclic shifts of the ZC sequence configured as a synchronization sequence for the overlaid OFDM LP-WUS.

To limit the complexity, the choice of which ZC to be used as a synchronization sequence needs to take into account the cell-ID and a UE-ID or paging group ID. Taking the cell-ID into account means that a UE originally camped in a certain cell would look only for ZC sequences pertaining to only that cell. Different synchronization sequences from neighboring cells also reduces interference. Furthermore, taking a UE-ID or paging group-ID into account in the choice of the synchronization ZC sequence means that a given UE would search only for the ZC sequences pertaining to its UE-ID or paging group ID. Limiting the number of synchronization ZC sequences to correlate for in this manner, significantly reduces processing complexity for an OFDM-based LP-WUR receiver during synchronization. The cell-ID, UE-ID and/or paging group ID can be taken into account in choice of the ZC sequence root for example.

Proposal 7 – RAN1 to consider cell-ID, UE-ID and/or paging group-ID in the choice of the ZC sequence configured as a synchronization/preamble sequence for the overlaid OFDM LP-WUS.

For synchronization, the synchronization/preamble sequence can also be OFDM overlaid which is envisaged as an OOK signal. Furthermore, the synchronization sequence may also need to be repeated over multiple OFDM symbols to maximize coverage and minimize false alarm rate.

Proposal 8 – RAN1 to consider overlaying the ZC sequence configured as a synchronization/preamble sequence for the overlaid OFDM LP-WUS onto the OOK.

When deciding on the overlaid sequences, it is also important to consider the following:

- When carrying information, the information carried in the overlaid sequence should be only for the purpose of reducing the total length of the LP-WUS, received by OFDM based LP-WUR.

- Any additional information related to, e.g., cell information should be only included for the purpose of assisting OFDM-based LP-WUR detection. The cell information should not prevent the LP-WUS structure from including preamble in general structure of LP-WUS or carrying for instance cell ID in the preamble.

Proposal 9 – When carrying information, the information carried in the overlaid sequence should be only for the purpose of reducing the total length of the LP-WUS received by OFDM based LP-WUR without degrading performance of the OOK-based LP-WUR.

Proposal 10 – The OFDM overlaid sequence carrying cell information, should be only used for the purpose of assisting OFDM-based LP-WUR detection and not carrying any additional information that cannot be detected by the OOK-based LP-WUR.

### LP-WUS content and UE RRC operation

The following agreements were made, for UEs operating in IDLE/INACTIVE and CONNECTED mode, respectively.

On RRC IDLE/INACTIVE:

RAN1#117

Agreement For RRC idle/inactive state, down-select among the following options for at least indicating subgroup information using LP-WUS: - Option 1: A LP-WUS indicates a bitmap with each bit corresponding to one subgroup of N subgroups for part of, one or more PO(s), e.g., N is 8~16, 24 - Number of information bits for a LP-WUS is at least N, single LP-WUS to wake up one or more subgroups - Option 2: A LP-WUS indicates a codepoint value corresponding to one or more subgroup(s) from N subgroups for part of, one or more POs, e.g., N is 8~256 - Number of information bits for a LP-WUS is at least ceil (log2(X1)), where X1 is the number of codepoints indicating one or more subgroups. X1 is reported by companies, X1 could be smaller, equal to or larger than N. - Option 3: A LP-WUS indicates multiple codepoint values with each corresponding to one or more subgroup(s) from N subgroups for part of, one or more POs, e.g., N is 8~256 - Number of information bits for a LP-WUS is at least K*ceil (log2(X2)), where X2 is the number of codepoints indicating one or more subgroups. X2 is reported by companies, X2 could be smaller, equal to or larger than N. - K is the number of multiple codepoint values in a LP-WUS where K is larger than 1 - How to satisfy FAR is reported by companies, e.g., FEC/CRC - Note: multiple TDMed LP-WUSs can be used to support more subgroups for each option. - Note: Y% effective paging rate per PO is reported by companies - The followings are considered when down-select among options: - The number of supported UE subgroups per PO: M - Average network overhead to indicate the number of UE subgroups M per PO - False wake up rate due to subgroup-based indication, which will impact the power saving gain - Paging latency Note: Coverage target shall be met under 1%BLER, 1% FAR (for false alarm from noise)

On RRC_CONNECTED:

RAN1#118

Agreement Regarding the LP-WUS information to trigger PDCCH monitoring of RRC connected UEs (for non-CA case), select at least one from the following - Option 1: A bitmap with each bit corresponding to [one or more] UEs - Option 3: A codepoint value corresponding to [one or more] UEs - FFS details for extension of option 1 and/or 3 when UE is configured with CA

RAN1#119

For future meetings: Companies are encouraged to consider the following for future discussions Regarding the LP-WUS information to trigger PDCCH monitoring of RRC connected UEs, - Option 1: A bitmap with each bit corresponding to [one or more] UEs - 1A No CRC - Per bit coding, e.g., one bit is mapped to one of 2 codeword/sequences - The maximum number of codewords targeting per bit position in the bitmap for a UE is 1 - FFS coded bit length - Single MO - 1B No CRC - Bitmap-level coding, e.g., one bitmap of L bits is mapped to one of 2L codeword/sequences - The maximum number of codewords/sequences targeting for a UE is 2L-1 - Single MO - 1C With CRC - FFS CRC length - Single MO - Option 2: A codepoint value corresponding to [one or more] UEs - 2A only 1-1 mapping from codepoint to UE - The maximum number of codepoints targeting per MO for a UE is 1 - FFS the maximum number of MOs to be detected by a UE - 2B 1-to-1, 1-to-all mapping from codepoint to UEs - The maximum number of codepoints targeting per MO for a UE is up to 2 - FFS the maximum number of MOs to be detected by a UE - 2C 1-to-X mapping from codepoint to UEs, - The maximum number of codepoints targeting per MO for a UE is up to - FFS the maximum number of MOs to be detected by a UE - Note: L is the number of UEs associated monitoring the same WUS

Moreover, in RAN1#118, it is agreed to support Manchester coding for LP-WUS. The following are additionally agreed in RAN1#118bis to be investigated as alternatives to assist LP_WUR to meet target requirement.

Agreement Down-select among the following alternatives for LP-WUS information for OOK to meet target requirements: - adding CRC - channel coding other than Manchester coding, including rate matching if any - mapped to a binary sequence/codeword, include truncation if any - repetition, including rate matching if any - Note: combination of options above is not precluded - Note: combination of Manchester coding with above is assumed FFS how LP-WUS is transmitted within a MO/MOs

Depending on the LP-WUR architecture of the OOK-based LP-WUR, the level of power consumption vs noise figure and sensitivity can vary. Using longer signal sequences, containing more total energy, and a suitable low power digital base-band processing, see Figure 7, we can compensate for the worse performance and therefore allow for low detection error, i.e., mis-detection and false-alarm, rates. This lengthening of the signals can also be used for carrying additional information, beyond the pure wake-up trigger, as mentioned earlier. More specifically, we use the structure described in Figure 6 where we transmit sequences selected from sequences such as m-sequences with good correlation properties, which allows for low misdetection and false-alarm probabilities. The address spaces of these two sequences are typically much smaller than the M and L bits in the sequences, providing a degree of spreading that reduces error probabilities. With this, there is no need for any of the additional alternatives listed above to assist LP_WUR to meet target requirement.

From the proposed alternatives, in our view, while using bitmap to indicate the wake-up is flexible, this alternative is challenging as it does not fulfil the above conditions and thus it may be difficult to meet the required performance in terms of miss and false-alarm rates. Using multiple codepoints or sequences to be carried via the same LP-WUS may fulfil the above condition, but it will lead to higher power consumption since now the UE needs to monitor the channel for potentially much longer LP-WUS. Additionally, the detector may not be able to fully take advantage of the sequence correlation properties. Therefore, for IDLE/INACTIVE and CONNECTED modes, we support the alternative where a LP-WUS is a sequence or a codepoint is selected from sequences such as Kasami sequences, Gold sequences, m-sequences, with good auto- and cross-correlation properties.  This sequence or codepoint is signaled to wake up a UE or a group of UEs. This will correspond to option 3 for UEs in CONNECTED mode and option 2 for UEs operating in IDLE/INACTIVE mode.

Observation 6 – By selecting OOK-based LP-WUS from “longer” sequences with good auto- and cross-correlation properties, we can compensate for worse performance of the OOK-based LP-WUR and and therefore allow for low detection error without a need to use any of the additional alternative agreed in RAN1#118bis and RAN1#119.

Proposal 11 – From the alternatives, design LP-WUS to indicate wake-up based on option 2, based on the agreement in RAN1#118bis, for UEs in IDLE/INACTIVE mode.

Proposal 12 – From the alternatives, design LP-WUS to indicate wake up or sub-sequent action based on option 2C, from the discussion in RAN1#119, for UEs in CONNECTED mode.

# OOK LP-SS design

In this section we describe our view, based the agreement made in RAN1#119, on

- OOK-1/OOK-4 waveform for LP-SS

- overlaid OFDM sequences for LP-SS

- LP-SS structure and its content

- periodicities of LP-SS.

OOK-1/OOK-4 waveform

Agreement For the length L of LP-SS binary sequence, limit the selection to the following: - M=1, L= {4, 6, 8} - M=2, L= {8, 12, 16} - M=4, L= {16, 32} Agreement To compare LP-SS binary sequence performance, cross-correlation is deprioritized over sync accuracy, i.e., cross-correlation within a set of 4 sequences is further compared for different sets of sequences with similar sync accuracy. Agreement For the M value for LP-WUS and LP-SS, down-select one alternative from the following in RAN1#120: - Alt1: The M values for LP-WUS and LP-SS are always same. - Alt2: The M values for LP-WUS and LP-SS can be configured to be same or different. M value for LP-WUS cannot be larger than that of LP-SS. - Alt3: The M values for LP-WUS and LP-SS can be configured to be same or different.

Use of OOK-4 with M=4 has advantage over the other alternatives. When carrying the same number of bits, OOK-4 with M=4 has lower system overhead and can be detected with shorter delay, compared to the other candidate. Additionally, for the same LP-SS length, see RAN1#119 agreement above, a larger number of sequence space can be used, allowing for more flexibility in the signal design, e.g. accommodating more information such as cell ID related information in a LP-SS.

Proposal 13 – Consider OOK-4 transmission scheme with maximum M and L values for the transmission of the LP-SS.

Number of bits per OFDM symbol – The same LP-WUR is going to be used to receive both LP-SS and LP-WUS. It is therefore natural to  use the same value of M when generating LP-SS and LP-WUS.

Proposal 14 – RAN1 to consider having the same value of M for both LP-SS and LP-WUS.

LP-SS structure and content

Based on the RAN1 study and evaluation methodology, UE power saving gain cannot be observed if the existing Rel-18 MR RRM measurement periodicity for serving and neighbour cells are applied and UE MR enters in ultra-deep sleep during LP LP-WUS monitoring, therefore

- MR serving and neighbour cell measurement with further time domain relaxation than what is allowed in Rel-18 specification for IDLE/INACTIVE and/or

- at least serving cell RRM measurement offloaded from MR to LR is beneficial.

The aspect related to RRM measurement is under discussion in RAN2. In order to use LP-SS for both synchronization and RRM measurement purposes, the LP-SS needs to contain information related to/associated with serving cell and neighbor cell ID. During the last meeting, the following agreement on the number of LP_SS sequences was made.

Agreement The number of binary LP-SS sequences is 4.

To allow for good detection performance, i.e. low mis-detection and false-alarm probabilities, the binary LP-SS sequence type for the ‘ON-OFF’ pattern in a LP-SS should be selected from sequences with good auto- and cross-correlation properties such as m-sequences or Gold sequence. From the two candidates, Gold sequence, however, has better auto- and cross-correlation properties. The length of the sequences should be long enough so that the processing gain of the sequence can compensate for the worse sensitivity level of OOK-based LP-WUR improve on the LP-SS coverage.

Proposal 15 – Support inclusion of association of cell ID in the LP-SS content, e.g., by using LP-SS with max. value of M and L 
Proposal 16 – RAN1 to revise the agreed number of sequences and allow for larger number of sequences.

Proposal 17 – RAN1 to select LP-SS sequences from Gold sequence.

Discussion on LP-SS operation and its periodicity

Based on the RAN1 study and evaluation methodology, it is concluded that at least for LP-WUR that cannot receive existing PSS/SSS, periodic LP-SS signal is beneficial for the following functionalities: (a) RRM measurements by LP-WUR, (b) At least coarse time synchronization of LP-WUR. (c) At least coarse frequency synchronization of LP-WUR. During RAN1#118bis it was agreed to support short periodicities for LP-SS.

Agreement For LP-SS periodicity, support at least 320ms - FFS: 80ms,160ms, 640ms,1280ms, 2560ms, 5120ms, 10240ms

During RAN1#119, the followings were further agreed.

Agreement Regarding whether to support additional sync signal to LP-SS, down-selection from the following: - Option 1A: No additional sync signal, LP-SS periodicity =320ms - Additionally support at least one of [80ms,160ms] - FFS: whether different values can be applicable for different M values - Option 1B: No additional sync signal, LP-SS periodicity =320ms - No additional support of other periodicities - Option 2: With additional sync signal, LP-SS periodicity =320ms - FFS additionally support other LP-SS periodicities - FFS details on additional sync signal - FFS additional sync signal is configurable and/or conditionally present

In our view, the LP-SS needs to be designed such that it can be used for both synchronization purposes and measurements. However, during LP-WUS channel monitoring, the LP-WUR monitors the channel anyway for the potential LP-WUS. By including a preamble sequence in the LP-WUS structure, the aperiodic preamble or LP-WUS, which can be received by all the UEs listening to the same monitoring window, can also be used for synchronization. By introducing such preamble, the periodicity of the LP-SS can be relaxed, and the LP-SS can be transmitted only every number of the shortest LP-WUS monitoring window This allows to reduce the power consumption at the UE as the number of channels that the LP-WUR has to monitor is reduced. It also allows to configure the LP-SS with low periodicity and reduced resource and spectrum usage. To enable this, it is necessary to support longer LP-SS periodicity, e.g., 5120ms or 10240ms and include a preamble in the structure of LP-WUS Design.

Proposal 18 – Consider option 2 where the synchronization is done based on an aperiodic signal/sequence transmitted as part of LP-WUS.

Proposal 19 - Support longer periodicity of LP-SS to allow reduced resource, power consumption and spectrum usage.

# Conclusion

In this contribution, we have discussed our views on LP-WUS and LP-SS design and made the following observations and proposals.

Observation 1 – OOK-4 with M>1 has lower system overhead and can be detected with shorter delay using lower power LP-WUR, compared to OOK-1 or OOK-4 candidates with lower values of M.

Observation 2 - The maximum number of bits per OFDM symbol, M, needs to be calculated based on LP-WUS bit rate and according to channel delay spread, preventing ISI and the tolerable time/frequency errors by the LP-WUR.

Observation 3 – Using the same SCS configuration as other NR has less complexity for gNB.

Observation 4 - To allow for fine synchronization and to prevent the NW from always-on transmission of LP-SS with short periodicity, it is beneficial that the LP-WUS also includes a preamble prior to its data information.

Observation 5 – A use of a preamble before LP-WUS data part results in reduced power consumption at the UE while monitoring for LP-WUS. The LP-WUR first needs to only look for the preamble and it only continues to detect the data if the preamble is detected.

Observation 6 – By selecting OOK-based LP-WUS from “longer” sequences with good auto- and cross-correlation properties, we can compensate for worse performance of the OOK-based LP-WUR and and therefore allow for low detection error without a need to use any of the additional alternative agreed in RAN1#118bis and RAN1#119.

Proposal 1 – RAN1 to support setting M value to its maximum, i.e., M=4 for 30 kHz SCS and M=8 for 15 kHz to allow for low system overhead, lower latency and less power consumption at the LP-WUR.

Proposal 2 – RAN1 to support to use a unified specification for both OOK-1 and OOK-4.

Proposal 3 – RAN1 to support the same SCS configuration as other NR as it has less complexity for gNB.

Proposal 4 – Support LP-WUS structure with two fields, a preamble field for synchronization and cell identification purposes and a data field for indication of subsequent actions and/or wake-up group identity, depending on state of the operation.

Proposal 5 – RAN1 to support the use of one configured OFDM overlaid ZC sequence per UE and carry the information in a cyclic shift of the sequence.

Proposal 6 – RAN1 to support carrying information bits in cyclic shifts of the ZC sequence configured as a synchronization sequence for the overlaid OFDM LP-WUS.

Proposal 7 – RAN1 to consider cell-ID, UE-ID and/or paging group-ID in the choice of the ZC sequence configured as a synchronization/preamble sequence for the overlaid OFDM LP-WUS.

Proposal 8 – RAN1 to consider overlaying the ZC sequence configured as a synchronization/preamble sequence for the overlaid OFDM LP-WUS onto the OOK.

Proposal 9 – When carrying information, the information carried in the overlaid sequence should be only for the purpose of reducing the total length of the LP-WUS received by OFDM based LP-WUR without degrading performance of the OOK-based LP-WUR.

Proposal 10 – The OFDM overlaid sequence carrying cell information, should be only used for the purpose of assisting OFDM-based LP-WUR detection and not carrying any additional information that cannot be detected by the OOK-based LP-WUR.

Proposal 11 – From the alternatives, design LP-WUS to indicate wake-up based on option 2, based on the agreement in RAN1#118bis, for UEs in IDLE/INACTIVE mode.

Proposal 12 – From the alternatives, design LP-WUS to indicate wake up or sub-sequent action based on option 2C, from the discussion in RAN1#119, for UEs in CONNECTED mode.

Proposal 13 – Consider OOK-4 transmission scheme with maximum M and L values for the transmission of the LP-SS.

Proposal 14 – RAN1 to consider having the same value of M for both LP-SS and LP-WUS.

Proposal 15 – Support inclusion of association of cell ID in the LP-SS content, e.g., by using LP-SS with max. value of M and L 
Proposal 16 – RAN1 to revise the agreed number of sequences and allow for larger number of sequences.

Proposal 17 – RAN1 to select LP-SS sequences from Gold sequence.

Proposal 18 – Consider option 2 where the synchronization is done based on an aperiodic signal/sequence transmitted as part of LP-WUS.

Proposal 19 - Support longer periodicity of LP-SS  to allow reduced resource, power consumption and spectrum usage.

References

- RAN1#119 Chair’s notes

- RAN1#118bis Chair’s notes

- R1-2410232 LP-WUS operation in IDLE INACTIVE modes

- R1-2410233 LP-WUS operation in CONNECTED modes

- N. Mazloum and O. Edfors, "Interference-Free OFDM Embedding of Wake-Up Signals for Low-Power Wake-Up Receivers," in IEEE Transactions on Green Communications and Networking, vol. 4, no. 3, pp. 669-677, Sept. 2020,

- RAN1#116 Chair’s notes

- RAN1#116-bis Chair’s notes

- RAN1#117 Chair’s notes

- RAN1#118 Chair’s notes

- R2-2408710 Discussion on RRM aspects for LP-WUS/WUR

Appendix – Performance analysis and simulation results

In this section we present MDR performance for OOK-based LP-WUS based OOK-4 scheme and OFDM overlaid of option.  In table 1 we list the design parameters of a studied OOK-based LP-WUS and some of the important properties of the LP-WUR. This LP-WUS design has the following characteristics:

- Kasami sequences of equal length of 63 bits are selected to represent 504 cell identities and 16 group identities. This translates to spreading factors of 7 and 16 for the two parts of the 126-bit LP-WUS, which has a total time duration of 0.52 msec.

- Overlaid OFDM sequence with length 16 consisting of an m-sequence of length 15 and a random single bit. The overlaid OFDM sequence is generated based on option 1-1, i.e., before DFT/LS block.

- The LP-WUS occupies 16 OFDM-symbols, 11RBs (1 RB GB at each side of the LP-WUS bandwidth) with carrier frequency spacing of 30 kHz.

- When performing this simulation, we have taken into account a drift compensation, chosen cautiously to be 10 times that of the ultra-deep sleep of the main receiver.

| LP-WUS design parameters | Values | Notes |
| --- | --- | --- |
| (Np, Nd) | (4,4) | M=63 bits, L=63 bits, corresponding to total 8 OFDM symbols |
| WUS duration(us) | 0.52 msec | LP-WUS of 8 symbols @ SCS 15e3 is 500us. |
| No. of bits per OFDM symbol | 4 bits |  |
| LP-WUS SCS | 30 kHz |  |
| Main signal SCS | 30 kHz |  |
| No. of RBs used for LP-WUS transmission | 11 |  |
| LP-WUS Bandwidth | 3.96 MHz |  |
| Bit rate | 120 kbps | Manchester coded OOK |
| Guardband width | 360 kHz | 1 RB @ SCS 30kHz at each side |
| LP-WUR Parameters/characteristics | Values | Notes |
| Zero-IF architecture | yes | - |
| LP-WUR BW | 3.96 MHz | Butterworth, order of 5 |
| ADC | 1-bit ADC and Ideal ADC |  |

Table 1 – OOK-based LP-WUS and LP-WUR design parameters

Figure 7 - Misdetection rate vs SNR for 1-bit and Ideal ADC for TDL-C 300 channel

We can observe that the SNR at MDR 1% is within the SNR range agreed during RAN1#117 for the three studied case, fulfilling the requirement to meet msg3 coverage. We can also observe that the overlaid OFDM sequence does not degrade the performance of the OOK-based OFDM sequence when generated based option 1-1 whne using m-sequences as overlaid sequence.

Agreement The following SNR values for LP-WUR are reported by companies to achieve coverage of PUSCH for message3 for difference noise figures for calibration purposes: - NF of LR = NF of MR+ 8dB: the reported SNR value range is [-9.05,2.94] dB - NF of LR = NF of MR+ 5dB: the reported SNR value range is [-6.5, 5.58] dB - NF of LR = NF of MR+ 2dB: the reported SNR value range is [-4.04,7.95] dB - Note 1: The above is observed based on the following assumptions used for calibration: - Carrier frequency: 2.6 GHz - The number of Tx chains: 1 - MIL of MSG 3: use the average one in R17 coverage, i.e.,153.51 dB for non-redcap UE - Transmit antenna gain correction factors for WUS: up to company report - Noise Figure: All three values +2dB, +5dB, +8dB on top of NF of MR (7dB) are to be reported, SNR for different assumptions on NF are determined separately

Appendix – Agreements from earlier meetings

RAN1#118bis[2]

Working Assumption For overlaid OFDM sequences for LP-WUS in time or frequency domain, for OOK-1 and OOK-4 M=1 - Option 1-1(compromised): overlaid sequence(s) are the sequence(s) of an OOK on symbol before DFT/LS processing for OOK-4 with M=1. - ZC sequence in time domainDFT/LSIFFT to result in a frequency domain sequence that is ZC sequence or a sequence that is close to ZC sequence, no additional operation - It doesn’t preclude additional operation for OOK-4 with M>1. - DFT size is 2^n Note1: OOK-1 is considered as specific case of OOK-4 with M=1. Note: Concerns were raised by Ericsson and CATT that the above is not beneficial in terms of implementation complexity Agreement In case of overlaid OFDM sequence carrying information, support option 2: - Option 2: One sequence is selected from multiple candidates overlaid OFDM sequences on each OOK ‘ON’ symbol, and OFDM-based LP-WUR obtain LP-WUS information at least by overlaid OFDM sequence(s). Consider the following two sub-options for potential down-selection. - Option 2-1: The overlaid OFDM sequence(s) carry part of information bits of LP-WUS. OFDM-based LP-WUR can obtain the whole information bits by OFDM sequence(s) and location of the OFDM sequence(s)/OOK ‘ON’ symbols. - Option 2-2: The overlaid OFDM sequence(s) carry all information bits of LP-WUS. OFDM-based LP-WUR can obtain the whole information bits by the overlaid OFDM sequence(s) Note: the overlaid OFDM sequence in each OOK ‘ON’ symbol can be different according to information bits to be carried by the overlaid OFDM sequence within the LP-WUS. Agreement From RAN1 perspective, support X=11 PRBs for LP-WUS and LP-SS with SCS 15kHz (blanked guard RBs are not included) for a channel bandwidth equal or larger than 5 MHz Agreement To determine the binary sequences for LP-SS, for each M value, down-select the sequence length L from the corresponding candidate values: - M=1, L= {4,6,8} - M=2, L= {8,12,16,24} - M=4, L= {16,24,32,56} - FFS whether one or more than one L value (s) applied to each of M values M=1,2,4 - FFS the L values applied to other applicable SCS(s) - FFS the L values if other M values in addition to M=1,2,4 are supported. - FFS whether the time estimation averaged across multiple LP-SS occasions is applied - Note 1: with sequence length L, it includes Manchester coding (if supported for LP-SS) - Note 2: This doesn’t preclude any of three agreed sequence types Above applies at least for both 15kHz and 30kHz Agreement For LP-SS periodicity, support at least 320ms - FFS: 80ms,160ms, 640ms,1280ms, 2560ms, 5120ms, 10240ms Agreement For overlaid OFDM sequence at least for LP-WUS, - For OOK-4 with M>1, support ZC sequence for overlaid sequence. - FFS details Agreement From RAN1 perspective, support X=11 PRBs for LP-WUS and LP-SS with SCS 60kHz and 120kHz for FR2. Agreement For the SCS used for LP-WUS and LP-SS, further discuss the following options: - The single SCS is configured by gNB - The single SCS is determined by pre-defined rule Agreement In case of overlaid OFDM sequence not carrying information: - Option 1: Single overlaid sequence is on each OOK ‘ON’ symbol. - Note 1: multiple overlaid OFDM sequences are specified. - Note 2: gNB can configure different overlaid OFDM sequence(s) for different cells. Agreement Regarding the maximum number of candidate overlaid sequences to carry LP-WUS information per OOK ON chip for one cell, down-select from the below: - 4 - 8 - 16 - 32 - 64 for M not larger than 2 - FFS whether the same or different set of candidate overlaid sequences are used for different cells Agreement To determine the binary sequences for LP-SS, consider the following for performance comparison: - Sync accuracy T’: the achieved sync accuracy T’ for a set of 4 sequences with the same number of occupied OFDM symbols (timing estimation error smaller than ≤T’ us for P=90 % of the time for SNR=-3dB, -6dB (low priority)) - Assume the sliding window size is at least [-25us, 25us]. - RRM measurement accuracy X: the achieved measurement accuracy within range ± XdB for Q=90% measurements based on Y LP-SS samples within a period comparable to Z=the length of I-DRX cycle that is larger or equal to 1.28s - Assume X= 3.5dB shall be satisfied for LP-RSRP and LP-RSRQ, based on Y=1, 4 and Z=1.28s for comparison - Cross correlation: maximum cross-cor value/ maximum auto-cor value - FFS how to use cross-correlation for comparison - Note: refer to RAN4 RRM assumption for the inter-cell interference Agreement Down-select among the following alternatives for LP-WUS information for OOK to meet target requirements: - adding CRC - channel coding other than Manchester coding, including rate matching if any - mapped to a binary sequence/codeword, include truncation if any - repetition, including rate matching if any - Note: combination of options above is not precluded - Note: combination of Manchester coding with above is assumed FFS how LP-WUS is transmitted within a MO/MOs Agreement Support overlaid OFDM sequence(s) for LP-SS: - LP-SS reuses the overlaid OFDM sequence(s) specified for LP-WUS. The design on overlaid OFDM sequence(s) specified for LP-WUS doesn’t target for sync and RRM measurement performance based on overlaid OFDM sequence for LP-SS. - Whether to transmit LP-SS by using a specified overlaid OFDM sequence is configurable. - Applicable at least for OOK-1 and FFS for OOK-4 - From RAN1 perspective, it is not intended to introduce new RAN4 requirements specific to overlaid sequences Reminder to companies who are interested in LP-SS: - Evaluation results on binary sequences for LP-SS are to be collected in RAN1 #119 meeting for comparison.

RAN1#118 [8]

Agreement For RRC idle/inactive state, support the following option for at least indicating subgroup information using LP-WUS: - Option 2: A LP-WUS indicates a codepoint value corresponding to one or more subgroup(s) from N subgroups for part of, one or more POs - UE monitoring one or more MOs (up to X MOs) for the same beam within an LO is supported - Value of X is larger than 1. FFS on additional details of X. Agreement [H][FL1] Proposal 3.1-1: Confirm the Working Assumption that OOK-4 with M=4 for 15KHz SCS is supported for LP-WUS. Agreement For OOK-4 with M >1, support M=2 & M=4 (working assumption) for LP-WUS. - M=4 for 15KHz SCS - M=4 for 30KHz SCS (working assumption) - FFS M=1 for OOK-4. Agreement Regarding the LP-WUS information to trigger PDCCH monitoring of RRC connected UEs (for non-CA case), select at least one from the following - Option 1: A bitmap with each bit corresponding to [one or more] UEs - Option 3: A codepoint value corresponding to [one or more] UEs - FFS details for extension of option 1 and/or 3 when UE is configured with CA Agreement For overlaid OFDM sequences for LP-WUS, support option 1-1 for OOK-4 M>1. - Option 1-1: overlaid sequence(s) are the sequence(s) of an OOK on symbol before DFT/LS processing Agreement For overlaid OFDM sequences for LP-WUS, further down-selection between following two options for OOK-1 and OOK-4 with M=1(if supported) - Option 1-1: Overlaid sequence(s) are the sequence(s) of an OOK on symbol before DFT/LS processing - Option 2: Overlaid sequence(s) are the sequence(s) of an OFDM symbol before IFFT processing - Note: Different options for OOK-1 and OOK-4 with M=1 (if supported) is not precluded – in which case, it should be deemed necessary Agreement The number of binary LP-SS sequences is 4. Agreement To determine the binary sequences for LP-SS, the evaluation assumes the following: - Sync accuracy: timing estimation error smaller than T us for P=90 % of the time for at least SNR=-3dB and SNR=-6dB (lower priority), - T=2us(optional), 5us for OOK-1 - T=1us(optional), 2us for OOK-4 with M=2 - T=0.5us(optional), 1us for OOK-4 with M=4 - Other values of SNR, T, M are up to company report. - Additionally, companies can submit results on SINR with details on how the interference was modelled. - Assume one-shot estimation - The time error is based on 20ppm maximum frequency error for the detection of the first LP-SS - Note: Companies can assume other values inside of 20ppm maximum as long as the values are justified - RRM measurement accuracy: measurement accuracy within range ± XdB for Q=90% measurements based on Y LP-SS samples within a period comparable to Z=the length of I-DRX cycle that is larger or equal to 1.28s for at least SNR=-3dB and SNR=-6dB (lower priority), X, Y, and Z is up to company report, other values of SNR are up to company report. - As a starting point, the time error is based on 5-10ppm residual frequency error. - Note: 5-10ppm assumes frequency error correction is performed, e.g., RTC calibration and/or MR assistance - Note: Companies can assume other values outside of 5-10ppm as long as the values are justified - The frequency error: up to company report - Sampling rate: 3.84MHz (optional) or 7.68MHz assumed for 30kHz SCS. - Channel: AWGN and TDL-C 300ns is assumed. - Consider cross-correlation for 4 binary sequences under time and frequency error - Companies are encouraged to provide details on other additional simulation assumptions, high level description of their receiver algorithm, and assessment on power consumption Agreement For OOK-based LP-WUR, the LP-WUS design assumes the following: - As starting point, the time error is based on the 5-10ppm residual frequency error after frequency error correction without considering impact of drift. - The frequency error: up to company report. - Note: 5-10ppm assumes frequency error correction is performed, e.g., RTC calibration and/or MR assistance - Note: Companies can assume other values outside of 5-10ppm as long as the values are justified Agreement As a starting point, for both time error and frequency error, the overlaid OFDM sequence design of LP-WUS assumes that the residual frequency error is 0.1-5ppm for OFDM-based LP-WUR after frequency error correction without considering impact of drift. - Note: Companies can use any value within the above range with justification on the value (including impact on power consumption) Agreement Support Manchester coding for LP-WUS - FFS other coding schemes

During RAN1#117 [6], the following agreement on LP-WUS and LP-SS design were made.

Agreement Support overlaid OFDM sequence based on existing NR sequence type for LP-WUS - Down select among gold sequence, m sequence and ZC sequence - FFS the overlaid OFDM sequence is time or frequency domain sequence. - FFS how to reuse the existing sequences - Note: Strive to minimize the impact on OOK detection performance - If overlaid OFDM sequence is supported for LP-SS, the same sequence type is used for both LP-SS and LP-WUS Agreement For the LP-SS sequence used in a cell, - Option 1: the information necessary for determining the sequence is explicitly configured - FFS: Additional support of determining the sequence by predefined rule without configuration Agreement The LP-WUS and LP-SS design assumes the residual frequency error after frequency error correction without considering impact of drift, is up to X ppm for OOK-based LP-WUR. - FFS X which is no larger than 20ppm - Initial frequency error assumption: up to company report For the overlaid OFDM sequence design of LP-WUS, it is assumed that the residual frequency error for OFDM-based LP-WUR after frequency error correction without considering impact of drift is not larger than Y. - FFS Y which is no larger than 20ppm and lower than X - Initial frequency error assumption: up to company report Agreement For overlaid OFDM sequence(s) for LP-WUS in time or frequency domain, down-selection from the following: - Option 1-1: overlaid sequence(s) are the sequence(s) of an OOK on symbol before DFT/LS processing - The length of overlaid sequence(s) depends on the number of REs used for LP-WUS and the value of M - Option 1-2: overlaid sequence(s) are the sequence(s) of an OFDM symbol before DFT/LS processing - The length of overlaid sequence(s) depends on the number of REs used for LP-WUS - Option 2: overlaid sequence(s) are the sequence(s) of an OFDM symbol before IFFT processing - The length of overlaid sequence(s) depends on the number of REs used for LP-WUS - Option 3: overlaid sequence(s) are the sequence(s) of an OOK on symbol in time domain after IFFT processing - The length of overlaid sequence(s) depends on IFFT size and the value of M FFS: same or different options are applied for OOK-1 and OOK-4 M>1. Agreement Update agreement in last meeting as below: From RAN1 perspective, support X PRBs for LP-WUS and LP-SS with SCS 30kHz (blanked guard RBs are not included) for a channel bandwidth equal or larger than 5MHz - X to be down-selected between = 11 and 12 PRBs - FFS if other number of PRBs needed, for LP-SS and LP-WUS with a channel bandwidth equal or less than 5MHz FFS: Whether the above is applicable to FR2 Agreement Further down-select the number of binary LP-SS sequences for the ‘ON-OFF’ pattern: - 3 - 4 - 8 - 16 Agreement For the binary LP-SS sequence type for the ‘ON-OFF’ pattern in a LP-SS, further down-selection from the following: - Gold sequence - M sequence - Computer searched sequence - FFS: the length of LP-SS sequence Agreement The following SNR values for LP-WUR are reported by companies to achieve coverage of PUSCH for message3 for difference noise figures for calibration purposes: - NF of LR = NF of MR+ 8dB: the reported SNR value range is [-9.05,2.94] dB - NF of LR = NF of MR+ 5dB: the reported SNR value range is [-6.5, 5.58] dB - NF of LR = NF of MR+ 2dB: the reported SNR value range is [-4.04,7.95] dB - Note 1: The above is observed based on the following assumptions used for calibration: - Carrier frequency: 2.6 GHz - The number of Tx chains: 1 - MIL of MSG 3: use the average one in R17 coverage, i.e.,153.51 dB for non-redcap UE - Transmit antenna gain correction factors for WUS: up to company report - Noise Figure: All three values +2dB, +5dB, +8dB on top of NF of MR (7dB) are to be reported, SNR for different assumptions on NF are determined separately Agreement For evaluation of LP-WUS and LP-SS design to achieve coverage of PUSCH for message3 from RAN1 perspective, at least the following SNR values should be considered: - SNR=-3dB for NF of LR = NF of MR+ 8dB - SNR= -0.5dB for NF of LR = NF of MR+ 5dB - SNR=2dB for NF of LR = NF of MR+ 2dB - Note 1: The NF of MR is assumed as 7dB Agreement For RRC idle/inactive state, down-select among the following options for at least indicating subgroup information using LP-WUS: - Option 1: A LP-WUS indicates a bitmap with each bit corresponding to one subgroup of N subgroups for part of, one or more PO(s), e.g., N is 8~16, 24 - Number of information bits for a LP-WUS is at least N, single LP-WUS to wake up one or more subgroups - Option 2: A LP-WUS indicates a codepoint value corresponding to one or more subgroup(s) from N subgroups for part of, one or more POs, e.g., N is 8~256 - Number of information bits for a LP-WUS is at least ceil (log2(X1)), where X1 is the number of codepoints indicating one or more subgroups. X1 is reported by companies, X1 could be smaller, equal to or larger than N. - Option 3: A LP-WUS indicates multiple codepoint values with each corresponding to one or more subgroup(s) from N subgroups for part of, one or more POs, e.g., N is 8~256 - Number of information bits for a LP-WUS is at least K*ceil (log2(X2)), where X2 is the number of codepoints indicating one or more subgroups. X2 is reported by companies, X2 could be smaller, equal to or larger than N. - K is the number of multiple codepoint values in a LP-WUS where K is larger than 1 - How to satisfy FAR is reported by companies, e.g., FEC/CRC - Note: multiple TDMed LP-WUSs can be used to support more subgroups for each option. - Note: Y% effective paging rate per PO is reported by companies - The followings are considered when down-select among options: - The number of supported UE subgroups per PO: M - Average network overhead to indicate the number of UE subgroups M per PO - False wake up rate due to subgroup-based indication, which will impact the power saving gain - Paging latency - Note: Coverage target shall be met under 1%BLER, 1% FAR (for false alarm from noise)

During RAN1#116bis [6], the following agreement on LP-WUS and LP-SS design were made.

Agreement For OOK-4 with M >1, support M=2 & M=4 (working assumption) for LP-WUS. - FFS whether value of M depends on SCS - FFS M=1 for OOK-4 Agreement For evaluation purpose on LP-WUS, companies report the overlaid OFDM sequence(s), including: - Sequence(s) generation and how sequence(s) map in time or frequency domain (including any details with multiplexing and IFFT). - Number of candidate overlaid OFDM sequences used for information conveying - Including details on whether the number of candidate overlaid sequences is per OFDM symbol or per OOK symbol - How the proposed sequence design is processed by OFDM-based LP-WUR, e.g., in time domain or in frequency domain or in both time and frequency domain. Agreement Support to specify multiple binary LP-SS sequences for the ‘ON-OFF’ pattern: - The LP-SS sequence used in a cell is - Option 1: a sequence is configured - Option 2: a sequence is determined by predefined rule - FFS: Whether both options will be supported or only one will be supported - FFS: the number of LP-SS sequences - Note: Multiple sequences are used to differentiate LP-SS from different cells Agreement From RAN1 perspective, support X PRBs for LP-WUS and LP-SS with SCS 30kHz (blanked guard RBs are not included) for a channel bandwidth equal or larger than 5MHz - X to be down-selected between 11 and 12 PRBs - FFS the number of PRBs for 15kHz - FFS if other number of PRBs needed, for LP-SS and LP-WUS with a channel bandwidth equal or less than 5MHz FFS: Whether the above is applicable to FR2 Agreement For timing error evaluation purpose, the following two options for residual frequency error are considered: - Option 1: The maximum frequency error (Fe) of RTC/oscillator is assumed, companies report Fe value and the applied LP-WUR type. - Option 2: The residual frequency error (Fr) after frequency error correction/clock calibration by LR or after assistance from MR is assumed, companies report Fr value, how to achieve it and the applied LP-WUR type. Agreement For frequency error evaluation purpose, the following two options for residual frequency error are considered: - Option 1: The maximum frequency error (Fe) of oscillator is assumed, companies report Fe value and the applied LP-WUR type. - Option 2: The residual frequency error (Fr) after frequency error correction by LR or after assistance from MR is assumed, companies report Fr value, how to achieve it and the applied LP-WUR type. Working Assumption Support the following options for LP-SS - Option 1: OOK-1 - Option 2: OOK-4 with M=2,4, FFS:1,8,16 - FFS whether value of M depends on SCS - The SCS of a CP-OFDM symbol used for LP-SS generation is the same as that used for LP-WUS generation FFS how OOK-1 and OOK-4 are specified Agreement Regarding the LP-WUS information for idle/inactive UEs, at least consider the following： - Option 1: A bitmap with each bit corresponding to [one or more] subgroups - Option 2: A codepoint value corresponding to one or more subgroup(s) - Option 3: Multiple codepoint values with each corresponding to one or more subgroup(s) - Combination of above options are not precluded - FFS how to carry LP-WUS information, e.g., by encoded bits (with/without CRC) and/or by OOK sequence selection for ‘ON-OFF’ pattern for OOK symbols of LP-WUS. - FFS how to carry LP-WUS information by overlaid OFDM sequences. - It doesn’t preclude considering the configuration where a single candidate overlaid OFDM sequence is used - Other options are not precluded Agreement Regarding the LP-WUS information to trigger PDCCH monitoring of RRC connected UEs, at least consider the following： - Option 1: A bitmap with each bit corresponding to [one or more] UEs - Option 2: A codepoint value corresponding to one or part of UE identity, e.g., C-RNTI - Option 3: A codepoint value corresponding to [one or more] UEs - Option 4: Multiple codepoint values with each corresponding to [one or more] UE(s) - Option 5: Multiple bit blocks with each corresponding to [one or more] UE(s) - Combination of above options are not precluded. - FFS how to carry LP-WUS information, e.g, by encoded bits (with/without CRC) and/or by OOK sequence selection for ‘ON-OFF’ pattern for OOK symbols of LP-WUS. - FFS how to carry LP-WUS information by overlaid OFDM sequences. - It doesn’t preclude considering the configuration where a single candidate overlaid OFDM sequence is used - FFS details of LP-WUS information to trigger PDCCH monitoring (e.g. whether above is applicable to one or more serving cells) Conclusion: For calibration purposes, companies are encouraged to report the SNR to achieve the coverage of PUSCH for message3, at least with the following assumptions: - Carrier frequency: 2.6 GHz - The number of Tx chains: 1 - MIL of MSG 3: use the average one in R17 coverage, i.e.,153.51 dB for non-redcap UE - Transmit antenna gain correction factors for WUS: up to company report - Noise Figure: All three values +2dB, +5dB, +8dB on top of NF of MR (7dB) are to be reported, SNR for different assumptions on NF are determined separately Agreement For the purpose of further study and evaluation in RAN1, the following candidate sequences for the overlaid OFDM sequence are considered: - Gold sequence - M-sequence - ZC sequence - Chirp sequence - Walsh sequence - Golay sequence - Kasami sequence - Low density sequence - DFT/FFT sequence - QAM symbol-based sequence - Combinations and optimizations of above are not precluded Companies are encouraged to provide an assessment on performance, required complexity, and power consumption to support their preferred sequence. Companies are encouraged to provide details on their preferred sequence (e.g. references). Agreement Regarding the overlaid OFDM sequence(s) of LP-WUS, consider the following options: - Option 1: Single overlaid sequence is on each OOK ‘ON’ symbol or OFDM symbol duration. OFDM-based LP-WUR can obtain the whole information bits by the presence of the overlaid sequence. - Option 1-2: The overlaid OFDM sequence is pre-determined from multiple sequences. This sequence carry NO information bits of LP-WUS. OFDM-based LP-WUR can obtain the whole information bits by the OOK ON/OFF pattern. - Option 2: One sequence is selected from multiple candidates overlaid OFDM sequences on each OOK ‘ON’ symbol or OFDM symbol duration, and OFDM-based LP-WUR obtain LP-WUS information at least by overlaid OFDM sequence(s). Consider the following two sub-options. - Option 2-1: The overlaid OFDM sequence(s) carry part of information bits of LP-WUS. OFDM-based LP-WUR can obtain the whole information bits by OFDM sequence(s) and location of the OFDM sequence(s)/OOK symbols. - Option 2-2: The overlaid OFDM sequence(s) carry all information bits of LP-WUS. OFDM-based LP-WUR can obtain the whole information bits by the overlaid OFDM sequence(s) - Option 3: One sequence is selected from multiple candidates overlaid OFDM sequences on one or more OOK ‘ON’ symbols, and OFDM-based LP-WUR obtain LP-WUS information at least by overlaid OFDM sequence(s). - Option 4: Use of modulated overlay sequence with constellation point: overlay sequence acting as a spreading sequence and constellation point carrying information for OFDM-based LP-WUR. Other options are not precluded.

During RAN1#116[5], the following agreement on LP-WUS and LP-SS design were made.

Agreement Support both OOK-1 and OOK-4 for LP-WUS. - FFS how OOK-1 and OOK-4 are specified - For OOK-4, M<=4, FFS supported values - The SCS of a CP-OFDM symbol used for LP-WUS generation can be the same as one of the SCS(s) used for other NR transmissions in the same CP-OFDM symbol - FFS different SCS. Agreement Further study the following options for LP-SS: - Option 1: OOK-1 - Option 2: OOK-4 with M=1,2,4,[8] - The SCS of a CP-OFDM symbol used for LP-SS generation is the same as that used for LP-WUS generation - FFS: different SCS Agreement For LP-SS design from RAN1 perspective, consider at least the following as the design target: - For RRM measurement performed by LP-WUR based on LP-SS, UE can satisfy measurement accuracy based on X LP-SS samples within a period which is comparable to Y=the length of I-DRX cycle that is larger or equal to 1.28s. - FFS: X - Note: Y is chosen for evaluating LP-SS design. - Network overhead and network power consumption are to be considered Agreement The ‘ON-OFF’ pattern for OOK symbols of LP-SS is based on binary sequence(s) - FFS binary sequence(s) details, including the sequence type, the number of sequences, and the sequence length - FFS overlaid OFDM sequences, if supported Agreement For the overlaid OFDM sequence(s) for LP-SS, consider the following options for further down-selection: - Option 1: Do not specify the overlaid OFDM sequences(s) - Option 2: Specify the overlaid OFDM sequence(s) targeting for OOK waveform generation without targeting for sync and RRM measurement for OFDM-based LP-WUR using the overlaid sequence of LP-SS. - Option 3: Specify the overlaid OFDM sequence(s) targeting for OOK waveform generation and also targeting for sync and RRM measurement for OFDM-based LP-WUR using the overlaid sequence of LP-SS. - For Option 3, it is up to RAN4 to make decision on whether/how to define the RRM measurement requirement for OFDM-based LP-WUR using the overlaid sequence of LP-SS.