3GPP TSG RAN WG1 #120		  R1-2500698

Athens, Greece, February 17th – 21st, 2025

Source:	Panasonic

Title: 	Discussion on SBFD random access operation

Agenda Item:		9.3.2

Document for:	Discussion

Introduction

A work item on Evolution of NR duplex operation has been approved for Rel.19 [1]. This document provides our view on SBFD random access operation.

Discussion

PRACH configuration

Separate power control

For RACH configuration Option 1, the following agreement was reached on PRACH power control for additional RO in RAN1#119.

Agreement For RACH configuration Option 1, for support of separate power control parameters for PRACH transmission in additional-ROs and legacy-ROs, support separate preambleReceivedTargetPower for additional-ROs. - FFS: powerRampingStep, preamble maximum output power PCMAX, preambleTransMax, powerRampingStepHighPriority

Regarding FFS in the agreement, separate preamble maximum output power PCMAX  would be useful to reduce UE-to-UE CLI (e.g., configuring lower PCMAX for SBFD symbols makes it easier to control CLI due to UL transmission). In our understanding, PCMAX is intended for P-Max which is configured by RRC signaling. In the current specification, P-Max is configured per carrier. If separate PCMAX is applied only for PRACH, complexity would be increased because different PCMAX needs to be applied per symbol basis. Therefore, it would be simpler to apply separate PCMAX for a slot containing SBFD symbol, including channels/signals other than PRACH. Since slot configuration for SBFD is semi-statically configured, deterministic value of PCMAX can be applied on a slot-by-slot basis.

Proposal 1: Support separate maximum output power PCMAX for a slot containing SBFD symbol.

RO validation

For RACH configuration Option 2, the following agreement was reached for the case that additional ROs overlap with legacy ROs in In RAN1#119.

Agreement For RACH configuration Option 2, for the case that additional ROs overlap with legacy ROs, down-select from the following alternatives: - Alt-1: do not optimize this case. - Alt-2: the additional-ROs are considered invalid when overlapping with legacy-ROs.

Alt-2 would be clear on how to handle the overlap. However, considering that the overlap only occurs in flexible symbol, Alt-2 would not be essential because the overlap can be easily avoided by configuration (e.g., DL or UL symbol is used instead of flexible symbol). In addition, the overlap between ROs already exists in the current specification, but it is not optimized. Therefore, our preference is Alt-1.

Proposal 2: For the case that additional ROs overlap with legacy ROs for RACH configuration Option 2, support Alt-1.

PRACH configuration table

For RACH configuration Option 2, the following agreement was reached on interpretation of the parameter prach-ConfigurationIndex provided by the additional RACH configuration in in RAN1#119.

Agreement For RACH configuration Option 2, and for interpretation of the parameter prach-ConfigurationIndex provided by the additional RACH configuration, down-select from the following options: - Alt-1: introduce a subframe/slot offset (similar as prach-ConfigurationSOffset-IAB) to determine the subframe/slot number for ROs in SBFD symbols for FR1/FR2. - FFS: details of the subframe/slot offset. - Alt-2: introduce a new parameter indicating from an entry from a pre-defined/configured table to derive additional subframe/slot numbers. - Details are FFS. - Alt-3: introduce a bitmap indicating or puncturing subframe(s)/slot(s) for additional ROs in SBFD symbols. - Details are FFS. - Alt-4: no enhancement.

The existing random access configuration tables is not sufficient to configure RO in SBFD symbols because these tables are adjusted for UL slot location in TDD. Table 1 shows a part of random access configuration table as an example (only preamble format C2 in Table 6.3.3.2-3 in TS38.211 [2] are shown). According to Table 1, all entries in the table include subframe number 9 and some entries include subframe number 4 (see "Subframe number" column). If SBFD slot pattern is "DXXXU" for 15 kHz SCS (where "X" is SBFD slot), subframe 4 and 9 corresponds to UL slot (i.e., slot #4 and #9 are "U" slot in "DXXXU" pattern). In this case, subframe 4 and 9 would be useful to allocate RO in UL slot but would not be useful to allocate RO in SBFD slot. In addition, one motivation to introduce RACH configuration Option 2 would be to provide flexible configurations for additional-ROs compared to RACH configuration Option 1. Thus, our preference is either Alt-1, Alt-2, or Alt-3.

Among Alt-1, Alt-2, and Alt-3, Alt-1 can reuse the existing functionality introduced in Rel.16 IAB. Alt-2 and Alt-3 need to introduce new functionalities, e.g., a new parameter indicating an entry, or new bitmap. Therefore, our preference is Alt-1.

Proposal 3: For interpretation of the parameter prach-ConfigurationIndex provided by the additional RACH configuration for RACH configuration Option 2, support Alt-1.

Table 1  Part of Table 6.3.3.2-3 in 38.211[2] (Random access configurations for FR1 and unpaired spectrum)

| PRACH Configuration Index | Preamble format | nf mod x=y |  | Subframe number | Starting symbol | Number of PRACH slots within a subframe | , number of time-domain PRACH occasions within a PRACH slot | , PRACH duration |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 189 | C2 | 16 | 1 | 9 | 2 | 2 | 2 | 6 |
| 190 | C2 | 8 | 1 | 9 | 2 | 2 | 2 | 6 |
| 191 | C2 | 4 | 1 | 9 | 2 | 1 | 2 | 6 |
| 192 | C2 | 2 | 1 | 9 | 2 | 1 | 2 | 6 |
| 193 | C2 | 2 | 1 | 8,9 | 2 | 2 | 2 | 6 |
| 194 | C2 | 2 | 1 | 7,9 | 2 | 1 | 2 | 6 |
| 195 | C2 | 2 | 1 | 7,9 | 8 | 1 | 1 | 6 |
| 196 | C2 | 2 | 1 | 4,9 | 8 | 1 | 1 | 6 |
| 197 | C2 | 2 | 1 | 4,9 | 2 | 2 | 2 | 6 |
| 198 | C2 | 2 | 1 | 2,3,4,7,8,9 | 2 | 1 | 2 | 6 |
| 199 | C2 | 8 | 1 | 9 | 8 | 2 | 1 | 6 |
| 200 | C2 | 4 | 1 | 9 | 8 | 1 | 1 | 6 |
| 201 | C2 | 1 | 0 | 9 | 2 | 2 | 2 | 6 |
| 202 | C2 | 1 | 0 | 9 | 8 | 1 | 1 | 6 |
| 203 | C2 | 1 | 0 | 9 | 2 | 1 | 2 | 6 |
| 204 | C2 | 1 | 0 | 8,9 | 2 | 2 | 2 | 6 |
| 205 | C2 | 1 | 0 | 4,9 | 2 | 1 | 2 | 6 |
| 206 | C2 | 1 | 0 | 7,9 | 8 | 1 | 1 | 6 |
| 207 | C2 | 1 | 0 | 3,4,8,9 | 2 | 1 | 2 | 6 |
| 208 | C2 | 1 | 0 | 3,4,8,9 | 2 | 2 | 2 | 6 |
| 209 | C2 | 1 | 0 | 1,3,5,7,9 | 2 | 1 | 2 | 6 |
| 210 | C2 | 1 | 0 | 0,1,2,3,4,5,6,7,8,9 | 8 | 1 | 1 | 6 |

Msg3 enhancement

Msg3 repetition

In RAN1#119, the following agreement was reached on Msg3 PUSCH repetition.

Agreement For Msg3 PUSCH repetition Configuration 1, the valid symbol type is determined based on the symbol type of the first transmission occasion. - UE does not expect the first transmission occasion includes different symbol types (i.e., SBFD symbols and non-SBFD symbols). - FFS: Available slot counting on SBFD symbols.

Available slot counting for Msg3 repetition was FFS.  In Rel.17, Msg3 repetition was introduced with available slot counting. Thus, it would be beneficial to support Msg3 repetition with available slot counting even for SBFD symbol. For PUSCH repetition type A, the following agreement was made in AI 9.3.1 in RAN1#119.

Agreement It is clarified that ‘the first transmission/reception’ in the following agreement in RAN1#118 refers to the first transmission/reception occasion indicated by scheduling DCI. - It is not expected that the first transmission/reception occasion indicated by scheduling DCI is mapped to both SBFD and non-SBFD symbols. Agreement For Configuration 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only, the valid symbol type is determined as following. <Unrelated part omitted> - For dynamically scheduled transmissions/receptions, the valid symbol type is determined based on the symbol type of the first transmission/reception. <Unrelated part omitted>

We think that the above agreement can also be applied to Msg3 repetition.

Proposal 4: For Msg3 repetition, UE postpones transmissions in the invalid symbol type.

Msg3 power control

In RAN1#119, it was agreed to study Msg3 PUSCH power control for SBFD as follows.

Agreement Study how to determine the Msg3 PUSCH power control in SBFD symbols and non-SBFD symbols based on at least the following parameters. - preambleReceivedTargetPower, msg3-DeltaPreamble/deltaPreamble, TPC Command in RAR.

For preambleReceivedTargetPower, it was already agreed to support separate preambleReceivedTargetPower for additional-ROs in in RAN1#119. According to the current specification, preambleReceivedTargetPower can be used to calculate Msg3 transmission power. Thus, the relationship between PRACH and Msg3 would need to be clarified. There are following cases for the combination of PRACH and Msg3 transmission. Note that Case 4 is valid for CFRA case (i.e., since the early indication of SBFD aware UE is not supported, Case 4 is not valid for CBRA case).

- Case 1: PRACH is transmitted in legacy-RO, Msg3 is transmitted in non-SBFD symbols.

- Case 2: PRACH is transmitted in additional-RO, Msg3 is transmitted in SBFD symbols.

- Case 3: PRACH is transmitted in additional-RO, Msg3 is transmitted in non-SBFD symbols.

- Case 4: PRACH is transmitted in legacy-RO, Msg3 is transmitted in SBFD symbols.

When PRACH is transmitted in legacy-RO, power control parameters are configured for non-SBFD symbol. When PRACH is transmitted in additional-RO, power control parameters are configured for SBFD symbol. For Case 1 and Case 2, the symbol types of power control parameters are the same between PRACH and Msg 3 (i.e., non-SBFD symbol for Case 1 and SBFD symbol for Case 2). Then, the existing procedure can be applied to calculate Msg3 transmission power (i.e., preambleReceivedTargetPower which is used for PRACH transmission is applied to Msg3). For Case 3 and Case 4, the symbol types of power control parameters are different between PRACH and Msg 3. Thus, if separate preambleReceivedTargetPower for SBFD symbol is configured, how to use preambleReceivedTargetPower for Case 3 and Case 4 needs to be clarified. In our view, preambleReceivedTargetPower corresponding to symbol type can be used because preambleReceivedTargetPower is adjusted for each symbol type. For Case 3, preambleReceivedTargetPower for non-SBFD symbol is used. For Case 4, preambleReceivedTargetPower for SBFD symbol is used.

Proposal 5: If separate preambleReceivedTargetPower for SBFD symbol is configured, Msg3 transmission power is calculated by preambleReceivedTargetPower corresponding to symbol type of Msg3.

For Msg3 retransmission, symbol type can be different from initial Msg3 transmission (e.g., initial Msg3 transmission is transmitted in SBFD symbols, but Msg3 retransmission is transmitted in non-SBFD symbols). If symbol types between initial Msg3 transmission and retransmission are different, complexity to determine transmission power for Msg3 retransmission can be increased (e.g., use of power control parameters can be different). In order to adjust transmission power, to use TPC command can be considered. However, if required transmission power is very different between initial Msg3 transmission and Msg3 retransmission, to adjust by TPC command can be insufficient because TPC value range is limited (i.e., -1 dB to 3 dB for accumulated value by UL grant). Whether initial transmission and retransmission is restricted to controllable by the gNB scheduling. Therefore, we propose following.

Proposal 6: No specification modification related to Msg3 retransmission. Msg3 retransmission is not restricted to the same symbol type of initial Msg3 transmission. The possible power difference between initial Msg3 transmission and Msg3 retransmission is only to be managed by TPC command of the current specification regardless of the different symbol type.

2-Step RACH

In RAN1#116, the following agreement was reached on supporting 4-step RACH for SBFD symbol. Whether/how to support 2-step RACH is not concluded yet.

Agreement: For SBFD aware UEs in RRC CONNECTED state, support Type-1 random access procedure (4-step RACH) in SBFD symbols. - FFS Type-2 random access procedure (2-step RACH)

The main motivation to support 2-step RACH in SBFD symbol would be latency reduction. However, the latency of random access procedure in 4-step RACH in SBFD symbols can be reduced even if 2-step RACH is not supported (i.e., PRACH and Msg3 transmission opportunities can be increased by using UL subband). In order to support 2-step RACH for SBFD, at least the following points would need to be discussed.

- MsgA PRACH/PUSCH configuration for SBFD (e.g., separate configurations for legacy 2-step RACH and SBFD 2-step RACH)

- PO validation rule

- RO-to-PO mapping

The above issues would require discussion time especially for MsgA PRACH/PUSCH configuration for SBFD, because for 4-step RACH, PRACH configuration for SBFD required for a lot of discussion time (and is still ongoing). Since 2-step RACH is optional feature, we don't think that 2-step RACH in SBFD symbols is essential.

Proposal 7: Deprioritize to support 2-step RACH in Rel.19.

Conclusion

In this contribution, we made the following proposals.

Proposal 1: Support separate maximum output power PCMAX for a slot containing SBFD symbol.

Proposal 2: For the case that additional ROs overlap with legacy ROs for RACH configuration Option 2, support Alt-1.

Proposal 3: For interpretation of the parameter prach-ConfigurationIndex provided by the additional RACH configuration for RACH configuration Option 2, support Alt-1.

Proposal 4: For Msg3 repetition, UE postpones transmissions in the invalid symbol type.

Proposal 5: If separate preambleReceivedTargetPower for SBFD symbol is configured, Msg3 transmission power is calculated by preambleReceivedTargetPower corresponding to symbol type of Msg3.

Proposal 6: No specification modification related to Msg3 retransmission. Msg3 retransmission is not restricted to the same symbol type of initial Msg3 transmission. The possible power difference between initial Msg3 transmission and Msg3 retransmission is only to be managed by TPC command of the current specification regardless of the different symbol type.

Proposal 7: Deprioritize to support 2-step RACH in Rel.19.

- Reference

- RP-240789, "WID revision: Evolution of NR duplex operation: Sub-band full duplex (SBFD)", Huawei

- 3GPP TS 38.211 V18.5.0 (2024-12)