3GPP TSG RAN WG1 #120		R1-2500850

Athens, Greece, February 17th – 21st, 2025

Agenda item:	9.4.1

Source:	Samsung

Title:	Views on Physical channels design – modulation aspects

Document for:	Discussion and decision

# Introduction

Ambient Internet of Things (A-IoT) was studied by 3GPP in Rel-19 with solutions captured in [1]. RAN WGs start a Rel-19 WI with general scopes and objectives in [2].

This contribution discusses issues on modulation aspects of A-IoT physical channels design, including R2D CP handling, OFDM boundary alignment, indication of M values and D2R modulation aspects.

# R2D modulation: OOK-4

## CP handling

In legacy NR OFDM waveform generation, at transmitter side, the CP is copied from the end of the symbol and then prepended at the start of the corresponding symbol. The CP is removed at the receiver when capturing the samples for the FFT window and, therefore, it serves as a guard time to mitigate the inter-symbol interference. By prepending the CP in a circular manner, it also allows to maintain sub-carrier orthogonality and, thereby, preventing inter-carrier interference.

For R2D signal generation with OOK modulation, the CP insertion can be still beneficial in terms of reusing the existing OFDM hardware for OOK signal generation and also for other NR UEs served using OFDM waveform by maintaining the sub-carrier orthogonality. If reusing the same hardware for OFDM from NR Uu, CP will be inserted in the time domain before each OFDM symbol. However, the inserted CP may break the normal line coding scheme by introducing false rising/falling transition edge at the start of, the end of, or within the CP duration.

Therefore, R2D CP handling was discussed in A-IoT SI phase as a key issue to be solved. The following methods for R2D CP handling was agreed and captured in [1]:

For CP handling, the following candidate methods are studied, on the basis of e.g., CP impact on R2D timing acquisition, and decoding & performance of PRDCH, reader and device implementation complexities, interference between R2D and NR DL/UL if in the same NR band, spectrum efficiency. Method Type 1: Removal of CP at device without specified transmit-side. Method Type 2: Ensure the CP insertion of OFDM-based waveform will not introduce false rising/falling edge between the last OOK chip in OFDM symbol (n-1) and the first OOK chip in OFDM symbol n. For Method Type 1, two ways that CP location/length can be determined are studied: Alt M1-1: Device assumes same CP length for each OFDM symbol, i.e. does not distinguish exact CP length among different OFDM symbols Alt M1-2: Duration between transition edges is utilized by device to determine CP location/length, i.e. if the duration appears to be invalid based on known chip duration For Method Type 1, at least for Alt M1-1, device needs to be aware of or determine the boundary of an OFDM symbol (i.e. the beginning of an OFDM symbol) to determine CP location, including the start and the length of CP. Alt M1-1 and/or Alt M1-2 (if both are supported) can be up to device implementation which may depend on M values. For Method Type 2, two approaches regarding subcarrier orthogonality are studied: Alt M2-1: Method Type 2 retains subcarrier orthogonality, i.e. CP is copied from the end of an OFDM symbol. Alt M2-1-1: The first OOK chip(s) and the last OOK chip(s) in an OFDM symbol are the same. Alt M2-1-2: Ensure a transition edge occurs only at the start or only at the end of the CP, and no transition edge occurs during the CP. Alt M2-2: Method Type 2 does not retain subcarrier orthogonality. For Method Type 2, depending on the design, the chip duration generation of OOK-4 for M-chip per OFDM symbol transmission could possibly be determined by: - M, and the length of OFDM symbol with CP - M, and the length of OFDM symbol without CP - Depending on detailed solutions, chip duration may or may not be constant.

For Method Type 1, A-IoT devices are expected to determine CP position, and remove corresponding sampling points during the CP position, or the false decoding results of rising/falling transition edge within the CP duration. Given that the start of R2D transmission from the reader perspective is assumed to be aligned with the boundary of an NR OFDM symbol (including the CP) for in-band/guard-band operation, as captured in [1], in general, the CP removal can be performed by device counting the indices of sampling points and removing sampling points with indices corresponding to CP duration, e.g. sampling points 0~9, 138~146, 275~283, etc..

With this alternative, potential synchronization drift due to SFO may impact the CP removal and result in false rising/falling edge and degrades decoding reliability. Consequently, Method Type 1 increases the requirement on device synchronization accuracy. Figure 1 illustrates an example that how synchronization drift impacts the performance of CP removal and R2D decoding. The impact of synchronization drift will degrade the performance of both Alt M1-1 and Alt M1-2, which is analyzed in the following sections.

Figure 1 Examples of incorrect CP removal and false detection due to synchronization drift

Observation 1: The impact of synchronization inaccuracy should be carefully considered and justified for each alternative of CP handling Method Type 1.

Alt M1-1 proposes to solve the issue of different CP length between LCP of NR OFDM symbol 0, 7 and NCP of other OFDM symbols. According to 38.211, the length of LCP is 4.68μs and the length of NCP is 5.2μs under 15 KHz SCS. Assuming sampling frequency of 1.92Msps as baseline, LCP and NCP correspond to 10 and 9 sampling points, respectively. Therefore, when Alt M1-1, there exists offset of 1 sampling point per LCP. Assuming the offset is not accumulated across OFDM slots by device implementation, there will be an offset of 1 or 2 sampling points for some code chips.

The impact of the offset of sampling points depends on length of code chips. Assuming 15 KHz SCS and 1.92Msps sampling frequency, each OFDM symbol excluding CP part corresponds to 128 sampling points. For smaller M values, e.g. M=4, each code chip corresponds to 32 sampling points, and the offset due to Alt M1-1 may be negligible. However, for large M values, e.g. M=32, each code chip corresponds to 4 sampling points, thus Alt M1-1 will introduce a half of code chip offset, which is challenging for R2D decoding performance.

In addition, the impact of synchronization drift due to SFO may increase the offset of sampling points due to Alt M1-1. Assuming 0.5*10^5 ppm initial SFO as a moderate scenario, for a typical length of 20 bits payload size, the time drift can be 20%~40% of code chip length at the end of PRDCH transmission, depending on different transmission block structure assumptions. Even if considering synchronization calibration according to Manchester line encoding waveform, there are still residual SFO especially for large M value cases. As a combination result of SFO impact and sampling offset due to LCP/NCP length difference, a typical sampling offset of 25%~90% code chip length for large M values may be existed although the payload size is small. Therefore, Alt M1-1 is not appropriate for large M values.

Observation 2: Alt M1-1 is not appropriate for large M values due to sampling offset by not distinguishing LCP and SFO impacts. For instance, with M=32, a typical sampling offset of 25% ~ 90% of code chip length are observed for small payload size of 20 bits, and the sampling offset for large payload size e.g. 96 bits will further increase due to SFO impact, which may severely impacts R2D reception performance.

Alt M1-2 proposes to distinguish code chip and false chip during CP position by the duration between transition edges. An example is illustrated in Figure 2, wherein T denotes the nominal chip length. The actual chip length may have a slight variation from its nominal value, i.e., T±δ. Δ denotes the nominal CP length, which may also have a slight variation due to inaccurate clocks at the transmitter or receiver, signal propagation, or sampling timing error.

Figure 2 Example of Alt M1-2 for CP removal based on chip duration

Based on the rising/falling edge detection, the OOK chips can be detected as:

- A chip with duration T+Δ, e.g., C1 in symbol n, due to CP having the same value with the chip.

- A chip with duration Δ, e.g., CP in symbol n+1, due to CP having different value with the neighboring chips.

- A chip with duration T, 2T, …, etc., which are normal chips.

The receiver detects chips based on the detected chip duration as follows:

- If a detected chip duration is Δ, which may be further associated with relative variation parameter δ such that the detected chip duration falls into [Δ–δ, Δ+δ], the receiver identifies that the corresponding chip as a CP and ignores it as invalid.

- If a detected chip duration is T+Δ, which may be further associated with relative variation parameter δ such that the detected chip duration falls into [T+Δ–δ, T+Δ+δ], the receiver identifies that the corresponding chip is composed of CP plus a normal chip and detects it as a single chip.

- Otherwise, the detection decision is made based on the nominal chip duration T, i.e., an integer multiples of T with some variation ±δ.

Therefore, the performance of Alt M1-2 for CP handling may depend on two aspects: how strong the CP duration Δ differentiates with normal code chip length T or 2T, and how strong the time offset δ impacts the differentiation among {T+Δ, Δ, T or 2T}. For large M values, it is hard for the receiver to distinguish CP and code chip by the time lengths. For example, with M=12 or M=16, T corresponds to 10.67 and 8 sampling points, respectively, where LCP corresponds to 10 sampling points and NCP corresponds to 9 sampling points. Therefore, Alt M1-2 is not applicable for large M values, e.g. M>8.

In addition, synchronization inaccuracy will further decrease the performance of Alt M1-2 for large M values. As analyzed above, the time drift can be 20%~40% of code chip length for 20 bits payload size, corresponding to an offset of 1~4 sampling points. The SFO impacts will make Alt M1-2 more challenging to be applicable for M>8 values.

Observation 3: Alt M1-2 is not appropriate for large M values, e.g. M>8, due to similar chip length and SFO impacts. For instance, with M=12 or M=16, the time length of LCP/NCP and code chip have only 0~2 sampling points difference and become non-distinguishable at receiver side.

Combinations of Alt M1-1 and Alt M1-2 was also considered. However, with large M values under which Alt M1-1 cannot work, the introduction of Alt M1-2 could only introduce limited improvement, since the chip length and sampling offset may also have similar length, as illustrated in previous paragraphs. In addition, both alternatives are impacted by SFO severely under large M values, and the combination cannot resist the SFO impact as well. Therefore, Method type 1 may work for small M values, but are not preferred for large M values.

Proposal 1: Method type 1 with Alt M1-1, or Alt M1-2, or combination of both alternatives, is not preferred for large M values. For small M values, its performance can be further justified.

For Method Type 2, there are different alternatives on whether to retain subcarrier orthogonality. Alt M2-1 retains subcarrier orthogonality with two candidate alternatives Alt M2-1-1: The first OOK chip(s) and the last OOK chip(s) in an OFDM symbol are the same, and Alt M2-1-2: Ensure a transition edge occurs only at the start or only at the end of the CP, and no transition edge occurs during the CP.

In Alt M2-1-1, with small M values under which CP corresponds to single code chip, the code chips in CP duration and the first OOK chips are the same, therefore the start boundary of CP will keep same situation before CP insertion, and the end boundary of CP introduces no additional transition edge. Therefore, Alt M2-1-1 could work well under small M values with no additional transition edge.

With large M values under which CP corresponds to multiple code chips, e.g. M>16, Alt M2-1-1 needs to further ensure no transition edge within the inserted CP, and correspondingly no transition edges within the first and last OOK chips in an OFDM symbol. One possible solution is the first OOK chip(s) and/or the last OOK chip(s) are generated by padding chips, e.g. all '1' chips. Figure 3 illustrates an example of padding at the first OOK code chip(s) and last code chip(s) for every OFDM symbol. It should be noticed that although the padding code chips in the figure are shown as per chip at start and end of OFDM symbol, and the data encoded chips in the figure are shown as two chips, but the exact number of padding chips and encoded chips can be flexibly modified according to M values. For example, with M=24, wherein CP duration ≈ 1.875 (LCP)/1.688 (NCP) code chips, the first and last two OOK chips are padding chips, and the remaining 20 OOK chips are line encoded chips. The overhead of padding chips varies depending on M values as well. At least for large M values, the overhead can be acceptable.

After insert the padding chips, false rise/falling edge will not happen within CP, and may happen at the boundary of the padding chips. However, since the total length of {padding chips in OFDM symbol n-1, CP, padding chips in OFDM symbol n} (blue marked line in the figure) is much larger than code chip length, it becomes easier to be distinguished by the receiver and can better resist SFO impacts. Alt M2-1-1 can be used in combination with Alt M1-2 that the receiver removes the padding chips according to duration between transition edges and the expected CP position.

Figure 3 Example of Alt M2-1-1 with padding chips at first and last OOK chip(s)

In Alt M2-1-2, the principle can be similarly satisfied by padding code chips, e.g., only add the padding code chips at last OOK chips of every OFDM symbol. In this case, the voltage level of last OOK chips in OFDM symbol n-1 and inserted CP in OFDM symbol n are always the same, and transition edge may or may not happen at the boundary of inserted CP and first OOK chips of OFDM symbol n. Compared with Alt M2-1-1, the overhead of padding chips are reduced as the expense of decreased differentiation on padding length and code chip length. Therefore, Alt M2-1-2 can be considered as a trade-off method of Alt M2-1-1.

Proposal 2: Method Type 2 with Alt M2-1-1 or Alt M2-1-2, performed by insert padding chips at both start and end OOK chips or only at end OOK chips, is preferred, especially for large M values. For small M values, the overhead and performance can be further justified.

Therefore, the following is proposed for CP handling issue:

Proposal 3: For CP handling with large M values, Method Type 2 with Alt M2-1-1 or Alt M2-1-2 are supported. For CP handling with small M values, down-select between Method Type 1 with Alt M1-1/Alt M1-2/combination of both alternatives and Method Type 2 with Alt M2-1-1 or Alt M2-1-2. Further study the threshold of small/large M values.

Given that breaking the subcarrier orthogonality will introduce significantly increased interference, at least for in-band operation, Alt M2-2 is not preferred. According to [3], spectrum deployment includes in-band to NR and standalone in Rel-19, and R2D supports one solution for CP handling. Therefore, even for standalone deployment Alt M2-2 is not preferred as well.

Proposal 4: Alt M2-2: Method Type 2 does not retain subcarrier orthogonality, is not preferred for Rel-19 A-IoT.

## OFDM symbol boundary alignment

Physical layer structure design for R2D was discussed in A-IoT SI with OFDM symbol boundary alignment aspects captured in [1] as follows:

#### 6.1.1.1 R2D waveform, modulation and numerology An OFDM-based OOK waveform with subcarrier spacing of 15 kHz is studied for R2D, with OOK-1 for single-chip per OFDM symbol transmission, and OOK-4 for M-chip per OFDM symbol transmission, starting from the definitions in TR 38.869 [4]. For this waveform, the start of R2D transmission from the reader perspective is assumed to be aligned with the boundary of an NR OFDM symbol (including the CP) for in-band/guard-band operation. For the generation of this waveform, CP-OFDM and DFT-s-OFDM are both feasible. For M=1, i.e. use of OOK-1 or OOK-4 for single-chip per OFDM symbol transmission, CP-OFDM and DFT-s-OFDM are both feasible. For M>1, i.e. use of OOK-4 for M-chip per OFDM symbol transmission, DFT-s-OFDM is feasible. For R2D waveform generation using a DFT-s-OFDM based transmitter, from the transmitter perspective, when the generated number of chips for the R2D transmission does not fully occupy the last OFDM symbol, at least padding can be used.

As illustrated in Section 2.1, the long CP for NR OFDM symbol 0, 7 and normal CP for other NR OFDM symbols have different time length corresponding to 1 sampling points under 15 KHz SCS and 1.92Msps sampling frequency. At least when CP removal is applied for Method type 1 or combination of Method type 1+2, it is beneficial to ensure that receiver can distinguish LCP and NCP to accurately remove the sampling points within CP, especially for large M values.

Proposal 5: For the purpose of better solving CP handling issue, the start of R2D transmission should be aligned with NR half-slot boundary to facilitate the determination of underlying OFDM CP length and removal.

For R2D waveform generation using a DFT-s-OFDM based transmitter, when the generated number of chips for the R2D transmission does not fully occupy the last OFDM symbol, as illustrated by Figure 4, at least padding can be used for OFDM symbol boundary alignment, otherwise it will impact interference estimation from adjacent readers or coexisting NR system. The details of padding needs to be further discussed.

Figure 4 A illustration of a partially occupied last OFDM symbol

The content of padding can generally be consisted of bits or code chips with pre-defined value, e.g. padding with all-zero bits, or padding with low voltage level chips. In addition, if the PRDCH is attached with a postamble, given that postamble can be used to detect end of transmissions, the padding can also be repetitions of postamble to further improve the functionality of postamble and reduce miss-detections.

The length of padding is also relevant with CRC attachment and the existence of postamble. If zero-bits are used for padding, it can be added before CRC attachment, e.g., the length of padding bits can be determined and added in the baseband before the CRC attachment, if CRC is attached, such that the end of the PRDCH transmission after line encoding is aligned with the next OFDM symbol boundary. If zero-chips are used for padding, it can be added after CRC attachment, but it needs to be distinguished with PRDCH including CRC to ensure correct CRC check. If the PRDCH is attached with a postamble, the length of padding bits are determined such that the end of postamble is aligned with the next OFDM symbol boundary. Alternatively, a padding signal is added to PRDCH after line encoding and postamble, if exists. The padding signal can be left for reader implementation in this case.

Proposal 6: When the generated number of chips for the R2D transmission does not fully occupy the last OFDM symbol, padding is used.

- Study the following as content of padding: zero-bit(s), zero-code chip(s), repetition of postamble (if supported).

- Study whether padding is added before or after CRC attachment, depending on content of padding

- Regarding length of padding, if PRDCH is attached with a postamble, study the following options:

  - Option 1: the length of padding bits are determined such that end of postamble is aligned with the next OFDM symbol boundary

  - Option 2: the padding is added to PRDCH after line encoding and postamble, and end of the padding is aligned with the next OFDM symbol boundary

## Indication for OOK-4 M value

The number of OOK-4 chips per OFDM symbol, M values, was discussed in A-IoT SI with the following captured in [1]:

Table 6.1.1.4-1 is a starting point for study of M values and the associated minimum Btx,R2D value. The reader can use any transmission bandwidth greater than or equal to the minimum Btx,R2D value. Note: Depending on further study, the maximum value of M may be less than 32. Note: The performance can be better when transmission bandwidth greater than the minimum Btx,R2D, depending on device processing and transmit power constraint. Table 6.1.1.4-1: Starting point for M values and the associated minimum Btx,R2D value M; Minimum Btx,R2D # of PRBs. 1; 1. 2; 1. 4; 1. 6; 1. 8; 2. 12; 2. 16; 2. 24; 3. 32; 4.

Given that multiple M values can be supported by A-IoT system to accommodate different deployment scenarios and data rates, the practically used M value for a given PRDCH transmissions should be indicated. Since the device have limited capability, M values should be indicated before start of PRDCH reception to avoid blind detection of M values. Therefore, the M values should be indicated by preamble of PRDCH.

Proposal 7: For R2D transmission, the M value of a given PRDCH is indicated by preamble associated with the PRDCH.

Both explicit indication and implicit indication of M values can be studied. For explicit indication, it can be performed by preamble indicating information bits regarding M values or different sequences corresponding to M values. However, the explicit indication will increase the overhead of preamble or increases preamble detection complexity. Implicit indication can be performed by a pre-defined mapping between chip length of preamble and chip length of PRDCH. It can be easily implemented by the device without additional effort and is preferred for Rel-19 A-IoT.

Proposal 8: M values of PRDCH can be implicitly indicated by OOK chip length of preamble.

It may also be discussed whether PRDCH use the same M values for control information and data. Figure 5 illustrates a situation where L1 control of PRDCH indicates M value corresponding to a chip rate for the payload, which can be potentially different from the chip rate obtained from the preamble.

First of all, there is no clear motivation for having different chip rates between control and data as both control and data parts go through the same channel condition and have to be received. Also, it will increase the device complexity for switching chip rates quickly between control and data without clear motivation, which can be challenging for devices.  For the reception of data part, the device has to deduce the chip synchronization from that obtained from the clock-acquisition part rather than it can assume that the chip synchronization is directly conveyed over. This will require additional complexity as well as lose precision.

Proposal 9: For PRDCH reception, the M value is identical for both control and data parts.

Figure 5 PRDCH payload chip rate indicated by corresponding L1 control

# D2R modulation: OOK and BPSK

For D2R backscattering, OOK and BPSK modulations are supported per WID [2]. Since D2R modulation is non-OFDM based, it has no restrictions such as OFDM symbol boundary alignment or CP handling as R2D. In our view, both OOK and BPSK are feasible for device 1 and should be supported. BPSK especially has an advantage of coherent demodulation at the reader.

Proposal 10: For D2R backscattering, both OOK and BPSK modulations are supported per WID.

For the device to determine OOK chip or BPSK symbol rate, a device first needs to obtain a reference chip rate from the R2D preamble associated with the PRDCH providing the D2R scheduling information. Based on the reference chip rate, the device can be further indicated, via R2D control information in the PRDCH, OOK chip rate or BPSK symbol rate for D2R transmission with respect to the reference chip rate. As an example shown in Figure 6, D2R OOK chip rate or BPSK symbol rate may be indicated to be the same as the reference chip rate obtained from the R2D preamble, or increase/decrease factor from the reference chip rate can be indicated.

Proposal 11: To determine PDRCH OOK chip rate or BPSK symbol rate, a device first obtains a reference chip rate from the R2D preamble and multiplies rate factor to the reference chip rate, wherein the rate factor is indicated in the R2D control information in the corresponding PRDCH.

Figure 6 PDRCH chip/symbol rate determination

Figure 7 illustrates an example PDRCH signal structure with midamble. When BPSK modulation is used, the need for midamble is more pronounced to support coherent demodulation at the reader. Therefore, at least when BPSK modulation is used for PDRCH, midamble should be supported.

Proposal 12: Midamble is supported for D2R, at least, when BPSK modulation is used.

Figure 7 PDRCH signal structure with midamble

# Conclusion

In this contribution, we discussed modulation aspects for physical channel design with the following observations and proposals:

Observation 1: The impact of synchronization inaccuracy should be carefully considered and justified for each alternative of CP handling Method Type 1.

Observation 2: Alt M1-1 is not appropriate for large M values due to sampling offset by not distinguishing LCP and SFO impacts. For instance, with M=32, a typical sampling offset of 25% ~ 90% of code chip length are observed for small payload size of 20 bits, and the sampling offset for large payload size e.g. 96 bits will further increase due to SFO impact, which may severely impacts R2D reception performance.

Observation 3: Alt M1-2 is not appropriate for large M values, e.g. M>8, due to similar chip length and SFO impacts. For instance, with M=12 or M=16, the time length of LCP/NCP and code chip have only 0~2 sampling points difference and become non-distinguishable at receiver side.

Proposal 1: Method type 1 with Alt M1-1, or Alt M1-2, or combination of both alternatives, is not preferred for large M values. For small M values, its performance can be further justified.

Proposal 2: Method Type 2 with Alt M2-1-1 or Alt M2-1-2, performed by insert padding chips at both start and end OOK chips or only at end OOK chips, is preferred, especially for large M values. For small M values, the overhead and performance can be further justified.

Proposal 3: For CP handling with large M values, Method Type 2 with Alt M2-1-1 or Alt M2-1-2 are supported. For CP handling with small M values, down-select between Method Type 1 with Alt M1-1/Alt M1-2/combination of both alternatives and Method Type 2 with Alt M2-1-1 or Alt M2-1-2. Further study the threshold of small/large M values.

Proposal 4: Alt M2-2: Method Type 2 does not retain subcarrier orthogonality, is not preferred for Rel-19 A-IoT.

Proposal 5: For the purpose of better solving CP handling issue, the start of R2D transmission should be aligned with NR half-slot boundary to facilitate the determination of underlying OFDM CP length and removal.

Proposal 6: When the generated number of chips for the R2D transmission does not fully occupy the last OFDM symbol, padding is used.

- Study the following as content of padding: zero-bit(s), zero-code chip(s), repetition of postamble (if support).

- Whether padding is added before or after CRC attachment, depending on content of padding

- Regarding length of padding, if PRDCH is attached with a postamble, study the following options:

  - Option 1: the length of padding bits are determined such that end of postamble is aligned with the next OFDM symbol boundary

  - Option 2: the padding is added to PRDCH after line encoding and postamble, and end of the padding is aligned with the next OFDM symbol boundary

Proposal 7: For R2D transmission, the M value of a given PRDCH is indicated by preamble associated with the PRDCH.

Proposal 8: M values of PRDCH can be implicitly indicated by OOK chip length of preamble.

Proposal 9: For PRDCH reception, the M value is identical for both control and data parts.

Proposal 10: For D2R backscattering, both OOK and BPSK modulations are supported per WID.

Proposal 11: To determine PDRCH OOK chip rate or BPSK symbol rate, a device first obtains a reference chip rate from the R2D preamble and multiplies rate factor to the reference chip rate, wherein the rate factor is indicated in the R2D control information in the corresponding PRDCH.

Proposal 12: Midamble is supported for D2R, at least, when BPSK modulation is used.

# References:

- TR 38.769-201, Study on solutions for ambient IoT (Internet of Things)

- RP-243326, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR, 3GPP TSG RAN Meeting #106, Madrid, Spain, December 9th-12th, 2024

- RAN1 Chair’s Notes, 3GPP TSG RAN WG1 #119, Orlando, US, November 18th – 22nd, 2024.