| 3GPP TSG RAN WG1 #120 | R1-2501014 |
| --- | --- |
| Athens, Greece, February 17th – 21st, 2024 |  |

Agenda Item: 9.1.3

Source: MediaTek Inc.

Title:	Specification support for CSI prediction

Document for: Discussion & Decision

# Consistency between Training and Inference

Following the RAN1#118bis meeting [1], it was agreed that we need to identify which potential NW-side additional conditions, if any, may impact on the UE assumption for CSI prediction using a UE-sided model.

Agreement For consistency between training and inference, study to identify which potential NW-side additional conditions, if any, may impact on UE assumption for CSI prediction using UE-sided model, resulting non-negligible degradation on model generalization performance. Note: Companies are encouraged to provide generalization performance evaluation on the impact of NW-side additional conditions, if considered.

Companies also provide potential NW-side additional conditions that need further evaluation. We will further conduct generalized evaluations on these settings.

Agreement For generalization evaluation to identify potential NW-side additional conditions to ensure consistency between training and inference, consider one or more of the following aspects, if evaluated: - [Optional evaluation] Various tilt angle (e.g., [102] degrees, [110] degrees) - [Optional evaluation] Various TXRU mapping - With TXRU virtualization (e.g., for 32 port CSI-RS, [N1, N2, P] = [2, 8, 2] (baseline), [4,4,2] (optional) with (8x8x2) and (12 x 8 x 2) antenna elements) - Without TXRU virtualization (e.g., for 32 port CSI-RS, [N1, N2, P] = [2, 8, 2] (baseline), [4,4,2] (optional) with (2x8x2) and (4 x 4 x 2) antenna elements, respectively) - Other configurations are not precluded Note: for other evaluation assumptions, reuse baseline of TR 38.843 Note: report how to map antenna port to antenna elements Note: report the backbone/structure of AI/ML model used

## Generalization over Tilt angle

In this section, we evaluate the generalization performance of the base station (BS) downtilting angle. Both 102 degrees and 110 degrees are considered for UMa scenario. Other parameter settings refer to the Table 4-1, where the length of the observation window is 5/5ms, and the length of the prediction window is 1/5ms. We consider the following three cases agreed at RAN WG1 #110 [3]:

- Case 1: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a dataset from the same Scenario#A/Configuration#A

- Case 2: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B

- Case 3: The AI/ML model is trained based on training dataset constructed by mixing datasets from multiple scenarios/configurations including Scenario#A/Configuration#A and a dataset different than Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a dataset from a single Scenario/Configuration from the multiple scenarios/configurations

Table 1-1 shows the SGCS results of CSI prediction across different tilt angles, and Table 1-2 displays the NMSE results. In the table, the dark blue text refers to Case 1, black refers to Case 2, and light blue refers to Case 3. The brackets indicates the differences compared to Case 1. We can observe that all the cases show only minor differences (SGCS loss < 2%). In addition, there is not much difference or gain between mixed training and non-mixed training. This indicates that AI/ML-based CSI prediction is not significantly affected by tilt angles and can generalize well across different tilt angles.

Table 1-1: Generalization SGCS results for AI/ML-based CSI prediction over tilt angle

| Training Inference | 102 degrees | 110 degrees | Mixed [102, 110] degrees |
| --- | --- | --- | --- |
| 102 degrees | 0.9634 | 0.9448 (-1.93%) | 0.9550 (-0.87%) |
| 110 degrees | 0.9500 (-1.24%) | 0.9619 | 0.9607 (-0.12%) |

Table 1-2: Generalization NMSE results for AI/ML-based CSI prediction over tilt angle

| Training Inference | 102 degrees | 110 degrees | Mixed [102, 110] degrees |
| --- | --- | --- | --- |
| 102 degrees | -13.78 | -11.61 (+2.17) | -12.85 (+0.93) |
| 110 degrees | -12.24 (+1.44) | -13.68 | -13.42 (+0.26) |

For CSI prediction using UE-sided model, for consistency between training and inference, tilt angle at NW-side has negligible (SGCS loss < 2%) degradation on model generalization performance.

## Generalization over TXRU mapping

In this section, we evaluate the generalization performance of the various TXRU mappings. We consider the 32 ports CSI-RS with the following settings as per the agreement:

- With TXRU virtualization ([N1, N2, P] = [2, 8, 2], [4,4,2] with (8x8x2) and (12 x 8 x 2) antenna elements, respectively)

- Without TXRU virtualization ([N1, N2, P] = [2, 8, 2], [4,4,2] with (2x8x2) and (4 x 4 x 2) antenna elements, respectively)

Table 1-3 shows the SGCS results of CSI prediction across various TXRU mappings, and Table 1-4 displays the NMSE results. We can draw a similar conclusion as in section 1.1: all the cases exhibit only minor differences (SGCS loss < 2%). Based on these results, we can conclude that AI/ML-based CSI prediction is not significantly impacted by TXRU mapping and can generalize well across different TXRU mappings. This conclusion align with our analysis in the CSI compression agenda, where we found that the loss is less than 5% compared to Generalization Case 1.

The above simulation results show that AI/ML-based CSI prediction is not affected by different BS tilt angles and TXRU mappings, indicating that the AI/ML model generalizes well to these variations. A possible reason for this robustness is that CSI prediction primarily depends on small scale fading effects, such as multipath propagation and Doppler effects, which capture the rapid fluctuations in the channel. While tilt angles and TXRU mappings influence large scale effects like path loss and shadowing, these variations do not significantly impact the small scale fading characteristics that the AI/ML model is trained to predict. Therefore, the model's capability to accurately predict CSI is maintained across different tilt angles and TXRU configurations.

Table 1-3: Generalization SGCS results for AI/ML-based CSI prediction over TXRU mapping

| Training Inference | With TXRU virtualization |  | Without TXRU virtualization |  | Mixed the above four configurations |
| --- | --- | --- | --- | --- | --- |
|  | [N1, N2, P] = [2, 8, 2] with (8x8x2) AEs | [N1, N2, P] = [4, 4, 2] with (12x8x2) AEs | [N1, N2, P] = [2, 8, 2] with (2x8x2) AEs | [N1, N2, P] = [4, 4, 2] with (4x4x2) AEs |  |
| [2, 8, 2] with (8x8x2) AEs | 0.9613 | 0.9525 (-0.92%) | 0.9600 (-0.14%) | 0.9570 (-0.45%) | 0.9585 (-0.29%) |
| [4, 4, 2] with (12x8x2) AEs | 0.9553 (-0.29%) | 0.9581 | 0.9464 (-1.22%) | 0.9553 (-0.29%) | 0.9563 (-0.19%) |
| [2, 8, 2] with (2x8x2) AEs | 0.9533 (-0.17%) | 0.9388 (-1.69%) | 0.9549 | 0.9464 (-0.89%) | 0.9484 (-0.68%) |
| [4, 4, 2] with (4x4x2) AEs | 0.9338 (-0.84%) | 0.9281 (-1.44%) | 0.9340 (-0.82%) | 0.9417 | 0.9384 (-0.35%) |

Table 1-4: Generalization NMSE results for AI/ML-based CSI prediction over TXRU mapping

| Training Inference | With TXRU virtualization |  | Without TXRU virtualization |  | Mixed the above four configurations |
| --- | --- | --- | --- | --- | --- |
|  | [N1, N2, P] = [2, 8, 2] with (8x8x2) AEs | [N1, N2, P] = [4, 4, 2] with (12x8x2) AEs | [N1, N2, P] = [2, 8, 2] with (2x8x2) AEs | [N1, N2, P] = [4, 4, 2] with (4x4x2) AEs |  |
| [2, 8, 2] with (8x8x2) AEs | -13.38 | -12.54 (+0.84) | -13.25 (+0.13) | -12.78 (+0.60) | -13.11 (+0.27) |
| [4, 4, 2] with (12x8x2) AEs | -12.80 (+0.28) | -13.08 | -12.10 (+0.98) | -12.67 (+0.41) | -12.90 (+0.18) |
| [2, 8, 2] with (2x8x2) AEs | -12.52 (+0.05) | -11.28 (+1.29) | -12.57 | -11.51 (+1.06) | -12.00 (+0.57) |
| [4, 4, 2] with (4x4x2) AEs | -11.05 (+0.61) | -10.78 (+0.88) | -11.11 (+0.55) | -11.66 | -11.30 (+0.36) |

For CSI prediction using UE-sided model, for consistency between training and inference, TXRU mapping at NW-side has negligible (SGCS loss < 2%) degradation on model generalization performance.

## Associated ID and Performance Monitoring for Consistency

According to the Moderator's summary of the RAN1#118bis meeting [2], there is a proposal aimed at ensuring consistency between training and inference for CSI prediction that has not been fully discussed:

Proposal 3 For consistency between training and inference for CSI prediction using UE-sided model, further study the following options - Option 1: based on associated ID - FFS on what can be assumed by UE with the same associated ID across training and inference - Option 2: based on performance monitoring - FFS on details - Other options are not precluded

Since AI/ML-based CSI prediction targets UE-sided model, focusing on UE-sided data collection would be more effective. For UE-sided data collection, it is crucial to consider the impact of specific conditions under different configurations and scenarios of the data. Following downlink measurements by the UE (e.g., CSI-RS) and subsequent data collection, this information is processed and transmitted to a dedicated training entity or cloud server for model training. Due to various NW-side additional condition such as gNB-implemented antenna layout, antenna elements to TxRU mapping, and digital/analog beamforming, identical CSI-RS ports may exhibit different channel distributions observed at the UE under different settings.

To significantly improve the accuracy of CSI prediction models, it is necessary to categorize the collected data according to specific scenarios and configurations, enabling the development of targeted AI/ML models for these situations. To ensure consistency between training and inference processes, introducing an associated ID to identify these specific conditions is a straightforward solution. Ongoing discussions in the beam management agenda like existing framework and procedures can be reused. The associated ID based approach avoids revealing too much proprietary information, safeguarding the privacy of NW deployments, while allowing the UE to accurately select the appropriate AI/ML model in different situations.

However, in section 1.1 and section 1.2, we concluded that the AI/ML-based CSI prediction model can generalized well over tilt angle and TXRU mapping, which are part of the NW-side additional conditions. Therefore, it is necessary to study and further analyze which other NW-side additional conditions may impact the generalization of AI/ML-based CSI prediction models. If most of the common NW-side additional conditions do not impact the AI/ML-based CSI prediction model, then we need to further confirm whether using an associated ID for consistency between training and inference in CSI prediction is necessary. In this case, we recommend considering an alternative approach: performance monitoring-based methods. This allows UE to randomly execute their available models based on monitoring resources. Therefore, the NW does not have to manage numerous associated IDs, thus avoiding excessive transmission overhead from sending large volumes of data for various associated IDs. However, in a performance monitoring-based method, the UE must run multiple models or switch between various options to identify the best model until satisfactory performance is achieved. This process can be time-consuming and may increase complexity for the UE. If all models perform poorly during the monitoring check, it may indicate an inconsistency issue between training and inference. Consequently, it will take a considerable amount of time to fallback to the original non-AI method, making this approach less efficient compared to associated ID-based methods.

In conclusion, we propose that in the initial stage, we should confirm which kinds of NW-side additional conditions will impact the generalization of AI/ML-based CSI prediction models. Keeping the associated ID-based approach for consistency between training and inference for CSI prediction as the baseline is recommended since it can leverage the discussions from the beam management agenda. Nevertheless, performance monitoring-based approaches should also be considered as an alternative.

- Study which kinds of NW-side additional conditions will impact the generalization of AI/ML-based CSI prediction models.

- For consistency between training and inference for CSI prediction using UE-sided model, slightly propose the use of the associated ID as the baseline.

- Leverage and reuse the existing discussion and agreements on the beam management agenda for the associated ID framework and procedures as much as possible to reduce the discussion required for CSI prediction.

- Consider performance monitoring-based methods as an alternative approach, allowing the UE to dynamically select and switch between multiple models based on real-time performance monitoring to identify the best model for CSI prediction.

# Data Collection

## Data Collection mechanism

Regarding data collection for the UE-sided CSI prediction model, it is more convenient to collect the data on the UE-side for training, inference, and monitoring. Ensuring consistency between training and inference is crucial. Once this consistency is established, the next step is to design the triggering and requesting mechanism for UE-side data collection. Since the beam management case also involves a single-sided model, the corresponding configuration and mechanism can be reused.

- For CSI prediction using UE-sided model, the data collection mechanism can leverage the Rel-19 AI/ML-based beam management cases as much as possible.

## Capability Report

In addition to aligning the associated IDs, ensuring the CSI prediction capability is necessary for effective data collection, particularly regarding the observation window and the prediction window. This aspect can refer to the Rel-18 MIMO discussion. However, there may be different capability values for AI and non-AI methods. For example, in Rel-18 MIMO, the UE reports a value called N4, which represents the number of DD units and indicates how far it can predict. In case of AI-based CSI prediction, the UE can also report a similar format as N4 but with a different value compared to non-AI based CSI prediction. To differentiate between AI and non-AI cases, we could add an extra bit to the existing N4 value to indicate whether it represents AI or non-AI capability. The other potential capabilities of the UE that may need to be provided to the NW are: one bit to indicate whether it supports model fine-tuning, one bit to indicate whether it supports non-AI based CSI prediction (to determinate whether the model fallback process can be performed), and any other relevant information used for model monitoring.

- For AI/ML-based CSI prediction, UE needs to report the prediction capability to NW. The prediction capability can be the same format as the N4 value defined in the Rel-18 MIMO. To distinguish between AI and non-AI capabilities, an extra bit can be added to the existing N4 value.

- For AI/ML-based CSI prediction, UE needs to report the model monitoring related capabilities to NW. For example, one bit to indicate whether it supports model fine-tuning, one bit to indicate whether it supports non-AI based CSI prediction.

# Model Monitoring

## Ground Truth Configuration

In the model monitoring process, it is essential to calculate the performance metrics to monitor whether the model can effectively continue operating in the current scenario. To calculate these metrics, the ground truth data is required. For the CSI prediction problem, since it is the time series problem, we can directly use the currently received CSI as our ground truth data. The predicted CSI will be calculated based on the stored CSI. We can then calculate the metrics using both the ground truth and predicted CSIs. However, an issue may arise if there’s a mismatch between prediction and observation intervals; this concern will be highlighted in an upcoming section. Another question arises: If the NW does not configure the periodic/semi-persistent CSI-RS but instead configures aperiodic CSI-RS, additional ground truth will be necessary. We will also address this matter in the following section.

    - Mismatch between observation and prediction intervals

Figure 3-1 shows an example where both observation and prediction intervals are identical. In such cases, there is no requirement for NW to configure additional ground truth CSI-RS to compute performance metrics for the model monitoring process. However, if we consider an example where the observation interval (e.g., 5ms) is not equal to the prediction interval (e.g., 1ms), as shown in Figure 3-2, UE will be unable to acquire the ground truth CSI-RS since these CSI-RS occasions are not configured during the specified CSI-RS periodicity. In addition, more future CSI-RS need to be configured for calculating the performance metrics which will increase the transmission overhead.

Figure 3-1: Illustrate the case where observation and prediction interval are the same

Figure 3-2: Illustrate the case where observation and prediction interval are different

As a result, we propose to calculate the performance metrics solely depends on the predicted CSI-RS, which has identical intervals as the CSI-RS periodicity. Figure 3-3 illustrates our idea, where we calculate the performance metrics only based on the two dark green points at the same intervals as the observed CSI-RS.

Figure 3-3: Illustrate the proposed method for handling observation and prediction intervals is different

- Performance metrics should be calculated only based on the predicted CSI-RS that has the same intervals as the CSI-RS periodicity.

    - Aperiodic CSI-RS configuration

If the NW is configured with aperiodic CSI-RS, the model input can directly use the CSI-RS used for model inference. However, if the monitoring process is activated and the performance metrics need to be calculated, it is necessary to have the ground truth CSI. Therefore, the NW needs to configure additional future CSI-RS with the same periodicity as the observed CSI-RS sent to UE to obtain ground truth CSI. This additional configuration will result in extra transmitted overhead, causing longer latency and impacting the response time of model monitoring.

- For model monitoring purpose, the ground truth CSI-RS should be configured if the NW configures the aperiodic CSI-RS.

# Conclusion

In summary, based on the above discussions, we have the following observations:

- For CSI prediction using UE-sided model, for consistency between training and inference, tilt angle at NW-side has negligible (SGCS loss < 2%) degradation on model generalization performance.

- For CSI prediction using UE-sided model, for consistency between training and inference, TXRU mapping at NW-side has negligible (SGCS loss < 2%) degradation on model generalization performance.

- The AI/ML model trained on TXRU mapping can be generalized and inferenced on other TXRU mapping.

In summary, based on the observations and further analysis, we have the following proposals:

- Study which kinds of NW-side additional conditions will impact the generalization of AI/ML-based CSI prediction models.

- For consistency between training and inference for CSI prediction using UE-sided model, slightly propose the use of the associated ID as the baseline.

- Leverage and reuse the existing discussion and agreements on the beam management agenda for the associated ID framework and procedures as much as possible to reduce the discussion required for CSI prediction.

- Consider performance monitoring-based methods as an alternative approach, allowing the UE to dynamically select and switch between multiple models based on real-time performance monitoring to identify the best model for CSI prediction.

- For CSI prediction using UE-sided model, the data collection mechanism can leverage the Rel-19 AI/ML-based beam management cases as much as possible.

- For AI/ML-based CSI prediction, UE needs to report the prediction capability to NW. The prediction capability can be the same format as the N4 value defined in the Rel-18 MIMO. To distinguish between AI and non-AI capabilities, an extra bit can be added to the existing N4 value.

- For AI/ML-based CSI prediction, UE needs to report the model monitoring related capabilities to NW. For example, one bit to indicate whether it supports model fine-tuning, one bit to indicate whether it supports non-AI based CSI prediction.

- Performance metrics should be calculated only based on the predicted CSI-RS that has the same intervals as the CSI-RS periodicity.

- For model monitoring purpose, the ground truth CSI-RS should be configured if the NW configures the aperiodic CSI-RS.

# References

- Chairman's Notes RAN1#118bis, Hefei, China, October 14th-18th, 2024.

- R1-2409145	Summary #2 of CSI prediction	Moderator (LG Electronics)

- R1-2210753, Summary#7 of [110bis-e-R18-AI/ML-02], Moderator (Huawei), October 2022.

- R1-2410818 Summary #2 of CSI prediction	Moderator (LG Electronics)

# Appendix

Table 4-1: Parameter configuration for generalization test

|  | Dataset |
| --- | --- |
| Channel model | TR 38.901 Dense Urban Macro |
| Carrier frequency | 2GHz |
| Sub-carrier spacing | 15kHz |
| Antenna setup and port layouts at gNB | 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ, Cross-polarization |
| Antenna setup and port layouts at UE | 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ, Cross-polarization |
| Operating BW | 10MHz |
| Data type | Raw channel |
| UE distribution | 100% outdoor with UE speed 30km/h |
| Observation window (number/distance) | 5/5ms |
| Prediction window (number/distance between prediction instances/distance from the last observation instance to the 1st prediction instance) | 1/5ms/5ms |
| Spatial consistency model | Spatial consistency with procedure A |
| Channel estimation | Realistic CSI-RS channel estimation at UE |