3GPP TSG RAN WG1 #120								R1-2501081

Athens, Greece, February 17th – 21st, 2025

Agenda Item:	9.7.1

Source:	AT&T, FirstNet

Title:	ISAC channel model calibration and scenario parameters

Document for:	Discussion/Decision

# Introduction

A study item on channel modeling for Integrated Sensing and Communication (ISAC) was initially agreed in RAN#102 [1] and later updated at RAN#105 [2]. This study item will solely focus on defining a common channel modeling framework to enable wireless sensing in 5G-Advanced and will aim to support object detection and tracking use cases defined in SA1 [3][4]. For those use cases, the following objectives are identified for this study:

To develop a common framework for channel modeling, this contribution addresses further definitions of the deployment scenarios building off of previous agreements and channel model calibrations essential for establishing an ISAC channel model in 5G-Advanced (Rel-19) that will enable use cases for various sensing targets, e.g., object detection and/or tracking including UAVs, Automotive targets, and humans to meet the needs of commercial, public safety, and other verticals.

# Channel Model Calibration Overview

At RAN1#119 and prior RAN1 meetings, good progress was made with the agreements endorsed in the ISAC session report [5] regarding specific evaluation parameters for the UAV, Automotive, Human, AGV and Objects creating hazards sensing targets scenarios. However, to further the channel modeling work and enable channel model calibrations, initial direction for channel model calibration assumptions, parameters, and values must be addressed.

To further this effort, RAN1 agreed to the following email discussion [5]:

[Post-119-ISAC-01] – Jerome (AT&T)

Email discussion on simulation assumptions for channel model calibration, from January 8th-17th:

- Companies are encouraged to provide initial channel model calibration assumptions, parameters, and values

- Jerome to collect inputs, provide potential consolidated tables, proposals for consideration by January 17th

Decision: The discussions are to be carried over to RAN1#120.

The output of the email discussion was captured in R1-2500059 Email discussion summary on ISAC CM calibration assumptions [6] to collect some inputs from interested companies. The final moderator summary captured some views and feedback and the high-level details of CM calibration and some specific simulation assumptions for CM calibration (Stochastic Approach). The following discussion builds on those observations, and we present some potential proposals to progress the ISAC channel model calibration work.

## General Methodology for CM Calibration

As mentioned above, RAN1 can leverage existing agreements as a starting point for channel model calibration assumptions and leverage some of the initial discussions from the [Post-119-ISAC-01] calibration email discussion [6]. In the email discussion, there were two high-level channel model calibration methodology options that were discussed that RAN1 should have an agreement on a way forward.

One option is similar to the stochastic communication channel model calibration approach in TR 38.901 to be leveraged for ISAC channel modelling, which includes parameters and tables for at least the following: large scale parameters, where fast fading is not included and full calibration, which includes fast fading (for example [6] included tables for some of those details). The second option uses a stepwise approach for the ISAC channel model calibration simulations which would consist of deterministic models for the Tx/Rx-target channels, background channels, and combination of the target/background channels, e.g., CDL models limited to channel metrics based on received power and multipath/delay/angular spread.

As a third option, to sufficiently enable companies to compare their implementations of the channel model, RAN1 could leverage a hybrid approach to CM calibration whereby simulations would potentially consist of deterministic models for the Tx/Rx-target paths at least for the LOS paths (perhaps also NLOS). CDL models could be leveraged for channel metrics based on received power and multipath/delay/angular spread. In addition, a stochastic approach, similar to 38.901, may make the most sense for additional more complex components, such as the background channel and modeling of environmental objects. One drawback to this approach is that RAN1 will need to agree on which components should be derived deterministically or stochastically.

Observation 1: A hybrid approach to ISAC channel modelling calibrations could allow for a balance of realistic but not an overly complex means of ISAC CM simulations.

Proposal 1: RAN1 considers a hybrid approach to CM calibration whereby simulations would consist of deterministic models for the Tx/Rx-target paths at least for the LOS paths. Additionally, more complex components will be derived stochastically, such as the background channel and modeling of environmental objects.

Another aspect that is not yet resolved is calibration for spatial consistency. The majority of companies responding to the [Post-119-ISAC-01] email discussion [6] are supportive to calibrate spatial consistency. Details of the parameters for spatial consistency are FFS and should be further discussed.

Proposal 2: In addition to large scale calibration and full calibration that includes fast fading, RAN1 should consider calibration for spatial consistency as part of the ISAC channel modelling study. Details of the parameters for spatial consistency are FFS and should be further discussed.

## Calibration assumptions/parameters

Calibration assumptions should consider only a subset of parameters necessary to align implementations of the ISAC channel model, including target channel, background channel, RCS, etc., but not every parameter and permutation that may be necessary for future ISAC solution evaluations.

Given the current scope of the ISAC CM SID [2], RAN1 should consider all 5 sensing target types identified in the SID and associated calibration parameters should be represented in the new ISAC channel model calibration clauses in TR 38.901. Although there is limited TUs remaining for the ISAC CM study, no explicit prioritization for the different sensing modes/scenarios is required. This can be implied based on company contributions.

Proposal 3: RAN1 considers all 5 sensing target types identified in the SID should be represented in new TR 38.901 ISAC channel model calibration clause(s). With limited TUs remaining for the ISAC CM study, no explicit prioritization for the different sensing modes/scenarios is required but can be implied based on company contributions.

On specific ISAC CM calibration assumptions, parameters, and values, the company inputs captured in the xls in R1-2500059 Email discussion summary on ISAC CM calibration assumptions [6] can be used as a starting point to harmonize the calibration assumptions. One item that should be addressed at a high level is that for each of the sensing target xls tabs, there should be alignment for a given sensing target and scenario, or at least this aspect should be treated consistently across sensing target calibration parameters. For example, there is more than one tab for the automotive sensing target, for  a given scenario (highway, urban), yet other sensing targets have one tab with several scenarios included within the same tab. This should be made consistent across the different sensing targets.

Proposal 4: For ISAC CM calibration assumptions, the company inputs captured in R1-2500059 Email discussion summary on ISAC CM calibration assumptions [6] should be used as a starting point to harmonize the calibration assumptions. The calibration parameters for each sensing target and a given scenario should be consistently reflected in the tabular summary (tabs).

# Conclusion

In this contribution, we discussed the CM calibrations and deployment scenarios for developing the ISAC channel model. We made the following observations and proposals:

Observation 1: A hybrid approach to ISAC channel modelling calibrations could allow for a balance of realistic but not an overly complex means of ISAC CM simulations.

Proposal 1: RAN1 considers a hybrid approach to CM calibration whereby simulations would consist of deterministic models for the Tx/Rx-target paths at least for the LOS paths. Additionally, more complex components will be derived stochastically, such as the background channel and modeling of environmental objects.

Proposal 2: In addition to large scale calibration and full calibration that includes fast fading, RAN1 should consider calibration for spatial consistency as part of the ISAC channel modelling study. Details of the parameters for spatial consistency are FFS and should be further discussed.

Proposal 3: RAN1 considers all 5 sensing target types identified in the SID should be represented in new TR 38.901 ISAC channel model calibration clause(s). With limited TUs remaining for the ISAC CM study, no explicit prioritization for the different sensing modes/scenarios is required but can be implied based on company contributions.

Proposal 4: For ISAC CM calibration assumptions, the company inputs captured in R1-2500059 Email discussion summary on ISAC CM calibration assumptions [6] should be used as a starting point to harmonize the calibration assumptions. The calibration parameters for each sensing target and a given scenario should be consistently reflected in the tabular summary (tabs).

# References

[1] RP-234069 New SID: Study on channel modelling for Integrated Sensing and Communication (ISAC) for NR

[2] RP-242348 Revised SID: Study on channel modelling for Integrated Sensing and Communication (ISAC) for NR

[3] 3GPP TR22.837 Feasibility Study on Integrated Sensing and Communication, Rel. 19

[4] 3GPP TS 22.137 Service Requirements for Integrated Sensing and Communication, Stage 1, Rel. 19

[5] R1-2410846	Session notes for 9.7 (Study on channel modelling for Integrated Sensing And Communication for NR), Ad-Hoc Chair (Huawei). RAN1#119.

[6] R1-2500059 Email discussion summary on ISAC CM calibration assumptions.