3GPP TSG RAN WG1 #120	R1-2501134

Athens, Greece, February 17th – 21st, 2025

Source:	Sharp

Title:	Discussion on adaptation of common signal/channel transmissions

Agenda Item:	9.5.3

Document for:	Discussion and Decision

Introduction

In RAN1#119, following agreements are made on adaptation of common signal/channel transmissions [1]:

Agreement Reply to Q1(What is the relation in terms of time location before and after SSB adaptation?): - RAN1 agreed that at least SSB burst periodicity is adapted. - There are no RAN1 agreements to adapt the time location of the SSB burst other than the periodicity but RAN1 is still discussing other options. Agreement Reply to Q2(What is the relation in terms of frequency location before and after SSB adaptation?): - The frequency location is same before and after SSB adaptation. Agreement Reply to Q3(What is the spatial relation before and after SSB adaptation?): - There is no change to spatial relation (in terms of QCL assumption) for the same SSB index before and after SSB adaptation. - At least the case where there is no change to actually transmitted SSBs within a burst before and after SSB adaptation is supported. Further update to be made based on RAN1#119 progress if any. Agreement At least msg1-FrequencyStart can be configured separately for the additional PRACH resources at least for 4-step RACH. Agreement For DCI-based adaptation for additional PRACH resources, select only from the following alternatives: - Alt 1: (PRACH resource configuration level) DCI-based adaptation to indicate whether the additional PRACH resources provided by semi-static signalling are available or not - FFS: details - Alt 2: (subset of PRACH resource level) DCI-based adaptation to indicate whether a subset of the additional PRACH resources provided by semi-static signalling are available or not - FFS: Maximum number of subsets of the additional PRACH resources= [2 or 3 or 4 or 16] - FFS: whether the subset of the additional PRACH resources is in - Alt 2-1: RO level per SSB - Alt 2-2: SSB-to-RO mapping cycle level - Alt 2-3: PRACH association period level - Alt 2-4: PRACH association pattern period level - Alt 2-5: SFN level - Alt 2-6: Network configured time period Conclusion There is no RAN1 consensus to support SSB adaptation in time domain for Rel-19 NES-capable UE’s PCell (connected mode) Working Assumption For DCI-based adaptation for additional PRACH resources, - Select from the following options for carrying the adaptation indication in DCI format 1_0 with P-RNTI - Option 1: Use reserved bits in the DCI format - FFS: relation (if any) to TRS availability bits / short message indicator in the DCI format - Option 2: Use Bits 5-8 within the Short Message (from upper layers) - Note: Availability should be confirmed by checking with RAN2. - Option 3: Use bits available for both Option 1 and Option 2 FFS: Payload size for adaptation for additional PRACH resources Agreement For DCI-based adaptation for additional PRACH resources, select only from the following alternatives: - Alt 1: DCI-based adaptation to indicate whether the additional PRACH resources provided by semi-static signalling are available or not - [DCI payload size = 1 bit] - FFS: A single PRACH mask provided by semi-static signalling is used to identify the subset of the additional PRACH resources - FFS: details Agreement Confirm the following working assumption from RAN1#118bis. Working Assumption For DCI-based adaptation for additional PRACH resources, at least DCI format 1_0 can carry the adaptation indication for UEs in idle/inactive and connected mode. - P-RNTI is used

In this paper, we share the view on adaptation of common signal/channel transmissions for network energy saving.

Discussions

  - Adaptation of SSB in time domain

Applicable SSB type for adaptation of SSB

In previous RAN1 meetings, one main discussion point for SSB adaptation is the clarification of applicable SSB for the adaptation, and following has been agreed [1],[2]:

Agreement (RAN1#118) For adaptation mechanism(s) of SSB in time-domain, - For Rel-19 NES-capable UE’s PCell (Connected mode), adaptation of CD-SSB on sync raster is not supported - FFS: Adaptation for SSB that is not CD-SSB is supported (A2) - FFS: Adaptation for SSB not on sync raster is supported (A3) - For Rel-19 NES-capable UE’s SCell - Adaptation of SSB configured for the SCell is supported for the following cases - FFS: Adaptation for CD-SSB (B1) including UE impact compared to legacy operation where the SSB is configured with periodicity>20msec for SCell - Adaptation for SSB that is not CD-SSB on sync raster (B2’) - Adaptation for SSB that is not CD-SSB not on sync raster (B3’) Conclusion (RAN1#118bis) There is no consensus on the support of adaptation of SSB for idle mode UEs in Rel-19 Conclusion (RAN1#119) There is no RAN1 consensus to support SSB adaptation in time domain for Rel-19 NES-capable UE’s PCell (connected mode)

For Rel-19 NES-capable UE’s SCell

For this issue, the first point that we should consider is to avoid the impact on idle UEs by adapting CD-SSB. This point can be realized by cell barring using MIB or SIB1 in the current specification. Then, pure SCell (SCell only) can be realized by the barring method. For the pure SCell case, either CD-SSB or NCD-SSB can be used in the current specification, and it is an implementation matter whether NW uses CD-SSB or NCD-SSB. Therefore, regulation about adapting CD-SSB as specification is not necessary.

Observation 1: SSB for pure SCell (SCell only) can be realized by MIB/SIB1 barring.

Observation 2: Whether a SSB on Pure SCell (SCell only) is CD-SSB or NCD-SSB is up to gNB (NW) implementation.

Furthermore, NW can also adapt CD-SSB when there is no UE using a cell as PCell, allowing it to be used as PCell by one UE, according to the agreement for PCell (not to support adaptation of CD-SSB on sync raster for Rel-19 NES capable UE’s PCell (connected mode) in the RAN1#119).

As described above, there are some scenarios where CD-SSB can be used SCell without an impact on idle UEs and PCell implementation, and in such a case, the adaptation of CD-SSB can provide NES gain.

Proposal 1: For Rel-19 NES capable UE’s SCell (Connected mode), adaptation for CD-SSB is supported.

SSB configuration for SSB adaptation

In RAN1#118, FL has made a following proposal on SSB configuration for SSB adaptation,

Proposal 2.1.2a-rev1 For adaptation of SSB in time-domain, support SSB adaptation based on two SSB configurations where at least one configuration is active - At least SSB periodicity can be same or different between the SSB configurations - "ssb-PositionsInBurst" and "ss-PBCH-BlockPower" are same between the SSB configurations - FFS: whether "ssb-PositionsInBurst" and "ss-PBCH-BlockPower" can be different between the SSB configurations is supported - FFS: other settings between the SSB configurations - FFS: whether more than one SSB configuration can be active

In the previous meeting, it is almost consensus to use two SSB configurations where at least one configuration is active, and these two SSB configurations include at least SSB periodicity parameter. Then the remaining issues are whether the parameters other than SSB periodicity (e.g. ssb-PositionsInBurst and ss-PBCH-BlockPower) can be included in the configurations and whether more than one SSB configuration can be active.

From our view, for NES gain by “time domain adaptation”, the critical part is how to increase the opportunity of deeper sleep and for that, it is important to reduce frequent periodic SSB transmission. Therefore, compared with SSB periodicity, other parameters such as ssb-PositionsInBurst have quite less contribution on that perspective (as shown as Fig. 1).

Observation 3: Compared to SSB periodicity adaptation, the adaptation of the number/position of SSB (i.e. ssb-PositionsInBurst) has less contribution on NES gain in time domain (i.e. sleeping opportunity).

Figure 1: ranges for adaptation of “ssb-PositionsInBurst” and SSB burst periodicity

In addition, the adaptation of SSB transmission power (i.e. ss-PBCH-BlockPower) is not time-domain adaptation but is power-domain adaptation, which means out of the WI scope.

Observation 4: It is out of scope to consider to adapt SSB transmission power, since it is not adaptation in time domain but adaptation in power domain.

Considering the observation above, we prefer to focus on only SSB periodicity adaptation. When focusing on only SSB periodicity adaptation, there is no necessity to support more than one SSB configuration activation since SSB transmission of shorter periodicity always include SSB transmission of longer periodicity (as shown as Fig. 2).

Observation 5: If only SSB periodicity adaptation is supported, there is no necessity to support the case that more than one SSB configuration are active.

Figure 2: example of activation for two configurations

Proposal 2: For adaptation of SSB in time-domain, support SSB adaptation based on two SSB configurations where only one configuration is active

- Only SSB periodicity is different between the SSB configurations

- “ssb-PositionsInBurst" and "ss-PBCH-BlockPower" are same between the SSB configurations

  - Adaptation of PRACH in time domain

Time domain configuration for Additional PRACH resources

For the support of PRACH adaptation, one of the main remaining issues is how to configure the additional PRACH resources in time domain.

Agreement (from RAN1#118bis)

For adaptation of PRACH in time-domain, for determining the additional PRACH resources in time-domain,

- For Alt 1 (same PRACH configuration index for additional and legacy PRACH resources), select one or more of the following additional mechanism(s),

  - Opt 1-2a: up to N1 additional value(s) of (x, y)

    - FFS: value of N1(>=1)

  - Opt 1-2b: up to N2 additional value(s) of y

    - FFS: value of N2 (>=1)

  - Opt 1-3: Muting/masking ROs

  - Opt 1-4:

    - up to N4_1 additional timing offset(s) at frame-level

    - up to N4_2 additional timing offset(s) at slot-level

    - FFS: values of N4_1(>=1) and N4_2(>=1)

  - Opt 1-5: No additional mechanism is selected.

  - Note: x and y refer to the parameters from the random-access configuration tables (from 38.211)

- For Alt 2 (different PRACH configuration index for additional and legacy PRACH resources), select one or more of the following additional mechanism(s),

  - Opt 1-2a: up to N1 additional value(s) of (x, y)

    - FFS: value of N1(>=1)

  - Opt 1-2b: up to N2 additional value(s) of y

    - FFS: value of N2 (>=1)

  - Opt 1-3: Muting/masking ROs

  - Opt 1-4:

    - up to N4_1 additional timing offset(s) at frame-level

    - up to N4_2 additional timing offset(s) at slot-level

    - FFS: values of N4_1(>=1) and N4_2(>=1)

  - Opt 1-5: No additional mechanism is selected.

  - Note: x and y refer to the parameters from the random-access configuration tables (from 38.211)

From our view, there are two aspects to determine the applicable options:

Aspect 1: Collision issue between additional PRACH resources and PRACH resource for legacy UEs

Aspect 2: Scheduling flexibility of the additional PRACH resources for NES gain

Aspect 1: Collision between additional PRACH resources and PRACH resource for legacy UEs

In RAN1#118, SSB-to-RO mapping rule for previously supported 3 non-full overlapping cases has been discussed and it has been agreed that all cases follow legacy SSB-to-RO mapping rule. As a result, no new SSB-to-RO mapping rule will be introduced at least for non-full overlapping case. As next step, whether full overlapping case is supported or not needs to be clarified.

If the SSB-to-RO mapping for the additional PRACH resources follows legacy SSB-to-RO mapping rule and if a part of the valid ROs in additional PRACH resources full-overlaps (i.e. overlap both in frequency and time domain) with the valid ROs in legacy PRACH resources, the collision of different SSB indices for same RO may happen (as shown in Fig.3).

Figure 3: example of SSB-to-RO mapping collision issue

As same as non-full overlapping case, to avoid introducing new SSB-to-RO mapping rule, we prefer not to allow such full overlapping case of valid ROs. For that, following two approaches can be considered:

- Resource configurations for additional PRACH resources which include full/partial overlapping ROs in both time and frequency domains with PRACH resources are not allowed.

- Resource configurations for additional PRACH resources which include full/partial overlapping ROs in both time and frequency domains with PRACH resources are allowed, but the overlapping ROs are treated as invalid ROs for the additional PRACH resources.

To solve this issue, in RAN1#119, the introduction of following option 1-6B was discussed.

Proposal 4.1.5a-rev3 For adaptation of PRACH in time-domain, for determining the additional PRACH resources in time-domain, - For Alt 2 (different PRACH configuration index for additional and legacy PRACH resources), select Opt 1-6B: - Opt 1-6B: When an additional RO is overlapped with legacy RO in both time and frequency domain, the additional RO is invalid before the SSB-RO mapping - Note: the overlapped RO for legacy resource is not impacted. - This does not preclude the FFS on PRACH mask from the following agreement from RAN1#119). The single PRACH mask (if supported) is applied after the SSB-RO mapping. Agreement (from RAN1#119) For DCI-based adaptation for additional PRACH resources, select only from the following alternatives: - Alt 1: DCI-based adaptation to indicate whether the additional PRACH resources provided by semi-static signalling are available or not - [DCI payload size = 1 bit] - FFS: A single PRACH mask provided by semi-static signalling is used to identify the subset of the additional PRACH resources - FFS: details

Considering the impact to existing PRACH configuration, this Option 1-6B seems to be the simplest solution with less impact to solve the collision issue. Therefore, irrespective of the introduction of other options, we prefer to support this invalidation rule.

Proposal 3: RAN1 supports Option 1-6B: When an additional RO is overlapped with legacy RO in both time and frequency domain, the additional RO is invalid before the SSB-RO mapping

    - Note: the overlapped RO for legacy resource is not impacted.

Aspect 2: Scheduling flexibility of the additional PRACH resources for NES gain

Although the motivation of Opt 1-2a/1-2b/1-3/1-4 includes solving collision issue discussed in above, these options consider improving scheduling flexibility for the additional PRACH resources. We think the main use case of PRACH adaptation is NES-capable UE heavy cell (considering additional PRACH resources are only for such NES-capable UEs), and in such case, condensed PRACH resources is preferable to obtain NES gain with sleeping opportunity. Although all of these options can increase the scheduling flexibility, Option 1-2a/1-2b and 1-4 has concern that these options are effectively equivalent to introduce new PRACH configurations allocating ROs in the resource not used in existing PRACH configurations. To increase the scheduling flexibility with this concern, we assume masking approach is the good way to achieve condensed RO allocation.

Existing PRACH configurations introduced in Rel-15 are designed such as large amount of ROs with shorter periodicity and small amount of ROs with longer periodicity. On the other hand, considering NES gain by deeper sleeping opportunity, we prefer to have RO allocation with a large amount of condensed ROs with longer periodicity. To achieve this RO allocation with existing PRACH configurations, it is reasonable to use ROs in a part of multiple association periods by the masking in a unit of one or more of association periods. For example, following masking table in Table 1 can be considered. By adapting the available ROs by the mask index, NW can perform flexible RO allocation with existing PRACH configurations to set enough sleeping opportunity as NW desires.

Table 1: Example of association period mask index

| Association period mask index | Association periods which is available in additional PRACH resources |
| --- | --- |
| 0 | The first half of N association periods |
| 1 | The second half of N association periods |
| 2 | The first quarter of N association periods |
| 3 | The second quarter of N association periods |
| 4 | The third quarter of N association periods |
| 5 | The fourth quarter of N association periods |
| 6 | reserved |
| 7 | reserved |

Figure 4: example of additional PRACH resources with association mask index

Proposal 4: For additional PRACH resources, a single PRACH mask indicating an available subset of association periods is applied after the SSB-RO mapping.

Conclusion

In this contribution, we have the following observations and proposals:

Observation 1: SSB for pure SCell (SCell only) can be realized by MIB/SIB1 barring.

Observation 2: Whether a SSB on Pure SCell (SCell only) is CD-SSB or NCD-SSB is up to gNB (NW) implementation.

Proposal 1: For Rel-19 NES capable UE’s SCell (Connected mode), adaptation for CD-SSB is supported.

Observation 3: Compared to SSB periodicity adaptation, the adaptation of the number/position of SSB (i.e. ssb-PositionsInBurst) has less contribution on NES gain in time domain (i.e. sleeping opportunity).

Observation 4: It is out of scope to consider to adapt SSB transmission power, since it is not adaptation in time domain but adaptation in power domain.

Observation 5: If only SSB periodicity adaptation is supported, there is no necessity to support the case that more than one SSB configuration are active.

Proposal 2: For adaptation of SSB in time-domain, support SSB adaptation based on two SSB configurations where only one configuration is active

- Only SSB periodicity is different between the SSB configurations

- “ssb-PositionsInBurst" and "ss-PBCH-BlockPower" are same between the SSB configurations

Proposal 3: RAN1 supports Option 1-6B: When an additional RO is overlapped with legacy RO in both time and frequency domain, the additional RO is invalid before the SSB-RO mapping

    - Note: the overlapped RO for legacy resource is not impacted.

Proposal 4: For additional PRACH resources, a single PRACH mask indicating an available subset of association periods is applied after the SSB-RO mapping.

References

“RAN1 Chairman’s Notes,” RAN1 #119, November 2024

R1-2410888, “Final summary of AI 9.5.3 for R19 NES,” RAN1 #119, November 2024