| 3GPP TSG-RAN WG1 Meeting #120 Athens, Greece, February 17th-21st, 2025 |  | R1-2501169 |
| --- | --- | --- |

Agenda item:	9.8.2

Source: 	Qualcomm Incorporated

Title: 	Channel model adaptation or extension of TR38.901 for 7-24GHz

Document for:	Discussion/Decision

# Introduction

In this document we discuss two potential enhancements identified in RP-234018 that could be relevant to FR3 channel modeling when large antenna arrays are deployed in commercial networks.

In Section 2 we discuss near-field propagation and spherical wavefront modeling followed by a discussion on spatial non-stationarity in Section 3.

# Near-field propagation and spherical wavefront modeling

This section discusses our views on near-field propagation and spherical wavefront modelling.

## Clarifying SID Terminology

SID uses the term ‘near-field propagation’. However, in EM theory, ‘near field’ is used in close proximity to an antenna (of the order of a few wavelengths) where non-radiative behavior dominates radiative behavior.

Our understanding is that the intent was to study channel modeling in the far field but to more closely examine whether planar wavefronts hold good or whether spherical wavefront modeling would be required, especially as antenna array sizes get larger. We interpret the goal of the study to compare spherical wavefront modeling (SWM) with planar wavefront modeling (PWM) and assess whether spherical wavefront modeling is required for FR3 bands. In the rest of the document, any mention of near field modelling is to be interpreted as reference to spherical wavefront modeling. The two terms are used interchangeably.

Figure 1 Illustration of planar wavefront propagation and spherical wavefront propagation.

## Components of Spherical Wavefront Modelling

As agreed in the R1-118bis meeting, the following figures illustrate the impact a spherical wavefront could have on the equations used to compute the NLOS and LOS channel in 38.901.

Figure 2 Components of the LOS channel model that can be impacted by considerations of a spherical wavefront.

Figure 3 Components of the NLOS channel model that can be impacted by considerations of a spherical wavefront

The impact on the phase (red) is the most common aspect of SWM and is well studied. Planar wavefront assumes a linear shift in phase as a wave traverses across an antenna array. This fundamental assumption is altered once spherical wavefronts are assumed, where the individual locations of each antenna element pair begin to influence the phase/delay components. In addition to this, impact on angles of incidence and departure, doppler, etc, were also discussed.

In the following we present a short analysis on the impact to the delay and its relation to the impact on phase.

### On the impact of spherical wavefronts on delay

In the previous meeting the following agreement was made regarding the delay component for near field modelling.

Agreement For near-field channel, in principle, one of following options is considered to calculate the antenna element-wise delay of direct path between TRP and UE: - Option-1: The following formula is used to calculate the antenna element-wise delay of direct path: τ1,u,s=τ1-d3D-¦ru,s¦c where ru,s refers to the vector determined by the location of receive antenna element u and transmit antenna element s, d3D refers to the 3D distance between reference antenna at TRP and UE side. - The above formula is applied with the same principle as existing model in Clause 7.6.2 of TR 38.901, and the channel response of LOS ray between Rx antenna u and Tx antenna s at delay τ at time t is given by: Hu,s,1LOSt;τ=Frx,u,θθLOS,ZOA,u,s,ϕLOS,AOA,u,sFrx,u,ϕθLOS,ZOA,u,s,ϕLOS,AOA,u,sT100-1Ftx,s,θθLOS,ZOD,u,s,ϕLOS,AOD,u,sFtx,s,ϕθLOS,ZOD,u,s,ϕLOS,AOD,u,s exp-j2πd3Dλfexp-j2πru,s-d3Dλfexpj2πrrx,LOST∙vλ0tδτ-τ1,u,s - Option-2: The antenna element-wise delay of direct path is directly modeled based on the distance of direct path between antenna elements at TRP and UE side. - Following formula is adopted to model the antenna element-wise delay in the channel coefficient as: δτ-τ1,u,s, τ1,u,s=τ1-d3D-¦ru,s¦c where ru,s refers to the vector determined by the location of receive antenna element u and transmit antenna element s, d3D refers to the 3D distance between reference antenna at TRP and UE side. - Option-3: Confirm the following working assumption made in RAN1#118BIS: - “The existing model in Clause 7.6.2 of TR 38.901 is used to model the delay.” - Note: τ1 is same as that is defined in 7.5-30 in TR38.901

Note that in the previous meetings, we have already agreed to model the phase component for near-field modelling using the following expression: exp-j2πd3Dλoexp-j2πru,s-d3Dλo. This component takes the differences in the path lengths to different antenna pairs into account and reflects the results phase difference. In light of this change, if we are to separately change the delay component to reflect the differences in the path lengths to different antenna pairs, then the effect on phase gets accounted for twice over and would be technically incorrect. We suggest that no further changes be made to the delay component. If required, aspects of Clause 7.6.2 can be applied to reflect not the antenna pair specific delays, but the ray-specific delays. We summarize this in the following proposal.

Proposal 1: For near-field modelling, since the phase component is already modified to reflect distances for each antenna pair, no changes are required for modelling the delay component. If required, aspects of Clause 7.6.2 can be applied to reflect not the antenna-pair specific delays, but the ray-specific delays.

## SWM for Direct and Indirect Paths

On SWM for direct and indirect paths

In the previous meeting there was a distinction made between how direct and indirect paths are handled when considering spherical wavefronts.

While it is straightforward to model the impact of spherical wavefronts on direct paths, indirect paths present a significant challenge. The primary difficulty arises because understanding the nature of an indirect path’s wavefront incident on a receiver may depend on the location of the cluster associated with that indirect path and its impact on the radius of curvature associated with the wavefront.

Certain modelling approaches treat the clusters as a point source and assume the signal originates from the location of the cluster. It is however unclear if such an approach is well justified given that a cluster is usually a physical reflector or scatterer, and it cannot be assumed to behave like a point source. An alternate approach treats all indirect paths as being the result of specular reflections and assumes the incoming wavefront to originate from a location that corresponds to the absolute delay associated with that path. Such an approach would mirror the spatial consistency approach outlined in Section 7.6.3.2 of 38.901. The two approaches described above are illustrated in Figure 8.

| Treating clusters as point sources for near-field modeling | Treating clusters as specular reflectors for near-field modelling |
| --- | --- |

Figure 8 Two approaches to near field modelling: (a) treating cluster as point sources and (b) treat clusters as specular reflectors.

Recognizing that both the highlighted approaches may come into play depending on the nature of interaction a path has with the environment, the following agreement was made in the previous meeting to allow some paths to be modelled as specular paths and a few others to be modelled as non-specular paths.

Agreement For near-field channel, for the non-direct paths, the distance d1 between the BS and the 1st point associated with cluster is generated with one of following options that to be selected in RAN1#120: - Option-1: The distance d1 is generated directly following a specific distribution. - For the k1 clusters, d1 = speed of light times the absolute delay of the cluster - For other k2 clusters, the distance d1 is equal or less than the speed of light times the absolute delay of the cluster and generated following a specific distribution - FFS: The specific distribution, e.g., log-normal distribution for UMi - FFS: The values of k1 and k2 - The generation of absolute delay of the cluster is according to the procedure defined in Section 7.6.9 in TR 38.901 - FFS: The value for UMi, UMa, InH-Office. - Option-2: The distance d1 is generated by a scaling factor multiplied by the absolute distance corresponding to the absolute delay of the cluster, where the scaling factor is generated from a specific distribution. - For the k1 clusters, d1 = speed of light times the absolute delay of the cluster (i.e., the scaling factor is 1) - For other k2 clusters, the distance d1 is equal or less than the speed of light times the absolute delay of the cluster and the scaling factor generated following a specific distribution (i.e., the scaling factor is equal or less than 1) - FFS: Distribution of scaling factor, e.g., Beta distribution for UMa - FFS: The values of k1 and k2 - The generation of absolute delay of the cluster is according to the procedure defined in Section 7.6.9 in TR 38.901 - FFS: The value for UMi, UMa, InH-Office. Note 1: If d2 is agreed to be considered, the generation of d2 can follow the same principle with the generation of d1.

Before we present more details on the specifics of SWM for direct and indirect paths, it is important to highlight that SWM can be undertaken at different levels of granularity, i.e., it can be applied just to the direct path or it can be extended to some or all the indirect paths as well. The desired granularity is hard to assess and may best be left as an open parameter.

Further it is not clear whether indirect paths that scatter off rough surfaces will exhibit a wavefront that is spherical in nature. It is possible that depending on how the wavefront incident on a scatterer interacts with it, the gNB antennas may see a composite wavefront that is associated with different surfaces associated with the scatterer and thereby not perceived as a clear spherical wavefront across the entire panel.

Recognizing these issues, we suggest that SWM be applied only to a subset of the indirect paths. Of this subset, k1 paths are assumed to be due to specular reflections and the k2 paths are assumed to be due to non-specular reflections.

Proposal 2: Spherical wavefront modelling for indirect paths is applied only to a subset of K indirect paths out of a total of Ktotal paths. The exact choice of K is up to the requirements of the study requiring spherical wavefront modelling. Note: K=0 is not precluded as it can be useful when spherical wavefront modelling is only required for the direct path in certain scenarios or environments. Further, k1 + k2  = K≤ Ktotal.

In the following subsections, we further investigate the near fielding modelling approaches for indirect and direct paths that were discussed earlier. The focus is on numerical details, complexity, convergence, and finally on the overall impact to system level performance.

### SWM for the Direct Path

Figure 9 Direct path between UE and gNB.

Figure 9 illustrates the direct propagation path between the transmitter and receiver given a reference centre point in the transmit and receiver arrays. As can be seen from Figure 8, for each transmit antenna element s and receive antenna element u, there is a direct propagation path between the transmitter and receiver represented by ru,s whereas rref  represents the propagation path between the transmitter and receiver reference points.

The phase difference, which arises due to the varying positions of the transmit and receive antenna elements, can be modelled using expj2πλ0d3D-ru,s where d3D = |rref|. Similarly, to model the difference in angular domain parameters in both azimuth and elevation domains for each antenna element pair in the field patterns in the direction of the spherical basis vectors θ and ϕ can be calculated as follows Frx,u,θθLoS, ZOAu,s,ϕLoS, AOAu,sFrx,u,ϕθLoS,ZOAu,s,ϕLoS AOAu,s and Ftx,s,θθLoS, ZODu,s,ϕLoS AODsFtx,s,ϕθLoS, ZODu,s,ϕ,LoS AODu,s. Finally, the receive antenna element dependent doppler shift can be calculated as expj2πrrx,LoS,u,sT∙vλ0t. These modifications are mainly added to include the impact of angular domain parameters, phase and doppler. The modified channel model incorporating SWM over the existing stochastic model in TR 38.901 looks as follows:

Hu,s,1LOSt=Frx,u,θθLoS, ZOAu,s,ϕLoS, AOAu,sFrx,u,ϕθLoS,ZOAu,s,ϕLoS AOAu,sT100-1  Ftx,s,θθLoS, ZODu,s,ϕLoS AODsFtx,s,ϕθLoS, ZODu,s,ϕ,LoS AODu,sexp-j2πd3Dλ0expj2πλ0d3D-ru,sexpj2πrrx,LoS,u,sT∙vλ0t

### SWM for Indirect Paths

In the RAN1 119 agreement, there are two methods for generating cluster distances for indirect paths. Option-2 relies on determining the wavefront curvatures along the propagation path to derive a stochastic distribution of relative curvature of a ray between 0 and 1. These relative curvatures are then combined with the cluster path lengths (calculated as the speed of light multiplied by the absolute delay of the cluster) to determine the cluster distances. Option-1, on the other hand, generates the stochastic distribution based on the point associated with the cluster, which is the location of the first or last interaction in the propagation. This method directly generates the cluster distances, capping them at the cluster path lengths.

Theoretically, the distribution used to stochastically generate cluster distances should not solely depend on the first or last interaction point. Instead, the wavefront curvature-based method (Option-2) provides a more robust justification for using wavefront curvature at the gNB side as the distance between the BS/UE and a point associated with the cluster.

Proposal 3: The distribution used to stochastically generate the distances (d1 and d2) of a point associated with a cluster should not merely reflect the physical location of clusters. RAN1 to prioritize wavefront curvature-based approach to generate the distances (d1 and d2) of a point associated with a cluster.

### SWM for Indirect Path Using Stochastic Generation of the Point Associated with a Cluster

Given the structure of existing stochastic model TR 38.901, incorporating SWM using stochastic generation of the point associated with a cluster will require modifications to Step 5, Step 7, and Step 11 as highlighted in Figure 10.

Figure 10 Required modifications to incorporate spherical wavefront modelling using Stochastic Cluster Distances Generation under the structure of existing 38.901 model.

Step 5: Use the absolute cluster delays for indirect paths to stochastically generate the distances to the point(s) associated with a cluster.

Step 7: Calculate the observed angle of arrival/departure at each antenna element from the point(s) associated with each cluster.

Last point associated with a cluster

First point associated with a cluster

Step 11: Generate channel coefficients based on the parameters generated in the step 5 and step 7.

Figure 11 Indirect path between UE and gNB using stochastic generation of distances to point(s) associated with a cluster.

Figure 11 illustrates the indirect propagation path between the transmitter and receiver given a reference centre point in the transmit and receive arrays. As can be seen from the Figure 11, for each transmit antenna element s, there is a propagation path to a first point associated with a cluster represented by rtx,n,m,s for each cluster n and ray m. Similarly, for each receive antenna element u, there is a propagation path from a last point associated with a cluster represented by rrx,n,m,u for each cluster n and ray m. Additionally, there are paths from the reference point locations of transmit antenna array to a first and last point associated with a cluster represented by rtx,n,m  and rrx,n,m, respectively.

The phase difference, arising due to the varying positions of the transmit and receive antenna elements can be modelled using expj2πλ0rtx,n,m-rrx,n,m,s and expj2πλ0rrx,n,m-rrx,n,m,u, respectively. To model the difference in antenna gains due to differences in angular domain parameters in both azimuth and elevation domains, the antenna gains for each transmit/receive antenna element in the direction of the spherical basis vectors θ and ϕ can be calculated as follows Frx,u,θθn,m, ZOAu,ϕn,m, AOAuFrx,u,ϕθn,m, ZOAu,ϕn,m, AOAu and Ftx,s,θθn,m, ZODs,ϕn,m, AODsFtx,s,ϕθn,m, ZODs,ϕn,m, AODs. Finally, the receive antenna element dependent doppler shift can be calculated as expj2πrrx,n,m,uT∙vλ0t. The modified channel model for indirect paths incorporating SWM over the existing stochastic model in TR 38.901 looks as follows:

Hu,s,n,mNLOSt=PnMFrx,u,θθn,m, ZOAu,ϕn,m, AOAuFrx,u,ϕθn,m, ZOAu,ϕn,m, AOAuTexpjΦn,mθθκn,m-1expjΦn,mθϕκn,m-1expjΦn,mϕθexpjΦn,mϕϕ  Ftx,s,θθn,m, ZODs,ϕn,m, AODsFtx,s,ϕθn,m, ZODs,ϕn,m, AODs

expj2πλ0rrx,n,m-rrx,n,m,uexpj2πλ0rtx,n,m-rrx,n,m,sexpj2πrrx,n,m,uT∙vλ0t

#### Observations on convergence between PWM and SWM Using Option-2 (Stochastic Cluster Distance Generation)

This section uses SWM framework obtained using Option-2 with k1=2 clusters and cluster relative curvatures drawn from beta distribution with parameters α=3.81,β=1.62 to better understand how channels generated using SWM and PWM differ from each other. We do not consider doppler differences citing the concerns noted in Section 2.2.2. We conduct simulation analysis to understand the convergence of SWM and PWM channel models as a UE moves farther away from the gNB. Such an analysis sheds light on the set of UEs for which SWM is likely make a difference. For this purpose, we conduct simulation analysis by dropping UEs at a certain distance to compare the NMSE and SGCS between the legacy TR 38.901 stochastic model based on PWM approximation and the SWM framework. The formulas used for calculating NMSE and SGCS are given as follows:

NMSE (dB)= 10 log10vec(HSWM)-vec(HPWM)2vec(HPWM)2

SGCS = vec(HSWM) . vec(HPWM)HSWM2HPWM22

The simulation parameters are listed in the following table:

Table 1 Simulation parameters for comparing SWM Using TBCM and PWM.

| Layout | Urban Macro |
| --- | --- |
| Random UE Drops | 300 UEs dropped |
| gNb Antenna Downtilt | 94o |
| Antenna Spacing | dh = 0.5λ, dv = 0.8λ for gNb dh = 0.5λ, dv = 0.5λ for UE |
| Center Frequency | 13 GHz |
| gNb Antenna Elements Configuration | 4096 (64x32x2) |
| gNB Number of TxRUs | 256 |
| UE Number of TX/RX Antennas | 4 |
| UE Distribution | 20% UEs are outdoor and 80% UEs are indoor |
| k1 | 2 (strongest clusters) |

From the NMSE and SGCS plots for both LoS and NLoS UEs shown in Figure 12, we generally observe a downward trend in NMSE and an upward trend in SGCS between SWM and PWM channels as the distance between the UE and gNB increases, indicating some convergence of SWM to PWM beyond Rayleigh distances. However, some UEs, even those far from the gNB, exhibit significant differences between SWM and PWM channels. This behavior is counterintuitive since spherical wavefront properties are expected to converge to planar wavefront behavior at distances beyond the Rayleigh distance. We believe this issue is caused by the cluster distance being too small for high-power clusters, even when UEs are far away from the gNB.

UEs far away from gNB exhibiting spherical wavefront properties

Figure 12 NMSE and SGCS as the function of distance for comparing SWM obtained using Option-2 and PWM for both LoS and NLoS UEs in UMa layout.

Observation 1: The stochastic generation of point(s) associated with a cluster can lead to scenarios where strong clusters are associated with a point close to the gNB or the UE. This proximity magnifies the effects of the spherical wavefront, leading to a significant divergence between SWM and PWN channels even when UEs are far away from a gNB.

Proposal 4: RAN1 to further investigate whether channels associated with UEs far away from gNB exhibiting spherical wavefront properties is justified or whether any further mitigation measures are required.

#### Observations on System-Level Simulations Between SWM and PWM Models

In this section, we conduct system-level simulation analysis to understand the impact of SWM modeling using stochastic generation of cluster distances (Option-2) on communication metrics. The simulation parameters for system-level simulations are listed in the following table:

Table 2 Simulation parameters for System-Level Simulations for SWM (obtained using Option-2) for a UMa layout.

| System BW | 100 MHz |
| --- | --- |
| Layout | 1-cell Urban Macro – 1 UE/cell |
| Random UE Drops | 300 UEs dropped |
| gNb Tx Power | 49 dBm |
| gNb Antenna Downtilt | 94o |
| Antenna Spacing | dh = 0.5λ, dv = 0.8λ for gNb dh = 0.5λ, dv = 0.5λ for UE |
| Center Frequency | 13 GHz |
| gNb Antenna Elements Configuration | 4096 (64x32x2) |
| gNB Number of TxRUs | 256 |
| UE Number of TX/RX Antennas | 4 |
| UE Max Tx Power | 29 dBm |
| UE Distribution | 20% UEs are outdoor and 80% UEs are indoor |
| Penetration loss for Indoor UEs | 50% High-loss, 50% Low-loss |
| UE Speed | 3km/h for indoor and outdoor UEs |
| Traffic Models | Full-buffer |
| Scheduler | SU-MIMO |
| Slot Structure - DL | TDD: DDSU |
| Duration | 1 second |

From the results plotted in Figure 13 for both LoS and NLoS UEs, we observe no difference in mean CDF curves of mean UE throughput, UE rank, UE MCS, UE codebook SINR and coupling loss for SWM obtained using stochastic generation of cluster (Option-2) approach. However, the percentage differences in individual UE throughput between SWM and PWM for a few UEs can be as large as 23%, due to the differences in PWM and SWM channels caused by the stochastic generation of cluster. These system-level evaluations highlight an important observation: despite using stochastic generation of cluster, the impact on overall CDF curves of system-level communication metrics remains unnoticeable.

Figure 13 Communication metrics comparison for SWM obtained using stochastic generation of cluster and PWM for UEs in UMa scenario.

The preceding results and discussion lead us to make the following observation:

Observation 2: Using system-level evaluations, it is observed that the difference between PWM and SWM (using Option-2) is negligible for both LoS and NLoS UEs in UMa when comparing the CDF curves for UE throughput, UE rank and UE MCS.

# Spatial non-stationarity

This section discusses our views on spatial non-stationarity.

## Background

In FR3 bands, with the decrease in wavelengths, it is feasible to deploy large antenna arrays and realize a Massive-MIMO or a Giga-MIMO gNB.  Operating these large arrays may begin to sufficiently differ from current massive-MIMO panels that it may be important to categorize them under a new operating regime with more attention paid to the new challenges and opportunities brought about by them.

Once array dimensions exceed a certain threshold, typical assumptions on spatial wide-sense stationary properties of the channel model come into question. These large arrays could be impacted by partial radio exposure of the transmitter aperture i.e., not all antenna elements may be visible to each of the clusters and/or UEs due to LoS blockage. Relative size of scatterers or reflector and their location in reference to the gNB antenna array also impacts visibility via reflected paths.

Figure 31 Illustration of spatial non-stationarity of wireless channels when considering a large antenna array.

## Modelling Framework for Spatial Non-Stationarity on the gNB Side

In the previous RAN1 meetings, we have taken an either-or approach to selecting between blockage-based models and visibility region-based models for SnS. While blockage models are good to model partial blockage, they are unable to model scenarios where partial scattering or reflections occur. Since both approaches seem to have their merits, and to ensure sufficient support for all modes of SnS, it may make sense for RAN1 to pursue both options: a purely stochastic approach to modelling SNS on the gNB side along with a physical blockage-based approach that requires dropping blockers and determining their impact on the channel. The stochastic approach may be better suited to model partial scattering/reflection without there being a need to determine cluster locations or to drop physical objects.

A combination of both blockage models and visibility regions may be required to adequately model spatial non-stationarity depending on the specific scenario and environmental factors. While the direct path may be impacted by a blockage, indirect paths may be impacted by the limited dimensions of the underlying reflector or scatterer. Assuming both effects are to be sufficiently captured, we suggest the following:

Proposal 5: Spatial non-stationarity may arise due to incomplete scattering (depending on the dimensions of a reflectors/scatterer) or due to blockage (a physical entity obstructing the view of a portion of the array). To model spatial non-stationarity at the gNB side support a physical blockage approach and a stochastic approach.

- If spatial non-stationarity arises due to blockage, then reuse the existing blockage framework in 38.901 (with potential modifications) to model SnS.

- If spatial non-stationarity arises due to incomplete scattering and/or blockage, a stochastic approach based on visibility regions can be used to model SnS.

We also propose that these concepts be applied on a per-cluster basis rather than per ray. Physically, it seems to make sense that partial visibility may be associated with rays bouncing off a reflector or a scatter. Since reflectors/scatterers are likely to constitute unique clusters in the channel model, it is reasonable to apply the concept of visibility region on a per cluster basis. This may also help distinguish SnS effects from that of multi-path fading that impacts a tap’s magnitude across gNB antenna elements.

Proposal 6: If spatial non-stationarity is to be modeled, the concept of visibility region or blockage region is applied on a per cluster basis.

Going further, if visibility regions are used to model SnS, there are three components that were identified in the previous meeting:

Agreement For the modelling of spatial non-stationarity at BS side, if stochastic based approach, at least for unified visible probability (VP) and visibility region (VR), is adopted, the following steps are considered: - Step 1: Determine whether a cluster is impacted by SNS - Step 2: Generate the visibility region - Step 3: Calculate the power attenuation factor FFS: the details of each step Note: Merge of some of the steps is not precluded

In addition to the three steps outlined in the above agreement, it is also important to consider the probability that a UE at any given location is impacted by SnS on the gNB side. It is likely that only a fraction of the UEs in a cell are impacted by SnS and it may suffice to model the effects of SnS for this set of UEs. This is especially important to determine whether the LOS path between the UE and gNB is impacted by a partial blockage or not. We clarify this in the following:

Proposal 7: Not all UEs are impacted by SnS at the gNB side. If spatial non-stationarity is to be modeled using visibility regions at the gNB side, consider the probability of spatial non-stationarity impacting a UE at any given location.

The agreement is also not very clear if the direct path is to be modelled using the stochastic approach. It needs to be clarified that for a complete modelling of SnS on the gNB side, partial blockage of the direct path between a UE and a gNB is also considered. We clarify this in the following:

Proposal 8: If spatial non-stationarity is to be modeled using a stochastic approach, partial blockage of the direct path between a UE and gNB is also modelled.

We elaborate further on the three steps outlined in the above agreement in the following section.

### On Visibility Regions

While blockage models are focused on modelling the blockage leading to spatial non-stationarity, visibility regions sidestep the modelling details of a finite-dimensional reflector or scatterer and instead directly focus on the impact it has on the received signal at a large antenna array. The focus is on how the impact of a finite-dimensional scatterer is perceived via partial visibility. The following agreement was made to lay the basic foundations of this approach:

Agreement For the modelling of spatial non-stationarity, if visible probability or visibility region is adopted, - Visible probability or visibility region is modelled per cluster - FFS Ratio of UEs and clusters that have SNS impact - Rectangle can be considered as starting point for shape of VR with following alternatives to define the size: - Alt 1: VR size is defined as number of elements generated by a distribution - FFS distribution - Alt 2: VR size is derived based on distance between antenna array of BS and UE/cluster - Note: Cluster location is required in this alternative Alt 3: VR size is randomly generated with a minimum size limit

Building on this agreement, we propose to define visibility regions as rectangular portions of an array that are seen by a cluster. A minimum size on these visibility regions imposes a natural restriction on the panel sizes that experience spatial non-stationarity. Visibility regions can be drawn randomly for each cluster impacted by partial visibility, or alternately, visibility regions can be mapped to sub-arrays of the larger antenna panel --- this approach is likely to be more favourable from a complexity perspective for a system-level simulator.

Proposal 9: If spatial non-stationarity is to be modeled on the gNB side, visibility regions are assumed to be rectangular in shape with same orientation as the antenna array. The size of visibility regions is not dependent on exact cluster location but can be dependent on cluster power. The placement/location of the visibility region on the panel is determined in a stochastic manner.

Similar to blockage, once the visibility regions are defined, we need to focus on defining appropriate attenuation models for each of the visibility regions with consideration for how these models are defined at the edges of a visibility region with particular attention to how the transitions across the edges of a visibility region are handled. Further, for portions of the array that fall outside the visibility region, the paths or rays associated with the impacted cluster can either be modelled as non-existent or alternately be significantly attenuated. These aspects are captured in the following proposal:

Proposal 10: If spatial non-stationarity is to be modeled on the gNB side using visibility regions, for portions of the array that fall outside the visibility region of a cluster, the cluster power decreases linearly in dB domain from the edge of the visibility region with a certain roll off factor.

## Modelling Framework for Spatial Non-Stationarity on the UE side.

In the previous meetings the following agreements were made regarding modelling SnS on the UE side.

Working Assumption For the modelling of spatial non-stationarity at UE side mainly due to close proximity, if blocker-based approach is adopted, the Option-3 is supported with following details: - Three typical cases are introduced: one hand grip, dual hand grip, head and one hand grip. - FFS: probability of each case in the simulation. - For each case, to model the attenuation for each of antenna in the candidate antenna location defined in the UE antenna model for handheld device with one of following alternatives: - Alt 1: Introduce a fixed value for each candidate antenna position per case. - Alt 2: Introduce a distribution for candidate antenna position. - FFS: distribution per antenna position(s) and per case. Agreement For the modelling of spatial non-stationarity at BS side, if physical blocker-based approach is adopted, for blockage model B, the procedure to determine the number of blockers and locations are same as existing blockage model B. For the modelling of spatial non-stationarity at UE side, if blocker-based approach is adopted, an attenuation per antenna element is introduced, the following options can be considered with down-selection by RAN1#120: - Option-1: generated by leveraging the existing blockage Model-B with potential updates. - At most 2 hand type blockers and one head type blocker for a specific UE is assumed. - Option-2: generated by leveraging the existing blockage Model-A with potential updates - Option-3: generated by a distribution or a fixed value due to the hand grip and/or head proximity - FFS: the details, e.g., how to determine the value of loss for the case as one hand grip, a dual hand grip and/or head proximity

There are two issues at stake here. The first relates to whether the effects of UE-side SnS due to hand/body blockage are captured as a new blockage model or not. The second is focused on how exactly to model the effects of hand/body blockage. On the first aspect, we suggest that UE-side SnS be captured separately from the existing blockage models. Even though there is some precedence in modelling blockage on the UE side in Section 7.6.4.1 of 38.901, a different approach may be adopted to model SnS and it may be best to capture this in an independent section.

On the second aspect, each of the three UE states, UE-side SnS can be simply modelled as an attenuation factor that impacts a subset of antennas. The attenuation factor is assumed to be generated randomly from given range of values for each impacted antenna. In addition to these attenuation factors being state-specific, they may also be frequency-range specific with potentially a different range for FR1, FR2, and the 7-24 GHz frequencies. The losses may also depend on whether discrete or module-based antenna architectures are assumed. The subset of antennas impacted in any given UE state can be identified by reusing the UE antenna model for handheld devices that was agreed in the previous meeting. Finally, the model needs to probabilistically determine the UE impacted by UE-side SnS and assign individual probabilities for each of the three states. We summarize this framework in the following proposal.

Proposal 11: For the modelling of spatial non-stationarity at UE side, introduce a new blockage model that generates a loss due to three possible hand and/or head grips based on a certain probability for each state and a certain distribution of losses associated with each state. Additional considerations are as follows:

- Reuse the UE antenna model for handheld devices to identify the subsets of antenna impacted in different UE states.

- Attenuation factors for the impacted antennas are chosen from a range of values at random.

- Determine attenuation factors/tables for each state and each frequency range, with consideration to module-based and discrete antenna architectures.

# Conclusion

We make the following observations and proposals in this document:

On spherical wavefront modeling:

Proposal 1: For near-field modelling, since the phase component is already modified to reflect distances for each antenna pair, no changes are required for modelling the delay component. If required, aspects of Clause 7.6.2 can be applied to reflect not the antenna-pair specific delays, but the ray-specific delays.

Proposal 2: Spherical wavefront modelling for indirect paths is applied only to a subset of K indirect paths out of a total of Ktotal paths. The exact choice of K is up to the requirements of the study requiring spherical wavefront modelling. Note: K=0 is not precluded as it can be useful when spherical wavefront modelling is only required for the direct path in certain scenarios or environments. Further, k1 + k2  = K≤ Ktotal.

Proposal 3: The distribution used to stochastically generate the distances (d1 and d2) of a point associated with a cluster should not merely reflect the physical location of clusters. RAN1 to prioritize wavefront curvature-based approach to generate the distances (d1 and d2) of a point associated with a cluster.

Observation 1: The stochastic generation of point(s) associated with a cluster can lead to scenarios where strong clusters are associated with a point close to the gNB or the UE. This proximity magnifies the effects of the spherical wavefront, leading to a significant divergence between SWM and PWN channels even when UEs are far away from a gNB.

Proposal 4: RAN1 to further investigate whether channels associated with UEs far away from gNB exhibiting spherical wavefront properties is justified or whether any further mitigation measures are required.

Observation 2: Using system-level evaluations, it is observed that the difference between PWM and SWM (using Option-2) is negligible for both LoS and NLoS UEs in UMa when comparing the CDF curves for UE throughput, UE rank and UE MCS.

On spatial non-stationarity

Proposal 5: Spatial non-stationarity may arise due to incomplete scattering (depending on the dimensions of a reflectors/scatterer) or due to blockage (a physical entity obstructing the view of a portion of the array). To model spatial non-stationarity at the gNB side support a physical blockage approach and a stochastic approach.

- If spatial non-stationarity arises due to blockage, then reuse the existing blockage framework in 38.901 (with potential modifications) to model SnS.

- If spatial non-stationarity arises due to incomplete scattering and/or blockage, a stochastic approach based on visibility regions can be used to model SnS.

Proposal 6: If spatial non-stationarity is to be modeled, the concept of visibility region or blockage region is applied on a per cluster basis.

Proposal 7: Not all UEs are impacted by SnS at the gNB side. If spatial non-stationarity is to be modeled using visibility regions at the gNB side, consider the probability of spatial non-stationarity impacting a UE at any given location.

Proposal 8: If spatial non-stationarity is to be modeled using a stochastic approach, partial blockage of the direct path between a UE and gNB is also modelled.

Proposal 9: If spatial non-stationarity is to be modeled on the gNB side, visibility regions are assumed to be rectangular in shape with same orientation as the antenna array. The size of visibility regions is not dependent on exact cluster location but can be dependent on cluster power. The placement/location of the visibility region on the panel is determined in a stochastic manner.

Proposal 10: If spatial non-stationarity is to be modeled on the gNB side using visibility regions, for portions of the array that fall outside the visibility region of a cluster, the cluster power decreases linearly in dB domain from the edge of the visibility region with a certain roll off factor.

Proposal 11: For the modelling of spatial non-stationarity at UE side, introduce a new blockage model that generates a loss due to three possible hand and/or head grips based on a certain probability for each state and a certain distribution of losses associated with each state. Additional considerations are as follows:

- Reuse the UE antenna model for handheld devices to identify the subsets of antenna impacted in different UE states.

- Attenuation factors for the impacted antennas are chosen from a range of values at random.

- Determine attenuation factors/tables for each state and each frequency range, with consideration to module-based and discrete antenna architectures.

# References

- Jaeckel S, Raschkowski L, Börner K, Thiele L. QuaDRiGa: A 3-D multi-cell channel model with time evolution for enabling virtual field trials. IEEE transactions on antennas and propagation. 2014 Mar 6;62(6):3242-56. URL: https://quadriga-channel-model.de/

- RP-234018, New SID: Study on channel modelling enhancements for 7-24GHz for NR, 3GPP TSG RAN Meeting #102,  Edinburgh, UK, 11th – 15th December, 2023.

- Frode Bohagen, Pal Orten, Geir E. Oien, “On spherical vs. plane wave modeling of line-of-sight MIMO channels”, IEEE Transactions on Communications, Volume: 57, Issue: 3, pp 841 – 849, March 2009.

- L. Liu et al., “The Cost 2100 MIMO Channel Model,” IEEE Wireless Commun., vol. 19, no. 6, pp. 92–99, Dec. 2012.

- Jose Flordelis, Xuhong Li, Ove Edfors, Fredrik Tufvesson, “Massive MIMO Extensions to the COST 2100 Channel Model: Modeling and Validation”, IEEE Transactions on Wireless Communications, Volume: 19, Issue: 1, pp. 380 – 394, January 2020.

- E. D. Carvalho, A. Ali, A. Amiri, M. Angjelichinoski and R. W. Heath, "Non-Stationarities in Extra-Large-Scale Massive MIMO," in IEEE Wireless Communications, vol. 27, no. 4, pp. 74-80, August 2020.

- C. F. López, C. -X. Wang and Y. Zheng, "A 3D Non-Stationary Wideband Massive MIMO Channel Model Based on Ray-Level Evolution," in IEEE Transactions on Communications, vol. 70, no. 1, pp. 621-634, Jan. 2022

- S. Wu, C. X. Wang, H. Haas, E. H. M. Aggoune, M. M. Alwakeel, and B. Ai, “A non-stationary wideband channel model for massive MIMO communication systems,” IEEE Trans. Wireless Commun., vol. 14, no. 3, pp. 1434–1446, Mar. 2015,

- C. -X. Wang, Z. Lv, X. Gao, X. You, Y. Hao and H. Haas, "Pervasive Wireless Channel Modeling Theory and Applications to 6G GBSMs for All Frequency Bands and All Scenarios," in IEEE Transactions on Vehicular Technology, vol. 71, no. 9, pp. 9159-9173, Sept. 2022, doi: 10.1109/TVT.2022.3179695.

- S. Payami and F. Tufvesson, "Channel measurements and analysis for very large array systems at 2.6 GHz," 2012 6th European Conference on Antennas and Propagation (EUCAP), Prague, Czech Republic, 2012, pp. 433-437

- X. Gao, F. Tufvesson and O. Edfors, "Massive MIMO channels — Measurements and models," 2013 Asilomar Conference on Signals, Systems and Computers, Pacific Grove, CA, USA, 2013, pp. 280-284

- R. Feng, C. -X. Wang, J. Huang, Y. Zheng, F. Lai and W. Zhou, "Mutual Coupling Analysis of 6G Ultra-Massive MIMO Channel Measurements and Models," ICC 2022 - IEEE International Conference on Communications, Seoul, Korea, Republic of, 2022, pp. 956-961.

- Y. Zheng, C. -X. Wang, J. Huang, R. Feng and J. Thompson, "Measurements and Characteristics Analysis of 6G Ultra-Massive MIMO Wireless Channels with Different Antenna Configurations and Scenarios," in IEEE Transactions on Vehicular Technology, vol. 72, no. 8, pp. 9720-9732, Aug. 2023

- J. Chen, X. Yin, X. Cai, and S. Wang, “Measurement-based massive MIMO channel modeling for outdoor LoS and NLoS environments,” IEEE Access, vol. 5, pp. 2126–2140, 2017.

- À. O. Martínez, E. De Carvalho and J. Ø. Nielsen, "Towards very large aperture massive MIMO: A measurement-based study," 2014 IEEE Globecom Workshops (GC Wkshps), Austin, TX, USA, 2014, pp. 281-286.

- J. Huang, C.-X. Wang, R. Feng, J. Sun, W. Zhang, and Y. Yang, “Multi-frequency mmWave massive MIMO channel measurements and characterization for 5G wireless communication systems,” IEEE J. Sel. Areas Commun., vol. 35, no. 7, pp. 1591–1605, Jul. 2017.

- J. Li et al., “Cluster-based 3-D channel modeling for massive MIMO in subway station environment,” IEEE Access, vol. 6, pp. 6257–6272, 2018.

- J. Li, B. Ai, R. He, M. Yang, Z. Zhong, and Y. Hao, “A cluster-based channel model for massive MIMO communications in indoor hotspot scenarios,” IEEE Trans. Wireless Commun., vol. 18, no. 8, pp. 3856–3870, Aug. 2019.

- R1-2406140, “Discussion on Channel model adaptation/extension of TR38.901 for 7-24GHz”, 3GPP TSG RAN WG1 #118, Maastricht, Netherlands, August, 2024.

- R1-2410492, “Discussion on Channel model adaptation/extension of TR38.901 for 7-24GHz”, 3GPP TSG RAN WG1 #119, Orlando, USA, November, 2024.

# Annex

## Agreements Made in RAN1-119 Meeting

Including near-field propagation and spatial non-stationarity Agreement For near-field channel, the following formula is adopted to model the angular domain parameters of direct path between TRP and UE as antenna element-wise channel parameter: Frx,u,θθLOS,ZOA,u,s,ϕLOS,AOA,u,sFrx,u,ϕθLOS,ZOA,u,s,ϕLOS,AOA,u,sT100-1Ftx,s,θθLOS,ZOD,u,s,ϕLOS,AOD,u,sFtx,s,ϕθLOS,ZOD,u,s,ϕLOS,AOD,u,s Where θLOS,ZOA,u,s, ϕLOS,AOA,u,s, θLOS,ZOD,u,s, ϕLOS,AOD,u,s are the respective antenna element-wise elevation arrival angles, azimuth arrival angles, elevation departure angles and azimuth departure angles of LoS path between the transmit antenna element s and receive antenna element u. Working Assumption - For the modelling of spatial non-stationarity at BS side, if physical blocker-based approach is adopted, the nearest K blockers from BS are selected. - For the modelling of spatial non-stationarity at BS side, if physical blocker-based approach is adopted, at least for blockage model B, the following parameters to define the building edge are introduced in the Table 7.6.4.2-5 in TR 38.901: Typical set of blockers; Blocker dimensions; Mobility pattern. Outdoor; Building edge; Cartesian: w=50 m; h=20 m; Stationary. - In the blockage model B, the following equation is used to calculate the attenuation caused by the building edge: - LdB=20log100.5-F, where F represents one of Fh1, Fh2, Fv1, and Fv2 defined in equation 7.6-29 in section 7.6.4.2 of TR 38.901. Agreement For the modelling of spatial non-stationarity at BS side, if stochastic based approach, at least for unified visible probability (VP) and visibility region (VR), is adopted, the following steps are considered: - Step 1: Determine whether a cluster is impacted by SNS - Step 2: Generate the visibility region - Step 3: Calculate the power attenuation factor FFS: the details of each step Note: Merge of some of the steps is not precluded Working Assumption For the modelling of spatial non-stationarity at UE side mainly due to close proximity, if blocker-based approach is adopted, the Option-3 is supported with following details: - Three typical cases are introduced: one hand grip, dual hand grip, head and one hand grip. - FFS: probability of each case in the simulation. - For each case, to model the attenuation for each of antenna in the candidate antenna location defined in the UE antenna model for handheld device with one of following alternatives: - Alt 1: Introduce a fixed value for each candidate antenna position per case. - Alt 2: Introduce a distribution for candidate antenna position. - FFS: distribution per antenna position(s) and per case. Agreement For near-field channel, to generate the channel parameter, the reference point is defined as: - The physical center of the antenna array/center of the device. Conclusion For near-field channel, no changes are expected on following parameter for direct path between TRP and UE: - Doppler shift Conclusion For near-field channel, no changes are expected on following parameter for non-direct path between TRP and UE: - Doppler shift Agreement For near-field channel, the following formula is adopted to model the angular domain parameters of non-direct path between TRP and UE as antenna element-wise channel parameter: Frx,u,θθn,m,ZOA,u,ϕn,m,AOA,uFrx,u,ϕθn,m,ZOA,u,ϕn,m,AOA,uTexpjΦn,mθθkn,m-1expjΦn,mθϕkn,m-1expjΦn,mϕθexpjΦn,mϕϕFtx,s,θθn,m,ZOD,s,ϕn,m,AOD,sFtx,s,ϕθn,m,ZOD,s,ϕn,m,AOD,s where θn,m,ZOA,u,ϕn,m,AOA,u, θn,m,ZOD,s,ϕn,m,AOD,s are the ray-wise angular domain parameters of ray m cluster n between the transmit antenna element s and receive antenna element u. Agreement For the modelling of spatial non-stationarity at BS side, if physical blocker-based approach is adopted, the rotation and power variation calculation are conducted in ray level. Agreement For near-field channel, for the non-direct paths, the distance d1 between the BS and the 1st point associated with cluster is generated with one of following options that to be selected in RAN1#120: - Option-1: The distance d1 is generated directly following a specific distribution. - For the k1 clusters, d1 = speed of light times the absolute delay of the cluster - For other k2 clusters, the distance d1 is equal or less than the speed of light times the absolute delay of the cluster and generated following a specific distribution - FFS: The specific distribution, e.g., log-normal distribution for UMi - FFS: The values of k1 and k2 - The generation of absolute delay of the cluster is according to the procedure defined in Section 7.6.9 in TR 38.901 - FFS: The value for UMi, UMa, InH-Office. - Option-2: The distance d1 is generated by a scaling factor multiplied by the absolute distance corresponding to the absolute delay of the cluster, where the scaling factor is generated from a specific distribution. - For the k1 clusters, d1 = speed of light times the absolute delay of the cluster (i.e., the scaling factor is 1) - For other k2 clusters, the distance d1 is equal or less than the speed of light times the absolute delay of the cluster and the scaling factor generated following a specific distribution (i.e., the scaling factor is equal or less than 1) - FFS: Distribution of scaling factor, e.g., Beta distribution for UMa - FFS: The values of k1 and k2 - The generation of absolute delay of the cluster is according to the procedure defined in Section 7.6.9 in TR 38.901 - FFS: The value for UMi, UMa, InH-Office. Note 1: If d2 is agreed to be considered, the generation of d2 can follow the same principle with the generation of d1. Working Assumption For near-field channel, for the non-direct paths, the impact of spherical wavefront is optionally considered at UE side. Agreement For near-field channel, in principle, one of following options is considered to calculate the antenna element-wise delay of direct path between TRP and UE: - Option-1: The following formula is used to calculate the antenna element-wise delay of direct path: τ1,u,s=τ1-d3D-¦ru,s¦c where ru,s refers to the vector determined by the location of receive antenna element u and transmit antenna element s, d3D refers to the 3D distance between reference antenna at TRP and UE side. - The above formula is applied with the same principle as existing model in Clause 7.6.2 of TR 38.901, and the channel response of LOS ray between Rx antenna u and Tx antenna s at delay τ at time t is given by: Hu,s,1LOSt;τ=Frx,u,θθLOS,ZOA,u,s,ϕLOS,AOA,u,sFrx,u,ϕθLOS,ZOA,u,s,ϕLOS,AOA,u,sT100-1Ftx,s,θθLOS,ZOD,u,s,ϕLOS,AOD,u,sFtx,s,ϕθLOS,ZOD,u,s,ϕLOS,AOD,u,s exp-j2πd3Dλfexp-j2πru,s-d3Dλfexpj2πrrx,LOST∙vλ0tδτ-τ1,u,s - Option-2: The antenna element-wise delay of direct path is directly modeled based on the distance of direct path between antenna elements at TRP and UE side. - Following formula is adopted to model the antenna element-wise delay in the channel coefficient as: δτ-τ1,u,s, τ1,u,s=τ1-d3D-¦ru,s¦c where ru,s refers to the vector determined by the location of receive antenna element u and transmit antenna element s, d3D refers to the 3D distance between reference antenna at TRP and UE side. - Option-3: Confirm the following working assumption made in RAN1#118BIS: - “The existing model in Clause 7.6.2 of TR 38.901 is used to model the delay.” - Note: τ1 is same as that is defined in 7.5-30 in TR38.901