3GPP TSG RAN WG1 #120							         R1-2501203

Athens, Greece, February 17th – 21st, 2025

Source:	NTT DOCOMO, INC.

Title:	Discussion on coding and CRC aspects of physical channel design for Ambient IoT

Agenda Item:	9.4.2

Document for: 	Discussion and Decision

- Introduction

At the RAN#106 meeting, a new WID “Solutions for Ambient IoT (Internet of Things) in NR” was approved [1]. The objectives are as follows.

General Scope The definitions provided in TR 38.848, TR 38.769, and decisions, etc. made during the Rel-19 SI in RAN WGs are taken into this WI, and the following is the exclusive general scope: - The overall objective shall be to standardize the following Ambient IoT device: Device 1: ~1 µW peak power consumption, has energy storage, RF envelope detector receiver, initial sampling frequency offset (SFO) up to 10X ppm, neither R2D nor D2R amplification in the device. The device’s D2R transmission is backscattered on a carrier wave provided externally. - Deployment scenario 1 with Topology 1, according to D1T1-B. - FR1 licensed spectrum in FDD, with R2D in DL spectrum and D2R and CW in UL spectrum. - Spectrum deployment in-band to NR and standalone, with A-IoT BS located indoor. - Traffic types DO-DTT, DT, for rUC1 (indoor inventory) and rUC4 (indoor command). - Carrier wave transmission for waveform 1 only, without hopping, per the following cases in TR 38.769: - Case 1-4 for D1T1-B - Proximity determination via Solution 1 in TR 38.769 only. - Device (un)availability via Direction 1 in TR 38.769 only. WGs begin their discussions from the decisions already made in TR 38.769, with the following refinements for the scope: The following objectives are set, within the General Scope: - RAN1 scope: - PRDCH and PDRCH, which are the only physical channels in R2D and D2R, respectively. - R2D and D2R signal(s) - Multiplexing/multiple access in R2D is by only TDMA, and in D2R is by only TDMA and FDMA. - R2D supports only OOK-4 modulation, one solution for CP handling. D2R backscattering supports only OOK and BPSK modulations. - R2D transmission supports only the Manchester line code in TR 38.769 - D2R transmission supports: - Either the Manchester line code in TR 38.769 or no line code (one to be down-selected); and - A corresponding small frequency shift method according to the options in TR 38.769. - R2D does not support FEC. D2R supports only convolutional code with generator polynomials as per TS 36.212 (unless RAN1 decides to use other generator polynomials by RAN1#120bis). - PRDCH and PDRCH both support transmission without CRC, and with CRC as per the generator polynomials in TS 38.212 (unless RAN1 decides to use other generator polynomials by RAN1#120bis) for 6-bit CRC and 16-bit CRC. Cases to use which length of CRC, or no CRC, to be decided in RAN1. - D2R supports physical layer repetition transmission. R2D does not support physical-layer repetition transmission.

As captured in WID objective, PRDCH and PDRCH, which are the only physical channels in R2D and D2R are specified in Rel-19. In this contribution, we discuss the line coding, FEC, repetition and CRC aspects of these physical channels.

- Discussion

  - R2D (PRDCH) and D2R (PDRCH)

    - Line coding

According to the WID, Manchester coding is applied to PRDCH while either Manchester coding or no line coding is applied for PDRCH.

During the SI, for Manchester coding, the bit-to-chip mapping was assumed as; bit 0→chips{10}, bit 1→chips{01}. With this codeword, Manchester coding can provide the following characteristics according to the TR 38.769[2];

- The same codeword length for information bit 0 and 1, then scheduling time domain resources would not be different depending on the amount of 0 and 1

- Transitions during Manchester code transmissions can be utilized for recovering the signal’s clock

- The signal does not lose information or distort when fed to a DC blocker because each Manchester encoded symbol maintains a constant DC level, i.e. always the same average power

To preserve these benefits, the same codeword mapping should be supported for both R2D and D2R (if Manchester coding is supported).

Proposal 1: For Manchester encoding, support the bit-to-chip mapping defined in TR 38.769; bit 0→chips{10}, bit 1→chips{01}.

For PRDCH, some details may be further discussed subject to CP handling.

For PDRCH, as captured in WID objective, it should be further discussed whether Manchester coding or no line coding is applied. Different from PRDCH, FEC is applied to PDRCH and the performance of Manchester coding combined with FEC may need to be assessed and compared with the case where no line coding is applied. On the other hand, even without line coding, small frequency shift would be applied to D2R. Given that line coding discussion is highly related to the discussion of small frequency shift scheme, the characteristic should be discussed assuming small frequency shift together, which is discussed in section 2.2.1.

    - CRC

According to the WID objective, CRC is supported for both PRDCH and PDRCH while it may not be applied to specific case(s). Based on the SI, 6-bit CRC and 16-bit CRC are considered and their polynomials are in TS 38.212 in general unless RAN1 decides to use other generator polynomials by RAN1#120bis.

TBS for 6-bit and 16-bit CRC

As explained above, 6-bit CRC and 16-bit CRC are supported for PRDCH and PDRCH. During the SI, CRC overhead and false alarm rate/probability of undetected error with each 6-bit CRC and 16-bit CRC were studied for possible message sizes. While some exceptions such as no CRC attachment/16-bit CRC attachment for a specific message type regardless of TBS can be further discussed, general principle on determination of whether 6-bit CRC or 16-bit CRC is applied based on TBS can be specified. Based on the study on CRC overhead and FAR, it was observed that they are trade-off in general. Considering the tradeoff, i.e., to achieve moderate FAR with moderate CRC overhead, TBS of [24] bits can be the border for 6-bit CRC and 16-bit CRC.

Proposal 2: For CRC for PRDCH and PDRCH, 6bit CRC should be assumed for TBS<[24]bits, otherwise, 16bit CRC should be applied.

- Exception depending on message type, e.g., no CRC attachment or 6/16-bit CRC regardless of TBS, can be further discussed if any.

Fig.1: CRC overhead of 6-bit CRC and 16-bit CRC for each TBS

Polynomial

According to the current specification of NR, polynomials for 6-bit CRC and 16-bit CRC are specified as follows.

TS 38.212: Denote the input bits to the CRC computation by , and the parity bits by , where is the size of the input sequence and is the number of parity bits. The parity bits are generated by one of the following cyclic generator polynomials: - … - for a CRC length ; - … - for a CRC length .

Given that the above CRC in TS38.212 was not designed especially for low complexity device, further complexity reduction on CRC polynomial can be considered for A-IoT device.

Meanwhile the polynomial for 6-bit CRC can be reused as it is, for 16-bit CRC, polynomial incorporating that for CRC-6, such as D16+D11+D6+D5+1, has been studied which can provide equivalent performance with reducing device complexity. In that sense, we believe it is worth to consider a new polynomial for 16-bit CRC instead of reusing the polynomial for in TS38.212.

Proposal 3: For 6-bit CRC for PRDCH and PDRCH, reuse polynomial for CRC-6 in TS38.212.

Proposal 4: For 16-bit CRC for PRDCH and PDRCH, consider the following polynomial.

- gCRC16D=D16+D11+D6+D5+1

  - D2R (PDRCH)

    - Small frequency shift

During the SI, the necessity/purpose of small frequency shift was identified that it is necessary at least to avoid the CW interference. In addition, as we discussed in our companion paper[3], FDMA should be considered for D2R transmission of Msg1 for which contention-based access is assumed.

Proposal 5: For D2R, support small frequency shift at least for D2R transmission of Msg1.

During the SI, how to realize small frequency shift was studied for the case with Manchester coding and without line coding each.

TR38.769: For OOK and BPSK, small frequency shifts are studied: - For applying with Manchester line codes with a repetition number R ≥ 1: - Option 1: Each Manchester codeword is repeated by a codeword repetition number R, within the same time duration Tb corresponding to an information bit, where R = Tb/(2 × chip length), such that the amount of small frequency shift in Hz is R/Tb = 1/(2 × chip length). - Option 2: By multiplying the Manchester codeword with a square wave corresponding to the small frequency shift, the time duration Tb corresponding to each information bit includes R number of square wave periods, where R = Tb/(2 * chip length), such that the amount of small frequency shift in Hz is R/Tb = 1/(2 × chip length). The multiplication operation is performed as either an XOR or XNOR operation between a Manchester codeword corresponding to the information bit and the square wave for the small frequency shift. - … - If no D2R line code is used, by using a square-wave corresponding to the small frequency-shift, the time duration Tb corresponding to each information bit includes R number of square wave periods generated by 2R OOK chips [0, 1, 0, 1 …]/[1, 0, 1, 0 ..] or BPSK chips [-1, +1, -1, +1, …]/[+1, -1, +1, -1, …], such that the amount of small frequency shift in Hz is R/Tb.

Based on the above description, small frequency shift range in Hz is defined as R/ Tb, where R is repetition number of codeword or number of square wave period and time duration Tb corresponding to an information bit.

In our understanding, Tb can be the fixed value in time while R and chip length would be scaled with the target small frequency shift range, or vice versa, i.e., R or chip length is the fixed value and Tb would be scaled with the target small frequency shift range.

Proposal 6: For the small frequency shift for D2R, discuss how to determine the Tb, R and chip length.

- Opt.1: One fixed value is specified

- Opt.2: Explicitly indicated via R2D from set of values

- Opt.3: Implicitly indicated, i.e., calculated based on the other parameters.

- FFS: Detailed values for each of Tb, R and chip length.

Fig.2: Small frequency shift with and without line coding (R=2).

During the SI, based on the above definition of bandwidth was defined for each small frequency shift scheme. Based on the definition of bandwidth, it can be observed that small frequency shift by Manchester coding Opt.1/no line coding case has less bandwidth for main lobe which corresponds to lower effective noise power and larger coverage while small frequency shift with Manchester coding Opt.2 has lower sidelobe. Considering the difference on these spectrum characteristics, the performance on FDMA would be affected and it should be taken into account which of Manchester coding opt.1/2 or no line code should be applied.

Proposal 7: For small frequency shift scheme, one option should be down-selected considering its spectrum characteristics, performance of small frequency shift and performance combined with FEC.

    - FEC

Based on WID objective, FEC is applied to PDRCH and tail biting convolutional coding in TS36.212, i.e., constraint length 7 and code rate 1/3 should be supported at least.

Whether other constraint length and/or coding rate than that in TS36.212 should be supported can be further discussed.

TS36.212 A tail biting convolutional code with constraint length 7 and coding rate 1/3 is defined. The configuration of the convolutional encoder is presented in figure 5.1.3-1. The initial value of the shift register of the encoder shall be set to the values corresponding to the last 6 information bits in the input stream so that the initial and final states of the shift register are the same. Therefore, denoting the shift register of the encoder by, then the initial value of the shift register shall be set to Figure 5.1.3-1: Rate 1/3 tail biting convolutional encoder The encoder output streams, and correspond to the first, second and third parity streams, respectively as shown in Figure 5.1.3-1.

Constraint length

Smaller number of shift register than 7, e.g., 6 or 4, can reduce the complexity on device/reader while performance would be degraded. Considering the performance/spectrum efficiency degradation, we don’t see the necessity on further complexity reduction for shift register.

Larger number of shift register than 7, e.g., 8, can improve the performance while it would increase the complexity on device/reader exponentially.

Proposal 8: For FEC on PDRCH, other constraint length than 7 is not necessary.

Coding rate

Higher coding rate can improve spectrum efficiency. However, the higher code rate, e.g., 1/2, can be supported by puncturing based on the lower mother code rate, e.g., 1/3, of FEC as well. Lower coding rate can improve the performance while the target coverage can be achieved with 1/3 code rate in general and also coverage compensation can be supported by repetition.

Proposal 9: For FEC on PDRCH, other code rate than 1/3 is not necessary.

- Higher coding rate such as 1/2 can be supported by e.g., puncturing, if necessary.

    - Repetition

According to the WID objective, physical layer repetition is supported for D2R. During the SI, the following types of repetition were defined.

TR38.769: Block level: All the bits received from higher layers and/or physical layer (according to what is present) after CRC attachment (if used) are blockwise repeated Rblock times Bit level type 1: Each bit after CRC attachment (if used) is repeated Rbit times Bit level type 2: Each bit after both CRC attachment (if used) and FEC (if used) is repeated Rbit times Chip level: Each chip after line coding (if used) or after square wave modulation (if used) is repeated Rchip time. NOTE: This is equivalent to extending the duration of each chip by Rchip times.

As captured in TR, chip-level repetition is equivalent to longer chip length and should be discussed as a part of chip length discussion.

For bit-level repetition and block-level repetition, it was reported in TR38.769 [2] that block-level repetition outperforms bit-level repetition. In addition, in our understanding, bit-level repetition, especially type 2, is equivalent to small frequency shift by Manchester opt.1 or no line coding which is the repetition of Manchester coding codeword or repetition of square wave period. In that sense, if either of them is support, it is not necessary to consider bit-level repetition further. for block-level repetition, the details such as repetition factor, whether not only PDRCH but also D2R preamble should be repeated, whether to support repetition for D2R transmission of each message of Msg1, Msg3 and/or Step C2 etc.

Proposal 10: For D2R, at least block-level repetition should be supported.

- FFS: Repetition factor

- FFS: Which of Msg1, Msg3 and Step C2 (if present) supports repetition

- FFS: Whether preamble should be repeated

- Conclusion

In this contribution, we discussed modulation aspects on physical channel design for A-IoT device. Based on the discussion, we made following observations and proposals.

R2D and D2R

Proposal 1: For Manchester encoding, support the bit-to-chip mapping defined in TR 38.769; bit 0→chips{10}, bit 1→chips{01}.

Proposal 2: For CRC for PRDCH and PDRCH, 6bit CRC should be assumed for TBS<[24]bits, otherwise, 16bit CRC should be applied.

- Exception depending on message type, e.g., no CRC attachment or 6/16-bit CRC regardless of TBS, can be further discussed if any.

Proposal 3: For 6-bit CRC for PRDCH and PDRCH, reuse polynomial for CRC-6 in TS38.212.

Proposal 4: For 16-bit CRC for PRDCH and PDRCH, consider the following polynomial.

- gCRC16D=D16+D11+D6+D5+1

D2R

Proposal 5: For D2R, support small frequency shift at least for D2R transmission of Msg1.

Proposal 6: For the small frequency shift for D2R, discuss how to determine the Tb, R and chip length.

- Opt.1: One fixed value is specified

- Opt.2: Explicitly indicated via R2D from set of values

- Opt.3: Implicitly indicated, i.e., calculated based on the other parameters.

- FFS: Detailed values for each of Tb, R and chip length.

Proposal 7: For small frequency shift scheme, one option should be down-selected considering its spectrum characteristics, performance of small frequency shift and performance combined with FEC.

Proposal 8: For FEC on PDRCH, other constraint length than 7 is not necessary.

Proposal 9: For FEC on PDRCH, other code rate than 1/3 is not necessary.

- Higher coding rate such as 1/2 can be supported by e.g., puncturing, if necessary.

Proposal 10: For D2R, at least block-level repetition should be supported.

- FFS: Repetition factor

- FFS: Which of Msg1, Msg3 and Step C2 (if present) supports repetition

- FFS: Whether preamble should be repeated

Reference

[1] RP-243326, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, RAN#106, Dec. 2024.

[2] TR 38.769 V19.0.0, “Study on solutions for Ambient IoT (Internet of Things) in NR”, Dec. 2024.

[3] RI-2501205, Discussion on other aspects for Ambient IoT, NTT DOCOMO, INC.