3GPP TSG-RAN WG1 Meeting #120	Tdoc R1-2501245

Athens, Greece, February 17th – 21st, 2025

Agenda Item:	9.2.1

Source:	Ericsson

Title:	Remaining issues for UE-initiated beam management

Document for:	Discussion

# 1	Introduction

The scope of NR MIMO phase 5 was endorsed in RAN#102 meeting including the following objective:

- Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management

  - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching

  - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting

In previous RAN1 meetings, some agreements were made on UE-initiated/event-driven beam management. In this contribution, we discuss next level of details on UE-initiated or event-driven reporting.

# 2	Discussion

## 2.1	Event

In RAN1#117, the following agreement was made:

Agreement

Regarding the triggering event determination for Event 2:

- If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs.

  - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam

Above feature is subject to UE capability.

- Basic feature: Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs

FFS: Whether the above is captured in RAN1 or RAN2 specification.

The agreement is illustrated in Figure 1. The UE counts the number of event-2 instances in a window, and as soon as M event-2 instances occur within the window, the UE initiates the beam reporting, using the first available instance of the first UL channel.

Figure 1: Illustration of the agreement of event evaluation. The UE evaluates how many event-2 instances occur within a (sliding) window. Every event-2 instance is represented by “Ind” in the figure.

Up till now, events have been captured in RAN1 and RAN2 specifications. This includes beam failure detection, where L1 provides indications to MAC, which are then counted. Based on the counting, beam failure is then declared in MAC, and the associated actions are initiated in MAC. On the other hand, RAN1 specifications do not contain any counters: RAN1 specifications are stateless. We prefer to keep this separation, and propose:

The event evaluation for UE-initiated beam reporting is captured in a RAN2 specification.

The same proposal was made in the RAN2 contributions [2][3][4][5][6], which indicates that it is feasible to capture the event evaluation in the RAN2 specifications. Furthermore, RAN2 has made the following agreements for LTM:

MAC layer handles the event evaluation and measurement report triggering. (RAN2#127)

The entire event evaluation procedure is handled by MAC based on the latest L1 measured results reported by L1. (RAN2#127bis)

Hence, RAN2 will describe event evaluation for LTM in the MAC specification, and it would make a lot of sense to reuse that description also for UE-initiated beam management.

Note that the implementation in the UE is independent on where the triggering is captured, and that there is no performance impact on this decision. For example, the latency does not depend on the document where the functionality is captured.

### 2.1.1	Further details on Event-2

In RAN1#118bis, the following agreement was reached:

Agreement

Regarding RS measurement for the current beam for Event 2, for enabling one of either Scheme-1 or Scheme-2 by NW in Option-2a, the following implicit manner is supported:

- If the RS(s) for new beam are CSI-RS configured in a CSI-RS resource set configured with repetition, Scheme-1 is enabled; otherwise, Scheme-2 is enabled.

In legacy, we rely on the CSI-ResourceConfig to define measurement resources:

We propose to reuse the CSI-ResourceConfig also for UE-initiated beam management:

Reuse the RRC IE CSI-ResourceConfig to define the new beams for UE-initiated beam management.

This means that the parameter newBeamResourceSetEvent2-r19 in the RRC parameter list should be removed.

In RAN1#119, the following was agreed:

Agreement

Regarding for the evaluation periodicity for determining Event-2 instance [at least when DRX is not configured], at least Alt-1 is supported for the single-CC beam reporting (for case that the CSI report configuration and RS for the current beam and new beam(s) are in the same CC).

- Alt-1: The periodicity of the current beam RS should be the same as that of the new beam RS(s).

  - The evaluation periodicity is the same as the periodicity of the current and new beam RS(s).

- Alt-2: The periodicity of the current beam RS can be different from that of the new beam RS(s)

  - Alt-2_1: The evaluation periodicity is the same as the periodicity of the current beam RS.

  - Alt-2_2: The evaluation periodicity is the same as periodicity of the new beam RS.

  - Alt-2_3: The evaluation periodicity is the same as shortest periodicity of the current beam RS and new beam RS(s).

  - Alt-2_4: The evaluation periodicity is the maximum of {X ms, shortest periodicity of the current beam RS and new beam RS(s)}.

  - Alt-2_5: The evaluation periodicity is the same as largest periodicity of the current beam RS and new beam RS(s).

Note: There is the same periodicity for the new beam RS(s).

FFS: Down-selection among above Alt(s) for cross-CC beam reporting.

Strong concerns were expressed by Huawei and CATT that if only Alt-1 is supported in the end, the feature will not be practical.

Send an LS to RAN4. Final LS in R1-241XXXX.

In RAN1#119, RAN1 started to discuss NW configuration limitations, rather than discussing the real issue: what is the event evaluation periodicity. In our view, this was really a sidetrack. In the end, the reference signal configuration is up to the NW, and unless there are UE implementation issues related to a certain NW configuration, 3GPP should not introduce parameter restrictions. During the discussion, no such UE implementation issue was brought forward: instead RAN1 was debating if it was a possible or likely configuration. To iterate: it is not the responsibility of 3GPP to discuss if certain parameter settings are likely!

We also note that the event evaluation periodicity should be defined for all events. Strictly speaking, RAN1 has not agreed that new beams must be configured for event-1: since new beams are not absolutely necessary, Alt-1 in the agreement is not really operational.

To define a criterion that is independent of the new beams, the natural thing is to define the evaluation periodicity based on the current beam:

The event evaluation periodicity is identical to the periodicity of the current beam RS.

Note that RAN4 is discussing the measurement periodicity, i.e., how frequent the UE is required to perform measurements on RSs associated with current and new beams. The corresponding requirements for the legacy L1-RSRP measurements are provided in Table 9.5.4.1-1 and Table 9.5.4.1-2 in [7] for FR1 and FR2, respectively. For ease of reference, we replicate Table 9.5.4.1-2 below:

Table 9.5.4.1-2: Measurement period TL1-RSRP_Measurement_Period_SSB for FR2

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(1.5*M*P*N)*TDRX |
| Note: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. |  |

Table 1: Table 9.5.4.1-2 from [7].

In Table 1, M is related to if measurement restriction is configured, N is related to the Rx beam sweeping, and P is related to if the L1-RSRP measurement occasions can be shared with L3 measurements. In our understanding, the event evaluation periodicity does not have to be the same as the measurement periodicity: the UE may evaluate the event even when no new measurement of the current or any new beam has been received.

In RAN1#118bis, the following was agreed

Agreement

Regarding the triggering event determination for Event 2, the event instance(s) counting is per new beam. Further study candidate condition(s) of resetting the counting including whether resetting is needed.

When considering counter resets, it is useful to compare with other counter reset mechanisms in NR:

For RLM, the counter (N310, N311) and timer (T310) are reset at reconfiguration with sync, and at reconfiguration of the parameters related to RLM (the counters/timers and the reference signals

For BFR, the counter (BFI_COUNTER) is reset at RRC reconfiguration of any of the parameters used for BFR, and also when the reference signal for beam failure detection is changed.

It thus makes sense to reset counters at RRC reconfiguration of the associated parameters:

The event instance counting is reset for all new beams when the parameters associated with event-driven reporting are reconfigured.

Typically, the NW updates the indicated TCI state so that the measured RSRP on the RS associated with the indicated TCI state is higher than the RSRP measured on RSs associated with other TCI states. Hence, after a change in the indicated TCI state, event conditions related to the indicated TCI state would typically be false, and the counter would automatically be reset. However, RAN1 has already stated that the counting is per new beam. Then it makes sense to apply the same thinking to the current beam: the counting is applied per current beam, meaning that if the current beam changes, separate counting should be applied. In other words, once the indicated TCI state is updated, the event instance counting should be reset for events related to the current beam:

For event-1 and event-2, the event instance counting is reset for all new beams when the indicated TCI state is updated.

Once the event instance counter triggers the measurement reporting, i.e., when the UE informs the NW that an event occurred, it makes sense to also reset the counter:

The event instance counter is reset when it reaches its maximum value.

Note that Proposal 6 only applies for the new beam where the event instance counter reached its maximum value.

With Proposal 6, the risk of having multiple consecutive triggers will be reduced: the NW only needs to know that the event occurred once.

### 2.1.2	Event-1 and Event-7

In RAN1#119, the following agreements were made

Agreement

On UE-initiated/event-driven beam reporting, regarding trigger events, the following working assumption in RAN1#118bis is confirmed:

On UE-initiated/event-driven beam reporting, regarding trigger events, besides for Event-2, Event-1 and Event-7 are both supported.

- Event-1: Quality of the current beam is worse than a certain threshold.

- Event-7: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the Q-th best quality.

  - Q is RRC configured with subjective to UE capability signalling

    - UE may only indicate a single candidate value or not support Event-7.

- The additionally supported events will reuse the same design as event 2 – unless there is consensus to do otherwise

- The additionally supported events will be lower priority compared to event 2.

Agreement

On UE-initiated/event-driven beam reporting, for Event 1, down-select at least one among the following options for RS measurement

- Option-1: RS resource set for new beam is NOT configured in the CSI reporting configuration, and an explicit RRC selection for scheme-1 and scheme-2 is introduced.

- Option-2: RS resource set for new beam is configured in the CSI reporting configuration, and the following implicit manner for enabling one of either scheme-1 or scheme-2 is used:

  - If the RS(s) for new beam are CSI-RS configured in a CSI-RS resource set configured with repetition, Scheme-1 is enabled; otherwise, Scheme-2 is enabled.

In the agreement, there is a statement that event-1 and event-7 will reuse the same design as event-2 as far as possible. In our view, this is straightforward for most properties of event-2. In the following, we highlight how to do this. Since we aim to reuse as much as possible, we focus on how to make it work, rather than discussing if certain properties are useful or not.

In the second agreement above, two options were listed on how to select scheme-1 or scheme-2 for Event 1. First thing to notice is that also for Event-1, it is beneficial to have the UE reporting a set of new beams:

For event-1, it should be possible to configure the UE to report RSs.

The NW could use that report to trigger additional measurements, or even change the indicated TCI state. This is the paradigm used with SCell BFR, where the UE can also provide a measurement report of suitable candidates.

With Proposal 7, Option-2 in the third agreement above is the only possible solution. Having a unified solution for all three events is also a great advantage. Hence, we propose

For event-1, RS resource set for new beam is configured in the CSI reporting configuration, and the following implicit manner for enabling one of either scheme-1 or scheme-2 is used: if the RS(s) for new beam are CSI-RS configured in a CSI-RS resource set configured with repetition, Scheme-1 is enabled; otherwise, Scheme-2 is enabled.

For event-1 and event-7 we summarize our proposals in Table 2.

|  | Event-1 | Event-7 |
| --- | --- | --- |
| Quality measure | L1-RSRP | L1-RSRP |
| Report | N new beams Current beam (if configured) | N2 new beams Current beam+identifier (if configured) Additional contents FFS |
| Configuration of new beams | Explicitly configured in one RS resource set | Explicitly configured in one RS resource set. FFS: The reporting of RSs corresponding to activated TCI states |
| Report container | Mode A and mode B | Mode A and mode B |
| Window/counter | M instances within a window | M instances within a window |

Table 2: Details for event-1 and event-7.

For the report after event-7, it is relevant for the UE to report N2 new beams, with the corresponding configuration. We use N2, rather than N, for event-7, to highlight that up to L RSs should be reported, where L can be up to 8:

For event-7, it should be possible to configure the UE to report up to 8 RSs.

The idea with event-7 is to update the set of activated TCI states, the report should include information on what TCI states to activate, and what TCI states to deactivate:

The report for event-7 should include information on what TCI states to activate, and what TCI states to deactivate.

Note that including an identifier is necessary, since the NW does not know which TCI state corresponds to the activated TCI state with the Qth best quality.

## 2.2	Requesting UL resources for transmission of the report

One central issue when designing a UE-initiated beam report is how the resources for the transmission of the UL report are provided to the UE. In RAN1#117, the following was agreed

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Mode-A, the DCI format in Step-2 comprises UL-grant DCI format, and the second channel in Step-3 is at least PUSCH.

- The UL-grant DCI format at least comprises DCI format 0_1/0_2.

  - FFS: DCI format 0_3

- FFS: How to trigger the UEI beam report by the UL-grant DCI format

- FFS: the DCI format in Step-2 comprises DL-grant DCI format, and the second channel in Step-3 is PUCCH.

  - 1-bit field in the DL-grant DCI format is introduced to indicate the transmission of the UEI beam report

    - The PUCCH resource for HARQ-ACK transmission can be reused to carry both the HARQ-ACK and UEI beam report.

  - The DL-grant DCI format at least comprises DCI format 1_1/1_2.

    - FFS: DCI format 1_3

The agreement contains a number of FFSs. The first FFS discusses if UL-grant after step-1 can be DCI format 0_3. In our view, DCI format 0_3 should be supported:

The UL-grant DCI format comprises DCI format 0_1/0_2/0_3.

Note that if the UE is scheduled using DCI format 0_3, the beam report will be mapped to the cell with the smallest serving cell index among the scheduled cells indicated by scheduled cells indicator field or frequency domain resource assignment field, just as in legacy.

When a UE is scheduled using DCI format 0_3, the UCI is transmitted on one of the scheduled cells.

In RAN1#119, the following was agreed

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding first PUCCH channel configuration, Alt-2 is supported for both mode-A and mode-B: the first PUCCH channel is a new UCI type

- Alt-2 (new UCI type): Introduce RRC parameter, e.g., firstPUCCHResourceConfig- UEIBR, for the periodic PUCCH resource configuration.

  - It is RAN1’s understanding that the RRC parameter is NOT associated with SchedulingRequestId.

  - 1-bit to PUCCH resource is encoded by reusing the encoding mechanism of positive/negative SR.

  - The dedicated RRC parameter at least comprises the following:

    - periodicityAndOffset

    - PUCCH-ResourceID

- Above applies at least for the single CC case.

- Reuse multiplexing/dropping rule(s) of SR as baseline

  - FFS: overlapping with SR/LRR or PUSCH

Note: Further details on first PUCCH retransmission (if supported) for mode A and mode B will be continue to be separately discussed in RAN1.

The first channel is used to indicate to the NW that an event has occurred, both for mode A and mode B. In the following, we will use the term BEI (beam event indicator) for the new UCI.

For mode A, by transmitting BEI the UE informs the NW that an event has occurred and requests resources to transmit the corresponding report. To ensure that the NW receives the BEI, the UE should retransmit the BEI until it receives the grant:

The UE retransmits the BEI until it receives a DCI format that indicates a resource for a second UL channel.

To control the retransmissions, we propose to introduce the same functionality as for the scheduling request:

The retransmissions for the BEI are controlled as the scheduling request: a prohibit timer that controls the interval between the retransmissions, and a maximum number of retransmissions that controls the maximum number of retransmissions.

Note that these are the only conditions that control the retransmissions. In particular, note that the UE performs the retransmissions even if the event condition is no longer fulfilled:

The UE retransmits the BEI even if the event condition is no longer fulfilled.

Observation 3 may seem surprising that first, but the logic is that the NW wants to know the first time the event was fulfilled, and the procedure should make sure that the NW receives this information. If the UE would stop the retransmissions when the event is no longer fulfilled, the information that the event was fulfilled would be lost.

With the prohibit timer, the UE does not necessarily retransmit the BEI in every PUCCH occasion: if the prohibit timer is running, the UE would not retransmit. Separating the PUCCH periodicity and the BEI retransmission interval facilitates a small PUCCH periodicity (which enables a small latency) while still limiting the rate of retransmissions.  Keep in mind that the NW processing delay can be longer than the interval between PUCCH occasions.

Once the maximum number of BEI retransmissions is exhausted, the UE simply stops transmitting the BEI. This is different from the SR procedure, where the UE initiates a RACH procedure when the maximum number of SRs have been transmitted.

For mode B, the maximum number of BEI transmissions would be set to 1. Otherwise, all the other properties of the BEI would be the same as for mode A.

Discontinuous reception (DRX) is commonly used to reduce the UE power consumption. With DRX, the UE monitors PDCCH more sparsely, and this is true also when UEIBM is enabled. Note that once an SR procedure is initiated, the UE resumes full PDCCH monitoring to receive the corresponding grant. It makes sense to apply the same procedure also after transmission of the first channel:

Once the UE transmitted the BEI, it resumes frequent PDCCH monitoring.

Without Proposal 13, the NW could only send the grant for the second channel in the DRX active period, which may significantly delay the report. RAN1 would then also have to decide when the UE would again reinitiate the sparse PDCCH monitoring.

In RAN1#119, the following was agreed:

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the multiplexing/dropping rule(s) of 1-bit first PUCCH, further study at least the following cases:

- Case-1: The 1-bit first PUCCH is collided/overlapped with a PUCCH carrying normal SR and/or a PUCCH with normal LRR

- Case-2: The 1-bit first PUCCH is collided/overlapped with a PUSCH

- Case-3: The 1-bit first PUCCHs corresponding to different CSI configuration for UE-initiated/event-driven beam reporting are overlapping in the time domain.

The first thing to note is that we need to define multiplexing/dropping rules for all UCI combinations. In NR Rel-15, the following UCI types exist:

ACK/NACK (A/N) – without multiplexing, A/N can only be transmitted over PUCCH

CSI – can be transmitted over PUCCH or PUSCH

SR – without multiplexing, SR can only be transmitted over PUCCH.

We also need to define multiplexing/dropping rules when the new UCI overlaps with a PUSCH transmission.

The current UCI multiplexing rules due to overlapping corresponding PUCCHs are summarized in Table 3.

| Multiplexing rules for UCI on PUCCH |  |
| --- | --- |
| CSI+SR | SR bit(s) is appended to CSI bits. Bits are appended for all SR configurations, but only one of the bits can be set to one, irrespective of how many SRs were triggered. It is up to the UE which bit to set to one. UCI is sent using CSI PUCCH resource |
| CSI+A/N | CSI is appended to A/N. UCI is sent using CSI PUCCH resource if A/N is only DL SPS A/N. Otherwise, PUCCH resource based on A/N PUCCH resources is used. |
| CSI+A/N+SR | CSI and SR bit(s) are appended to A/N. Bits are appended for all SR configurations, but only one of the bits can be set to one, irrespective of how many SRs were triggered. It is up to the UE which bit to set to one. UCI is sent using an A/N PUCCH resource based on UCI size |
| 3 or more A/N bits + SR | SR bit(s) is appended to A/N. Bits are appended for all SR configurations, but only one of the bits can be set to one, irrespective of how many SRs were triggered. It is up to the UE which bit to set to one. UCI is sent using an A/N PUCCH resource based on UCI size |
| 1 or 2 A/N bits + SR | Use additional constellation points in A/N sequence (Cyclic-shift of A/N sequence). Only one SR is multiplexed, up to the UE which one to choose. |

Table 3: The current UCI multiplexing rules on PUCCH for overlapping PUCCHs.

RAN1 agreed to reuse the UCI multiplexing rules for SR as baseline. Looking at Table 3, we realize that this solution is straightforward when the BEI is multiplexed with CSI or A/N on PUCCH: then the BEI will be multiplexed as SR:

Reuse the UCI multiplexing rules for SR when BEI PUCCH resource is overlapped with CSI and or ACK/NACK on PUCCH resource.

For the case when both BEI and SR PUCCH resources overlap with CSI and/or A/N PUCCH resources, the same procedures for SR can be reused for BEI in determination of number of BEI bits and their corresponding representation.

Reuse the UCI multiplexing rules for SR when BEI  PUCCH resource is overlapped with CSI and SR PUCCH resources and or with ACK/NACK and SR PUCCH resources.

If we extend the legacy dropping rules, when both an SR and a BEI is triggered, the UE must determine which to send: even for the cases where the UE reserves bits for both SR and BEI, the UE would only indicate that one of the SR configurations and BEI occurred. From Table 3, we realize that in legacy, if multiple overlapping SRs are triggered, it is up to the UE which one to indicate to the NW. The simplest way to reuse the legacy multiplexing rules is that when BEI and SR overlap, it is up to the UE which one to indicate:

The simplest way to extend the legacy multiplexing rules is when BEI and SR overlap, it is up to the UE to determine to indicate a positive BEI or SR.

However, this may lead to that the UL data is delayed, which is very harmful, also for DL data rates. It may also lead to that an LRR is dropped. Therefore, we propose

When only BEI and SR PUCCH resources overlap (i.e. no overlapping CSI and/or A/N PUCCH resources), if a BEI collides with a positive SR, the BEI is dropped.

For the cases where the SR/BEI is appended to the CSI and/or A/N, the UE would indicate a zero value for the bit(s) corresponding to BEI. In other words, the any SR is prioritized over BEI.

The remaining issue is to discuss what happens when the BEI collides with a PUSCH. When an SR collides with a PUSCH, the SR is simply dropped: the UE may in most cases simply include the UL data that triggered the SR. Note that SR is dropped also when the UE cannot transmit any UL data, for example when the PUSCH is a retransmission, or when the PUSCH includes only UCI.

In legacy, the UE drops the SR also when no UL data can be transmitted.

The logic behind the dropping rule is that the SR is anyway retransmitted and missing one SR transmission occasion would not have a noticeable impact on performance. Also, for the most common collision where the UE is provided a grant to transmit new UL data, that data would simply be included in the PUSCH, or the UE could send a BSR.

If the UE-initiated beam report was carried in a MAC CE, it would have been a natural solution to simply include the report in the PUSCH: the exact same logic would apply. With the decision to use UCI as the container, the UE cannot send the report if it is provided with a grant to send new UL data, since the NW would not be able to decode the UL transmission.

The question is if this difference is important enough to motivate the introduction of a completely new multiplexing rule. The NW can avoid the issue by utilizing these two mechanisms:

The NW may choose not to schedule the UE with PUSCH in a symbol where the UE is configured with a PUCCH resource for BEI. The NW may of course schedule other UEs in these symbols.

In case the NW still wants to schedule a UE with PUSCH in a symbol where a PUCCH resource for BEI is configured, the NW may trigger the UE to send the report, in case an event was triggered. In other words, the NW would space in the UL grant for the UE to send the report. For the cases when no event has occurred, these bits would be wasted. Since the report is most cases around 20 bits, this overhead is acceptable.

- Based on the above reasoning, we propose to reuse the SR multiplexing rules also for the BEI:

When only BEI PUCCH resource overlaps with a PUSCH (i.e. no overlapping CSI and/or A/N PUCCH resources), if a BEI collides with a PUSCH, the BEI is dropped.

The alternative to Proposal 17 would be to extend the A/N multiplexing rule to BEI. In other words, the NW would always reserve resources for the UE to include the BEI in the PUSCH transmission. In case an event occurred, the UE would include a positive value of the BEI in these reserved resources, and if no event occurred, the UE would include a negative value. Since the BEI is just a small number of bits, this would typically be encoded using the coding for small block lengths described in [10], section 5.3.3. For these short block lengths, there is no CRC, and the NW cannot perform any error detection: based on the decoded bits, the NW would have to decide what value the UE transmitted. Since the BEI is multiplexed into a PUSCH, where the modulation and coding schemes are based on the current link quality, the performance of a multiplexed BEI is unclear:

The performance of a BEI multiplexed into a PUSCH is unclear.

Note that when detecting BEI, there will be a risk of missed detection, and false detection.

In RAN1#118bis, the following was agreed:

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the triggering procedure in Step-2 of Mode-A

- Reuse CSI request field in DCI format 0_1/0_2 to trigger the transmission of the UEI beam report

  - If a CSI trigger state associated with UEI beam report configuration(s) is indicated by the CSI request field in DCI format 0_1/0_2, the UE transmits the corresponding UEI beam report(s) in the second PUSCH scheduled by the DCI format 0_1/0_2

  - FFS: DCI format 0_3

- FFS: Whether a CSI trigger state should be dedicated to UE-initiated/event-driven beam reporting, i.e., not associated with legacy AP-CSI report configuration.

The report triggered in step-2 of mode A is very similar to a normal aperiodic beam report: in particular, the configuration will be similar: the CSI report configuration will be similar, as will the configuration of the triggering offsets. As we see it, we could also reuse the CSI processing criteria, and report priorities: there seems to be no reason to deviate from what we have:

The report triggered in step-2 of mode A reuses properties from a normal aperiodic beam report, when it comes to configuration, CSI processing criteria and report priorities.

Note that false SR detection happens from time to time: as a rule of thumb, SR is dimensioned to have 0.1% false detection probability, but it is difficult to achieve that level of accuracy in practice. This means that the UE will receive the grant to transmit the step-2 beam report also when no event has occurred. To avoid complicated error cases, the UE should be prepared to send the step-2 beam report in response to a request from the NW, even when no event has been triggered:

The UE sends the step-2 beam report of mode A in response to the CSI request from the NW even when no event has occurred.

## 2.3	Reporting

In RAN1#119, RAN1 agreed

Agreement

On UE-initiated/event-driven beam reporting, at least one of the following is supported as an extension on L1-RSRP report format depending on Event-2.

- Option-1: For each of reported CRI/SSBRI, to introduce additional indication of whether the CRI/SSBRI satisfies the condition of Event-2.

  - The presence of this field is enabled by RRC with subjective to UE capability.

  - The presence of this field is only for the case that N > 1 and the time window and M are configured

- Option-2: For each of reported CRI/SSBRI, to introduce additional indication of the number of Event-2 instances for the CRI/SSBRI(s) within a time window.

  - The presence of this field is enabled by RRC with subjective to UE capability.

  - The presence of this field is only for the case that N > 1 and the time window and M are configured.

- Option-3: No further enhancement.

Note: As agreed in RAN1#117, at least one of the reported CRI/SSBRI(s) should satisfy the condition of Event-2

FFS: Whether/how to handle the case if UE has not sent any first PUCCH associated for UE-initiated/event-driven beam reporting, for Mode-A.

With the time window/counter functionality agreed above, the beam that triggered the event is not necessarily the beam with the highest RSRP at the beam reporting time. This is illustrated in Figure 2 below.

Figure 2: An illustration of varying beam quality. In this event-2 example, candidate beam 2 is consistently better than the current beam.

In the example in Figure 2, the NW configures event-2 with a 2dB threshold. The NW also configures a time window of 80ms, and M=4, i.e., 4 instances must fulfill the condition within 80ms for the event to be triggered. In this example, candidate beam 1 triggers at time 400ms, but candidate beam 2 has the largest RSRP. With the window/counter, this is generally true:

The beam that triggered the event is not necessarily the beam with the highest RSRP.

The window/counter was introduced to avoid ping pong beam switches, and there was a wide understanding in RAN1 that this was an important feature. However, to avoid ping pong, the window/counter in the UE is not sufficient: the NW must also know which beam triggered the event. Otherwise, the NW has no choice but to switch to the beam with the highest RSRP. Clearly, there is no reduction in ping pong in this case:

There is no reduction in ping pong beam switches if the NW is unaware of which beam triggered the event, i.e., which beam consistently fulfilled the condition.

Based on this argument, the following proposal follows:

For every reported new beam, the UE indicates if it fulfilled the event condition or not.

The current agreements facilitate the configuration of multiple events to the UE: different event types, different CCs, different new beam configurations. When the UE is configured with multiple events, it must be possible for the NW to determine which event has occurred. One option is to define multiple instances of the first UL channel, and associate different events with different instances. However, this solution is very inefficient. Therefore, it is much more efficient to introduce information about the event in the second UL channel:

Introduce an event identifier in the report.

Without such an identifier, the NW cannot determine which event triggered the report.

## 2.4	Reduce TCI state activation time

The TCI state activation delay is described in [7], and is copied below:

8.10.3	MAC-CE based TCI state switch delay

If the target TCI state is known, upon receiving PDSCH carrying MAC-CE for indication of UE-specific PDCCH TCI state as defined in clause 6.1.3.15 of TS 38.321 [7]  in slot n, UE shall be able to receive PDCCH with target TCI state of the serving cell on which TCI state switch occurs at the first slot that is after slot n+ THARQ + 3Nslotsubframe,µ+ TOk*(Tfirst-SSB + TSSB-proc) / NR slot length. The UE shall be able to receive PDCCH with the old TCI state until slot n+ THARQ + 3Nslotsubframe,µ .Where THARQ is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3];

-	Tfirst-SSB is time to first SSB transmission after MAC CE command is decoded by the UE; The SSB shall be the QCL-TypeA or QCL-TypeC to target TCI state

-	TSSB-proc = 2 ms;

-	TOk = 1 if target TCI state is not in the active TCI state list for PDSCH, 0 otherwise.

The activation time is dominated by the term TOk*(Tfirst-SSB + TSSB-proc).

In RAN1#119, RAN1 agreed

Agreement

Study the following to reduce beam application latency after a UEI/ED beam report is sent

- Alt-1: After confirmation/acknowledgement from NW, apply new beam without RRC configuration signaling or MAC-CE signaling

  - after sending a UE-initiated beam report, the UE could store the QCL properties of the SSB associated with the reference signal reported in the beam report

  - update TCI state(s) with the reported new beam(s)

  - activate new beam(s) without additional SSB reception

- Alt-2: After receiving a TCI state activation command to activate a TCI state(s), if the new beam(s) associated with the TCI state(s) is reported as synchronized in the UEI/ED beam report, the TCI state(s) becomes applicable for DL reception without additional SSB reception.

  - Note: A reported new beam is determined as synchronized by UE, if the UE stores the QCL properties associated with the reported new beam(s) after the UEI/ED beam report is sent

  - FFS: How to inform a reported new beam in a UEI/ED beam report (i.e., introducing one-bit indicator for each reported new beam or all the reported new beam are assumed to be synchronized)

- Alt-3: After sending a UE-initiated beam report, the UE could store the QCL properties of the SSB associated with the reference signal reported in the beam report.

  - In such case, at the reception of a subsequent reception of Unified TCI States Activation/Deactivation MAC CE, the UE activates new beam(s) without additional SSB reception

- Other alternatives are not precluded

Comparing the alternatives in the agreement, Alt-1 requires that a new acknowledgement of the report is introduced. Alt-1 is also a step towards UE-controlled beam switching, which would be a fundamental change in beam management procedures, and in RRM procedures in general. Therefore, we do not support Alt-1.

The difference between Alt-2 and Alt-3 is small. With Alt-3, the UE is required to remember the QCL properties of all the SSBs the UE reported, and a subsequent activation of a TCI state associated with any of the reported SSBs would not require a reception of an additional SSB, leading to TOk=0.  For Alt-2, the UE is only required to remember the QCL properties of a subset of the SSBs it reported, and this is indicated by the UE in the report. A subsequent activation of a TCI state associated with an SSB the UE indicated as synchronized would not require a reception of an additional SSB, leading to TOk=0, but a subsequent activation of a TCI state associated with an SSB the UE reported by was not indicated as synchronized would require a reception of an additional SSB, leading to TOk=1.

The difference between Alt-2 and Alt-3 is small, and we would be OK with either:

To reduce the TCI state activation time, support Alt-2 or Alt-3.

## 2.5	RRC parameters

In RAN1#119, RRC parameters were discussed offline and the output was collected in [9]. It is assumed that all these parameters are configured inside a CSI-ReportConfig. Here we will comment on these parameters and suggest a few new.

Parameters currently in the RRC parameter excel:

reportTransmissionMode: This parameter is used to differentiate between mode A and mode B. This parameter should be replaced by a configuration for the second UL channel: when a configuration for the second UL channel is present, the UE should apply mode B. If a configuration for the second UL channel is absent, the UE should apply mode A.

eventThreshold: This parameter is used to control the threshold in the event evaluation. This parameter should be applicable for event-1 and event-7 as well.

eventInstanceCount, eventDetectionTimeWindowLength: These parameters control the event evaluation. The event triggers when a condition is fulfilled eventInstanceCount within the time eventDetectionTimeWindowLength.  These parameters should be applicable for event-1 and event-7 as well.

enabledCurrentBeamReport: This parameter is used to instruct the UE to include a measurement of the current beam in the report. This parameter should be applicable for event-1 and event-7 as well.

nrofReportedRS-UEIBR: This parameter is used to control how many new beams are included in total in the report. The UE always includes the beam that triggered the event.

newBeamResourceSetEvent2: This parameter is used to configure the new beams. We note that the CSI-ReportConfig already includes the mandatory IE resourcesForChannelMeasurement, and we propose to reuse that rather than defining a new IE.

eventType: The parameter is used to configure which event is configured.

valueOfQ: This parameter is used to configure the value Q for event-7. Here we propose to change the name of the parameter: RAN1 used Q during the discussion, but here it would be preferable to use a more descriptive name.

configuredResourceForSecondChannelOfModeB: This parameter describes the resources used for transmission of the second channel in ModeB. Also in this case, the parameter is using naming used in RAN1, which is somewhat enigmatic: the addition ForSecondChannelOfModeB is quite unnecessary, and we propose to remove that.

carrier: The excel sheet indicates that this is an old parameter. Also, the parameter description is the same as in legacy.

New proposed parameters:

eventIndicatorChannel: This parameter provides the configuration for the first UL channel and has fields periodicityAndOffset, PUCCH-ResourceID. We also see the need to have a prohibit timer and maximum number of retransmissions as part of the eventIndicatorChannel configuration. Note that we use a descriptive name of the parameter, rather than using cryptic names used during the RAN1 standardization.

eventId: The UE includes this parameter when it sends the report. The NW would use this to determine which event has occurred. In principle, we could use the reportConfigId for this purpose, but that identifier would be unnecessarily long, since not all CSI report configurations are associated with UE-initiated beam reporting.

Parameters in the CSI report configuration that are reused and relevant:

resourcesForChannelMeasurement: This parameter is used to provide the new beams to the UE. This can be either a list of CSI-RS resource sets or a list of CSI SSB resource sets. For UE initiated beam reporting, we do not support a combination of CSI-RS resource sets and CSI SSB resource sets.

reportConfigType: this field is only valid for mode A and must in this case be set to aperiodic. The reportSlotOffsetList describes when the UE should send the report when triggered with this report. In practice, this parameter must be the same for all mode A events: otherwise, the NW will not be able to receive the report. Alternatively, we could introduce a new report in parallel (UEinitiated), which would in that case require the inclusion of slot offset as for an aperiodic report.

- In summary, we propose the following RRC parameter updates:

| Parameter name in the spec | Description |
| --- | --- |
| reportTransmissionMode-r19 | This parameter is used to inform one report transmission mode on UCI based beam report procedure for UE-initated/event-driven beam reporting. - ModeA: UCI in a dynamically scheduled PUSCH - ModeB: UCI in a pre-configured UL resource(s) Note: This par may be removed later if it can be inferred from other configurations |
| eventThreshold-r19 | This parameter is used to inform a threshold value for trigger event detection regarding Event-1, Event-2 and Event-7. |
| eventDetectionTimeWindowLength-r19 | This parameter is used to inform the time window for triggering event determination regarding Event-1, Event-2 and Event-7 |
| eventInstanceCount-r19 | This parameter is used to inform the number of event instances for at least one same new beam within a configured time window that the UE can initiate UEIBM report. This parameter applies to Event-1, Event-2 and Event-7. |
| enabledCurrentBeamReport-r19 | This parameter is to enable or disable whether current beam is always reported. - When enabled by RRC, the current beam + N beams from the measurement RSs for new beam(s) are reported l - Note: The reported current beam is NOT counted in the N reported beams. - When disabled by RRC, N beams are reported. |
| nrofReportedRS-UEIBR-r19 | This parameter is used to inform the number of reported RS(s) in a report format for UE-initiated/event-driven beam reporting. |
| newBeamResourceSetEvent2-r19 | This parameter is used to configure the new beam resource set for Event 2 (Opt1) |
| valueofQ currentBeamIndex | Defines the current beam for event-7. This could be made a field under eventType, since it is only relevant for event-7. |
| configuredResourceForSecondChannelOfModeB | Used to configure resources for sending the report. |
| eventIndicatorChannel | This parameter is used to inform the UE how to transmit the first UL channel, and contains the fields periodicityAndOffset, PUCCH-ResourceID, and prohibitTimer, and ueibr-TransMax. |
| eventId | The UE inserts this parameter in the report to inform the NW which event occurred. |
| resourcesForChannelMeasurement | Append the field description to state that this field describes the new beam configuration for UE-initiated beam reporting |
| reportConfigType | Append the field description to state that this field should be set to aperiodic when configuredResource is not configured. |

# Conclusion

In the previous sections we made the following observations:

Observation 1	The report for event-7 should include information on what TCI states to activate, and what TCI states to deactivate.

Observation 2	When a UE is scheduled using DCI format 0_3, the UCI is transmitted on one of the scheduled cells.

Observation 3	The UE retransmits the BEI even if the event condition is no longer fulfilled.

Observation 4	The simplest way to extend the legacy multiplexing rules is when BEI and SR overlap, it is up to the UE to determine to indicate a positive BEI or SR.

Observation 5	In legacy, the UE drops the SR also when no UL data can be transmitted.

Observation 6	The performance of a BEI multiplexed into a PUSCH is unclear.

Observation 7	The beam that triggered the event is not necessarily the beam with the highest RSRP.

Observation 8	There is no reduction in ping pong beam switches if the NW is unaware of which beam triggered the event, i.e., which beam consistently fulfilled the condition.

Based on the discussion in the previous sections we propose the following:

Proposal 1	The event evaluation for UE-initiated beam reporting is captured in a RAN2 specification.

Proposal 2	Reuse the RRC IE CSI-ResourceConfig to define the new beams for UE-initiated beam management.

Proposal 3	The event evaluation periodicity is identical to the periodicity of the current beam RS.

Proposal 4	The event instance counting is reset for all new beams when the parameters associated with event-driven reporting are reconfigured.

Proposal 5	For event-1 and event-2, the event instance counting is reset for all new beams when the indicated TCI state is updated.

Proposal 6	The event instance counter is reset when it reaches its maximum value.

Proposal 7	For event-1, it should be possible to configure the UE to report RSs.

Proposal 8	For event-1, RS resource set for new beam is configured in the CSI reporting configuration, and the following implicit manner for enabling one of either scheme-1 or scheme-2 is used: if the RS(s) for new beam are CSI-RS configured in a CSI-RS resource se configured with repetition, Scheme-1 is enabled; otherwise, Scheme-2 is enabled.

Proposal 9	For event-7, it should be possible to configure the UE to report up to 8 RSs.

Proposal 10	The UL-grant DCI format comprises DCI format 0_1/0_2/0_3.

Proposal 11	The UE retransmits the BEI until it receives a DCI format that indicates a resource for a second UL channel.

Proposal 12	The retransmissions for the BEI are controlled as the scheduling request: a prohibit timer that controls the interval between the retransmissions, and a maximum number of retransmissions that controls the maximum number of retransmissions.

Proposal 13	Once the UE transmitted the BEI, it resumes frequent PDCCH monitoring.

Proposal 14	Reuse the UCI multiplexing rules for SR when BEI PUCCH resource is overlapped with CSI and or ACK/NACK on PUCCH resource.

Proposal 15	Reuse the UCI multiplexing rules for SR when BEI  PUCCH resource is overlapped with CSI and SR PUCCH resources and or with ACK/NACK and SR PUCCH resources.

Proposal 16	When only BEI and SR PUCCH resources overlap (i.e. no overlapping CSI and/or A/N PUCCH resources), if a BEI collides with a positive SR, the BEI is dropped.

Proposal 17	When only BEI PUCCH resource overlaps with a PUSCH (i.e. no overlapping CSI and/or A/N PUCCH resources), if a BEI collides with a PUSCH, the BEI is dropped.

Proposal 18	The report triggered in step-2 of mode A reuses properties from a normal aperiodic beam report, when it comes to configuration, CSI processing criteria and report priorities.

Proposal 19	The UE sends the step-2 beam report of mode A in response to the CSI request from the NW even when no event has occurred.

Proposal 20	For every reported new beam, the UE indicates if it fulfilled the event condition or not.

Proposal 21	Introduce an event identifier in the report.

Proposal 22	To reduce the TCI state activation time, support Alt-2 or Alt-3.

# References

RP-234007, New WID: NR MIMO Phase 5, Samsung (Moderator), RAN#102, Edinburgh, Scotland, December 2023

R2-2409200, Initial analysis on RAN2 impact for Rel-19 NR MIMO Ph5, Huawei/HiSilicon, RAN2#127bis, Hefei, China, October 2024

R2-2408092, RAN2 Impacts of Rel-19 NR MIMO, CMCC, RAN2#127bis, Hefei, China, October 2024

R2-2408125, Discussion on UE-initiated/event-driven beam management, vivo, RAN2#127bis, Hefei, China, October 2024

R2-2408667, User plane and Control plane impacts from MIMO, Ericsson, RAN2#127bis, Hefei, China, October 2024

R2-2408795, Discussion on the design considerations for MIMO Phase 5, Qualcomm Incorporated, RAN2#127bis, Hefei, China, October 2024

3GPP TS 38.133, NR; Requirements for support of radio resource management, RAN#101, Bangalore, India, v18.3.0, September 2023

3GPP TS 38.214 V18.2.0, NR; Physical layer procedures for data, RAN1#103, Maastricht, The Netherlands, March 2024

R1-2409592, List of RRC and MAC CE impact for Rel-19 MIMO Ph5, Moderator (Samsung), RAN1#119, Orlando, USA, November 2024

3GPP TS 38.212 V18.4.0, NR; Physical layer procedures for data, RAN1#105, Melbourne, Australia, September 2024