3GPP TSG-RAN WG1 Meeting #120	R1-2501266

# Athens, Greece, 17 – 21 February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.212 | CR | DRAFT | rev | - | Current version: | 17.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Soft buffer size determination for 1024QAM rank restricted UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_DL1024QAM_FR1-Core |  |  |  |  |  | Date: |  |  | 2024-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For 1024QAM, two UE capabilities were defined in Release 17: pdsch-1024QAM-FR1-r17 Indicates whether the UE supports 1024QAM modulation scheme for PDSCH for FR1 as defined in TS 38.211 [6], MCS and CQI feedback tables based on 1024QAM modulation order as defined in TS 38.214 [12]. pdsch-1024QAM-2MIMO-FR1-r17 Indicates whether the UE supports 1024QAM modulation scheme for PDSCH with maximum 2 MIMO layers for FR1 as defined in TS 38.211 [6], MCS and CQI feedback tables based on 1024QAM modulation order as defined in TS 38.214 [12]. The TS38.212 calculation for soft buffer size doesn’t take the rank restricted UE type into account, but calculates the soft buffer size for LDPC bit selection according to the configured maximum modulation and configured maximum rank, even though with the UE indicating pdsch-1024QAM-2MIMO-FR1-r17 and being configured with maximum modulation of 1024QAM and with maximum rank > 2 will not use the maximum modulation and maximum rank at the same time. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarifying that 1024QAM rank-restricted UE (indicating pdsch-1024QAM-2MIMO-FR1-r17 capability) calculates its HARQ buffer size using 1024QAM only if the maximum rank is not larger than 2 also for the lower modulation orders. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear if the 1024QAM rank-restricted UE (indicating pdsch-1024QAM-2MIMO-FR1-r17 capability) should calculate the soft buffer size according to maximum modulation and maximum rank even if the maximum rank > 2 and thus the maximum rank and maximum modulation never happen together. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the first version of the CR |  |  |  |  |  |  |  |  |

### 5.4.2	Rate matching for LDPC code

The rate matching for LDPC code is defined per coded block and consists of bit selection and bit interleaving. The input bit sequence to rate matching is . The output bit sequence after rate matching is denoted as .

#### 5.4.2.1	Bit selection

The bit sequence after encoding  from Clause 5.3.2 is written into a circular buffer of length  for the -th coded block, where  is defined in Clause 5.3.2.

For the -th code block, let  if  and  otherwise, where, ,  is determined according to Clause 6.1.4.2 in [6, TS 38.214] for UL-SCH and Clause 5.1.3.2 in [6, TS 38.214] for DL-SCH/PCH, assuming the following:

For one TB for DL-SCH with PDSCH scheduled by DCI format 4_0/4_1/4_2,

-	if the PDSCH is scheduled by DCI format 4_1/4_2,

-	maximum number of layers is given by X, where

-	if the higher layer parameter maxMIMO-Layers of pdsch-ConfigMulticast is configured, X is given by that parameter;

-	otherwise, X equals to 1;

-	if the higher layer parameter mcs-Table given by a pdsch-ConfigMulticast for at least one common frequency resource (CFR) is set to 'qam256', maximum modulation order Qm=8 is assumed for DL-SCH; otherwise a maximum modulation order Qm=6 is assumed for DL-SCH;

-	if the PDSCH is scheduled by DCI format 4_0,

-	maximum number of layers is 1;

-	if the higher layer parameter mcs-Table given by a pdsch-ConfigMCCH is set to 'qam256', maximum modulation order Qm=8 is assumed for DL-SCH; otherwise a maximum modulation order Qm=6 is assumed for DL-SCH;

-	if the higher layer parameter mcs-Table given by a pdsch-ConfigMTCH is set to 'qam256', maximum modulation order Qm=8 is assumed for DL-SCH; otherwise a maximum modulation order Qm=6 is assumed for DL-SCH;

-	nPRB=nPRB,LBRM is given by Table 5.4.2.1-1, where the value of nPRB,LBRM for DL-SCH is determined according to the size of the associated CFR if configured to the UE;

-	maximum coding rate of 948/1024;

-	NRE=156∙nPRB;

-	C is the number of code blocks of the transport block determined according to Clause 5.2.2.

For one TB for UL-SCH, or for one TB for DL-SCH/PCH except for DL-SCH with PDSCH scheduled by DCI format 4_0/4_1/4_2,

-	maximum number of layers for one TB for UL-SCH is given by X, where

-	if the higher layer parameter maxMIMO-Layers of PUSCH-ServingCellConfig of the serving cell is configured, X is given by that parameter

-	elseif the higher layer parameter maxRank of pusch-Config of the serving cell is configured, X is given by the maximum value of maxRank across all BWPs of the serving cell

-	otherwise, X is given by the maximum number of layers for PUSCH supported by the UE for the serving cell

-	maximum number of layers for one TB for DL-SCH/PCH is given by the minimum of X and 4, where

-	if the higher layer parameter maxMIMO-Layers of PDSCH-ServingCellConfig of the serving cell is configured, X is given by that parameter

-	otherwise, X is given by the maximum number of layers for PDSCH supported by the UE for the serving cell

-	if the higher layer parameter mcs-Table-r17 or mcs-TableDCI-1-2-r17 given by a pdsch-Config for at least one DL BWP of the serving cell is set to 'qam1024', and

-	if the UE indicated support for pdsch-1024QAM-FR1-r17, maximum modulation order Qm=10 is assumed for DL-SCH,

-	else if the UE indicated support for pdsch-1024QAM-2MIMO-FR1-r17 and X ≤ 2, maximum modulation order Qm=10 is assumed for DL-SCH,

-	else if the UE indicated support for pdsch-1024QAM-2MIMO-FR1-r17 and X > 2, maximum modulation order Qm=8 is assumed for DL-SCH,

-	else if the higher layer parameter mcs-Table or mcs-TableDCI-1-2 given by a pdsch-Config for at least one DL BWP of the serving cell is set to 'qam256', maximum modulation order  is assumed for DL-SCH; otherwise a maximum modulation order  is assumed for DL-SCH;

-	if the higher layer parameter mcs-Table or mcs-TableTransformPrecoder or mcs-TableDCI-0-2 or mcs-TableTransformPrecoderDCI-0-2 given by a pusch-Config or the higher layer parameter mcs-Table or mcs-TableTransformPrecoder given by configuredGrantConfig for at least one UL BWP of the serving cell is set to 'qam256', maximum modulation order  is assumed for UL-SCH; otherwise a maximum modulation order  is assumed for UL-SCH

-	maximum coding rate of 948/1024;

-	 is given by Table 5.4.2.1-1, where the value of  for DL-SCH is determined according to the initial downlink bandwidth part if there is no other downlink bandwidth part configured to the UE;

-	;

-	 is the number of code blocks of the transport block determined according to Clause 5.2.2.

Table 5.4.2.1-1: Value of

| Maximum number of PRBs across all configured DL BWPs and UL BWPs of a carrier for DL-SCH and UL-SCH, respectively, or Maximum number of PRBs across all CFRs of a carrier for DL-SCH with PDSCH scheduled by DCI format 4_0/4_1/4_2 |  |
| --- | --- |
| Less than 33 | 32 |
| 33 to 66 | 66 |
| 67 to 107 | 107 |
| 108 to 135 | 135 |
| 136 to 162 | 162 |
| 163 to 217 | 217 |
| Larger than 217 | 273 |

Denoting by  the rate matching output sequence length for the -th coded block, where the value of  is determined as follows:

Set

for  to

if the -th coded block is not scheduled for transmission as indicated by CBGTI according to Clause 5.1.7.2 for DL-SCH and 6.1.5.2 for UL-SCH in [6, TS 38.214]

;

else

if

;

else

;

end if

;

end if

end for

where

-	 is the number of transmission layers that the transport block is mapped onto;

-	 is the modulation order;

-	 is the total number of coded bits available for transmission of the transport block;

-	 if CBGTI is not present in the DCI scheduling the transport block and  is the number of scheduled code blocks of the transport block if CBGTI is present in the DCI scheduling the transport block.

Denote by  the redundancy version number for this transmission ( = 0, 1, 2 or 3), the rate matching output bit sequence , , is generated as follows, where  is given by Table 5.4.2.1-2 according to the value of  and LDPC base graph:

;

;

while

if

;

;

end if

;

end while

Table 5.4.2.1-2: Starting position of different redundancy versions,

|  | LDPC base graph 1 | LDPC base graph 2 |
| --- | --- | --- |
| 0 |  |  |
| 1 |  |  |
| 2 |  |  |
| 3 |  |  |