3GPP TSG-RAN WG1 Meeting #120	R1-2501272

Athens, Greece, February 17th – 21nd, 2025

Agenda Item:	9.2.2

Source:	Ericsson

Title:	Remaining issues on CSI enhancements for large antenna arrays and CJT

Document for:	Discussion, Decision

# 1	Introduction

In the work item description (WID) for NR MIMO Phase 5 we have the following objectives related to CSI enhancement and CJT [1].

- Specify CSI support for up to 128 CSI-RS ports, targeting FR1

  - Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks

  - Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s)

  - Extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design

  - SRS port grouping and its association to the two codewords for the 6/8Rx low complexity receiver supporting more than 4 layers, with legacy codebook

    - No enhancement on codeword-to-layer mapping, DL resource allocation, CSI feedback, and DCI format

    - Note: Whether to support 6Rx with more than 4 layers is to be decided in RAN4 Rel-19 RF enhancements WI

- Specify UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD

- Inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH

In RAN1#116 to RAN#118, the topics were discussed, and some agreements were reached. In this paper, we provide Ericsson’s views on the remaining issues related to the above objectives.

# 2	CSI Support for up to 128 ports

## 2.1 Extension of Rel-19 Type I codebooks to number of ports ≤32

During pre-RAN1#119 offline email discussion [3], it was discussed whether to extend Rel-19 Type I codebooks introduced for 48, 64, and 128 ports to smaller of ports as well.  We provide some simulation results to compare the performances between Rel-15 and Rel-19 Type I codebooks for 32 ports.  The results are provided in Table 2 for ranks 1-4, and the evaluation assumptions are provided in the appendix in Table 4.

As can be seen in the results, Rel-19 Type I Scheme A (no array splitting) shows around 3% (4%) gain at 50% RU and around 7% (8%) gain at 70% RU when compared to the Rel-15 baseline for mean user throughput (cell edge throughput).  Rel-19 Type I Scheme B shows around 7% (12%) gain at 50% RU and around 16% (24%) gain at 70% RU when compared to the Rel-15 baseline for mean user throughput (cell edge throughput). ​

Our results suggest that Rel-19 Type I Scheme B provides larger gains than Rel-19 Type I Scheme A.  Based on the results, we support extension of Rel-19 Type I codebooks to number of ports smaller or equal to 32.  To avoid introduction of too many UE capabilities, our preference is to introduce this extension to only 16, 24, and 32 ports.

- Proposal 1:  For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, extend the agreed Scheme-A and Scheme-B to 16, 24, and 32 CSI-RS ports, for all applicable RI values with K=1 only.

Table 2: Comparison between Rel-15 and Rel-19 Type I codebooks for 32 ports for ranks 1-4

## 2.3 Mapping of CBSR and scaling factor bitmaps to beam groups

Although a N1⋅O1⋅N2⋅O2X1⋅X2-bit group based CBSR is agreed in Rel-19, the details of which beam group a given bit in the N1⋅O1⋅N2⋅O2X1⋅X2-bit CBSR bitmap is mapped to still needs to be solved.  Let the gth (g=1, 2, …,N1⋅O1⋅N2⋅O2X1⋅X2)  beam group be composed of the following X1X2 2-D DFT vectors:

vxg,1,xg,2 ,   vxg,1+1,xg,2,   …  vxg,1+X1-1,xg,2 ,

vxg,1,xg,2+1 ,   vxg,1+1,xg,2+1,   …  vxg,1+X1-1,xg,2+1 ,

…

vxg,1,xg,2+X2-1 ,   vxg,1+1,xg,2+X2-1,   …  vxg,1+X1-1,xg,2+X2-1 ,

Then the gth beam group is mapped to the gth bit in the CBSR bitmap where

xg,1=ag,1-1X1

xg,2=(ag,2-1)X2

ag,2=mod(g-1, N2O2X2)+1

ag,1=gX2N2O2

An example illustrating the mapping between the bits in the CBSR bitmap and beam groups is shown in Figure 2,  where N1=6, N2=3, and O1=O2=4.  Furthermore, the values of X1 and X2 are both 4 in the example of Figure 2. The resulting beam group mapping is shown in below.

Figure 2.  An example illustrating the mapping between the bits in the CBSR bitmap and beam groups

- Proposal 2:  The gth (g=1, 2, …, ) bit in the CBSR bitmap is mapped to the following beam group:

- where

Similarly, the gth group of bits B3(g)=b3(g,2)b3(g,1)b3(g,0) for the scaling factor can be mapped to the following beam group of X'1X'2 2-D DFT vectors

vxg,1,xg,2 ,   vxg,1+1,xg,2,   …  vxg,1+X'1-1,xg,2 ,

vxg,1,xg,2+1 ,   vxg,1+1,xg,2+1,   …  vxg,1+X'1-1,xg,2+1 ,

…

vxg,1,xg,2+X'2-1 ,   vxg,1+1,xg,2+X'2-1,   …  vxg,1+X'1-1,xg,2+X'2-1 ,

where

xg,1=ag,1-1X'1

xg,2=(ag,2-1)X'2

ag,2=mod(g-1, N2O2X'2)+1

ag,1=gX'2N2O2

- Proposal 3:  The gth (g=1, 2, …, ) group of 3-bits for the scaling factor is mapped to the following beam group:

- where

# 3		Remaining issues on enhancement for CJT

## 3.1 On linking the CJTC Dd and Rel-18 eType-II CJT CSI reports

In RAN1#118bis, the following agreements were captured for the cases when linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured:

Agreement

For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured, confirm the following working assumptions as agreement with the following refinement:

- When separately triggered, the delay offset value to be compensated is the latest reported delay offset (DO) whose reporting instance’s last symbol is before the first symbol of DCI triggering of the CJT CSI reporting

- When jointly triggered, the delay offset value to be compensated is the reported delay offset (DO) in the same reporting instance

FFS: Whether an additional UE procedure is needed when the reported DO value is ‘out of range’

FFS: Whether the Dd report codepoints need to be reinterpreted from intervals/ranges to values when the linkage mechanism is configured, or in general, a rule is needed to determine the value of DO for the compensation for each CMR for CJT CSI report (including if multiple DOs are reported)

Agreement

For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured with two separate triggers, support to include an indicator in the trigger for a Rel-18 eType-II CJT CSI, which indicates whether the UE should perform delay offset (DO) compensation based on the linked CJTC Dd report when calculating the Rel-18 Type-II CJT CSI or not.

- This feature is a separate UE capability

- No new DCI field is introduced. This does not preclude increasing the bit width for CSI request.

- FFS: Details on signalling design for the indicator including whether it is per CSI-RS resource/Dd value and the associated UE behaviour(s)

Agreement

For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured with two separate triggers, to indicate whether or not the UE should perform delay offset (DO) compensation based on the latest linked CJTC Dd report when calculating the Rel-18 Type-II CJT CSI, introduce a 1-bit indicator per CSI trigger state:

- The 1-bit indicator is an RRC parameter

- This is done without increasing the bit-width of the CSI request field of the DCI triggering a Rel-18 CJT eType-II CJT CSI report

- FFS (RAN1#119): Whether the 1-bit indicator applies to all the NTRP CSI-RS resources, or to 1-bit indicator per CSI-RS resource

- FFS (RAN1#119): How this applies to a single CSI trigger state associated with >1 CSI reports

Agreement

For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured, support linking the CMRs in the two CSI Report Settings so that UE knows which CMRs in the two report settings correspond to the same TRP.

- Based on a fixed correspondence between the set of TRS resource set IDs in sequential order and the set of CSI-RS resource IDs in sequential order of configuration in their respective Resource Setting

FFS: linking, when the number of resources configured for CJT CSI is < number of resource sets configured for CJT Dd, in case of separate triggers

Agreement

For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured with a joint trigger:

- Reuse the CPU occupation and active resource counting for the Rel-18 eType-II CJT

- FFS (RAN1#119): Re timeline, decide whether to reuse or further relax the timeline for the Rel-18 eType-II CJT

Conclusion

For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured with a joint trigger, there is no consensus in supporting P/SP CSI-RS as the CMR for the Rel-18 eType-II CJT reporting (in addition to the agreed AP CSI-RS).

When linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured, a remaining FFS is whether additional UE procedures are needed when the reported DO value is ‘out of range.’ With an “out of range” report, the network has no information what DO value is, except that the delay is probably large and CJT would not work properly with all or a subset of the TRPs. Hence, what seems reasonable to perform at the UE side is that the UE does not perform pre-compensation when the UE reports an “out of range” value as part of the Dd report.

- Proposal 4: 	When linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured, and the reported DO value is ‘out of range’, the UE is not expected to perform pre-compensation.

Another FFS is whether there should be a mapping between the Dd report codepoints and the DO values to be compensated. We believe that there is no need for rules determining the value of DO associated with different reported codepoints for the compensation for each CMR for CJT CSI report. This is because with proper resolution and possible processing of multiple DO reports, the benefit of such mappings and the quantization error will be minor. Thus, no further specification work is required for such minor optimizations.

- Proposal 5: 	For DO compensation, it is up to UE to use a value in the interval represented by the reported codepoint.

## 3.2 Reporting Phase Offsets for Reciprocity Based CJT

On phase offset reporting for reciprocity based CJT, the following agreements were made in prior RAN1 meetings:

Agreement

For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), regarding the support of sub-band reporting (>1):

- Denoting the number of reported sub-band phase-offset values as NSB-P, and the sub-bands are indexed as {0, 1, …, NSB-P –1}, support, as a separate UE capability from wideband (=1) phase offset reporting, reporting, in one CSI reporting instance, {(n,, n,, n,NSB-P), n=0, 1, …, NTRP – 1, n≠nref}

  - The alphabet for n, follows the previously agreed alphabet for =1, including the ‘invalid’ state

- FFS:

  - Supported sub-band size(s), e.g. following the legacy CSI sub-band definition, vs {1, 2, 4, 8, 16 PRBs}

  - Whether the UE performs measurement over the entire configured CSI reporting band WCSI

  - If needed, mechanism to limit CSI reporting overhead (e.g. maximum NSB-P)

  - If needed, configuration of whether the CSI-RSs are MRT-beamformed in frequency-domain or not

Note: UE is not required to perform DO/FO compensation for sub-band phase offset calculation

Note: There is a source R1-2407184 showing for frequency-domain-MRT-precoded CSI-RSs, inter-TRP phase has a linear rotation in frequency-domain, and is associated with a single delay component.

Agreement

For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), regarding the support of sub-band reporting (>1), the maximum value of configured NSB-P is 16 per CSI reporting setting.

Agreement

For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), regarding the configured associated SRS resource,

- Confirm the following working assumption as agreement: “the configured associated SRS resource can be either periodic, semi-persistent, or aperiodic”

- When periodic or semi-persistent associated SRS resource is configured, the SRS transmission occasion for determining the reference UE antenna port corresponds to the latest SRS transmission occasion before the occasions of the NTRP CSI-RS resources used for measuring ‘cjtc-P’ report

  - FFS: Whether ‘the earliest SRS transmission occasion after the NTRP CSI-RS occasions’ is also supported as an option

  - FFS: Whether determination of SRS transmission occasion is needed for aperiodic associated SRS resource, and if so, how

In RAN1#118bis, it was agreed that, when periodic or semi-persistent associated SRS resource is configured, the SRS transmission occasion for determining the reference UE antenna port corresponds to the latest SRS transmission occasion before the occasions of the NTRP CSI-RS resources used for measuring ‘cjtc-P’ report.

A further FFS is whether the earliest SRS transmission occasion after the NTRP CSI-RS occasions should also be supported. We do not see a need in supporting such an option. As for determination of SRS transmission occasion for aperiodic SRS resource, UE should always assume the latest triggered SRS before receiving the trigger for phase report.

- Proposal 6: 	For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), the SRS transmission occasion for determining the reference UE antenna is the latest aperiodic SRS transmission occasion for antenna switching before receiving the trigger for phase offset reporting.

- Proposal 7: 	When periodic or semi-persistent associated SRS resource is configured, “the earliest SRS transmission occasion after the NTRP CSI-RS occasions” is not supported as an option for the SRS transmission occasion for determining the reference UE antenna port.

# 4.  Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1:  For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, extend the agreed Scheme-A and Scheme-B to 16, 24, and 32 CSI-RS ports, for all applicable RI values with K=1 only.

Proposal 2:  The gth (g=1, 2, …, ) bit in the CBSR bitmap is mapped to the following beam group:

where

Proposal 3:  The gth (g=1, 2, …, ) group of 3-bits for the scaling factor is mapped to the following beam group:

where

Proposal 4: 	When linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured, and the reported DO value is ‘out of range’, the UE is not expected to perform pre-compensation.

Proposal 5: 	For DO compensation, it is up to UE to use a value in the interval represented by the reported codepoint.

Proposal 6: 	For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), the SRS transmission occasion for determining the reference UE antenna is the latest aperiodic SRS transmission occasion for antenna switching before receiving the trigger for phase offset reporting.

Proposal 7: 	When periodic or semi-persistent associated SRS resource is configured, “the earliest SRS transmission occasion after the NTRP CSI-RS occasions” is not supported as an option for the SRS transmission occasion for determining the reference UE antenna port.

# 5.  References

RP-242394, WID revision: NR MIMO Phase 5, Samsung (Moderator), RAN meeting #105 Melbourne, Australia, September 9-12, 2024.

Word Radiocommunications Conference 2023 (WRC-23), Provisional Final Acts, Dec. 2023.

R1-2409587, Moderator summary for OFFLINE discussion on Rel-19 CSI enhancements, RAN1#119 Orlando, USA, November 18-22, 2024.