3GPP TSG RAN WG1 #120							    R1-2501380

Athens, Greece, February 17th – 21st, 2025

Source:	Moderator (MediaTek)

Title:	FL summary 3 for on-demand SIB1 in idle/inactive mode

Agenda item:	9.5.2

Document for:	Discussion and Decision

Introduction

This document is used to assist the discussions for “9.5.2 On-demand SIB1 for idle/inactive mode UEs” of the Rel-19 working item on “Enhancements of network energy savings for NR” based on proposals from companies.

This contribution provides discussion points in Section 2, resulted RAN1 conclusion/agreement in Section 3 (TBD), and reference (companies tdoc list) in Section 4.

Discussion points

- (Closed) Issue 1: Reference time point to determine the on-demand SIB1 window starting time

Background

In RAN1 #119, the following are agreed:

Agreement

The reference time point to determine the window starting time for on-demand SIB1 is based on the RAR window for UL WUS wherein UE successfully received a RAR.

- FFS: Use starting or ending slot of the RAR window as reference time

On the reference time point, companies’ views in RAN1 #120 can be summarized below:

- Option 1: Using starting slot of the RAR window

  - Lenovo, ETRI, DOCOMO, Ericsson, FUTURWEI, vivo, Nokia, Panasonic, Interdigital, Google,

- Option 2: Using ending slot of the RAR window

  - Apple, Samsung, Transsion, DENSO, Qualcomm, Huawei/HiSilicon, ZTE, Spreadtrum/UNISOC, CATT, China Telecom, CMCC, Tejas, OPPO, Fujitsu, Xiaomi, MTK

From moderator’s perspective, both starting slot or ending slot can work. Using starting slot can reduce some access latency while using ending slot may be a more obvious way. Moderator would like to first try the majority view.

- FL Proposal 1-1

The reference time point to determine the window starting time for on-demand SIB1 is based on the ending slot of the RAR window for UL WUS wherein UE successfully received a RAR.

| Company | Support or not | Comment |
| --- | --- | --- |
| CMCC | Support |  |
| InterDigital | No | Given the relatively long timeline ranging from UL-WUS transmission to RAR reception, reducing the latency for receiving PDCCH for OD-SIB1 is important. As such, we support using the starting slot of the RAR window. |
| Google | Support |  |
| ZTE, Sanechips | Support | I agree that Proposal 1-1 is simple solution we can do. And for option 1, although some benefit from access latency aspect can be achieved, we need to handle one complicate case where exists overlapped part between RAR window and window for on-demand SIB1. |
| Spreadtrum | Support | Proposal 1-1 is a common solution for timeline determination. |
| Samsung | Support | Rather than designing a different mechanism, it is good to count on what current spec supports for OSI. |
| Ericsson |  | This is a minor issue, but among the two options, starting slot is a better option as it can reduce the access latency, in particular for UEs that can monitor for both RAR and SIB1 simultaneously. |
| Huawei& Hisilcon 1 | Support | However, we think a clarification is still needed. Right now, in the current spec, the RAR window ends once the UE receive the RAR. This current behaviour could lead to ambiguity in the reference time. |
| Qualcomm | Support |  |
| Sharp | Support | We agree with FL that both options works, then we slightly prefer option 2 to simply avoid the invalid case. |
| Panasonic | No | Option 1 has clearer merits of lower latency and network flexibility. We need to understand better the technical merits of Option 2. |
| Apple | support |  |
| LG |  | We are fine with either option. In addition, we can make the two options the same in terms of access latency by allowing Option 2 to configure/indicate a negative offset value. |
| Xiaomi | Support | Alt 2 is a lean and qualified solution. In particular, it will not introduce additional complexity for UE processing pipeline. |
| NEC | Support |  |
| Nokia/NSB | Not support | First, we need to clarify whether common-Preamble and common-RNTI can be assume for OD-SIB1 operation. Moreover, to our understanding, there can be more than 1 RAR transmitted by the gNB within the RAR window for UE RAR monitoring. Thus, using “ending slot” clearly introduce extra delay for UE to start monitoring OD-SIB1, as the scenario shown in below figure, where the start time of the OD-SIB1 window can be the same as the UE RAR window, (meaning that the starting time of RAR window is used as reference time for the OD-SIB1 window), and the NW gNB can decide by implementation when it starts to transmit the OD-SIB1 within the OD-SIB1window. As an example shown in below in the figure, the gNB starts to transmit the OD-SIB1 after the transmission of the 1st RAR. @FL, if time allows, I can briefly introduce a bit details of this figure during offline discussion if feasible. Thanks! |
| Futurewei | Not support | The discussion of the reference point for SIB1 should be based on UE behavior. If after transmitting WUS request for OD-SIB the UE starts monitoring SIB1, there is no need to wait until the end of RAR window. |
| ETRI |  | Agree with Ericsson. However, we don’t have a strong preference and are also okay with the proposal. |
| Fraunhofer | OK | Same view as Ericsson and ETRI |
| Fujitsu | Support | The definition of ending slot needs to be clarified. Based on the existing spec, a UE may stop RAR window after successful reception of a RAR. In this case, it needs to be clarified which of the following two alternatives the ending slot refers to: - The ending slot of the RAR window is the slot when the UE stops the RAR window - The ending slot of the RAR window is N slots after the start of the RAR window, where N is provided by ra-ResponseWindow in the UL WUS configuration |
| Lenovo |  | Agree with FL’s understanding that both are workable. Option 1 is still preferred because of the benefit on latency. |
| DCM | No | We see no advantage to Option 2 over Option 1. |
| vivo |  | We prefer Option 1 since it clearly have latency benefit compared to Option 2. |
| Tejas | Support |  |
| CATT | OK |  |
| Moderator |  | Strong concern: Nokia, has figure to explain Need clarification on the ending slot as in legacy spec UE can end the RAR window after receiving RAR: Huawei, Fujitsu, Nokia Can be discussed during offline/online session. |
| China Telecom | Support | Option 2 is more straightforward and won’t increase the complexity for UE to detect the RAR and OD-SIB1 and the same time. |

- FL Proposal 1-2

The reference time point to determine the window starting time for on-demand SIB1 is based on the ending slot of the RAR window for UL WUS wherein UE successfully received a RAR.

- The ending slot of the RAR window is N slots after the start of the RAR window, where N is provided by ra-ResponseWindow in the UL WUS configuration.

- (Closed) Issue 2: Starting time of the time window for on-demand SIB1

Background

In RAN1 #118, the following are agreed:

Agreement

At least for Case-2: For further study of type 0 PDCCH monitoring occasions for on demand SIB1, on the starting time and duration of the time window of type 0 PDCCH monitoring occasions, RAN1 to down select from the following two options:

- Option 1: starting time and duration are indicated in RAR of the UL-WUS transmission

- Option 2: starting time and duration are indicated in the UL WUS configuration

In RAN1 #119, the following are agreed:

Agreement

The duration of the time window for on-demand SIB1 is indicated in the UL WUS configuration.

- Unit of the duration is in slot

- FFS: Starting time

Figure 2-1: Definition of time window for type 0 PDCCH monitoring. The NES cell indicates the time offset XUE1 to UE1 and time offset XUE2 to UE2 to coordinate the time window. [31, Ericsson]

Fig 2-2: Example of on-demand SIB1 monitoring window [29, Qualcomm]

For starting time (time offset) of the time window for on-demand SIB1, companies’ views in RAN1 #120 can be summarized below:

- Option 1: Indicated in RAR

  - Apple, LG, Ericsson, OPPO, Panasonic, Sony, NEC, InterDigital

- Option 2: Indicated in UL WUS configuration

  - MTK, Lenovo, LG, Transsion, DENSO, DOCOMO, CEWIT, Google, ZTE, Spreadtrum/UNISOC, CATT, China Telecom, Tejas,

- Option 3: Predefined in spec considering UE processing time

  - Samsung, Qualcomm, CMCC, Nokia, Fujitsu, Xiaomi

For how to indicate the starting time (time offset), this has been discussed for multiple meetings and the situation and companies’ stands seem to remain the same. Moderator suggests trying to follow the RAN1 #118 agreement and majority view.

- FL Proposal 2-1

The time offset between the reference time point and the starting time for the time window of on-demand SIB1 is indicated in the UL WUS configuration.

- Unit of the time offset is in slot

| Moderator |  | Strong concern (should be in RAR): Interdigital, Ericsson, Sony, Apple Strong concern (should use predefined window): Samsung, Huawei, Nokia, Futurewei - Starting time of OD-SIB1 window is the same as the RAR window (offset=0) or UL transmission time Can be discussed during offline/online session. |
| --- | --- | --- |

| Company | Support or not | Comment |
| --- | --- | --- |
| CMCC |  | Fine with this proposal but we think the offset can also be predefined. |
| InterDigital | No | Our preference is for the start time offset to not to be indicated in the UL WUS configuration as it does not benefit both from NW and UE perspectives. From NW perspective, having the flexibility to dynamically update the starting time for OD-SIB1 is an important energy savings feature at the NES cell since the same OD-SIB1 transmission can be used for multiple UEs. From UE perspective, providing the start time offset in the UL WUS configuration can incur unnecessarily longer monitoring PDCCH duration to account for delayed transmission of OD-SIB1. These issues can be avoided by providing the start time offset in the RAR. |
| Google |  | OK |
| ZTE, Sanchips | Support | Based on the processing during last meeting, we are likely to go with option 2. Moreover, option 1 may show NES benefit by not extending the window for on-demand SIB1, but that requires prior information of receiving UL WUS from multiple UEs, we think it may be not practical. |
| Spreadtrum | Support |  |
| Ericsson | Not support | Same view as InterDigital. Option 1 (RAR) is more flexible, it can provide coordination benefits among UEs. Having the offset in the UL WUS means that there it need to be synchronized between network nodes, which adds unnecessary complexity. |
| Samsung | Not | From our understanding, the necessity of starting time is to consider RAR processing time. Still, we think pre-determined value is enough as legacy PDSCH processing time. |
| Sony | Not support | We prefer Option 1 (Indicated in RAR). To our understanding, the difference between Option 1 (Indicated in RAR) and Option 2/3 (Indicated in UL WUS configuration / Predefined in spec) lies in the flexibility of the time gap between the reference time point and the starting time of the window. In Option 1, gNB can indicate different time gaps to each UE that transmitted UL WUS after receiving each SIB1 transmission request. If the gNB coordinates the starting time of the monitoring window for each UE, the gNB can efficiently transmit on-demand SIB1 at the same transmission occasion to each UE, and each UE can receive the on-demand SIB1 without unnecessary extension of the SIB1 monitoring window. |
| Huawei& Hisilcon 1 | Not | The unit of time offset and its value can be FFS with the aim to be discussed with RRC parameters discussion. We do not see any gain or need for the UE to be flexibility configured with start and duration of the window once the reference time is known. Especially if the reference time is the end of the RAR window which is already configurable reference time (from 1ms to 10ms). Hence, we are not convinced by this proposal. The time offset can be predefined in the spec to reduce the UL WUS configuration size. |
| Qualcomm |  | We prefer to fix it in the spec. |
| Sharp |  | We are OK to align with the indication of duration. |
| Panasonic |  | In our understanding, the signalling design should allow network start to transmit OD-SIB1 more flexibility, considering the ongoing traffic to be scheduled and resource availability. By UL WUS configuration, we see such limitation. |
| CEWiT | Support |  |
| Apple | No | If the offset is a fixed value for all UEs, we do not see any benefits to indicate it in the UL WUS configuration, rather, Option 3 is the way to go. However, we believe that including it in the RAR would provide NW enough flexibility for SIB1 transmission and also benefit for UE power, therefore, our preference is Option 1. |
| LG | Support | Support indicating the starting time of the time window for on-demand SIB1 by the UL WUS configuration, and additionally support indicating that information by RAR. |
| Xiaomi | Yes | We can live with the current proposal. |
| NEC |  | We agree with the intention to have configurability in RRC for this, but the phrasing needs to be carefully selected so that it doesn’t lead to any complications later. For example, we do not know how SIB1 will be transmitted within the time window, does the first CORESET0 monitoring occasion occur at the start of time window or can it be flexibly allocated? The answer to these questions can give us an idea that how this time offset should be defined and agreed during the meeting. |
| Nokia/NSB | Not support | As explained in above in issue-1, the starting time of OD-SIB1 window is the same as the RAR window. There is no need of introducing “offset” for the starting time of OD-SIB1. |
| Futurewei |  | We have a similar preference as Nokia that the SIB1 window starts as soon the WUS is transmitted. The SIB1 window duration is expected to be larger than RAR window. |
| ETRI | OK | Either Option 2 or Option 3 is fine. |
| Fraunhofer | Not | Agree with Apple. Option 1 or Option 3 are better. |
| Fujitsu |  | Our first preference is using a predefined value for the time offset, but we can compromise to this proposal if the majority support it. |
| Lenovo | Support | Indicating in RAR means high standard impact. |
| DCM | Support |  |
| vivo | Support | We are not OK with Option 1 since it will make RAR more complexed without clear benefit. |
| Tejas | Support |  |
| CATT | OK |  |
| Moderator |  | Strong concern (should be in RAR): Interdigital, Ericsson, Sony, Apple Strong concern (should use predefined window): Samsung, Huawei, Nokia, Futurewei - Starting time of OD-SIB1 window is the same as the RAR window (offset=0) or UL transmission time Can be discussed during offline/online session. |
| China Telecom | Support | Since the starting time of OD-SIB1 is an offset to UL RAR window, we think the NES benefits of Option1 is rather small compared with Option 2. |
| CEWiT | Support | Indicating in RAR means high standard impact, since it will make RAR more complex. |

- (Closed) Issue 3: Whether/how to support transmission for on-demand SIB1 with the association of SSB(s) based on a received UL-WUS

Background

On whether/how to support transmission of on-demand SIB1 with the association with SSB(s) based on a received UL-WUS in R19, companies views in RAN1 #120 are collected below:

- Yes:

  - vivo, CMCC (additional beams can be configured via UL WUS configuration),

CEWiT, Ericsson (additional beams can be indicated in RAR),

Transsion, LG, [Qualcomm],

- No (follow legacy on-demand OSI):

  - CATT (follow legacy on-demand OSI that PDCCH for an SI message is transmitted in at least one PDCCH monitoring occasion corresponding to each transmitted SSB),

MTK, Fujitsu (no NES gain, decrease the probability of successful SIB1 reception),

DENSO, Huawei/HiSilicon, ZTE, Tejas,

- Up to NW implementation:

  - Sharp

For this topic, it has been discussed for multiple meetings and the situation and companies’ stands seem to remain the same.

Moderator tends to think Fujitsu made good points that:

- If UE’s assumption regarding PDCCH for OD-SIB1 is based on the transmitted UL WUS, it may decrease the probability of successful SIB1 reception.

- Besides, restricting transmitted beams of on-demand SIB1 does not result in network energy saving from a system perspective.

- TS 38.331 clause 5.2.2.3.2

“The UE assumes that, in the SI window, PDCCH for an SI message is transmitted in at least one PDCCH monitoring occasion corresponding to each transmitted SSB and thus the selection of SSB for the reception SI messages is up to UE implementation.”

and hence wants to try the following with slight majority:

- FL Proposal 3-1

After transmitting a UL-WUS, UE assumes that PDCCH for an on-demand SIB1 is transmitted in at least one PDCCH monitoring occasion corresponding to each transmitted SSB (same as legacy on-demand OSI).

| Moderator |  | Strong concern: Samsung, Ericsson, Sharp, vivo Potential outcome: RAN1 has no consensus to support the following: - The UE assumes that, in the OD-SIB1 window, PDCCH for an OD-SIB1 message is transmitted in PDCCH monitoring occasions corresponding to at least the SSB associated with the PRACH for UL-WUS. Note: In legacy, the UE assumes that, in the SI window, PDCCH for an SI message is transmitted in at least one PDCCH monitoring occasion corresponding to each transmitted SSB. Can be discussed during offline/online session. |
| --- | --- | --- |

| Company | Support or not | Comment |
| --- | --- | --- |
| CMCC |  | We support that PDCCH for an on-demand SIB1 is transmitted in at least one PDCCH monitoring occasion. For the corresponding SSB(s), our understanding is that if OD-SIB1 corresponding to each transmitted SSB is transmitted, NES gain might be limited or even negative when the request rate for OD-SIB1 is relatively large. Therefore, we suggest that the OD-SIB1 can be transmitted only with the beam direction corresponding with UL WUS, or the actual transmitted OD-SIB1 beams can be configured by gNB (e.g. in WUS configuration). |
| InterDigital | Support |  |
| Google |  | We think spec only needs to define where to monitor OD-SIB1. We suggest the following proposal: After transmitting a UL-WUS, UE monitors that PDCCH for an on-demand SIB1 in the PDCCH monitoring occasion(s) corresponding to the SSB associated with the PRACH for UL-WUS. |
| ZTE, Sanechips | Support | It’s simple to be the same as legacy OSI. |
| Spreadtrum | Support | Transmitting OD-SIB1 corresponding to each transmitted SSB is beneficial for UEs who need OD-SIB1 but don’t send UL WUS yet. |
| Samsung | Not support | In our point of view, leaving as implementation would be enough. We are still wondering on the necessity of proposal. In addition, based on our observation during evaluation, OD-SIB1 transmission related to SSB corresponding to UL WUS has provided better NES gain. Regarding the decrease the probability of successful SIB1 reception from Fujitsu, it would be good discussion point, but we are not sure that transmitting OD-SIB1 on PDCCH occasion corresponding to each transmitted SSB always provides better performance. In addition, we think it can be trade-off to get better NES gain. |
| Ericsson | Not support | Network energy savings can be achieved by restricting OD-SIB1 to SSBs associated with RO of UL WUS. This was found during the SI, where almost all companies simulated under the assumption that OD-SIB1 is provided in only one SSB. Regarding the reduced probability of successful SIB1 decoding –there is no simulation evidence, we expect that this effect is insignificant. And in the worst case that the UE receives RAR but fails to decode SIB1, then it only needs to send UL WUS again. That’s a better option than gNB always sending OD-SIB1 in all SSBs. |
| Sony | Support |  |
| Huawei& Hisilcon 1 | yes | We support this proposal. |
| Qualcomm | Support |  |
| Sharp | Not support | Comparing with OD-OSI case, we need to consider more energy saving which means unnecessary SIB1 transmission should be avoided unless UE’s request. Regarding the NES gain, it depends how often the SIB1 is requested by NES UE’s in NES cell. We believe simultaneous requests from multiple UEs is not major case, and single OD-SIB1 transmission with limited SSBs contributes energy saving per the request. |
| Panasonic |  | We may understand the intention if it means UE expect a full set of beams for OD-SIB1. We may not need the condition “After transmitting a UL-WUS”, as gNB is not always obliged to transmit OD-SIB1 after being triggered. |
| CEWiT | Not support | Restricting OD-SIB1 to SSBs associated with the UL WUS provide significant network energy savings as shown during the SI, where most companies simulated scenarios assuming OD-SIB1 was provided through a single SSB. Further, how the QCL relation will be assumed by a UE between WUS and SIB1 if associated with different SSBs. |
| Apple |  | OK with the proposal. |
| LG | Not support | Support transmission of on-demand SIB1 with the association with SSB(s) based on a received UL-WUS in R19. It is more beneficial in terms of gNB’s energy saving since gNB doesn’t need to transmit SIB1 for all beams. |
| Xiaomi | Support | Legacy mechanism works well for OD-SIB1. The procedure of OD-SIB1 and the procedure of OD-SI are same. We don’t see necessity to define something different from current mechanisms. |
| NEC | Support |  |
| Nokia/NSB | Not support | What we have observed from the simulation results during study phase is that, network energy saving gain is obtained when NW transmits OD-SIB1 that is associated with the UL-WUS transmission. |
| Futurewei |  | We think that OD-SIB1 transmission should be associated with UL-WUS transmission opportunities for energy saving purposes. However, if the OD-SIB1 transmit window is limited, we could be OK with monitoring all the OD-SIB1 opportunities (to be defined) in the transmission window. |
| ETRI | Not support | From RAN1 perspective, we do not think this proposal is needed. Leaving it to gNB implementation is enough. If a similar behaviour is already captured in RAN2 specification, RAN2 can do the same thing for the OD-SIB1 if needed. |
| Fraunhofer | Not support | Unnecessary SIB1 transmissions should be avoided for NES gains. |
| Fujitsu | Support |  |
| DCM | Not support | It is not desirable to define in spec that UE expects all SIB1 beams should be transmitted corresponding to ssb beams. |
| vivo | Not support | Mandating gNB to transmit SIB1 corresponding to each SSB is not good for NES as mentioned by several companies already. We suggest the following proposal: After transmitting a UL-WUS, UE assumes that PDCCH for an on-demand SIB1 is transmitted in at least one PDCCH monitoring occasion corresponding to the SSB associated with the UL-WUS. Whether on-demand SIB1(s) is transmitted or not in at least one PDCCH monitoring occasion corresponding to other SSB(s) is up to implementation. |
| Tejas | Support |  |
| CATT | Support | If no consensus, follow legacy which is the current wording in the spec |
| Moderator |  | Strong concern: Samsung, Ericsson, Sharp, vivo Potential outcome: RAN1 has no consensus to support the following: - The UE assumes that, in the OD-SIB1 window, PDCCH for an OD-SIB1 message is transmitted in at least one PDCCH monitoring occasion corresponding to the SSB associated with the PRACH for UL-WUS. Note: In legacy, the UE assumes that, in the SI window, PDCCH for an SI message is transmitted in at least one PDCCH monitoring occasion corresponding to each transmitted SSB. Can be discussed during offline/online session. |
| China Telecom | Support |  |
| OPPO | Support | if this proposal is not supported, it implies that the network can implement whatever it wants. In this case, there is no longer needed to set the time window for SIB1 reception as the network may also not transmit anything in the time window. |
| Transsion | Support |  |

- FL Proposal 3-1-2

The UE assumes that, in the OD-SIB1 window, PDCCH for an OD-SIB1 message is transmitted in at least one PDCCH monitoring occasion corresponding to the SSB associated with the PRACH for UL-WUS.

- Note: In legacy, the UE assumes that, in the SI window, PDCCH for an SI message is transmitted in at least one PDCCH monitoring occasion corresponding to each transmitted SSB.

| Company | Support or not | Comment |
| --- | --- | --- |
| Ericsson | Support | It is unreasonable to request e.g. an FR2 NES cell with 64 SSB to sweep SIB1 in all directions when there is just one UE in one beam requesting SIB1, that will waste a lot of energy in contradiction to the WI objective. Up to gNB implementation, SIB1 can also be transmitted in additional PDCCH monitoring occasions associated with other SSBs. Hence, if issues with SIB1 reception probability are observed, the gNB can adapt its transmission behaviour. |
| CEWiT | Support | We support the tranmissions corresponding only to the SSB associated with the PRACH for UL-WUS. Further, the note is not needed and can be removed. Its uncertain whether transmitting OD-SIB1 on the PDCCH occasion corresponding to each transmitted SSB consistently improves performance since this case may not always occurs where a UE receives RAR but not SIB1. |

- FL Proposal 3-1-3 (with text update from LG during offline discussion)

The UE assumes that, in the OD-SIB1 window, PDCCH for an OD-SIB1 message is transmitted in at least PDCCH monitoring occasions corresponding to at least the SSB associated with the PRACH for UL-WUS.

- Note: In legacy, the UE assumes that, in the SI window, PDCCH for an SI message is transmitted in at least one PDCCH monitoring occasion corresponding to each transmitted SSB.

- FL Proposal 3-1-4

RAN1 has no consensus to support the following:

- The UE assumes that, in the OD-SIB1 window, PDCCH for an OD-SIB1 message is transmitted in PDCCH monitoring occasions corresponding to at least the SSB associated with the PRACH for UL-WUS.

Note: In legacy, the UE assumes that, in the SI window, PDCCH for an SI message is transmitted in at least one PDCCH monitoring occasion corresponding to each transmitted SSB.

- (Closed) Issue 4: How to use PDCCH-ConfigSIB1 in MIB of NES Cell

Background

In RAN1 #118, the following are agreed:

Agreement

For type 0 PDCCH monitoring occasions of on demand SIB1, searchSpaceZero and controlResourceSetZero for on-demand SIB1 are provided from UL WUS configuration if SSB on NES cell is on sync raster and K_SSB is not equal to 30 in FR1 or 14 in FR2.

In RAN1 #119, the following is agreed:

Agreement

On how to use PDCCH-ConfigSIB1 for R19 NES-capable UE in MIB of NES Cell when K_SSB = 30 in FR1 or K_SSB = 14 in FR2 on NES cell if SSB of NES cell is on the sync raster, down-select from the following options:

- Option 1: Use it to indicate frequency assistance information to search SSB for Cell A

  - FFS: Details on the range/granularity of the frequency assistance information

  - FFS: Potential impact to RAN3

- Option 2: Use it to indicate searchSpaceZero and controlResourceSetZero of OD-SIB1

- Option 3: Above feature is not supported.

Companies’ view in RAN1 #120 are collected below:

On how to use PDCCH-ConfigSIB1 in MIB of NES Cell when K_SSB = 30 in FR1 or K_SSB = 14 in FR2 on NES cell

- Option 1: Use it to indicate frequency assistance information to search SSB for Cell A

  - Apple, Samsung, Lenovo, LG, Transsion, Sharp, Ericsson, Huawei/HiSilicon, OPPO, Nokia, InterDigital, Sony

- Option 2: Use it to indicate searchSpaceZero and controlResourceSetZero of OD-SIB1

  - Lenovo, ETRI, MTK, DOCOMO, [CAICT], Spreadtrum/UNISOC, CMCC, vivo, NEC, Transsion

- Option 3: Above feature is not supported

  - ETRI, ZTE, CAICT, Fraunhofer, FUTUREWEI, ZTE, China Telecom, Tejajs, Panasonic, Xiaomi

For type 0 PDCCH monitoring occasions of on demand SIB1, searchSpaceZero and controlResourceSetZero for on-demand SIB1 are provided from UL WUS configuration if SSB on NES cell is on sync raster and K_SSB is equal to 30 in FR1 or 14 in FR2.

- Combined with RAN1 #118 agreement  regardless of K_SSB value

- Qualcomm, Sharp, Huawei/HiSilicon, China Telecom, Nokia

This topic has been discussed multiple meetings and moderator suggests trying the following with slight majority:

- FL Proposal 4-1

For type 0 PDCCH monitoring occasions of on demand SIB1, searchSpaceZero and controlResourceSetZero for on-demand SIB1 are provided from UL WUS configuration if SSB on NES cell is on sync raster, regardless of K_SSB value.

| Moderator |  | Concern: CMCC, NEC, vivo May try the following suggested by Huawei/Qualcomm/Lenovo: - For type 0 PDCCH monitoring occasions of on demand SIB1, searchSpaceZero and controlResourceSetZero for on-demand SIB1 are provided from UL WUS configuration if SSB on NES cell is on sync raster, regardless of K_SSB value carried in the SSB on NES cell. |
| --- | --- | --- |

| Company | Support or not | Comment |
| --- | --- | --- |
| CMCC |  | For the case that Kssb = 30 in FR1 and 14 in FR2, K_SSB = 30/14, our understanding is that since the bit field for PDCCH-ConfigSIB1 is reserved according to TS 38.213, it is recommended that the corresponding bit field can be used to indicate searchSpaceZero and controlResourceSetZero of OD-SIB1 for R19 NES-capable UE, which can reduce the overhead for WUS configuration and therefore increase the overall NES gain. |
| InterDigital | Support |  |
| Google |  | OK |
| ZTE, Sanechips | Support | For the discussion purpose, benefits and details from option 1 and option 2 can be provided by proponents, then people understand whether or not it is essential or worth to have these enhancements. |
| Samsung | Support |  |
| Ericsson | Support |  |
| Sony | Support |  |
| Huawei& Hisilcon 1 | Support | To remove ambiguity since we have K_ssb value configured in WUS configuration and we have restrictions on the values of SSBs on NES cell once they are on sync raster. We propose this addition in blue For type 0 PDCCH monitoring occasions of on demand SIB1, searchSpaceZero and controlResourceSetZero for on-demand SIB1 are provided from UL WUS configuration if SSB on NES cell is on sync raster, regardless of K_SSB value carried in the SSB on NES cell. |
| Qualcomm |  | Same view as Huawei. |
| Sharp | Support |  |
| Panasonic | Support |  |
| Apple | Support |  |
| LG | Support but | We wonder whether K_SSB value smaller than 24 in FR1 or 12 in FR2 is included or not in this proposal. |
| Xiaomi | Support |  |
| NEC | Not support | We prefer the provision of searchSpaceZero and controlResourceSetZero for on-demand SIB1 using MIB when K_SSB = 30 in FR1 or K_SSB = 14 in FR2. This will allow reduction in WUS configuration overhead. |
| Nokia/NSB | Support |  |
| Futurewei | OK |  |
| ETRI | OK | Although we do not see the necessity with this K_SSB value range, we are okay with the proposal for consistency with the previous agreement. |
| Fujitsu | Support |  |
| Lenovo | Support | Huawei’s revision is supported. |
| DCM | OK |  |
| vivo | Not support | Option 2 can reduce WUS configuration overhead. Besides, when a UE is camping on a NES cell, option 2 provide a way for gNB to update searchSpaceZero and controlResourceSetZero if needed. |
| Tejas | Support |  |
| Moderator |  | Concern: CMCC, NEC, vivo May try the following suggested by Huawei/Qualcomm/Lenovo: - For type 0 PDCCH monitoring occasions of on demand SIB1, searchSpaceZero and controlResourceSetZero for on-demand SIB1 are provided from UL WUS configuration if SSB on NES cell is on sync raster, regardless of K_SSB value carried in the SSB on NES cell. |
| China Telecom | Support |  |
| Transsion | Support |  |
| CEWiT | Support |  |

- FL Proposal 4-1-1

For type 0 PDCCH monitoring occasions of on demand SIB1, searchSpaceZero and controlResourceSetZero for on-demand SIB1 are provided from UL WUS configuration if SSB on NES cell is on sync raster, regardless of K_SSB value carried in the SSB on NES cell.

- FL Proposal 4-2

On how to use PDCCH-ConfigSIB1 for R19 NES-capable UE in MIB of NES Cell when K_SSB = 30 in FR1 or K_SSB = 14 in FR2 on NES cell if SSB of NES cell is on the sync raster, adopt the following option:

- Option 1: Use it to indicate frequency assistance information to search SSB for Cell A

  - FFS: Details on the range/granularity of the frequency assistance information

| Moderator |  | Concern on Option 1: Spreadtrum, Panasonic, NEC, Fraunhofer, Fujitsu, vivo, CATT, Xiaomi, ZTE - NEC: Benefits not clear - Vivo: Benefits not clear, SSB in the range of 24~29 can already give indication of Cell A’s SSB. Complicated details on the range/granularity with limited time left.` - Fujitsu: Legacy UE can also use K_SSB = 30/14 in FR1/FR2, which may create ambiguity - Xiaomi: Marginal benefits with big spec impact - Huawei/Ericsson: The benefits of assistance on finding Cell A is easy to be seen. - LG: Before UE finds Cell A, UE can obtain already some information on Cell A by NES cell - Samsung: Legacy assistance frequency information on finding the CD-SSB is limited in frequency range, but NES cell and Cell A may be far away in frequency, which may need a new design. - Qualcomm: Potential RAN4 impact on designing the new freq range Can try Huawei’s version below: On how to use PDCCH-ConfigSIB1 for R19 NES-capable UE in MIB of NES Cell when K_SSB = 30 in FR1 or K_SSB = 14 in FR2 on NES cell if SSB of NES cell is on the sync raster, adopt the following option: - Option 1: Use it to indicate frequency assistance information to search SSB for a Cell A - FFS: Details on the range/granularity of the frequency assistance information If not agreeable, moderator would suggest to go with Option 3: - Option 3: Above feature is not supported Can be discussed during offline/online session. Concern on Option 2: Ericsson, Xiaomi, Samsung, LG, Nokia, Sharp, InterDigital, Fraunhofer, Tejas |
| --- | --- | --- |

| Company | Support or not | Comment |
| --- | --- | --- |
| CMCC |  | We think this should be further discussed, please see our reply in proposal 4-1. |
| InterDigital | Support |  |
| Google |  | OK |
| ZTE, Sanechips |  | The FFS part and details should be clarified firstly, then companies know the scenarios and benefits if any. |
| Spreadtrum | Not | If K-SSB is not to indicate searchSpaceZero and controlResourceSetZero of OD-SIB1, it can be reserved value as legacy spec. And this issue can be discussed later after Proposal 4-1 is determined. |
| Samsung | Support |  |
| Ericsson | Support | For the UE, to find Cell A quickly is important and a bottleneck in the procedure to acquire OD-SIB1. Any enhancements that can simplify that are good. |
| Sony | Support |  |
| Huawei& Hisilcon 1 | Support | Minor correction in blue On how to use PDCCH-ConfigSIB1 for R19 NES-capable UE in MIB of NES Cell when K_SSB = 30 in FR1 or K_SSB = 14 in FR2 on NES cell if SSB of NES cell is on the sync raster, adopt the following option: - Option 1: Use it to indicate frequency assistance information to search SSB for a Cell A - FFS: Details on the range/granularity of the frequency assistance information |
| Sharp | Support | We believe the assistance information contributes to decrease the number of search for SSB of Cell A. |
| Panasonic | No | We think it is redundant feature, as UE anyway starts from cell searching, during which Cell A should be identifiable if the channel condition is sufficiently good. If Cell A channel condition is not sufficiently good, procedure of Option 1 will not be helpful anyway. |
| Apple | Support |  |
| LG | Support | If a UE detects a first SS/PBCH block and determines that a CORESET for Type0-PDCCH CSS set is not present, and for k_SSB=30 for FR1 or for k_SSB=14 for FR2, the UE may determine the Cell A’s global synchronization channel number (GSCN) of a SSB set as NGSCNReference+ K⋅NGSCNOffset. NGSCNReference is the GSCN of NES Cell’s SSB, NGSCNOffset value corresponding to a code-point for “16×controlResourceSetZero+searchSpaceZero” (as shown in Table 1 below), and K value can be any positive integer value greater than or equal to 1 which can be predefined or configured from the Cell A. Table 1. Example of NGSCNOffset values for k_SSB=30 for FR1 or k_SSB=14 for FR2 kSSB; 16×controlResourceSetZero +searchSpaceZero; NGSCNOffset. 30 for FR1 or 14 for FR2; 0, 1, …, 255; -127, -126, …, 127, 128. |
| Xiaomi |  | It is not critical issue. To keep interpretation of PDCCH-ConfigSIB1 when K_SSB=30 for FR1 and K_SSB=14 for FR2 as legacy works well. The way of proposal 4-2 may bring significant specification impacts, which is not preferred. |
| NEC | Not Support | We prefer the provision of searchSpaceZero and controlResourceSetZero for on-demand SIB1 using MIB when K_SSB = 30 in FR1 or K_SSB = 14 in FR2. This will allow reduction in WUS configuration overhead. |
| Nokia/NSB | Support |  |
| Futurewei |  | We could be OK if the majority wants, even it may add additional complexity. For instance, what happens if Cell A is not in the frequency range specified by these numbers? What is UE behaviour? Can UE find a Cell A farther in frequency? |
| ETRI |  | Different from the case of K_SSB = 24~29 for FR1 and 12~14 for FR2, when K_SSB = 30 in FR1 or K_SSB = 14 in FR2, a new GSCN table may need to be defined. Further discussion may be needed. |
| Fraunhofer | Not | Same view as Panasonic. And we think the benefits are minimal as indicated by ZTE. |
| Fujitsu |  | The necessity of this option needs to be clarified first. |
| Lenovo | Support |  |
| DCM |  | We think it is safer and better to postpone this discussion until RAN1 decides if/how to indicate OD-SIB1 TX status, as the number of usable bits in the MIB/PBCH is very limited. |
| vivo | Not support | For the case of K_SSB=24-29 for FR1 and 12-14 for FR2, it can already indicate a CD-SSB with SIB1 transmission. What’s the additional benefit compared to this? |
| Moderator |  | Concern: Spreadtrum, Panasonic, NEC, Fraunhofer, Fujitsu, vivo, CATT, Xiaomi, ZTE, China Teleocm Can try Huawei’s version below: On how to use PDCCH-ConfigSIB1 for R19 NES-capable UE in MIB of NES Cell when K_SSB = 30 in FR1 or K_SSB = 14 in FR2 on NES cell if SSB of NES cell is on the sync raster, adopt the following option: - Option 1: Use it to indicate frequency assistance information to search SSB for a Cell A - FFS: Details on the range/granularity of the frequency assistance information If not agreeable, moderator would suggest to go with Option 3: - Option 3: Above feature is not supported Can be discussed during offline/online session. |
| China Telecom | Not Support | We share similar view as vivo. We don’t see the benefits for Option 1. |
| OPPO |  | if the proposal cannot get agreed, the only wayforward would be option 3. Then the proposal 4-1 should not be objected by any company. |
| Transsion | Support |  |

- FL Proposal 4-2-1

On how to use PDCCH-ConfigSIB1 for R19 NES-capable UE in MIB of NES Cell when K_SSB = 30 in FR1 or K_SSB = 14 in FR2 on NES cell if SSB of NES cell is on the sync raster, Option 3 is adopted.

- Option 1: Use it to indicate frequency assistance information to search SSB for Cell A

  - FFS: Details on the range/granularity of the frequency assistance information

  - FFS: Potential impact to RAN3

- Option 2: Use it to indicate searchSpaceZero and controlResourceSetZero of OD-SIB1

- Option 3: Above feature is not supported.

- (Closed) Issue 5: Whether to support SSB with on-demand SIB1 not on sync raster

Background

On whether to support SSB with on-demand SIB1 not on sync raster, companies’ views in RAN1 #120 are collected below

- Yes:

  - CATT:

    - When SSB transmitted in the NES cell is not on sync raster and if SIB1 is not currently transmitted but can be triggered by UL WUS, the kSSB in PBCH payload of NES cell is set to reserved value, i.e. kSSB=30 for FR1 and kSSB =14 for FR2.

    - When SSB transmitted in the NES cell is not on sync raster and SIB1 is being transmitted, the SSB of NES cell shall be CD-SSB, i.e. kSSB <24 for FR1 and kSSB <12 for FR2.

  - Huawei

    - NES cell is allowed to send SSB off sync raster as a way to bar legacy UEs, in which case the cell A WUS configurations, corresponding to this NES cell, provides the frequency location of these SSBs.

  - OPPO, Sharp

  - DCM, LG

    - searchSpaceZero and controlResourceSetZero for on-demand SIB1 are provided from MIB on NES cell

- No:

  - ETRI, Fraunhofer, InterDigital, DENSO, Ericsson, FUTURWEI

  - Apple (avoid requiring IDLE/INACTIVE UEs measuring on SSBs off-sync-raster),

This topic has been discussed for multiple meetings. Moderator suggests having a last try for support of this case.

- FL Proposal 5-1

It is up to gNB implementation to use SSB on or off the sync raster for NES cell.

- FFS: Details when SSB is off the sync raster for NES cell

| Moderator |  | Concern: InterDigital, Ericsson, ETRI, DCM, vivo, Samsung If not agreeable, potential results would be: - RAN1 has no consensus to support SSB off the sync raster for NES cell. Can be discussed during offline/online session. |
| --- | --- | --- |

| Company | Support or not | Comment |
| --- | --- | --- |
| InterDigital |  | We understand the frequency location deployment for SSBs can be up to NW implementation. However, the impacts for configuring the SSBs of NES cell off sync raster seems to outweigh any benefits, in our view. For example, when the NES cell’s SSB are configured off sync raster, the NES cell itself becomes invisible to any idle/inactive UEs and the SSBs of Cell A should now compensate to ensure reliable coverage for all UEs that may camp on it. Also, in the case when multiple NES cells are associated with a Cell A, it is also not clear how the Cell A can provide a suitable UL WUS configuration to the UE since the NES cell itself is not detectable by the UE. |
| ZTE, Sanechips | Support |  |
| Samsung | Support |  |
| Ericsson | Not support | With SSB off the sync raster, the procedures for inter-cell measurements becomes more complicated: An idle UE cannot just search the synchronization grid for reselection candidates, it has to take additional steps to first acquire UL WUS config, and then search both the synchronization raster and additional frequencies. |
| Huawei& Hisilcon 1 | Support | SSB on sync raster could have the following properties and advantages: - SSB with K_SSB <= 23 for FR1 and K_SSB < 11 for FR2, per RAN1#118bis agreement. - Indicates frequency assistance information to search SSB for a Cell A. If not possible, then SSB off sync raster can be used [can be agreed]. Because in this case SSB on sync raster is wasting UE time and energy. - Legacy Idle/inactive UEs are bared in the current spec. - NES Idle/inactive UEs are bared till they acquire WUS conf. SSB of sync raster could have the following properties and advantages - The value of K_SSB can be fixed to below or above 23 in FR1 and below or above 11 for FR2 [can be agreed]. - Legacy Idle/inactive UEs are bared in the current spec i.e. NES cell will be invisible to these UEs. - NES Idle/inactive UEs are bared till they acquire WUS conf. These two options (i.e., SSB on sync raster and SSB of sync raster) give the gNB the flexibility to adapt to the scenarios and decide when it is suitable to send SSB off sync raster in order not to waist the time and the energy of legacy and new UEs or when to use SSB on sync raster to help legacy and new UEs to find a cell to camp on. |
| Qualcomm |  | We think SSB on sync raster should be sufficient for this feature. Furthermore, if Proposal 4-2 is agreed, it is better to support SSB on sync raster only. |
| Sharp | Support | As mentioned by CATT “When SSB transmitted in the NES cell is not on sync raster and SIB1 is being transmitted, the SSB of NES cell shall be CD-SSB, i.e. kSSB <24 for FR1 and kSSB <12 for FR2.” This design can reduce UL-WUS transmission from UE since if the kSSB can be set to kSSB <24 for FR1 and kSSB <12 for FR2, the UE doesn’t have to transmit UL-WUS. From the NW’s perspective, it doesn’t need to transmit RAR corresponding to the UL-WUS. Thus, both UE and NW can save power. |
| Apple |  | Considering SSB on off-raster requires IDLE/INACTIVE UEs measuring on SSBs off-sync-raster, we prefer to keep it on sync raster. |
| LG | Support |  |
| NEC | Support |  |
| Futurewei | OK |  |
| ETRI | Not support | In our understanding, reception of SSB off sync-raster for camping in idle mode seems a new UE behaviour whose benefit is not clear. It would complicate the UE implementation, and also seems out of scope since the WID says: Note 1: No modification of SSB will be discussed under this objective |
| Fraunhofer | Not support | Agree with Ericsson and Apple. |
| DCM | Not support | RAN2’s assumption is that Rel-19 OD-SIB1 UEs will identify and move to OD-SIB1 cell based on the legacy concept (idle mode RRM measurement and cell re-selection). The legacy concept is based on SSB on sync raster. |
| vivo | Not support | In TS 38.300, it is clearly said that cell reselection is based on CD-SSB on sync raster only as shown below: 9.2.1.2 Cell Reselection A UE in RRC_IDLE performs cell reselection. The principles of the procedure are the following: - Cell reselection is always based on CD-SSBs located on the synchronization raster (see clause 5.2.4). We prefer to keep this as legacy. There is no strong reason to break such basic rules. |
| Moderator |  | Strong concern: InterDigital, Ericsson, ETRI, DCM, vivo If not agreeable, potential results would be: - RAN1 has no consensus to support SSB off the sync raster for NES cell. Can be discussed during offline/online session. |
| China Telecom |  | Support. |
| OPPO | Support | there is a clear majority support for this proposal. |
| Transsion | Not Support | In RAN1#118bis meeting, we reached the following agreement: Agreement（RAN1#118bis） For further work of on-demand SIB1 in idle/inactive mode, it is assumed that: - SSB transmitted in the NES cell supporting OD-SIB1 is with K_SSB > 23 for FR1 and K_SSB >11 for FR2 if it is transmitted on sync raster. - UE determines a cell supporting OD-SIB1 based at least on UL WUS configuration from Cell A. From the agreement, if the SSB transmitted in the NES cell is not on sync raster, the K_SSB <= 23 for FR1 and K_SSB <=11 for FR2. When K_SSB <= 23 for FR1 and K_SSB <=11 for FR2, the SSB should be the CD-SSB on sync raster for cell A or non-NES cell. Therefore, SSB on sync raster is enough for NES cell supporting OD-SIB1. |
| Samsung | Not Support | After checking with our RAN2 colleagues, we figure out that it can bring large spec impact for cell selection. From our understanding, NES cell will be used as PCell for camping and connection for idle/inactive UE. And, current spec supports only SSB on sync raster for PCell selection. If SSB off the sync raster is supported for NES cell, then large spec impact is needed in RAN2 spec. So, we'd like to share our concern and leave as RAN2 discussion. |

- FL Poposal 5-1-2

RAN1 has no consensus to support SSB off the sync raster for NES cell.

- Issue 6: Indication of broadcast status of OD SIB1, and switch between NES cell and Cell A

Background

In RAN2 #127b, the following is agreed

Agreement

A cell for which SIB1 request configuration is available, can periodically broadcast SIB1.

Agreement

If UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.

On whether/how to have indication of broadcast status of OD SIB1, companies’ views in RAN1 #120 are collected below

Samsung:

Proposal 4: RAN1 further studies the UE behavior and how/whether to indicate switching of SIB1 transmission

states between the following states

•  State A: No SIB1 transmission

•  State B: OD-SIB1 transmission within time window

•  State C: Periodic always-on SIB1 transmission as legacy SIB1 operation

ETRI:

Proposal  6:  If  UE  receives  the  UL  WUS  configuration  of  a  NES  cell,  it  reinterprets  the  ssb-

SubcarrierSpacing field in the MIB of the NES cell when K_SSB > 23/11 for FR1/FR2, using it to indicate  the  presence  or  absence  of  SIB1  transmission.  Otherwise,  UE  interprets  the  ssb-SubcarrierSpacing field according to the legacy specification.

Proposal 8: If a Rel-19 OD-SIB1-capable UE identifies a NES cell that is not transmitting regular SIB1, the UE ignores the cellBarred field in the MIB. In other words, the UE is allowed to camp on or access the NES cell, regardless of whether the cellBarred field is set to ‘barred’ or ‘not barred’.

Proposal 10: If the broadcast status of SIB1 is introduced, study the following UE behaviors in RRC

connected mode:

o  When SIB1 is set to ‘broadcasting’, UE releases the UL WUS resource if it was previously

configured

o  When  SIB1  is  set  to  ‘notBroadcasting’,  UE  assumes  that  the  UL WUS  resource  remains

available – either for itself or for other UEs in RRC idle/inactive mode within the same cell.

LG:

Proposal #4:  Before a  UE  transmits  UL  WUS for  a  NES cell,  the  UE  can  start monitoring SIB1

PDCCH on the NES cell.

-  If the UE fails to receive SIB1 PDCCH for a certain duration, it transmits UL WUS. FFS how to define the duration

-  If the UE succeeds to receive SIB1 PDCCH, it expects that SIB1 will be transmitted for a time window. FFS how to define the time window

Proposal #5: After a UE transmits UL WUS for a NES cell and SIB1 PDCCH is detected before the RAR is received, the UE considers OD-SIB1 procedure as successful.

Qualcomm:

Proposal 4: Support indication of whether SIB1 is currently transmitted on-demand or broadcast by a NES cell via SSB.

•  FFS: how to indicate in SSB.

Ericsson:

Proposal 10  If UE has UL WUS configuration for a cell with not CD-SSB, then the UE checks at least one candidate Type0-PDCCH CSS occasion which matches the assistance information for the SIB1 transmission occasions before it can request OD-SIB1.

Proposal 12  The UE should continue to monitor for SIB1 until the next RACH occasion in which it can transmit UL WUS.

CAICT:

Proposal 3:  SI change indication or etwsAndCmasIndication of NES cell and DCI 1_0 on Cell A or NES

cell are first preference to indicate the broadcast status of OD SIB1 when can UE transmit UL-WUS to the

NES cell.

CEWiT:

Proposal 3: Support indicating the transmission status of on-demand SIB1 using spare bit in MIB.

Huawei/HiSilicon

Proposal 9: Do not support SIB1 transmission state indication (e.g. Broadcast or not broadcast) by NES Cell MIB. That is, switching from broadcast SIB1 to not broadcast SIB1 or visa versa has no impact to NES MIB content.

ZTE Corporation, Sanechips

Observation 1: If the OD-SIB1 function is switched off on a NES cell, the SSB on the NES cell contains K_SSB < 24 for FR1 and K_SSB < 12 for FR2.

Observation 2: Checking the NES cell states (whether the SIB1 is being broadcasted) through blind detection may cause a large access delay and increase the UE power consumption.

Observation 3: Using the value of K_SSB to indicate whether the SIB1 on the NES cell is currently broadcasting is beneficial for the UE power consumption and has small specification impact.

Proposal 1: Using the K_SSB to indicate whether or not SIB1 is being broadcasted on the NES cell.

- K_SSB < 24 for FR1 and K_SSB < 12 for FR2 indicates that the on-demand SIB1 function is switched off.

- K_SSB=30 for FR1 and K_SSB =14 for FR2 indicates that the on-demand SIB1 is being broadcast.

- K_SSB > 23 for FR1 (excluding K_SSB =30) and K_SSB >11 for FR2 (excluding K_SSB=14) indicates that the on-demand SIB1 is not broadcast.

Spreadtrum, UNISOC

Proposal 1: For SSB on sync raster case, K_SSB can be used to indicate the state of SIB1, e.g. broadcasting or not.

CATT

Proposal 2: To comply with RAN2 agreement, NES cell needs to indicate whether SIB1 is currently being transmitted (due to UL WUS request or SIB1 update), or it is not currently being transmitted and thus should be requested by UL WUS.

Proposal 3: For on-demand SIB1, the kSSB in PBCH payload of NES cell should support identifying the SIB1 transmission states, i.e. SIB1 is being broadcasted or SIB1 is not being broadcasted thus it can be requested by UL WUS.

Proposal 4: If NES cell transmits SSB on sync raster and the kSSB = 30 for FR1 or kSSB = 14 for FR2, on demand SIB1 is currently being broadcasted.

vivo

Observation 2: It is already clear that UE will check whether NES cell becomes a normal cell where OD-SIB1 function is switched off (i.e. case 1) and no further enhancements is needed.

Observation 3: UE power and NW energy would be highly impacted if no enhancements is designed for checking the temporary broadcast status of SIB1 before UL-WUS transmission.

- For UE, additional UL-WUS transmission is unnecessary if SIB1 is broadcasting.

- For NW, RAR transmission for additional UL-WUS is unnecessary if SIB1 is broadcasting.

Observation 4: Opportunities for blind detection of SIB1 before UL-WUS transmission can be effectively reduced based on network indication.

Proposal 1: RAN1 prefers not to rely on UE blind detection to check if SIB1 is currently being broadcasted from the perspective of both UE power saving and NW energy saving.

Proposal 2: UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell based on network indication.

Proposal 3: PBCH can be used to indicate whether OD-SIB1 is being broadcasted in a time window or not. FFS: Spare bit or kssb in MIB can be considered for broadcasting status indication.

Tejas Networks Limited

Proposal 1:  Before CELL-A becomes NES CELL, only the required parameters (Example: raPreambleStartIndex, K_SSB, SIB1-RequestPeriod etc.,) are sent in the UL-WUS configuration to the UE, rest of the parameters are taken by UE from valid SI itself.

Proposal 3:  Before NES CELL becomes CELL-A or vice versa, details about the SIB1 broadcast status information and the switching time can be indicated in the UL-WUS configuration to the UE.

Proposal 4:  Before CELL-A become NES CELL, legacy UEs under NES CELL should be handover to the other CELLs which transmit SIB1 always periodically.

- FFS: Details on handover

Proposal 5:  When CELL-A becomes NES CELL, NES CELL (i.e., switched from CELL-A) will no more be transmitting the CD-SSB on synchronization raster, which was transmitted by CELL-A before switching.

Proposal 13:  Supporting case 2, i.e., “SIB1 is currently being periodically broadcasted in NES cell due to the UL WUS sent by another UE, and the broadcasted SIB1 is limited to a time window with an indicated duration. The UE needs to check whether SIB1 is broadcasted in the cell before sending UL WUS”.

Proposal 14:  Supporting case 1, i.e., “Semi-statically switch between NES cell and cell A.”

Proposal 15:  Not supporting the indication of OD-SIB1 transmission status using SIBx of CELL-A.

Proposal 16:  Not supporting the indication of OD-SIB1 transmission status using spare bit in MIB, as the MIB periodicity is 80 msec.

Proposal 17:  Not supporting the indication of OD-SIB1 transmission status using the K_SSB in MIB of NES CELL.

Proposal 18:  Repurpose the existing parameter (Example: Half frame index/dmrs-TypeA-Position) in the MIB/PBCH of the NES, to indicate the OD-SIB1 transmission status in the NES CELL. The actual value of the repurposed parameter can be indicated in the UL-WUS.

FFS: List of parameters

Proposal 19:  Use the parameter in the MIB/PBCH (Example: By repurposing the existing parameters in the MIB/PBCH) to indicate the OD-SIB1 transmission status in the NES CELL.

Nokia, Nokia Shanghai Bell

Proposal-3: Having common preamble for OD-SIB1 considered, RAN1 to discuss the procedure for UE to monitor on-demand SIB1 transmission requested by other UEs before UE to transmit UL WUS for on-demand SIB1

Observation-6: Based on TS38.212, UE is not expecting change of MIB payload within one MIB TTI of 80ms.

Proposal-11: RAN1 to consider that the network switching between broadcasting SIB1 and on-demand SIB1 is at least 80ms via MIB indication, meaning that UE expects the same value of K_SSB within 80ms.

Vodafone:

Proposal 1: For single-carrier scenario, when a Cell A switches to become NES Cell, upon receiving UL WUS it should fall back to become Cell A rather than sending OD-SIB1 as NES Cell. Alternatively, the gNB should not switch to NES mode without ensuring that it will not lead to service degradation for the UEs under its coverage.

•  FFS: Discuss how to capture this in specification, e.g. Stage 2 specification (TS 38.300).

Denso:

Proposal 1:  Support the semi-static switch of SIB1 transmission state between NES cell and cell A by SSB (kssb) of the cell.

CAICT:

Proposal 4: If NES cell becomes a normal cell, which means that the OD-SIB1 function is switched off, the indication can be the MIB spare bit of NES cell or the k_SSB=30 for FR1 or k_SSB=14 for FR2 in MIB of NES cell.

OPPO

Proposal 6: RAN1 to discuss the UE behavior for case Cell A switching to NES Cell

Panasonic

Observation 1: As per the endorse way forward in RAN#106, a Cell A can switch between being NES Cell and Cell A.

Proposal 2: For clearer UE behaviour, below possible NES cell states should be further discussed and clarified:

- SSB-only state

- Active but SIB1-less state

- Cell A/Fully active state

Proposal 3: After OD-SIB1 is triggered and transmitted, it needs to be clarified that the OD-SIB1 can be broadcasted periodically without being limited to a time window, which supports that NES Cell becomes a Cell A.

On whether/how to have indication of broadcast status of OD SIB1, companies’ views in RAN1 #120 are summarized below

Support indication of whether SIB1 is currently transmitted on-demand or broadcast by a NES cell

- Yes: ZTE, ETRI, Qualcomm, MTK, CAICT, CEWiT, Huawei/HiSilicon (in RAR, not in MIB), ZTE/Sanechips, Spreadtrum/UNISOC, CATT, Tejas, vivo, Nokia

- No: LG, Ericsson (UE checks at least one candidate Type0-PDCCH CSS occasion from the UL-WUS configuration)

Considering the majority preference, moderator has the following proposal:

- FL Proposal 6-1

Support indication by a NES cell to indicate its SIB1 transmission state among the following states:

- State A: No SIB1 transmission

- State B: OD-SIB1 transmission within a time window

- State C: Periodic always-on SIB1 transmission as legacy SIB1 operation (i.e. same behavior as Cell A)

FFS: How to do the indication.

FFS: Whether/how UE behavior is impacted when a NES cell switches its state from one to another

| Moderator |  | Strong concern: CMCC, Ericsson, Nokia, Lenovo Ambiguity on state C: Huawei No need of switch between state A and B: Fujitsu Up to RAN2: ETRI Possible update (from Samsung): RAN1 to further study the following SIB1 transmission states: - State A: No SIB1 transmission - State B: OD-SIB1 transmission within a time window - State C: Periodic always-on SIB1 transmission as legacy SIB1 operation (i.e. same behavior as Cell A) FFS: Whether/How to do the indication for switching between states. FFS: Whether/how UE behavior is impacted when a NES cell switches its state from one to another Can be discussed during offline/online session. |
| --- | --- | --- |

| Company | Support or not | Comment |
| --- | --- | --- |
| CMCC |  | We do not think an explicit indication for OD-SIB1 transmission status. If an explicit indication of OD-SIB1 is introduced (e.g. in MIB), the following scenario can be considered: - UE receives MIB, MIB indicates OD-SIB1 is transmitted, UE receives SIB1; - UE receives MIB, MIB indicates OD-SIB1 is transmitted, UE falls to receive SIB1; - UE receives MIB, MIB indicates OD-SIB1 is not transmitted If there is no explicit indication for OD-SIB1, UE can monitor PDCCH MO associated with OD-SIB1 to check whether OD-SIB1 is transmitted. Therefore, the following scenario can be considered: - UE monitors PDCCH MO associated with OD-SIB1, UE receives SIB1; - UE monitors PDCCH MO associated with OD-SIB1, UE falls to receive SIB1; Compared with the above scenario: - 1 vs 4: the overhead is [detection in MIB+1 detection in PDCCH] vs [one detection in PDCCH], consider that checking OD-SIB1 transmission status is not a frequent behaviour from UE side, no obvious difference on the overhead is observed. - 2/3 vs 5: [one detection in MIB (+1 detection in PDCCH)] vs [1 detection in PDCCH], consider that checking OD-SIB1 transmission status is not a frequent behaviour from UE side, no obvious difference on the overhead is observed. Based on the above analysis, we do not find the obvious benefit on explicit indication for OD-SIB1 transmission. |
| Google |  | Support |
| ZTE, Sanechips | Support | We support the network indication of SIB1 transmission state. |
| Samsung | Support | As starting point of discussion, we support FL’s proposal. We can see many different views on SIB1 transmission states and solutions to indicate states for clear UE behaviour. Therefore, we’d like to be clear the possible states for easy discussion. |
| Ericsson | Not support | Agree with CMCC. With K_SSB, a cell can indicate periodic always-on SIB1 transmission as legacy (State C) or OD-SIB1 provisioning (State A or State B). State B, OD-SIB1 transmission within a time window, lasts for a very brief period and we think that NW indication is not necessary nor worth the effort. Instead, a UE can check for SIB1 with blind-decoding as in legacy. This can be done quickly and efficiently with NW providing SIB1 periodicity and offset as assistance information. UE checking one candidate Type0-PDCCH occasion would be enough. |
| Huawei& Hisilcon 1 | Not | First, State C is not clear. Is it legacy cell (NES cell become normal cell) or is it an NES cell (NES cell decides of its own to provide OD-SIB1 without receiving a WUS). To remove this ambiguity, we suggest adding a forth state or spitting state C to two sub-cases. Second, we do not see the need of indications of all these states to the UEs by NES cell without clear behaviours to be defined by the UE or clear indication to be agreed upon. For example: We do not support this indication in MIB, however we are open to discuss other indication signalling. |
| Qualcomm | Support |  |
| Samsung2 |  | Sorry for multiple comments. From our perspective, proposal is to define possible scenarios in order to study whether additional indications are needed for each SIB1 transmission states. Hence, we suggest the updated FL’s proposal as below: - Updated FL Proposal 6-1 For the further study of SIB1 transmission states, RAN1 focuses its studies on the following states: Support indication by a NES cell to indicate its SIB1 transmission state among the following states: - State A: No SIB1 transmission - State B: OD-SIB1 transmission within a time window - State C: Periodic always-on SIB1 transmission as legacy SIB1 operation (i.e. same behavior as Cell A) FFS: Whether/How to do the indication for switching between states. FFS: Whether/how UE behavior is impacted when a NES cell switches its state from one to another After clarifying scenarios, whether it is necessary to indicate the switching between states can be further discussed. Also, how to indicate can be further discussed on specific switches, if needed. |
| Sharp |  | We are not sure whether explicit dynamic indication of State B is needed, or NES UEs just try to monitor PDCCH for OD-SIB1. |
| Panasonic | Support |  |
| CEWiT | Support |  |
| Apple | Support |  |
| LG | Not support | We think that the indicating the SIB1 transmission state of NES Cell by UL WUS configuration from Cell A should be a baseline. We do not prefer to indicate state change by using PBCH of the NES Cell. |
| Xiaomi |  | We tend to agree with CMCC and Ericsson. Furthermore, we are confused on case A. Before UE to request OD-SIB1 transmission, there is no SIB1 transmission. It is the basic assumption on OD-SIB1 operation and can be covered by case B. |
| Nokia/NSB |  | How is this explicit indication is signaled from NW to UE? Is it via MIB? If it is MIB, to our understanding, the latency introduced for such status indication updates will be very large. |
| Futurewei |  | We support explicit indication, and open for further discussions of the details. |
| ETRI | For RRC connected UE | At least, providing SIB1 broadcast status to RRC connected UEs may be beneficial. Since the legacy specification already supports configuration of broadcast status of other SIs, the similar mechanism can be applied to SIB1. However, this discussion may be a RAN2 scope. A potential RAN1 issue may include assumption on UL WUS resources in RRC connected mode. |
| Fraunhofer | Not support | Agree with CMCC and Ericsson |
| Fujitsu |  | The indication of switch 1 (switch between state A and state B) is not needed. |
| vivo | Support | Indicating SIB1 transmission state is good to both gNB and UE energy consumption. From UE side, it avoids unnecessary WUS transmission and from NW side, it avoids unnecessary RAR transmission. |
| Moderator |  | Strong concern: CMCC, Ericsson, Nokia Ambiguity on state C: Huawei No need of switch between state A and B: Fujitsu Up to RAN2: ETRI Possible update (from Samsung): RAN1 to further study the following SIB1 transmission states: - State A: No SIB1 transmission - State B: OD-SIB1 transmission within a time window - State C: Periodic always-on SIB1 transmission as legacy SIB1 operation (i.e. same behavior as Cell A) FFS: Whether/How to do the indication for switching between states. FFS: Whether/how UE behavior is impacted when a NES cell switches its state from one to another Can be discussed during offline/online session. |
| OPPO | partially support | we don’t support the status C, which is a normal cell status and can be indicated by legacy way. |
| China Telecom | Not Support. | Agree with CMCC/Ericsson/Xiaomi. We think the motivation of this proposal is for UE who want OD-SIB1 to check whether there are SIB1s transmitted in the NES Cell. For State A and C, it can already be distinguished with K_SSB value as agreed in previous meeting; And for State B, we think there is no difference for UE as long as there are no legacy SIB1 transmitted on the Cell. |

- FL Proposal 6-1-2

RAN1 to further study whether/how UE determines one of the following states.

It’s up to RAN2 to study how UE determines one of the following states.

- State A: No SIB1 transmission

- State B: OD-SIB1 transmission within a time window

- State C1: Periodic on-demand SIB1 transmission as legacy SIB1 operation (i.e. same behavior as Cell A)

- State C2: NES cell falls back to the legacy cell

- State C: Periodic always-on SIB1 transmission as legacy SIB1 operation (i.e. same behavior as Cell A)

FFS: Whether/how UE behavior is impacted when a NES cell switches its state from one to another

Qualcomm: Change RAN2 agreement from “UE check …” to “UE may check …”

| Qualcomm | RAN2 Agreements 7. A cell for which SIB1 request configuration is available, can periodically broadcast SIB1. 8. If UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell. We suggest the following to move forward with this Issue: Proposal 4-2-1a - From RAN1 perspectives, if UE has SIB1 request configuration of a cell, it is up to UE implementation on deciding whether the UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell. - Send LS to RAN2 |
| --- | --- |
| Ericsson | We should separate between the two cases mentioned in the RAN2 agreement: (1) SIB1 is periodically broadcasted (legacy behaviour) and (2) SIB1 is provided on demand. - For (1), the UE should check K_SSB in PBCH. - For (2), if Proposal 3-1-2 is accepted, then we acknowledge that in FR2 it makes little sense that UE should check if OD-SIB1 is provided on demand, in response to the request of another UE. |
| Lenovo | We think a unified approach (as stated in the RAN2 agreement – both always-on and on-demand are intended to be treated in the same way) is reasonable. If we cannot achieve consensus, we think it makes sense to revisit the RAN2 agreement (No 8 quoted by Qualcomm) and leave the detailed way of detecting up to the UE, but without requiring any additional signalling by the gNB Proposal: From RAN1 perspective, if a UE has SIB1 request configuration of a cell, and if the UE has detected a SIB1 transmission within the most recent [20/160 ms] from the cell, the UE shall not request SIB1 for the cell. |

RAN2 #127b Agreement

A cell for which SIB1 request configuration is available, can periodically broadcast SIB1.

RAN2 #127b Agreement

If UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.

- FL Proposal 6-1-3 (From Samsung)

RAN1 to further study whether/how UE determines one of the following states for the NES cell.

- State A: No SIB1 transmission

- State B: OD-SIB1 transmission within a time window

- State C1: Periodic on-demand SIB1 transmission as legacy SIB1 operation (i.e. same behavior as Cell A)

- State C2: NES cell falls back to the legacy cell

- State C: Periodic always-on SIB1 transmission as legacy SIB1 operation (i.e. same behavior as Cell A)

FFS: Whether/how UE behavior is impacted when a NES cell switches its state from one to another

- FL Proposal 6-1-3-2 (Revsied from last online discussion)

RAN1 to further study whether/how UE determines one of the following states for the NES cell.

- State A: No SIB1 transmission

- State B: SIB1 is currently being periodically broadcasted (like legacy UE)

- State C1: SIB1 is provided on demand within a time window (no SIB1 transmission and UE has to sent SIB1 request)

- State C2: SIB1 is provided within a time window (SIB1 is being transmitted due to request from another UE or paging for SI update)

FFS: Whether/how UE behavior is impacted when a NES cell switches its state from one to another

- FL Proposal 6-1-4 (From Apple)

It’s up to RAN2 to study how UE determines the SIB1 transmission states for the NES cell.

- FL Proposal 6-1-5 (From Qualcomm)

From RAN1 perspectives, if UE has SIB1 request configuration of a cell, it is up to UE implementation on deciding whether the UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.

- Send LS to RAN2

- FL Proposal 6-1-6 (From Lenovo)

From RAN1 perspective, if a UE has SIB1 request configuration of a cell, and if the UE has detected a SIB1 transmission within the most recent [20/160 ms] from the cell, the UE shall not request SIB1 for the cell.

- Note: It is up to UE implementation whether to monitor SIB1 before SIB1 request

- Send LS to RAN2

Supported by: LG, Qualcomm, Lenovo

At most a conclusion: Futurewei

- FL Proposal 6-1-7

It’s up to RAN2 to study how UE checks if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.

RAN2 #127b Agreement

A cell for which SIB1 request configuration is available, can periodically broadcast SIB1.

RAN2 #127b Agreement

If UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.

- Issue 7: Contents of the RAR for UL WUS

Background

In RAN1 #118, the following is agreed that UE is expected to receive RAR for UL WUS:

Agreement

RAN1 assumes the UE is expected to receive the RAR responding to the preamble transmission for Msg1-based on-demand SIB1 procedure, as the baseline.

In RAN1 #118b, the following is agreed

Agreement

RAN1 to discuss the contents of RAR in response to UL WUS using the legacy RAR for OSI as a starting point

In RAN1 #119, the following is agreed

Agreement

At least the contents of RAR in response to SI request are included in RAR in response to UL WUS

Companies’ view in RAN1 #120 are collected below.

Huawei/HiSilicon

Proposal 6: The RAR responding to the preamble transmission for Msg1-based on-demand SIB1 procedure should contain at least:

- The status of the transmission of the OD-SIB1 e.g., sib1-BroadcastStatus {broadcast; not broadcast}.

  - If sib1-BroadcastStatus = broadcast, then during the OD-SIB1 time window the UE could monitor type 0 PDCCH monitoring occasions and do not send further WUS requests.

  - If sib1-BroadcastStatus = not broadcast, then during the OD-SIB1 time window the UE does not need to monitor type 0 PDCCH monitoring occasions nor send further WUS requests.

Spreadtrum/UNISOC

Proposal 4: RAPID in RAR in response to UL WUS is enough.

vivo

Proposal 9: The RAR for UL-WUS to request OD-SIB1 should be the same size as legacy RAR for CBRA/CFRA or for OSI to avoid the negative impacts on legacy UEs.

Observation 9: Align the size between the RAR for UL-WUS and the RAR for CFRA/CBRA may result in resource waste.

Google

Proposal 5: Do not support additional content in the RAR for on-demand SIB1.

NEC

Proposal 4: WUS response message from the network shall not include UL grant, timing advance command or Temporary C-RNTI. Discuss the applicability of RAPID depending on whether the WUS response message also includes response for other RACH resources.

Companies’ view in RAN1 #120 on additional contents to RAR in response to UL WUS on top of contents of RAR in response to SI request are summarized below.

- Add status of the transmission of the OD-SIB1: Huawei/HiSilicon

- Do not add anything: Spreadtrum/UNISOC, vivo, Google, [NEC]

Considering majority of companies prefer not to add anything, and vivo mentions a valid point to moderator that

- The RAR for UL-WUS to request OD-SIB1 should be the same size as legacy RAR for CBRA/CFRA or for OSI to avoid negative impacts on legacy UEs while

  - align the size between the RAR for UL-WUS and the RAR for CFRA/CBRA may result in resource waste.

Moderator has the following proposal:

- FL Proposal 7-1

Do not support additional contents to RAR in response to UL WUS on top of contents of RAR in response to SI request.

| Company | Support or not | Comment |
| --- | --- | --- |
| CMCC | Support | Agree with vivo that enhancing RAR for OD-SIB1 may impact legacy UE. |
| InterDigital |  | Given that this proposal may be related to other issues (e.g. proposal 2-1 in issue 2), we suggest postponing discussion on this proposal until progress is made on others. |
| Google | Support |  |
| ZTE, Sanechips | Support | Besides, we are open to FFS Huawei’s proposal 6 in order to prohibit useless UL WUS transmissions from UE side. |
| Spreadtrum | Support |  |
| Samsung | Support |  |
| Huawei& Hisilcon 1 | Not | This discussion is related to the previous one in 6-1. We think RAR can be used to indicate OD-SIB1 transmission (switching between States A and B). Additionally, using no RAR as a NACK from the gNB is not useful for the UEs , send WUS, when the gNB receive their request but decided not to send OD-SIB1 in the next RAR window (s). these UEs will keep ramping up their powers and re-sending WUS. Finally, regarding the point raised by VIVO. In RAN2 spec, the size of RAR MAC subheader is not restricted in the TS38.321. It only has the octet aligned length requirement. Although there is no space to include the si-BroadcastStatus in the current legacy MAC subheader. The si-BroadcastStatus could be introduced with 1 bit indication by extending the MAC subheader octet length. |
| Sharp |  | Although we are basically fine on it, it can be discussed after other points (e.g. broadcasting status) are clarified. |
| Apple | No | We think we can discussion the other proposal first, e.g., Proposal 2-1. |
| LG |  | Share the view with InterDigital |
| Xiaomi | Support |  |
| NEC | Support |  |
| Futurewei | OK |  |
| ETRI | Support |  |
| Fujitsu | Support |  |
| DCM | Support |  |
| vivo | Support | As we mentioned in our Tdoc, different RARs for multiple UEs in response to different PRACH preambles are included in one single message. If different RAR size as legacy is used, it may cause impact on how to read RAR from UE side. Keeping the same RAR size as OSI is needed. |
| Tejas | Support |  |
| China Telecom | Support. |  |
| Transsion | Support |  |
| Moderator |  | Concern: Huawei, Apple Can wait after discussion of FL proposal 2-1 and 6-1. |

- FL Proposal 7-1-2

Other than the SIB1 transmission status indication, do not support additional contents to RAR in response to UL WUS on top of contents of RAR in response to SI request.

- FFS: Whether to include the SIB1 transmission status indication into the RAR in response to UL WUS

- Issue 8: Relation between on-demand SIB1 and on-demand OSI procedure

Background

In RAN1 #120, there are following proposals from companies to discuss the relation between on-demand SIB1 and on-demand OSI procedure:

LG:

Proposal #12: Discuss how to guarantee on-demand SIB1 transmission while the UE receives SIBx (other than SIB1) or performs the existing on-demand SI procedure.

QC:

Proposal 5: Support joint PRACH request to trigger both OD-SIB1 and a subset of OD-OSI, e.g. SIB2/SIB3/SIB4.

•  FFS: parameters in UL-WUS config to support joint PRACH request.

Moderator tends to think the following statements from Qualcomm makes some sense:

- “The motivation to support joint PRACH request to trigger both OD-SIB1 and a subset of OD-OSI, e.g. SIB2/SIB3/SIB4, is that when the UE reselects to NES cell, it needs to acquire not only SIB1 but also SIB2-4 for camping information and cell reselection parameters. If SIB1-4 is on-demand in the cell, the UE would need to perform multiple separate RACH procedures (one for SIB1 and one or more for SIB2-4), which is inefficient. Having one RACH procedure to request SIB1-4 is much more efficient from both UE and NW perspectives.”

Moderator hence has the following proposal:

- FL Proposal 8-1

Support joint PRACH request to trigger both OD-SIB1 and a subset of OD-OSI, e.g. SIB2/SIB3/SIB4.

- FFS: parameters in UL-WUS config to support joint PRACH request.

| Moderator |  | Out of scope: Google, Spreadtrum, Samsung, Apple, Xiaomi Not needed: ZTE, Futurewei, Fujitsu, DCM, vivo, Tejas, China Telecom, OPPO, Transsion @Nokia: Previously we agreed not to have joint PRACH request for UL-WUS and entering connected mode. This one is joint PRACH request for OD-SIB1 and OD-OSI, which are both used for idle mode. Can be discussed during offline/online session with lower priority. |
| --- | --- | --- |

| Company | Support or not | Comment |
| --- | --- | --- |
| CMCC |  | If such mechanism is not introduced, UE can first request for OD-SIB1, and then obtain other OSI with the legacy OD-OSI mechanism and the information in OD-SIB1. Therefore, such mechanism can be an optimization, we are open for it. |
| Google |  | This seems to be out of scope. |
| ZTE, Sanechips | Not support | We understand this way may be efficient, but it brings more spec impacts for both RAN1 and RAN2. |
| Spreadtrum | Not | This topic is deprioritized in R-19 |
| Samsung | Not | It seems out of scop in WI. We also think it would be good way for designing on-demand SIB operation. But, with WI description, it will be out of scope, and more spec impact is expected. |
| Sharp | Support | Considering OSI request always requires SIB1, simultaneous request with OD-SIB1 can be considered to avoid unnecessary RACH procedure. |
| Apple | No | Similar comment that it is out of WI scope |
| LG |  | We are open to discuss |
| Xiaomi | Not | We share same view as other companies that it is out of scope. |
| NEC | Not support | Whether to support joint triggering of OD-SIB1 and OSI is dependent on the current triggers defined for OSI which is present in RAN2 spec. Hence, before proceeding with this, RAN2 needs to tell us whether it is even feasible that OD-SIB1 and OSI can be requested by the UE simultaneously. |
| Nokia/NSB |  | This proposal is quite the same as one of the early discussions we had, which was not agreed, isn’t so? |
| Futurewei |  | We do not see it necessary |
| Fujitsu |  | The use case of joint PRACH request for OD-SIB1 and a subset of OSI is unclear. The UE needs to obtain SIB1 to check the broadcast status of other SIB, such as SIB2/SIB3/SIB4. If the status for the SIB that the UE needed is “notBroadcasting”, the UE transmits request for that SIB. Therefore, the UE has already acquired OD-SIB1 before deciding to send a request for an OSI. It seems that joint PRACH request is not needed. |
| DCM | Not support | Agree with Fujitsu. UE cannot identify OSI transmission status before reception of (OD-)SIB1. |
| vivo | Not support | UE may not need SIB1/SIB3/SIB4 when requesting SIB1. |
| Tejas | Not support | There can be a case where, NES CELL can bar some UEs by indicating the barring information in the SIB1. In this case, if UE decode SIB1, and understand that it is barred from accessing the CELL, then triggering other SIBs along with SIB1 could waste resources as UE will not decode other SIBs transmitted along with SIB1. And the amount of NES gain using this method may not be significant. If CELL accessibility info is available at the UE through UL-WUS configuration, we can consider this feature. |
| Moderator |  | Out of scope: Google, Spreadtrum, Samsung, Apple, Xiaomi Not needed: ZTE, Futurewei, Fujitsu, DCM, vivo, Tejas, China Telecom, OPPO, Transsion @Nokia: Previously we agreed not to have joint PRACH request for UL-WUS and entering connected mode. This one is joint PRACH request for OD-SIB1 and OD-OSI, which are both used for idle mode. Can be discussed during offline/online session with lower priority. |
| China Telecom | Not Support | Agree with Fujitsu/DCM. |
| OPPO | Not support |  |
| Transsion | Not Support | SIB1 should be transmitted before OD-OSI. |

- Issue 9: Parameters/contents inside the UL WUS configuration

Background

In RAN1 #117, it is agreed to use the Table I below from [R1-2405106, Ericsson] as a starting point to discuss the required parameters/contents inside the UL WUS configuration.

Table I.

| Purpose | Parameters |  |  |
| --- | --- | --- | --- |
| To which cell does the config applies | NES-CellId | PhysCellId |  |
|  |  | ARFCN-ValueNR |  |
| WUS transmission | SIB1-RequestConfig | ss-PBCH-BlockPower |  |
|  |  | rach-OccasionsSIB1 | Prach-ConfigurationIndex |
|  |  |  | msg1-FDM |
|  |  |  | msg1-FrequencyStart |
|  |  |  | zeroCorrelationZoneConfig |
|  |  |  | preambleReceivedTargetPower |
|  |  |  | preambleTransMax |
|  |  |  | powerRampingStep |
|  |  |  | ra-ResponseWindow |
|  |  |  | ssb-perRACH-Occasion |
|  |  | sib1-RequestPeriod |  |
|  |  | sib1-RequestResource | ra-PreambleStartIndex |
|  |  |  | ra-AssociationPeriodIndex |
|  |  |  | ra-ssb-OccasionMaskIndex |
|  |  | rsrp-ThresholdSSB |  |
|  |  | prach-RootSequenceIndex |  |
|  |  | msg1-SubcarrierSpacing |  |
|  |  | restrictedSetConfig |  |
|  |  | tdd-UL-DL-ConfigurationCommon |  |
|  | frequencyInfoUL | frequencyBandList |  |
|  |  | absoluteFrequencyPointA |  |
|  |  | offsetToCarrier |  |
|  |  | p-Max |  |
|  |  | frequencyShift7p5khz |  |
| SIB1 reception | pdcch-ConfigSIB1 | ssb-SubcarrierOffset |  |
|  |  | controlResourceSetZero |  |
|  |  | searchSpaceZero |  |
| RAR Reception | pdcch-ConfigOD-SIB1-RAR | controlResourceSet |  |
|  |  | monitoringSlotPeriodicityAndOffset |  |
|  |  | Duration |  |
|  |  | monitoringSymbolsWithinSlot |  |
|  |  | aggregationLevels |  |

In RAN1 #118-bis, the following are agreed:

Agreement

For the purpose of “to which cell does the configuration applies”, at least the following parameters are included in the UL-WUS configuration.

| Purpose | Parameters |  |
| --- | --- | --- |
| To which cell does the configuration applies | NES-CellId | PhysCellId |
|  |  | ARFCN-ValueNR |

Note: ARFCN-ValueNR is used to indicate the absolute radio frequency channel number (ARFCN) for SSB of NES cell.

Agreement

For the purpose of “WUS transmission”, at least the following parameters are included in the UL-WUS configuration:

- rsrp-ThresholdSSB

- prach-RootSequenceIndex

- msg1-SubcarrierSpacing

- restrictedSetConfig

Note: In legacy spec, the parameters above are under the IE RACH-ConfigCommon

Agreement

For the purpose of “WUS transmission”, at least the following parameters to indicate “frequency information for UL-WUS transmission” are included in the UL-WUS configuration.

| Purpose | Parameters |
| --- | --- |
| WUS transmission | frequencyBandList |
|  | absoluteFrequencyPointA |
|  | offsetToCarrier |
|  | p-Max |
|  | frequencyShift7p5khz (working assumption)ULSubCarrierSpacing |

Agreement

For the purpose of “WUS transmission”, at least the following parameters are included in the UL-WUS configuration.

| Purpose | Parameters |  |  |
| --- | --- | --- | --- |
| WUS transmission | SIB1-RequestConfig | ss-PBCH-BlockPower |  |
|  |  | SSB-positionInBurst |  |
|  |  | tdd-UL-DL-ConfigurationCommon |  |
|  |  | rach-OccasionsSIB1 | Prach-ConfigurationIndex |
|  |  |  | msg1-FDM |
|  |  |  | msg1-FrequencyStart |
|  |  |  | zeroCorrelationZoneConfig |
|  |  |  | preambleReceivedTargetPower |
|  |  |  | preambleTransMax |
|  |  |  | powerRampingStep |
|  |  |  | ra-ResponseWindow |
|  |  |  | ssb-perRACH-Occasion |
|  |  | sib1-RequestPeriod |  |
|  |  | sib1-RequestResources | ra-PreambleStartIndex |
|  |  |  | ra-AssociationPeriodIndex |
|  |  |  | ra-ssb-OccasionMaskIndex |

Note:

- In legacy spec, ss-PBCH-BlockPower/SSB-positionInBurst/tdd-UL-DL-ConfigurationCommon are under the IE ServingCellConfigCommon/ServingCellConfigCommonSIB

- Msg1-FrequencyStart is with respect to the first RB of the UE carrier determined by offsetToCarrier and absoluteFrequencyPointA

Agreement

Search space for RAR in response to UL WUS on a NES Cell can be provided by the UL WUS configuration. If not, follow the search space zero for the NES Cell.

In RAN1 #119, the following are agreed:

Agreement

The parameter “n-TimingAdvanceOffset” is included in the UL-WUS configuration.

Agreement

For parameters agreed for UL WUS, whether it is mandatory or optional is for further discussion.

Agreement

At least for RO validation determination for WUS transmission, the parameter “ssb-PeriodicityServingCell” is included in the UL-WUS configuration at least for TDD system.

- FFS: FDD system

Agreement

Confirm the working assumption below from RAN1 #118-bis to include it in the UL-WUS configuration.

- (working assumption) ULSubCarrierSpacing

Agreement

CORESET0 of NES cell is used for RAR CORESET of that NES cell.

Agreement

At least for the case where SSB is transmitted on sync-raster, the indication of the quantity K_SSB (defined in TS 38.211 as subcarrier offset from subcarrier 0 in common resource block NCRBSSB to the lowest-numbered subcarrier of the SS/PBCH block) is included in the UL-WUS configuration for FDD and TDD NES cell.

Agreement

The duration of the time window for on-demand SIB1 is indicated in the UL WUS configuration.

- Unit of the duration is in slot

- FFS: Starting time

Companies’ views for this topic in RAN1 #120 are collected below:

Samsung:

Proposal 5: Support the following additional RRC parameters in UL WUS configuration:

- Remaining configuration related to initial UL BWP for the configuration of RO:

  - genericParameters in initialUplinkBWP

- Configurations related to accessibility, e.g. cell selection information and accessible UE type

Note: details are up to RAN2

ETRI:

Proposal 1: For agreed UL WUS parameters, follow the legacy specification regarding its mandatory

or optional presence unless otherwise agreed.

Proposal 2: For agreed UL WUS parameters, regarding their mandatory or optional presence and

applicability to TDD and/or FDD, adopt the followings:

o  PhysCellId and ARFCN-ValueNR are mandatory

o  frequencyBandList and absoluteFrequencyPointA are present in IE FrequencyInfoUL only

for FDD (as in the legacy specification)

o  ssb-PeriodicityServingCell is used for RO validation purpose only during the SIB1 request

procedure, and therefore is present only in TDD

o  K_SSB is mandatory

o  searchSpaceZero and controlResourceSetZero are mandatory (at least for the agreed K_SSB

values)

o  Duration of OD-SIB1 window is optional (as in the legacy OD-OSI specification)

Proposal 3: Include supplementaryUplink, along with the sub-parameter frequencyShift7p5khz, in the

UL WUS configuration.

Proposal 4: For any parameter that exists in both the UL WUS configuration and SIB1, by default,

the value in the UL WUS configuration is aligned with the value in OD-SIB1 unless an exceptional

allowance is separately agreed.

Ericsson:

Proposal 9  Include assistance parameters in the UL WUS configuration that informs the UE of the SIB1 transmission/repetition occasions. For example: a) SIB1 periodicity and offset, or b) SIB1 search space mask indicating which of the occasions are used by the NES cell

Proposal 11  The assistance parameters in the WUS configuration that informs the of the SIB1 transmission/repetition occasions are also valid after the on-demand SIB1 acquisition procedure.

Huawei, HiSilicon

Proposal 7: For OD-SIB1, the following information elements could be added to the required parameters/contents as part of UL WUS configuration.

- carrierBandwidth

- RAN1 clarifies initial uplink BWP related parameter is needed or not

- If needed, Msg1-FrequencyStart is with respect to the first RB of the initial uplink BWP

- If not needed, the RO is confined in an initial uplink BWP determined by UE itself and UE assumes this initial uplink BWP is confined in the carrier bandwidth and the center of this initial uplink BWP is aligned with the center of initial DL BWP for TDD.

- RAN1 clarifies frequencyBandList refer to the IE within FrequencyInfoUL-SIB

Proposal 8: If the value in OD-SIB1 IE is different with the value of the corresponding IE in the WUS configuration, then the value in OD-SIB1 IE overrides the value of the corresponding IE in the WUS configuration, otherwise the corresponding IE is not configured in OD-SIB1.

ZTE Corporation, Sanechips

Observation 9: If the parameter “ssb-PeriodicityServingCell” is absent, the UE can apply the value 5ms as legacy.

Observation 10: The parameter “ra-ssb-OccasionMaskIndex” may be not needed when separated RO is used.

Proposal 7: The parameter “ssb-PeriodicityServingCell”, “ra-ssb-OccasionMaskIndex” and “ra-PreambleStartIndex” should be optional for both TDD system and FDD system.

CATT

Proposal 12: The parameter “ssb-PeriodicityServingCell” is not needed in the UL-WUS configuration for FDD system.

vivo

Proposal 4: The design principle for WUS configuration is to minimize the size of mandatory parameters in WUS configuration as much as possible.

Observation 5: UE needs to distinguish whether the received WUS configuration applies to the serving cell or other NES cells.

Proposal 5: If NES cell indicates a WUS configuration applying to NES cell itself, some parameters can follow the corresponding values in the SIB1 of NES cell itself to further reduce the payload of WUS configuration.

Proposal 6: For distinguishing whether the received WUS configuration applies to the serving cell or to other NES cells,

- if the IE PhysCellIdList is present in WUS configuration, the WUS configuration is applied to the NES cell(s) in the Cell ID list.

- if the IE PhysCellIdList is not present in WUS configuration, the WUS configuration is applied to serving cell.

Observation 6: DL point A may not need to be explicitly indicated in WUS configuration.

Observation 7: For TDD system, offsetToPointA and kssb are present for DL point A and further derived UL point A. For FDD, absoluteFrequencyPointA is present for UL transmission.

Proposal 7: Based on above design principles, we support above Table1 WUS configuration.

Observation 8: RAR decoding would be highly impacted if NES UE doesn’t know the preamble range for CFRA/CBRA and the preamble range for legacy OSI when shared RO.

Proposal 8: Adding totalNumberOfRA-Preambles into the WUS configuration.

Proposal 13: The duration of the time window for on-demand SIB1 can be configured in WUS configuration.

Proposal 15: Support separate WUS resource configuration for RedCap UEs (e.g., named as SIB1-RequestResourceRedCap) to be configured in WUS configuration if associated NES cell supports RedCap feature.

Google

Proposal 1: Support to configure the RA-RNTI by UL-WUS configuration

Proposal 2: Support to configure an initial BWP by the UL-WUS configuration

- The NW can transmit the RAR and on-demand SIB1 based on the bandwidth of the initial BWP

Fujitsu

Proposal 6. In FDD system, the parameter “ssb-PeriodicityServingCell” is not included in the UL WUS configuration.

Proposal 7. RAN1 to discuss the configuration of a common RNTI by UL WUS configuration for DCI scrambling, specifically for the RAR in response to UL WUS.

Xiaomi

Proposal 4: For parameters agreed for UL WUS, they are mandatory.

Proposal 5: The parameter “ssb-PeriodicityServingCell” is included in the UL-WUS configuration for FDD system.

Huawei, HiSilicon

Proposal 8: If the value in OD-SIB1 IE is different with the value of the corresponding IE in the WUS configuration, then the value in OD-SIB1 IE overrides the value of the corresponding IE in the WUS configuration, otherwise the corresponding IE is not configured in OD-SIB1.

CMCC

Proposal 5: For specific content in WUS configuration, support the following parameters:

- SIB1 reception, which includes:

  - Configuration of CORESET0 and CSS0 for the case that 24≤K_SSB≤29 for FR1 and 12≤K_SSB ≤13 for FR2.

  - Actual K_SSB value for FDD system.

  - Duration of SIB1 reception time window.

- RAR reception, which includes:

  - Configuration of searchspace for PDCCH associated with RAR.

Nokia, Nokia Shanghai Bell

Proposal-12: Barring specific UE should be indicated as parameters/contents inside the UL WUS configuration to avoid unnecessary SIB1 requests.

Companies’ views for additional parameters to be included into the UL WUS configuraiton in RAN1 #120 are summarized below:

- genericParameters in initialUplinkBWP

- Yes: Samsung, [Huawei] with following comments,

    - If included, Msg1-FrequencyStart is with respect to the first RB of the initial uplink BWP

    - If not needed, the RO is confined in an initial uplink BWP determined by UE itself and UE assumes this initial uplink BWP is confined in the carrier bandwidth and the center of this initial uplink BWP is aligned with the center of initial DL BWP for TDD.

- cell selection information and accessible UE type

- Yes: Samsung

- supplementaryUplink with the sub-parameter frequencyShift7p5khz

- Yes: ETRI

- Assistance parameters that informs the UE of the SIB1 transmission/repetition occasions. For example: a) SIB1 periodicity and offset, or b) SIB1 search space mask indicating which of the occasions are used by the NES cell

- Yes: Ericsson

- totalNumberOfRA-Preambles

- Yes: vivo

- SIB1-RequestResourceRedCap

- Yes: vivo

- RA-RNTI

- Yes: Google, Fujitsu

- ssb-PeriodicityServingCell for FDD cell

- Yes: Xiaomi

- No: CATT, Fujitsu

- initialDownlinkBWP

- Yes: Google

- cell barring information

- Yes: Nokia (to avoid unnecessary SIB1 requests)

- FL Proposal 9-1

Please elaborate whether you think each of the following parameters are needed in the UL WUS configuration:

- genericParameters in initialUplinkBWP

- cell selection information and accessible UE type

- supplementaryUplink with the sub-parameter frequencyShift7p5khz

- Assistance parameters that informs the UE of the SIB1 transmission/repetition occasions

- totalNumberOfRA-Preambles

- SIB1-RequestResourceRedCap

- RA-RNTI

- ssb-PeriodicityServingCell for FDD cell

- initialDownlinkBWP

- cell barring information

| Company | Comment |
| --- | --- |
| Google | Support initialUplinkBWP, RA-RNTI, and initialDownlinkBWP |
| Samsung | We support 1st sub-bullet and 10th sub-bullet. Regarding the 1st sub-bullet, at least for FDD, it is necessary to transmit UL WUS. Regarding the 10th sub-bullet, to reduce meaningless OD-SIB1 triggering, cell barring information can be transmitted in UL WUS configation. |
| Huawei& Hisilcon 1 | - carrierBandwidth needs to be included in UL WUS configuration, otherwise the UE does not know the MPR for UL WUS transmission. 2. If initialUplinkBWP is included, then Msg1-FrequencyStart should be with respect to the first RB of the initial uplink BWP rather than the first RB of the UE carrier. If initialUplinkBWP is not included, the UE may not be able to send UL WUS due to the uplink center is not aligned with DL center for TDD. So even no initial uplink BWP parameter in UL WUS configuration, the RO should also be confined in an initial uplink BWP determined by UE itself and UE assumes this initial uplink BWP is confined in the carrier bandwidth and the center of this initial uplink BWP is aligned with the center of initial DL BWP for TDD. |
| Qualcomm | We don’t think any IEs listed above is needed. |
| ETRI | Support adding supplementaryUplink for UL coverage extension of TDD NES cells. |
| Fujitsu | Support to include a common RA-RNTI in UL WUS configuration. With the common RA-RNTI, a RAR corresponds to one UE’s UL WUS can also be received by other UEs. We believe it is beneficial for network energy saving. |
| Tejas | - genericParameters in initialUplinkBWP May not be required as UE can still find the PRACH frequency location using absoluteFrequencyPointA, msg1-FrequencyStart and offsetToCarrier parameters for FDD case. Whereas for TDD case using Offset to Point A, Offset to carrier and msg1-FrequencyStart parameters UE can find the frequency location for sending UL-WUS. - cell selection information and accessible UE type Support this, as it will eliminate unnecessary SIB1 triggers. - supplementaryUplink with the sub-parameter frequencyShift7p5khz Ok, if majority support. - Assistance parameters that inform the UE of the SIB1 transmission/repetition occasions Not support. - totalNumberOfRA-Preambles Support - SIB1-RequestResourceRedCap Support - RA-RNTI Not support. - ssb-PeriodicityServingCell for FDD cell Not support. - initialDownlinkBWP Ok, if majority support. - cell barring information Support. |
| Moderator | Common RA-RNTI in UL WUS configuration: Google, Fujitsu Cell barring information: Nokia, Samsung, Tejas cell selection information and accessible UE type: Samsung, Tejas carrierBandwidth (for MPR calculation): Huawei, MTK initialUplinkBWP: Google, Samsung, Huawei totalNumberOfRA-Preambles: vivo, Tejas SIB1-RequestResourceRedCap: vivo, Tejas Can try above during offline/online discussions. |

- FL Proposal 9-1-1 (CATT, Fujitsu)

The parameter “ssb-PeriodicityServingCell” is not included in the UL-WUS configuration for TDD system.

- FL Proposal 9-1-2 (Google, Fujitsu)

Include common RA-RNTI in the UL WUS configuration.

- FL Proposal 9-1-3 (Nokia, Samsung, Tejas)

Include cell barring information in the UL WUS configuration.

- FL Proposal 9-1-4 (Samsung, Tejas)

Include cell selection information in the UL WUS configuration.

- FL Proposal 9-1-5 (Samsung, Tejas)

Include accessible UE types in the UL WUS configuration.

- FL Proposal 9-1-6 (Huawei, MTK)

Include carrierBandwidth (for MPR calculation) in the UL WUS configuration.

- FL Proposal 9-1-7 (Google, Samsung, [Huawei])

Include initialUplinkBWP in the UL WUS configuration.

- FL Proposal 9-1-8 (vivo, Tejas)

Include totalNumberOfRA-Preambles in the UL WUS configuration.

- FL Proposal 9-1-9 (vivo, Tejas)

Include SIB1-RequestResourceRedCap in the UL WUS configuration.

- FL Proposal 9-2

If the value in OD-SIB1 IE is different with the value of the corresponding IE in the WUS configuration, then the value in OD-SIB1 IE overrides the value of the corresponding IE in the WUS configuration.

| Moderator |  | Huawei’s version: - If the value in OD-SIB1 IE is different with the value of the corresponding IE in the WUS configuration, then the value in OD-SIB1 IE overrides the value of the corresponding IE in the WUS configuration. Otherwise it is up to gNB to decide that the corresponding IE is not configured in OD-SIB1. ETRI’s version: - For any parameter that exists in both the UL WUS configuration and SIB1, by default, the value in the UL WUS configuration is aligned with the value in OD-SIB1 unless an exceptional allowance is separately agreed. Can be discussed during offline/online session. Rapporteur suggestion: - It’s up to RAN2 to discuss whether the value in OD-SIB1 IE can be different from the value of the corresponding IE in the WUS configuration, and how to define the UE behavior if the values are different. |
| --- | --- | --- |

| Company | Support or not | Comment |
| --- | --- | --- |
| CMCC |  | Fine with the proposal but this can be discussed after the design of WUS configuration is finished. |
| Google |  | It seems this can be handled by RAN2. |
| Huawei& Hisilcon 1 | Support | We think the proposal is a reasonable behaviour of the UE when it sees two similar IEs configured in WUS and OD-SIB1. However, more important for NES gain is to minimize the OD-SIB1 overhead transmission as much as possible especially when repeating information do not add additional information to the NES UEs. If the value in OD-SIB1 IE is different with the value of the corresponding IE in the WUS configuration, then the value in OD-SIB1 IE overrides the value of the corresponding IE in the WUS configuration. Otherwise it is up to gNB to decide that the corresponding IE is not configured in OD-SIB1. |
| NEC | Support |  |
| Futurewei | OK |  |
| ETRI |  | So far, no use case has been identified where assigning different values to the same parameter in the UL WUS configuration and SIB1 is necessary or beneficial. Rather, any misalignment between the two could unnecessarily complicate UE implementation and potentially bring additional issues in the parameter interpretation. In that sense, we have a slightly different proposal. Alternative proposal: For any parameter that exists in both the UL WUS configuration and SIB1, by default, the value in the UL WUS configuration is aligned with the value in OD-SIB1 unless an exceptional allowance is separately agreed. |
| Tejas | Support |  |
| Moderator |  | Huawei’s version: - If the value in OD-SIB1 IE is different with the value of the corresponding IE in the WUS configuration, then the value in OD-SIB1 IE overrides the value of the corresponding IE in the WUS configuration. Otherwise it is up to gNB to decide that the corresponding IE is not configured in OD-SIB1. ETRI’s version: - For any parameter that exists in both the UL WUS configuration and SIB1, by default, the value in the UL WUS configuration is aligned with the value in OD-SIB1 unless an exceptional allowance is separately agreed. Can be discussed during offline/online session. |

- FL Proposal 9-2-1

Huawei’s version:

- If the value in OD-SIB1 IE is different with the value of the corresponding IE in the WUS configuration, then the value in OD-SIB1 IE overrides the value of the corresponding IE in the WUS configuration. Otherwise it is up to gNB to decide that the corresponding IE is not configured in OD-SIB1.

ETRI’s version:

- For any parameter that exists in both the UL WUS configuration and SIB1, by default, the value in the UL WUS configuration is aligned with the value in OD-SIB1 unless an exceptional allowance is separately agreed.

Rapporteur suggestion:

- It’s up to RAN2 to discuss whether the value in OD-SIB1 IE can be different from the value of the corresponding IE in the WUS configuration, and how to define the UE behavior if the values are different.

| Company | Support which version | Comment |
| --- | --- | --- |

- FL Proposal 9-2-2 (suggested by Rapporteur)

It’s up to RAN2 to discuss whether the value in OD-SIB1 IE can be different from the value of the corresponding IE in the WUS configuration, and how to define the UE behavior if the values are different.

- Issue 10: Other topics

- Issue 10-A: NES Cell identification

Ericsson:

Proposal 8  If UE has UL WUS configuration of a cell with SSB on sync raster and k SSB  < 24/12 (FR1/FR2), then the UE ignores the UL WUS configuration and monitors for SIB1 according to legacy mechanisms.

- Issue 10-B: UL WUS power control procedure enhancement

CMCC

Proposal 6: Support reuse the power control parameter (e.g. transmission power, power ramping counter) for UL WUS while transmitting PRACH for initial access.

- FFS how the ensure the time validity.

- Issue 10-C: Validicity of UL WUS configuration

Panasonic:

Proposal 4: After OD-SIB1 is transmitted for a duration and then stops being transmitted, UE assumption should be further clarified on whether and how long the paging and PRACH configurations are valid.

CATT:

Proposal 14: How to ensure the validity of UL WUS configuration received from cell A should be studied.

- Issue 10-D: UE to monitor the RAR for SIB1 requests sent by other UEs

LG:

Proposal #6: To support UEs to monitor the RAR for SIB1 requests sent by other UEs, the following

two methods can be considered:

1)  UE  monitors  DCI  scrambled  by  RA-RNTI  which  is  associated  with  RO  allocated  for  on-

demand SIB1 but not associated with RO containing PRACH transmitted by UE

2)  Introduce  a  new  RNTI  (i.e.,  cell-specific  or  beam-specific  RNTI)  to  monitor  RAR

corresponding to UL WUS for requesting SIB1

3) Use only PDCCH (i.e., without RAR MAC CE) as the RAR where the PDCCH contains RAPID

directly

- Issue 10-E: Combination of OD-SIB1 with other features

vivo

Proposal 16: Whether some other features such as NRU and SUL can be combined with OD-SIB1 needs conclusion.

- Issue 10-F: UL-WUS on NES cell overlapping with paging occasion on normal cell

MTK

Proposal 5: A potential issue of paging dropping may arise if UL-WUS on NES cell overlapped with paging occasion on Cell A as shown in Figure 1. RAN1 can further discuss how to solve this issue.

Figure 1. An Example of UL-WUS on NES cell overlapped with paging occasion on normal cell

- Issue 10-G: DCI based WUS response

NEC

Proposal 5: RAN1 to study DCI based WUS response from the network.

- FL Proposal 10-1

Companies are invited to provide support/comments on the proposals above (Issue 10 series) or bring up other proposals which (you think) should be treated in this meeting. Moderator would try to devise further discussion based on received comments.

- Proposal receiving support from multiple companies would be visited first

  - Example comment format: MTK supports Proposal X from company Y with the reason that …

| Company | Comment |
| --- | --- |
| Samsung | Okay to have discussion on issue 10-F to clarify the UE behaviour for camping. |

Resulted RAN1 conclusion/agreement

TBD.

- 4 References (all from RAN1 #120)

- R1-2500054	Discussion of on-demand SIB1 for idle/inactive mode UEs	FUTUREWEI

- R1-2500078	Discussion on on-demand SIB1 for eNES	Huawei, HiSilicon

- R1-2500129	Discussion on on-demand SIB1 for NES	ZTE Corporation, Sanechips

- R1-2500174	Discussion on on-demand SIB1 for idle/inactive mode UEs	Spreadtrum, UNISOC

- R1-2500229	Discussion on on-demand SIB1	CATT

- R1-2500264	Discussion on on-demand SIB1 for idle/inactive mode UEs	China Telecom

- R1-2500292	Discussion on on-demand SIB1 for UEs in idle/inactive mode	CMCC

- R1-2500354	Discussion on on-demand SIB1 for idle/inactive mode UEs	vivo

- R1-2500398	On demand SIB1 for idle/inactive UEs	Tejas Networks Limited

- R1-2500441	Discussion on the enhancement to support on demand SIB1 for idle/inactive mode UE OPPO

- R1-2500477	On-demand SIB1 for Idle/Inactive mode UEs	Nokia, Nokia Shanghai Bell

- R1-2500490	Discussion on on-demand SIB1 for idle/inactive mode UEs	Panasonic

- R1-2500526	Discussion on on-demand SIB1 for idle/inactive mode UEs	InterDigital, Inc.

- R1-2500553	On-demand SIB1 for Idle/Inactive Mode UE	Google

- R1-2500622	Discussion on on-demand SIB1 for UEs in idle/inactive mode	NEC

- R1-2500629	Discussion on on-demand SIB1 transmission for network energy savings	Fujitsu

- R1-2500654	On-demand SIB1 for idle/inactive mode UEs	Sony

- R1-2500737	Discussion on on-demand SIB1 for idle/inactive mode UEs	Xiaomi

- R1-2500786	On On-demand SIB1 for IDLE/INACTIVE mode UEs	Apple

- R1-2500855	On-demand SIB1 for idle/inactive mode UEs	Samsung

- R1-2500885	On-demand SIB1 for idle/inactive mode UEs	Lenovo

- R1-2500914	On-demand SIB1 for idle/inactive mode UEs	ETRI

- R1-2500954	On-demand SIB1 for idle/inactive mode UEs	LG Electronics

- R1-2500968	Discussion on on-demand SIB1 transmission for idle/inactive mode UEs Transsion Holdings

- R1-2501021	On-demand SIB1 for idle or inactive mode UEs	MediaTek Inc.

- R1-2501049	Views on On-demand SIB1 for idle-inactive UEs	Vodafone

- R1-2501089	Discussion on on-demand SIB1 for idle/inactive mode UEs	DENSO CORPORATION

- R1-2501133	Discussion on on-demand SIB1 transmission for idle UEs	Sharp

- R1-2501161	On-demand SIB1 procedure	Qualcomm Incorporated

- R1-2501207	Discussion on on-demand SIB1 for idle/inactive mode UEs	NTT DOCOMO, INC.

- R1-2501252	On-demand SIB1 for UEs in idle/inactive mode for NES	Ericsson

- R1-2501263	Discussion on on-demand SIB1 in idle/inactive mode	CAICT

- R1-2501278	Discussion on  on-demand SIB1	CEWiT

- R1-2501299	On-demand SIB1 for NES	Fraunhofer IIS, Fraunhofer HHI