3GPP TSG RAN-WG1 Meeting #120			R1-2501409

Athens, Greece, 17-21 February, 2025

Source: 	Moderator (Huawei)

Title:	FL summary #3 for Ambient IoT: “9.4.1 Physical channels design – modulation aspects”

Document for:	Discussion and decision

Agenda item:	9.4.1

Introduction

According to the chair’s agenda and contributions submitted to this meeting from companies, this feature lead summary is organized to discuss the following topics:

- R2D numerology - Including the SCS, alignment and padding - R2D waveform - R2D modulation - Including the M values and associated R2D bandwidth - R2D CP handling - R2D chip duration - D2R modulation - Others

The draft FL proposals will be found in each section with the following way of naming:

Proposal 2.2.1a-v1: XXX … - ‘2.2.1’ is the section number in this document - ‘2.2.1’ is the section number in this document - ‘a’ is marked as the serial of proposals under this section, e.g. different proposals in this section will be numbered as {a, b, c, …} - ‘v1’ is the version of a proposal, and will be numbered as {v1, v2, v3, …}

Please follow the guidelines for offline discussions and input to FL summary

- Proposals/questions marked as [open] are candidates to be discussed in this meeting.

- Once a proposal/question marked as [closed] then no need further discussion in this meeting.

- Proposals/questions marked as [H] are assumed to be in high priority for the upcoming discussion round in this meeting.

- Proposals/questions marked as [M] are assumed to be in medium priority for the upcoming discussion round in this meeting.

- For the first-round discussion, please kindly provide your views by 21:00 Monday local time. Due the lack of time and no offline before the first online session on Monday afternoon 14:30, early inputs before that are highly appreciated. FL intends to go through and start from less controversial proposals during the first online.

R2D numerology

Related proposals

| Source | Proposal |
| --- | --- |
| Futurewei | Proposal 2: For an OFDM-based waveform with subcarrier spacing of 15 kHz, Btx,R2D is the integer multiples of 180 kHz. |
| Huawei, HiSilicon | Proposal 1: For the R2D transmission of Ambient IoT, SCS 15 kHz is supported. Proposal 2: A decision on the granularity of Btx,R2D is not critical, but if one is needed, Alt 2 (Integer multiple(s) of 180 kHz) is supported. |
| Spreadtrum, UNISOC | Proposal 1: When the generated number of chips for the R2D transmission does not fully occupy the last OFDM symbol, the padding should be used by reader that has no specification impact. |
| China Telecom | Proposal 7: From the transmitter perspective, the start of R2D transmission is assumed to be aligned with the boundary of an NR OFDM symbol (including the CP) for in-band/guard-band operation. Proposal 8: For OOK-4 waveform generation, the end alignment in the last NR OFDM symbol is necessary. - If the generated number of chips for the R2D transmission does not fully occupy the last OFDM symbol, at least padding can be used, other methods are not precluded. Proposal 11: Support only 15KHz SCS for R2D transmission. Proposal 13: Support the transmission bandwidth of 1RB (180kHz) for R2D as baseline. - Integer multiple(s) of 180 kHz (FFS: what integer(s)). |
| CMCC | Proposal 11: For R2D OFDM-based waveform with subcarrier spacing of 15 kHz, Btx,R2D takes values of Alt 2: Integer multiple(s) of 180 kHz. Proposal 15: From device point of view, the end of R2D transmission is not required to be aligned with the boundary of an NR symbol for in-band/guard-band operation. It is up to reader implementation. |
| vivo | Observation 8: No special handling is needed if the R2D transmission ends in middle of OFDM symbol. - From R2D Tx perspective, up to reader implementation to fill in the remaining chips to align with OFDM symbol boundary. - From Rx perspective, no impact on detection (padding is transparent) for AIoT device. |
| OPPO | Proposal 3: Support Alt1, i.e. Btx,R2D including 180 kHz, 360 kHz . |
| NEC | Proposal 7: Padding can be inserted into R2D postamble to meet the last OFDM symbol boundary. |
| Samsung | Proposal 6: When the generated number of chips for the R2D transmission does not fully occupy the last OFDM symbol, padding is used. - Study the following as content of padding: zero-bit(s), zero-code chip(s), repetition of postamble (if supported). - Study whether padding is added before or after CRC attachment, depending on content of padding - Regarding length of padding, if PRDCH is attached with a postamble, study the following options: - Option 1: the length of padding bits are determined such that end of postamble is aligned with the next OFDM symbol boundary - Option 2: the padding is added to PRDCH after line encoding and postamble, and end of the padding is aligned with the next OFDM symbol boundary |
| LGE | Proposal 8: For forward compatibility, R2D transmission bandwidth Btx,R2D are configured by integer multiple(s) of the subcarrier spacing (Alt 3). |
| MediaTek | Proposal 2: For R2D transmission bandwidth, support Alt 2 in TR 38.769, i.e., Btx,R2D is integer multiple(s) of 180 kHz. |
| Sharp | Observation 1: Restriction(s) on the resource grid, if any, should be already specified in Rel-19. Observation 2: For forward compatibility with future evolution of A-IoT e.g. deployment on other bands, SCS for R2D waveform generation should not be a fixed value. Proposal 1: For waveform generation of an R2D transmission (including preamble and PRDCH), OFDM baseband signal generation for legacy NR DL, UL and SL is reused. - Configuration of the resource grid is up to the Reader in Rel-19. Proposal 2: SCS for R2D waveform generation is determined on a per-band basis. Proposal 3: Regarding R2D bandwidths from a Reader perspective, Alt-2 (i.e. integer multiple(s) of 180 kHz) is assumed in Rel-19. |
| HONOR | Proposal 1: R2D only support 15kHz SCS. Proposal 2: Support Alt2 in which Integer multiple(s) of 180 kHz is used for R2D transmission bandwidth. |
| InterDigital | Proposal 3: Transmission bandwidth of the R2D channel is integer multiples of a PRB. |
| NTT DOCOMO | Proposal 2: For R2D, 15 kHz SCS should be assumed in Rel-19, i.e., 14 OFDM symbols correspond to 1ms. Proposal 4: For PRDCH resource allocation unit in time, discuss the following alternatives. - Alt.1: Manchester code codeword, e.g., two chips. - Alt.2: OFDM symbol, i.e., M chips. Proposal 7: For R2D, integer multiple(s) of 180 kHz should be considered for Btx,R2D. - FFS: Maximum number of PRB for each M value is 12. |
| WILUS | Proposal 2: In R2D transmission, if padding is required at the last OFDM symbol, two alternatives can be considered for handling the padding: - Alt 1: Fill the padding with either all ‘0’ or ‘1, depending on the level of the first chip of the OFDM symbol. Alt 2: If there are more than four paddings chips to be filled, the first and second padding chips can be set to the same level as the last information chip. Then the remaining padding can be filled with ‘01’ pattern, ‘10’ pattern, or a combination of both, with the level of the last padding chip equal to the level of the first chip of the current OFDM symbol. |
| IIT Kanpur | Proposal 3: For in-band AIoT operation SCS should be fixed. Preferable 15KHz. |

Discussion

Sub-carrier spacing

From inputs to this meeting, FL observes 7 companies [Futurewei, Huawei, CTC, CMCC, HONOR, DOCOMO, IIT Kanpur] talk about 15kHz SCS while 1 company [Sharp] mentioned SCS for R2D waveform generation is determined on a per-band basis.

Refer to the study during SI, only an OFDM-based OOK waveform with subcarrier spacing of 15 kHz is studied for R2D in the TR 38.769.

Based on above, FL suggests the following proposal:

[Closed][H] Proposal 2.2.1a-v1:

For R2D transmission, only subcarrier spacing of 15 kHz is supported (as per TR 38.769), and no other SCS are further considered.

| Company | Views |
| --- | --- |
| Spreadtrum | Support |
| CMCC | Support |
| Panasonic | OK |
| Nokia | OK. We don’t see the necessity to support 30 KHz for A-IoT communications. |
| QC | Fine to us. |
| vivo | Support |
| Huawei, HiSilicon | Support. This is the priority case for first deployments. |
| Xiaomi | Support the proposal. |
| FUTUREWEI | Support |
| Ericsson | Agree. |
| Tejas Networks | Support and the M-value also should be fixed accordingly |
| Lenovo | Support |
| ZTE, Sanechips | Okay. |
| LGE | Okay with the proposal. |
| OPPO2 | OK. |
| Samsung | OK |
| CATT | OK |
| FL | It was agreed on Monday online session |

Granularity of Btx,R2D

From inputs to this meeting, 10 companies [Futurewei, Huawei, CTC, CMCC, OPPO, MediaTek, Sharp, HONOR, InterDigital, DOCOMO] propose the R2D transmission bandwidth as integer multiple(s) of 180 kHz while 1 company [LGE] proposes integer multiple(s) of the subcarrier spacing for forward compatibility. Among of them, 1 company [Huawei] also mentioned a decision on the granularity of Btx,R2D is not critical.

Refer to the study during SI and TR 38.769, there was no further discussion on this after 3 candidate alternatives identified for the transmission bandwidth, Btx,R2D from a reader perspective. Late on, integer number of PRBs are used when discuss the minimum R2D transmission bandwidth associated with M values. The one proponent of integer multiple(s) of the subcarrier spacing is talking potential scenarios but not in the Rel-19 scope.

Based on above, FL suggests the following proposal:

[Closed][H] Proposal 2.2.2a-v1:

Btx,R2D is defined as integer multiple(s) of 180 kHz.

| Company | Views |
| --- | --- |
| Spreadtrum | We are fine with integer multiple(s) of the subcarrier spacing not only for forward compatibility, but also can achieve more flexibility for resource utilization for Rel-19 A-IoT. Additionally, we also have one observation: if considering the guardband of Bocc,R2D, the total R2D resource will not be integer multiple(s) of 180 kHz. It is benefit for system deployment with Bocc,R2D with integer multiple(s) of 180kHz and Btx,R2D with non-integer multiple(s) of 180 kHz. |
| CMCC | Support |
| Panasonic | Support |
| Nokia | Support |
| QC | Fine to us. |
| vivo | Support. |
| Huawei, HiSilicon | This is fine. There is some overlap between the alternatives. It is also acceptable to directly determine value for Btx,R2D. |
| Xiaomi | Support the proposal. |
| FUTUREWEI | Support |
| Ericsson | Fine. However, it might be better to have Bocc,R2D be a multiple of 180 kHz, and then leave it to RAN 4 to decide the guardband size, when without guardband Bocc,R2D is equavalent to Btx,R2D. |
| Tejas Networks | Support |
| Lenovo | Support |
| ZTE, Sanechips | Okay. |
| OPPO2 | Reader could configure the R2D transmission bandwidth flexibly as equal or larger than the minimum PRBs associated with M values. We think it is enough to define the minimum R2D transmission PRBs associated with M values. |
| Samsung | OK |
| DOCOMO | OK |
| CATT | OK |
| FL | Based on the offline discussion, FL further updates the proposal and merge it into Proposal 4.2.2a-v2. |

Time domain alignment

From inputs to this meeting, 1 company [CTC] propose from the transmitter perspective, the start of R2D transmission is assumed to be aligned with the boundary of an NR OFDM symbol (including the CP) for in-band/guard-band operation. Although only one company has the proposal on OFDM symbol alignment it seems it is also in companies mind when they talk about the issue padding to last OFDM symbol.

During the SI, RAN1 have studied this and concluded that for OFDM-based OOK waveform the start of R2D transmission from the reader perspective is assumed to be aligned with the boundary of an NR OFDM symbol (including the CP) for in-band/guard-band operation in TR 38.769.

Based on above, FL suggests the following proposal:

#### [Open][M] Proposal 2.2.3a-v1:

FL conclusion: For R2D OFDM symbol alignment, TR 38.769 already gives us:

#### 6.1.1.1 R2D waveform, modulation and numerology An OFDM-based OOK waveform with subcarrier spacing of 15 kHz is studied for R2D, with OOK-1 for single-chip per OFDM symbol transmission, and OOK-4 for M-chip per OFDM symbol transmission, starting from the definitions in TR 38.869 [4]. For this waveform, the start of R2D transmission from the reader perspective is assumed to be aligned with the boundary of an NR OFDM symbol (including the CP) for in-band/guard-band operation.

| Company | Is anything needed further to the TR? |
| --- | --- |
| Spreadtrum | Nothing needed to the TR. Additionally, we have one question: from the FL perspective, whether R2D OFDM symbol alignment need to be specified? |
| QC | Fine to align the start of R2D with the start of NR OFDM symbol boundary (including CP). |
| Huawei, HiSilicon | Nothing further needed. We think this does need to be reflected in specification, and with this conclusion in RAN1 chair notes, the relevant editor should get the signal. |
| Xiaomi | No additional clarification is required. |
| Ericsson | Ok. |
| ZTE, Sanechips | The R2D transmission is generated per OFDM symbol, so we think that the end of R2D transmission from the reader perspective should be also aligned with the boundary of an NR OFDM symbol (including the CP). How to attach padding for R2D transmission can be further discussed. |
| FL | To ZTE: end of R2D transmission alignment to OFDM symbol boundary should be same thing as the proposal 2.2.3b in terms of padding. |

1 company [CTC] propose that for OOK-4 waveform generation, the end alignment in the last NR OFDM symbol is necessary while 1 company [CMCC] propose from device point of view, the end of R2D transmission is not required to be aligned with the boundary of an NR symbol for in-band/guard-band operation. It is up to reader implementation.

There also 6 companies talk about R2D padding if the end of R2D transmission is not align with the OFDM symbol boundary. 3 companies [Spreadtrum, vivo, CMCC] think it is up to reader implementation since it is transparent to A-IoT device while 3 companies [NEC, Samsung, WILUS] proposed solutions for padding. 1 company [NTT DOCOMO] propose to discuss PRDCH resource allocation unit in time and suggest two alternatives: Alt.1: Manchester code codeword, e.g., two chips and Alt.2: OFDM symbol, i.e., M chips. FL understand it may be related to OFDM symbol boundary alignment and padding. 1 company [ZTE] propose Calculation 5: the total length of M chip(s) should be equal to the duration of OFDM symbol, i.e. the smallest time unit of resource allocation which it seems the smallest time unit of R2D resource allocation in their mind is OFDM symbol.

According to the TR 38.769. It studied for R2D waveform generation using a DFT-s-OFDM based transmitter, from the transmitter perspective, when the generated number of chips for the R2D transmission does not fully occupy the last OFDM symbol, at least padding can be used. But whether the padding is up to reader implementation or need to be specified is not determined.

Based on above, FL suggests the following proposal:

[Closed][M] Proposal 2.2.3b-v1:

As per TR 38.769, for the OFDM-based OOK-4 waveform, from the reader perspective, when the generated number of chips for the R2D transmission does not fully occupy the last OFDM symbol, padding is used to align with the boundary of the last NR OFDM symbol for in-band operation. Further to the TR,

- Padding is to be down-selected between the following alternatives:

  - Alt 1: Padding is up to reader implementation and transparent to A-IoT device.

  - Alt 2: Specify the details of padding

| Company | Views |
| --- | --- |
| Spreadtrum | We prefer Alt 1. On the one hand, the OFDM symbol boundary is transparent to A-IoT device. On the other hand, reader can demodulation by blind detection. |
| CMCC | We are fine with the proposal and support Alt 1. From device point of view, as long as the device knows the end of R2D transmission by postamble or control information, it will stop monitoring, it does not care what is following. Hence, there is no need to specify such padding operation from device perspective. It is up to gNB whether to send anything for the remaining OFDM symbol. |
| Nokia | Prefer Alt 1. |
| QC | Alt 1 is preferred. |
| vivo | Support Alt 1. For OOK waveform, any ON/OFF combination can be generated. Hence, the tail part can be left to reader implementation. |
| Huawei, HiSilicon | Does the question relate only to the content of the padding, or generally to e.g. how the device understands timing with reference to start/end/length of the padding? |
| Xiaomi | General fine with the proposal. There is no necessity to standardize the padding details, as the device can complete the entire receiving process prior to receiving the padding. |
| Ericsson | We prefer Alt 1. |
| Tejas Networks | We support Alt-1. However, the padding related information such as, the padding is added before postamble or after postamble should be conveyed to the device. |
| Lenovo | Prefer Alt 1. |
| ZTE, Sanechips | In our views, Alt 1 is feasible to ensure that the padding transmitted by reader is transparent to A-IoT device. |
| LGE | Support Alt 2. While the implementation of padding may depend on the reader, providing explicit specifications regarding padding can yield significant advantages. For instance, thoughtfully designed padding could function similarly to a postamble, enhancing the overall structure and clarity of the data. |
| OPPO2 | If the TB for R2D transmission is integer multiple(s) of M values, it is no need to introduce the padding chips. |
| Samsung | We are fine to discuss the alternatives. Our preference is Alt 2. If some garbage signal is added after the indicated end of PRDCH or preamble by implementation, it can trigger false detection of R2D signal, e.g., SIP. We thus prefer specification support of padding, e.g., in baseband signal generation. |
| CATT | Open to discuss the benefits on the options. Alt.1 has less impact on the work of the specs, but Alt.2 might offer some benefits that need further discussion. |
| FL | Based on the inputs above, update as the following Proposal 2.2.3b-v2. |

#### [Open][M] Proposal 2.2.3b-v2:

As per TR 38.769, for the OFDM-based OOK-4 waveform, from the reader perspective, when the generated number of chips for the R2D transmission does not fully occupy the last OFDM symbol, padding is used to align with the boundary of the last NR OFDM symbol for in-band operation. Further to the TR,

- Padding is to be down-selected between the following alternatives:

  - Alt 1: The content of padding is up to reader implementation and transparent to A-IoT device.

  - Alt 2: Specify the detail content of padding

| Company | Views |
| --- | --- |

R2D waveform

Related proposals

| Source | Proposal |
| --- | --- |
| vivo | Observation 9: Larger pre-DFT size with truncation can achieve more flattened ON/OFF chips compared with small pre-DFT size without truncation. Proposal 5: Same number of L samples value for each of M chips per OFDM symbol, and N’=2α2∙3α3∙5α5 shall be fulfilled, where N’=M*L. Proposal 6: The pre-DFT output length N’ can be greater than X, and the output of the pre-DFT can be truncated to fit Tx BW with X subcarriers. Proposal 7: If the output of the pre-DFT is directly mapped to subcarriers without reordering, the ‘centre frequency component’ is actually mapped to lowest frequency of the RB resource, which may lead to the OOK chip not flattened, and not desired for edge detection for Manchester coded R2D transmission. Proposal 8: Re-ordering of the pre-DFT/truncation (if used) output, i.e., DFT shift, is needed to make sure the direct current in baseband shifted to the centre of frequency resource. Proposal 9: DFT shift is needed for correct mapping the frequency domain components correctly, and equivalent operation can be performed in one of the steps of step 2/4/5 per section 4.4 of TR 38.769. - Alt-1: Change of resource mapping before IFFT operation, i.e., achieved in Step 4 - Alt-2: Phase rotation/scrambling of the samples before DFT operation, i.e., achieved in Step 2 - Alt-3: Change of IFFT formula, i.e., achieved in Step 5 Proposal 10: There are parameters not applicable for AIoT R2D if reusing NR OFDM baseband signal generation. It is not clean and straightforward to re-define these parameters for AIoT, which may make the baseband generation formula quite complicated. |
| TCL | Observation 1: For M=1, OOK-4 can be generated by CP-OFDM and DFT-OFDM, and for M>1, OOK-4 can be generated by DFT-OFDM. Observation 2: The lowest threshold of ON bit for R2D signal transmission may impact on the performance of RF ED for energy detection method. Observation 3: If edge method is used for start-indicator detection, the first level will not be detected by device but it can activate/wake-up device to receive following R2D signal, e.g., clock-acquisition. Observation 4: For edge detection, if higher PAPR is generated in OOK-4 time domain waveform, device may detect wrongly OOK symbol Observation 5: Rel-19 do not need to consider the impact of multipath effect on frequency spectrum power. Proposal 1: Consider how to keep OOK-4 symbol with low PAPR or flat waveform during transmission. Proposal 2: How to solve signal distortion because of truncation operation for DFT-s-OFDM operation should be considered in WI phase. |
| OPPO | Observation 6: Overlaid OFDM sequence over OOK chip could help flatten the spectrum, though Ambient IoT device has no capability to detect OFDM sequence, which may or may not have specification impact. Proposal 2: For the better detection performance of R2D transmission and flatten the spectrum, support overlaid OFDM sequence over OOK-ON chip when OOK-4 waveform generation. |
| EURECOM | Proposal 2: Specify time-domain sequence mapped to ON-chip. |
| NEC | Proposal 3: OFDM baseband signal generation from OOK chip values to time-continuous signal s(t) needs to be specified in normative work. Proposal 4: Cyclic-shift based procedure is proposed for OFDM baseband signal generation from OOK chip values to time-continuous signal s(t). Companies could select a base sequence by implementation and cyclic shift the based sequence to each chip to generate the signals on that chip. |
| Xiaomi | Proposal 1: RAN1 supports only DFT-s-OFDM for the generation of R2D waveforms. |
| Apple | Proposal 1: It should be discussed whether to specify how to generate the OOK-4 waveform. |
| InterDigital | Proposal 1: Support DFT-s-OFDM for R2D waveform generation. Proposal 2: The detailed DFT operation for R2D waveform generation is left to reader implementation and is not specified. |
| Qualcomm | - Observation 18: When N'=X, for some M and #RB combinations, we have non-integer number of samples per chip. - When N'=X, for M={1,2,4,6}, any # of RBs have integer number of per-chip input samples to DFT input. - When N'=X, odd # of RBs do not work well with M={8, 16, 24, 32}. - When N'=X, even # of RBs work well for M = {8, 12, 24} - When N'=X, M=16 and 32 does not work well in general. - Observation 19: Using larger N’ values than X can ensure number of samples per OOK chip for DFT input be integer. However, discarding N’-X DFT outputs will degrade sharpness of waveform, which degrades performance. |
| NTT DOCOMO | - Proposal 1: For R2D waveform, DFT-s-OFDM is assumed for OOK-4. |

Discussion

Waveform generation

From inputs to this meeting, FL observes 3 companies [vivo, OPPO, EURECOM] propose certain details of DFT-s-OFDM generation in their contributions in terms of different aspects. 1 company [NEC] generally saying OFDM baseband signal generation from OOK chip values to time-continuous signal s(t) needs to be specified in normative work with proposing a specific solution. 1 company [TCL] propose how to solve signal distortion because of truncation operation for DFT-s-OFDM operation should be considered in WI phase. 1 company [China Telecom] mentioned “FFS: The corresponding parameters such as the value of M chips, L samples, N’-points DFT, X subcarriers and N-points IDFT” in their R2D modulation section. 1 company [Qualcomm] mentioned some details of DFT-s-OFDM generation for M values analysis without proposing specification efforts. 3 companies [Xiaomi, InterDigital, NTT DOCOMO] propose to support DFT-s-OFDM waveform while 1 company [InterDigital] also proposes that the detailed DFT operation for R2D waveform generation is left to reader implementation and is not specified. 1 company [Apple] propose to discuss whether to specify how to generate the OOK-4 waveform while they mention since the device will only use envelop detection to detect OOK symbols, there is no strict need to specify the exact waveform. If the waveform generation is not specified, it can be discussed in RAN4 whether a reference waveform should be defined for performance requirement.

In the TR 38.769, there is a section 4.4 describing the waveform generation for DFT-s-OFDM (copy as follows) with saying for evaluation purposes.

With reference to the R2D waveform described in Clause 6.1.1.1, for evaluation purposes the waveform for DFT-s-OFDM is generated as follows: 1. The time domain OOK signal is the M chips of one OFDM symbol. 2. A chip is represented (e.g. upsampled) by L samples - Companies to report L 3. An N’-points DFT is performed on the samples of one OFDM symbol to obtain the frequency domain signal. - Companies to report N’, e.g. N’=128 or equal to X 4. Map the frequency domain signal obtained by N’-points DFT to the X subcarriers of Btx,R2D. - Companies report how to map and report X 5. An N-points IDFT is performed to obtain the time domain signal. - Companies to report N, and how value was selected Note: Companies report whether/how CP samples are added.

When the first time to discuss above steps during SI there is a justification in the feature lead summary (R1-2405441) with saying “Although it is possible that the R2D waveform generation may be not specified, it will have in e.g. RAN4 certain time domain characteristics that ensure it is ‘good’. For evaluation purposes in RAN1, it seems we need to agree on how we will model the generation. Note that the below is an attempt to harmonize understanding of signal generation among companies for evaluation purposes. It does not imply any specification of the reader’s signal generation.”

FL understand the above steps captured in the TR section 4.4 does not have the intention to be specified and also there is no further discussion if anything need to be specified.

Based on above, FL suggests the following proposal:

#### [Open][H] Proposal 3.2.1a-v1:

For R2D transmission, from reader perspective, DFT-s-OFDM waveform is supported for OOK-4 modulation. For the details of DFT-s-OFDM waveform, down-selection between the followings:

- Option 1: Details of DFT-s-OFDM waveform generation is up to reader implementation and is not specified in RAN1.

  - This does not preclude RAN4 to discuss R2D waveform generation themselves when defining requirement(s), if needed.

- Option 2: Certain specification of DFT-s-OFDM waveform generation is needed in RAN1.

  - FFS: identify what potential details of 5 steps in section 4.4 of TR 38.769 needs to be specified.

| Company | Views |
| --- | --- |
| Spreadtrum | We prefer option 1. |
| CMCC | We are fine with the proposal, and prefer Option 1. |
| Panasonic | We think option 1 can be the possibility. But before to conclude it, we prefer to obtain the view from RAN4. If RAN4 see the need on this, we think the reference waveform itself should be described in RAN1 spec. |
| Nokia | Option1 |
| QC | The waveform generation method is related to CP handling scheme discussion. So, it is better to defer its discussion until we have conclusion on CP handling scheme. |
| vivo | Support this proposal. Either option would be OK. Besides, even for option 2, if the parameter and details are not fully specified for all steps in OOK4 generation, some RAN4 requirements on waveform may still needed. |
| Huawei, HiSilicon | Option 1, from the RAN1 perspective. So much as needed in RAN1 will result from how many PRBs are used for a given M-chips/symbol decision. |
| Xiaomi | General fine with the proposal. We prefer option 1. It makes things simpler for the reader when creating waveforms, while still meeting the basic requirements. |
| Ericsson | We support option 1 |
| Tejas Networks | We support Option 1. |
| Lenovo | Support Option 1. |
| ZTE, Sanechips | In our views, we support Option 2 to ensure a consistent understanding of waveform generation procedure between reader and device. Other aspects with impacts on interference, adjacent channel power ratio (ACPR) and so on can be discussed by RAN4. |
| OPPO2 | OK with the proposal. No need to specify the details of 5 steps, it is enough to determine the characteristics of frequency and time domain for the waveform. |
| Samsung | Option 1. We don’t see what potential details of 5 steps in section 4.4 of TR 38.769 needs to be specified. |
| DOCOMO | We support option 1. |
| FL | FL understand the majority view is Option 1. FL would like to further ask companies, from specification perspective, what details of the “DFT-s-OFDM waveform” in mind that no need to specify? If nothing specified, from specification perspective, how to describe “For R2D transmission, from reader perspective, DFT-s-OFDM waveform is supported for OOK-4 modulation.”? |

#### [Open][H] Question 3.2.1a-v1:

- What details of DFT-s-OFDM waveform generation is not needed to be specified in RAN1?

- If nothing specified, from specification perspective, how to describe “For R2D transmission, from reader perspective, DFT-s-OFDM waveform is supported for OOK-4 modulation.”?

| Company | Views |
| --- | --- |

R2D Modulation

Related proposals

| Source | Proposal |
| --- | --- |
| Ericsson | Proposal 5 The modulation order (M) for OOK-4 can be down-selected as follows: Number of PRBs; Maximum M value. 1 PRB; 2. 2 PRBs; 4. 3 PRBs; 4. 4 PRBs; 8. greater than 4 PRBs; 8. |
| ZTE, Sanechips | - Proposal 1: Support the M values captured in TR38.769. - Proposal 2: Define the maximum Btx,R2D # of PRBs for different M values. |
| Huawei, HiSilicon | Observation 1: M = 32 with minimum transmission bandwidth 4 PRBs cannot achieve BLER 10% and cannot work well for the R2D transmission. Observation 2: The M value set of M = {2, 6, 24} provides evenly SNR gap for the R2D transmission. Observation 3: M = 6 with minimum 1RB achieves the maximum transmission efficiency with minimum 1 PRB and higher transmission efficiency than M = 8 with minimum 2 PRBs. Observation 4: If M values with small gap e.g., M = 6 and M = 4, M = 8 are all in the M value table, false detection rate performance of the clock acquisition will be degraded. Observation 5: Similar as RFID, the clock for receiving R2D chips corresponding to any of M value is up to device implementation. Observation 6: We consider to support the M value set of {2, 6, 24} in the following aspects: - Including M = 24 is necessary since it can achieve comparable chip rate as UHF RFID and good performance. - Including M = 6 is necessary since it can achieve the maximum transmission efficiency with minimum Btx,R2D 1 PRB and higher transmission efficiency compared with M = 8 with minimum Btx,R2D 2 PRBs. - Evenly SNR gap with the M value set {2, 6, 24} is sufficient for different coverage/data rate. - No FDR performance loss for the clock acquisition performance with distinguished gap among M values in the set of {2, 6, 24}. Proposal 3: Specifying the minimum Btx,R2D per M value is required in order to guarantee the system performance. Proposal 4: There is no need to specify the maximum Btx,R2D per M value, due to the device’s RF-ED receiver architecture. Proposal 5: The maximum M value is required to ensure the corresponding R2D chip length comparable with UHF RFID minimum chip length. Proposal 6: M = 32 is not supported of the M value set for the R2D transmission. Proposal 7: The maximum M value is M = 24 for the R2D transmission. Proposal 8: The M values determination requires down-selection of the M values table of TR 38.769. - M = {2, 6, 24} is a sufficient set. Proposal 9: For the appropriate M = {2, 6, 24} set, the minimum values of Btx,R2D agreed in TR 38.769 are used, since they give sufficient performance. Proposal 10: Specify the following table of {M, minimum Btx,R2D} values: M; Minimum Btx,R2D PRBs. 2; 1. 6; 1. 24; 3. |
| Spreadtrum, UNISOC | Proposal 6: M value should not be equal to the integer multiples of 6, i.e., 6, 12, 24. Proposal 7: M values with 1, 4, 8, 16, 32 are supported together with the following table 3. Table 3: M values and the associated minimum Btx,R2D value M; Minimum Btx,R2D # of PRBs. 1; 1. 4; 1. 8; 2. 16; 2. 32; 4. |
| CATT | Proposal 6: For Device 1, the R2D signal should only support the following M values: {1, 2, 4, 8, 16}. Proposal 13: From the device perspective, the specific transmission bandwidth for R2D signal/channel, which is greater than or equal to the minimum Btx,R2D value, should be up to gNB implementation. |
| China Telecom | Proposal 1: For R2D transmission, reuse OOK-4 with M-chip per OFDM symbol transmission from LP-WUS as a starting point, with further refinement if needed. - FFS: The corresponding parameters such as the value of M chips, L samples, N’-points DFT, X subcarriers and N-points IDFT. Proposal 2: For R2D transmission, support OOK-4 with no larger than 4-chip per OFDM symbol transmission as baseline. - No need to support M value larger than 16 for A-IoT device 1. Proposal 3: Update the table of M value and the associated minimum Btx,R2D value as follows. M; Minimum Btx,R2D # of PRBs. 1; 1. 2; 1. 4; 1. 6; 1. 8; 2. 12; 2. 16; 2. 24; 3. 32; 4. Proposal 12: From RAN1 perspective, no need to define the potential guard band for R2D transmission. - It can be left to RAN4 work or up to implementation based on the different network deployment and interference scenarios. Proposal 14: From RAN1 perspective, no need to define the associated intra A-IoT guard-bands totalling Bguard,D2R for D2R transmission. - It can be left to RAN4 work or up to implementation based on the scheduling mechanism with different network deployment and interference scenarios. |
| CMCC | Observation 5: Considering the offset of sampling clock, it is difficult for a device to determine the M value based on the sampling points per chip, especially if adjacent M values are supported. Proposal 7: Down-selection of M values can be considered. M = 2, 6, 24 can be used as a start point. Proposal 8: The recommended minimum Btx,R2D number of PRBs for M=16 and M=24 can be updated to, - M=16, the minimum Btx,R2D number of PRBs is 3; - M=24, the minimum Btx,R2D number of PRBs is 4; |
| vivo | Proposal 2: M=32 suffers material performance degradation, especially with the presence of CP, with BLER error floor at 20% BLER. Proposal 3: M=32 is not supported due to performance degradation, and the upbound of the M value for R2D can be determined between 16 or 24 - M=16 can be considered to achieve a comparable R2D data rate as RFID. - M=24 can be considered to achieve a higher date rate than RFID Proposal 4: All M values below 16/24 in Table 6.1.1.4-1 of TR 38.769 are supported. |
| Nokia | Proposal 1: RAN1 clarifies whether to support OOK-1 for R2D Proposal 6: RAN1 should decide a set of (M, Btx,R2D) should be supported, which needs to be considered for CP handling. |
| OPPO | Observation 1: If the M values larger than 16, R2D transmission needs more than 2 PRBs. Observation 2: For the M values [8,12,16], considering the same minimum frequency resource i.e. 2PRBs, the detection performance of R2D transmission would decrease as the value of M increase. Observation 3: There are three relationships between the time duration of OOK chip and CP as following according to different M values. - In case of M≤8, the OOK chip duration is significantly larger than CP duration. - In case of M=12, the OOK chip duration is very close to CP duration. - In case of M>12, the OOK chip duration is significantly smaller than CP duration. Observation 4: In case of M>12, the CP part would copy more than one OOK chip from the OFDM symbol part. Observation 5: For the R2D transmission, not see the necessity to support too large peak data rate. Peak data rate 56kbps is enough. Proposal 1: Limit the maximum value of M to 8, i.e. support M=1,2,4,8. |
| Tejas | Observation 1: As R2D signals are transmitted using OOK-4 generated by OFDM modulator, it is important to define the M-value for OOK-4 modulation for all DL message signals. Proposal 3: we propose the following three options to decide the optimum M value. - Option 1: Msg0 contains known preamble from which the device can estimate the channel condition and choose an optimum M value from a lookup table stored in the device and convey the same through msg. - Option 2: Msg1 contains known preamble from which the reader can estimate the channel condition and choose an optimum M value from a lookup table stored in the reader and the M value will be used for subsequent DL transmissions. - Option 3: An optimum value can be decided based on the following parameters: - Received signal strength of the UL message signals received form the device - Block error rate - Data rate and latency |
| EURECOM | Observation 1: Given the same total transmit power, larger values of M outperform smaller M while minimizing both transmission resources and transmission time. Observation 2: Bandwidths larger than the minimum BW increase performance. Proposal 1: Limit the number of possible transmission BW configurations, e.g. to one low, medium and high BW configuration. |
| NEC | Proposal 1: To support the required data rate of ambient IoT’s service, large M value, e.g., M = 16 or 32, is specified in normative work. Proposal 5: OOK modulation depth could be considered. Proposal 6: Reader make sure a time duration of OOK-ON power before R2D transmission which is used by the device to determine the threshold of OOK-ON and OOK-OFF detection (something like AGC). |
| Sony | Proposal 1: For R2D, OOK-4 modulation is supported with a maximum value of M = 8. |
| Xiaomi | Observation 7: When determining the M value, it is crucial to consider the number of information bits to ensure that the device can receive all necessary information within a short time frame, thereby avoiding additional transmission delays. Observation 8: The bandwidth allocated by the reader for transmitting PRDCH depends on its specific implementation, provided that it meets the minimum bandwidth requirements. Proposal 4: RAN1 should support an appropriate M-value range to accommodate varying information bit sizes while ensuring optimal data transmission efficiency. |
| Samsung | Proposal 7: For R2D transmission, the M value of a given PRDCH is indicated by preamble associated with the PRDCH. Proposal 8: M values of PRDCH can be implicitly indicated by OOK chip length of preamble. Proposal 9: For PRDCH reception, the M value is identical for both control and data parts. |
| Fujitsu | Proposal 3: The maximum M value for device 1 is 12 or 16. |
| LGE | Observation 5: Supporting the M values that are integer multiples of 6 (i.e., M = 6, 12, 24) result in non-integer number of samples per chip duration, which would add complexity to the transmitter. Proposal 6: Support at least M = 1, 2, 4, 8. - FFS other M values such as M = 16, 32 considering CP handling issues - FFS other M values such as M = 6, 12, 24 considering the potential impact of non-integer number of samples per OOK symbol on transmitter complexity Proposal 7: Consider defining a basic set of M values (e.g., M = 1, 2, 4, 8) to be supported by all devices in Rel-19. - FFS defining extended set(s) to be supported based on device capabilities or by devices in later releases. Proposal 9: Discuss if the maximum configurable R2D transmission bandwidth can be reduced to e.g., 6 PRBs from the 12 PRBs currently in TR 38.769. |
| Lenovo | Proposal 2: For R2D transmission, limit the maximum M value to (M < 8) |
| HONOR | Proposal 3: Down select the supported M values, e.g. {2,6,24}. |
| InterDigital | Proposal 4: Adopt the following table for M and corresponding number of PRBs for R2D transmission. Table 1 M and corresponding number of RBs M; Minimum # of PRBs. 1; 1. 2; 1. 4; 1. 6; 1. 8; 2. 12; 2. 16; 2. 24; 3. 32; 4. |
| Qualcomm | - Observation 14: RFID reader can use any data rate between 26.6kbps and 128kbps. - Observation 15: In R2D with Manchester coding, M=4 (28kbps) to M=16(112bkps) provides similar range as that supported by RFID. - Observation 16: Small M = 1 and 2 can be considered to support extended coverage compared to that of RFID. - Observation 17: Device can use identified M value in detecting OFDM symbol boundary, CP handling (if any), and device clock calibration. - Observation 18: When N'=X, for some M and #RB combinations, we have non-integer number of samples per chip. - When N'=X, for M={1,2,4,6}, any # of RBs have integer number of per-chip input samples to DFT input. - When N'=X, odd # of RBs do not work well with M={8, 16, 24, 32}. - When N'=X, even # of RBs work well for M = {8, 12, 24} - When N'=X, M=16 and 32 does not work well in general. - Observation 19: Using larger N’ values than X can ensure number of samples per OOK chip for DFT input be integer. However, discarding N’-X DFT outputs will degrade sharpness of waveform, which degrades performance. - Observation 20: For CP handling method M2-2, M=32 does not work for AWGN channel. - Observation 21: For CP handling scheme M1-1, M=16 is the maximum supportable M in AWGN channel. - Observation 22: Using different M value for SIP and PRDCH could reduce false detection of SIP. - Proposal 2: A-IoT device should be able to identify indicated M value based on CAP. - Proposal 3: For PRDCH, support a set of M values = {1, 2, 4, [6/8], 12, 16} if CP handling scheme M1-1 is adopted. One of 6 and 8 used for SIP is to be dropped from this set. - Proposal 4: For PRDCH, support a set of M values = {1, 2, 4, [6/8], 12, 24} If CP handling scheme M2-2 is adopted. One of 6 and 8 used for SIP is to be dropped from this set. - Proposal 5: SIP uses a fixed M = [6/8], which is not to be used for PRDCH. - Proposal 6: Minimum necessary R2D transmission bandwidths associated with M values can be fixed/concluded only after the necessary functionalities and requirements on R2D OOK edges are clear. - Proposal 7: The maximum R2D bandwidth is 24RBs for all M values. |
| NTT DOCOMO | Observation 1: For PRDCH, to achieve more than one hundred kbps data rate, at least M=16 should be supported considering that Manchester coding is applied. Observation 2: For PRDCH, for CP handling Method 1, at least Alt.1 can work for any M value assuming that device can identify the CP location. Observation 3: For PRDCH, for CP handling Method 2 Alt.1, the characteristic of Manchester coding such as transition edge at the middle of codeword is not preserved for last chips when M=16 or larger. Observation 4: For PRDCH, CP handling Method 2 Alt.1 may not work for M=2 assuming the codeword of Manchester coding as; bit 0→chips{10}, bit 1→chips{01}. Observation 5: For PRDCH, CP handling Method 2 Alt.2 can work for any M value. Proposal 5: For PRDCH, at least 1, 4, 8 and 16 should be supported for M value of OOK-4. - FFS: Whether M=2 and/or 24 or larger can be supported subject to CP handling Method. Proposal 6: For R2D, support minimum transmission bandwidth for each M value of OOK-4 which is captured in table 6.1.1.4-1 in TR 38.769. Proposal 7: For R2D, integer multiple(s) of 180 kHz should be considered for Btx,R2D. - FFS: Maximum number of PRB for each M value is 12. |
| IITM | Observation 1: For a given SNR, the performance of the R2D decreases as the value of M increases. Therefore, M should be scheduled based on the link performance. Proposal 2: In R2D, the number of chips per OFDM symbol should be selected based on the link performance in the previous D2R messages. |

Discussion

M values

A table of starting point for study of M values and the associated minimum Btx,R2D value is in the TR 38.769. Companies gives their views for M values of this table in contributions to this meeting.

Table 6.1.1.4-1 is a starting point for study of M values and the associated minimum Btx,R2D value. The reader can use any transmission bandwidth greater than or equal to the minimum Btx,R2D value. Note: Depending on further study, the maximum value of M may be less than 32. Note: The performance can be better when transmission bandwidth greater than the minimum Btx,R2D, depending on device processing and transmit power constraint. Table 6.1.1.4-1: Starting point for M values and the associated minimum Btx,R2D value M; Minimum Btx,R2D # of PRBs. 1; 1. 2; 1. 4; 1. 6; 1. 8; 2. 12; 2. 16; 2. 24; 3. 32; 4.

FL observes that 2 companies [ZTE, InterDigital] propose to take all the M values in the table while 15 companies [Ericsson, Huawei, Spreadtrum, CATT, China Telecom, CMCC, vivo, OPPO, Sony, Fujitsu, LGE, Lenovo, HONOR, Qualcomm, NTT DOCOMO] talk about down-selection of M values from the table.

For the down-selection of M values from the table, companies discuss different aspects in their papers. FL observes that common interests are shown in the following aspects:

- Comparable chip/data rate to UHF RFID

  - Maximum M value and smallest M value determines the range of R2D chip/data rate

- Performance

  - Performance of a certain M value

  - Evenly SNR distribution cross different M values

- Transmission efficiency

  - Considering the efficiency together with Btx,R2D

- Ambiguity of device detecting different M values

  - Distinguish from adjacent M values

  - Distinguish from M values and CP

  - Distinguish from M values and R2D SIP signal

- Integer or non-integer samples per chip

For R2D chip/data rate, some companies mentioned it is necessary that defining A-IoT maximum M value has to support comparable chip/data rate to RFID. 4 companies [Huawei, vivo, Qualcomm, LGE] show that M equal to 32 does not work due poor performance.

Based on above, FL suggests the following proposal:

[Closed][H] Proposal 4.2.1a-v1:

For the OOK-4 modulation for M-chip per OFDM symbol transmission, the maximum M value is down-selected from the followings:

- Alt 1: 32

  - Achieve much higher chip/data rate than RFID

- Alt 2: 24

  - Achieve comparable R2D chip rate and a bit higher data rate than RFID

- Alt 3: 16

  - Achieve a bit lower R2D chip rate and comparable R2D data rate than RFID

- Alt 4: 8

  - Achieve much lower R2D chip/data rate than RFID

FL encourage companies to share views to check whether we can pick one of them or further reduce the candidates.

| Company | Views |
| --- | --- |
| Spreadtrum | From our view, M = 32 is the first priority, and M=16 is the second priority. |
| CMCC | Considering the largest M should ensure that the R2D data rate is competitive, e.g., larger than 320 kbps, we slightly prefer Alt2:24 as the maximum M value |
| Panasonic | We prefer Alt1 or 2 to support high data rate. |
| Nokia | It shouldn’t be less than 24. It doesn’t make sense to support lower data rate than the of the existing RFID. |
| QC | Maximum M value also depends on CP handling scheme. So, it would be good to discuss CP handling scheme first. In principle, we prefer A-IoT to support the set of data rates comparable to that of RFID. We prefer Alt 2 or 3 depending on CP handling scheme can provide similar data rate as RFID. 32 is too high to be reliably supported. |
| vivo | Support either Alt 2 or Alt 3. And 32 seems not work under most CP handling method. |
| Huawei, HiSilicon | Alt 2. Alt 3 and Alt 4 do not match to RFID, while solutions exist for handling higher maximum M value. Those higher maximum M values are within the ability of the device 1 and reader, and with proper selection of other M values, the device is able to detect amongst them with sufficient performance. Hence providing maximum M=24 does not come with a downside, in our view, compared to a lower maximum value. Alt 1, M = 32 with minimum transmission bandwidth 4 PRBs cannot achieve BLER 10% and cannot work well for the R2D transmission according to our evaluations. At a sampling rate of 1.92 MHz, the samples of each chip for M = 32 is only 4, which is insufficient for the detection of the receiver. Since the rate achieved by M=32 is beyond what is needed, we think M=32 is not needed. |
| Xiaomi | General fine with the proposal. Although a higher M value can lead to a higher data rate, the resulting performance loss cannot be ignored. Therefore, the feasibility of larger M values needs to be carefully evaluated. In addition to evaluating performance, it is also necessary to consider the overhead of frequency domain resources, and there should be a reasonable compromise between the M value and the minimum bandwidth. |
| FUTUREWEI | We prefer Alt1: 32, which provides a much higher R2D data rate than RFID. (the shortest low power segment in RFID is 3.125 us). The 2nd choice is Alt2: 24. |
| Ericsson | We support Alt 4. Considering several criteria including combining gain of energy detection, and robustness against time and frequency offsets, the impact of CP insertion. We believe that maximum M-value feasible option is 8. |
| Tejas Networks | A lower value of M can be chosen as default M value for the first DL message signal. For the subsequent DL signals, the M value can be updated according to the channel condition. |
| Lenovo | Prefer Alt 4, but open for Alt 3 |
| ZTE, Sanechips | In our views, A-IoT should achieve a comparable and a bit higher data rate. Therefore, Alt 2 is supported. |
| LGE | Support at least Alt 4. Consideration must be given to scenarios where the cyclic prefix encompasses multiple chips (e.g., M=24, 32), particularly in conjunction with the methodology employed for handling the cyclic prefix. |
| OPPO2 | Support Alt4. In our views, the chip/data rate comparable with RFID is not the only aspect for the down-selected. Coverage, inventory counting efficiency, performance for detection and so on also need to be considered. Suggest to remove the sub-bullets about the chip/date rate when do the down-selected, or add more aspects for down-selected, e.g. the performance for detection, impact on the design of CP handling. |
| Samsung | Considering the commercial compatibility with RFID, we prefer to remove Alt 3 and Alt 4. For down-selection between Alt 1 and 2, we think the motivation should include not only the data rate, but also the performances of different M values from system design perspective, e.g. CP handling. Therefore, the down-selection can be postponed depending on other proposal’s progress. |
| DOCOMO | Which max M can work is subject to CP handling, then we think it is better to defer the discussion and wait for the progress in CP handling Method. |
| CATT | Support Alt 3. |
| FL | Based on the 1st offline session discussion, FL try to further update the proposal as the following Proposal 4.2.1a-v2, please continue to share your views. |

#### [Open][H] Proposal 4.2.1a-v2:

For the OOK-4 modulation for M-chip per OFDM symbol transmission

- The maximum M value is no less than 16, and to be down-selected from the followings:

  - Alt 1: 32

  - Alt 2: 24

  - Alt 3: 16

- Up to the maximum M value, at least [3] M values to be supported from Table 6.1.1.4-1 (as per TR 38.769)

  - FFS: the exact M values

| Company | Views |
| --- | --- |

For the M values, FL observes that different companies propose divergent M value sets and at least including 3 M values for different R2D chip/data rate. It seems not easy to reach consensus on candidate M values directly.

Based on above, FL suggests the following proposal:

[Closed][H] Proposal 4.2.1b-v1:

For R2D, up to the maximum M value, RAN1 select M values from the following two options:

- Option 1: all M values up to the maximum M value

- Option 2: at least 3 of the M values for [low, medium, high] R2D chip/data rate

  - FFS: the selection of detail M values can consider at least the following aspects

    - Performance and SNR distribution cross different M values

    - Transmission efficiency together with minimum Btx,R2D

    - Ambiguity of device detecting different M values

    - Integer or non-integer samples per R2D OOK-4 chip

  - FFS: whether need more M values

[Closed][H] Proposal 4.2.1b-v2:

For R2D, up to the maximum M value, RAN1 select M values from the following two options:

- Option 1: all M values (as per TR 38.769) up to the maximum M value

- Option 2: at least 3 of the M values for [low, medium, high] R2D chip/data rate subset of M values (as per TR 38.769) up to the maximum M value

  - FFS: the selection of detail M values is to be done can considering at least the following aspects

    - Performance gap of and SNR distribution cross under different M values

    - Transmission efficiency together with minimum Btx,R2D

    - Ambiguity of device detecting different M values

    - Integer or non-integer samples per R2D OOK-4 chip

    - [Flexibility in time domain resource for R2D transmission]

    - CP handling

  - FFS: whether need more M values

| Company | Views |
| --- | --- |
| Spreadtrum | We prefer Option 2. There is no need to support so many M values, especially when some values are relatively close. |
| CMCC | We support option 2. Assuming that the device can correct sampling point based on, e.g., start-indicator part of the R2D time acquisition signal, and after calibration, the SFO error is around 10%, the calculated sampling points per chip show that the device may have problem in determining the correct M values, especially for adjacent M values. For example, if both M = 6 and M = 8 are supported, the device cannot distinguish the M values used for R2D transmission. Therefore, some down-selections of these candidate M values can be considered. |
| Panasonic | Option 2 |
| Nokia | The support of all values may have a negative impact on the performance as the A-IoT device has to differentiate the M values. We think option 2 is a better way to go. |
| QC | We suggest following modification. We think we don’t need to start from 3 values. We can first study (i.e., evaluate performance) which M values can be detected reliably, and then, decide which M values to support. Note that RFID can select any data rate between its min and max. RFID data rate choice is quite flexible. For R2D, up to the maximum M value, RAN1 select M values from the following two options: - Option 1: all M values up to the maximum M value - Option 2: at least 3 of the M values for [low, medium, high] R2D chip/data rate subset of M values up to the maximum M value - FFS: the selection of detail M values is to be done can considering at least the following aspects - Performance and SNR distribution cross under different M values - Transmission efficiency together with minimum Btx,R2D - Ambiguity of device detecting different M values - Integer or non-integer samples per R2D OOK-4 chip - FFS: whether need more M values |
| vivo | Option 1 As for further down selection below Max M value, option 1 is supported to provide sufficient flexibility in time domain resource for R2D transmission. We prefer also add bullet ‘flexibility in time domain resource for R2D transmission’, if option 2 is kept. Regarding ‘Ambiguity of device detecting different M values’, in our understanding it depends on the preamble design for R2D, if SIP and CAP part can provide sufficient timing information to determine M value for follow-up detection, this ambiguity issue should not be considered. For the ‘Integer or non-integer samples per R2D OOK-4 chip’, - We do not see this as a real issue, since length difference among chips is negligible at transmitter side, and the non-integer samples per OOK4 chip is eliminated by transient period between ON and OFF chip from transmitter perspective. - Besides, if the number of samples is counted at AIoT device side, it is not possible to always ensure the same number of samples per chips, due to sampling rate is up to implementation and may varies due to clock accuracy. |
| Huawei, HiSilicon | Option 2. Selecting all values in the TR up to max(M) produces a set which does not permit good detection performance in terms of false-detection rate at the device, due to close spacing of the chip length, particularly in the presence of device 1 level of SFO. |
| Xiaomi | We prefer option 2. Some of the adjacent M values are very close to each other in the performance evaluation, and not all of these M candidates need to be supported. |
| FUTUREWEI | We support Option2 to have at least 3 M values. Considering the longest low power segment is 12.5 us in RFID, M=2 is a good starting value for the minimal M value. And M=32 (or 24) is a good starting value to consider for the maximal M value, considering the shortest low power segment in RFID is 3.125 us. In addition to the minimal and the maximal M values, another middle range M could be chosen. |
| Ericsson | We also prefer option 2. |
| Tejas Networks | We prefer option 2 including more parameters such as: - Received signal strength of the UL message signals received form the device - Block error rate - Data rate and latency |
| Lenovo | We prefer option 2, to limit the allowed M values that cover low, medium and high data rate. |
| ZTE, Sanechips | In our views, we are not sure how “3” is determined. Therefore, we suggest Option 2 is modified as below, i.e., starting point is 2 values, further study whether to include more values. Furthermore, we think it is also worthy to study whether to support similar data rate with D2R. And the need of evaluation of the SNR distribution requires further justification. #### [Open][H] Proposal 4.2.1b-v1: For R2D, up to the maximum M value, RAN1 select M values from the following two options: - Option 1: all M values up to the maximum M value in TR - Option 2: at least 2 3 of the M values, i.e. the maximum M value and M=1 for [low, medium, high] R2D chip/data rate - FFS: the selection of detail M values can consider at least the following aspects - Performance and SNR distribution cross different M values - Transmission efficiency together with minimum Btx,R2D - Ambiguity of device detecting different M values - Integer or non-integer samples per R2D OOK-4 chip - Whether to support comparable data/chip rate with D2R - FFS: whether need more M values |
| OPPO2 | OK with the proposal. |
| Samsung | We prefer option 2. In addition, we think the value of 3 can be in bracket to make the proposal more open and flexible, and the purpose can be an example as e.g. for [low, medium, high] R2D chip/data rate. |
| DOCOMO | For option 1, it should be clarified all the M values in TR38.769 otherwise it is unclear all the M values intend to support all the integer values up to max M etc. For first sub-bullet for option 2, CP handling method should be considered. For example, according to our observation, M=2 may not work for CP handling Type 2 Alt.1-1. |
| CATT | We prefer Option 1. There is no need to limit 3 M values in our view. |
| FL | Update to v2 according to the inputs. Please continue to v2. |

Based on the 1st offline discussion, FL further update the proposal and merge it into Proposal 4.2.1a-v2.

BW associated with M values

The BW associated with M values in TR 38.769 is the minimum transmission bandwidth Btx,R2D. FL observes from the inputs to this meeting, 4 companies [Huawei, CATT, CTC, CMCC] propose minimum transmission bandwidth Btx,R2D in their contributions. Among of them, 3 company [Huawei, CATT, CTC] propose no need to define maximum transmission bandwidth Btx,R2D or no need to define the potential guard band for R2D transmission where it can be up to reader implementation or left to RAN4 work based on the different network deployment and interference scenarios. Alternatively, 1 company [ZTE] propose to define the maximum Btx,R2D # of PRBs for different M values while 1 company [NTT DOCOMO] propose to FFS: Maximum number of PRB for each M value is 12. 1 company [EURECOM]propose to limit the number of possible transmission BW configurations, e.g. to one low, medium and high BW configuration but it is not clear to FL which bandwidth is the intention. On the other hand, 1 company [CMCC] propose the recommended minimum Btx,R2D number of PRBs for M=16 and M=24 can be updated to 3 and 4 respectively.

FL thinks we do not need to spend time updating minimum Btx,R2D values when it is probable that a number of affected M values will be removed. Hence, FL will return to setting the minimum number of PRBs after progress on M values. There are some discussion whether need to specify maximum Btx,R2D in addition to minimum Btx,R2D.

Based above, FL suggests the following proposal:

[Closed][H] Proposal 4.2.2a-v1:

Proposal: RAN1 does not specify the exact number of PRBs a reader shall use for Btx,R2D.

- For R2D, specify the minimum Btx,R2D # of PRBs associated to each M value.

- For R2D, for the maximum Btx,R2D # of PRBs down-select from the followings:

  - Option 1: No need to specify the maximum Btx,R2D # of PRBs in RAN1

    - This does not preclude RAN4 to discuss the maximum Btx,R2D # of PRBs themselves when defining requirement(s), if needed.

  - Option 2: Specify the maximum Btx,R2D # of PRBs and FFS from the followings

    - A single maximum Btx,R2D # of PRBs for all M values

    - A single maximum Btx,R2D # of PRBs for each M value

| Company | Views |
| --- | --- |
| Spreadtrum | General fine with this proposal. We prefer Option 1. |
| CMCC | Considering the maximum Btx,R2D # of PRBs, we prefer Option 1. |
| Panasonic | Support |
| Nokia | We think anyway one of WG should define the maximum BW, which is also related to the performance. In our view, we just need to decide a single maximum value that the system support. |
| QC | We think minimum is more important to RAN1 perspective. We think minimum value better to be increased than current values, if possible, to provide certain level of performance. For maximum, instead of making it open ended, RAN1 can decide a number as maximum PRBs to support. So, we prefer Option 2 - A single maximum Btx,R2D # of PRBs for all M values. |
| vivo | Prefer option 1. The max Btx,R2D # of PRBs seems belong to system parameter that the max BW can be allocated to AIoT R2D, which is typically RAN4 topic. |
| Huawei, HiSilicon | OK. Option 1. With an RF-ED at the device receiver, it is not necessary to specify the maximum number of PRBs. It can be left to the deployment and operator to decide the appropriate trade-off between R2D waveform quality for edge detection at the device, and the available spectrum resources. |
| Xiaomi | Fine with the proposal. For the maximum Btx,R2D # of PRBs, Option 1 is preferred. |
| FUTUREWEI | We are ok with the proposal and prefer Option 1. |
| Ericsson | We prefer option 1. |
| Tejas Networks | We prefer option 2. |
| Lenovo | Prefer Option 1 |
| ZTE, Sanechips | okay |
| LGE | Support Option 1. There is no clear benefit about Option 2. If there is a necessity about defining maximum # of PRBs for R2D, |
| OPPO2 | Prefer option 1. |
| Samsung | OK with the minimum value case. Considering time limitation, we prefer Opt 1 for maximum value case. |
| DOCOMO | Support this proposal. For max bandwidth, we prefer to keep both option at this point. |
| CATT | Fine with the proposal. We prefer Option 1. |
| FL | During the 1st offline session, the following is offline consensus. The newly added Note is trying to address Spreadtrum concern to Proposal 2.2.2a-v1 |

#### [Open][H] Proposal 4.2.2a-v2:

Proposal: RAN1 does not specify the exact number of PRBs a reader shall use for Btx,R2D.

- For R2D, specify the minimum Btx,R2D # of PRBs associated to each M value.

  - Note: Reader can transmit any Btx,R2D subject to this minimum Btx,R2D and the agreed subcarrier spacing of 15 kHz

- For R2D, no need to specify the maximum Btx,R2D # of PRBs in RAN1

  - This does not preclude RAN4 to discuss the maximum Btx,R2D # of PRBs themselves when defining requirement(s), if needed.

| Company | Views |
| --- | --- |

Note that 1 company [China Telecom] proposes that from RAN1 perspective, no need to define the potential guard band for R2D transmission. It can be left to RAN4 work or up to implementation based on the different network deployment and interference scenarios.

Based on above, FL suggest the following proposal:

[Closed][M] Proposal 4.2.2b-v1:

For R2D, Bocc,R2D is not specified in RAN1.

- This does not preclude RAN4 to discuss Bocc,R2D themselves when defining requirement(s), if needed.

| Company | Views |
| --- | --- |
| Spreadtrum | Fine with this proposal. However, we prefer occupy bandwidth can be integer multiple(s) of 180kHz as we mentioned in Proposal 2.2.2a-v1. |
| CMCC | Support. For the occupied bandwidth, the potential guard band is used to ensure that the interference between A-IoT and NR is acceptable. The coexistence issue and the amount of guard band is up to RAN4 study. |
| Panasonic | Support |
| Nokia | OK |
| QC | We don’t need to make an agreement for now. Let RAN4 start discussion and see whether RAN1 needs to do anything on this. |
| vivo | Support. For R2D, Bocc,R2D, depends on SFO requirements, and percentage of power would be included in the bandwidth, which seems RAN4 expertise. Hence, no need to specify in RAN1. |
| Huawei, HiSilicon | Agree, this can be left to RAN4. |
| Xiaomi | OK. |
| FUTUREWEI | Ok with the proposal. |
| Ericsson | Fine with us. |
| Lenovo | Fine with the proposal |
| ZTE, Sanechips | Okay |
| OPPO2 | Support the proposal. |
| CATT | OK |
| FL | During 1st offline discussion, the following is the offline consensus. |

#### [Open][M] Proposal 4.2.2b-v1:

For R2D, Bocc,R2D is not specified in RAN1.

- This does not preclude RAN4 to discuss Bocc,R2D themselves when defining requirement(s), if needed.

| Company | Views |
| --- | --- |

R2D CP handling

Related proposals

| Source | Proposal |
| --- | --- |
| Ericsson | Observation 1 For R2D CP handling Method 1, at least for Alt 1, the device needs to be aware of the boundary of the OFDM symbol (i.e., the beginning of the OFDM symbol) to determine the CP location. Observation 2 Method Type 2 does not need to be aware of OFDM symbol boundaries, but this comes at the cost of spectrum inefficiency or the loss of subcarrier orthogonality. Proposal 1 Method Type 1 Alt 1 and Method Type 1 Alt 2 are considered UE implementations (and down-selection between Alt 1 and Alt 2 is up to implementation). Proposal 2 Method Type 2 Alt 1 is not supported (since it restricts scheduling and degrades spectral efficiency due to parity chips). Proposal 3 Method 2 Type Alt 2 is not supported (since it does not support subcarrier orthogonality, which prevents in-band deployment). |
| Futurewei | Observation 1: The CP handing methods need to support large M values such as 16, 24 and 32 to achieve a comparable or better R2D data rate compared to UHF RFID. Observation 2: The R2D Timing-Acquisition preamble waveform provides a reference point to Ambient IoT devices to detect the beginning of the NR OFDM symbol. The device uses this reference point to locate and discard the samples corresponding to the CP section. Observation 3: Due to SFO in the device, the location of the reference point may drift as the device receives the R2D transmission. As a result, the device may discard samples not corresponding to the assumed CP section and degrade the detection performance Observation 4: Method Alt M1-2 is simple to implement and does not need a reference point as the Alt M1-1. Observation 5: Method Alt M1-2 does not work for large M values when the CP length is not distinguishable from the length of OOK chip. Observation 6: Method Alt M1-2 will result in non-constant chip length that may degrade the decoding performance. Observation 7: Method Alt M2-1-2 may result in the duration of OOK chips at the beginning or end of an OFDM symbol different from other OOK chips and impact the device decoding performance. Observation 8: Method Alt M2-1-2 only works for cases where the OOK chip duration is longer than CP length. In another word Method Alt M2-1-2 does not work for large M (>14) values for SCS 15k Hz. Observation 9: Adopting Alt M2-1-1 and considering Manchester line-code codewords where the first n1 chip(s) and the last n2 chip(s) of the codewords are the same can help ensure consistent chip duration at the device mitigating the impacts of CP insertion. Observation 10: Adopting Alt M2-1-1 and considering a circular shift of the M chips’ samples before CP insertion at the reader side can help ensure consistent chip duration at the device and simpler CP handling by redundant chip(s) dropping. Proposal 1: For CP handling in R2D, adopt Method Alt M2-1-1 and consider: - For small M': Manchester line-code codewords where the first n1 chip(s) and the last n2 chip(s) of the codewords are the same. - For large M': circular shift of the M chips’ samples before CP insertion at the reader side in addition to chip skipping/dropping at the device side. |
| ZTE, Sanechips | Observation 1: CP handling methods of Alt M1-1 and Alt M1-2, i.e. removal of CP at device depends on device implementation so that no normative work is required. Observation 2: CP handling method type 1 (i.e., removal of CP at device) can mitigate the potential impacts of non-constant chip duration, but without any other enhancement, it will cause performance loss in the cases of inaccurate timing synchronization, timing error accumulation caused by false determination of the transition edge, especially for larger M values. Observation 3: Alt M2-1-1 doesn’t work in the case when M=2. In other cases, the overhead of the redundant chip(s) can be up to 33.3%. Observation 4: For Alt M2-1-2, it is assumed the R2D data transmission starts from the last chip in the first OFDM symbol and has no additional overhead, i.e., the first odd number of chip(s) in the first OFDM symbol can be reserved for preamble transmission. Observation 5: For Alt M2-1-2, in the case of M≤16 that CP length is less than or approximately equal to a chip duration, the CP concatenated with either the first or the second chip of the Manchester codeword doesn’t introduce false transition edge. Observation 6: For Alt M2-1-2 wherein codeword is always set across CP, when M≥24 and the CP length is larger than a chip duration, no additional edge is introduced by CP removal as long as the edge detection error does not exceed the length of chip. However, for Alt 1 of Method Type 1, the additional transition edge introduced by CP removal cannot be completely eliminated without any careful design of chip duration. Observation 7: Alt M2-2 does not retain subcarrier orthogonality which would cause a certain interference to NR UEs and prevent in-band deployment. Observation 8: Alt M2-1-2 has better BLER performance than Alt M1-1, and Alt M1-1 has error floor. - Proposal 3: It is necessary to ensure that no false transition edge is introduced under the case of device supporting CP removal. Proposal 4: Alt M2-1-2 is proposed to be used for CP handling because of the advantages that it does not introduce additional overhead and provides better performance. |
| Huawei, HiSilicon | Observation 7: Alt M1-1 is achievable within theimplementation complexity for device 1.Observation 8: Alt M1-1 is based on the last rising or falling edge included in the OFDM symbol. Observation 9: For Alt M1-1, CP length difference among OFDM symbols leads to only negligible performance loss even when M is as large as 24. Observation 10: Alt M1-2 is not a complete solution to CP handling, since it does not work for M values which result in CP length > chip length. Observation 11: Alt M2 assumes the device does not identify the CP location and remove/skip the CP part. Observation 12: Alt M2-1 suffers performance loss compared with Alt M1-1 for small M value due to the non-constant chip length around CP. The performance of Alt M2-1 is ~1dB worse than Alt M1-1 for M = 6. Observation 13: Alt M2-1 cannot work for large M value since the device cannot identify the CP location and will misunderstand the CP part as the information part. Observation 14: For R2D CP handling, Alt M2-2 will destroy the subcarrier orthogonality for in-band operation. Observation 15: For R2D CP handling, if Alt M2-2 is used for standalone operation, it certainly adds complexity to the system and may increase device implementation complexity to distinguish such difference or impact on the R2D receiving if such difference is agnostic to device. Proposal 11: For R2D CP handling, Alt M1-1 is supported since it can work for all M values. Proposal 12: For R2D CP handling, Alt M1-2 is a part of a likely implementation of Alt M1-1 for small values of M. Proposal 13: For R2D CP handling, Alt M2-1 is not supported for small M value due to the performance loss and increasing implementation complexity for the transmitter. Proposal 14: For R2D CP handling, Alt M2-1 is not supported for large M value since it cannot work. Proposal 15: For R2D CP handling, Alt M2-2 is not supported for all the deployment operations. |
| Spreadtrum, UNISOC | Observation 1: Method Type 1 (Alt M1-1 and/or M1-2) can be supported as an optional method by device implementation with no specification impact. Proposal 3: A mandatory solution (e.g., one of Method Type 2) for CP handling with specification impact should be adopted in R19 A-IoT. Proposal 4: CP is not be part of an OOK chip for definition. Proposal 5: Alt M2-1-1 should be supported for R2D transmission CP handling. |
| CATT | Observation 1: For Method Type 2-M2-2, existing mechanisms can resolve or mitigate the inter-system interference between the A-IoT system and the NR system as follows: - Solution 1: The NR DL/UL signal and the A-IoT R2D/D2R signal can be transmitted using TDM. - Solution 2: The legacy NR UE and A-IoT devices can be deployed in different deployment scenarios, such as deploying the A-IoT reader/devices indoor while the NR UEs outdoor. - Solution 3: Using large guard band to mitigate the interference between A-IoT system and NR system. - Solution 4: gNB could utilize some existing mechanisms, such as using the pulse shaping filter, to minimize the interference from the R2D signal/channel to the adjacent NR channels during the transmission. Observation 2: For Method Type 2-Alt M2-1-1, the overheads caused by the parity-check chip(s) (i.e. last chip(s)) and CP are 29.9% and 53.3% for M = 8 and 4, respectively, when the CP is not included in first chip. Proposal 1: Since the A-IoT D2R does not have the orthogonality with UL NR signal, NR A-IoT R2D signal would not need to maintain a strict orthogonality with legacy DL NR signal. Proposal 2: Method Type 2-M2-2, which extends the legacy OFDM symbol duration without NR CP operation, should be supported for R2D signal transmission due to its lower complexity and more efficient spectrum resource utilization compared to other methods. Proposal 3: The Method 1-1 and 1-2 of Type 1 CP handling, i.e. Removal of CP at device without explicitly specification of transmitter to simplify the A-IoT device detection should not be supported in Rel-19. Proposal 4: Method Type 2-Alt M2-1-1, where the first OOK chip(s) and the last OOK chip(s) in an OFDM symbol are the same, should not be supported due to the unnecessary signal overhead it introduces. |
| China Telecom | Proposal 4: For CP handling if only one solution is allowed, support to remove CP at device without specified transmit-side as a starting point, and details can be FFS. Proposal 5: For CP handling of A-IoT R2D transmission, only normal CP is supported. Proposal 6: For CP handling of Method Type 2, it needs to retain the subcarrier orthogonality. |
| CMCC | Observation 1: The interpretation of "one solution for CP handling" in the WID is that either subcarrier orthogonal solution or non-orthogonal solution is supported, and for a given M value, supporting multiple solutions are not allowed. Observation 2: Discussion on combination of different alternatives to solve the CP issues under different M values is in the scope. And it is more important to consider low and medium M value cases. Observation 3: The following Method Type 2, Alt M2-1-2.1 with M≤6 can avoid false rising/falling introduced by CP: - Alt M2-1-2.1: Change a=b≠c case to a=b=c case by changing OOK-4 M value to adjacent M+1 for OFDM symbol n. - where a, b and c is the last chip of symbol n, first chip of symbol n+1 and last chip of symbol n+1. Observation 4: The following Method Type 2, Alt M2-1-2.2 with M≤6 can avoid false rising/falling introduced by CP: - Alt M2-1-2.2: Odd number of chips of PRDCH is transmitted in the first OFDM symbols, while M value is kept unchanged during the remaining OFDM symbols. Proposal 1: For R2D CP handling of OFDM based OOK waveform, normal CP is considered in the study. Proposal 2: Do not support both orthogonal and non-orthogonal CP handling solution, down-selection from orthogonal and non-orthogonal CP handling solution is needed. Proposal 3: For CP handling, Alt. M2-2 which cannot retain subcarrier non-orthogonality is precluded. Proposal 4: For Method Type 2 with Manchester coding for R2D, Alt M2-1-2 can be realized by, - M2-1-2.1: Change a=b≠c case to a=b=c case by changing OOK-4 M value to adjacent M+1.The first OFDM symbol carries odd value M chips, and the following OFDM symbols carry even value M chips. - where a, b and c is the last chip of symbol n, first chip of symbol n+1 and last chip of symbol n+1. - M2-1-2.2: Odd number of chips of PRDCH is transmitted in the first OFDM symbols, while M value is kept unchanged during the remaining OFDM symbols. Proposal 5: For M≤6, support Method Type 2, Alt M2-1-2 to solve CP issues, both Alt M2-1-2.1 and Alt M2-1-2.2 can be further studied for down-selection. Proposal 6: For M≥8, Method Type 1 with Alt M1-1 can be supported. |
| vivo | Observation 1: Method Type 1 works effectively for all M values, and it is up to device implementation on Alt M1-1 and/or Alt M1-2 is/are used. There are no RAN1 specification impacts. Observation 2: Alt M2-1-1 implemented by inserting parity chip introduces additional overhead and extends R2D transmission duration, therefore reducing the resource efficiency and power efficiency. Observation 3: For small M values, Alt M2-1-2 alone can work. However, the presence of the CP decreases the chip estimation accuracy as well as synchronization performance. Observation 4: For large M values, Alt M2-1-2 must work together with Method Type 1 (CP removal at device side). However, if Method Type 2 must work together with Method Type 1, it violates the single CP handling solution required by WID. Observation 5: For Alt M2-1-2 with the first chip of R2D being placed at the end of the first OFDM symbol, the location of the last transition edge in a OFDM symbol is dependent on the carried information bits, therefore complicating device behaviour to determine the symbol boundary. Observation 6: For Alt M2-1-2 with the first chip of PRDCH being placed at the end of the preceding OFDM symbol, if the same power level/amplitude level should be maintained for all ON chips, Tx power changes across symbols, which would complicate reader power control, and decrease the effective SNR and thus restrict the coverage when gNB needs to transmit by maximum Tx power for deep coverage device; if the same symbol level Tx power should be maintained, the power level/amplitude level of ON chips changes depending on information bits, which makes it more challenging for device to identify ON-OFF transitions with a single threshold. Observation 7: For Method type 2 Method 2-2, non-orthogonality and interference are not significant issues. The main challenge is that signal generation without CP insertion may increase complexity when using an existing OFDM transmitter. Proposal 1: Method Type 1 is supported. It is up to device implementation on Alt M1-1 and/or Alt M1-2 is/are used. There is no RAN1 specification impacts. |
| Nokia | Proposal 2: The scope of RAN1 discussions for CP handling is normal CP only. Proposal 3: RAN1 should not pursue Alt M1-2 and Alt M2-2 to handle CP. Proposal 4: Before making decision on CP handling method, RAN1 should first decide the maximum value of M and a target or acceptable performance loss from CP handling from the device side. Proposal 5: Support CP handling at the device side such as Alt M1-1 or support CP handling method at the reader effective for both small and large M. |
| TCL | Observation 6: Assume that one resource block (RB) carries one bit and 14 symbols within one slot (e.g., 30kHz SCS), modulation order is 1 and coding rate is 1, then M equals 10 (one chip length=3.4us and CP=1.7us). Observation 7: If CP has high level and the first chip is bit 0 based on Manchester encoding, false judgement may happen at device side. Observation 8: For Method 1 to hand CP, SFO/CP length/CP-level/transition edge will impact the performance of recognizing CP. Observation 9: For R2D preamble, there is no any previous SFO information to compensate the impact of SFO. Observation 10: The end of CP could add time domain gap to compensate the impact of SFO based on the evaluation of SFO at reader side. Observation 11: If transition edge period is 10us and CP is 5us for edge envelop detection, CP on next OFDM symbol might be overlap with transition edge. In this case, device could detect only transition edge even CP has been received by device. Observation 12: From TR 38.769, there are two options for M2-1, including Alt M2-1-1 and Alt M2-1-2. Observation 13: If CP length is longer than chip length, device can identify CP and detect high level chip when same level can be considered between CP and first chip, however, false judgement for CP may happen at device side because of the existing SFO at device side when considering different level between CP and first chip. Observation 14: When M is larger than one value (e.g., 8), CP length will large one chip length, which will bring error detection at device side. Proposal 3: Discuss how/whether to calculate CP length based on NR CP calculation. Proposal 4: CP design without line code is supported for reducing specification efforts. Proposal 5: Support Method 1 to hand CP and some issues as below should be considered for WI, including -- the impact of SFO -- CP length design based on M value -- ON/OFF level and adjacent chip level -- the impact of transition edge Proposal 6: Consider time domain gap at the end of CP to solve the CP false distinguish to avoid the impact of SFO for M1-1. Proposal 7: Define minimum transition edge period for envelop detection based on CP length to avoid overlap between transition edge and CP. Proposal 8: Support CP is only ON level for Method 1 and Method 2. Proposal 9: Support CP length is fixed or feasible, which is up to the choose of M1-1 or M 1-2 to handle CP. Proposal 10: Support M2-1 is used for retaining subcarrier orthogonality. Proposal 11: Support M2-1-2 of M 2-1 to retain subcarrier orthogonality and down-select M2-1-1 of M2-1. Proposal 12: Consider time domain gap at the end of CP to solve the CP false distinguish to avoid the impact of SFO for M2-1. Proposal 13: Support CP length is not longer than one chip length for M2-1. Proposal 14: Support CP length is longer than one chip length if CP is high level while first chip is high level for M2-1. Proposal 15: Limited M value (e.g., M<8) should be considered for M2-1. Proposal 16: M2-2 should not supported for WI discussion. |
| OPPO | Observation 7: Compared with Method Type 2, Method Type 1 would introduce additional complexity to the implementation of device 1. Observation 8: Method type 1, 2-1, and 2-2 (with CP insertion)are not applicable to all M values considered in SI phase. Proposal 4: CP handling method is deferred until M values to be supported in Rel-19 are decided. |
| Tejas | Observation 2: In order to decode the PRDCH frame including the control information, the device should have the information of CP length of the OOK-4 modulated signal generated from OFDM modulator even before receiving the control information. Proposal 4: CP length can be derived as a function of M-value, which is detected from the length of clock-acquisition signal. The device estimates the M-value of the received PRDCH signal from the clock-acquisition signal and derive the CP length form the M-value. |
| Panasonic | Proposal 1: No specific CP insertion could be sufficient for device 1. |
| NEC | Proposal 2: For CP handling for R2D transmission, support Alt M2-1 of method type 2. |
| Sony | Proposal 2: For CP handling, RAN1 supports Alt M2-1, where a known signal shape in the tail-portion of the R2D signal is copied to the CP. |
| Xiaomi | Observation 1: Method type 1 imposes more stringent requirements on the device, yet it effectively simplifies the complexity on the reader side. Observation 2: For Alt-M1-1 in R2D CP handling, its performance remains to be further validated due to the challenges in establishing a reliable reference point and potential errors in its underlying assumptions. Observation 3: For Alt-M1-2 in R2D CP handling, its performance is significantly influenced by the configuration of the M value. To support Alt-M1-2, the device is also required to feature a more precise clock mechanism. Observation 4: Method type 2 mitigates the influence of false rising/falling edges on PRDCH reception, and overall, this approach has minor impact on device and reader implementation. The extent of this impact varies depending on the specific implementation schemes. Observation 5: For Alt-M2-1 in R2D CP handling, various implementations can be adopted, even for the same candidate alternative. Different implementation schemes exert distinct influences on protocols and have varying effects on the implementation in devices and readers. Observation 6: Alt-M2-2 in R2D CP handling lacks significant advantages and has a considerable impact on other NR transmissions, making it also unsuitable for A-IoT transmission. Proposal 2: RAN1 at least supports Alt-M2-1 in R2D CP handling, and the implementation of Alt-M2-1 could be down selected from the following cases. - Case1: Same OOK chip lengths within information part of an OFDM symbol - Case2: Vary OOK chip lengths within information part of an OFDM symbol - Case3: CP as the first OOK chip length within an OFDM symbol Proposal 3: RAN1 should not support Alt-M2-2 because of adverse effects on co-existence and implementation |
| Apple | Proposal 2: For R2D CP handling for OOK-4 with M>1, down-select between the following two options: - Option 1: The CP handling is left it to AIoT device implementation, and the Time Acquisition Signal (TAS) is designed such that it is agnostic to CP (i.e. it does not require special CP handling at the UE) and the device can derive the start of an OFDM symbol. - The device can handle the CP via a combination of Alt M1-1 and Alt M1-2. - Option 2: Alt Method Type 1 - FFS Alt 1-1 or Alt 1-2 - The device may still need to handle the CP using e.g. Alt M1-1. |
| Samsung | Observation 1: The impact of synchronization inaccuracy should be carefully considered and justified for each alternative of CP handling Method Type 1. Observation 2: Alt M1-1 is not appropriate for large M values due to sampling offset by not distinguishing LCP and SFO impacts. For instance, with M=32, a typical sampling offset of 25% ~ 90% of code chip length are observed for small payload size of 20 bits, and the sampling offset for large payload size e.g. 96 bits will further increase due to SFO impact, which may severely impacts R2D reception performance. Observation 3: Alt M1-2 is not appropriate for large M values, e.g. M>8, due to similar chip length and SFO impacts. For instance, with M=12 or M=16, the time length of LCP/NCP and code chip have only 0~2 sampling points difference and become non-distinguishable at receiver side. Proposal 1: Method type 1 with Alt M1-1, or Alt M1-2, or combination of both alternatives, is not preferred for large M values. For small M values, its performance can be further justified. Proposal 2: Method Type 2 with Alt M2-1-1 or Alt M2-1-2, performed by insert padding chips at both start and end OOK chips or only at end OOK chips, is preferred, especially for large M values. For small M values, the overhead and performance can be further justified. Proposal 3: For CP handling with large M values, Method Type 2 with Alt M2-1-1 or Alt M2-1-2 are supported. For CP handling with small M values, down-select between Method Type 1 with Alt M1-1/Alt M1-2/combination of both alternatives and Method Type 2 with Alt M2-1-1 or Alt M2-1-2. Further study the threshold of small/large M values. Proposal 4: Alt M2-2: Method Type 2 does not retain subcarrier orthogonality, is not preferred for Rel-19 A-IoT. Proposal 5: For the purpose of better solving CP handling issue, the start of R2D transmission should be aligned with NR half-slot boundary to facilitate the determination of underlying OFDM CP length and removal. |
| Fujitsu | Observation 1: - The usage of the CP of the AIoT R2D signal is mainly to protect the adjacent NR signal. - The CP inserted into the R2D signal does not help the detection of the R2D signal at all, because - The AIoT devices have no capability to utilize this orthogonality to remove the interference coming from the adjacent NR DL signal. - The orthogonality is in frequency domain rather than time domain; and, - The AIoT devices cannot demodulate the received signal in the frequency domain which obviously exceeds its capability. - The AIoT devices are equipped with RF ED only and without any high-quality passband filter. - The AIoT devices can neither extract the R2D signal from the mixed-up signal nor suppress the impact of the NR signal. Observation 2: Another potential advantage of inserting CP into the R2D signal may be to make sure that the timeline of producing NR DL signal can somehow be shared with the generation of the AIoT R2D signal that is beneficial to reusing the legacy network equipment to transmit AIoT R2D signal. Proposal 1: Do not support Alt M2-2 for the CP handling in Rel-19. Proposal 2: Support Alt M1-2 and Alt M2-1-2 for the CP handling in Rel-19 unless any infeasibility is confirmed. |
| LGE | Observation 1: Using Method Type 1 Alt 2 alone, Ambient IoT devices can handle the CP for M values up to 8. Observation 2: Using Method Type 1 Alt 2 combined with Method Type 1 Alt 1, Ambient IoT devices can also handle the CP with larger (>8) M values. Observation 3: Using Alt M2-1 combined with Alt M1-1 assuming Manchester coding is used for R2D transmission, Ambient IoT devices can handle CP for M values up to 24. Observation#4: Even with the combination of Alt M2-1 and Alt M1-1, false rising/falling edge occurs when M=32 because multiple chips are within CP duration. Proposal 1: Support Method Type 1 for CP handling method for Ambient IoT which can support larger (>8) M values with the combination of Alt M1-1 and Alt M1-2, and also provides better resource utilization with less specification impact. |
| MediaTek | Proposal 1: Regarding CP handling method in TR 38.769, method type 1 is supported with - Alt M1-2 for OOK-4 M ≤6 - Alt M1-1 for OOK-4 M >6 |
| Lenovo | Proposal 1: For CP handling, support removing CP at device without impact on transmitter. FFS: Down select between AltM1-1 and AltM1-2 for determining the CP length based on the required complexity. |
| Sharp | Observation 4: At least for small values of M (e.g. with chip lengths larger than CP duration), CP handling Method Type 1 is sufficient, and choice of Alt M1-1 or Alt M1-2, or any potential combination of Alt M1-1 and Alt M1-2, could be left to device implementation. Observation 5: For large values of M (e.g. with chip lengths comparable to or smaller than CP duration), more discussions are necessary on whether to adopt CP handling Method Type 1 or CP handling Method Type 2. Proposal 5: At least for small values of M (e.g. with chip lengths larger than CP duration), adopt CP handling Method Type 1. |
| HONOR | Observation 1: All the Alt M2-1 solutions are not feasible for larger M values, such as M=24. Observation 2: CP handling of Alt M2-2 does not retain subcarrier orthogonality which would cause interference to NR system, and increase the complexity of the transmitter. Observation 3: A-IoT devices can support Method Type 1 by implementing Alt M1-1 or Alt M1-2 depending on M value indicated by preamble without standard effort. Proposal 4: For CP handling, support Method Type 1 in which removal of CP at device side. |
| InterDigital | Proposal 5: Support removal of CP at the device without specified transmit-side. |
| Qualcomm | - Observation 1: For CP handling Method M1-1, poor clock accuracy affects the identification of CP location and removal of CP and degrade performance. - Observation 2: For CP handling Method M1-1, during CAP detection, device may need to adjust its internal clock or adjust internal clock tick threshold for the detected chip duration (i.e., M value) to make accurate removal of CP duration. - Observation 3: For CP handling Method M1-1, it may be hard to support M>=24 in AWGN channel. - Observation 4: For CP handling method M2-2, device cannot differentiate chips in CP and chips in actual OFDM symbols, which makes M2-2 fail in large M>=12. - Observation 5: CP handling method M2-1-1 is not applicable to the case that CP duration is longer than chip duration, e.g., M>12. - Observation 6: For CP handling scheme M2-1-2, the duration of chip before/after CP is dependent on the last information bit of the OFDM symbol. - Observation 7: For CP handling scheme M2-1-2, chip duration is not uniform. - Observation 8: For CP handling scheme M2-1-2, non-uniform chip duration or edge-to-edge duration makes it almost impossible to use embedded clocking information. - Observation 9: For CP handling scheme M2-1-2, varying center point of OOK bit makes it difficult to apply other detection schemes such as energy detection. - Observation 10: CP handling method M2-1-2 is not applicable to the case that CP duration is longer than chip duration, e.g., M>12. - Observation 11: CP handling Method Type 1 is implementation-based solutions. - Observation 12: CP handling method type 1 have burden mostly at device side. CP handling Method Type 2 has burden mostly at reader side. - Observation 13: Choice of CP handling scheme decides OOK chip duration. Proposal 1: For CP handling scheme, RAN1 to prioritize CP handling methods of Alt M1-1 and Alt M2-2. |
| NTT DOCOMO | - Proposal 8: For PRDCH, support CP handling Method Type 1. Proposal 9: Discuss how a device determines the OFDM symbol boundary for CP handling of PRDCH reception. E.g., Counting of samples from start/end of R-SIP/CAP of preamble, i.e., start/end of R-SIP/CAP is aligned with OFDM symbol boundary. Proposal 10: For PRDCH, for both CP handling Method Type 1 and Type 2, the impact on the difference of short CP symbol and long CP symbol, i.e., 16κ per 0.5ms, should be considered. |
| WILUS | Observation 1: When the sampling rate is 1.92 Msps, the number of samples per chip is not uniform across different M values. Specifically, for M = 6, 12, and 24, the number of samples per chip is inconsistent, which may introduce challenges in processing. Observation 2: For M = 16, 24, and 32, the chip length is smaller than the CP length, which can lead to additional edges occurring within the CP duration. These additional edges can degrade performance when using CP handling Alt.M1-1, as they may introduce discontinuities or increased interference. - Proposal 1: Support CP Handling Alt.M1-1 with parity chip depending on M values |
| IIT Kanpur | Observation 1: Method Type Alt M2-1-1 and Alt M2-1-2 can be implemented by restricting the M values to a small number X where the (X=1,2,4). Observation 2: Alt M2-1 could be implemented by adding one or more guard bits at the start and end of the OFDM symbol boundary. The guard bits can be selected based on the M value, though this would increase overhead and reduce spectral efficiency. This solution is independent of the M value. Observation 3: Implementing Alt M2-2 would increase transmitter complexity, as it would require a separate RF chain to transmit AIoT and NR signals. Additionally, this solution could lead to interference between the NR and AIoT systems if an appropriate guard band is not used. Observation 4: The SCS should be fixed for AIoT operation to facilitate device-side CP removal. The device must be aware of the symbol duration and the start-end boundaries of OFDM symbols to effectively remove the CP at the receiver. Observation 5: At least for device 1, the device-side solutions, such as, are not suitable for large M values. Additionally, the SFO experienced by device 1 would make the solution under method 1 less effective. For large M values, the chip duration may also become shorter than the CP duration, leading to potential false detection. Proposal 1: Method Type Alt M2-1-1 and Alt M2-1-2 should be considered for CP handling in R2D transmissions. Proposal 2: The number of chips, i.e., M, should be restricted to a small number of X where (X=1,2,4). Proposal 3: For in-band AIoT operation SCS should be fixed. Preferable 15KHz. Proposal 4: Device-based CP handling solutions such as Alt M1-1 and Alt M1-2 should be de-prioritized. |
| IITM | Proposal 1: For CP handling, Method type 1 should be prioritized since Method type 2 might incur additional complexities on the reader and the device side. |

Discussion

CP handling methods

For CP handling, the following candidate methods are studied as per TR 38.769.

- Method Type 1: Removal of CP at device without specified transmit-side. - Alt M1-1: Device assumes same CP length for each OFDM symbol, i.e. does not distinguish exact CP length among different OFDM symbols - Alt M1-2: Duration between transition edges is utilized by device to determine CP location/length, i.e. if the duration appears to be invalid based on known chip duration - Method Type 2: Ensure the CP insertion of OFDM-based waveform will not introduce false rising/falling edge between the last OOK chip in OFDM symbol (n-1) and the first OOK chip in OFDM symbol n. - Alt M2-1: Method Type 2 retains subcarrier orthogonality, i.e. CP is copied from the end of an OFDM symbol - Alt M2-1-1: The first OOK chip(s) and the last OOK chip(s) in an OFDM symbol are the same. - Alt M2-1-2: Ensure a transition edge occurs only at the start or only at the end of the CP, and no transition edge occurs during the CP. - Alt M2-2: Method Type 2 does not retain subcarrier orthogonality.

During the SI, details observations from companies to each method alternative can be found in TR 38.769.

For CP handling methods, the support and the main concern of each of them from the inputs to this meeting are summarized as follows:

- Method Type 1:

  - 19 companies [Ericsson, Huawei, China Telecom, CMCC, vivo, Nokia, TCL, Sharp, Apple, Fujitsu, LGE, Lenovo, MediaTek, HONOR, InterDigital, NTT DOCOMO, Qualcomm, WILUS, IITM] support at least one of Method Type 1. Among them, most of companies propose that Alt M1-1 and/or Alt M1-2 can be up to device implementation where [Spreadtrum] also mentioned this. In addition, one company [Sharp] propose Method Type 1 at least for small M values while one company [CMCC] propose Method Type 1 with Alt M1-1 for large M values.

  - On the other hand, 6 companies [ZTE, TCL, Samsung, IIT Kanpur, Futurewei, Xiaomi] show concerns that CP handling method type 1 (i.e., removal of CP at device) will cause performance loss due non-perfect CP removal by SFO impact, thus not preferred for larger M values.

    - Alt M1-1: 3 companies [Futurewei, Xiaomi, Samsung] report that due to SFO in the device, the device may non-perfectly discard CP and degrade the detection performance.

    - Alt M1-2: 5 companies [Futurewei, Huawei, Xiaomi, Samsung, LGE] Method Alt M1-2 does not work for large M values when the CP length is not distinguishable from the length of OOK chip or significantly influenced by the M value. 1 company [Nokia] report that RAN1 should not pursue Alt M1-2.

- Method Type 2 Alt M2-1:

  - 11 companies [Futurewei, Spreatrum, CMCC, ZTE, TCL, NEC, Sony, Xiaomi, Fujitsu, Samsung, WILUS] support at least one of Method Type 2 Alt 2-1.

  - On the other hand, 4 companies [Huawei, HONOR, Futurewei, vivo] report that Alt M2-1 solutions does not work for larger M values. 1 company [Huawei] report that Alt M2-1 suffers performance loss compared with Alt M1-1 for small M value.

    - Alt M2-1-1: 3 companies [ZTE, CATT, vivo] report that the overhead of the redundant chip(s) introduces overhead up to 53.3% and extends R2D transmission duration, therefore reducing the resource efficiency and power efficiency. 1 company [ZTE] observes that Alt M2-1-1 doesn’t work in the case when M=2. 1 company [Qualcomm]

    - Alt M2-1-2: 2 companies [Futurewei, vivo] report Method Alt M2-1-2 does not work for large M values. 1 company [Futurewei] report it may result in the duration of OOK chips at the beginning or end of an OFDM symbol different from other OOK chips and impact the device decoding performance. 1 company [Qualcomm] report varying center point of OOK bit makes it almost impossible to use embedded clocking information and difficult to apply other detection schemes such as energy detection. 1 company [vivo] report for Alt M2-1-2 with the first chip of R2D being placed at the end of the first OFDM symbol, the location of the last transition edge in a OFDM symbol is dependent on the carried information bits, therefore complicating device behaviour to determine the symbol boundary. 1 company [vivo] report for Alt M2-1-2 with the first chip of PRDCH being placed at the end of the preceding OFDM symbol leads to unequal numbers of ON and OFF chips in a OFDM symbol, which changes with information bits in the OFDM symbol. If the same power level/amplitude level should be maintained for all ON chips, Tx power changes across symbols, which would complicate reader power control, and decrease the effective SNR and thus restrict the coverage when gNB needs to transmit by maximum Tx power for deep coverage device; if the same symbol level Tx power should be maintained, the power level/amplitude level of ON chips changes depending on information bits, which makes it more challenging for device to identify ON-OFF transitions with a single threshold

- Method Type 2 Alt M2-2:

  - 3 company [CATT, Panasonic Quacomm] explicitly or implicitly propose Alt M2-2 which does not retain subcarrier orthogonality while 1 company [Qualcomm] among them also propose to prioritize Alt M1-1.

  - On the other hand, 25 companies [Ericsson, Futurewei, ZTE, Huawei, Spreadtrum, CTC, CMCC, vivo, Nokia, TCL, NEC, Sony, Xiaomi, Apple, Samsung, Fujitsu, LGE, MediaTek, Lenovo, Sharp, HONOR, InterDigital, NTT DOCOMO, WILUS, IIT Kanpur] are only proposing CP handling method which retains OFDM subcarrier orthogonality (from Alt M1-1, M1-2, M2-1-1, M2-1-2). Among of them, 11 companies [Ericsson, ZTE, Huawei, CTC, CMCC, vivo, Nokia, TCL, Xiaomi, Fujitsu, HONOR] explicitly propose to not support Alt M2-2 which does not retain subcarrier orthogonality.

Based on above, FL suggests the following proposal:

[Closed][H] Proposal 5.2.1a-v1:

For R2D CP handling, Alt M2-2 (as per TR 38.769) is deprioritized in Rel-19.

| Company | Views |
| --- | --- |
| Spreadtrum | Support. |
| CMCC | Support |
| Panasonic | In our understanding, the merit of CP is not for the orthogonality but rather some of transmitter chain of reader can be reused. As the complexity of Tx chain of reader would be quite simple process, and considering the hardware simplicity for device 1, we think to have M2-2 would be beneficial for device 1. |
| Nokia | OK |
| QC | Although M2-2 does not maintain orthogonality, what matters is the amount of inter carrier interference (ICI) generated from it. Given that we consider indoor reader and pathloss and penetration loss from indoor reader to outdoor UEs, we see that the impact of ICI is not that severe. TxWOLA and RxWOLA could significantly reduce ICI impact. We see that M2-2 is clean slate design, which makes easier device implementation; there is nothing to do in device side. So, we recommend to consider M2-2 together with M1-1. |
| Huawei, HiSilicon | Support. We do not support a design which introduces interference between A-IoT and NR, due to non-orthogonality, when orthogonality without interference could have been achieved. The ability to use the same BS hardware to generate A-IoT together with NR is a simplifying advantage compared to a design that presumes hardware differentiation. |
| Xiaomi | OK. |
| FUTUREWEI | Support the proposal We need to correct some text above. See below - Method Type 2 Alt M2-1: 11 companies [Futurewei, Spreatrum, CMCC, ZTE, TCL, NEC, Sony, Xiaomi, Fujitsu, Samsung, WILUS] support at least one of Method Type 2 Alt 2-1. On the other hand, 4 companies [Huawei, HONOR, Futurewei, vivo] report that Alt M2-1 solutions does not work for larger M values. In our contribution we indicate that only Alt M2-1-2 does not work for large M. In fact, in our contribution, the method M2-1-1 we propose works for large M. |
| Ericsson | We support this proposal. Alt M2-2 since it does not support subcarrier orthogonality, which prevents in-band deployment |
| Tejas Networks | Support |
| Lenovo | Support deprioritizing Alt M2-2 |
| ZTE, Sanechips | Okay. |
| LGE | Support the proposal. |
| OPPO2 | For M2-2, it at least could be applied for all the M values, and would not introduce false rising/falling edge. If ‘only one solution for CP handling’ means only one method could be selected for all the M values, M2-2 should also be considered. |
| Samsung | Support |
| DOCOMO | OK |
| CATT | Not support. The evaluation results have shown that Alt M2-2 does not necessarily bring a significant degradation of the performance. On the other hand, it provides significant benefits in term of the design and the implementation, including the consistency between R2D and D2R design. |
| FL | Update as the tentative version from the Chair notes |
| Vivo2 | It seems FL may have overlooked one of our observations on Alt M2-1-2, could you please add the following observation to the summary so that companies can consider in the next meeting? Alt M2-1-2 with the first chip of PRDCH being placed at the end of the preceding OFDM symbol leads to unequal numbers of ON and OFF chips in a OFDM symbol, which changes with information bits in the OFDM symbol. If the same power level/amplitude level should be maintained for all ON chips, Tx power changes across symbols, which would complicate reader power control, and decrease the effective SNR and thus restrict the coverage when gNB needs to transmit by maximum Tx power for deep coverage device; if the same symbol level Tx power should be maintained, the power level/amplitude level of ON chips changes depending on information bits, which makes it more challenging for device to identify ON-OFF transitions with a single threshold |
| FL | To Vivo2, thanks for the reminder, added. |

#### [Open][H] Proposal 5.2.1a-v2:

For R2D CP handling, Alt M2-2 (as per TR 38.769) is not considered in Rel-19.

| Company | Views |
| --- | --- |

For CP handling Method Type 1, as per TR 38.769, it studied that “Method Type 1, at least for Alt M1-1, device needs to be aware of or determine the boundary of an OFDM symbol (i.e. the beginning of an OFDM symbol) to determine CP location, including the start and the length of CP.” For this meeting, some companies further analyse that Alt M1-2 does not need to be aware of or determine the OFDM symbol boundaries. FL observes that 3 companies [Apple, Qualcomm, NTT DOCOMO] further analyze how device is aware of or determines the OFDM symbol boundaries. Where [Apple] mentioned the Time Acquisition Signal (TAS) is designed such that it is agnostic to CP (i.e. it does not require special CP handling at the UE) and the device can derive the start of an OFDM symbol. [Qualcomm] mentioned the first CP depends on the shape of the last bit in the first OFDM symbol. Thus, depending on design of last of CAP and CP, the boundary may or may not have edge. [NTT DOCOMO] propose that e.g., counting of samples from start/end of R-SIP/CAP of preamble, i.e., start/end of R-SIP/CAP is aligned with OFDM symbol boundary. Some companies also mentioned for Alt M1-2, device does not need to be aware of or determine the boundary of an OFDM symbol.

For CP handling Method Type 2 Alt M2-1, some companies propose detail design for Alt M2-1-1 and Alt M2-1-2. Some of the design seems requiring combination of Method 1 otherwise it can’t support large M values e.g. [Futurewei] propose M2-1-1 but it requires chip skipping/dropping at the device side. [ZTE] propose M2-1-2 but it seems using Method 1-1 in the meantime at device side. [vivo] observe that for large M values, Alt M2-1-2 must work together with Method Type 1 (CP removal at device side). [Ericsson, Huawei] observe that no special CP handling is required at the A-IoT device side and Method Type 2 does not need to be aware of OFDM symbol boundaries and the device does not identify the CP location and remove/skip the CP part. Some companies [CMCC, Fujitsu, WILUS] directly propose combination of Method 1 and Method 2-1. The details design for CP handling Method Type 2 Alt M2-1 are divergent from different companies, while some companies only indicate their preference of Alt M2-1-1 or M2-1-2 or M2-1 but not provide design.

Based on above, FL suggests the following proposals:

[Closed][H] Proposal 5.2.1b-v1:

For R2D CP handling,

- At least Method Type 1 (as per TR 38.769) is supported for M value larger than 12

  - Alt M1-1 and/or Alt M1-2 is up to device implementation

  - The device is aware of or determines the boundary of an OFDM symbol using the R2D timing acquisition signal (R-TAS), at least for Alt M1-1

    - Detail is up to R2D preamble design

- FFS: whether/how to additionally support Method Type 2 Alt M2-1-1 or Alt M2-1-2 (as per TR 38.769)

| Company | Views |
| --- | --- |
| Spreadtrum | We have strong concern for Alt M1. For Alt M1, there are multiple detailed implementation methods. For different M, the performance of these methods has very large differences, and the performance is different to guarantee. Both for Alt M1-1 and M1-2, can be supported as an optional method for CP handling from implementation perspective. The performance of this method is highly depended on the implementation of the manufacturer. According to WID, “one solution for CP handling” means one mandatory solution should be specified to guarantee good performance for different M and have capability to eliminate the impact of SFO. Hence, specification-based method should be considered, e.g., M2-1-1 or M2-1-2. In short, we propose to firstly discuss one specification-based solution. |
| CMCC | First we want to clarify that our understanding on “one solution for CP handling” is either subcarrier orthogonal solution or subcarrier non-orthogonal solutions in the TR is supported. In addition, for a given M value, only one CP handling alternative is supported and supporting multiple solutions are not allowed. In this sense, it is still allowed to discuss a combined solution of different alternatives in the TR to solve the CP issue under different M values. Regarding Method Type 1 for larger M values (e.g., M>8), the device can use Alt. M1-1 to determine the CP location and remove it as the chip duration is less than the CP duration. For smaller M values, as it corresponds to more practical R2D data rate, we believe that some optimization can be considered that the CP handling solution is not required additional implementation complexity for device, and Alt M2-1-2 can be supported with detailed design. |
| Panasonic | We are not supportive of M1. As the CP remove is purely device procedures without specific design from transmitter or signal design perspective, there might be some mis-detection when CP is not properly removed. |
| Nokia | We first would like to discuss the WID description about the “single solution”. Are we going to support different methods depending on M values ? Given that the Alt M1-1 is the solution leaning on the device implementation, we are okay to discuss a specified solution below. |
| QC | One solution It would be good to clarify “one solution” first. It seems that different companies have different views on this. M1 (no use CP part for OOK) vs M2(use CP part for OOK) M1-1/1-2 are implementation issue. So, in RAN1, what we need to decide is whether to use CP portion or not for ambient IoT purpose. If CP part is used, then, solution goes with M2. If CP part is not used, then, solution goes with M1 and we leave CP handling as implementation issue at device side. Combination of CP handling schemes Supporting combination of schemes depending on M value increases complexity at device. So, we prefer not to go to this direction. |
| vivo | Basically we are fine with the first bullet. But the WID requires a single CP handling method, so we first need to align the understanding of this requirement. Our interpretation is that a single method from the following should be adopted for all M values. According to our previous evaluation in the SI phase, the performance of Method Type 1 is good for both small and large M values. Therefore, Method Type 1 should also be supported for M ≤ 12. On the other hand, using different methods for different M values is not preferred. The R2D Preamble also involves CP handling and should follow the same CP handling method as the PRDCH. However, at the beginning of the Preamble detection phase, the device does not yet know the M value and must perform blind detection. Since Method Type 2 alone does not work for large M values (e.g., when the CP length exceeds the chip duration), it must be combined with CP removal at the device side, which in our understanding also violates the WID. Even if the combination is allowed, adopting an M-dependent method (e.g., using Method Type 1 for some M values and a combination of Type 1 and Type 2 for others) would increase the complexity of device detection and lead to potential M confusion. Therefore, FFS should be removed #### [Open][H] Proposal 5.2.1b-v1: For R2D CP handling, - At least Method Type 1 (as per TR 38.769) is supported for M value larger than 12 all M values - Alt M1-1 and/or Alt M1-2 is up to device implementation - The device is aware of or determines the boundary of an OFDM symbol using the R2D timing acquisition signal (R-TAS), at least for Alt M1-1 - Detail is up to R2D preamble design FFS: whether/how to additionally support Method Type 2 Alt M2-1-1 or Alt M2-1-2 (as per TR 38.769) |
| Huawei, HiSilicon | We agree with this proposal. Alt M1 works effectively and is very simple. In our Tdoc R1-2500153 Figure 5, the simulation results show that Alt M1-1 can achieve 10-1 BLER under M=24, TBS=20bit, [104 ppm, 105 ppm] SFO, etc. As shown in Proposal 5.2.1c-v1, Alt M2x is unclear now, proponents need to provide more details about the designs so that RAN1 can further discuss. It’s hard to discuss at high level. |
| Xiaomi | We have concerns on this proposal for the following reason: Due to the limitation of peak power of device1, it is risky to remove CP through device 1. Since SFO is not considered for processing, sampling bias is a high-probability event. This makes the performance of Method Type 1 questionable. |
| FUTUREWEI | We do not support the proposal. According to the TR, unlike M1, M2 does not prohibit receive side operation in addition to transmit side. So the following highlighted statement from above text is not accurate - For CP handling Method Type 2 Alt M2-1, some companies propose detail design for Alt M2-1-1 and Alt M2-1-2. Some of the design seems requiring combination of Method 1 otherwise it can’t support large M values e.g. [Futurewei] propose M2-1-1 but it requires chip skipping/dropping at the device side. M2 will offer better performance and easier implementation |
| Ericsson | Overall, the proposal looks fine. We believe the down-selection between Alt 1 and Alt 2 should be left to implementation. Method Type 2 (Alt M2-1-1 or Alt M2-1-2) restricts the possible M-value, limits scheduling flexibility, and degrades spectral efficiency due to parity chips, so it can also be deprioritized. |
| Lenovo | Fine with the proposal |
| ZTE, Sanechips | For the case of larger M values, chip duration is smaller so that performance may be impacted by the false transition edge introduce by CP. We suggest that the feasibility of Method Type 1 for larger M values should be further evaluated. |
| LGE | Support the proposal. |
| OPPO2 | As per the WID only one solution for CP handling. In our views, this proposal means method type 1 is down-selected for all M values once supported one value of M larger than 12. It needs to clarify the understanding for ‘one solution for CP handling’. |
| Samsung | We disagree this proposal. As FL also captured, there are several companies show the drawback and performance loss of Method Type 1 for large M values. Purely Method Type 1, including Alt M1-1, or Alt M1-2, or combination of M1-1 and 1-2, can hardly satisfy the reliability requirement for M>12 cases. |
| DOCOMO | For the WID description “one solution for CP”, in our understanding, it was not well-discussed which interpretation is correct, e.g., single solution for PRDCH regardless of M value or single solution for PRDCH for each M value but can be different for different M value etc. In our view, if different CP handling is supported for different M value, it would complicate both device and reader operation, hence a single solution should be supported regardless of M value, otherwise WID revision is necessary in RANP. We suggest the following update for the first bullet; At least Method Type 1 (as per TR 38.769) is supported for all the M values larger than 12 |
| FL | According to the comments from 1st offline session, FL update the proposal and split it into two Proposal 5.2.1b-1-v2 and Proposal 5.2.1b-2-v2 |

#### [Open][H] Proposal 5.2.1b-1-v2:

If any part of R2D preamble needs CP handling, same CP handling solution applies for both R2D preamble and PRDCH.

| Company | Views |
| --- | --- |

#### [Open][H] Proposal 5.2.1b-2-v2:

For CP handling,

- For Method Type 1 (as per TR 38.769)

  - For purposes of evaluating Method Type 1, it is assumed the device is aware of or determines the boundary of an OFDM symbol using the R2D timing acquisition signal (R-TAS), at least for Alt M1-1

    - Detail is up to R2D preamble design

  - Using Alt M1-1 and/or Alt M1-2 is up to device implementation as per TR 38.769

  - FFS: applicable M values

- For Method Type 2 Alt M2-1 (as per TR 38.769)

  - Method Type 1 needs to be also used for CP handling at least when CP part of an OFDM symbol is longer than a R2D chip.

  - FFS: detail design of Alt M2-1-1 or Alt M2-1-2 [from the followings

    - Alt M2-1-1

      - Candidate 1:

        - For small M': Manchester line-code codewords where the first n1 chip(s) and the last n2 chip(s) of the codewords are the same.

        - For large M': circular shift of the M chips’ samples before CP insertion at the reader side in addition to chip skipping/dropping at the device side

      - Candidate 2:

        - Where a known signal shape in the tail-portion of the R2D signal is copied to the CP.

      - Candidate 3:

        - Insert padding chips at both start and end OOK chips or only at end OOK chips

    - Alt M2-1-1

      - Candidate 4:

        - Change a=b≠c case to a=b=c case by changing OOK-4 M value to adjacent M+1 for OFDM symbol n.

          - where a, b and c is the last chip of symbol n, first chip of symbol n+1 and last chip of symbol n+1.

      - Candidate 5:

        - Odd number of chips of PRDCH is transmitted in the first OFDM symbols, while M value is kept unchanged during the remaining OFDM symbols.]

  - FFS: applicable M values with or without combination of Method Type 1

- For further down-selection, companies are encouraged to provide evaluation among the followings to RAN1#120bis:

  - Method Type 1

  - Method Type 2 Alt M2-1-1 with or without combination of Method Type 1

  - Method Type 2 Alt M2-1-2 with or without combination of Method Type 1

| Company | Views |
| --- | --- |

#### [Open][H] Proposal 5.2.1c-v1:

Proponents of the following candidates are requested to check and update if necessary, the expression and list of candidates of the Alt M2-1, from their papers in this meeting.

- Candidate 1 [Futurewei Alt M2-1-1]:

  - For small M': Manchester line-code codewords where the first n1 chip(s) and the last n2 chip(s) of the codewords are the same.

  - For large M': circular shift of the M chips’ samples before CP insertion at the reader side in addition to chip skipping/dropping at the device side

- Candidate 2 [Sony Alt M2-1-1]:

  - Where a known signal shape in the tail-portion of the R2D signal is copied to the CP.

- Candidate 3 [CMCC Alt M2-1-2]:

  - Change a=b≠c case to a=b=c case by changing OOK-4 M value to adjacent M+1 for OFDM symbol n.

    - where a, b and c is the last chip of symbol n, first chip of symbol n+1 and last chip of symbol n+1.

- Candidate 4 [CMCC, ZTE, Alt M2-1-2]:

  - Odd number of chips of PRDCH is transmitted in the first OFDM symbols, while M value is kept unchanged during the remaining OFDM symbols.

- Candidate 5 [Samsung Alt M2-1-1]:

    - Insert padding chips at both start and end OOK chips or only at end OOK chips

Please indicate concrete description if you want to add/modify candidates:

| Company | Views |
| --- | --- |
| Spreadtrum | See our comments in Proposal 5.2.1b-v1. |
| Nokia | We prefer to have a solution feasible for overall candidate M values, which would be candidate 5. |
| Huawei, HiSilicon | For Alt M2x, it’s hard to discuss at high level. So we hope proponent can make their design clear and complete to help discussions. For example, for the current candidates, we feel the following aspects are unclear. == For candidate 1, n1 equals to n2 or not? If not, what does it mean “n1 … n2 are the same”? For candidate 2, “a known signal shape” means a fixed sequence in spec? how long is it? For candidate 3, “…by changing OOK-4 M value to adjacent M+1 for OFDM symbol n…”, what does this mean? How reader generate R2D signal using M+1? For candidate 4, how to implement “odd number of chips”? drop the last chip? Is this a typo “s” in ”first OFDM symbols”? For candidate 5, for small M (e.g., M=2), the overhead is too large? |
| FUTUREWEI | Correction here our method is M2-1-1, not M2-1-2 - Candidate 1 [Futurewei Alt M2-1-1]: Answer to Huawei: yes n1=n2 |
| Ericsson | Please see our comments in proposal 5.2.1b-v1. We believe Method Type 2 (Alt M2-1-1 or Alt M2-1-2) restricts the possible M-value, limits scheduling flexibility, and degrades spectral efficiency due to parity chips, so it can be deprioritized. There is no need to discuss the details of Method Type 2 Alt M2-1-1 or Alt M2-1-2 before “FFS: whether/how to additionally support Method Type 2 Alt M2-1-1 or Alt M2-1-2 (as per TR 38.769)”. |
| ZTE, Sanechips | We also support candidate 4. #### [Open][H] Proposal 5.2.1c-v1: Proponents of the following candidates are requested to check and update if necessary, the expression and list of candidates of the Alt M2-1, from their papers in this meeting. - Candidate 1 [Futurewei Alt M2-1-2]: - For small M': Manchester line-code codewords where the first n1 chip(s) and the last n2 chip(s) of the codewords are the same. - For large M': circular shift of the M chips’ samples before CP insertion at the reader side in addition to chip skipping/dropping at the device side - Candidate 2 [Sony Alt M2-1-1]: - Where a known signal shape in the tail-portion of the R2D signal is copied to the CP. - Candidate 3 [CMCC Alt M2-1-2]: - Change a=b≠c case to a=b=c case by changing OOK-4 M value to adjacent M+1 for OFDM symbol n. - where a, b and c is the last chip of symbol n, first chip of symbol n+1 and last chip of symbol n+1. - Candidate 4 [CMCC, ZTE Alt M2-1-2]: - Odd number of chips of PRDCH is transmitted in the first OFDM symbols, while M value is kept unchanged during the remaining OFDM symbols. - Candidate 5 [Samsung Alt M2-1-1]: - Insert padding chips at both start and end OOK chips or only at end OOK chips |

2 companies [China Telecom, Nokia] propose that only normal CP is supported for CP handling. FL think as per TR 38.769, 15 kHz SCS only support normal CP, thus dealing with proposals relating to extended CP is not needed unless an applicable SCS is agreed.

R2D chip duration

Related proposals

| Source | Proposal |
| --- | --- |
| Ericsson | Proposal 4 M chips correspond to M ON/OFF segments within one A-IoT OFDM symbol before CP insertion. |
| ZTE, Sanechips | Observation 9: If the number of sampling points is the ceiling/floor of (1/M) × {OFDM symbol's sample points excluding/including CP part}, the total of sampling points of M chips may not match the number of OFDM symbol's sample points excluding/including CP part. It will lead to non-orthogonality and misalignment issues. Observation 10: If Chip duration = (1/M) × {OFDM symbol duration including CP part}, the OFDM symbol with M chips may be non-orthogonal to NR and the chip duration is not a constant value in the cases of different CP lengths. Observation 11: If Chip duration = (1/M) × {OFDM symbol duration excluding CP part}, the total duration of M chips cannot align with the OFDM symbol if the number of IFFT points of an OFDM symbol cannot be evenly divided by M. For example, if all chip duration is determined by ceiling operation, the total length of M chips may be larger than an OFDM symbol duration, resulting into non-orthogonality issue. - Proposal 5: The chip duration for OOK-4 based OOK waveform should be discussed. The solutions of chip duration definition summarized in [3] can be considered as starting point. - Proposal 6: In order to ensure that the total length of M chip(s) is equal to the duration of OFDM symbol, - for the case of larger M values (e.g. M≥24), dedicated chip duration is introduced to ensure no additional transition edge introduced by CP, i.e. chip duration is equal to CP length. - for other cases, - if the number of IFFT points of an OFDM symbol can be evenly divided by M, chip duration = (1/M) × {OFDM symbol duration excluding CP part}; - if the number of IFFT points of an OFDM symbol cannot be evenly divided by M, chip duration = or , where . Proposal 7: Consider Calculation 5 for determining the chip duration. |
| Huawei, HiSilicon | Proposal 16: For R2D, the time unit, and minimum unit of resource allocation, is defined as the chip length, equal to 1/M OFDM symbol length without CP calculated as (1/SCS)/M. |
| Spreadtrum, UNISOC | Proposal 4: CP is not be part of an OOK chip for definition. |
| CATT | Proposal 5: The R2D chip duration should be defined as that of an OOK chip of OOK-4 signal, which is equal to 1/M of an OFDM symbol length (excluding the CP), where M≥1. |
| CMCC | Proposal 12: In R2D, a chip - Corresponds to one modulated symbol for OOK-4. - Chip duration = (1/M) × {OFDM symbol duration excluding CP part}. Proposal 13: For R2D link, from device perspective, floating chip duration that T0-∆T0≤Tchip≤T0+∆T0 is valid, and FFS ∆T0T0. |
| OPPO | Figure 2. Illustration of device detection for R-TAS and PRDCH Observation 9: For Method Type 2, from device perspective, after obtaining the OOK chip duration from clock-acquisition part, it would detect the PRDCH based the OOK chip duration without change, as illustration in Figure 2. Figure 3. Illustration of the signals of R2D transmission for reader and device Observation 10: For Method Type 2, Reader and device have different procedure for R2D transmission, as illustration in Figure 3. - At reader side, Reader would do the CP insertion when generate the waveform R2D transmission. - At device side, device detect the R2D transmission based the OOK chip duration obtained from the clock-acquisition part. Proposal 5: For Method Type 2, support the chip duration generation of OOK-4 for M-chip per OFDM symbol transmission be determined by: - M, and the length of OFDM symbol with CP Proposal 6: For Method Type 2, from device perspective, the OOK chip duration obtained from clock-acquisition part means the time duration of one OOK-on or one OOK-off chip after CP insertion. Chip duration is always constant. - When M=1, OOK chip duration always includes the CP part. - When M>1, OOK chip duration may include the CP part or not, i.e. chip duration include the CP part for the 1st OOK chip within one OFDM symbol, and not include the CP part for the other OOK chips within one OFDM symbol. Proposal 7: For Method Type 2, from reader perspective - If the OOK chip duration means the time duration of one OOK-on or one OOK-off chip within one OFDM symbol before CP insertion. Chip duration may or may not be constant. - When M=1, chip duration is constant. - When M>1, chip duration is not constant, the chip duration of 1st OOK chip is shorter than the other OOK chips within one OFDM symbol, i.e. the chip duration of 1st OOK chip + CP = the chip duration of other OOK chips within one OFDM symbol - If the OOK chip duration means the time duration of one OOK-on or one OOK-off chip within one OFDM symbol after CP insertion. Chip duration is constant. - When M>1, the 1st chip duration includes CP part, while others not include CP part. |
| Xiaomi | Observation 9: The duration corresponding to the M-value including the CP enhances the performance of R2D by extending the OOK chip length and mitigating ambiguities in OOK chip. Proposal 2: RAN1 at least supports Alt-M2-1 in R2D CP handling, and the implementation of Alt-M2-1 could be down selected from the following cases. - Case1: Same OOK chip lengths within information part of an OFDM symbol - Case2: Vary OOK chip lengths within information part of an OFDM symbol - Case3: CP as the first OOK chip length within an OFDM symbol Proposal 5: M-value corresponding to the duration containing CP at least need to be supported for R2D in RAN1. |
| LGE | Proposal 3: R2D chip duration is indicated in CAP of R-TAS. Proposal 4: The chip durations for L1 R2D control (if supported) and R2D data are the same. Proposal 5: The following two alternatives for indicating R2D chip duration in CAP can be considered. - Alt.1) CAP indicating the R2D chip duration directly indicates the R2D chip duration (without partitioning) - Alt.2) CAP indicating the R2D chip duration consists of two parts the combination of which indicates the R2D chip duration |
| Sharp | Observation 3: R2D chip length for R2D start-indicator-part (SIP) should correspond to a relatively large value of M, even if the value of M for the corresponding clock-acquisition-part (CAP) or PRDCH is small. Proposal 4: For R2D, different chips within one OFDM symbol can correspond to different values of M. |
| NTT DOCOMO | Proposal 3: For R2D, chip length can be defined depending on supported CP handling Method. - For CP handling Method 1, one chip length corresponds to (one OFDM symbol excluding CP)/M. - For CP handling Method 2, one chip length corresponds to (one OFDM symbol including CP)/M. |

Discussion

R2D chip duration

8 companies [Ericsson, Huawei, CATT, ZTE, Spreadtrum, CMCC, OPPO, NTT DOCOMO] propose that Option 1 from the followings that M chips correspond to M ON/OFF segments within one A-IoT OFDM symbol before CP insertion while among of them, 3 companies [OPPP, ZTE, NTT DOCOMO] propose some condition on this case. [ZTE] discuss 5 different ways of calculation of R2D chip duration and propose Option 3 for some cases as well. 2 companies [CMCC, ZTE] propose Option 2 from device perspective. In addition, 2 companies [Xiaomi, Sharp] discuss whether the R2D chip duration same or vary within one OFDM symbol while 1 company [LGE] propose that the chip durations for L1 R2D control (if supported) and R2D data are the same.

R2D chip duration is denoted as C, depending on CP handling method and/or M value, C is further down-selected from the followings:

- Option 1: C = (1/SCS) / M

  - Ericsson, Huawei, CATT, ZTE, Spreadtrum, CMCC, OPPO, NTT DOCOMO

- Option 2 (from device perpective): (1/SCS) / M - delta_C<= C <= (1/SCS) / M + delta_C

  - Where, delta_C is a floating chip duration

  - FFS: delta_C/C

  - ZTE, CMCC

- Option 3: C = ceil {(1/SCS) / M} or C = floor {(1/SCS) / M}

  - ZTE

- Option 4: C = (1/SCS + CP) / M

  - OPPO

- Option 5: C = (1/SCS) / M + CP

  - OPPO, Xiaomi, NTT DOCOMO

- Option 6: Dedicated chip duration is introduced to ensure no additional transition edge introduced by CP, i.e. chip duration is equal to CP length.

  - ZTE

Based on above, FL suggests the following proposal to explain the issue to all companies. Since the TR indicates this definition to be a consequence of the CP handling, FL would intend to pursue the choice of definition after CP handling is clear enough.

#### [Open][M] Proposal 6.2.1a-v1:

At least from reader perspective, depending on CP handling and/or M values (if supported), the length of a R2D chip duration (noted as ‘C’) can be derived from the followings:

- Option 1: C = (1/SCS) / M

- Option 2: C = ceil {(1/SCS) / M} or C = floor {(1/SCS) / M}

- Option 3: C = (1/SCS + CP) / M

- Option 4: C = (1/SCS) / M + CP

- Option 5: Dedicated chip duration C is introduced to ensure no additional transition edge introduced by CP

- FFS: from device perspective, whether to define the following on top of above options

  - C - delta_C<= Cd <= C + delta_C

  - Where, delta_C is a floating chip duration and FFS: delta_C/C

| Company | Views |
| --- | --- |
| Spreadtrum | General fine with this proposal. We prefer Option 1. However, the definition of chip is highly related to the method for CP handling. So, we suggest discussing this issue after determining a CP handling method. |
| CMCC | We are generally fine with the proposal and prefer Option 1 to define R2D chip duration. Furthermore, the SFO and CP handling will affect the judgement of a valid chip duration from device perspective. Therefore, a "floating chip duration" can be defined and used for the device to determine a valid chip. |
| Nokia | Option 1 |
| QC | Defer discussion until CP handling scheme is decided. |
| vivo | Support option 1. For option 2, whether ceil and floor needed seems depending on whether the IFFT size can be divided by M value, while typically FFT size is not specified in RAN1, hence we do not see this option needed. Besides, even if there would be such cases, we can also define chip length with equal length in RAN1, and allow certain chip length variance in RAN4 requirements. For Option 3/4/5, it seems to be tied with certain CP handling method, which should be determined first. |
| Huawei, HiSilicon | The TR indicates R2D chip duration is a consequence of CP handling. We suggest that further work on this can hence follow later. This may well result in a smaller list of Options to spend time considering. |
| Xiaomi | General fine with the proposal. |
| Ericsson | We support Option 1. M chips correspond to M ON/OFF segments within one A-IoT OFDM symbol before CP insertion. The definition of chip duration is related to CP handling and cannot be accurately defined before the down-selection of a CP handling method. |
| ZTE, Sanechips | In our views, it is unfriendly to define an absolute chip duration calculated by (1/SCS)/M because the resultant chip duration will be a floating value. Reader/device with different clock rates or calculation accuracies may result into different understanding of chip durations. Therefore, we propose that the number of sampling points based on the smallest time duration Tc in TS 38.211 can be reused to define the chip duration. Moreover, the option 2 proposed by us is also in terms of sampling number , not an absolute value. The following revisions for the proposal are suggested. #### [Open][M] Proposal 6.2.1a-v1: At least from reader perspective, depending on CP handling and/or M values (if supported), the length of a R2D chip duration (noted as ‘C’) can be derived from the followings: - Option 1: C = (1/SCS) / M - Option 2: C = ceil {(1/SCS) / M} or C = floor {(1/SCS) / M}If the number of IFFT points of an OFDM symbol can not be evenly divided by M, C=Nf,refM∙Ts or C=Nf,refM∙Ts, where Ts/Tc=64; Otherwise, C=Nf,refM . - dedicated chip duration C is introduced to ensure no additional transition edge introduced by CP, e.g. the last chip duration is equal to CP length. - Tc=1480∙103∙4096 specified in TS 38.211. - Option 3: C = (1/SCS + CP) / M - Option 4: C = (1/SCS) / M + CP - Option 5: Dedicated chip duration C is introduced to ensure no additional transition edge introduced by CP - FFS: from device perspective, whether to define the following on top of above options - C - delta_C<= Cd <= C + delta_C - Where, delta_C is a floating chip duration and FFS: delta_C/C |
| LGE | Support Option 1 for simplicity at reader side. |
| OPPO2 | Considering the SFO of UE, it may cause different samplings numbers for different chips. It is better to transmit the R2D signal with the same R2D chip duration after the CP insertion at reader side. We think option 3 is better for device1. |
| Samsung | We generally support option 1. |
| CATT | We are fine with option 1. |

D2R modulation

Related proposals

| Source | Proposal |
| --- | --- |
| ZTE, Sanechips | Observation 12: OOK modulation scheme is beneficial for device energy harvesting because device 1 does not need to reflect and can conserve energy for the low power levels of D2R transmission. For BPSK modulation scheme, it can be used to achieve better coverage performance due to less modulation loss. - Proposal 8: Considering the tradeoff between energy harvesting efficiency and D2R coverage performance, it is proposed that both OOK and BPSK are mandatory modulation schemes for device 1. - Proposal 9: When the device supports both OOK and BPSK, the reader can configure the device to perform OOK or BPSK modulation for D2R transmission. - Proposal 10: It is proposed that same modulation scheme can be used for D2R RS and data transmission by considering the same backscattering transmission mode and the lower overhead of modulation scheme configuration at reader side. |
| Huawei, HiSilicon | Proposal 17: For D2R OOK modulation, after line coding, bit “1” of the line codeword is mapped to the real-valued modulation symbol {1} and bit “0” of the codeword is mapped to the real-valued modulation symbol {0} in baseband. Proposal 18: For D2R BPSK modulation, after line coding, the “1” of the line codeword is mapped to the real-valued modulation symbol {1} and bit “0” of the codeword is mapped to the real-valued modulation symbol {-1} in the baseband. Proposal 19: D2R scheduling to indicate for OOK modulation and BPSK modulation is not required since the reader can implement blind detection on both modulations. |
| Spreadtrum, UNISOC | Proposal 8: The modulation OOK or BPSK used by a device is up to device implementation. Proposal 9: Only 2SB modulation is supported in Rel-19. |
| CATT | Proposal 10：For D2R backscattering from Device 1, support only the BPSK modulation due to its superior BLER performance gain compared to that of OOK modulation. |
| CMCC | Proposal 9: Either OOK modulation or BPSK modulation is implemented by a device, which is not known by the reader. Proposal 10: The followings are considered in the further discussion, - Two impedance states are enough for either OOK or BPSK modulation. - Phase continuity of two ON chips are assumed to be kept for OOK modulation. |
| vivo | Observation 10: It is not necessary and not feasible for reader to distinguish between OOK and BPSK modulation. The D2R receiver can detect OOK and BPSK using the same receiver implementation. |
| Nokia | Proposal 7: RAN1 should support BPSK and OOK with 2SB modulation for D2R backscattering. |
| TCL | Observation 18: When device send D2R signal with BPSK modulation, reader needs to detect D2R signal using coherent detection. Proposal 21: To keep the detection accuracy at reader side, phase difference between +1 and -1 should be set one reasonable range, e.g, ¦phi (+1) -phi (-1)¦>N (e.g., N=90 degree). |
| OPPO | Proposal 8: A device should use BPSK if it supports such modulation scheme, or OOK otherwise. |
| Sony | Proposal 3: For the D2R link, RAN1 clarifies which of the following options applies: - Option 1: RAN1 will support either only OOK or only BPSK modulations. - Option 2: Device is required to support both OOK and BPSK modulations. FFS mechanism for UE to determine the appropriate modulation. - Option 3: Choice of modulation is up to device implementation. Reader is required to demodulate both OOK and BPSK modulations. |
| Xiaomi | Observation 12: The reader cannot determine whether the backscattered signal is generated by OOK modulation or BPSK modulation because of envelope detection. Observation 13: OOK and BPSK modulation schemes each possess distinct advantages and disadvantages. From a device implementation perspective, the choice of modulation mode depends on specific implementation details and requirements. Proposal 7: Only 2SB modulation should be supported for A-IoT in RAN1 because of the limitations of device1 implementation. |
| Apple | Proposal 3: Whether to support OOK and/or BPSK is up to AIoT device implementation, and which modulation to use for D2R transmission is up to the AIoT device. - The reader should blindly detect whether OOK or BPSK is used. |
| Samsung | Proposal 10: For D2R backscattering, both OOK and BPSK modulations are supported per WID. |
| IIT Kharagpur | Observation 1: OOK consumes the least power and is ideal for ultra-low-power applications, while BPSK offers better noise immunity with moderate power consumption. Observation 2: Device 2b, which generates its own carrier wave, can manage phase continuity better, making MSK feasible but at the cost of higher power consumption. Observation 3: MSK requires continuous impedance switching, leading to higher circuit activity and making it unsuitable for energy-harvesting and backscatter-based devices. Proposal 1: Prioritizing the OOK and BPSK modulation for device-1 and device-2a D2R transmission. Proposal 2: MSK may be considered only for Device 2b, while spectral efficiency is prioritized over power efficiency. Proposal 3: Deprioritize MSK for low-power A-IoT devices, particularly Devices 1 and 2a, due to power and complexity limitations. |
| LGE | Proposal 2: The 2SB modulation should be assumed for D2R transmission of Ambient IoT. |
| Lenovo | Proposal 3: For D2R transmission support only double side band transmission Proposal 4: For D2R transmission specify how the modulation scheme is selected at the device. |
| Sharp | Proposal 6: RAN1 to discussion the following two options for D2R modualtion: - Option 1: the device can choose which of OOK modulation and BPSK modulation to use for D2R backscattering. - Option 2: the Reader has full control of which of OOK modulation and BPSK modulation to use for D2R backscattering. Proposal 7: Double-side-band modulation is used for D2R backscattering. |
| Qualcomm | Observation 26: Supporting 1SB requires additional HW and increases device complexity. Proposal 8: For D2R modulation, support both OOK and BPSK. Device to support at least one of them. Proposal 9: Support 2SB for device 1. |
| NTT DOCOMO | Proposal 11: For PDRCH modulation scheme, down-select from the following alternatives; - Alt.1: A-IoT device supports both OOK and BPSK by default. - Alt.2: A-IoT device supports BPSK by default. Proposal 12: For PDRCH transmissions, discuss how to determine the modulation scheme for D2R if an A-IoT device supports both OOK and BPSK. - E.g., Modulation scheme for PDRCH is indicated via R2D. |
| IIT Kanpur | Proposal 5: Further down selection of D2R waveform should be considered. |

Discussion

OOK and BPSK

In the WID scope it is stated as: D2R backscattering supports only OOK and BPSK modulations. FL observes that inputs to this meeting are mainly discuss how the two modulations to be used.

Based on above, FL suggests the following proposal:

[Closed][H] Proposal 7.2.1a-v1:

For a D2R transmission

- The modulation OOK or BPSK is chosen by the device implementation

| Company | Views |
| --- | --- |
| Spreadtrum | Yes |
| CMCC | Support |
| Panasonic | Prefer to have the default of either OOK or BPSK. |
| Nokia | We are not sure why a reader should perform blind detection. In case there are many devices connected to a single reader, from the reader perspective, it probably prefer to indicate them to use an aligned modulation scheme. |
| QC | fine |
| vivo | Support. We do not think it is feasible for reader to distinguish OOK and BPSK based on backscattering. And clarify that, with this proposal, it imply reader does not need to indicate the modulation. |
| Huawei, HiSilicon | OK. The reader is able to detect both the modulations in a common way, or by blind algorithm, etc. No signaling is needed for this. |
| Xiaomi | Support. |
| FUTUREWEI | Ok to the proposal. |
| Ericsson | For D2R modulation, we need to either down-select between OOK and BPSK or study a method for identifying the modulation scheme. - Option 1: Blind decoding - Option 2: Signaling |
| Tejas Networks | Yes. However, the information should be conveyed to the reader through L1 control information. |
| Lenovo | 1 bit Signaling is enough and useful to avoid blind search at reader. |
| ZTE, Sanechips | Okay. |
| LGE | When multiple devices are connected to the reader, the selection of modulation schemes (OOK or BPSK) by the devices can introduce additional complexity for the reader in its efforts to blindly detect the modulation formats employed by the supporting devices. It may be beneficial to indicate modulation scheme to device by reader. |
| OPPO2 | Support |
| FL | It was agreed on Monday online session |

2 companies [Huawei, CMCC] discuss some further details of OOK or BPSK modulation in terms of constellation mapping from [Huawei] or phase continuity from [CMCC].

Based on above, FL suggests the following proposals:

[Closed][M] Proposal 7.2.1b-v1:

For D2R OOK modulation

- After line coding, bit “1” of the line codeword is mapped to the real-valued modulation symbol {1} and bit “0” of the codeword is mapped to the real-valued modulation symbol {0} in baseband.

| Company | Views |
| --- | --- |
| Spreadtrum | Fine with this proposal. |
| CMCC | We are fine with the proposal. |
| Nokia | OK |
| QC | Suggest following modified version For D2R OOK modulation - After line coding, data bit “1” of the line codeword is mapped to the real-valued modulation symbol {1} and data bit “0” of the codeword is mapped to the real-valued modulation symbol {0} in baseband. |
| Huawei, HiSilicon | OK |
| FUTUREWEI | Support |
| Lenovo | Ok |
| ZTE, Sanechips | Okay. |
| OPPO2 | Support |
| Samsung | OK |
| SONY1 | I understand that the line code produces chips, not bits. In this case, we suggest the following modification: For D2R OOK modulation - After line coding, chip bit “1” of the line codeword is mapped to the real-valued modulation symbol {1} and chip bit “0” of the codeword is mapped to the real-valued modulation symbol {0} in baseband. |
| FL | Update accordingly to Proposal 7.2.1b-v2 |

#### [Open][M] Proposal 7.2.1b-v2:

For D2R OOK modulation

- After line coding or square wave, bit chip “1” of the line codeword is mapped to the real-valued modulation symbol {1} and bit chip “0” of the codeword is mapped to the real-valued modulation symbol {0} in baseband.

| Company | Views |
| --- | --- |

#### [Open][M] Proposal 7.2.1c-v1:

For D2R OOK modulation

- Phase continuity of two ON chips are assumed to be kept for OOK modulation.

| Company | Views |
| --- | --- |
| Spreadtrum | Support this proposal with conclusion. |
| CMCC | Support |
| Panasonic | We are not sure why phase continuity of two ON chips are required to be assumed for OOK modulation. |
| Nokia | OK |
| QC | Fine with following modification. For D2R OOK modulation - Phase continuity of CW during two consecutive ON chips are assumed to be kept for OOK modulation. |
| Huawei, HiSilicon | Would like to understand precisely what this proposal means from the proponent (CMCC). Is that the phase of any two ON chips is the same? In that case, is the effect achieved already if Proposal 7.2.1b-v1 is agreed, since all ON will be ‘1’? |
| FUTUREWEI | Ok with the proposal |
| Ericsson | The benefits and necessity of phase continuity between two ON chips need further study. We suggest updating the proposal to: [Open][M] Proposal 7.2.1c-v1: For D2R OOK modulation - FSS: Phase continuity of two ON chips are assumed to be kept for OOK modulation. |
| ZTE, Sanechips | Okay. |
| OPPO2 | What is the impact on device 1? |
| Samsung | We would like to ask the technical motivation of this proposal. For OOK modulation only, why phase continuity has to be strictly required? In addition, the ON chips may be consecutive or non-consecutive, and we also have concern on the feasibility of non-consecutive case. |
| SONY1 | Agree with Huawei that this agreement is not required if Proposal 7.2.1b-v1 is agreed. This agreement would have been needed if the “ON” of ON-OFF keying could take either value +1 or -1 (i.e. reflect or reflect-and-invert). |

[Closed][M] Proposal 7.2.1d-v1:

For D2R BPSK modulation

- After line coding, the “1” of the line codeword is mapped to the real-valued modulation symbol {1} and bit “0” of the codeword is mapped to the real-valued modulation symbol {-1} in the baseband.

| Company | Views |
| --- | --- |
| Spreadtrum | Fine with this proposal. |
| CMCC | We are fine with the proposal. |
| QC | Suggest following modification. For D2R BPSK modulation - After line coding, The data “1” of the line codeword is mapped to the real-valued modulation symbol {1} and data bit “0” of the codeword is mapped to the real-valued modulation symbol {-1} in the baseband. |
| Huawei, HiSilicon | OK |
| FUTUREWEI | Ok with the proposal |
| ZTE, Sanechips | Okay. |
| OPPO2 | Support. |
| Samsung | OK |
| SONY1 | I find this text problematic in the context of this agreement: Agreement For D2R transmission with small frequency shift +/- R/Tb Hz, where time duration Tb corresponding to a bit (that is after FEC if applied) and R = Tb/(2 × D2R chip length): - the transmitted 2R chips for bit 1 and bit 0 are [0 1 0 1 …] and [1 0 1 0 …], respectively, for OOK modulation - the transmitted 2R chips for bit 1 and bit 0 are [-1 +1 -1 +1 …] and [+1 -1 +1 -1 …], respectively, for BPSK modulation. - Note: The case of R=1 is equivalent to using a Manchester line code without repeating each Manchester codeword. The chip polarities for BPSK are +1/-1 according to that agreement. There is no “0” chip for BPSK, but this proposal says how we modulate a “0” chip. Why doesn’t it talk about how to modulate a “-1” chip, which would be consistent with the agreement quoted above? I also don’t understand why we are talking about “bits” being produced by the line code. Doesn’t the line code produce chips? Our suggestion is to update the proposal as: For D2R BPSK modulation - After line coding, the “1” of the line codeword is mapped to the real-valued modulation symbol {1} and the bit “0” of the codeword is mapped to the real-valued modulation symbol {-1} in the baseband. And to correct the above-quoted agreement to: Agreement For D2R transmission with small frequency shift +/- R/Tb Hz, where time duration Tb corresponding to a bit (that is after FEC if applied) and R = Tb/(2 × D2R chip length): - the transmitted 2R chips for bit 1 and bit 0 are [0 1 0 1 …] and [1 0 1 0 …], respectively, for OOK modulation - the transmitted 2R chips for bit 1 and bit 0 are [0 1 0 1 …] [-1 +1 -1 +1 …] and [+1 -1 +1 -1 …] [1 0 1 0 …], respectively, for BPSK modulation. - Note: The case of R=1 is equivalent to using a Manchester line code without repeating each Manchester codeword. |
| FL | Update to Proposal 7.2.1d-v2 |

#### [Open][M] Proposal 7.2.1d-v2:

For D2R BPSK modulation

- After line coding or square wave, chip “1” of the line codeword is mapped to the real-valued modulation symbol {1} and bit chip “0” of the codeword is mapped to the real-valued modulation symbol {-1} in the baseband.

| Company | Views |
| --- | --- |

1SB/2SB

7 companies [Lenovo, LGE, Nokia, Qualcomm, Sharp, Spreadtrum, Xiaomi] propose that only 2SB is supported while no company propose 1SB.

Based on above, FL suggests the following proposal:

[Closed][H] Proposal 7.2.2a-v1:

For D2R, only 2SB (Double sideband) modulation is supported.

| Company | Views |
| --- | --- |
| Spreadtrum | Support. |
| CMCC | Support. |
| Panasonic | Support. |
| Nokia | Support |
| QC | fine |
| vivo | Support. |
| Huawei, HiSilicon | OK. Device 1 hardware does not prefer 1SB generation complexity. |
| Xiaomi | Support. |
| FUTUREWEI | Ok with the proposal |
| Ericsson | Fine. |
| Lenovo | Support |
| ZTE, Sanechips | Okay. |
| LGE | Support the proposal. |
| OPPO2 | Support. |
| CATT | Support. |
| FL | It was agreed on Monday online session |

Others

Related proposals

| Source | Proposal |
| --- | --- |
| Spreadtrum, UNISOC | Proposal 2: The carrier wave (with waveform) should be captured in the specification; otherwise, it is difficult for the implementation people to determine how the D2R waveform is generated. |
| ZTE, Sanechips | Observation 13: As captured in TR 38.796, the bandwidth of D2R transmission with small frequency shift is not changed for different R values, which always equals to 2/Tb for double sideband modulation. Proposal 11: D2R chip duration can be defined as the time duration of a modulated symbol. Proposal 12: For D2R transmission without double sideband modulation, the bit duration (Tb) is associated with D2R transmission bandwidth (Btx,D2R), i.e. Tb=2/Btx,D2R . Proposal 13: Define D2R bandwidth/bit duration to facilitate at least the indication of D2R scheduling information. Proposal 14: Potential impacts of FDM, data rate and clock acquisition on the definition of D2R bit duration Tb, chip duration Tchip and transmission bandwidth Btx,D2R in Table 1 should be considered as the starting point. Proposal 15: It is proposed that the bit duration Tb and transmission bandwidth Btx,D2R defined by option 2 in Table 3 can be considered as starting point for D2R. |
| CATT | Observation 4: If the maximum occupied bandwidth of PDRCH is consistent with that of PRDCH, the FDMA would not be supported or would only support a limited number of SFS factors, when the chip rate is higher than 120kbps. Proposal 7: The D2R chip duration should be defined as the duration of each channel coding bit, which is related to the chip rate from the physical layer transmission perspective. Proposal 8: The square-wave chip length should be further defined with the following relationship, when the SFS is considered. D2R chip length = 2*R* square-wave chip length, where R is the number of square-wave periods corresponding a D2R chip. Proposal 9: For Device 1, the chip rate of PDRCH should not be higher than 240kbps with M=16. Proposal 11: The TBS tables should be introduced for simple indication of TBS and the corresponding TTI, which are aligned with the NR time domain resource assignment. Proposal 12: Adopt the following TBS tables for PRDCH and PDRCH: The TBS table for PRDCH (3 bits TBS indication) TBS index; TBS (bits); Coded bits size; TTI length (slots). M=1; M=2; M=4; M=8; M=16; M=32. 1; 22; 56; 4; 2; 1; 1; 1; 1. 2; 40; 112; 8; 4; 2; 1; 1; 1. 3; 96; 224; 16; 8; 4; 2; 1; 1. 4; 208; 448; 32; 16; 8; 4; 2; 1. 5; 432; 896; 64; 32; 16; 8; 4; 2. 6; 656; 1344; 96; 48; 24; 12; 6; 3. 7; 880; 1792; 128; 64; 32; 16; 8; 4. 8; 1104; 2240; 160; 80; 40; 20; 10; 5. The TBS table for PDRCH (3 bits TBS indication) TBS index; TBS (bits); Coded bits size; TTI length (slots). M=1; M=2; M=4; M=8; M=16; M=32. 1; 21; 112; 8; 4; 2; 1; 1; 1. 2; 40; 168; 12; 6; 3; 2; 1; 1. 3; 96; 336; 24; 12; 6; 3; 2; 1. 4; 208; 672; 48; 24; 12; 6; 3; 2. 5; 432; 1344; 96; 48; 24; 12; 6; 3. 6; 656; 2016; 144; 72; 36; 18; 9; 5. 7; 880; 2688; 192; 96; 48; 24; 12; 6. 8; 1104; 3360; 240; 120; 60; 30; 15; 8. |
| China Telecom | Proposal 9: The NR symbol boundary alignment is not necessary for D2R transmission. Proposal 10: Both the first and last symbol occupied by D2R transmission can be regarded as the allocated reception resource at reader side. |
| CMCC | Observation 6: For D2R transmission within [TR2D_min, TR2D_max] from a R2D transmission, it is impossible to align the D2R transmission within the NR symbols and/or slots. Observation 7: For D2R transmission timing TR2D following R2D control indication, it is up to gNB or intermediate UE whether to align the D2R transmission within the NR symbols and/or slots. Furthermore, the impact of large SFO should be considered to verify the feasibility of such alignment among different devices. Proposal 14: Defining a unified time unit to represent the granularity of time domain resources can be considered in A-IoT. Proposal 16: D2R transmission is not required to be aligned with NR symbols and/or slots. |
| TCL | Observation 15: OOK only modulates “1” and “0” of D2R signal without line code, which is in-consistence D2R time domain signal. Observation 16: If no D2R line code is used, by using a square-wave corresponding to the small frequency-shift, the time duration Tb corresponding to each information bit includes R number of square wave periods generated by 2R OOK chips [0, 1, 0, 1 …]/[1, 0, 1, 0 ..]. Observation 17: If no D2R line code is used, by using a square-wave corresponding to the small frequency-shift, the time duration Tb corresponding to each information bit includes R number of square wave periods generated by 2R BPSK chips [-1, +1, -1, +1, …]/[+1, -1, +1, -1, …]. Observation 19: For D2R transmission of AIoT, MCS-like is associated with D2R modulation (e.g., OOK and BPSK), channel coding (e.g., convolutional code), line code (e.g., with or w/o line code, repetition factor) and block/bit/chip-level repetition (e.g., repetition number). Proposal 17: For CAP transmission, - Support CAP length is longer than 1 OFDM symbol - Support first OFDM symbol of CAP insert CP - If CAP occupy multiple OFDM symbol, consider how/whether to design CP insertion for subsequent OFDM symbol. Proposal 18: Support OOK modulation without line code to be as binary sequence for channel/interference/SFO estimation at reader side. Proposal 19: Consider other OOK sequences without line code to meet the requirement of channel/interference/SFO estimation at reader side, e.g., [111..11] or [000111...] or other types. Proposal 20: Consider other BPSK sequences without line code to meet the requirement of channel/interference/SFO estimation at reader side, e.g., [+1,+1,+1..+1,+1] or [+1,+1,+1,-1,-1,-1...] or other types. Proposal 22: Discuss the definition of MCS-like for D2R transmission based on OOK/BPSK modulation, channel coding, line code and block/bit/chip-level repetition. |
| OPPO | Proposal 9: D2R transmission at least supports the same candidate chip rate/chip length as the R2D transmission, e.g. 14kcps, 28kcps, 56kcps, 112kcps. |
| Tejas | Proposal 1: The control information and system information can be generated in two ways: 1) concatenating the control information frame and data frame together before sending to the physical layer processing and 2) passing the information bits separately through differential encoder and concatenating the differential encoded information to the CRC attached and line coded data before passing through the modulation block. Proposal 2: Depending on the feasibility of the system and targeted SNR or other system performance matrices, a method of transmitting control information and data can be down selected from the two methods mentioned above. |
| Panasonic | Proposal 2: No SFO estimation / correction can be sufficient for device 1. Proposal 3: For device 1, counter and digital logic can be used to calibrate transmission waveform. |
| NEC | Proposal 8: Reader supports full-duplexed FDD, i.e., from reader’s perspective, it can transmit PRDCH on DL spectrum and receive PDRCH on UL spectrum (for different ambient IoT devices) at the same time. |
| Xiaomi | Observation 10: Regardless of which option for design of the clock-acquisition part is used, inserting CP in the clock acquisition process can adversely affect the detection performance of the clock-acquisition part. Since devices may acquire clocks across multiple time-domain symbols, the sampling counts are affected by the irregular ON/OFF durations introduced by CP insertion. Observation 11: When the M value is relatively small, CP insertion exerts a greater impact on the performance of clock acquisition because there are not enough correct ON/OFF durations. Proposal 6: RAN1 needs to mitigate the impact of CP insertion on clock acquisition performance. For instance, the entire CP could be reserved for a placeholder level to prevent incorrect ON/OFF durations. |
| Samsung | Proposal 11: To determine PDRCH OOK chip rate or BPSK symbol rate, a device first obtains a reference chip rate from the R2D preamble and multiplies rate factor to the reference chip rate, wherein the rate factor is indicated in the R2D control information in the corresponding PRDCH. Proposal 12: Midamble is supported for D2R, at least, when BPSK modulation is used. |
| MediaTek | Proposal 3: For D2R transmission, transmission bandwidth of RE level and RB level are supported. |
| Sharp | Proposal 8: A set of D2R bandwidths (including a number of values as integer multiple of SCS and other values as integer multiple of PRBs) is pre-defined for backscattering transmissions in Rel-19. |
| Qualcomm | - Observation 23: Minimum and Maximum RFID BL data rates are 5 kbps and 640kbps respectively. - Observation 24: In terms of RFID FL data rate, RFID BL data rate range is given as 16FL data rate≤BL data rate≤9.69FL data rate. - Observation 25: The wide range of BL data rate implies that FL and BL data rate can be almost independently chosen by reader. |
| NTT DOCOMO | Proposal 13: For D2R, chip duration definition should be one of a pre-defined set of pulse time durations. - FFS: candidate values - FFS: how to determine the chip length for each D2R transmission - E.g., D2R chip length is implicitly indicated based on the transmission bandwidth, small frequency shift factor, repetition number of line coding codeword/square wave period. - E.g., set of D2R chip lengths are pre-defined and one of them is explicitly indicated. Proposal 14: For D2R, discuss the candidate chip length considering at least small frequency shift factor, the target peak rate for D2R and decoding performance. |
| IITM | Proposal 3: For R2D and D2R signals, the TB size should be chosen based on the channel conditions. |

Discussion

CW waveform

1 company [Spreadtrum] propose that the carrier wave (with waveform) should be captured in the specification; otherwise, it is difficult for the implementation people to determine how the D2R waveform is generated. FL understand this is already included in the WID statement.

Based on above, FL suggest the following proposal:

#### [Open][M] Proposal 8.2.1a-v1:

FL conclusion: For CW waveform, A-IoT WID RP-243326 and TR 38.769 already give us:

It is left up to specification drafting phase to capture the following.

- Carrier wave transmission for waveform 1 only, without hopping, per the following cases in TR 38.769: - Case 1-4 for D1T1-B

- 6.8.2 CW characteristics Candidates for the CW for D2R backscattering are waveforms consisting of: Waveform 1: A single-tone unmodulated sinusoid, also referred to as 'a single tone'. Waveform 2: Two single tones.

| Company | Is anything needed further to the WID and TR? |
| --- | --- |
| Spreadtrum | We are fine specification editor drafting to handle this issue. Our intention is to emphasize the need to present how to generate the D2R waveform in the TS. |
| Panasonic | Which frequency is used as the carrier wave is RAN4 specification? We would like to know whether CW is available at the time of the reception of PRDCH. We assume CW should be available at the time of PDRCH transmission. |
| vivo | It seems this issue can be addressed if there is some RF requirements for CW signal defined in RAN4. |

Others

Other proposals from companies discuss things e.g., D2R bandwidth/chip rate/data rate, D2R alignment, TBS, MCS-like etc. which seems either better to be discussed in other agenda e.g. 9.4.2 or 9.4.4 or else. FL will not pursue to discuss those in this meeting.

Offline/Online proposals

Monday afternoon online

[Open][H] Proposal 7.2.2a-v1:

For D2R, only 2SB (Double sideband) modulation is supported.

[Open][H] Proposal 2.2.1a-v1:

For R2D transmission, only subcarrier spacing of 15 kHz is supported (as per TR 38.769), and no other SCS are further considered.

[Open][H] Proposal 7.2.1a-v1:

For a D2R transmission

- The modulation OOK or BPSK is chosen by the device implementation

[Open][H] Proposal 5.2.1a-v1:

For R2D CP handling, Alt M2-2 (as per TR 38.769) is deprioritized in Rel-19.

[Open][H] Proposal 5.2.1b-v1:

For R2D CP handling,

- At least Method Type 1 (as per TR 38.769) is supported for M value larger than 12

  - Alt M1-1 and/or Alt M1-2 is up to device implementation

  - The device is aware of or determines the boundary of an OFDM symbol using the R2D timing acquisition signal (R-TAS), at least for Alt M1-1

    - Detail is up to R2D preamble design

- FFS: whether/how to additionally support Method Type 2 Alt M2-1-1 or Alt M2-1-2 (as per TR 38.769)

[Open][H] Proposal 4.2.1a-v1:

For the OOK-4 modulation for M-chip per OFDM symbol transmission, the maximum M value is down-selected from the followings:

- Alt 1: 32

  - Achieve much higher chip/data rate than RFID

- Alt 2: 24

  - Achieve comparable R2D chip rate and a bit higher data rate than RFID

- Alt 3: 16

  - Achieve a bit lower R2D chip rate and comparable R2D data rate than RFID

- Alt 4: 8

  - Achieve much lower R2D chip/data rate than RFID

[Open][M] Proposal 8.2.1a-v1:

FL conclusion: For CW waveform, A-IoT WID RP-243326 and TR 38.769 already give us:

It is left up to specification drafting phase to capture the following.

- Carrier wave transmission for waveform 1 only, without hopping, per the following cases in TR 38.769: - Case 1-4 for D1T1-B

- 6.8.2 CW characteristics Candidates for the CW for D2R backscattering are waveforms consisting of: Waveform 1: A single-tone unmodulated sinusoid, also referred to as 'a single tone'. Waveform 2: Two single tones.

[Open][H] Proposal 3.2.1a-v1:

For R2D transmission, from reader perspective, DFT-s-OFDM waveform is supported for OOK-4 modulation. For the details of DFT-s-OFDM waveform, down-selection between the followings:

- Option 1: Details of DFT-s-OFDM waveform generation is up to reader implementation and is not specified in RAN1.

  - This does not preclude RAN4 to discuss R2D waveform generation themselves when defining requirement(s), if needed.

- Option 2: Certain specification of DFT-s-OFDM waveform generation is needed in RAN1.

  - FFS: identify what potential details of 5 steps in section 4.4 of TR 38.769 needs to be specified.

[Open][H] Proposal 4.2.2a-v1:

Proposal: RAN1 does not specify the exact number of PRBs a reader shall use for Btx,R2D.

- For R2D, specify the minimum Btx,R2D # of PRBs associated to each M value.

- For R2D, for the maximum Btx,R2D # of PRBs down-select from the followings:

  - Option 1: No need to specify the maximum Btx,R2D # of PRBs in RAN1

    - This does not preclude RAN4 to discuss the maximum Btx,R2D # of PRBs themselves when defining requirement(s), if needed.

  - Option 2: Specify the maximum Btx,R2D # of PRBs and FFS from the followings

    - A single maximum Btx,R2D # of PRBs for all M values

    - A single maximum Btx,R2D # of PRBs for each M value

[Open][H] Proposal 2.2.2a-v1:

Btx,R2D is defined as integer multiple(s) of 180 kHz.

[Open][M] Proposal 4.2.2b-v1:

For R2D, Bocc,R2D is not specified in RAN1.

- This does not preclude RAN4 to discuss Bocc,R2D themselves when defining requirement(s), if needed.

Tuesday morning offline

[Open][H] Proposal 5.2.1b-v1:

For R2D CP handling,

- At least Method Type 1 (as per TR 38.769) is supported for M value (if supported) larger than 12

  - Alt M1-1 and/or Alt M1-2 is up to device implementation

  - The device is aware of or determines the boundary of an OFDM symbol using the R2D timing acquisition signal (R-TAS), at least for Alt M1-1

    - Detail is up to R2D preamble design

- FFS: whether/how to additionally support Method Type 2 Alt M2-1-1 or Alt M2-1-2 (as per TR 38.769)

[Open][H] Proposal 4.2.1a-v1:

For the OOK-4 modulation for M-chip per OFDM symbol transmission, the maximum M value is down-selected from the followings:

- Alt 1: 32

  - Achieve much higher chip/data rate than RFID

- Alt 2: 24

  - Achieve comparable R2D chip rate and a bit higher data rate than RFID

- Alt 3: 16

  - Achieve a bit lower R2D chip rate and comparable R2D data rate than RFID

- Alt 4: 8

  - Achieve much lower R2D chip/data rate than RFID

[Open][H] Proposal 4.2.1b-v2:

For R2D, up to the maximum M value, RAN1 select M values from the following two options:

- Option 1: all M values (as per TR 38.769) up to the maximum M value

- Option 2: at least 3 of the M values for [low, medium, high] R2D chip/data rate subset of M values (as per TR 38.769) up to the maximum M value

  - FFS: the selection of detail M values is to be done can considering at least the following aspects

    - Performance gap of and SNR distribution cross under different M values

    - Transmission efficiency together with minimum Btx,R2D

    - Ambiguity of device detecting different M values

    - Integer or non-integer samples per R2D OOK-4 chip

    - [Flexibility in time domain resource for R2D transmission]

    - CP handling

  - FFS: whether need more M values

[Open][H] Proposal 4.2.2a-v1:

Proposal: RAN1 does not specify the exact number of PRBs a reader shall use for Btx,R2D.

- For R2D, specify the minimum Btx,R2D # of PRBs associated to each M value.

- For R2D, for the maximum Btx,R2D # of PRBs down-select from the followings:

  - Option 1: No need to specify the maximum Btx,R2D # of PRBs in RAN1

    - This does not preclude RAN4 to discuss the maximum Btx,R2D # of PRBs themselves when defining requirement(s), if needed.

  - Option 2: Specify the maximum Btx,R2D # of PRBs and FFS from the followings

    - A single maximum Btx,R2D # of PRBs for all M values

    - A single maximum Btx,R2D # of PRBs for each M value

[Open][H] Proposal 2.2.2a-v1:

Btx,R2D is defined as integer multiple(s) of 180 kHz.

[Open][M] Proposal 4.2.2b-v1:

For R2D, Bocc,R2D is not specified in RAN1.

- This does not preclude RAN4 to discuss Bocc,R2D themselves when defining requirement(s), if needed.

[Open][H] Proposal 3.2.1a-v1:

For R2D transmission, from reader perspective, DFT-s-OFDM waveform is supported for OOK-4 modulation. For the details of DFT-s-OFDM waveform, down-selection between the followings:

- Option 1: Details of DFT-s-OFDM waveform generation is up to reader implementation and is not specified in RAN1.

  - This does not preclude RAN4 to discuss R2D waveform generation themselves when defining requirement(s), if needed.

- Option 2: Certain specification of DFT-s-OFDM waveform generation is needed in RAN1.

  - FFS: identify what potential details of 5 steps in section 4.4 of TR 38.769 needs to be specified.

[Open][M] Proposal 2.2.3b-v1:

As per TR 38.769, for the OFDM-based OOK-4 waveform, from the reader perspective, when the generated number of chips for the R2D transmission does not fully occupy the last OFDM symbol, padding is used to align with the boundary of the last NR OFDM symbol for in-band operation. Further to the TR,

- Padding is to be down-selected between the following alternatives:

  - Alt 1: Padding is up to reader implementation and transparent to A-IoT device.

  - Alt 2: Specify the details of padding

[Open][M] Proposal 7.2.1b-v1:

For D2R OOK modulation

- After line coding, bit “1” of the line codeword is mapped to the real-valued modulation symbol {1} and bit “0” of the codeword is mapped to the real-valued modulation symbol {0} in baseband.

[Open][M] Proposal 7.2.1b-v2:

For D2R OOK modulation

- After line coding or square wave, bit chip “1” of the line codeword is mapped to the real-valued modulation symbol {1} and bit chip “0” of the codeword is mapped to the real-valued modulation symbol {0} in baseband.

[Open][M] Proposal 7.2.1c-v1:

For D2R OOK modulation

- Phase continuity of two ON chips are assumed to be kept for OOK modulation.

[Open][M] Proposal 7.2.1d-v2:

For D2R BPSK modulation

- After line coding or square wave, chip “1” of the line codeword is mapped to the real-valued modulation symbol {1} and bit chip “0” of the codeword is mapped to the real-valued modulation symbol {-1} in the baseband.

[Open][M] Proposal 8.2.1a-v1:

FL conclusion: For CW waveform, A-IoT WID RP-243326 and TR 38.769 already give us:

It is left up to specification drafting phase to capture the following.

- Carrier wave transmission for waveform 1 only, without hopping, per the following cases in TR 38.769: - Case 1-4 for D1T1-B

- 6.8.2 CW characteristics Candidates for the CW for D2R backscattering are waveforms consisting of: Waveform 1: A single-tone unmodulated sinusoid, also referred to as 'a single tone'. Waveform 2: Two single tones.

[Open][H] Proposal 5.2.1a-v1:

For R2D CP handling, Alt M2-2 (as per TR 38.769) is deprioritized in Rel-19.

Wednesday morning online

[Open][M] Proposal 4.2.2b-v1:

For R2D, Bocc,R2D is not specified in RAN1.

- This does not preclude RAN4 to discuss Bocc,R2D themselves when defining requirement(s), if needed.

[Open][H] Proposal 4.2.2a-v2:

Proposal: RAN1 does not specify the exact number of PRBs a reader shall use for Btx,R2D.

- For R2D, specify the minimum Btx,R2D # of PRBs associated to each M value.

  - Note: Reader can transmit any Btx,R2D subject to this minimum Btx,R2D and the agreed 15 kHz SCS

- For R2D, no need to specify the maximum Btx,R2D # of PRBs in RAN1

  - This does not preclude RAN4 to discuss the maximum Btx,R2D # of PRBs themselves when defining requirement(s), if needed.

[Open][H] Proposal 4.2.1a-v2:

For the OOK-4 modulation for M-chip per OFDM symbol transmission

- The maximum M value is no less than 16, and to be down-selected from the followings:

  - Alt 1: 32

  - Alt 2: 24

  - Alt 3: 16

- Up to the maximum M value, at least [3] M values to be supported from Table 6.1.1.4-1 (as per TR 38.769)

  - FFS: the exact M values

[Open][H] Proposal 5.2.1b-1-v2:

If any part of R2D preamble needs CP handling, same CP handling solution applies for both R2D preamble and PRDCH.

[Open][H] Proposal 5.2.1b-2-v2:

For CP handling,

- For Method Type 1 (as per TR 38.769)

  - For purposes of evaluating Method Type 1, it is assumed the device is aware of or determines the boundary of an OFDM symbol using the R2D timing acquisition signal (R-TAS), at least for Alt M1-1

    - Detail is up to R2D preamble design

  - Using Alt M1-1 and/or Alt M1-2 is up to device implementation as per TR 38.769

  - FFS: applicable M values

- For Method Type 2 Alt M2-1 (as per TR 38.769)

  - Method Type 1 needs to be also used for CP handling at least when CP part of an OFDM symbol is longer than a R2D chip.

  - FFS: detail design of Alt M2-1-1 or Alt M2-1-2 [from the followings

    - Alt M2-1-1

      - Candidate 1:

        - For small M': Manchester line-code codewords where the first n1 chip(s) and the last n2 chip(s) of the codewords are the same.

        - For large M': circular shift of the M chips’ samples before CP insertion at the reader side in addition to chip skipping/dropping at the device side

      - Candidate 2:

        - Where a known signal shape in the tail-portion of the R2D signal is copied to the CP.

      - Candidate 3:

        - Insert padding chips at both start and end OOK chips or only at end OOK chips

    - Alt M2-1-1

      - Candidate 4:

        - Change a=b≠c case to a=b=c case by changing OOK-4 M value to adjacent M+1 for OFDM symbol n.

          - where a, b and c is the last chip of symbol n, first chip of symbol n+1 and last chip of symbol n+1.

      - Candidate 5:

        - Odd number of chips of PRDCH is transmitted in the first OFDM symbols, while M value is kept unchanged during the remaining OFDM symbols.]

  - FFS: applicable M values with or without combination of Method Type 1

- For further down-selection, companies are encouraged to provide evaluation among the followings to RAN1#120bis:

  - Method Type 1

  - Method Type 2 Alt M2-1-1 with or without combination of Method Type 1

  - Method Type 2 Alt M2-1-2 with or without combination of Method Type 1

[Open][M] Proposal 7.2.1b-v2:

For D2R OOK modulation

- After line coding or square wave, bit chip “1” of the line codeword is mapped to the real-valued modulation symbol {1} and bit chip “0” of the codeword is mapped to the real-valued modulation symbol {0} in baseband.

[Open][M] Proposal 7.2.1c-v1:

For D2R OOK modulation

- Phase continuity of two ON chips are assumed to be kept for OOK modulation.

[Open][M] Proposal 7.2.1d-v2:

For D2R BPSK modulation

- After line coding or square wave, chip “1” of the line codeword is mapped to the real-valued modulation symbol {1} and bit chip “0” of the codeword is mapped to the real-valued modulation symbol {-1} in the baseband.

[Open][M] Proposal 2.2.3b-v2:

As per TR 38.769, for the OFDM-based OOK-4 waveform, from the reader perspective, when the generated number of chips for the R2D transmission does not fully occupy the last OFDM symbol, padding is used to align with the boundary of the last NR OFDM symbol for in-band operation. Further to the TR,

- Padding is to be down-selected between the following alternatives:

  - Alt 1: The content of padding is up to reader implementation and transparent to A-IoT device.

  - Alt 2: Specify the detail content of padding

[Open][H] Proposal 3.2.1a-v1:

For R2D transmission, from reader perspective, DFT-s-OFDM waveform is supported for OOK-4 modulation. For the details of DFT-s-OFDM waveform, down-selection between the followings:

- Option 1: Details of DFT-s-OFDM waveform generation is up to reader implementation and is not specified in RAN1.

  - This does not preclude RAN4 to discuss R2D waveform generation themselves when defining requirement(s), if needed.

- Option 2: Certain specification of DFT-s-OFDM waveform generation is needed in RAN1.

  - FFS: identify what potential details of 5 steps in section 4.4 of TR 38.769 needs to be specified.

[Open][M] Proposal 8.2.1a-v1:

FL conclusion: For CW waveform, A-IoT WID RP-243326 and TR 38.769 already give us:

It is left up to specification drafting phase to capture the following.

- Carrier wave transmission for waveform 1 only, without hopping, per the following cases in TR 38.769: - Case 1-4 for D1T1-B

- 6.8.2 CW characteristics Candidates for the CW for D2R backscattering are waveforms consisting of: Waveform 1: A single-tone unmodulated sinusoid, also referred to as 'a single tone'. Waveform 2: Two single tones.

[Open][H] Proposal 5.2.1a-v2:

For R2D CP handling, Alt M2-2 (as per TR 38.769) is not considered in Rel-19.

Wednesday afternoon offline

Proposal

For R2D, for the OOK-4 modulation for M-chip per OFDM symbol transmission:

- The maximum M value is no less than 16, and to be down-selected from 32, 24 or 16 at RAN1#120bis.

- Up to the maximum M value, [3 or 4 or 5] M values to be supported from Table 6.1.1.4-1 (as per TR 38.769)

  - FFS: the exact M values

- Note: all the supported M values are supported for the PRDCH

Proposal

If any part of R2D preamble needs CP handling, same CP handling solution applies for both R2D preamble and PRDCH.

[Open][H] Proposal 5.2.1b-2-v2:

For CP handling,

- For Method Type 1 (as per TR 38.769)

  - For purposes of evaluating Method Type 1, it is assumed the device is aware of or determines the boundary of an OFDM symbol using the R2D timing acquisition signal (R-TAS), at least for Alt M1-1

    - Detail is up to R2D preamble design

  - Using Alt M1-1 and/or Alt M1-2 is up to device implementation as per TR 38.769

  - FFS: applicable M values

- For Method Type 2 Alt M2-1 (as per TR 38.769)

  - Method Type 1 needs to be also used for CP handling at least when CP part of an OFDM symbol is longer than a R2D chip where the CP or the whole bit/chip with CP removed.

  - FFS: detail design of Alt M2-1-1 or Alt M2-1-2 [from the followings

    - Alt M2-1-1

      - Candidate 1:

        - For M < 6: Use Alt M1-1

        - For M >=6: circular shift of the M chips’ samples before CP insertion at the reader side in addition to bit/chip skipping/dropping at the device side after bit decoding

      - Candidate 2:

        - Where a known signal shape in the tail-portion of the R2D signal is copied to the CP.

      - Candidate 3:

        - Insert padding chips at both start and end OOK chips or only at end OOK chips

    - Alt M2-1-2

      - Candidate 4:

        - Change a=b≠c case to a=b=c case by changing OOK-4 M value to adjacent M+1 for OFDM symbol n.

          - where a, b and c is the last chip of symbol n, first chip of symbol n+1 and last chip of symbol n+1.

      - Candidate 5:

        - Odd number of chips of PRDCH is transmitted in the first OFDM symbols, while M value is kept unchanged during the remaining OFDM symbols.]

  - FFS: applicable M values with or without combination of Method Type 1

- For further down-selection, companies are encouraged to provide evaluation (including overhead) among the followings to RAN1#120bis:

  - Method Type 1

  - Method Type 2 Alt M2-1-1 with or without combination of Method Type 1

  - Method Type 2 Alt M2-1-2 with or without combination of Method Type 1

[Open][M] Proposal 7.2.1b-v2:

For D2R OOK modulation

- After line coding or square wave, bit chip “1” of the line codeword is mapped to the real-valued modulation symbol {1} and bit chip “0” of the codeword is mapped to the real-valued modulation symbol {0} in baseband.

[Open][M] Proposal 7.2.1c-v1:

For D2R OOK modulation

- Phase continuity of two ON chips are assumed to be kept for OOK modulation.

[Open][M] Proposal 7.2.1d-v2:

For D2R BPSK modulation

- After line coding or square wave, chip “1” of the line codeword is mapped to the real-valued modulation symbol {1} and bit chip “0” of the codeword is mapped to the real-valued modulation symbol {-1} in the baseband.

[Open][M] Proposal 2.2.3b-v2:

As per TR 38.769, for the OFDM-based OOK-4 waveform, from the reader perspective, when the generated number of chips for the R2D transmission does not fully occupy the last OFDM symbol, padding is used to align with the boundary of the last NR OFDM symbol for in-band operation. Further to the TR,

- Padding is to be down-selected between the following alternatives:

  - Alt 1: The content of padding is up to reader implementation and transparent to A-IoT device.

  - Alt 2: Specify the detail content of padding

[Open][H] Proposal 3.2.1a-v1:

For R2D transmission, from reader perspective, DFT-s-OFDM waveform is supported for OOK-4 modulation. For the details of DFT-s-OFDM waveform, down-selection between the followings:

- Option 1: Details of DFT-s-OFDM waveform generation is up to reader implementation and is not specified in RAN1.

  - This does not preclude RAN4 to discuss R2D waveform generation themselves when defining requirement(s), if needed.

- Option 2: Certain specification of DFT-s-OFDM waveform generation is needed in RAN1.

  - FFS: identify what potential details of 5 steps in section 4.4 of TR 38.769 needs to be specified.

[Open][M] Proposal 8.2.1a-v1:

FL conclusion: For CW waveform, A-IoT WID RP-243326 and TR 38.769 already give us:

It is left up to specification drafting phase to capture the following.

- Carrier wave transmission for waveform 1 only, without hopping, per the following cases in TR 38.769: - Case 1-4 for D1T1-B

- 6.8.2 CW characteristics Candidates for the CW for D2R backscattering are waveforms consisting of: Waveform 1: A single-tone unmodulated sinusoid, also referred to as 'a single tone'. Waveform 2: Two single tones.

[Open][H] Proposal 5.2.1a-v2:

For R2D CP handling, Alt M2-2 (as per TR 38.769) is not considered in Rel-19.

Thursday morning offline

[Open][H] Proposal 5.2.1b-2-v2:

For CP handling which retains subcarrier orthogonality,

- For Method Type 1 (as per TR 38.769)

  - For purposes of evaluating Method Type 1, it is assumed the device is aware of or determines the boundary of an OFDM symbol using the R2D timing acquisition signal (R-TAS), at least for Alt M1-1

    - Detail is up to R2D preamble design

  - Using Alt M1-1 and/or Alt M1-2 is up to device implementation as per TR 38.769

  - FFS: applicable M values

- For Method Type 2 Alt M2-1 (as per TR 38.769)

  - Method Type 1 needs to be also used for CP handling at least when CP part of an OFDM symbol is longer than a R2D chip where the CP or the whole bit/chip with CP removed at device side.

  - FFS: detail design of Alt M2-1-1 or Alt M2-1-2 [from the followings

    - Alt M2-1-1

      - Candidate 1:

        - For M less than 6: Use Alt M1-1

        - For M >=6: circular shift of the M chips’ samples before CP insertion at the reader side in addition to bit/chip skipping/dropping at the device side after bit decoding

      - Candidate 2:

        - Where a known signal shape in the tail-portion of the R2D signal is copied to the CP.

      - Candidate 3:

        - Insert padding chips at both start and end OOK chips or only at end OOK chips

    - Alt M2-1-2

      - Candidate 4:

        - Change a=b≠c case to a=b=c case by changing OOK-4 M value to adjacent M+1 for OFDM symbol n.

          - where a, b and c is the last chip of symbol n, first chip of symbol n+1 and last chip of symbol n+1.

      - Candidate 5:

        - Odd number of chips of PRDCH is transmitted in the first OFDM symbols, while M value is kept unchanged during the remaining OFDM symbols.]

  - FFS: applicable M values with or without combination of Method Type 1

- For further down-selection, companies are encouraged to provide evaluation (including overhead) among the followings to RAN1#120bis:

  - Method Type 1

  - Method Type 2 Alt M2-1-1 with or without combination of Method Type 1

  - Method Type 2 Alt M2-1-2 with or without combination of Method Type 1

  - Note: The evaluation for CP handling at least includes a small M value from {2,4,6} and a large M value from {16, 24, 32}

[Open][H] Proposal 5.2.1a-v2:

For CP handling which does not retain subcarrier orthogonality,

- For Method Type 2 Alt M2-2 (as per TR 38.769)

  - Option 1: For R2D CP handling, Alt M2-2 (as per TR 38.769) is not considered in Rel-19.

  - Option 2: FFS detail design of Alt M2-2 from [the followings:

    - Candidate 1:

      - A duration of OFDM symbol with CP has integer number of OOK symbols. CP is not copied from the end of OFDM symbol. Total 14 OFDM symbols fit in a slot.

    - Candidate 2:

      - A duration of OFDM symbol w/o CP has integer number of OOK symbols. No CP is added. Total 15 OFDM symbols (w/o CP) fit in a slot.]

[Open][M] Proposal 7.2.1d-v2:

For D2R BPSK modulation

- After line coding or square wave, chip “1” of the line codeword is mapped to the real-valued modulation symbol {1} and bit chip “0” of the codeword is mapped to the real-valued modulation symbol {-1} in the baseband.

[Open][M] Proposal 7.2.1b-v2:

For D2R OOK modulation

- After line coding or square wave, bit chip “1” of the line codeword is mapped to the real-valued modulation symbol {1} and bit chip “0” of the codeword is mapped to the real-valued modulation symbol {0} in baseband.

[Open][M] Proposal 7.2.1c-v1:

For D2R OOK modulation

- Phase continuity of two ON chips are assumed to be kept for OOK modulation.

[Open][H] Proposal 3.2.1a-v1:

For R2D transmission, from reader perspective, DFT-s-OFDM waveform is supported for OOK-4 modulation. For the details of DFT-s-OFDM waveform generation of OOK-4 modulation, down-selection between the followings:

- Option 1: Details of DFT-s-OFDM waveform generation for OOK-4 modulation is up to reader implementation and is not specified in RAN1.

  - This does not preclude RAN4 to discuss R2D waveform generation themselves when defining requirement(s), if needed.

- Option 2: Certain specification of DFT-s-OFDM waveform generation for OOK-4 modulation is needed in RAN1.

  - FFS: identify what potential details of 5 steps in section 4.4 of TR 38.769 needs to be specified.

[Open][M] Proposal 2.2.3b-v2:

As per TR 38.769, for the OFDM-based OOK-4 waveform, from the reader perspective, when the generated number of chips for the R2D transmission does not fully occupy the last OFDM symbol, padding is used to align with the boundary of the last NR OFDM symbol for in-band operation. Further to the TR,

- Padding is to be down-selected between the following alternatives:

  - Alt 1: The content of padding is up to reader implementation and transparent to A-IoT device.

  - Alt 2: Specify the detail content of padding

Thursday afternoon online

Proposal

For R2D, for the OOK-4 modulation for M-chip per OFDM symbol transmission:

- The maximum M value is no less than 16, and to be down-selected from 32, 24 or 16 at RAN1#120bis.

- Up to the maximum M value, [3 or 4 or 5] M values to be supported from Table 6.1.1.4-1 (as per TR 38.769)

  - FFS: the exact M values

- Note: all the supported M values are supported for the PRDCH

#### [Open][H] Proposal 5.2.1b-2-v2:

For CP handling which retains subcarrier orthogonality,

- For Method Type 1 (as per TR 38.769)

  - For purposes of evaluating Method Type 1, it is assumed the device is aware of or determines the boundary of an OFDM symbol using the R2D timing acquisition signal (R-TAS), at least for Alt M1-1

    - Detail is up to R2D preamble design

  - Using Alt M1-1 and/or Alt M1-2 is up to device implementation as per TR 38.769

  - FFS: applicable M values

- For Method Type 2 Alt M2-1 (as per TR 38.769)

  - Method Type 1 and/or removal of chip(s)/bit(s) adjacent to CP needs to be also used for CP handling at least when CP part of an OFDM symbol is longer than a R2D chip where the CP or chip/bit adjacent to CP or the whole bit/chip with CP is removed at device side.

  - FFS: detail design of Alt M2-1-1 or Alt M2-1-2 from the followings:

    - Alt M2-1-1

      - Candidate 1:

        - For M less than 6: Use Alt M1-1

        - For M >=6: circular shift of the M chips’ samples before CP insertion at the reader side in addition to bit/chip skipping/dropping at the device side after bit decoding

      - Candidate 2:

        - Where a known signal shape in the tail-portion of the R2D signal is copied to the CP.

      - Candidate 3:

        - Insert padding chips at both start and end OOK chips or only at end OOK chips

    - Alt M2-1-2

      - Candidate 4:

        - Change a=b≠c case to a=b=c case by changing OOK-4 M value to adjacent M+1 for OFDM symbol n.

          - where a, b and c is the last chip of symbol n, first chip of symbol n+1 and last chip of symbol n+1.

      - Candidate 5:

        - Odd number of chips of PRDCH is transmitted in the first OFDM symbols, while M value is kept unchanged during the remaining OFDM symbols.

  - FFS: applicable M values with or without combination of Method Type 1

- For further down-selection, companies are encouraged to provide evaluation (including overhead) among the followings to RAN1#120bis:

  - Method Type 1

  - Method Type 2 Alt M2-1-1 with or without combination of Method Type 1

  - Method Type 2 Alt M2-1-2 with or without combination of Method Type 1

  - Note: The evaluation for CP handling at least includes a M value from {2,4,6,8}, a M value from {12, 16}, and a M value from {24, 32}

  - [Note: In this evaluation, the power cross OFDM symbols is assumed to be same]

#### [Open][H] Proposal 5.2.1a-v2:

For CP handling which does not retain subcarrier orthogonality,

- For Method Type 2 Alt M2-2 (as per TR 38.769)

  - Option 1: For R2D CP handling, Alt M2-2 (as per TR 38.769) is not considered in Rel-19.

  - Option 2: FFS detail design of Alt M2-2 from the following:

    - Candidate 1:

      - A duration of OFDM symbol with CP has integer number of OOK symbols. CP is not copied from the end of OFDM symbol. Total 14 OFDM symbols fit in a slot.

Offline status: no consensus whether or not to consider Alt M2-2 in Rel-19.

#### [Open][M] Proposal 7.2.1d-v2:

For D2R BPSK modulation

- After line coding or square wave, chip “1” of the line codeword is mapped to the real-valued modulation symbol {1} and bit chip “0” of the codeword is mapped to the real-valued modulation symbol {-1} in the baseband.

#### [Open][M] Proposal 7.2.1b-v2:

For D2R OOK modulation

- After line coding or square wave, bit chip “1” of the line codeword is mapped to the real-valued modulation symbol {1} and bit chip “0” of the codeword is mapped to the real-valued modulation symbol {0} in baseband.

#### [Open][M] Proposal 7.2.1c-v1:

For D2R OOK modulation

- Phase continuity of two ON chips are assumed to be kept for OOK modulation.

#### [Open][H] Proposal 3.2.1a-v1:

For R2D transmission, from reader perspective, DFT-s-OFDM waveform is supported for OOK-4 modulation. For the details of DFT-s-OFDM waveform generation of OOK-4 modulation, down-selection between the followings:

- Option 1: Details of DFT-s-OFDM waveform generation for OOK-4 modulation is up to reader implementation and is not specified in RAN1.

  - This does not preclude RAN4 to discuss R2D waveform generation themselves when defining requirement(s), if needed.

- Option 2: Certain specification of DFT-s-OFDM waveform generation for OOK-4 modulation is needed in RAN1.

  - FFS: identify what potential details of 5 steps in section 4.4 of TR 38.769 needs to be specified.

#### [Open][M] Proposal 2.2.3b-v2:

As per TR 38.769, for the OFDM-based OOK-4 waveform, from the reader perspective, when the generated number of chips for the R2D transmission does not fully occupy the last OFDM symbol, padding is used to align with the boundary of the last NR OFDM symbol for in-band operation. Further to the TR,

- Padding is to be down-selected between the following alternatives:

  - Alt 1: The content of padding is up to reader implementation and transparent to A-IoT device.

  - Alt 2: Specify the detail content of padding

Agreements achieved in RAN1#120 Athens

Agreement

For D2R, only 2SB (Double sideband) modulation is supported.

Agreement

For R2D transmission, only subcarrier spacing of 15 kHz is supported (as per TR 38.769), and no other SCS are further considered.

Agreement

For a D2R transmission, it is up to device implementation to use OOK or BPSK modulation. It is assumed that a device operates consistently with the same D2R modulation in the same inventory/command round.

Conclusion

For R2D, Bocc,R2D is not specified in RAN1.

- This does not preclude RAN4 to discuss Bocc,R2D themselves when defining requirement(s), if needed.

Agreement

RAN1 does not specify the exact bandwidth a reader shall use for Btx,R2D.

- For R2D, specify the minimum Btx,R2D # of PRBs associated to each M value.

- For R2D, whether to specify the maximum Btx,R2D # of PRBs is up to RAN4

- Note: Reader can transmit any Btx,R2D subject to this minimum Btx,R2D and the agreed 15 kHz subcarrier spacing, and subject to other limitations that are up to RAN4.

Conclusion

For CW waveform, A-IoT WID RP-243326 and TR 38.769 already give us:

It is left up to specification drafting phase how to capture the following.

- Carrier wave transmission for waveform 1 only, without hopping, per the following cases in TR 38.769: - Case 1-4 for D1T1-B

- 6.8.2 CW characteristics Candidates for the CW for D2R backscattering are waveforms consisting of: Waveform 1: A single-tone unmodulated sinusoid, also referred to as 'a single tone'. Waveform 2: Two single tones.

References

- RP-243326	New Work Item: Solutions for Ambient IoT (Internet of Things)

- TR 38.769	Study on solutions for Ambient IoT (Internet of Things) in NR

- R1-2500033	Modulation aspects for Ambient IoT	Ericsson

- R1-2500044	Discussion on modulation aspects for A-IoT physical channel	FUTUREWEI

- R1-2501372	Discussion on Ambient IoT modulation	ZTE Corporation, Sanechips

- R1-2500153	Physical channels design on modulation	Huawei, HiSilicon

- R1-2500169	Discussion on modulation aspects of physical channels design for Ambient IoT	Spreadtrum, UNISOC

- R1-2500224	Ambient IoT physical channel design and modulation	CATT

- R1-2500259	Discussion on physical channels design about modulation aspects for Ambient IoT	China Telecom

- R1-2500287	Discussion on modulation aspects of physical channel design	CMCC

- R1-2500349	Discussion on Modulation Aspects of Physical Channels Design	vivo

- R1-2500394	AIoT Physical channels design - modulation aspects	Nokia

- R1-2500424	Discussion on modulation aspects for Ambient IoT physical design	TCL

- R1-2500450	Discussion on modulation aspects of A-IoT	OPPO

- R1-2500479	Discussion on modulation aspects for A-IoT physical channel	Tejas Network Limited

- R1-2500497	Discussion on Physical Channel Modulation Aspects for Ambient-IoT	EURECOM

- R1-2500583	A-IoT Modulation Aspects	Panasonic

- R1-2500610	Discussion on modulation aspects of ambient IoT	NEC

- R1-2500651	Modulation aspects for Ambient IoT	Sony

- R1-2500732	Discussion on modulation aspects for Ambient IoT	Xiaomi

- R1-2500778	On modulation aspects for Ambient IoT	Apple

- R1-2500850	Views on Physical channels design – modulation aspects	Samsung

- R1-2500936	Modulation for R2D and D2R	Fujitsu

- R1-2500947	Discussion on D2R modulation schemes for A-IoT	IIT Kharagpur

- R1-2500949	A-IoT PHY layer design - waveform and modulation aspects	LG Electronics

- R1-2501016	A-IoT - PHY modulation aspects	MediaTek Inc.

- R1-2501071	Discussion on modulation aspects for Ambient IoT physical channel design	Lenovo

- R1-2501092	Discussion on modulation aspects	Sharp

- R1-2501108	Discussion on Physical channels design for Ambient IoT-modulation aspects	HONOR

- R1-2501117	Modulation aspects of physical channel design for Ambient IoT	InterDigital, Inc.

- R1-2501156	Physical channels design – modulation aspects	Qualcomm Incorporated

- R1-2501202	Discussion on modulation aspects of physical channel design for Ambient IoT	NTT DOCOMO, INC.

- R1-2501287	Discussion on modulation aspects of physical channels design for Ambient IoT	WILUS Inc.

- R1-2501310	Discussion on modulation related aspects of AIoT	IIT Kanpur

- R1-2501351	Discussion on Physical channels design for Ambient IoT	Indian Institute of Tech (M)