3GPP TSG RAN WG1 Meeting #120	R1-2501462

Athens, Greece, February 17th – 21st, 2025

Source:	Moderator (ZTE)

Title:	Summary of discussion on transmission power reduction for STxMP in TS 38.213

Agenda Item:	8.1

Document for:	Discussion and Decision

Introduction

In this contribution, the summary of discussion on transmission power reduction as per the following draft CR in [1] was provided.

- TS 38.214 V17.11.0

## 7.5 Prioritizations for transmission power reductions For single cell operation with two uplink carriers or for operation with carrier aggregation or for operation with a candidate cell configured by LTM-Config or for operation with multiple panel simultaneous uplink transmission configured by multipanelSchemeSDM or multipanelSchemeSFN or sTx-2Panel or multipanelSFN-Scheme, if a total UE transmit power for PUSCH or PUCCH or PRACH or SRS transmissions on serving cell(s) or on a candidate cell, if any, in a frequency range in a respective transmission occasion i would exceed PCMAX(i), where PCMAX(i) is the linear value of PCMAX(i) in transmission occasion i as defined in [8-1, TS 38.101-1] for FR1 and [8-2, TS 38.101-2] for FR2, the UE allocates power to PUSCH/PUCCH/PRACH/SRS transmissions according to the following priority order (in descending order) so that the total UE transmit power for transmissions on serving cell(s) or on a candidate cell, if any, in the frequency range is smaller than or equal to PCMAX(i) for that frequency range in every symbol of transmission occasion i. If the UE transmits SRS on multiple SRS resources according to Clause 6.2.1.4 of [6, TS 38.214], the UE allocates power so that all REs of the SRS transmission have same power. <Unchanged part is omitted>

Discussion

According to the agreement reached in RAN1#112 meeting [2], for STxMP, the maximum Tx power when the UE determines UL Tx power for the PUSCH/PUCCH transmission occasion(s) or antenna port(s) are intended to be discussed after receiving RAN4 reply on UE power limitation for STxMP in FR2. Till now, the configured UE maximum output power for STxMP has already been captured in RAN1 and RAN4 specifications.

Agreement On unified TCI framework extension, if an indicated joint/UL TCI state(s) applies to a PUSCH/PUCCH/SRS transmission occasion(s) or antenna port(s), the UE shall determine UL Tx power for the PUSCH/PUCCH/SRS transmission occasion(s) or antenna port(s) based on the UL PC parameter setting for PUSCH/PUCCH/SRS, if any, and the PL-RS included in the indicated joint/UL TCI state - FFS: For STxMP, the maximum Tx power when the UE determines UL Tx power for the PUSCH/PUCCH transmission occasion(s) or antenna port(s) (discussed after receiving RAN4 reply on UE power limitation for STxMP in FR2) - FFS: Default UL PC parameter setting(s) if one or both of indicated joint/UL TCI states applied to PUSCH/PUCCH/SRS transmission occasion(s) or antenna port(s) does/do not include the UL PC parameter setting(s) for PUCCH/PUSCH/SRS

According to clauses 7.1 and clause 7.2 in TS 38.213, in case of STxMP, where UE determines to apply both the first TCI-State or TCI-UL-State and the second TCI-State or TCI-UL-State in a PUCCH/PUSCH transmission occasion, UE should determine PUCCH/PUSCH transmission power for the first and second TCI-State or TCI-UL-State based on the UE configured maximum output power for the first and second TCI-State or TCI-UL-State, respectively. The UE configured maximum output power for each of joint/UL TCI states indicated for simultaneous transmission to multiple directions are defined in RAN4 specifications.

According to the following conditions excerpted from the LS from RAN4 regarding the configured Tx power for STxMP in FR2 [3], we have the following considerations.

RAN4 would like to confirm to RAN1 the conditions to be simultaneously met by a UE for STxMP: 1. The total EIRP over all panels cannot exceed the existing EIRP limitation. 2. The total TRP over all panels cannot exceed the existing TRP limitation. 3. The sum of per-panel EIRP power limitation for STxMP can be greater than the existing EIRP limitation. a. See condition #1, which must always be obeyed. b. In cases where the beams for each panel have low enough spatial overlap, this clause allows EIRP for each panel to individually approach the existing EIRP limitation. c. RAN4 has not completed spec. definition work for ‘per panel EIRP power limitation’. 4. No explicit ‘per-panel’ TRP limit is deemed necessary.

- Condition-1: EIRP represents the limitation of the total transmission power of the entire device. Regardless of the number of panels, the total EIRP must not exceed this limitation.

- Condition-2: TRP focuses on the total radiated power of the entire device across all directions, while EIRP represents the maximum power in a specific direction. Both EIRP and TRP serve as respective metrics for presenting total Tx power limitation.

- Condition-3: Although the sum of per-panel EIRP power limitation may exceed the existing EIRP limit, during actual uplink transmission, the total EIRP still cannot exceed the limitation. This may require sufficient spatial isolation between beams to avoid excessive total EIRP caused by simultaneous high-power transmissions.

Meanwhile, according to the following excerpted from clause 6.2K.4 in TS 38101-2, it can be observed that the above conditions have been captured in RAN4 specifications.

### 6.2K.4 Configured transmitted power for simultaneous transmission to multiple directions <Unrelated parts are omitted> The configured UE maximum output power PCMAX,f,c,k shall be set such that the corresponding measured peak EIRP PUMAX,f,c,k is within the following bounds PPowerclass + ΔPIBE – MAX(MAX(MPRf,c,k, A- MPRf,c,k,) + ΔMPRSTxMP + ΔMBP,n, P-MPRf,c,k) – MAX{T(MAX(MPRf,c,k, A- MPRf,c,k,) + ΔMPRSTxMP), T(P-MPRf,c,k)} – ΔTSTxMP ≤ PUMAX,f,c,k ≤ EIRPmax and PUMAX,f,c, the corresponding measured peak EIRP for carrier f of a serving cell c, aggregated over all indicated joint/UL TCI states in a given direction, satisfies over all directions PUMAX,f,c ≤ EIRPmax while the corresponding measured total radiated power PTMAX,f,c is bounded by PTMAX,f,c ≤ TRPmax <Unrelated parts are omitted>

Based on the above analysis, we can assume that the sum of per-panel power limitation for STxMP can be greater than the existing power limitation for a given power class while the sum of actual transmission power for STxMP cannot be greater than the existing power limitation for a given power class.

For scenarios involving two uplink carriers, carrier aggregation and LTM, transmission power reduction operation is specified in clause 7.5 in TS 38.213 if the total UE transmit power for uplink transmissions on serving cells or on a candidate cell in a respective transmission occasion would exceed the linear value of maximum output power in the transmission occasion.

Therefore, for STxMP in a single carrier of a single serving cell, the total UE transmit power for simultaneously transmitted PUSCH or PUCCH on the serving cell in a frequency range in a respective transmission occasion i can exceed the linear value of PCMAX(i) in transmission occasion i. Then, transmission power reduction should also be operated to allocate power to the PUSCH/PUCCH transmissions so that the total UE transmit power for transmissions on the serving cell is not greater than PCMAX(i) in transmission occasion i.

In light of the above, we have the following proposal and the corresponding CR can be found in [3].

Proposal 1: Support transmission power reduction for multiple panel simultaneous uplink transmission of a single carrier of a single serving cell.

Proposal 1: Support transmission power reduction for multiple panel simultaneous uplink transmission of a single carrier of a single serving cell so that the total UE transmit power for PUSCH or PUCCH does not exceed PCMAX(i).

- FFS: How to determine the priority order between the two panels for SDM/SFN/mDCI STxMP schemes.

Companies are welcome to provide comments of the following questions in the table below.

- Q1: Do you agree with Proposal 1 to capture that the sum of actual transmission power for STxMP cannot be greater than the existing output power limitation? If not, please explain whether/how to address this issue.

- Q2: If your answer to Q1 is yes, do you agree with the draft CR provided in [1]? If not, please suggest anything else (e.g., modifications or different context) properly.

Companies’ views

| Company | Comments |
| --- | --- |
| Huawei/HiSilicon | In principle, agree with STxMP power reduction. However, priority order between the two panels should be determined. Otherwise agreeing on the CR as is just creates more confusion. current priority orders in 7.5 of 38.213 are for two different PUSCH/PUCCH with the same or different priorities. However, in SDM, UE transmit different layers of the same PUSCH from the two panels and in SFN UE transmit only one PUSCH. We suggest to agree with the modified version of the proposal in the discussion paper as follows and leave the priority determination as FFS: Proposal 1: Support transmission power reduction for multiple panel simultaneous uplink transmission of a single carrier of a single serving cell so that the total UE transmit power for PUSCH or PUCCH does not exceed PCMAX(i). - FFS: How to determine the priority order between the two panels for SDM/SFN/mDCI STxMP schemes. |
| vivo | Not needed. Total UE transmit power for simultaneously UL transmission on one serving cell will not exceed the linear value of PCMAX(i) in transmission occasion i because it is determined based on sum of pUMAX,f,c and pTMAX,f,c. And maximum transmission power of two panels are derived to be not greater than these two parameters. |
| Mod | @Huawei/Hisilicon: Thanks for discussion. Your update of Proposal 1 seems good to me, let’s check if we can take a baby step of capture transmission output power reduction for STxUL scheme, and then further study on how to determine the priority rule. @vivo: Thanks for discussion. As commented in Monday online session, I agree with you the total maximum output power limitation of STxUL transmission was already ensured by PUMAX,f,c and PTMAX,f,c as specified in TS 38.101-2. However, regarding whether/how does UE reduce the total output power for STxUL transmission when it exceeds the maximum output power limitation is still missing in the specs. In the meanwhile, it is worth to be noted, in case of CA operation, SUL and LTM, the total maximum output power limitation was specified by RAN4 in TS 38.101-2 while how to reduce the total output power to meet the limitation was specified by RAN1 in TS 38.213. @All: Please check and provide your views on the updated Proposal 1 from Huawei/HiSilicon. |
| OPPO | We are fine with the updated proposal. As mentioned by Huawei, the signals in different panels usually have the same type and priority, which cannot be covered by 7.5. Additional rule at least between two panels is needed. |
| Samsung | Q1. We can discuss whether transmission power reduction could be specified in RAN1 or not. According to RAN4’s LS, power reduction might be needed due to total EIRP/total TRP over all panels not to exceed the existing EIRP/TRP limitation for a given power class. This seems that RAN4 already captured as above (PUMAX,f,c and PTMAX,f,c are bounded up to EIRPmax and TRPmax respectively). Q2. We are open to clarify after deciding answer of Q1. |
| Mod | Thanks for comments of the discussion, companies standpoint of the latest Proposal 1 can be collected as follows, let’s try whether it can be acceptable to the opponent in online session. Proposal 1: Support transmission power reduction for multiple panel simultaneous uplink transmission of a single carrier of a single serving cell so that the total UE transmit power for PUSCH or PUCCH does not exceed PCMAX(i). - FFS: How to determine the priority order between the two panels for SDM/SFN/mDCI STxMP schemes. Support/fine: Huawei/HiSilicon, ZTE, OPPO, Samsung Not support: vivo |

Conclusion

Proposal 1: Support transmission power reduction for multiple panel simultaneous uplink transmission of a single carrier of a single serving cell so that the total UE transmit power for PUSCH or PUCCH does not exceed PCMAX(i).

- FFS: How to determine the priority order between the two panels for SDM/SFN/mDCI STxMP schemes.

References

- R1-2500536, Draft CR on transmission power reduction for STxMP in TS 38.213, ZTE Corporation, Sanechips

- Chair notes RAN1#112 eom, Feb 27th - Mar 3rd, 2023, 3GPP RAN1

- R4-2306657, Reply LS on configured Tx power for STxMP in FR2, Apr, 2023, RAN4