3GPP TSG RAN WG1#120		R1-2501489

Athens, Greece, February 17th – 21st, 2025

Agenda item:	8.1

Source:	Moderator (Samsung)

Title:	Summary #2 of discussion on the timeline issue for STxMP

Document for:	Discussion and Decision

# Introduction

This contribution aims to collect and summarize company views on the timeline issue for STxMP as discussed in [1].

Please consider entering the contact information below for better coordination for this discussion.

| Company | Contact(s) | Email address(es) |
| --- | --- | --- |
| Samsung (Moderator) | Sa Zhang | sa.zhang@samsung.com |
| ZTE | Yang Zhang | zhang.yang220@zte.com.cn |

# Background

In [1] Samsung proposes a CR to capture the following agreements made in RAN1#119.

Agreement: Introduce following FG for UE STxMP processing capability for codebook. - FG name - UE STxMP processing capability for codebook - Component - 1. Require additional timeline to process multiple TBs for codebook multi-DCI based STx2P PUSCH+PUSCH for DG+DG - Note. This FG can be applied for CG+DG also if UE can support those FG. - Prerequisite - 40-6-3a - Consequence if not supported - UE should process multiple TBs within legacy timeline - Type - Per FSPC - Need of FR1/FR2 differentiation - FR2 only - Note - candidate values: - UE reports candidate value independently for each SCS in unit of symbols - For 15kHz SCS: {1,2,4} - For 30kHz SCS: {1,2,4,8} - For 60kHz SCS: {2,4,8,16} - For 120kHz SCS: {4,8,16,32} - For 480kHz SCS: {16,32,64,128} - For 960kHz SCS: {32,64,128,256} - Note: SCS above is the minimum between SCS of the scheduling DCI and SCS of the scheduled PUSCH. - Mandatory or optional - Optional with capability signaling Agreement: Introduce following FG for UE STxMP processing capability for non-codebook. - FG name - UE STxMP processing capability for non-codebook - Component - 1. Require additional timeline to process multiple TBs for noncodebook multi-DCI based STx2P PUSCH+PUSCH for DG+DG - Note. This FG can be applied for CG+DG also if UE can support those FG. - Prerequisite - 40-6-3b - Consequence if not supported - UE should process multiple TBs within legacy timeline - Type - Per FSPC - Need of FR1/FR2 differentiation - FR2 only - Note - candidate values: - UE reports candidate value independently for each SCS in unit of symbols - For 15kHz SCS: {1,2,4} - For 30kHz SCS: {1,2,4,8} - For 60kHz SCS: {2,4,8,16} - For 120kHz SCS: {4,8,16,32} - For 480kHz SCS: {16,32,64,128} - For 960kHz SCS: {32,64,128,256} - Note: SCS above is the minimum between SCS of the scheduling DCI and SCS of the scheduled PUSCH. - Mandatory or optional - Optional with capability signaling

The CR is copied below.

## 6.4 UE PUSCH preparation procedure time If the first uplink symbol in the PUSCH allocation for a transport block, including the DM-RS, as defined by the slot offset K2 and Koffset, if configured, and the start S and length L of the PUSCH allocation indicated by 'Time domain resource assignment' of the scheduling DCI and including the effect of the timing advance, is no earlier than at symbol L2, where L2 is defined as the next uplink symbol with its CP starting Tproc,2=max(N2+d2,1+d2+d3)(2048+144)⋅κ2-μ⋅TC+Text+Tswitch,d2,2 after the end of the reception of the last symbol of the PDCCH carrying the DCI scheduling the PUSCH, then the UE shall transmit the transport block. When the PDCCH reception includes two PDCCH candidates from two respective search space sets, as described in clause 10.1 of [6, TS 38.213], for the purpose of determining the last symbol of the PDCCH carrying the DCI scheduling the PUSCH, the PDCCH candidate that ends later in time is used. - N2 is based on µ of Table 6.4-1 and Table 6.4-2 for UE processing capability 1 and 2 respectively, where µ corresponds to the one of (µDL, µUL) resulting with the largest Tproc,2, where the µDL corresponds to the subcarrier spacing of the downlink with which the PDCCH carrying the DCI scheduling the PUSCH was transmitted and µUL corresponds to the subcarrier spacing of the uplink channel with which the PUSCH is to be transmitted, and κ is defined in clause 4.1 of [4, TS 38.211]. - For operation with shared spectrum channel access in FR1, is calculated according to [4, TS 38.211], otherwise =0. - If the first symbol of the PUSCH allocation consists of DM-RS only, then d2,1 = 0, otherwise d2,1 = 1. - If the UE is configured with multiple active component carriers, the first uplink symbol in the PUSCH allocation further includes the effect of timing difference between component carriers as given in [11, TS 38.133]. - If the scheduling DCI triggered a switch of BWP, d2,2 equals to the switching time as defined in [11, TS 38.133], otherwise d2,2=0. - If a PUSCH of a larger priority index would overlap with a PUCCH of a smaller priority index and the PUCCH and PUSCH are not simultaneously transmitted, and the UE is not provided uci-MuxWithDiffPrio for the primary PUCCH group or uci-MuxWithDiffPrioSecondaryPUCCHgroup for the secondary PUCCH group, d2 for the PUSCH of a larger priority is set as reported by the UE; otherwise d2 = 0. - For a UE that supports capability 2 on a given cell, the processing time according to UE processing capability 2 is applied if the high layer parameter processingType2Enabled in PUSCH-ServingCellConfig is configured for the cell and set to 'enable', - If the PUSCH indicated by the DCI is overlapping with one or more PUCCH channels, then the transport block is multiplexed following the procedure in clause 9.2.5 of [6, TS 38.213], otherwise the transport block is transmitted on the PUSCH indicated by the DCI. - If uplink switching gap is triggered as defined in clause 6.1.6, equals to the switching gap duration and for the UE configured with higher layer parameter uplinkTxSwitchingOption set to 'dualUL' for uplink carrier aggregation µUL=min(µUL,carrier1, µUL,carrier2), otherwise . - If the UE is configured with sTx-2Panel, d3 for the PUSCH to be simultaneously transmitted with another PUSCH with a different value of coresetPoolIndex on a same serving cell is set as reported by the UE; otherwise d3 = 0. Otherwise the UE may ignore the scheduling DCI. The value of is used both in the case of normal and extended cyclic prefix. Table 6.4-1: PUSCH preparation time for PUSCH timing capability 1 PUSCH preparation time N2 [symbols]. 0; 10. 1; 12. 2; 23. 3; 36. 5; 144. 6; 288. Table 6.4-2: PUSCH preparation time for PUSCH timing capability 2 PUSCH preparation time N2 [symbols]. 0; 5. 1; 5.5. 2; 11 for frequency range 1. ## *** Unchanged parts are omitted ***

# Discussion

## 1st round

Q1: Do you agree with the CR proposed in [1]?

| Company | View |
| --- | --- |
| ZTE | Agree in principle. Regarding the description of MDCI based STxMP PUSCH+PUSCH transmission, it is proper to use the wording as specified in TS 38.214, i.e., “two PUSCHs that are fully/partially overlapping in time domain and are fully/partially/non-overlapping in frequency domain”. Besides, the description of “the value of coresetPoolIndex” for the former PUSCH also needs to be captured. Consequently, the following updates were provided. - If the UE is configured with sTx-2Panel, d3 for the PUSCH that associated with a value of coresetPoolIndex to be fully/partially overlapping in time domain and to be fully/partially/non-overlapping in frequency domainwith another PUSCH that associated with a different value of coresetPoolIndex on a same serving cell is set as reported by the UE; otherwise d3 = 0. |

Q2: Any other comments?

| Company | View |
| --- | --- |

## 2nd round

During the Monday online session, it was confirmed by companies that the agreement should be captured in RAN1 specifications. As clarified during online session, from Moderator’s perspective, both CRs can work. Companies are encouraged to provides comments on the wording of the draft CR, if any, before 12 a.m. Tuesday (local time).

Q3: Which option do you prefer to capture the CR in Rel-18 TS 38.214 clause 6.4?

- Option 1: Samsung’s CR provided in R1-2500831

- Option 2: ZTE’s update based on Samsung’s CR provided in R1-2500831.

-	If the UE is configured with sTx-2Panel, d3 for the PUSCH that associated with a value of coresetPoolIndex to be fully/partially overlapping in time domain and to be fully/partially/non-overlapping in frequency domainwith another PUSCH that associated with a different value of coresetPoolIndex on a same serving cell is set as reported by the UE; otherwise d3 = 0.

- Option 3: HW’s update as below.

- d3 is set as reported by UE if the UE is configured with sTx-2Panel and the PUSCH is associated with a value of coresetPoolIndex and is fully/partially overlapping in time domain and is fully/partially/non-overlapping in frequency domain with another PUSCH that is associated with a different value of coresetPoolIndex; otherwise d3 = 0.

| Company | View |
| --- | --- |
| Huawei/HiSilicon | OK with ZTE modification in principle. But the sentence doesn’t read well. Suggest the following text …. - d3 is set as reported by UE if the PUSCH is associated with a value of coresetPoolIndex and is fully/partially overlapping in time domain and is fully/partially/non-overlapping in frequency domain with another PUSCH that is associated with a different value of coresetPoolIndex; otherwise d3 = 0. |
| Mod | Regarding HW’s update, it seems the RRC parameter is needed, please check Option 3 with moderator’s update in highlighted yellow text. |
| Ericsson | Support Option 3 with moderator’s update |

# Conclusion

Three versions are proposed by three companies, it is proposed to do down selection online.

Proposal: Down select from the following two CRs for the timeline issue of STxMP for Rel-18 TS 38.214 clause 6.4.

Option 1: Samsung’s CR provided in R1-2500831

Option 2: ZTE’s update based on Samsung’s CR provided in R1-2500831.

-	If the UE is configured with sTx-2Panel, d3 for the PUSCH that associated with a value of coresetPoolIndex to be fully/partially overlapping in time domain and to be fully/partially/non-overlapping in frequency domainwith another PUSCH that associated with a different value of coresetPoolIndex on a same serving cell is set as reported by the UE; otherwise d3 = 0.

Option 3: HW’s update as below.

- d3 is set as reported by UE if the UE is configured with sTx-2Panel and the PUSCH is associated with a value of coresetPoolIndex and is fully/partially overlapping in time domain and is fully/partially/non-overlapping in frequency domain with another PUSCH that is associated with a different value of coresetPoolIndex on a same serving cell; otherwise d3 = 0.

# Reference

[1] R1-2500831	Draft CR on timeline for STxMP	Samsung