3GPP TSG RAN WG1 #120			R1-250XXXX

Athens, Greece, February 17th – 21st, 2025

Agenda Item:	9.13

Source:	Ad-Hoc Chair (Huawei)

Title:	Session notes for 9.13 LTE-based 5G Broadcast Phase 2

Document for:	Discussion, Decision

## LTE-based 5G Broadcast Phase 2

Please refer to RP-243290 for detailed scope of the WI for LTE-based 5G Broadcast Phase 2.

[120-R19-Broadcast] Email discussion on Rel-19 LTE-bad 5G Broadcast Phase 2 – Jordi (EBU)

- To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc

R1-2500872	Time-frequency interleaver design for MBMS dedicated cells	Samsung

R1-2501057	Introduction of WI “Time and Frequency Interleaving for LTE-based 5G Terrestrial Broadcast” 	EBU

### Time-frequency interleaver design for MBMS dedicated cells

R1-2500148	Discussion on time-frequency interleaver design for LTE-based 5G Broadcast	ZTE Corporation, Sanechips

R1-2500150	On time-frequency interleavers for LTE-based 5G Broadcast	Huawei, HiSilicon

R1-2500582	Design of Time and Frequency Interleaver for LTE-based 5G Broadcast	Shanghai Jiao Tong University

R1-2501058	Details of Time and Frequency Interleaving for LTE-based 5G Terrestrial Broadcast 	EBU

R1-2501179	Time and Frequency Interleaving for 5G Broadcast	Qualcomm Incorporated

R1-2501432	FL summary#1 on Rel-19 LTE-based 5G Broadcast Phase 2	Moderator (EBU)

Agreement

For TBS scaling by N, down-select one of the two options below:

- Option 1: Calculate the total number of PRBs NRBtotal as NRBtotal=NRBDL×N, wherein, NRBDL is system bandwidth;

    - If NRBtotal≤110, reuse the transport block size table in TS 36.213 and according to the obtained ITBS and NRBtotal, obtain the corresponding TBS;

    - Else, reuse the transport block size table in TS 36.213 and according to the obtained ITBS and NRBDL, obtain the corresponding TBS’; and calculate the final TBS as TBS=round{TBS'×N}

The operation “round{x}” is mapping x to the closest TBS from all the existing TBS values in section 7.1.7.2 TS 36.213.

- Option 2: Obtain the initial TBS (TBS') as per current specifications, and calculate the final TBS as TBS=round{TBS'×N}

  - The operation “round{x}” is mapping x to the closest TBS from all the existing TBS values in section 7.1.7.2 TS 36.213.

Agreement

At least values of N={1,4,8} are supported, where N=1 indicates time interleaving is disabled.

  - FFS: other values

Agreement

LBRM is applied for MCH (excluding MCCH and MSI) with time interleaving

  - FFS spec impact

Agreement

For PMCH with time interleaving, the maximum TBS a UE supports for the scaled TB is derived based on DL-SCH as per TS 36.306 set by UE category.

  - FFS when the UE category has multiple maximum TBS

Agreement

The maximum number of transport block bits received within a TTI defined by the UE category should include the number of bits for unicast and broadcast, where the number of bits for broadcast is the one after being scaled up by N.

Agreement

The basic transmission pattern of PMCH with time interleaving consists of a set of (M x N) consecutive MBSFN subframes, excluding MCCH and MSI, where two transmissions of the same TB are separated by (M-1) MBSFN subframes, excluding MCCH and MSI.

Agreement

Specify time interleaving per MBMS session/MTCH without supporting interleaving across different MBMS sessions/MTCH.

Agreement

Regarding configuration for time interleaving transmission, the following options can be considered:

- Option 1: Per MBSFN area/MCCH configuration, e.g., via mcch-Config

- Option 2: Per PMCH configuration, e.g., via pmch-Config

- Option 3: Per MBMS session, e.g., via MBMS-SessionInfo or via MAC CE

Agreement

RAN1 to further study the potential issue of mismatch between MSI periodicity and the basic transmission pattern of (M x N) subframes.

Agreement

The equation of k0 for time interleaving is modified to avoid puncturing of systematic bits.

- Further study the exact equation for k0 for the n-th subframe of a transmission belonging to a same TB k0n, n∈{0, 1, 2, …, N-1}), including

  - Option 1: for each CB of the TB, k0n=2RsubblockTC+rvnEmin, where Emin=NLQmG'/C (as per TS 36.212), and rvn∈{0…N-1} is the redundancy version of the n-th subframe.

  - Option 2: For each CB of the TB, follow the principle of NR TBoMS to determine k0n

  - Option 1 and Option 2 assumes all CBs of the TB have the same RV index for all CBs of the TB, indexed by k0n

- FFS whether different CBs of the TB mapped to each subframe may have different RV indices (e.g., indexed by different k0n per CB)

Agreement

The modulation symbol vector (prior to frequency interleaving) y=y0,y1,…,yZ-1 is interleaved, according to the principles of subclause 5.1.4.1.1 of TS 36.212 as follows

- Set the number of columns of the frequency interleaver, C=X, set the number of rows R to be the minimum integer satisfying Z≤R×C, where Z is the number of data REs in one OFDM symbol

  - FFS: How to determine X

- Append y with <NULL> elements, as required, to obtain yapp

- Write the elements of yapp into the R×C-matrix column-wise, with yapp(0) in column 0 of row 0.

- FFS whether the rows and elements in each row will be permuted, with details to be further studied, e.g., intra-row cyclic shifts, inter-row permutations

- Obtain the vector yΠ by reading the elements of the above R×C-matrix row-wise, excluding the <NULL>s.

- The vector yΠ shall then be mapped to resource elements, according to clause 6.3.5 of TS 36.211.

FFS: whether to apply a cyclic shift offset, and the details of the cyclic shift offset for the concatenated frequency interleaver outputs of all OFDM symbols before the mapping to REs of a subframe, where the cyclic shift offset is dependent on the RV index.

Agreement

The granularity of the frequency interleaver is 1 RE.

R1-2501433	FL summary#2 on Rel-19 LTE-based 5G Broadcast Phase 2	Moderator (EBU)