3GPP TSG RAN WG1 #120			R1-250XXXX

Athens, Greece, February 17th – 21st, 2025

Agenda Item:	9.7

Source:	Ad-Hoc Chair (Huawei)

Title:	Session notes for 9.7 Study on channel modelling for ISAC for NR

Document for:	Discussion, Decision

## Study on channel modelling for Integrated Sensing And Communication (ISAC) for NR

Please refer to RP-242348 for detailed scope of the SI.

[120-R19-ISAC] Email discussion on Rel-19 ISAC channel model – Yingyang (xiaomi)

- To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc

R1-2500998	Draft CR for TR 38.901 to introduce channel model for ISAC	Xiaomi, AT&T

### ISAC deployment scenarios

Please provide your inputs on calibrations is to this sub-agenda item.

R1-2500059	Email discussion summary on ISAC CM calibration assumptions	Moderator (AT&T)

R1-2500071	Deployment scenarios for ISAC channel model	Huawei, HiSilicon

R1-2500234	Discussion on ISAC deployment scenarios	CATT, CICTCI

R1-2500266	Discussion on ISAC deployment scenarios	China Telecom

R1-2500297	Discussion on ISAC deployment scenarios	CMCC, China Southern Power Grid

R1-2500312	Discussion on ISAC scenario	CALTTA

R1-2500360	Views on Rel-19 ISAC deployment scenarios	vivo

R1-2500413	Discussion on ISAC deployment scenarios and requirements	EURECOM

R1-2500417	Discussion on ISAC deployment scenarios	Tiami Networks

R1-2500462	Discussion on ISAC deployment scenarios and calibration	OPPO

R1-2500482	Discussion on ISAC Deployment Scenarios 	Tejas Network Limited

Late submission

R1-2500576	Discussion on ISAC deployment scenarios	ZTE Corporation, Sanechips

R1-2500659	Discussion on ISAC deployment scenarios	Sony

R1-2500679	Discussion on ISAC Deployment Scenarios	Nokia, Nokia Shanghai Bell

R1-2500684	Discussion on ISAC deployment scenarios	SK Telecom

R1-2500691	Deployment scenarios for integrated sensing and communication with NR	NVIDIA

R1-2500742	Deployment scenarios and evaluation assumptions for ISAC channel model	Xiaomi

R1-2500747	Discussion on ISAC deployment scenarios	InterDigital, Inc.

R1-2500755	Discussion on ISAC Deployment Scenarios	Ericsson

R1-2500796	Discussion on ISAC deployment scenarios	Apple

R1-2500860	Discussion on  ISAC deployment scenarios	Samsung

R1-2500891	Considerations on ISCA deployment scenarios	CAICT

R1-2501011	Discussion on ISAC deployment scenarios for Automotive	Continental Automotive

R1-2501026	Discussion on ISAC deployment scenario	MediaTek Inc.

R1-2501045	Discussion on ISAC deployment scenarios	LG Electronics

R1-2501059	Discussion on ISAC deployment scenarios 	Lenovo

R1-2501081	ISAC channel model calibration and scenario parameters	AT&T, FirstNet

R1-2501135	Discussion on ISAC channel calibration	BUPT, CMCC

R1-2501166	Discussion on ISAC deployment scenarios	Qualcomm Incorporated

R1-2501076	FL Summary #1 on ISAC Scenarios and Calibrations	Moderator (AT&T)

R1-2501077	FL Summary #2 on ISAC Scenarios and Calibrations	Moderator (AT&T)

Agreement

      - For ISAC channel modelling calibration, RAN1 considers both large-scale and full-scale calibration to include parameters and values for at least the following:

    - large scale parameters, where fast fading is not included

    - full-scale calibration parameters, which includes fast fading.

- NOTE0: one part of calibration work does not include additional components and does not include spatial consistency

    - FFS: whether spatial consistency is specified as an additional component for ISAC CM

- NOTE1: additional calibrations including spatial consistency can also be considered case by case for different scenarios.

- NOTE2: Inclusion of EO in ISAC CM calibrations can also be considered case by case for different scenarios.

Agreement

Calibration of ISAC CM includes separate calibration of the target channel and of the background channel

      - FFS: additional calibration for the combined channel (combination of target and background channel).

R1-2501078	FL Summary #3 on ISAC Scenarios and Calibrations	Moderator (AT&T)

### ISAC channel modelling

R1-2500072	Channel modelling for ISAC	Huawei, HiSilicon

R1-2500179	Discussion on ISAC channel modeling	Spreadtrum, UNISOC

R1-2500235	Discussion on ISAC channel modelling	CATT, CICTCI

R1-2500267	Discussion on ISAC channel modelling	China Telecom

R1-2500298	Discussion on channel modeling methodology for ISAC	CMCC, BUPT, SEU, PML

R1-2500313	Discussion on ISAC channel modelling	CALTTA

R1-2500361	Views on Rel-19 ISAC channel modelling	vivo, BUPT

R1-2500414	Discussion on ISAC channel modeling	EURECOM

R1-2500418	Discussion on ISAC Channel Modeling	Tiami Networks

R1-2500463	Study on ISAC channel modelling	OPPO

R1-2500483	Discussion on ISAC channel modelling	Tejas Network Limited

R1-2500577	Discussion on channel modelling for ISAC	ZTE Corporation, Sanechips

R1-2500626	ISAC Channel Modeling and Measurement Validation	BUPT, CMCC, VIVO

R1-2500660	Discussion on Channel Modelling for ISAC	Sony

R1-2500680	Discussion on ISAC channel modeling	Nokia, Nokia Shanghai Bell

R1-2500681	Discussion on ISAC Channel Modeling	NIST

R1-2500685	Discussion on ISAC channel modelling	SK Telecom

R1-2500692	Channel modeling for integrated sensing and communication with NR	NVIDIA

R1-2500743	Discussion on ISAC channel model	Xiaomi, BJTU, BUPT

R1-2500748	Discussion on ISAC channel modeling	InterDigital, Inc.

R1-2500756	Discussion on ISAC Channel Modelling	Ericsson

R1-2500797	Discussion on ISAC channel modelling	Apple

R1-2500861	Discussion on ISAC channel modelling	Samsung

R1-2500892	Considerations on ISAC channel modelling	CAICT

R1-2500979	Discussion on ISAC Channel Modelling 	Panasonic

R1-2501027	Discussion on ISAC channel modelling	MediaTek Inc.

R1-2501046	Discussion on ISAC channel modelling	LG Electronics

R1-2501060	Discussion on Channel Modelling for ISAC	Lenovo

R1-2501082	Discussions on ISAC Channel Modeling	AT&T

R1-2501167	Discussion on ISAC channel modelling	Qualcomm Incorporated

R1-2501212	Discussion on ISAC channel modeling	NTT DOCOMO, INC.

R1-2500999	Summary #1 on ISAC channel modelling	Moderator (Xiaomi)

Agreement

For bistatic/monostatic RCS

- RCS values/pattern for a scattering point of a target for bistatic sensing is generated by A*B1*B2 (i.e., Option 3 from the agreement in RAN1 #118bis)

- RCS values/pattern obtained by setting the same incident/scattered angle in the RCS model for bistatic sensing should be aligned with RCS for monostatic sensing

Agreement

RCS model and application in ISAC channel generation

- To define the RCS model (RCS=A*B1*B2) for a scattering point of a target, when the target type is vehicle, large size UAV, human with RCS model 2, AGV

  - The values/pattern of the product of component A and B1, i.e., A*B1 is given per target type, expressed in dBsm scale

  - Component A is expressed in dBsm scale. B1 is dependent on A*B1 and value of component A.

    - A is equal to a single value per target type

      - FFS: this allows different values for the same target type with different size, if needed

      - FFS: this allows different values for monostatic and bistatic sensing, if needed

  - Component B2 follows log-normal distribution. The mean μB2_dB and variance σB2_dB2 used to characterize 10lgB2 satisfied a fixed relation μB2_dB=-ln1020σB2_dB2.

- In the procedure of generating ISAC target channel, B1*B2 is applied after coupling of rays for a STX-SPST link and the corresponding SPST-SRX link before path dropping

- In the procedure of generating ISAC target channel, the following power scaling factor is applied in the last step in target channel generation (i.e., step 14 in the running CR).

LTX-SPST-RX=PLdBd1+PLdBd2+10lgc24πf2-10lgσRCS,A+SFdB,1+SFdB,2

Where,

    - PLdBd1 is pathloss between Tx and SPST, where d1 is the distance between Tx and SPST

    - PLdBd2 is pathloss between Rx and SPST, where d2 is the distance between SPST and Rx

    - σRCS,A is the value of RCS component A

    - SFdB,1,SFdB,2 are shadow fading respectively generated for the Tx- SPST link and SPST -Rx link referring to step 4 in section 7.5, TR 38.901

    - Note: for monostatic sensing, PLdBd1=PLdBd2,SFdB,1=SFdB,2

Agreement

RCS upper bound: k equals to 3 is adopted to derive the upper bound of RCS component B2, kσ, where σ is the standard deviation of B2 in dB

Agreement

For reducing options for reference TRs: for sensing scenario UMi, UMa, RMa, InH, InF, UMi-AV, UMa-AV, and RMa-AV, the reference TR to generate a TRP-TRP channel is:

- TRP-TRP link of scenario UMi, UMa, InH, and InF following the option based on TR 38.901 defined in section A.3 of TR 38.858

  - For InF, hUE is changed to the same height as the BS

- TRP-UE link of scenario RMa defined in section 7 of TR 38.901 by setting hUE=35m

- FFS: whether to add very low power clusters

R1-2501000	Summary #2 on ISAC channel modelling	Moderator (Xiaomi)

Agreement

For vehicle with single/multiple scattering points:

- For mono-static, the RCS=A*B=A*B1*B2 for a scattering point of a vehicle is generated by

  - The values/pattern A*B1, i.e., rcsdB(θ,φ) is deterministic based on incident/scattered angles

rcsdB(θ,φ)=Gmax-min-σVdBθ+σHdB φ,σmax

Where,

σVdBθ=-min12θ-θcenterθ3dB2, σmax,

σHdB φ=-min12φ-φcenterφ3dB2, σmax,

For example, in case of vehicle with multiple scattering points:

|  | φcenter | φ3dB, n | θcenter | θ3dB,n | Gmax | σmax | Applicable Range of θ | Applicable Range of φ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Left | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] |
| Back | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] |
| Right | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] |
| Front | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] |
| Roof | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] |

  - Note: the applicable angular range is 360 degrees per row in horizontal domain in case of vehicle with multiple scattering points, and the applicable angular range is < 360 degrees per row in horizontal domain in case of vehicle with a single scattering point.

    - FFS: angular continuity

Working assumption

For modelling background channel for monostatic sensing:

Solution A: (previous Option 1)

- drop N reference point(s) following certain distribution, and then the background channel is generated based on the channel generated as in existing TR between the real Tx and the reference point for a scenario.

  - The distance between Tx and the reference point, and the height of the reference point follow Gamma distribution,

drp_2D_n~Γα1, β1+c1

hrp_n~Γα2, β2+c2

  - FFS: value of N (N<4)

- FFS: whether to add additional very low power clusters

- FFS: any update to parameters e.g. angular distribution and delay spread

Agreement

The existing spatial consistency model in TR 38.901 is reused to model correlation of links between one TRP and different STs/UEs.

Agreement

Spatial consistency is not modelled at least for the following links

- Case 1: the links from/to two non-co-located TRPs

  - Link TRP1-X and link TRP2-X, where node X can be a target, a UE or another TRP

  - Link TRP1-X and link TRP2-Y, where node X, Y can be a target, a UE or another TRP

- Case 2: the two links with different LOS/NLOS condition

- Case 3: background channel for monostatic vs. any link (Tx-ST, ST-Rx, background channel) for bistatic

R1-2501001	Summary #3 on ISAC channel modelling	Moderator (Xiaomi)

R1-2501002	Summary #4 on ISAC channel modelling	Moderator (Xiaomi)