3GPP TSG RAN WG1 #120		                                   R1-2501387

Athens, Greece, February 17th – 21st, 2025

Source:		Ad-Hoc Chair (NTT DOCOMO)

Title:			Session Notes for 8.2

Agenda Item:		8.2

Document for: 	Endorsement

- 8.2	Rel-18 UE Features

The maximum number of contributions per company/organization/university is limited to 1 per agenda item unless stated otherwise.

[120-R18-UE_features] Email discussion on Rel-18 UE features – Hiroki (DOCOMO)

- To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc

R1-2501385	Summary of discussion on Rel-18 NR UE features	Moderator (NTT DOCOMO, INC.)

### NR_MIMO_evo_DL_UL

Agreement:

Introduce following FG for UE 8Tx PUSCH processing capability for codebook.

- FG name

  - UE 8Tx PUSCH processing capability for codebook

- Component

  - 1. Require additional timeline to process two TBs for 8Tx PUSCH

- Prerequisite

  - 40-7-1 or 40-7-2

- Consequence if not supported

  - UE should process two TBs within legacy timeline

- Type

  - Per FS

- Need of FR1/FR2 differentiation

  - n/a

- Note

  - candidate values:

  - UE reports candidate value independently for each SCS in unit of symbols

  - For 15kHz SCS: {1,2,4}

  - For 30kHz SCS: {1,2,4,8}

  - For 60kHz SCS: {2,4,8,16}

  - For 120kHz SCS: {4,8,16,32}

  - For 480kHz SCS: {16,32,64, 128}

  - For 960kHz SCS: {32,64,128, 256}

  - Note: SCS above is the minimum between SCS of the scheduling DCI and SCS of the scheduled PUSCH

  - Note that the proposed UE capability is reported by UE only when UE reports the Maximum number of PUSCH MIMO layers to be {5,6,7,8}

- Mandatory or optional

  - Optional with capability signaling

Agreement:

Introduce following FG for UE 8Tx PUSCH processing capability for non-codebook.

- FG name

  - UE 8Tx PUSCH processing capability for non-codebook

- Component

  - 1. Require additional timeline to process two TBs for 8Tx PUSCH

- Prerequisite

  - 40-7-1 or 40-7-2

- Consequence if not supported

  - UE should process two TBs within legacy timeline

- Type

  - Per FS

- Need of FR1/FR2 differentiation

  - n/a

- Note

  - candidate values:

  - UE reports candidate value independently for each SCS in unit of symbols

  - For 15kHz SCS: {1,2,4}

  - For 30kHz SCS: {1,2,4,8}

  - For 60kHz SCS: {2,4,8,16}

  - For 120kHz SCS: {4,8,16,32}

  - For 480kHz SCS: {16,32,64, 128}

  - For 960kHz SCS: {32,64,128, 256}

  - Note: SCS above is the minimum between SCS of the scheduling DCI and SCS of the scheduled PUSCH

  - Note that the proposed UE capability is reported by UE only when UE reports the Maximum number of PUSCH MIMO layers to be {5,6,7,8}

- Mandatory or optional

  - Optional with capability signaling

### NR_Mob_enh2

Agreement:

Reply to RAN2 that following per-band capabilities refer to the source band.

- ue-TA-Measurement-r18

- ltm-BeamIndicationJointTCI-r18

- ltm-BeamIndicationSeparateTCI-r18

- rach-EarlyTA-Measurement-r18

- ltm-MAC-CE-JointTCI-r18

- ltm-MAC-CE-SeparateTCI-r18

- ta-IndicationCellSwitch-r18

R1-2500833	UE features for Rel-18 work items	Samsung

R1-2501142	UE features for Rel-18	Qualcomm Incorporated

Withdrawn

R1-2501325	Rel-18 UE Features	Huawei, HiSilicon