3GPP TSG RAN WG1 #120                                        R1-25xxxxx

Athens, Greece, February 17–21, 2025

Source:	Moderator (ZTE)

Title:	FLS#1 on power control of PSFCH in TS 38.213

Agenda item:	8.1

Document for:	Discussion and Decision

# Introduction

In this RAN1 meeting, a CR R1-2500580 [1] is submitted on power control of PSFCH in TS 38.213.

# Discussion

As described in clause 16.3.0 in TS 38.213, for the operation of PSFCH, there are four cases as follows:

- Case 1: SL operation without shared spectrum channel access.

- Case 2: SL operation with shared spectrum channel access, when sl-TransmissionStructureForPSFCH is not provided.

- Case 3: SL operation with shared spectrum channel access, when sl-TransmissionStructureForPSFCH = ' dedicatedInterlace '.

- Case 4: SL operation with shared spectrum channel access, when sl-TransmissionStructureForPSFCH = ' commonInterlace'.

For operation without shared spectrum channel access, a UE is provided by sl-PSFCH-RB-Set a set of MPRB, setPSFCH PRBs in a resource pool for PSFCH transmission with HARQ-ACK information in a PRB of the resource pool. A UE can be provided by sl-RB-SetPSFCH a set of MPRB, setPSFCH PRBs in a resource pool for PSFCH transmission with conflict information in a PRB of the resource pool. A UE expects that different PRBs are (pre)configured for conflict information and HARQ-ACK information. For a number of Nsubch sub-channels for the resource pool, provided by sl-NumSubchannel, and a number of PSSCH slots associated with a PSFCH slot that is less than or equal to NPSSCHPSFCH, the UE allocates the i+j⋅NPSSCHPSFCH⋅Msubch, slotPSFCH, i+1+j⋅NPSSCHPSFCH⋅Msubch, slotPSFCH-1 PRBs from the MPRB, setPSFCH PRBs to slot i among the PSSCH slots associated with the PSFCH slot and sub-channel j, where Msubch, slotPSFCH=MPRB, setPSFCHNsubch⋅NPSSCHPSFCH, 0≤i<NPSSCHPSFCH, 0≤j<Nsubch, and the allocation starts in an ascending order of i and continues in an ascending order of j. The UE expects that MPRB, setPSFCH is a multiple of Nsubch∙NPSSCHPSFCH. For operation with shared spectrum channel access, when sl-TransmissionStructureForPSFCH is not provided and within RB-set k, for the n-th candidate PSFCH transmission occasion, 1≤n≤NoccasionPSFCH, a UE determines a set of MPRB, set, kPSFCH,n PRBs in a resource pool based on the n-th indication provided by sl-PSFCH-RB-SetList or sl-IUC-RB-SetList for PSFCH transmission with HARQ-ACK information or conflict information, respectively. The UE expects that different PRBs are (pre)configured for conflict information and HARQ-ACK information. For a number of Nsubchk sub-channels where all PRBs of each sub-channel are located in RB-set k and a number of PSSCH slots associated with a PSFCH slot that is less than or equal to NPSSCHPSFCH, the UE allocates the i+j⋅NPSSCHPSFCH⋅Msubch, slot,kPSFCH,n, i+1+j⋅NPSSCHPSFCH⋅Msubch, slot,kPSFCH,n-1 PRBs from the MPRB, set, kPSFCH,n PRBs to slot i among the PSSCH slots associated with the PSFCH slot and sub-channel j, where Msubch, slot,kPSFCH,n=MPRB, set,kPSFCH,nNsubchk⋅NPSSCHPSFCH, 0≤i<NPSSCHPSFCH, 0≤j<Nsubchk, and the allocation starts in an ascending order of i and continues in an ascending order of j. The UE expects that MPRB, set, kPSFCH,n is a multiple of Nsubchk∙NPSSCHPSFCH. For operation with shared spectrum channel access, when sl-TransmissionStructureForPSFCH = ' dedicatedInterlace ' and within RB-set k, a UE determines, based on sl-PSFCH-RB-SetList, all PRBs of an interlace for one PSFCH transmission with HARQ-ACK information in the resource pool. Within RB-set k, the UE determines, based on sl-IUC-RB-SetList, all PRBs of an interlace for one PSFCH transmission with conflict information in the resource pool. For the n-th candidate PSFCH transmission occasion, 1≤n≤NoccasionPSFCH, the UE determines a set of interlaces that includes a number Minterlace,kPSFCH,n of interlaces based on the n-th indication provided by sl-PSFCH-RB-SetList or sl-IUC-RB-SetList for HARQ-ACK information or conflict information, respectively. The UE expects that different interlaces are determined for conflict information and HARQ-ACK information. The set of interlaces are indexed in an ascending order of interlace indexes. For each interlace of the set of interlaces, all PRBs in the interlace are available for PSFCH transmission. For a number of Nsubchk sub-channels in RB-set k and a number of PSSCH slots that is not larger than NPSSCHPSFCH and is associated with a slot for PSFCH transmission, the UE allocates the i+j⋅NPSSCHPSFCH⋅Msubch, slot,kPSFCH,n, i+1+j⋅NPSSCHPSFCH⋅Msubch, slot,kPSFCH,n-1 interlaces from the Minterlace,kPSFCH,n interlaces to slot i and sub-channel j, where Msubch, slot,kPSFCH,n=Minterlace,kPSFCH,nNsubchk⋅NPSSCHPSFCH, 0≤i<NPSSCHPSFCH, 0≤j<Nsubchk. The allocation starts in an ascending order of i and continues in an ascending order of j. The UE expects that Minterlace,kPSFCH,n is a multiple of Nsubchk∙NPSSCHPSFCH. For operation with shared spectrum channel access, when sl-TransmissionStructureForPSFCH = ' commonInterlace' and within RB-set k, a UE determines a subset of PRBs in a first interlace and, based on sl-PSFCH-RB-SetList, a subset of NPRBPSFCH PRBs in a second interlace for a PSFCH transmission with HARQ-ACK information in a resource pool, or based on sl-IUC-RB-SetList, a subset of NPRBPSFCH PRBs in a second interlace for a PSFCH transmission with conflict information in a resource pool. An index of the first interlace is provided by sl-PSFCH-CommonInterlaceIndex. The NPRBPSFCH PRBs in the second interlace are provided by sl-NumDedicatedPRBs-ForPSFCH where, for the n-th candidate PSFCH transmission occasion, 1≤n≤NoccasionPSFCH, and for each interlace l, the UE determines MPRB,k, lPSFCH,n PRBs based on the n-th indication provided by sl-PSFCH-RB-SetList or sl-IUC-RB-SetList for HARQ-ACK information or conflict information, respectively. The UE expects that different subsets of NPRBPSFCH PRBs are determined for conflict information and HARQ-ACK information. The UE expects that MPRB,k,lPSFCH,n is a multiple of NPRBPSFCH. For interlace l, the UE determines a PRB subset with index s to include PRBs NPRBPSFCH⋅s, NPRBPSFCH⋅s+1, …, NPRBPSFCH⋅s+1-1, 0≤s≤MPRB,k,lPSFCH,n/NPRBPSFCH-1. The UE determines the Msubset,kPSFCH,n PRB subsets by ordering the PRB subsets first in an ascending order of PRB subset index within an interlace and second in ascending order of interlace index, where Msubset,kPSFCH,n=lMPRB,k,lPSFCH,n/NPRBPSFCH. For a number of Nsubchk sub-channels in RB-set k and a number of slots for PSSCH transmissions that is not larger than NPSSCHPSFCH and is associated with a slot for PSFCH transmission, the UE allocates the i+j⋅NPSSCHPSFCH⋅Msubch, slot,kPSFCH,n, i+j⋅NPSSCHPSFCH⋅Msubch, slot,kPSFCH,n+1, …, i+1+j⋅NPSSCHPSFCH⋅Msubch, slot,kPSFCH,n-1 PRB subsets from the Msubset,kPSFCH,n PRB subsets to slot i among the slots for PSSCH transmissions that are associated with the slot and sub-channel j for PSFCH transmissions, where Msubch, slot,kPSFCH,n=Msubset,kPSFCH,nNsubchk⋅NPSSCHPSFCH and 0≤i<NPSSCHPSFCH, 0≤j<Nsubchk. The allocation starts in an ascending order of i and continues in an ascending order of j. The UE expects that Msubset,kPSFCH,n is a multiple of Nsubchk∙NPSSCHPSFCH.

While in clause 16.2.3, for the cases which are applicable, only three cases are listed, case 2 above was missing. The related description of case 2 above can be added in clause 16.2.3 in TS 38.213.

## 2.1 Round-1

Proposal: Endorse CR R1-2500580 for PSFCH power control in Clause 16.2.3 in TS 38.213:

| Reason for change: | As described in clause 16.3.0, for the operation of PSFCH, there are four cases as follows: Case 1: SL operation without shared spectrum channel access. Case 2: SL operation with shared spectrum channel access, when sl-TransmissionStructureForPSFCH is not provided. Case 3: SL operation with shared spectrum channel access, when sl-TransmissionStructureForPSFCH = ' dedicatedInterlace '. Case 4: SL operation with shared spectrum channel access, when sl-TransmissionStructureForPSFCH = ' commonInterlace'. While in clause 16.2.3, for the cases which PPSFCH,one are applicable, only three cases are listed, case 2 above was missing. Therefore, the related description of case 2 above should be added in clause 16.2.3 in TS 38.213. |
| --- | --- |
| Summary of change: | Adding ‘or for operation with shared spectrum channel access, when sl-TransmissionStructureForPSFCH is not provided’. |
| Consequences if not approved: | Power control of PSFCH is not captured completely for the operation with shared spectrum channel access, when sl-TransmissionStructureForPSFCH is not provided. |
| Clauses affected: | 16.2.3 |

<Unchanged parts are omitted> # 16.2.3 PSFCH A UE with Nsch,Tx,PSFCH scheduled PSFCH transmissions for HARQ-ACK information and conflict information, and capable of transmitting a maximum of Nmax,PSFCH PSFCHs, determines a number NTx,PSFCH of simultaneous PSFCH transmissions and a power PPSFCH,k(i) for a PSFCH transmission k, 1≤k≤NTx,PSFCH, on all the resource pools in PSFCH transmission occasion i on active SL BWP b of carrier f as - if dl-P0-PSFCH is provided, PPSFCH,one=PO,PSFCH+10log102μ+αPSFCH⋅PL [dBm] where - PPSFCH,one is applicable for - the PRB of the PSFCH transmission for operation without shared spectrum channel access or for operation with shared spectrum channel access, when sl-TransmissionStructureForPSFCH is not provided, - each PRB in the interlace of the PSFCH transmission for operation with shared spectrum channel access and sl-TransmissionStructureForPSFCH = ''dedicatedInterlace'', - each PRB in the subset of PRBs in the second interlace of the PSFCH transmission for operation with shared spectrum channel access and sl-TransmissionStructureForPSFCH = 'commonInterlace' - PO,PSFCH is a value of dl-P0-PSFCH-r17, if using the parameter is supported by the UE and the parameter is provided; else dl-P0-PSFCH-r16 if provided - αPSFCH is a value of dl-Alpha-PSFCH, if provided; else, αPFSCH=1 - PL=PLb,f,c(qd) when the active SL BWP is on a serving cell c, as described in clause 7.1.1 except that - the RS resource is the one the UE uses for determining a power of a PUSCH transmission scheduled by a DCI format 0_0 in serving cell c when the UE is configured to monitor PDCCH for detection of DCI format 0_0 in serving cell c - the RS resource is the one corresponding to the SS/PBCH block the UE uses to obtain MIB when the UE is not configured to monitor PDCCH for detection of DCI format 0_0 in serving cell c - for operation with shared spectrum channel access and sl-TransmissionStructureForPSFCH = ‘commonInterlace’, PPSFCH,k(i) includes the power on PRBs in both the first and second interlaces and, for more than one PSFCH transmissions from the UE, the power on any PRB in the first interlace is not accumulated among the more than one PSFCH transmissions within a same RB set and is same as the power PPSFCH,PRB,first,k(i) on the PRB in the first interlace for PSFCH transmission k. - if Nsch,Tx,PSFCH≤Nmax,PSFCH - if PPSFCH,one+10log10NPSFCH,one≤PCMAX, where PCMAX is determined for Nsch,Tx,PSFCH PSFCH transmissions according to [8-1, TS 38.101-1] and - NPSFCH,one=Nsch,Tx,PSFCH for operation without shared spectrum channel access or for operation with shared spectrum channel access, when sl-TransmissionStructureForPSFCH is not provided - NTx,PSFCH=Nsch,Tx,PSFCH and PPSFCH,k(i)=PPSFCH,one [dBm] - NPSFCH,one=k=1Nsch,Tx,PSFCHNPSFCH,one,kinterlace for operation with shared spectrum channel access and sl-TransmissionStructureForPSFCH = 'dedicatedInterlace', where NPSFCH,one,kinterlace is the number of PRBs in the interlace for PSFCH transmission k - NTx,PSFCH=Nsch,Tx,PSFCH and PPSFCH,ki=PPSFCH,one+10log10NPSFCH,one,kinterlace [dBm], where the power on one PRB in the interlace for PSFCH transmission is PPSFCH,PRB,k(i)=PPSFCH,one - NPSFCH,one=Nsch,Tx,PSFCH⋅NPSFCH,oneinterlace2+NPSFCH,oneinterlace1⋅10(-PPSFCH,offset/10) for operation with shared spectrum channel access and sl-TransmissionStructureForPSFCH = 'commonInterlace', where NPSFCH,oneinterlace2 is provided by sl-NumDedicatedPRBs-ForPSFCH, PPSFCH,offset is provided by sl-PSFCH-PowerOffset, and NPSFCH,oneinterlace1 is the number of PRBs in the first interlace for all Nsch,Tx,PSFCH PSFCH transmissions after excluding PRBs for PSFCH transmissions as described in Clause 16.3.0 - NTx,PSFCH=Nsch,Tx,PSFCH and PPSFCH,ki=PPSFCH,one+10log10NPSFCH,oneinterlace2+NPSFCH,one,kinterlace1⋅10(-PPSFCH,offset/10) [dBm], where NPSFCH,one,kinterlace1 is the number of PRBs in the first interlace within the same RB set of PSFCH transmission k after excluding PRBs for PSFCH transmissions as described in Clause 16.3.0, and the power on one PRB in the first interlace for PSFCH transmission is PPSFCH,PRB,first,k(i)=PPSFCH,PRB,second,ki-PPSFCH,offset and the power on one PRB in the subset of PRBs in the second interlace for PSFCH transmission is PPSFCH,PRB,second,k(i)=PPSFCH,one, where PPSFCH,offset is provided by sl-PSFCH-PowerOffset - else - UE autonomously determines NTx,PSFCH PSFCH transmissions first with ascending order of corresponding priority field values as described in clause 16.2.4.2 over the PSFCH transmissions with HARQ-ACK information, if any, and then with ascending order of priority value over the PSFCH transmissions with conflict information, if any, such that NTx,PSFCH≥max1,i=1KMi where Mi, for 1≤i≤ 8, is a number of PSFCHs with priority value i for PSFCH with HARQ-ACK information and Mi, for i>8, is a number of PSFCHs with priority value i-8 for PSFCH with conflict information and K is defined as - the largest value satisfying PPSFCH,one+10log10NPSFCH,oneK≤PCMAX where PCMAX is determined according to [8-1, TS 38.101-1] for transmission of all PSFCHs in i=1KMi, if any - NPSFCH,oneK=max1,i=1KMi for operation without shared spectrum channel access or for operation with shared spectrum channel access, when sl-TransmissionStructureForPSFCH is not provided - NPSFCH,oneK=k=1max1,i=1KMiNPSFCH,one,kinterlace for operation with shared spectrum channel access and sl-TransmissionStructureForPSFCH = 'dedicatedInterlace' - NPSFCH,oneK=max1,i=1KMi⋅NPSFCH,oneinterlace2+NPSFCH,oneinterlace1,K⋅10(-PPSFCH,offset/10) for operation with shared spectrum channel access and sl-TransmissionStructureForPSFCH = 'commonInterlace', where NPSFCH,oneinterlace1,K is the number of PRBs in the first interlace for the max1,i=1KMi PSFCH transmissions after excluding PRBs for PSFCH transmissions as described in Clause 16.3.0 - zero, otherwise and - PPSFCH,k(i)=minPCMAX-10log10(NTx,PSFCH),PPSFCH,one [dBm] for operation without shared spectrum channel access or for operation with shared spectrum channel access, when sl-TransmissionStructureForPSFCH is not provided - PPSFCH,ki=minPCMAX-10log10k=1NTx,PSFCHNPSFCH,one,kinterlace,PPSFCH,one+10log10NPSFCH,one,kinterlace [dBm] for operation with shared spectrum channel access and sl-TransmissionStructureForPSFCH = 'dedicatedInterlace', where the power on one PRB in the interlace for PSFCH transmission is PPSFCH,PRB,k(i)=minPCMAX-10log10k=1NTx,PSFCHNPSFCH,one,kinterlace,PPSFCH,one - PPSFCH,ki=minPCMAX-10log10NTx,PSFCH⋅NPSFCH,oneinterlace2+NPSFCH,oneinterlace1,K⋅10-PPSFCH,offset10 ,PPSFCH,one+10log10NPSFCH,oneinterlace2+NPSFCH,one,kinterlace1⋅10-PPSFCH,offset/10 [dBm] for operation with shared spectrum channel access and sl-TransmissionStructureForPSFCH = 'commonInterlace', where NPSFCH,oneinterlace1,K is the number of PRBs in the first interlace for the NTx,PSFCH PSFCH transmissions after excluding PRBs for PSFCH transmissions as described in Clause 16.3.0, NPSFCH,one,kinterlace1 is the number of PRBs in the first interlace for PSFCH transmission(s) among the NTx,PSFCH PSFCH transmissions which are within the same RB set of PSFCH transmission k after excluding PRBs for PSFCH transmissions as described in Clause 16.3.0, and the power on one PRB in the first interlace for PSFCH transmission is PPSFCH,PRB,first,k(i)=PPSFCH,PRB,second,ki-PPSFCH,offset and the power on one PRB in the subset of PRBs in the second interlace for PSFCH transmission is PPSFCH,PRB,second,k(i)=minPCMAX-10log10NTx,PSFCH⋅NPSFCH,oneinterlace2+NPSFCH,oneinterlace1,K⋅10-PPSFCH,offset/10 ,PPSFCH,one, where PPSFCH,offset is provided by sl-PSFCH-PowerOffset where PCMAX is defined in [8-1, TS 38.101-1] and is determined for the NTx,PSFCH PSFCH transmissions - else - the UE autonomously selects Nmax,PSFCH PSFCH transmissions with ascending order of corresponding priority field values as described in clause 16.2.4.2 - if PPSFCH,one+10log10NPSFCH,one,max≤PCMAX, where PCMAX is determined for the Nmax,PSFCH PSFCH transmissions according to [8-1, TS 38.101-1] - NPSFCH,one,max=Nmax,PSFCH [dBm] for operation without shared spectrum channel access or for operation with shared spectrum channel access, when sl-TransmissionStructureForPSFCH is not provided - NTx,PSFCH=Nmax,PSFCH and PPSFCH,k(i)=PPSFCH,one [dBm] - NPSFCH,one,max=k=1Nmax,PSFCHNPSFCH,one,kinterlace for operation with shared spectrum channel access and sl-TransmissionStructureForPSFCH = 'dedicatedInterlace', where NPSFCH,one,kinterlace is the number of PRBs in the interlace for the PSFCH transmission k - NTx,PSFCH=Nmax,PSFCH and PPSFCH,ki=PPSFCH,one+10log10NPSFCH,one,kinterlace [dBm], where the power on one PRB in the interlace for PSFCH transmission is PPSFCH,PRB,k(i)=PPSFCH,one - NPSFCH,one,max=Nmax,PSFCH⋅NPSFCH,oneinterlace2+NPSFCH,oneinterlace1⋅10(-PPSFCH,offset/10) for operation with shared spectrum channel access and sl-TransmissionStructureForPSFCH = 'commonInterlace', where NPSFCH,oneinterlace2 is provided by sl-NumDedicatedPRBs-ForPSFCH, PPSFCH,offset is provided by sl-PSFCH-PowerOffset, and NPSFCH,oneinterlace1 is the number of PRBs in the first interlace for all Nmax,PSFCH PSFCH transmissions after excluding PRBs for PSFCH transmissions as described in Clause 16.3.0 - NTx,PSFCH=Nmax,PSFCH and PPSFCH,ki=PPSFCH,one+10log10NPSFCH,oneinterlace2+NPSFCH,one,kinterlace1⋅10(-PPSFCH,offset/10) [dBm], where NPSFCH,one,kinterlace1 is the number of PRBs in the first interlace for PSFCH transmission(s) among all Nmax,PSFCH PSFCH transmissions within the same RB set of PSFCH transmission k after excluding PRBs for PSFCH transmissions as described in Clause 16.3.0, and the power on one PRB in the first interlace for PSFCH transmission is PPSFCH,PRB,first,k(i)=PPSFCH,PRB,second,ki-PPSFCH,offset and the power on one PRB in the subset of PRBs in the second interlace for PSFCH transmission is PPSFCH,PRB,second,k(i)=PPSFCH,one, where PPSFCH,offset is provided by sl-PSFCH-PowerOffset - else - the UE autonomously selects NTx,PSFCH PSFCH transmissions in ascending order of corresponding priority field values as described in clause 16.2.4.2 over the PSFCH transmissions with HARQ-ACK information, if any, and then with ascending order of priority value over the PSFCH transmissions with conflict information, if any, such that NTx,PSFCH≥max1,i=1KMi where Mi, 1≤i≤8, is a number of PSFCHs with priority value i for PSFCH with HARQ-ACK information and Mi, i>8, is a number of PSFCHs with priority value i-8 for PSFCH with conflict information and K is defined as - the largest value satisfying PPSFCH,one+10log10NPSFCH,oneK≤PCMAX where PCMAX is determined according to [8-1, TS 38.101-1] for transmission of all PSFCHs in i=1KMi, if any - NPSFCH,oneK=max1,i=1KMi for operation without shared spectrum channel access or for operation with shared spectrum channel access, when sl-TransmissionStructureForPSFCH is not provided - NPSFCH,oneK=k=1max1,i=1KMiNPSFCH,one,kinterlace for operation with shared spectrum channel access and sl-TransmissionStructureForPSFCH = 'dedicatedInterlace' - NPSFCH,oneK=max1,i=1KMi⋅NPSFCH,oneinterlace2+NPSFCH,oneinterlace1,K⋅10(-PPSFCH,offset/10) for operation with shared spectrum channel access and sl-TransmissionStructureForPSFCH = 'commonInterlace', where NPSFCH,oneinterlace1,K is the number of PRBs in the first interlace for the max1,i=1KMi PSFCH transmissions after excluding PRBs for PSFCH transmissions as described in Clause 16.3.0 - zero, otherwise and - PPSFCH,k(i)=minPCMAX-10log10(NTx,PSFCH),PPSFCH,one [dBm] for operation without shared spectrum channel access or for operation with shared spectrum channel access, when sl-TransmissionStructureForPSFCH is not provided - PPSFCH,k(i)=minPCMAX-10log10(k=1NTx,PSFCHNPSFCH,one,kinterlace),PPSFCH,one+10log10NPSFCH,one,kinterlace [dBm] for operation with shared spectrum channel access and sl-TransmissionStructureForPSFCH= 'dedicatedInterlace', where the power on one PRB in the interlace for PSFCH transmission is PPSFCH,PRB,k(i)=minPCMAX-10log10k=1NTx,PSFCHNPSFCH,one,kinterlace,PPSFCH,one - PPSFCH,k(i)=minPCMAX-10log10(NTx,PSFCH⋅NPSFCH,oneinterlace2+NPSFCH,oneinterlace1,K⋅10(-PPSFCH,offset/10)),PPSFCH,one+ 10log10NPSFCH,oneinterlace2+NPSFCH,one,kinterlace1⋅10(-PPSFCH,offset/10) [dBm] for operation with shared spectrum channel access and sl-TransmissionStructureForPSFCH = 'commonInterlace', where NPSFCH,oneinterlace1,K is the number of PRBs in the first interlace for the NTx,PSFCH PSFCH transmissions after excluding PRBs for PSFCH transmissions as described in Clause 16.3.0, NPSFCH,one,kinterlace1 is the number of PRBs in the first interlace for PSFCH transmission(s) among the NTx,PSFCH PSFCH transmissions which are within the same RB set of PSFCH transmission k after excluding PRBs for PSFCH transmissions as described in Clause 16.3.0, and the power on one PRB in the first interlace for PSFCH transmission is PPSFCH,PRB,first,k(i)=PPSFCH,PRB,second,ki-PPSFCH,offset and the power on one PRB in the subset of PRBs in the second interlace for PSFCH transmission is PPSFCH,PRB,second,k(i)=minPCMAX-10log10NTx,PSFCH⋅NPSFCH,oneinterlace2+NPSFCH,oneinterlace1,K⋅10-PPSFCH,offset/10 ,PPSFCH,one, where PPSFCH,offset is provided by sl-PSFCH-PowerOffset where PCMAX is determined for the NTx,PSFCH simultaneous PSFCH transmissions according to [8-1, TS 38.101-1] - else - PPSFCH,k(i)=PCMAX-10log10(NTx,PSFCH) [dBm] for operation without shared spectrum channel access or for operation with shared spectrum channel access, when sl-TransmissionStructureForPSFCH is not provided - PPSFCH,k(i)=PCMAX-10log10(k=1NTx,PSFCHNPSFCH,one,kinterlace)+10log10NPSFCH,one,kinterlace [dBm] for operation with shared spectrum channel access and sl-TransmissionStructureForPSFCH = 'dedicatedInterlace', where the power on one PRB in the interlace for PSFCH transmission is PPSFCH,PRB,k(i)=PCMAX-10log10k=1NTx,PSFCHNPSFCH,one,kinterlace - PPSFCH,k(i)=PCMAX-10log10(NTx,PSFCH⋅NPSFCH,oneinterlace2+NPSFCH,oneinterlace1⋅10(-PPSFCH,offset/10))+ 10log10NPSFCH,oneinterlace2+NPSFCH,one,kinterlace1⋅10(-PPSFCH,offset/10) [dBm] for operation with shared spectrum channel access and sl-TransmissionStructureForPSFCH = 'commonInterlace', where NPSFCH,oneinterlace1 is the number of PRBs in the first interlace for the NTx,PSFCH PSFCH transmissions after excluding PRBs for PSFCH transmissions as described in Clause 16.3.0, NPSFCH,one,kinterlace1 is the number of PRBs in the first interlace for PSFCH transmission(s) among the NTx,PSFCH PSFCH transmissions which are within the same RB set of PSFCH transmission k after excluding PRBs for PSFCH transmissions as described in Clause 16.3.0, the power on one PRB in the first interlace for PSFCH transmission is PPSFCH,PRB,first,k(i)=PPSFCH,PRB,second,ki-PPSFCH,offset and the power on one PRB in the subset of PRBs in the second interlace for PSFCH transmission is PPSFCH,PRB,second,k(i)=PCMAX-10log10NTx,PSFCH⋅NPSFCH,oneinterlace2+NPSFCH,oneinterlace1⋅10-PPSFCH,offset/10, where PPSFCH,offset is provided by sl-PSFCH-PowerOffset - for operation with shared spectrum channel access and sl-TransmissionStructureForPSFCH = ‘commonInterlace’, PPSFCH,k(i) includes the power on PRBs in both the first and second interlaces and, for more than one PSFCH transmissions from the UE, the power on any PRB in the first interlace is not accumulated among the more than one PSFCH transmissions within a same RB set and is same as the power PPSFCH,PRB,first,k(i) on the PRB in the first interlace for PSFCH transmission k. where the UE autonomously determines NTx,PSFCH PSFCH transmissions with ascending order of corresponding priority field values as described in clause 16.2.4.2 over the PSFCH transmissions with HARQ-ACK information, if any, and then with ascending order of priority value over the PSFCH transmissions with conflict information, if any, such that NTx,PSFCH≥1 and where PCMAX is determined for the NTx,PSFCH PSFCH transmissions according to [8-1, TS 38.101-1]. For resource pools configured with PSFCH resources overlapping in time, the UE either expects not to be provided with dl-P0-PSFCH or dl-Alpha-PSFCH in any of the resource pools, or expects to be provided with the same values of dl-P0-PSFCH and the same values of dl-Alpha-PSFCH for all the resource pools. <Unchanged parts are omitted>

### View collection

Please provide your views on the above CR:

| Company | Agree? (Yes or no) | Comments |
| --- | --- | --- |
| Samsung | Agree | Ok with the CR in general. One suggestion on the wording, the following update could make the meaning more clear: for operation without shared spectrum channel access, or for operation with shared spectrum channel access and when sl-TransmissionStructureForPSFCH is not provided. |

# Conclusion

xxx

# Reference

- R1-2500580	Draft CR on power control of PSFCH in TS 38.213	ZTE Corporation, Sanechips