3GPP TSG- RAN WG1 #120	R1-2xxxxxx

# Athens, Greece, February 17th – 21st, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Draft CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | - | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of UE-initiated/event-driven beam management |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_Ph5 |  |  |  |  |  | Date: |  |  | 2025-02-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduction of UE-initiated/event-driven beam management |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of UE-initiated/event-driven beam management |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE-initiated/event-driven beam management would not be supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.212, 38.213 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<omitted text>

5.2.1.5.4 Event-triggered reporting (NEW)

For a UE configured with a CSI-ReportConfig with the higher layer parameter eventType-r19 set to ‘event2’, for periodic reference signals with the same periodicity,

if during the eventDetectionTimeWindowLength-r19, the L1-RSRP [measured and counted independently] for eventInstanceCount-r19 times for any of the reference signals configured by newBeamResourceSetEvent2-r19 is an eventThreshold-r19 greater than the L1-RSRP of

-	the reference signal indicated by the DCI field tci-PresentInDCI if the NZP-CSI-RS-ResourceSet is configured with repetition set to 'on',  else

-	the  SSB which is QCLed with the measured reference signal,

the UE transmits ‘0’ on a PUCCH format 0 or format 1 indicated by firstPUCCHResourceConfig-UEIBR-r19, and

the UE reports nrofReportedRS-UEIBR-r19 CRIs and, when enabledCurrentBeamReport-r19 is configured as ‘yes’, the reference signal indicated by the DCI field tci-PresentInDCI, or the SSB which is QCLed with this reference signal:

-	on PUSCH if reportTransmissionMode-r19 is configured as ‘ModeA’ and as indicated by the CSI trigger state in the CSI request field in DCI format 0_1/0_2, or

-	on CG-PUSCH if reportTransmissionMode-r19 is configured as ‘ModeB’ and as indicated by configuredResourceForSecondChannelOfModeB-r19 and X symbols after sending the PUCCH , and where the periodicity of the PUCCH and CG-PUSCH channels is the same.

For a UE configured with a CSI-ReportConfig with the higher layer parameter eventType-r19 set to ‘event7’, for periodic reference signals with the same periodicity,

if during the eventDetectionTimeWindowLength-r19, the L1-RSRP [measured and counted independently] for eventInstanceCount-r19 times for any of the periodic activated TCI state reference signals configured by tci-StatesToAddModList is an eventThreshold-r19 greater than the L1-RSRP of the reference signal with the valueOfQ-r19 highest L1-RSRP out of the activated TCI state reference signals,

the UE transmits ‘0’ on a PUCCH format 0 or format 1 indicated by firstPUCCHResourceConfig-UEIBR-r19, and

the UE reports nrofReportedRS-UEIBR-r19 CRIs:

-	on PUSCH if reportTransmissionMode-r19 is configured as ‘ModeA’ and as indicated by the CSI trigger state in the CSI request field in DCI format 0_1/0_2, or

-	on CG-PUSCH if reportTransmissionMode-r19 is configured as ‘ModeB’ and as indicated by configuredResourceForSecondChannelOfModeB-r19 and X symbols after sending the PUCCH, and where the periodicity of the PUCCH and CG-PUSCH channels is the same.

For a UE configured with a CSI-ReportConfig with the higher layer parameter eventType-r19 set to ‘event1’, …

<omitted text>