Agreement List 9.2.4 (Rel-19 Asymmetric DL sTRP/UL mTRP)

# Crossed out in red = superseded by later agreements

# Issue 1: PL offset

[116] Agreement

For the asymmetric DL sTRP/UL mTRP deployment scenarios, support to associate a UL TCI state with a PL offset:

- When a UL TCI state associated with a PL offset is applied for the PUSCH/PUCCH/SRS transmission, the UE shall calculate the Tx power of the PUSCH/PUCCH/SRS based on the DL PL RS and PL offset associated with this UL TCI state.

  - Reuse the legacy uplink power control formulation by replacing legacy PL with UL PL which is derived from the DL PL RS and the PL offset.

  - FFS: The UE can update UL PL in a way that new UL PL = current UL PL + an update delta indicated by the NW.

- Note: it does not intend to increase the number of maintained PLs per cell.

- FFS: whether to support associating joint TCI state (if supported) with a PL offset.

Further study whether/how to apply a PL offset on PDCCH-order PRACH transmission too.

- FFS: how to determine the Tx beam of PRACH towards UL TRP

- Note: this does not imply to support 2 TA for single-DCI based system.

[116] Agreement

Down-select one from the following alternatives:

- Alt1: Use only RRC to update the PL offset associated with the UL TCI state

- Alt2: In addition to RRC, MAC-CE can be used to update the PL offset associated with the UL TCI state

  - FFS: Details on MAC CE

[116] Agreement

For the asymmetric DL sTRP/UL mTRP deployment scenarios, separate DL/UL TCI state mode of Rel-17/18 unified TCI framework can be configured for both FR1 and FR2.

- Joint TCI state mode can be configured at least for FR1

[116bis] Agreement

For FR1, a joint TCI state can be associated with a PL offset.

- When a joint TCI state associated with a PL offset is applied for the PUSCH/PUCCH/SRS transmission, the UE shall calculate the Tx power of the PUSCH/PUCCH/SRS based on the DL PL RS and PL offset associated with this joint TCI state.

  - Reuse the legacy uplink power control formulation by replacing legacy PL with a PL which is derived from the DL PL RS and the PL offset.

  - FFS: The UE can update UL PL in a way that new UL PL = current UL PL + an update delta indicated by the NW.

[116bis] Agreement

Support applying PL offset on PDCCH-order PRACH towards a UL TRP in FR1.

- Note: The DL reference timing determination for PDCCH-order PRACH transmission to an UL TRP is still based on the DL RS defined in current RAN4 specification

- Above is subject to a separate UE capability signaling

[116bis] Agreement

Consider and down-select one from the following alts for indicating a PL offset for PDCCH-order PRACH transmission at least for FR1.

- Alt1: RRC configures multiple PL offset values in PRACH-Config and PDCCH-order DCI indicates one of them through one DCI field.

- Alt2: PDCCH order DCI indicates one PL offset value

- Alt3: The PL offset associated with one of the indicated joint/UL TCI state for UL TRP in unified TCI framework is applied on the PDCCH-order PRACH transmission

- Alt4: The PDCCH order DCI indicates one TCI state associated with a PL offset and the associated PL offset is applied on the PRACH transmission.

- Alt5: RRC configures one PL offset value for PRACH and the PDCCH order DCI indicates whether this PL offset value is applied on PRACH transmission or not.

Note: Other alternatives are not precluded

[116bis] Agreement

For the association between PL offset and joint/UL TCI state, consider and down-select one from the following Alts:

- Alt1a: One PL offset value is configured in a joint or UL TCI state by RRC only

- Alt1b: One PL offset value is configured in a joint or UL TCI state by RRC. A MAC CE can update the PL offset value(s) for joint or UL TCI state(s).

- Alt2a: A list of PL offset configurations is configured by RRC in BWP/CC and each PL offset configuration contains one PL offset value. One new RRC parameter is introduced in a joint or UL TCI state to indicate one of the configured PL offset configurations.

- Alt2b: A list of PL offset configurations is configured by RRC in BWP/CC and each PL offset configuration contains one PL offset value. One new RRC parameter is introduced in a joint or UL TCI state to indicate one of the configured PL offset configurations. A MAC CE can update the association between a joint or UL TCI state and PL offset configuration

- Alt3: A list of PL offset configurations is configured by RRC in BWP/CC and each PL offset configuration contains one PL offset value.  A MAC CE can activate/indicate one PL offset configuration for each activated joint or UL TCI state. In each joint or UL TCI state, the initial PL offset value is 0dB.

- Alt4: A list of PL offset values is provided in a joint or UL TCI state by RRC. Each PL offset value is applied to a corresponding measured PL range.

Other alternatives are not precluded.

[117] Agreement

For indicating a PL offset for PDCCH-order PRACH transmission at least for FR1, further study and down-select one from the Alt1 and Alt3 by RAN1#118 meeting:

- Alt1: RRC configures multiple PL offset values [in PRACH-Config] and PDCCH-order DCI indicates one of them through one DCI field

- Alt3: The PL offset associated with one of the indicated joint/UL TCI state for UL TRP in unified TCI framework is applied on the PDCCH-order PRACH transmission

- FFS: the details of DCI field design.

[117] Agreement

For the association between PL offset and joint/UL TCI state, support the following

- Alt1b: One PL offset value is configured in a joint or UL TCI state by RRC, where different PL offset values can be configured to different joint or UL TCI states. A MAC CE can update the PL offset value(s) for joint or UL TCI state(s).

[117] Conclusion

There is no consensus on the following proposal:

Proposal 1.6: Support to update a UL PL for a joint/UL TCI state as follows:

- When this joint/UL TCI state is activated and it is not in the current active TCI state list, a UL PL is calculated as: UL PL = PL estimated from DL PL RS – the value of PL offset.

- When this joint/UL TCI state is activated and it is in the current active TCI state list, the UE updates the UL PL as: new UL PL = current UL PL + the updated delta indicated by the NW.

[117] Agreement

For the asymmetric DL sTRP/UL mTRP scenarios, study and decide the value range and candidate values of PL offset value

[117] Agreement

For the asymmetric DL sTRP/UL mTRP scenarios, study whether/how to consider PL offset in PHR calculation, including Type 1 PHR based on actual PUSCH transmission, Type 1 PHR based on reference PUSCH, Type 3 PHR based on actual SRS and Type 3 PHR based on reference SRS

[117] Conclusion

For the asymmetric DL sTRP/UL mTRP deployment scenario, reuse the rel-17 unified TCI/ICBM and rel-18 unified TCI framework:

- When rel-17 unified TCI/ICBM is configured:

  - For FR1: one joint TCI state or {one DL TCI state + one UL TCI state} can be applied.

  - For FR2: {one DL TCI state + one UL TCI state} can be applied.

- When rel-18 unified TCI is configured:

  - For FR1: up to two joint TCI states or {one DL TCI state + up to two UL TCI state} can be applied.

  - For FR2: {one DL TCI state + up to two UL TCI states} can be applied.

[118]Agreement

For indicating a PL offset for PDCCH-order PRACH transmission at least for FR1, support Alt3:

- Alt3: The PL offset associated with one of the indicated joint/UL TCI state for UL TRP in unified TCI framework is applied on the PDCCH-order PRACH transmission

  - FFS the detailed design of DCI format: e.g., how to indicate one of the indicated joint/UL TCI states or whether to apply the PL offset in the indicated TCI state or not.

[118]Agreement

For the asymmetric DL sTRP/UL mTRP scenarios, the value range of PL offset includes at least [-10, 60] dB

- FFS: Extending the range to lower than -10dB

- Step size is 4dB

[118]Agreement

For the asymmetric DL sTRP/UL mTRP scenarios, support to include PL offset in the calculation of Type 1 PHR based on actual PUSCH transmission and Type 1 PHR based on reference PUSCH

[118] Agreement

Study whether to support Type 3 PHR reporting in a serving cell/BWP where the UE is configured with two separate SRS CLPC adjustment states.

- Continue to study whether to support including PL offset in the calculation of Type 3 PHR.

[118] Agreement

- Study whether/how to apply PL offset for SRS resource set when the SRS resource set is not configured with TCI state

- Study whether/how to apply one of the two separate SRS CLPC adjustment states on the SRS resource set when the SRS resource set is not configured with TCI state

  - E.g., defining i0 as the default CLPC for SRS resource set in this case. E.g,, configure one of the separate SRS CLPC adjustment states to the SRS resource set.

[118bis] Agreement

The lower limit of the value range of PL offset is extended to -12 dB

[118bis] Agreement

For indicating PL offset for PDCCH-order PRACH, introduce a new 1-bit DCI field in DCI format 1_0:

- This DCI field exists when the corresponding RRC parameter (which is a new RRC used to configure the presence of this 1-bit DCI field) is enabled and at least one TCI state is configured with PL offset.

- When one joint/UL TCI state is indicated in Rel-17 unified TCI,

  - the bit field index 0 of this field indicates that PL offset is not included in the PRACH transmission power calculation

  - the bit field index 1 of this field indicates that the PL offset associated with the indicated TCI state is included in the PRACH transmission power.

- FFS: Whether the bit field can be used to indicate other information

- FFS: When two joint/UL TCI states are indicated in Rel-18 unified TCI

[118bis] Agreement

For indicating PL offset for PDCCH-order PRACH, when two joint/UL TCI states are indicated in Rel-18 unified TCI, down-select one from the following Alts for the 1-bit DCI field in DCI 1_0 which indicates the application of PL offset on PRACH transmission:

- Alt2:

  - the bit field index 0 of this field indicates that the PL offset associated in the first indicated joint/UL TCI state is included in the PRACH transmission power calculation

  - the bit field index 1 of this field indicates that the PL offset associated in the second indicated joint/UL TCI state is included in the PRACH transmission power calculation.

  - FFS: Whether a restriction that only one of the indicated joint/UL TCI states can be configured with PL offset is needed.

FFS: If other information can be indicated by this same 1-bit DCI field for the PDCCH-order PRACH transmission

[119] Agreement

Support to apply PL offset on PDCCH-order PRACH in FR2

- The design for applying PL offset on PDCCH-order PRACH in FR1 is fully reused here.

Note: there is no extra enhancement for Tx beam determination for PDCCH-order PRACH in FR2 in Rel-19

[119] Agreement

The answer to the Question 1 in LS R1-2409353 is:

- From the perspective of UE: if UE is configured with PL offset in joint/UL TCI state(s), UE does not expect to receive SSB from UL TRP(s), else, UE may expect to receive SSB from UL TRP(s).

[120] Conclusion

There is no RAN1 consensus to support the following proposal:

Support the UE to report either Type1 PHR or Type 3 PHR in a serving cell configured with one UL carrier and two separate SRS CLPC adjustment states. The UE determines to report Type1 PHR or Type 3 PHR according to:

- UE provides the Type 1 PHR if both the Type 1 PHR and the Type 3 PHR are based on respective actual transmissions or on respective reference transmissions.

- UE provides the PHR that is based on a respective actual transmission if either the Type 1 PHR or the Type 3 PHR is based on a respective reference transmission.

This is subject to UE capability.

This is feature is enabled by one new RRC parameter.

[120] Conclusion

There is no consensus to introduce the restriction that only one of the indicated joint/UL TCI states can be configured with PL offset.

[120] Conclusion

There is no consensus to include PL offset in Type 3 PHR calculation.

[120] Conclusion

In asymmetric DL sTRP/UL mTRP deployment scenario, the PDCCH-order PRACH to UL TRP is CFRA.

# Issue 2: separate SRS CLPC

[116] Agreement

To facilitate the asymmetric DL sTRP/UL mTRP deployment scenarios, support two closed-loop PC adjustment states for SRS in one CC, both of which are separate from that of the PUSCH.

[116] Agreement

Study how to indicate TPC command for those two SRS CLPC adjustment states through DCI when the UE is configured two SRS CLPC adjustment states, down-select from the following options:

- Option 1: enhance the legacy DCI format 2_3 of higher layer parameter srs-TPC-PDCCH-Group = typeA;

- Option 2: enhance the legacy DCI format 2_3 of higher layer parameter srs-TPC-PDCCH-Group = typeB;

- Option 3: enhance the legacy DCI format 2_3 of higher layer parameter srs-TPC-PDCCH-Group = typeA and typeB;

- Option 4: enhance DCI format 1_1 and/or 0_1 to indicate TPC for SRS CLPC adjustment states

- Option 5: enhance the legacy DCI format 2_3 by introducing a new Type for higher layer parameter srs-TPC-PDCCH-Group

- Option 6: new DCI format to indicate TPC for SRS CLPC adjustment states

- Other options are not precluded.

For the Options1, 2, 3 and 5, consider at least the following Alts as possible examples:

- Alt1: In DCI format 2_3, add one additional TPC command for each CC configured with two SRS CLPC adjustment states,

  - the first TPC command is associated with the first SRS CLPC adjustment state and the second TPC command is associated with the second SRS CLPC adjustment state.

- Alt2: Introduce one 1-bit closed-loop-indicator field for each TPC command in DCI format 2_3

  - This 1-bit closed-loop-indicator indicates the first SRS CLPC adjustment state or the second SRS CLPC adjustment state.

- Alt3: use two different TPC-SRS-RNTIs for DCI format 2_3:

  - DCI format 2_3 with CRC scrambled with the first TPC-SRS-RNTI and the second TPC-SRS-RNTI indicates the TPC command for the first and second SRS CLPC adjustment state, respectively.

- Alt4: Implicit method:

[116] Agreement

To support two SRS CLPC adjustment states, study and possibly down-select at least one from the following Alts:

- Alt1: SRS CLPC adjustment state is associated with SRS resource set

- Alt2: When the parameter srs-PowerControlAdjustmentStates is set to 'separateClosedLoop', closedLoopIndex-r17 in the TCI state indicates one of the SRS CLPC adjustment states

- Alt3: Add one extra parameter in P0AlphaSet-r17 of TCI state to indicate one of those two SRS CLPC adjustment states

- Alt4: SRS CLPC adjustment state is associated with SRS resource usage type

Note: Other alternatives are not precluded

[116bis] Agreement

For a UE configured with two SRS CLPC adjustment states, support Alt2 for indicating one of the SRS CLPC adjustment states to SRS:

- Alt2: When the parameter srs-PowerControlAdjustmentStates is set to 'separateClosedLoop', closedLoopIndex-r17 for SRS in the TCI state indicates one of the SRS CLPC adjustment states

  - The candidate value of i0 and i1 in closedLoopIndex-r17 for SRS refers to the first and the second CLPC adjustment state separate from PUSCH, respectively

Ericsson raised concerns on Alt2 due to potential issues with beam management

[116bis] Agreement

For indicating TPC command for those two SRS CLPC adjustment states through DCI when the UE is configured with two SRS CLPC adjustment states, support Option3:

- Option 3: enhance the legacy DCI format 2_3 of higher layer parameter srs-TPC-PDCCH-Group = typeA and typeB;

[116bis] Agreement

For enhancing DCI format 2_3 for indicating TPC command for two SRS CLPC adjustment states, support Alt2:

- Alt2: Introduce one 1-bit closed-loop-indicator field for each TPC command in DCI format 2_3

  - This 1-bit closed-loop-indicator indicates the first SRS CLPC adjustment state or the second SRS CLPC adjustment state.

Note: this 1-bit indicator is present for the CC where two SRS CLPC adjustment states are configured.

[116bis] Agreement

For the asymmetric DL sTRP/UL mTRP deployment scenarios, study whether and how to indicate TPC command for SRS CLPC adjustment states through DCI format 1_1 and/or 0_1 when the UE is configured two SRS CLPC adjustment states.

[117] Agreement

In Rel-19, the value range of starting bit of block in DCI format 2-3 is extended from 1~31 to 1~X.

- FFS the value of X>31

FFS: Condition under which the above is applicable

[117] Agreement

Introduce a new RRC parameter per BWP/CC to indicate that two separate SRS CLPC adjustment states are configured for SRS in a BWP/CC

[118] Agreement

About the extended value range 1~X of starting bit of blocks in DCI format 2_3 in Rel-19, down-select one from the following Alts in RAN1#118bis:

- Alt1: X = 45 (to be captured in RAN2 spec)

  - This feature is a separate UE capability and is appliable to any rel-19 UE who supports this UE capability, regardless this UE supports two separate SRS CLPC adjustment states or not.

  - Note: X=45 can be used for operations in FR1 in shared spectrum or FR2-2 and X = 43 otherwise

- Alt2: X = 44 (to be captured in RAN2 spec)

  - This feature is only applicable to UE who is configured with two separate SRS CLPC adjustment states.

  - Note: X=44 can be used for operations in FR1 in shared spectrum for FR2-2 and X = 42 otherwise

- Above extended value range is applied to startingBitOfFormat2-3.

[118bis] Agreement

About the extended value range 1~X of starting bit of blocks in DCI format 2_3 in Rel-19, support Alt1:

- Alt1: X = 45 (to be captured in RAN2 spec)

  - This feature is a separate UE capability and is appliable to any Rel-19 UE who supports this UE capability, regardless this UE supports two separate SRS CLPC adjustment states or not.

  - Note: X=45 can be used for operations in FR1 in shared spectrum or FR2-2 and X = 43 otherwise

[118bis] Agreement

Support DCI format 1_1 to indicate TPC command for SRS CLPC adjustment state(s) separate from PUSCH:

- (Working Assumption) Introduce a 2-bit TPC command field to indicate TPC command for SRS associated with separate SRS CLPC adjustment state where:

  - The 2-bit TPC command field is present if UE reports supporting a dedicated UE capability, and a corresponding RRC parameter is configured (which is a new RRC to enable this).

- (Working Assumption) Introduce a 1-bit SRS close-loop indicator to indicate one of the two separate SRS CLPC adjustment states for the TPC command

  - The 1-bit SRS close-loop indicator is present if UE reports supporting another dedicated UE capability and a corresponding RRC parameter is configured (which is a new RRC to enable this) and two separate SRS CLPC adjustment states are configured.

[119] Agreement

The working assumption part of the following previous agreement is confirmed.

Support DCI format 1_1 to indicate TPC command for SRS CLPC adjustment state(s) separate from PUSCH:

- (Working Assumption) Introduce a 2-bit TPC command field to indicate TPC command for SRS associated with separate SRS CLPC adjustment state where:

  - The 2-bit TPC command field is present if UE reports supporting a dedicated UE capability, and a corresponding RRC parameter is configured (which is a new RRC to enable this).

- (Working Assumption) Introduce a 1-bit SRS close-loop indicator to indicate one of the two separate SRS CLPC adjustment states for the TPC command

  - The 1-bit SRS close-loop indicator is present if UE reports supporting another dedicated UE capability and a corresponding RRC parameter is configured (which is a new RRC to enable this) and two separate SRS CLPC adjustment states are configured.

[119] Agreement

For an SRS resource set not configured with followUnifiedTCI-StateSRS, regarding how to determine the PL offset and the CLPC adjustment state, down-select from the following Alts:

- Alt1: The PL offset and CLPC adjustment state indicated by the TCI state configured to the SRS resource with lowest ID (i.e., reusing the rule specified in Rel-17) and no extra enhancement.

- Alt2: The PL offset and CLPC adjustment state indicated by the TCI state configured to the SRS resource with lowest ID (i.e., reusing the rule specified in Rel-17), the SRS with lowest ID is always configured with one joint/UL TCI state, the “referenceSignal” in a UL TCI state is optional.

- Alt3:

  - When the SRS resource with lowest ID is configured with a TCI state, The PL offset and CLPC adjustment state indicated by that TCI state are applied to those SRS resources (i.e., reusing the rule specified in Rel-17)

  - Otherwise, one of the two separate SRS CLPC adjustment states and one PL offset are configured in the SRS resource set, and they are applied on the SRS resources.

- Alt4:

  - When the SRS resource with lowest ID is configured with a TCI state, The PL offset and CLPC adjustment state indicated by that TCI state are applied to those SRS resources (i.e., reusing the rule specified in Rel-17)

  - Otherwise, one PL offset and one CLPC adjustment state are applied on the SRS resources.

    - FFS how to define the PL offset and CLPC adjustment state

[120] Agreement

- The legacy TPC application mechanisms specified in legacy spec are reused for the TPC command for separate SRS CLPC adjustment state(s) indicated in DCI format 1_1 in Rel-19 and the TPC command for separate SRS CLPC adjustment states indicated in DCI format 2_3 in Rel-19.

  - One example specification changes in TS38.213 can be

7.3.1 UE behavior *** Unchanged parts are omitted *** - δSRS,b,f,c(m) is jointly coded with other TPC commands in a PDCCH with DCI format 2_3, as described in clause 11.4 or provided by TPC command field XXX in a PDCCH with DCI format 1_1 *** Unchanged parts are omitted *** - hb,f,c(i)=δSRS,b,f,c(i) if the UE is not configured for PUSCH transmissions on active UL BWP b of carrier f of serving cell c, or if srs-PowerControlAdjustmentStates indicates separate power control adjustment states between SRS transmissions and PUSCH transmissions, and tpc-Accumulation is provided, and the UE detects a DCI format 2_3 or DCI format 1_1 carrying DCI field XXX KSRS,min symbols before a first symbol of SRS transmission occasion i, where absolute values of δSRS,b,f,c are provided in Table 7.1.1-1 *** Unchanged parts are omitted ***

- Note: how to capture this in specification is up to editor.

- In Rel-19, the TPC command for separate SRS CLPC adjustment state(s) indicated in DCI format 1_1 and the TPC command for separate SRS CLPC adjustment states indicated in DCI format 2_3 are applicable to periodic SRS, semi-persistent SRS and aperiodic SRS.

[120] Agreement

Study whether/how to reset the CLPC of PUSCH/PUCCH/SRS based on the following alternatives (including if any enhancement is necessary and other alternatives are not precluded)

- Alt1: For a UE configured with two TAGs, when the UE receives a RAR of a PDCCH-order PRACH, according to the new 1-bit DCI field PL offset indication in DCI format 1_0:

  - When there is one indicated joint/UL TCI state in Rel-17 Unified TCI:

    - If the 1-bit DCI field is 0, the UE resets the CLPC that is different from the CLPC associated with the indicated joint/UL TCI state.

    - If the 1-bit DCI field is 1, the UE resets the CLPC associated with the indicated joint/UL TCI state.

  - When there are two indicated joint/UL TCI states in Rel-18 Unified TCI:

    - If the 1-bit DCI field is 0, the UE resets the CLPC associated with the first indicated joint/UL TCI state;

    - If the 1-bit DCI field is 1, the UE resets the CLPC associated with the second indicated joint/UL TCI state

- Alt 2: For a UE configured with two TAGs, one TAG ID is indicated in the RAR

  - The UE resets the closed loop power control adjustment state associated with a joint/UL TCI state associated with the indicated TAG.

# Issue 3: 2 TA

[118bis] Agreement

Support 2TA for the asymmetric DL sTRP/UL mTRP deployment scenarios:

- Remove the restriction that coresetPoolIndex needs to be configured for the 2TA feature.

- One downlink reference timing is supported and applied to both TAGs.

  - (FFS) Note: UE autonomous TA adjustment is only applicable to the first TAG

- One single n-TimingAdvanceoffset is configured and applied to both TAGs.

- Any of the TCI states can be associated with any one of the two TAGs.

- The RAR carrying TA adjustment for those 2 TAGs is reused for Rel-19 2TA

- The MAC CE based TA adjustment for 2 TAGs is reused for Rel-19 2TA.

- Introduce the optional UE capability of “Overlapping UL transmission reduction” for Rel-19 2TA

  - If UE does not report this UE capability, UE does not expect two UL transmissions associated with different TAGs are overlapped.

- FFS: UE does not expect that in intra-slot TDM PUSCH type-B repetition transmission, two consecutive repetitions associated with different TAGs are overlapped.

[119] Agreement

For a UE supporting the UE capability of “Overlapping UL transmission reduction” of Rel-19 2TA:

- The overlapping duration of the latter one of two overlapping UL transmissions is reduced, where the two overlapped UL transmissions are associated with two TAGs.

[120] Agreement

For a UE provided with SSB-MTC-AddtionalPCI and not configured with multi-DCI based mTRP, support to reuse the DCI field ‘PRACH association indicator’ in DCI format 1_0 to indicate PL RS for PDCCH-order PRACH:

- The bit field index 0 of this field indicates the DL RS that DMRS of PDCCH order DCI is QCLed with is used as PL RS for PRACH;

- The bit field index 1 of this field is mapped to the additional PCI associated with the active TCI states and indicates the indicated SSB in this DCI is used as PL RS for PRACH:

  - In this case, the PRACH configuration associated with addition PCI is used.

- This DCI field is present when the corresponding RRC parameter is configured and multi-DCI based mTRP is not configured.

# Stop