3GPP TSG RAN WG1 Meeting #120bis	R1-2501709

Wuhan, China, April 7 – 11, 2025

Agenda Item:	9.4.4

Source:	Futurewei

Title:	Discussion on multiple access, scheduling and timing aspects for A-IoT

Document for:	Discussion and Decision

# Introduction

The objectives of the revised Ambient IoT work item [1] are listed in Appendix A. The RAN1-related objectives are being treated across four agenda items.

This agenda item provides proposals for multiplexing/multiple access, scheduling information, and timing relationships based on the agreements [5] and FL summary [4].

# Discussion

## Background

The WID has specific requirements for the R2D and D2R links, as captured in the following table.

Table 1. WID requirements

|  | R2D | D2R |
| --- | --- | --- |
| Channels | PRDCH is only physical channel in R2D | PDRCH the only physical channels in D2R |
| Multiplexing | Multiplexing/multiple access in R2D is by only TDMA | Multiplexing/multiple access in D2R is by only TDMA and FDMA. |

In the FL summary, there were several suggestions for RAN1#120bis [4].

- Whether to support TDMA of X>1 and if supported, the maximum value of X is [2].

Companies are encouraged to provide analysis from the feasibility (e.g., counter) and performance (e.g., resource efficiency or latency) perspective in the next meeting.

- For the following timing relations, companies are encouraged to provide the justifications on the necessity, function/purpose. (i.e., whether 3GPP need to define these parameter(s)) [table for FL summary presented in section 2.2]

- For Msg2 reception,

  - Whether it is necessary to define the Msg2 monitoring window for device 1

  - Note It is a separate discussion from the processing time requirement

  - Following two Figures give example for cases of TDMA of X=1 and TDMA of X>1 if supported (the figures are placed in section 2.3.2)

## Timing

The FL proposed the entries for this table to be completed / agreed.

Table 2. FL summary table for timing [5].

| Timings | Description in TR 38.769 | Necessity/Function/Purpose (Example) Whether 3GPP need to define these parameter(s)? |
| --- | --- | --- |
| TR2D_min | Minimum Time between a R2D transmission and the corresponding D2R transmission following it | Requirement on device E.g., Time required for device processing from the end of R2D Rx to the earliest start of the corresponding D2R Tx from device perspective |
| TR2D_max | Define a maximum time TR2D_max between a R2D transmission and the corresponding D2R transmission following it, so that the device transmits D2R transmission within [TR2D_min, TR2D_max]. | Requirement on device Time required for device processing from the end of R2D Rx to the latest start of the corresponding D2R Tx from device perspective |
| TD2R_min | Minimum Time between a D2R transmission and the corresponding R2D transmission following it. | E.g., Requirement on device Time required for device processing from the end of D2R Tx to the earliest start of the corresponding R2D Rx from device perspective |
| TD2R_max | When a R2D transmission in response to a D2R transmission is expected for A-IoT Msg2 response to A-IoT Msg1 for the A-IoT device, study to define a maximum time TD2R_max between the D2R transmission and the corresponding R2D transmission following it, so that the R2D transmission timing is expected to be within [TD2R_min, TD2R_max]. |  |

We are unsure whether the FL is proposing a revision of the TR description, which can be captured in a specification, or providing examples (which may not get captured in a specification).

For TR2D_min, the intent of the description is correct but a revision to refine the time interval is necessary.

Proposal 1. For A-IoT devices, support the timing TR2D_min with the following revision:

Minimum Time between the end of a R2D transmission and the start of the corresponding D2R transmission following it

For TR2D_max, a similar revision of the description is necessary.

Proposal 2. For A-IoT devices, support the timing TR2D_max with the following revision:

Define a maximum time TR2D_max between the end of a R2D transmission and the start of the corresponding D2R transmission following it, so that the device starts a transmits D2R transmission within [TR2D_min, TR2D_max].

For TD2R_min, a similar revision of the description is necessary.

Proposal 3. For A-IoT devices, support the timing TD2R_min with the following revision:

Minimum Time between the end of a D2R transmission and the start of the corresponding R2D transmission following it.

For TD2R_max, the description within the TR should be modified for a specification. Note for the Msg2 discussions, this timing TDR2_max may be applied for at least FDMA.

Proposal 4. For A-IoT devices, support the timing TD2R_max with the following description:

Maximum time between the end of a D2R transmission and the start of the corresponding R2D transmission following it, so that the R2D transmission timing is expected to be within [TD2R_min, TD2R_max] when the corresponding R2D transmission is the expected A-IoT Msg2 response to A-IoT Msg1 for the A-IoT device

## Random access

### Msg1

For FDMA, the agreement is captured from a reader perspective. It is expected that RAN2 will specify details on how the device is provided information for the set of Y candidate frequency domain resources and on how the device selects a frequency domain resource for the transmission of Msg. 1. The value of Y depends on the results of the discussions in AI 9.4.2.

For TDMA, both X=1 and X>1 can be supported. For example, a reader can specify X=4 time domain resources. An X>1 device can use all four time domain resources while an X=1 device uses the first time domain resource. A reader needs to ensure that a transmission from an X=1 device does not affect reception of the second time domain resource. As for the maximum number of time domain resources, we support a maximum of four as we have shown that there is efficiency with four resources when the guard time between each resource increases [3].

Proposal 5: For a A-IoT device supporting X>1 TDMA, support 4 time domain resources.

### Msg2

In RAN1#120, several agreements for Msg2 were reached.

Agreement Support following two options for Msg2 transmission in response to multiple Msg1 transmissions, which are initiated by a R2D transmission triggering random access. - Option 1: A PRDCH for Msg2 transmission corresponds to a A-IoT Msg1 received from one device - Option 2: A PRDCH for Msg2 transmission corresponds to multiple A-IoT Msg1 received from different devices - FFS the maximum number of Msg1 responses that can be carried within a PRDCH for Msg2.

With option 2 of the agreement, there is an FFS for the maximum number of Msg1 responses. Due to the absence of FEC on the R2D link, a large sized Msg2 is more prone to bit errors, which impacts Msg3 scheduling and increases latency. For example, if the size of each response is 40 bits (16 bits RN + 8 bits MAC + 16 bits for scheduling), then 160 bits are needed for four responses.

Observation 1: The maximum number of Msg1 responses that can be carried within a PRDCH for Msg2 is a function of the maximum TBS for reliable reception of Msg2.

The FL provided two figures for discussion of Msg2 timing for TDMA cases of X=1 and X>1 in Fig. 1 and Fig. 2, respectively.

Fig. 1. TDMA of X=1 [4]

For an X=1 device, as soon as the time value TD2R_min elapses, it begins to monitor for a Msg2. Instead of using timers (e.g., TD2R_max), the device monitors R2D messages. If no corresponding Msg2 was received before the next Paging message, the device can assume its Msg1 transmission failed. No other timers are necessary.

Fig. 2. TDMA of X>1 [4]

The behavior for X>1 devices can be similar. After a device transmits Msg1, it waits until at least TD2R_min elapses before the device monitors for Msg2. If the device does not observe a Msg2 in response to the device’s Msg1 transmission, the device monitors for either a Paging message (which would indicate that the reception of a Msg1 transmission was unsuccessful) or a next Msg2. Note the behavior is appropriate for case of a device unsuccessfully receiving a Msg2 that does contain the response to the Msg1 transmission.

Proposal 6: For TDMA, a device that transmitted A-IoT Msg1 monitors a Msg2 transmission that corresponds to a A-IoT Msg1 from the device. If the device does not receive the Msg2 transmission by the next paging message, the device considers the A-IoT Msg1 transmission failed.

### Msg3

For Msg3, there was an agreement for FDMA.

Agreement Support FDMA of D2R transmissions for Msg3 from multiple devices in response to a PRDCH for Msg2 transmission corresponding to multiple Msg1 during access procedure. Agreement Support the following option(s) for frequency domain resource for Msg3 transmission: - Option 2: The frequency domain resource for Msg3 can be determined based on explicit indication in the PRDCH for Msg2 transmission for one or multiples devices. - FFS: support option 1 as a default if PRDCH does not provide the indication or based on a rule - Option 1: the frequency domain resource for Msg3 reuses the frequency domain resource for Msg1 for the same device

Option 2 is the general solution while the FFS represents an optimization. Further. the reader may choose to reallocate resources based on other considerations and the number of successful responses.

Proposal 7: For the agreements for frequency domain resources for Msg3 transmission, drop the FFS.

# Conclusion

The following observations / proposals were discussed in this contribution.

Timing

Proposal 1. For A-IoT devices, support the timing TR2D_min with the following revision:

Minimum Time between the end of a R2D transmission and the start of the corresponding D2R transmission following it

Proposal 2. For A-IoT devices, support the timing TR2D_max with the following revision:

Define a maximum time TR2D_max between the end of a R2D transmission and the start of the corresponding D2R transmission following it, so that the device starts a transmits D2R transmission within [TR2D_min, TR2D_max].

Proposal 3. For A-IoT devices, support the timing TD2R_min with the following revision:

Minimum Time between the end of a D2R transmission and the start of the corresponding R2D transmission following it.

Proposal 4. For A-IoT devices, support the timing TD2R_max with the following description:

Maximum time between the end of a D2R transmission and the start of the corresponding R2D transmission following it, so that the R2D transmission timing is expected to be within [TD2R_min, TD2R_max] when the corresponding R2D transmission is the expected A-IoT Msg2 response to A-IoT Msg1 for the A-IoT device

Msg1 / Msg2

Proposal 5: For a A-IoT device supporting X>1 TDMA, support 4 time domain resources.

Observation 1: The maximum number of Msg1 responses that can be carried within a PRDCH for Msg2 is a function of the maximum TBS for reliable reception of Msg2.

Proposal 6: For TDMA, a device that transmitted A-IoT Msg1 monitors a Msg2 transmission that corresponds to a A-IoT Msg1 from the device. If the device does not receive the Msg2 transmission by the next paging message, the device considers the A-IoT Msg1 transmission failed.

Proposal 7: For the agreements for frequency domain resources for Msg3 transmission, drop the FFS.

# References

RP-250796, “Revised WID: Solutions for Ambient IoT (Internet of Things) in NR”, CMCC, Huawei, T-Mobile USA Inc, RAN#107, Mar. 12-14, 2025.

TR 38.769, Study on solutions for Ambient IoT (Internet of Things) in NR

R1-2500047, “Discussion on multiple access, scheduling and timing aspects for A-IoT”, Futurewei, RAN1#120, Feb. 17-21, 2025

R1-2501625, “Final FL summary on other aspects for Rel-19 Ambient IoT”, Moderator (vivo), RAN1#120, Feb. 17-21, 2025

R1-2501547, “Session notes for 9.4 (Study on solutions for Ambient IoT in NR)”, Chairman, RAN1#120, Feb. 17-21, 2025

# Appendices

## Appendix A: Work item objectives [1]

This WI is to standardize RAN aspects of Ambient IoT, a new 3GPP IoT technology, suitable for deployment in a 3GPP system, which relies on ultra-low complexity devices with ultra-low power consumption for the very-low end IoT applications. The work shall provide clear differentiation, i.e. addressing use cases and scenarios that cannot otherwise be fulfilled based on existing 3GPP LPWA IoT technology e.g. NB-IoT including with reduced peak Tx power. General Scope The definitions provided in TR 38.848, TR 38.769, and decisions, etc. made during the Rel-19 SI in RAN WGs are taken into this WI, and the following is the exclusive general scope: - The overall objective shall be to standardize the following Ambient IoT device: Device 1: ~1 µW peak power consumption, has energy storage, RF envelope detector receiver, initial sampling frequency offset (SFO) up to 10X ppm, neither R2D nor D2R amplification in the device. The device’s D2R transmission is backscattered on a carrier wave provided externally. - Deployment scenario 1 with Topology 1, according to D1T1-B. - FR1 licensed spectrum in FDD, with R2D in DL spectrum and D2R and CW in UL spectrum. - Spectrum deployment in-band to NR and standalone, with A-IoT BS located indoor. - Traffic types DO-DTT, DT, for rUC1 (indoor inventory) and rUC4 (indoor command). - Carrier wave transmission for waveform 1 only, without hopping, per the following cases in TR 38.769: - Case 1-4 for D1T1-B - Proximity determination via Solution 1 in TR 38.769 only. - Device (un)availability via Direction 1 in TR 38.769 only. WGs begin their discussions from the decisions already made in TR 38.769, with the following refinements for the scope: The following objectives are set, within the General Scope: - RAN1 scope: - PRDCH and PDRCH, which are the only physical channels in R2D and D2R, respectively. - R2D and D2R signal(s) - Multiplexing/multiple access in R2D is by only TDMA, and in D2R is by only TDMA and FDMA. - R2D supports only OOK-4 modulation, one solution for CP handling. D2R backscattering supports only OOK and BPSK modulations. - R2D transmission supports only the Manchester line code in TR 38.769 - D2R transmission supports: - Either the Manchester line code in TR 38.769 or no line code (one to be down-selected); and - A corresponding small frequency shift method according to the options in TR 38.769. - R2D does not support FEC. D2R supports only convolutional code with generator polynomials as per TS 36.212 (unless RAN1 decides to use other generator polynomials by RAN1#120bis). Applying or not applying the FEC to D2R is specified by ensuring it is under the reader control and applies to all devices targeted by the reader. - PRDCH and PDRCH both support transmission without CRC, and with CRC as per the generator polynomials in TS 38.212 (unless RAN1 decides to use other generator polynomials by RAN1#120bis) for 6-bit CRC and 16-bit CRC. Cases to use which length of CRC, or no CRC, to be decided in RAN1. - D2R supports physical layer repetition transmission. R2D does not support physical-layer repetition transmission. - RAN2 scope: - Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission: - A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Temporary identifier is not supported, unless required by SA WGs. Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes. - A-IoT Random access, including re-access for failure handling. Contention-based and contention-free cases are supported. For the contention-based random access, only Solution 1 (3-step only) is included (unless RAN2 decides to use Solution 3 (unified solution) by RAN2#129). - A-IoT data transmission, including data (re-)transmission for failure handling. Segmentation is supported at least in D2R. - Only MAC layer is included - RAN3 scope: - Specify necessary architectural aspects, and signaling and procedures between A-IoT RAN and A-IoT CN to support the A-IoT functions, assuming an architecture of aggregated gNB, including: - Inventory and command operations - Device location reporting at reader ID granularity Note: The above A-IoT functions are supported over the existing NG interface, based on architecture(s) defined by RAN3/SA2. - RAN4 scope: - Specify RF requirements for Ambient-IoT BS, device 1, and CW - RF requirements for Type 1-C Ambient-IoT BS - RF requirements for device 1 - RF requirements for CW - Specify RRM core requirements for device 1, if necessary - Study and develop OTA test methodology for A-IoT device 1 - Consider test methods specified in TR 38.870 as starting point. Take test system reuse, test system complexity and test time into account, when developing test methods suitable for Ambient IoT. - Develop the preliminary Measurement Uncertainty (MU) assessment for the test system - Use band n8 as an example band Note 1: Coordination with SA2 and SA3 is expected. Updates to the WID objectives should be considered if needed. Note 2: This WI shall target for an IoT segment well below the existing 3GPP IoT technologies, e.g. NB-IoT, eMTC, RedCap, etc. The WI shall not aim to replace existing 3GPP LPWA technologies.

## Appendix B – agreements from RAN1#120 [5]

Agreement Support FDMA with Y≥1 D2R transmissions for Msg.1 from multiple devices in response to a R2D transmission triggering random access. - The maximum value of Y >1 is determined in AI 9.4.2, based on feasible values of small frequency shift to be decided in AI 9.4.2. - Signalling details should be discussed and determined in AI 9.4.4. Agreement Support following two options for Msg2 transmission in response to multiple Msg1 transmissions, which are initiated by a R2D transmission triggering random access. - Option 1: A PRDCH for Msg2 transmission corresponds to a A-IoT Msg1 received from one device - Option 2: A PRDCH for Msg2 transmission corresponds to multiple A-IoT Msg1 received from different devices - FFS the maximum number of Msg1 responses that can be carried within a PRDCH for Msg2. Agreement Support FDMA of D2R transmissions for Msg3 from multiple devices in response to a PRDCH for Msg2 transmission corresponding to multiple Msg1 during access procedure. Agreement Support the following option(s) for frequency domain resource for Msg3 transmission: - Option 2: The frequency domain resource for Msg3 can be determined based on explicit indication in the PRDCH for Msg2 transmission for one or multiples devices. - FFS: support option 1 as a default if PRDCH does not provide the indication or based on a rule - Option 1: the frequency domain resource for Msg3 reuses the frequency domain resource for Msg1 for the same device Agreement The following is supported for indicating the D2R chip duration: - The D2R chip duration is indicated in R2D control information from predefined a set of D2R chip duration values Agreement The payload size (i.e. TBS-like) for PDRCH transmission with variable size is explicitly indicated in the corresponding R2D control information.