3GPP TSG RAN WG1 Meeting #120bis 																																										R1-2501732

Wuhan, China, April 7th – 11th, 2025

Agenda Item: 9.15.2

Source: MediaTek Inc.

Title: 	UE features for NR MIMO Phase 5

Document for: Discussion and Decision

Introduction

This paper provides our views on Rel-19 UE features for NR MIMO Phase 5. Detailed proposals are described in the following sections.

Discussion

Enhancements for UE-initiated/event-driven beam management

FG 59-1-1 (UE-initiated/event-driven beam management for Mode A)

This FG on supporting at least Mode A should be the basic FG for UEI/ED beam reporting, and Event-2 based measurement and reporting should be the basic feature as well. If a UE indicating it doesn’t support this FG, the whole feature of UEI/ED beam reporting is not supported by the UE even UE could support the other events or Mode B. Hence, we prefer removing the wording of “for Event-2 and Mode A” in the field of “Consequence if the feature is not supported by the UE”.

According to the following agreement, the basic FG should also support that the first PUCCH and second PUSCH are from the same PUCCH group.

Agreement On cross-CC beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Event-2, the following working assumption is confirmed with the following modification. - The first PUCCH and the second PUSCH associated for UE-initiated/event-driven beam reporting are from different PUCCH groups - Subject to separate UE capability - Note: From RAN1 perspective, the above does NOT introduce any spec impact except for UE capability signaling

Proposal 1: Support FG 59-1-1 with the following changes:

| 59. NR_MIMO_Ph5 | 59-1-1 | UE-initiated/event-driven beam management for Mode A | 1. Support of UE-initiated/event-driven beam report based on one event instance 2. Support of Event-2 based measurement and report 3. Support of Mode A UE-initiated/event-driven beam report 4. Maximum number of the configured RS(s) for new beam in the RS resource set 5. Support of current beam measurement by using QCL RS in the indicated TCI state and the corresponding QCL SSB for Scheme-1 and Scheme-2, respectively 6. Support the first PUCCH and second PUSCH from the same PUCCH group | FFS | yes | n/a | UEI/ED beam report is not supported [for Event-2 and Mode A] | FFS | FFS | FFS | FFS | [Component 4 candidate values: {1, 2, … , 64}] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

FG 59-1-2 (UE-initiated/event-driven beam management for Mode B)

For component 2, as FG 59-1-1 is the prerequisite FG of this FG and we don’t expect UE will have different capabilities on the maximum number of the new beam RS(s) in the RS resource set for support Mode A and Mode B. Hence, we suggest removing the component 2 and the relevant note.

Proposal 2: Support FG 59-1-2 with the following changes:

| 59. NR_MIMO_Ph5 | 59-1-2 | UE-initiated/event-driven beam management Mode B | 1. Support of Mode B UE-initiated/event-driven beam report [2. Maximum number of the configured RS(s) for new beam in the RS resource set] [3. Supported values of X] | FG 59-1-1 | yes | n/a | UEI/ED beam report Mode-B is not supported for Event-2 | FFS | FFS | FFS | FFS | [Component 2 candidate values: {0,1,…,[N]}] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

FG 59-1-4 (UE-initiated/event-driven beam management for Event-1 based measurement and report [for Mode A])

This FG is a separate FG for optional support of Event-1, and FG 59-1-1 should be the prerequisite FG of this FG. We prefer to make this FG general regardless of UE capability on supporting Mode A only or Mode A+ Mode B. With FG 59-1-1 as the prerequisite, UE supporting this FG must support at least Mode A. If UE supports FG 59-1-1 + FG 59-1-2 + FG 59-1-4, then UE supports Even-1/Event-2 with ether Mode A or Mode B. Hence, we suggest removing the wording of “for Mode A” in the fields of “Components” and “Consequence if the feature is not supported by the UE”.

For component 1, the window will not be provided if FG 5-1-3 is supported. Hence, we suggest removing the wording of “within a time window”.

For component 2, as FG 59-1-1 is the prerequisite FG of this FG, it can be removed since they are already included in FG 59-1-1.

For component 3, event-dependent UE capability for maximum number of the configured RS(s) for new beam measurement in the RS resource set is needed. Hence, we prefer to keep this component.

Proposal 3: Support FG 59-1-4 with the following changes:

| 59. NR_MIMO_Ph5 | 59-1-4 | UE-initiated/event-driven beam management for Event-1 based measurement and report [for Mode A] | 1. Support of Event-1 based measurement and report that L1-RSRP of the current beam becomes worse than a configured threshold based on one event instance [within a time window] [2. Support of current beam measurement by using QCL RS in the indicated TCI state and the corresponding QCL SSB for Scheme-1 and Scheme-2, respectively] [3. Maximum number of the configured RS(s) for new beam measurement in the RS resource set] [FFS: memory] | 59-1-1 | yes | n/a | UE-initiated/event-driven beam management for Event-1 based measurement is not supported [for Mode A] | Per Band | n/a | FFS: FR2 only | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

FG 59-1-5 (UE-initiated/event-driven beam management for Event-1 based measurement and report [for Mode A])

Analogues to FG 59-1-4, This FG is a separate FG for optional support of Event-7, and FG 59-1-1 should be the prerequisite FG of this FG. The FF should be general regardless of UE capability on supporting Mode A only or Mode A+ Mode B. Hence, we suggest removing the wording of “for Mode A” in the fields of “Components” and “Consequence if the feature is not supported by the UE”.

For component 1, the window will not be provided if FG 5-1-3 is supported. Hence, we suggest removing the wording of “within a time window”.

For component 3, it should be kept because the RS should be derived from the activated TCI state the Q-th best quality instead of the indicated TCI state for FG 59-1-1.

For component 4, according to the following agreement, one more component should be added for indicating the supported value(s) of Q.

Agreement On UE-initiated/event-driven beam reporting, regarding trigger events, the following working assumption in RAN1#118bis is confirmed: On UE-initiated/event-driven beam reporting, regarding trigger events, besides for Event-2, Event-1 and Event-7 are both supported. - Event-1: Quality of the current beam is worse than a certain threshold. - Event-7: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the Q-th best quality. - Q is RRC configured with subjective to UE capability signalling - UE may only indicate a single candidate value or not support Event-7. - The additionally supported events will reuse the same design as event 2 – unless there is consensus to do otherwise - The additionally supported events will be lower priority compared to event 2.

Proposal 4: Support FG 59-1-5 with the following changes:

| 59. NR_MIMO_Ph5 | 59-1-5 | UE-initiated/event-driven beam management for Event-7 based measurement and report [for Mode A] | 1. Support of Event-7 that L1-RSRP of at least one new beam becomes a threshold value better than the RS derived from the activated TCI state with the Q-th best quality based on one event instance [within a time window] 2. Maximum number of the configured RS(s) for new beam measurement and event evaluation in the RS resource set [3. Support of the RS derived from the activated TCI state with the Q-th best quality measurement by using QCL RS in the activated TCI state with the Q-th best quality and the corresponding QCL SSB for the activated TCI state with the Q-th best quality for Scheme-1 and Scheme-2, respectively] 4. Supported values of Q | 59-1-1 | yes | n/a | UE-initiated/event-driven beam management for Event-7 based measurement is not supported [for Mode A] | Per Band | n/a | FFS: FR2 only | n/a | Component 1 candidate values for Q: [{1, 2, ..., 8}] Component 2 candidate values: {1, 2, ..., 64} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(New FG) FG 59-1-7 (UE-initiated/event-driven beam management for Event-1 based measurement and report [for Mode A])

In additional to above FGs, we propose to add three new FGs based on the following RAN1 agreement:

Agreement On UE-initiated/event-driven beam reporting, Option-1 is supported as an extension on L1-RSRP report format depending on Event-2. - Option-1: For each of reported CRI/SSBRI, to introduce additional indication of whether the CRI/SSBRI satisfies the condition of Event-2. - The presence of this field is enabled by RRC with subjective to UE capability. - Note: The presence of this field is only for the case that N > 1 and the time window and M are configured Above is NOT applicable for event 1.

Proposal 5: Introduce the following new FG according to RAN1 agreement:

| 59. NR_MIMO_Ph5 | 59-1-7 | Additional indication of whether the CRI/SSBRI satisfies the condition of Event-2 and Event-7 | Support additional indication of whether the CRI/SSBRI satisfies the condition of Event-2 and Event-7 | FG 59-1-3 | yes | n/a | Additional indication of whether the CRI/SSBRI satisfies the condition of Event-2 and Event-7 is not supported | FFS | FFS | FFS | FFS |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Enhancement for asymmetric DL sTRP/UL mTRP scenarios

(New FG) FG 59-4-1c (PL offset for Type 1 PHR calculation)

According to the following agreement, PL offset for Type 1 PHR calculation is supported, but it is missed in current UE feature list. Thus, we propose to introduce one new FG.

Agreement For the asymmetric DL sTRP/UL mTRP scenarios, support to include PL offset in the calculation of Type 1 PHR based on actual PUSCH transmission and Type 1 PHR based on reference PUSCH

Proposal 6: Introduce separate FG 59-4-1c PL offset for Type 1 PHR calculation as follows:

| 59. NR_MIMO_Ph5 | 59-4-1c | PL offset for Type 1 PHR calculation | Support to include PL offset in the calculation of Type 1 PHR based on actual PUSCH transmission and Type 1 PHR based on reference PUSCH | 59-4-1a, 59-4-1b | yes | n/a | PL offset for Type 1 PHR calculation is not supported | FFS | FFS | FFS | FFS |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

FG 59-4-3 (Two SRS closed-loop power control adjustment states separate from PUSCH)

According to the following agreement, two SRS CLPC adjustment states separate from PUSCH should be supported based on Rel-17 unified TCI framework (using closedLoopIndex-r17 in a TCI state), thus FG23-1-1 should be the prerequisite FG.

Agreement For a UE configured with two SRS CLPC adjustment states, support Alt2 for indicating one of the SRS CLPC adjustment states to SRS: - Alt2: When the parameter srs-PowerControlAdjustmentStates is set to 'separateClosedLoop', closedLoopIndex-r17 for SRS in the TCI state indicates one of the SRS CLPC adjustment states - The candidate value of i0 and i1 in closedLoopIndex-r17 for SRS refers to the first and the second CLPC adjustment state separate from PUSCH, respectively.

Proposal 7: FG23-1-1 should be the prerequisite FG for FG59-4-3.

| 59. NR_MIMO_Ph5 | 59-4-3 | Two SRS closed-loop power control adjustment states separate from PUSCH | Support of two separate SRS closed loop indices separate from PUSCH | 23-1-1 | yes | n/a | Two separate SRS closed loop indexes is not supported | FFS | FFS | FFS | FFS |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

FG 59-4-5 (Overlapping UL transmission reduction)

Since FG59-4-5 is supported only when the UE is with two TA enhancement, FG59-4-4a and  FG59-4-4b should be the prerequisite FG.

Proposal 8: FG59-4-4a and  FG59-4-4b should be the prerequisite FG for FG59-4-5.

| 59. NR_MIMO_Ph5 | 59-4-5 | Overlapping UL transmission reduction | Support of reducing the overlapping duration of the later of the two time-domain overlapping UL transmissions when the UE is with two TA enhancement | 59-4-4a, 59-4-4b | yes | n/a | Reducing the overlapping duration of the later of the two time-domain overlapping UL transmissions is not supported | FFS | FFS | FFS | FFS | Note: If UE does not support this feature, UE does not expect the two UL transmissions to overlap | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

FG 59-4-6 (MAC-CE update of PL offset value(s))

FG59-4-1a, FG59-4-1b, FG59-4-2a and FG59-4-2b should be the prerequisite FG for FG59-4-6.

Proposal 9: FG59-4-1a, FG59-4-1b, FG59-4-2a and FG59-4-2b should be the prerequisite FG for FG59-4-6.

| NR MIMO Phase 5 | 59-4-6 | MAC-CE update of PL offset value(s) | Support of MAC-CE update of the configured PL offset value(s) | 59-4-1a, 59-4-1b, 59-4-2a, 59-4-2b | yes | n/a | MAC-CE update of the configured PL offset value(s) is not supported | FFS | FFS | FFS | FFS |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

FG 59-4-8, FG 59-4-9a, FG 59-4-9b (DCI format 1_1 to indicate TPC for separate SRS CLPC adjustment state)

Based on current FGs, we already have separate FGs (FG 59-4-9a and FG 59-4-9b) for support of DCI format 1_1 to indicate TPC for separate SRS closed loop index(es) under separate DL/UL TCI state mode. Thus, FG59-4-8 should be used for “DCI format 1_1 to indicate TPC for a separate SRS CLPC adjustment state”. Note that there is no closed loop index for SRS CLPC adjustment state is there is only one.

Proposal 10: Support the following modifications for FG 59-4-8, FG 59-4-9a, FG 59-4-9b:

| 59. NR_MIMO_Ph5 | 59-4-8 | DCI format 1_1 to indicate TPC for a separate SRS CLPC adjustment state closed loop index(es) | Support of DCI format 1_1 to indicate TPC for a separate SRS CLPC adjustment state closed loop index(es) | FFS | yes | n/a | DCI 1_1 indicating TPC command for a separate SRS CLPC adjustment state closed loop index(es) is not supported | FFS | FFS | FFS | FFS | FFS: Further partitioning of this FG based on existing and future agreements (e.g., whether to split based on deployment scenario such asymmetric DL sTRP/UL mTRP and other scenarios) and also whether to have one FG is “DCI format 1_1 for TPC command indicatinon for a separate SRS CLPC adjustment state” and another is “DCI format 1_1 for TPC command indication for two separate SRS CLPC adjustment states” | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 59. NR_MIMO_Ph5 | 59-4-9a | DCI format 1_1 to indicate TPC for one of two separate SRS closed loop indexes under separate DL/UL TCI state mode | Support of DCI format 1_1 to indicate TPC for one of separate SRS closed loop index(es) under separate DL/UL TCI state mode | FFS | yes | n/a | DCI 1_1 indicating TPC for one of two separate SRS closed loop indexes under separate DL/UL TCI state mode is not supported | FFS | FFS | FFS | FFS |  | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-4-9b | DCI format 1_1 to indicate TPC for one of two separate SRS closed loop indexes under joint TCI state mode | Support of DCI format 1_1 to indicate TPC for one of separate SRS closed loop index(es) under joint TCI state mode | FFS | yes | n/a | DCI 1_1 indicating TPC for one of two separate SRS closed loop indexes under joint TCI state mode is not supported | FFS | FFS | FFS | FFS |  | Optional with capability signalling |

Conclusion

Based on the discussion, we made the following proposals:

Enhancements for UE-initiated/event-driven beam management

Proposal 1: Support FG 59-1-1 with the following changes:

| 59. NR_MIMO_Ph5 | 59-1-1 | UE-initiated/event-driven beam management for Mode A | 1. Support of UE-initiated/event-driven beam report based on one event instance 2. Support of Event-2 based measurement and report 3. Support of Mode A UE-initiated/event-driven beam report 4. Maximum number of the configured RS(s) for new beam in the RS resource set 5. Support of current beam measurement by using QCL RS in the indicated TCI state and the corresponding QCL SSB for Scheme-1 and Scheme-2, respectively 6. Support the first PUCCH and second PUSCH from the same PUCCH group | FFS | yes | n/a | UEI/ED beam report is not supported [for Event-2 and Mode A] | FFS | FFS | FFS | FFS | [Component 4 candidate values: {1, 2, … , 64}] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 2: Support FG 59-1-2 with the following changes:

| 59. NR_MIMO_Ph5 | 59-1-2 | UE-initiated/event-driven beam management Mode B | 1. Support of Mode B UE-initiated/event-driven beam report [2. Maximum number of the configured RS(s) for new beam in the RS resource set] [3. Supported values of X] | FG 59-1-1 | yes | n/a | UEI/ED beam report Mode-B is not supported for Event-2 | FFS | FFS | FFS | FFS | [Component 2 candidate values: {0,1,…,[N]}] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 3: Support FG 59-1-4 with the following changes:

| 59. NR_MIMO_Ph5 | 59-1-4 | UE-initiated/event-driven beam management for Event-1 based measurement and report [for Mode A] | 1. Support of Event-1 based measurement and report that L1-RSRP of the current beam becomes worse than a configured threshold based on one event instance [within a time window] [2. Support of current beam measurement by using QCL RS in the indicated TCI state and the corresponding QCL SSB for Scheme-1 and Scheme-2, respectively] [3. Maximum number of the configured RS(s) for new beam measurement in the RS resource set] [FFS: memory] | 59-1-1 | yes | n/a | UE-initiated/event-driven beam management for Event-1 based measurement is not supported [for Mode A] | Per Band | n/a | FFS: FR2 only | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 4: Support FG 59-1-5 with the following changes:

| 59. NR_MIMO_Ph5 | 59-1-5 | UE-initiated/event-driven beam management for Event-7 based measurement and report [for Mode A] | 1. Support of Event-7 that L1-RSRP of at least one new beam becomes a threshold value better than the RS derived from the activated TCI state with the Q-th best quality based on one event instance [within a time window] 2. Maximum number of the configured RS(s) for new beam measurement and event evaluation in the RS resource set [3. Support of the RS derived from the activated TCI state with the Q-th best quality measurement by using QCL RS in the activated TCI state with the Q-th best quality and the corresponding QCL SSB for the activated TCI state with the Q-th best quality for Scheme-1 and Scheme-2, respectively] 4. Supported values of Q | 59-1-1 | yes | n/a | UE-initiated/event-driven beam management for Event-7 based measurement is not supported [for Mode A] | Per Band | n/a | FFS: FR2 only | n/a | Component 1 candidate values for Q: [{1, 2, ..., 8}] Component 2 candidate values: {1, 2, ..., 64} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 5: Introduce the following new FG according to RAN1 agreement:

| 59. NR_MIMO_Ph5 | 59-1-7 | Additional indication of whether the CRI/SSBRI satisfies the condition of Event-2 and Event-7 | Support additional indication of whether the CRI/SSBRI satisfies the condition of Event-2 and Event-7 | FG 59-1-3 | yes | n/a | Additional indication of whether the CRI/SSBRI satisfies the condition of Event-2 and Event-7 is not supported | FFS | FFS | FFS | FFS |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Enhancement for asymmetric DL sTRP/UL mTRP scenarios

Proposal 6: Introduce separate FG 59-4-1c PL offset for Type 1 PHR calculation as follows:

| 59. NR_MIMO_Ph5 | 59-4-1c | PL offset for Type 1 PHR calculation | Support to include PL offset in the calculation of Type 1 PHR based on actual PUSCH transmission and Type 1 PHR based on reference PUSCH | 59-4-1a, 59-4-1b | yes | n/a | PL offset for Type 1 PHR calculation is not supported | FFS | FFS | FFS | FFS |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 7: FG23-1-1 should be the prerequisite FG for FG59-4-3.

| 59. NR_MIMO_Ph5 | 59-4-3 | Two SRS closed-loop power control adjustment states separate from PUSCH | Support of two separate SRS closed loop indices separate from PUSCH | 23-1-1 | yes | n/a | Two separate SRS closed loop indexes is not supported | FFS | FFS | FFS | FFS |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 8: FG59-4-4a and FG59-4-4b should be the prerequisite FG for FG59-4-5.

| 59. NR_MIMO_Ph5 | 59-4-5 | Overlapping UL transmission reduction | Support of reducing the overlapping duration of the later of the two time-domain overlapping UL transmissions when the UE is with two TA enhancement | 59-4-4a, 59-4-4b | yes | n/a | Reducing the overlapping duration of the later of the two time-domain overlapping UL transmissions is not supported | FFS | FFS | FFS | FFS | Note: If UE does not support this feature, UE does not expect the two UL transmissions to overlap | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 9: FG59-4-1a, FG59-4-1b, FG59-4-2a and FG59-4-2b should be the prerequisite FG for FG59-4-6.

| NR MIMO Phase 5 | 59-4-6 | MAC-CE update of PL offset value(s) | Support of MAC-CE update of the configured PL offset value(s) | 59-4-1a, 59-4-1b, 59-4-2a, 59-4-2b | yes | n/a | MAC-CE update of the configured PL offset value(s) is not supported | FFS | FFS | FFS | FFS |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 10: Support the following modifications for FG 59-4-8, FG 59-4-9a, FG 59-4-9b:

| 59. NR_MIMO_Ph5 | 59-4-8 | DCI format 1_1 to indicate TPC for a separate SRS CLPC adjustment state closed loop index(es) | Support of DCI format 1_1 to indicate TPC for a separate SRS CLPC adjustment state closed loop index(es) | FFS | yes | n/a | DCI 1_1 indicating TPC command for a separate SRS CLPC adjustment state closed loop index(es) is not supported | FFS | FFS | FFS | FFS | FFS: Further partitioning of this FG based on existing and future agreements (e.g., whether to split based on deployment scenario such asymmetric DL sTRP/UL mTRP and other scenarios) and also whether to have one FG is “DCI format 1_1 for TPC command indicatinon for a separate SRS CLPC adjustment state” and another is “DCI format 1_1 for TPC command indication for two separate SRS CLPC adjustment states” | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 59. NR_MIMO_Ph5 | 59-4-9a | DCI format 1_1 to indicate TPC for one of two separate SRS closed loop indexes under separate DL/UL TCI state mode | Support of DCI format 1_1 to indicate TPC for one of separate SRS closed loop index(es) under separate DL/UL TCI state mode | FFS | yes | n/a | DCI 1_1 indicating TPC for one of two separate SRS closed loop indexes under separate DL/UL TCI state mode is not supported | FFS | FFS | FFS | FFS |  | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-4-9b | DCI format 1_1 to indicate TPC for one of two separate SRS closed loop indexes under joint TCI state mode | Support of DCI format 1_1 to indicate TPC for one of separate SRS closed loop index(es) under joint TCI state mode | FFS | yes | n/a | DCI 1_1 indicating TPC for one of two separate SRS closed loop indexes under joint TCI state mode is not supported | FFS | FFS | FFS | FFS |  | Optional with capability signalling |