3GPP TSG RAN WG1 #120b	R1-2501776

Wuhan, China, April 7th - 11th, 2025

Source:	Nokia

Title:	AIoT Physical channels design - modulation aspects

Agenda item:	9.4.1

Document for:	Discussion and Decision

# Introduction

At RAN#106, a new work item “Solutions for Ambient IoT (Internet of Things) in NR” (Ambient_IoT_Solutions) was approved; the WID was most recently revised in [1].

The following objective(s) is/are relevant for the present agenda item:

- RAN1 scope: - PRDCH and PDRCH, which are the only physical channels in R2D and D2R, respectively. - R2D and D2R signal(s) - Multiplexing/multiple access in R2D is by only TDMA, and in D2R is by only TDMA and FDMA. - R2D supports only OOK-4 modulation, one solution for CP handling. D2R backscattering supports only OOK and BPSK modulations. - R2D transmission supports only the Manchester line code in TR 38.769 - D2R transmission supports: - Either the Manchester line code in TR 38.769 or no line code (one to be down-selected); and - A corresponding small frequency shift method according to the options in TR 38.769. - R2D does not support FEC. D2R supports only convolutional code with generator polynomials as per TS 36.212 (unless RAN1 decides to use other generator polynomials by RAN1#120bis). - PRDCH and PDRCH both support transmission without CRC, and with CRC as per the generator polynomials in TS 38.212 (unless RAN1 decides to use other generator polynomials by RAN1#120bis) for 6-bit CRC and 16-bit CRC. Cases to use which length of CRC, or no CRC, to be decided in RAN1. - D2R supports physical layer repetition transmission. R2D does not support physical-layer repetition transmission.

# Discussion

## R2D

### M values

In the SI phase, RAN1 discussed the potential values of M and agreed a table, which is captured in TR 38.769. Also, RAN1 discussed the granularity of the transmission bandwidth Btx,R2D.

The study defines the following bandwidths for R2D: - Transmission bandwidth, Btx,R2D from a reader perspective: The frequency resources used for transmitting R2D. For an OFDM-based waveform with subcarrier spacing of 15 kHz, Btx,R2D ≤ twelve PRBs, and would be down-selected among: - Alt 1: Including 180 kHz, 360 kHz - Alt 2: Integer multiple(s) of 180 kHz - Alt 3: Integer multiple(s) of the subcarrier spacing - Occupied bandwidth, Bocc,R2D from a reader perspective: The frequency resources used for transmitting R2D, and potential guard band. - Bocc,R2D ≥ Btx,R2D. Table 6.1.1.4-1 is a starting point for study of M values and the associated minimum Btx,R2D value. The reader can use any transmission bandwidth greater than or equal to the minimum Btx,R2D value. Note: Depending on further study, the maximum value of M may be less than 32. Note: The performance can be better when transmission bandwidth greater than the minimum Btx,R2D, depending on device processing and transmit power constraint. Table 6.1.1.4-1: Starting point for M values and the associated minimum Btx,R2D value M; Minimum Btx,R2D # of PRBs. 1; 1. 2; 1. 4; 1. 6; 1. 8; 2. 12; 2. 16; 2. 24; 3. 32; 4.

In the previous meeting, we made the following agreement for the transmission bandwidth and the maximum value of M.

Agreement

RAN1 does not specify the exact bandwidth a reader shall use for Btx,R2D.

- For R2D, specify the minimum Btx,R2D # of PRBs associated to each M value.

- For R2D, whether to specify the maximum Btx,R2D # of PRBs is up to RAN4

- Note: Reader can transmit any Btx,R2D subject to this minimum Btx,R2D and the agreed 15 kHz subcarrier spacing, and subject to other limitations that are up to RAN4.

Agreement

For R2D, for the OOK-4 modulation for M-chip per OFDM symbol transmission:

- The maximum M value is no less than 16, and to be down-selected from 32, 24 or 16 at RAN1#120bis.

To determine the maximum value of M, RAN1 should first consider the maximum data rate required for A-IoT data communications. This value also impacts other design aspects, such as the CP handling method, which varies based on company preferences. As a result, it has been challenging to make progress on both the maximum value of M and CP handling simultaneously. Thus, when determining the maximum value of M, RAN1 should prioritize maximizing the data rate.

Proposal 1: RAN1 should decide the maximum value of M solely based on the required maximum data rate. E.g., M=24 would provide comparable performance to RFID.

In addition, RAN1 also should decide which M values will be supported. RAN1 have discussed this, which is in FL summary (R1-2501409). It would be unnecessary to support all M values listed in the table 6.1.1.4-1, which was captured as a starting for discussions. We are not sure if readers really need full flexibility such as supporting two adjacent M values such as 6 and 8. Given that we are only considering device type 1, rather than supporting full flexibility, it would be better to design to decrease false detection rate of M values at the device. Assuming the maximum value is 24, we propose 2, 6, 12, and 24 as a set of the supporting M values.

Proposal 2: RAN1 supports a subset of M values listed in Table 6.1.1.4-1 of TR 38.769. E.g., M= 2, 6, 12, and 24.

In this meeting, at least RAN1 should agree how many M values will be supported, but the M values may affect other design features, so it is desirable to decide it in this meeting.

### CP handling

RAN1 identified the following potential solutions as follows:, which is from TR 38.769.

- Method Type 1: Removal of CP at device without specified transmit-side. For Method Type 1, two ways that CP location/length can be determined are studied: - Alt M1-1: Device assumes same CP length for each OFDM symbol, i.e. does not distinguish exact CP length among different OFDM symbols - Alt M1-2: Duration between transition edges is utilized by device to determine CP location/length, i.e. if the duration appears to be invalid based on known chip duration - Method Type 2: Ensure the CP insertion of OFDM-based waveform will not introduce false rising/falling edge between the last OOK chip in OFDM symbol (n-1) and the first OOK chip in OFDM symbol n. For Method Type 2, two approaches regarding subcarrier orthogonality are studied: - Alt M2-1: Method Type 2 retains subcarrier orthogonality, i.e. CP is copied from the end of an OFDM symbol. - Alt M2-1-1: The first OOK chip(s) and the last OOK chip(s) in an OFDM symbol are the same. - Alt M2-1-2: Ensure a transition edge occurs only at the start or only at the end of the CP, and no transition edge occurs during the CP. - Alt M2-2: Method Type 2 does not retain subcarrier orthogonality.

In the previous meeting, RAN1 discussed this CP-handling issue. Based on the study outcome, RAN1 made a further progress at RAN1 #120 meeting as follows:

Agreement

For CP handling which retains subcarrier orthogonality, the candidate methods are clarified as follows:

- For Method Type 1 (as per TR 38.769)

  - For purposes of evaluating Method Type 1, it is assumed the device is aware of or determines the boundary of an OFDM symbol using the R2D timing acquisition signal (R-TAS), at least for Alt M1-1

    - Detail is up to R2D preamble design

  - Using Alt M1-1 and/or Alt M1-2 is up to device implementation as per TR 38.769

  - FFS: applicable M values

- For Method Type 2 Alt M2-1 (as per TR 38.769)

  - Detail design of Alt M2-1-1 or Alt M2-1-2 from the followings:

    - Alt M2-1-1

      - Candidate 1:

        - For M less than 6: Use Alt M1-1

        - For M >=6: circular shift of the M chips’ samples before CP insertion at the reader side in addition to bit/chip skipping/dropping at the device side after bit decoding

      - Candidate 2:

        - Where a known signal shape in the tail-portion of the R2D signal is copied to the CP.

      - Candidate 3:

        - Insert padding chips at both start and end OOK chips or only at end OOK chips

    - Alt M2-1-2

      - Candidate 4:

        - Change a=b≠c case to a=b=c case by changing OOK-4 M value to adjacent M +/- 1 for OFDM symbol n.

          - where a, b and c is the last chip of symbol n, first chip of symbol n+1 and last chip of symbol n+1.

      - Candidate 5:

        - Odd number of chips of PRDCH is transmitted in the first OFDM symbols, while M value is kept unchanged during the remaining OFDM symbols.

  - FFS: applicable M values with or without combination of Method Type 1

For CP handling which does not retain subcarrier orthogonality, the candidate methods are clarified as follows:

- For Method Type 2 Alt M2-2 (as per TR 38.769)

  - FFS detail design of Alt M2-2 from the following:

    - Candidate 1:

      - A duration of OFDM symbol with CP has integer number of OOK symbols. CP is not copied from the end of OFDM symbol. Total 14 OFDM symbols fit in a slot.

For further down-selection among candidate methods which retains subcarrier orthogonality, companies are encouraged to provide evaluation (including overhead) among the followings to RAN1#120bis:

- Method Type 1

- Method Type 2 Alt M2-1-1 with or without combination of Method Type 1

- Method Type 2 Alt M2-1-2 with or without combination of Method Type 1

- Note: The evaluation for CP handling at least includes a M value from {2,4,6,8}, a M value from {12, 16}, and a M value from {24, 32}

- Note: In this evaluation, the transmit power of the ON chips remains constant

In our understanding, RAN1 identified three distinct categories for handling the CP. For Method type 1, the impact on the specification would be relatively limited. In particular, for Alt M1-1, the device simply assumes a uniform CP duration across OFDM symbol(s), which may not require explicit specification support.

Method Type 2, which aims to retain orthogonality, includes multiple candidate approaches. However, the primary challenge in selecting a specific candidate appears to be the limited applicability of each option, especially when considering varying values of M or non-constant M across slots.

Candidate 1 and Candidate 2 of Method type 2 are not standalone solutions, as they inherently rely on Method Type 1. For Candidate 1, we should use Alt M1-1 for M<6. Candidate 2 copies the time-domain signals of the end of a symbol to the CP part. The device should be able to detect the symbol boundary and extract the samples of the signals in the CP part. Furthermore, alternating the value of M across slots would be an unfavorable implementation choice for both the AIoT devices and the gNB.

The simplest approach to maintaining orthogonality is to append one or more padding chips at the end of a symbol and copy chips from near the end of the symbol into the CP region. Depending on the chip definition, the duration of the padding chip may differ from that of a regular chip. For example, consider a scenario where the CP duration is longer than a single chip. In this case, a padding chip—set to a fixed value, such as '1'—can be inserted at the end of the symbol. The CP portion of the following symbol would then consist of one data chip (e.g., an on-chip) followed by the padding chip, as illustrated in Figure 1. Similarly, the previous symbol should also include a padding chip at the end of the previous symbol, which may likewise be padded with a value of '1'.

The device behaviour would proceed as follows: if an A-IoT device detects an On-chip or Off-chip between two padding chips, it determines that the data chip lies within the CP region and discards it—along with the surrounding padding chips—prior to data decoding. The resulting data rate loss due to CP handling is minimal, typically involving only the last chip of the symbol. Furthermore, the device does not need to estimate the symbol boundary or extract specific samples from the CP region, thereby simplifying implementation.

Figure 1. An illustrative example of CP-handling method

The AIoT device can differentiate the detected chips as a chip used for data communication (i.e., On-chip or Off-chip) and a padding chip.

If the time duration of the CP region is less than or equal to a single chip duration, the device will detect the padding chip and discard it accordingly. In cases where the CP duration exceeds one chip, the device discards both the padding chips and any detected On-chip or Off-chip located between two padding chips—where one padding chip is copied from the current symbol and the other from the previous symbol.

Based on the above, we have the following proposal:

Proposal 3: For CP handling, RAN1 supports insertion of padding chip at the end of the symbol and then copy one or more chips including data chip to the CP part, and support the following device behaviour: - In case the time duration of CP part is less than or equal to a chip duration, the device will detect the padding chip and discard it accordingly. - In cases where the CP duration exceeds one chip, the device discards both the padding chips and any detected On-chip or Off-chip located between two padding chips—where one padding chip is copied from the current symbol and the other from the previous symbol. The R2D chip length should be the same, but the padding chip duration may or may not be the same as the on/off-chip depending on design principles. The padding chip shouldn’t be detected by the device as a R2D chip for data transmission.

## D2R

### OOK/BPSK modulation

For D2R modulation, RAN1 made the following agreements:

Agreement

For D2R BPSK modulation

- After line coding or square wave, chip “1” is mapped to the real-valued modulation symbol {1} and chip “0” is mapped to the real-valued modulation symbol {-1} in the baseband.

Agreement

For D2R OOK modulation

- After line coding or square wave, chip “1” is mapped to the real-valued modulation symbol {1} and chip “0” is mapped to the real-valued modulation symbol {0} in baseband.

Agreement

For a D2R transmission, it is up to device implementation to use OOK or BPSK modulation. It is assumed that a device operates consistently with the same D2R modulation in the same inventory/command round.

When we were discussing the issue related to the last above agreement, we had a discussion on the necessity of the demodulation scheme information at the reader. Some companies mentioned the receiving method at the reader is the same between OOK and BPSK, which does not impact reader’s operation, so the reader does not need the information on which modulation scheme is used. First, OOK demodulation can be done by envelop detection, but the BPSK cannot be demodulated by the same way.

- The OOK signal can be represented by st=A⋅ xt⋅cos2πfct,  where xt=1 or 0,

- The BPSK signal can be represented by st=A⋅ cos2πfct+ϕt, where  ϕt=π or 0.

OOK modulation symbols can be demodulated using simple envelope detection, whereas BPSK requires phase-based demodulation process. The reader may initially assume the received signal uses OOK and attempt the simple envelope detection. However, this will fail if the actual signal is BPSK. The reader may first perform envelope detection over a sufficient number of modulation symbols. If the observed amplitude remains relatively constant, the reader may infer that the signal is BPSK and subsequently switch to a phase-based demodulation method. This raises several design considerations: How long should the reader continue envelope detection before determining the modulation scheme? What happens if the transmitted data consists of consecutive identical bits (e.g., a series of 0s or 1s), potentially masking amplitude variation at the start? Can it be guaranteed that a device will continue to use the same modulation scheme for the duration of its connection to a given reader?

The unknown information of modulation scheme affects receiving operation at the reader side. It is unclear how often the reader should attempt to detect which modulation scheme is used for a given AIoT device.

Given the potential for many devices to be connected to a reader, the reader should at least be able to indicate A-IoT devices to use a specific modulation scheme—either BPSK or OOK. If a device does not support the indicated modulation scheme, it may continue using its supported scheme; however, the reader would then be aware of this behavior.

This approach eliminates the need for the reader to always perform envelope detection first to determine the modulation scheme used by the device. Considering the possibility of numerous devices and simultaneous transmissions from multiple FDMed devices, it is preferable for the reader to know the modulation scheme in advance to avoid unnecessary processing. Additionally, if a device supports both modulation schemes, it should not switch between them without notification or explicit indication. Requiring the reader to estimate the modulation scheme used by a device each time it connects would not constitute good system design.

The receiving operation at the reader differs depending on whether the modulation scheme indication feature is supported.

Proposal 4: Support reader-side indication of the modulation scheme - either OOK and BPSK – to the AIoT devices.

# Conclusion

In this contribution we have made the following observations and proposals:

Observation 1:	The AIoT device can differentiate the detected chips as a chip used for data communication (i.e., On-chip or Off-chip) and a padding chip.

Observation 2:	The unknown information of modulation scheme affects receiving operation at the reader side. It is unclear how often the reader should attempt to detect which modulation scheme is used for a given AIoT device.

Observation 3:	The receiving operation at the reader differs depending on whether the modulation scheme indication feature is supported.

Proposal 1: RAN1 should decide the maximum value of M solely based on the required maximum data rate. E.g., M=24 would provide comparable performance to RFID.

Proposal 2: RAN1 supports a subset of M values listed in Table 6.1.1.4-1 of TR 38.769. E.g., M= 2, 6, 12, and 24.

Proposal 3: For CP handling, RAN1 supports insertion of padding chip at the end of the symbol and then copy one or more chips including data chip to the CP part, and support the following device behaviour: - In case the time duration of CP part is less than or equal to a chip duration, the device will detect the padding chip and discard it accordingly. - In cases where the CP duration exceeds one chip, the device discards both the padding chips and any detected On-chip or Off-chip located between two padding chips—where one padding chip is copied from the current symbol and the other from the previous symbol. The R2D chip length should be the same, but the padding chip duration may or may not be the same as the on/off-chip depending on design principles. The padding chip shouldn’t be detected by the device as a R2D chip for data transmission.

Proposal 4: Support reader-side indication of the modulation scheme - either OOK and BPSK – to the AIoT devices.

# References

- RP-250796, ” New WID on Rel-19 Ambient IoT”, RAN#107

- TR 38.769, “Study on solutions for Ambient IoT (Internet of Things) in NR”