3GPP TSG RAN WG1 Meeting #120bis	R1-2501780

Wuhan, China, April 7th – 11th, 2025

Source:	ZTE Corporation, Sanechips

Title:	Discussion on enhancements for UE-initiated/event-driven beam management

Agenda Item:	9.2.1

Document for:	Discussion and Decision

- Introduction

During RAN#105 meeting, a revised Rel-19 WID on NR MIMO phase 5 was approved as in [1]. Then, the following objective is identified for UE-initiated beam management.

- Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting

In this contribution, we discuss potential methods targeting for reducing overhead and latency of the UE-initiated beam report.

- UL signaling container

  - First PUCCH

In RAN1#120 meeting, the following agreement on multiplexing/dropping rule of first PUCCH was reached [2].

Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the multiplexing/dropping rule(s) of 1-bit first PUCCH, down-select one of the following rule for the Case-2: the 1-bit first PUCCH is collided/overlapped with a PUSCH - Option-1: Prioritize first PUCCH over PUSCH, i.e., PUSCH is dropped. - Option-3: Piggyback 1-bit indication of first PUCCH into the PUSCH. - FFS: If the PUSCH should be with UL-SCH or not for UEI beam report - Option-4: Reuse the SR dropping rules, i.e., the first PUCCH is dropped. - FFS: whether/how to handle the case of different PHY priorities.

Issue-1: First PUCCH transmission based on prohibit timer

To avoid too frequent UEI beam reporting and resource requests, a prohibit timer should be supported for the first PUCCH.

- In Mode-A, given that the second PUSCH to accommodate UEI beam reports is scheduled by the DCI, it is vital to enable continuous transmissions of the first PUCCH to request the second PUSCH before UE transmits the beam report in the second PUSCH. In particular, the report latency can be reduced in such way in case that the first PUCCH is not received by the network.

- In Mode-B, UE can always identify a valid transmission occasion of the second PUSCH following the first PUCCH transmission. However, since the second PUSCH may be dropped caused by collision handling, the time gap between the valid transmission occasions of the first PUCCH and second PUSCH may be quite large. To mitigate such impact, it is needed to start the prohibit timer after the second PUSCH transmission.

Proposal 1: On beam report transmission procedure for UE-initiated/event-driven beam reporting in both Mode-A and Mode-B, support to introduce a prohibit-timer for first PUCCH.

- In the case that triggering-event associated with a CSI report configuration is determined, if the prohibit timer is NOT running, the UE can transmit first PUCCH.

  - At the first symbol after the end of the second PUSCH transmission, the UE starts the prohibit timer.

Issue-2: UCI multiplexing/dropping rule for the first PUCCH

Multiplexing/dropping rule(s) of 1-bit first PUCCH are considered as follows.

- Case-2: The 1-bit first PUCCH is collided/overlapped with a PUSCH.

    - Unlike the functionality of legacy SR that is to request uplink resource for data transmission so that it is dropped when collided with a PUSCH with UL-SCH, it is proper to multiplex the 1-bit indication (i.e., beam report indication, BRI) of the first PUCCH which is to notify or request resources for UEIBR in the PUSCH for the sake of report latency reduction.

      - Given that the collided/overlapped PUSCH and the 1-bit first PUCCH cannot be associated with the same CSI report configuration anyways (which can be ensured by NW scheduling/configuration in both Mode-A and Mode-B), the case mentioned in the FFS part in Option-3 cannot be occurred accordingly.

Proposal 2: Regarding the multiplexing/dropping rule(s) of 1-bit first PUCCH for the Case-2 that the 1-bit first PUCCH is collided/overlapped with a PUSCH, support Option-3 that piggyback 1-bit indication of first PUCCH into the PUSCH.

- Case-3: Multiple 1-bit first PUCCHs corresponding to different CSI configurations for UE-initiated/event-driven beam reporting are overlapping in the time domain.

    - It is preferred to be up to UE implementation to handle such collision in this case, e.g., following the legacy rule for resolving collisions between multiple SRs.

Proposal 3: Regarding the multiplexing/dropping rule(s) of 1-bit first PUCCH for the Case-3 that multiple 1-bit first PUCCHs corresponding to different CSI configuration for UE-initiated/event-driven beam reporting are overlapping in the time domain, support that it is up to UE implementation to transmit only one of the 1-bit first PUCCHs.

- Case-4: The 1-bit first PUCCH is collided/overlapped with a PUCCH format 0/1 carrying HARQ-ACK information.

    - By referring to the legacy multiplexing rule of SR, the multiplexing of 1-bit indication and HARQ-ACK with no more than 2 bits depends on the PUCCH format of the 1-bit first PUCCH.

      - PUCCH format 0: The 1-bit first PUCCH is transmitted to carry the HARQ-ACK information additionally, and the mechanism on the change of cyclic shift values can be reused for multiplexing the 1-bit indication and HARQ-ACK information bits.

      - PUCCH format 1: The 1-bit first PUCCH is transmitted to carry the HARQ-ACK information only, and sequence for the first PUCCH transmission is determined based on the HARQ-ACK information.

Proposal 4: Regarding the multiplexing/dropping rule(s) of 1-bit first PUCCH for the Case-4 that the 1-bit first PUCCH is collided/overlapped with a PUCCH format 0/1 carrying HARQ-ACK information, support that the PUCCH resource of the 1-bit first PUCCH is to carry the multiplexing of the 1-bit indication and the HARQ-ACK information via cyclic shift when using PUCCH format 0, and the PUCCH resource of the 1-bit first PUCCH is to carry the HARQ-ACK information only when using PUCCH format 1.

- Case-5: The 1-bit first PUCCH is collided/overlapped with a PUCCH format 2/3/4 carrying HARQ-ACK or legacy CSI.

    - Similar to the multiplexing rule specified for scenarios when an SR overlaps with a PUCCH format 2/3/4 carrying HARQ-ACK and/or legacy CSI, the 1-bit indication should be multiplexed with the HARQ-ACK information and/or legacy CSI report.

    - Consequently, by reusing the multiplexing rules for SR, the 1-bit first PUCCH should be multiplexed with HARQ-ACK and/or legacy CSI via a UCI bit sequence.

      - In legacy, the UCI bit sequence for SR is generated based on ascending order of the values of SR-ID associated with the SRs, and it indicates a positive/negative SR among the SRs. UE transmits PUCCH for SR in the corresponding PUCCH resource only when the SR is positive.  Based on the following agreement, the 1-bit indication should be encoded by reusing the encoding mechanism of positive/negative SR, hence UE should transmit PUCCH for BRI in the corresponding PUCCH resource only when the BRI is positive.

      - However, since first PUCCH is NOT associated with a SchedulingRequestId, the UCI bit sequence should be generated by reusing the rule for SR with the following modification. The UCI bit sequence can be generated based on a first ascending order of the values of SR ID associated with the SRs and a second ascending order of the values of PUCCH resource ID associated with the first PUCCHs, and the UCI bit sequence can indicate a positive/negative SR or BRI among SRs and BRIs which are multiplexed with HARQ-ACK and/or legacy CSI.

Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding first PUCCH channel configuration, down-select one of Alt-1 and Alt-2 for both mode-A and mode-B. - Alt-1 (dedicated SR): Introduce RRC parameter, e.g., reportResourceRequest-UEIBR/reportNotification-UEIBR, corresponding to the one-bit indication in the first PUCCH channel - The RRC parameter is associated with the dedicated SchedulingRequestId. - Note: The detailed signaling is up to RAN2. - FFS: whether/how to enhance legacy SR to support notification of beam report transmission in Mode-B - Alt-2 (new UCI type): Introduce RRC parameter, e.g., firstPUCCHResourceConfig- UEIBR, for the periodic PUCCH resource configuration. - Note: The RRC parameter is NOT associated with SchedulingRequestId. - 1-bit to PUCCH resource is encoded by reusing the encoding mechanism of positive/negative SR. - The dedicated RRC parameter at least comprises the following: - periodicityAndOffset - PUCCH-ResourceID - Above applies at least for the single CC case. - Reuse multiplexing/dropping rule(s) of SR as baseline - FFS: overlapping with SR/LRR or PUSCH Note: Further details on first PUCCH retransmission for mode A and mode B will be separately discussed.

- Consequently, we have the following proposal and text proposal.

Proposal 5: Regarding the multiplexing/dropping rule(s) of 1-bit first PUCCH for the Case-5 that the 1-bit first PUCCH is overlapped with a PUCCH format 2/3/4 carrying HARQ-ACK information and/or CSI report(s), support to multiplex the 1-bit indication with the HARQ-ACK information and/or legacy CSI report via a UCI bit sequence.

- The UCI bit sequence generation reuses the legacy rule for SR with the following modification.

  - The UCI bit sequence is generated based on a first ascending order of the values of SR ID associated with the SRs and a second ascending order of the values of PUCCH resource ID associated with the first PUCCHs.

- The UCI bit sequence indicates a positive/negative SR or BRI.

Text Proposal 1: To adopt the following changes in section 9.2.5.1 in TS 38.213.

- 9.2.5.1 UE procedure for multiplexing HARQ-ACK or CSI and SR in a PUCCH In the following, a UE is configured to transmit K PUCCHs for respective K SRs in a slot, as determined by a set of schedulingRequestResourceId, a schedulingRequestResourceId associated with schedulingRequestID-BFR-SCell, a schedulingRequestResourceId associated with schedulingRequestID-BFR, a schedulingRequestResourceId associated with schedulingRequestID-BFR2 if the UE provides twoLRRcapability, and a schedulingRequestResourceId associated with schedulingRequestID-LBT-SCell, with SR transmission occasions that would overlap with a transmission of a PUCCH with HARQ-ACK information from the UE in the slot or with a transmission of a PUCCH with CSI report(s) from the UE in the slot. A UE is configured to transmit M PUCCHs for respective M BRIs in a slot, as determined by a set of pucch-ResourceId associated with firstPUCCHResourceConfig-UEIBR, with BRI transmission occasions that would overlap with a transmission of a PUCCH with HARQ-ACK information from the UE in the slot or with a transmission of a PUCCH with CSI report(s) from the UE in the slot. <Irrelevant part is omitted> If a UE would transmit a PUCCH with OACK HARQ-ACK information bits in a resource using PUCCH format 2 or PUCCH format 3 or PUCCH format 4 in a slot, as described in clauses 9.2.1 and 9.2.3, log2K+M+1 bits representing a negative/positive SR or BRI, firstly in ascending order of the values of schedulingRequestResourceId, a schedulingRequestResourceId associated with schedulingRequestID-BFR-SCell, a schedulingRequestResourceId associated with schedulingRequestID-BFR, a schedulingRequestResourceId associated with schedulingRequestID-BFR2 if the UE provides twoLRRcapability, and a schedulingRequestResourceId associated with schedulingRequestID-LBT-SCell, and secondly in ascending order of the values of pucch-ResourceId associated with firstPUCCHResourceConfig-UEIBR, are appended to the HARQ-ACK information bits and the UE transmits the combined OUCI=OACK+log2K+M+1 UCI bits in a PUCCH using a resource with PUCCH format 2 or PUCCH format 3 or PUCCH format 4 that the UE determines as described in clauses 9.2.1 and 9.2.3. If one of the SRs is a positive LRR, the value of the log2K+M+1 bits indicates the positive LRR. If one of the BRIs is a positive BRI and none of the SRs is a positive LRR, the value of the log2K+M+1 bits indicates the positive BRI. An all-zero value for the log2K+M+1 bits represents a negative SR value across all K SRs and a negative BRI across all M BRIs. If a UE would transmit a PUCCH with CSI report bits in a resource using PUCCH format 2 or PUCCH format 3 or PUCCH format 4 in a slot, log2K+M+1 bits representing corresponding negative/positive SR, firstly in ascending order of the values of schedulingRequestResourceId, a schedulingRequestResourceId associated with schedulingRequestID-BFR-SCell, a schedulingRequestResourceId associated with schedulingRequestID-BFR, a schedulingRequestResourceId associated with schedulingRequestID-BFR2 if the UE provides twoLRRcapability, and a schedulingRequestResourceId associated with schedulingRequestID-LBT-SCell, and secondly in ascending order of the values of pucch-ResourceId associated with firstPUCCHResourceConfig-UEIBR, are prepended to the CSI information bits as described in clause 9.2.5.2 and the UE transmits a PUCCH with the combined OUCI=log2K+M+1+OCSI UCI bits in a resource using the PUCCH format 2 or PUCCH format 3 or PUCCH format 4 for CSI reporting. If one of the SRs is a positive LRR, the value of the log2K+M+1 bits indicates the positive LRR. If one of the BRIs is a positive BRI and none of the SRs is a positive LRR, the value of the log2K+M+1 bits indicates the positive BRI. An all-zero value for the log2K+M+1 bits represents a negative SR value across all K SRs and a negative BRI across all M BRIs. <Irrelevant part is omitted>

  - Second PUSCH

In RAN1#118b meeting, the following agreement on the Type-1 CG PUSCH in Mode B was endorsed [3].

Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, for the case the pre-configured Type-1 CG PUSCH carry the beam report, for the second UL channel in Mode-B, at least option3 is supported: - Option-1: The same Type-1 CG PUSCH can carry UL-SCH, any other UCI, and the beam report. - Option-2: The Type-1 CG PUSCH is a dedicated type-1 CG PUSCH for carrying the beam report - Note: This PUSCH can NOT carry UL-SCH. This PUSCH can NOT carry any other UCI. - Option-3: The Type-1 CG PUSCH is a type-1 CG PUSCH for carrying the beam report - Note: This PUSCH can NOT carry UL-SCH. This PUSCH can carry any other UCI. FFS: whether Type-1 CG PUSCH can be transmitted if the pre-configured Type-1 CG PUSCH does NOT carry the beam report

Regarding the Type-1 CG PUSCH for carrying the beam report can carry any other UCI, the following clarifications needs to be reached.

  - Interpretation-1: UE multiplexes the HARQ-ACK information, 1-bit indication in first PUCCH, SR or legacy CSI report on the Type-1 CG PUSCH for carrying the beam report based on collision handling rules. UE does not multiplex a type of UCI on the Type-1 CG PUSCH for carrying the beam report if the dropping rule is applied rather than the multiplexing rule in case of collision occurs between the Type-1 CG PUSCH and the PUCCH carrying the UCI.

  - Interpretation-2: UE can always multiplex the HARQ-ACK information, 1-bit indication in first PUCCH, SR and/or legacy CSI report on the Type-1 CG PUSCH for carrying the beam report when PUCCH carrying the corresponding UCI overlaps with the Type-1 CG PUSCH.

Proposal 6: On beam report transmission procedure for UE-initiated/event-driven beam reporting, in order to clarify that the Type-1 CG PUSCH for carrying the beam report can carry any other UCI in Mode-B, support to down-select one of the following alternatives:

- Alt-1: UE multiplexes the HARQ-ACK information, 1-bit indication in first PUCCH, SR or CSI report on the Type-1 CG PUSCH for carrying the beam report based on collision handling rules.

- Alt-2: UE always can multiplex the HARQ-ACK information, 1-bit indication in first PUCCH, SR and/or CSI report on the Type-1 CG PUSCH for carrying the beam report in case of overlapping occurs.

Subsequently, the following two alternatives regarding UCI multiplexing/dropping rules for the case that Type-1 CG PUSCH for carrying the beam report can carry any other UCI should be considered.

  - Alt-1: Leverage the legacy UCI multiplexing/dropping rules as much as possible.

    - HARQ-ACK

      - When a PUCCH carrying HARQ-ACK overlaps with a PUSCH, the HARQ-ACK information is multiplexed on the PUSCH as per the legacy rules. This does not require additional specification efforts when the PUSCH is applied for carrying UEIBR.

    - BRI

      - Multiplexing BRI into the Type-1 CG PUSCH is preferred based on our analysis for Case-2 in section 2.1.

    - SR

      - By referring to the legacy rule, where UE drops PUSCH without UL-SCH in case that a PUCCH carrying positive SR overlaps with the PUSCH, UE should drop Type-1 CG-PUSCH carrying UEI beam reports without UL-SCH if the Type-1 CG-PUSCH is overlapped with a positive SR.

    - CSI

      - Based on the multiplexing rule as defined in TS 38.213 for PUSCH overlapped with PUCCH, CSI reports in the PUCCH are not multiplexed in the PUSCH unless the PUSCH does not carry CSI reports before multiplexing. Accordingly, by leveraging the legacy rule, when a PUSCH carrying a UEIBR overlaps with a PUCCH carrying CSI reports, UE does not multiplex the CSI reports from the PUCCH.

  - Alt-2: Introduce new rules for multiplexing any other UCI on the Type-1 CG PUSCH carrying UEI beam reports.

    - Except for HARQ-ACK information, new multiplexing rules are needed for multiplexing SR or P/SP CSI report in the Type-1 CG PUSCH carrying UEIBR.

    - For SR, the beta offset value for multiplexing SR in PUSCH and the mapping of the SR information on the REs of PUSCH are critical aspects that need to be discussed.

    - Regarding the P/SP CSI report in PUCCH, CSI priority value and CSI omission rule require further discussion if CSI report for UEIBR and P/SP CSI report can be multiplexed on the same UL resource. It should be noted that as per the previous agreement, if UEIBR is not carried on the second PUSCH, the UE cannot transmit the second PUSCH. In this sense, CSI priority of UEIBR carried on the Type-1 CG PUSCH can be higher than legacy CSI so that the second PUSCH cannot be dropped due to absence of UEIBR transmission.

- Overall, Alt-1 should be supported without introducing additional multiplexing/dropping rules. And we have the following proposal and text proposal.

Proposal 7: On beam report transmission procedure for UE-initiated/event-driven beam reporting, for the case the pre-configured Type-1 CG PUSCH carry the beam report and other UCI in Mode-B, support to leverage the legacy UCI multiplexing/dropping rules as much as possible, i.e.,

- UE can multiplex HARQ-ACK information and BRI in the pre-configured Type-1 CG PUSCH carrying the beam report.

- UE does not transmit the Type-1 CG-PUSCH carrying UEI beam reports when a PUCCH carrying positive SR overlaps with the Type-1 CG-PUSCH.

- UE does not multiplex CSI reports from the PUCCH in the pre-configured Type-1 CG PUSCH carrying the beam report.

Text Proposal 2: To adopt the following changes in section 9 in TS 38.213.

- 9 UE procedure for reporting control information <Irrelevant part is omitted> If a UE - would multiplex UCI in a PUCCH transmission that overlaps with a PUSCH transmission, and - the PUSCH and PUCCH transmissions fulfil the conditions in clause 9.2.5 for UCI multiplexing, the UE - multiplexes only HARQ-ACK information and BRI, if any, from the UCI in the PUSCH transmission and does not transmit the PUCCH if the UE multiplexes aperiodic, semi-persistent CSI reports or UEI-CSI reports in the PUSCH; - multiplexes only HARQ-ACK information, BRI and CSI reports, if any, from the UCI in the PUSCH transmission and does not transmit the PUCCH if the UE does not multiplex aperiodic, semi-persistent CSI reports or UEI-CSI reports in the PUSCH. <Irrelevant part is omitted>

  - Multiple CSI report configurations associated with one first PUCCH resource

In RAN1#120 meeting, the following agreement on one or multiple CSI report configurations being associated with one first PUCCH was endorsed [2].

Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, one PUCCH resource of first PUCCH can be associated with one or multiple CSI report configurations, regarding Event-2. - Only a single UEI beam report is carried in the second PUSCH. - Additional indication of one ‘CSI report configuration’ is provided in the report format. - The CSI report configurations associated with the same PUCCH resource are ordered in ascending order of corresponding CSI-ReportConfigId, the number of bits of the additional indication field is ceil(log2(N_CSIconfig)), where the N_CSIconfig denotes the number of CSI report configurations associated with the same PUCCH resource. - The payload size of the single UEI beam report is determined according to the max payload size among the associated CSI report configurations. - Zero-padding can be appended if the payload size of the UEI beam report is less than the max report payload. - The reported UEI beam report should satisfy the triggering condition. - If multiple UE initiated beam report procedures occur, down-select one of the following options: - Option-1: It is up to UE implementation to select one of configuration. - Option-2: The UEI beam report with highest priority is reported - Option-3: The report triggered in the latest measurement is reported in PUSCH - The multiple CSI report configurations associated with the one first PUCCH resource should be configured in the same CC. - Working Assumption: For Mode-A, the multiple CSI report configurations associated with the same PUCCH resource should be associated with a same CSI-AperiodicTriggerState. - For Mode-B, the multiple CSI report configurations associated with the same PUCCH resource should be associated with the same second configured PUSCH - FFS: Alt-2 Multiple UEI beam reports associated with the same PUCCH resource for first PUCCH can be transmitted in the second PUSCH. - If RAN1 cannot converge on the support of Alt-2 in RAN1#120bis, this alternative will be dropped from Rel-19

In scenarios where payload size of multiple CSI reports exceeds the capacity of a PUCCH or PUSCH, UE determines to transmits partial CSI reports in descending order of CSI priority. Similarly, the single beam report carried in the second PUSCH should have the highest priority among the multiple triggered UEI beam reports.

Regarding the determination of priority value of a UEI beam report, it is preferred to utilize a new formula independent to the one defined for legacy CSI reports. Except for serving cell index and CSI report configuration index, CSI report type (i.e., AP, SP or P) and CSI report contents (i.e., carrying L1-RSRP/L1-SINR or not) considered in the legacy formula are not needed for UEI beam reports.

In addition, as per discussion on the multiplexing of second PUSCH and other UCI in sub-section 2.2, it is suggested not to multiplex P or SP CSI reports from PUCCH in the second PUSCH. Consequently, it is unnecessary to discuss priority issues between UEI CSI reports and legacy CSI reports. Hence, an independent formula to determine the priority of UEI beam report should be considered. The corresponding proposal and text proposal are as follows.

Proposal 8: On beam report transmission procedure for UE-initiated/event-driven beam reporting, in case that one PUCCH resource of first PUCCH can be associated with one or multiple CSI report configurations and multiple UEI beam reports occur, support that only a single UEI beam report with the highest priority is carried in the second PUSCH (Option 2).

- The priority of a UEI beam report is determined based on a formula independent to the formula defined for legacy CSI reports.

Text Proposal 3: To adopt the following changes in section 5.2.5 in TS 38.214.

- 5.2.5 Priority rules for CSI reports <Irrelevant part is omitted> CSI reports are associated with a priority value PriiCSIy,k,c,s=2∙Ncells∙Ms∙y+Ncells∙Ms∙k+Ms∙c+s where - for aperiodic CSI reports to be carried on PUSCH for semi-persistent CSI reports to be carried on PUSCH, for semi-persistent CSI reports to be carried on PUCCH and for periodic CSI reports to be carried on PUCCH; - for CSI reports carrying L1-RSRP or L1-SINR and for CSI reports not carrying L1-RSRP or L1-SINR; - c is the serving cell index and Ncells is the value of the higher layer parameter maxNrofServingCells; - for a CSI report configured with ltm-CSI-ReportConfig, c is the serving cell index value where the report configuration is configured. - s is the reportConfigID and is the value of the higher layer parameter maxNrofCSI-ReportConfigurations. - for a CSI report configured with ltm-CSI-ReportConfig, s is the ltm-CSI-ReportConfigId and Ms is the value of the higher layer parameter maxNrofLTM-CSI-ReportConfigurations UEI-CSI reports are associated with a priority value PriiCSIc,s=Ms∙c+s where - c is the serving cell index where the report configuration is configured. - s is the reportConfigID and is the value of the higher layer parameter maxNrofCSI-ReportConfigurations. <Irrelevant part is omitted>

- Cross-CC beam report

In RAN1#120 meeting, the following agreement on cross-CC beam report was endorsed [2].

Agreement On cross-CC beam report measurement for UE-initiated/event-driven beam reporting, regarding Event-2, the following is supported - The current beam RS and new beam RS(s) are in the same CC - The above does NOT imply to preclude the cross-CC TCI indication. - The CC on which the indicated TCI state is applied, and the CC in which new beam RS(s) are can be the same or different. - FFS: Whether/how to introduce an RRC parameter to indicate the CC on which the indicated TCI state is applied.

When the CC on which the indicated TCI state is applied and the CC in which new beam RS(s) are from can be different, the CC in which new beam RS(s) are from is indicated by carrier indication and the CC on which the indicated TCI state is applied should be indicated by a new RRC parameter.

Proposal 9: On cross-CC beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Event-2, support to introduce new RRC parameter to indicate the CC in which the indicated TCI state associated with the CSI reporting configuration is applied.

For same-CC beam reporting, since triggering conditions of Event-1, Event-2 and Event-7 are inherently different, multiple CSI report configurations for UEIBR in a given CC should be associated with different trigger event types. For cross-CC beam reporting, UEI beam reporting for a CC can be configured in another CC, and then CSI report configurations in a given CC may be associated with same or different trigger event types. For example, UEIBR for CC#1 depending on Event-2, UEIBR for CC#1 depending on Event-7 and UEIBR for CC#2 depending on Event-2 can be configured in CC#1.

Proposal 10: On cross-CC beam report transmission procedure for UE-initiated/event-driven beam reporting, support that CSI report configurations in a given CC can be associated with same or different trigger event type(s).

- Trigger event detection

In RAN1#120 meeting, the following agreements regarding trigger event determination were endorsed [2].

Agreement Regarding triggering event determination for Event 2, at least Candidate #2 is supported for resetting the counting. - Candidate#1: RS reconfiguration/update or MAC-CE signaling (if supported) for new beam is received; - FFS: whether to reset the counting for all new beams. - FFS: whether to maintain the counting whose new beam is NOT updated. - Candidate#2: [The measured current beam based on] indicated TCI state is updated; - In such case, the UE need to reset the counting for all new beams. - Candidate#3: UEI beam report is transmitted; - FFS: Only reset the counting of new beams fulfilling triggering condition and reported by the UEI beam report - Candidate#4: NW response (e.g., DCI in step-2 of Mode-A) is detected. - Candidate#5: The time window expires - Candidate#6: The threshold for event evaluation is re-configured by RRC signaling - (FFS) Candidate#7: The RRC parameter(s) associated with the CSI report configuration for UEI beam report is reconfigured. - FFS: RRC parameter(s) - FFS: Other candidates Note: Whether this proposal is captured in RAN1 or RAN2 is a separate discussion point. Agreement Regarding triggering event determination for Event 2, on the measurement window for initiating the UE-initiated/event-driven beam reporting procedure, further study the following options for possible down-selection - Option-1: The measurement window is from T_PUCCH – T_proc – T_window to T_PUCCH – T_proc, where T_PUCCH is a transmission occasion of a first PUCCH, and T_proc is RRC configured. - Option-2: The measurement window is from T_Instance – T_window to T_Instance, where T_Instance is an evaluation occasion of event instance, and T_proc is RRC configured. - The UEI beam report in the second PUSCH is based on the most recent measurement for new/current beam RS(s). - Option-3: The length, slot offset and periodicity of a measurement window are configured per CSI report configuration by NW. - Option-4: If an Event-2 instance for a new beam is obtained at the time t, UE (re)starts the timer for the new beam, where the expiry time of the timer is equal to the NW-configured length of the time window (T_window) - Note: T_window is the agreed time window parameter for measurement. - Note: Other options are not precluded.

Reset the counting of trigger event instances

Besides Candidate#2, the other six candidates in the agreement to reset the counting of trigger event instances for a given UEIBR configuration should be discussed as well.

Given that the definition of the time window remains ambiguity, Candidate#5 should be postponed until the time window functions are specified as a timer in MAC layer. Candidate#3 and #4 should be deprioritized as they are relevant to the beam reporting procedure rather than the counting and the time window.

Candidate#1 and Candidate#6 can be considered as a subset of Candidate#7. In the case of reconfiguration of RRC parameters, measurement condition of the trigger event is changed, and resetting the counting ensures a reliable evaluation procedure. Therefore, Candidate#7 is preferred, and the RRC parameter(s) at least should comprise RS configuration for new beam and the threshold for event evaluation.

Additionally, for the sake of a simple and unified solution, when UE receives RS reconfiguration/update for new beams, resetting the counting of all new beams is preferred regardless of whether they are updated by the RS reconfiguration or not.

Proposal 11: Regarding triggering event determination within a time window for Event 2, on condition(s) of resetting the counting of trigger event instances, support Candidate#7.

- Candidate#7: The RRC parameter(s) associated with the CSI report configuration for UEI beam report is reconfigured.

  - RRC parameter(s) at least comprise RS configuration for new beam and the threshold for event evaluation.

  - FFS: Other RRC parameter(s).

Measurement window for triggering event determination

Considering the previous discussion on the first PUCCH transmission is based on prohibit timer, the prohibit timer based mechanism is typically specified in RAN2 specifications. While Option-1, which associates a time window with the transmission occasion of the first PUCCH, may lead to ambiguity in the beam report procedure if they are respectively captured by RAN1 and RAN2.

In addition, if the first PUCCH is associated with multiple CSI report configurations, transmission and prohibition of the first PUCCH based on a timeline and a time window in RAN1 specifications may be complicated. In contrast, the timer based procedure performed per CSI report configuration can minimize impacts between CSI report configurations associated with the same first PUCCH. Hence, we prefer to at least support Option-4.

Proposal 12: Regarding time window triggering event determination for Event 2, on the measurement window for initiating the UE-initiated/event-driven beam reporting procedure, support Option-4.

- UL signaling content

In RAN1#120 meeting, the following agreement regarding additional indication of report format for Event-2 was endorsed [2].

Agreement On UE-initiated/event-driven beam reporting, Option-1 is supported as an extension on L1-RSRP report format depending on Event-2. - Option-1: For each of reported CRI/SSBRI, to introduce additional indication of whether the CRI/SSBRI satisfies the condition of Event-2. - The presence of this field is enabled by RRC with subjective to UE capability. - Note: The presence of this field is only for the case that N > 1 and the time window and M are configured - Above is NOT applicable for event 1. Agreement On UE-initiated/event-driven beam reporting, for Event 1, support Option-2 for RS measurement: - Option-2: RS resource set for new beam is configured in the CSI reporting configuration, and the following implicit manner for enabling one of either scheme-1 or scheme-2 is used: - If the RS(s) for new beam are CSI-RS configured in a CSI-RS resource set configured with repetition, Scheme-1 is enabled; otherwise, Scheme-2 is enabled. - In such case, the report format for Event-2 is reused unless critical technical issue is identified - FFS: How to report the RSRP value of the current beam - FFS: RSRP of the current beam is always reported

- Event-2

With regard to the additional indication of whether the CRI/SSBRI satisfies the condition of Event-2, it is needed to use two bits for the additional indication to indicate the information as concluded in Table 1.

Table 1 Additional indication by 2-bit field to identify a new beam among the reported RSs

| Codepoint | Status of the reported RS for new beam |
| --- | --- |
| {11} | - Quality of the reported RS becomes a threshold value better than the current beam - The number of Event-2 instance for the reported RS is greater than or equal to a configured number M within a time window |
| {10} | - Quality of the reported RS becomes a threshold value better than the current beam - The number of Event-2 instance for the reported RS is less than a configured number M within a time window |
| {01} | - Quality of the reported RS (i.e., L1-RSRP) does not become a threshold value better than the current beam - The number of Event-2 instance for the reported RS is greater than or equal to a configured number M within a time window |
| {00} | - Quality of the reported RS (i.e., L1-RSRP) does not become a threshold value better than the current beam - The number of Event-2 instance for the reported RS is less than a configured number M within a time window |

Proposal 13: Regarding additional indication of whether the CRI/SSBRI satisfies the condition of Event-2, support  the 2-bit field of additional indication as follows:

  - The codepoint is ‘11’ if triggering event condition is met for the reported L1-RSRP value of the corresponding RS and event instances of the corresponding RS is greater than the configured counting number.

  - The codepoint is ‘10’ if triggering event condition is met for the reported L1-RSRP value of the corresponding RS and event instances of the corresponding RS is not greater than the configured counting number.

  - The codepoint is ‘01’ if triggering event condition is not met for the reported L1-RSRP value of the corresponding RS and event instances of the corresponding RS is greater than the configured counting number.

  - The codepoint is ‘00’ if triggering event condition is not met for the reported L1-RSRP value of the corresponding RS and event instances of the corresponding RS is not greater than the configured counting number.

If the additional indication can only be 1-bit field, the corresponding field interpretation can be clarified as follows.

  - Bit value 0

    - Event instances of the corresponding RS is not greater than the configured counting number or triggering event condition is not met for the reported L1-RSRP value of the corresponding RS.

  - Bit value 1

    - Event instances of the corresponding RS is greater than or equal to the configured counting number and triggering event condition is met for the reported L1-RSRP value of the corresponding RS.

Accordingly, we have the following proposal and corresponding text proposal.

Proposal 14: Regarding additional indication of whether the CRI/SSBRI satisfies the condition of Event-2, support the following clarification if the additional indication is 1-bit field:

  - Bit value is 0 if event instances of the corresponding RS is not greater than the configured counting number or triggering event condition is not met for the reported L1-RSRP value of the corresponding RS.

  - Bit value is 1 if event instances of the corresponding RS is greater than or equal to the configured counting number and triggering event condition is met for the reported L1-RSRP value of the corresponding RS.

Text Proposal 4: To adopt the following changes in section 5.2.1.4.3 in CR38.214.

5.2.1.4.3 L1-RSRP Reporting <Irrelevant part is omitted> For L1-RSRP reporting, if the higher layer parameter nrofReportedRS in CSI-ReportConfig is configured to be one, or if the higher layer parameters nrOfReportedCells and nrOfReportedRS-PerCell are both configured to be one, the reported L1-RSRP value is defined by a 7-bit value in the range [-140, -44] dBm with 1dB step size, if the higher layer parameter nrofReportedRS is configured to be larger than one, or if the higher layer parameter groupBasedBeamReporting is configured as 'enabled', or if the higher layer parameter groupBasedBeamReporting-r17 is configured, or if the higher layer parameter groupBasedBeamReporting-r18 is configured, or if any of the higher layer parameters nrOfReportedCells and nrOfReportedRS-PerCell is configured to be larger than one, the UE shall use differential L1-RSRP based reporting, where the largest measured value of L1-RSRP is quantized to a 7-bit value in the range [-140, -44] dBm with 1dB step size, and the differential L1-RSRP is quantized to a 4-bit value. The differential L1-RSRP value is computed with 2 dB step size with a reference to the largest measured L1-RSRP value which is part of the same L1-RSRP reporting instance. The mapping between the reported L1-RSRP value and the measured quantity is described in [11, TS 38.133]. If the higher layer parameter XYZ in CSI-ReportConfig is configured, the UE shall report Condition met indicator for any of the CRI or SSBRI. Condition met indicator is set to 0 if event instances of the corresponding RS is not greater than eventInstanceCount-r19 within eventDetectionTimeWindowLength-r19 or triggering event condition is not met eventThreshold-r19 for the reported L1-RSRP value of the corresponding RS, and Condition met indicator is set to 1 if event instances of the corresponding RS is greater than or equal to eventInstanceCount-r19 within eventDetectionTimeWindowLength-r19 and triggering event condition is met eventThreshold-r19 for the reported L1-RSRP value of the corresponding RS. <Irrelevant part is omitted>

- Event-7

Different from Event-2, the TCI state codepoint indication and differential L1-RSRP value for the Q-th best activated TCI state needs to be always reported. Otherwise, gNB cannot be aware of which activated TCI state(s) can be updated after receiving the beam report. Furthermore, for the sake of effective report, it may be advisable to report activated TCI states other than only the Q-th best activated TCI state to facilitate the subsequent TCI activation/deactivation. The maximum number of activated TCI states to be reported should be same as the number of reported new beams (i.e., N).

Accordingly, the L1-RSRP report format depending on Event-7 by reusing report format for Event-2 can be concluded as in Table 2.

Table 2 L1-RSRP report format depending on Event-7

CRI or SSBRI #1

CRI or SSBRI #2

...

CRI or SSBRI #N

L1-RSRP #1

Differential L1-RSRP #2

...

Differential L1-RSRP #N

Condition met indicator #1

Condition met indicator #2

...

Condition met indicator #4

Codepoint indication for the Q-th best activated TCI state

Differential L1-RSRP for the Q-th best activated TCI state

Codepoint indication for activated TCI state #1

Codepoint indication for activated TCI state #2

Codepoint indication for activated TCI state #3

Differential L1-RSRP for activated TCI state #1

Differential L1-RSRP for activated TCI state #2

Differential L1-RSRP for activated TCI state #3

Based on the above analysis, we have the following proposal.

Proposal 15: On UE-initiated/event-driven beam reporting, regarding UL signaling content(s) of L1-RSRP report depending on Event-7, the option-3 on Event-2 is taken as a starting point with the following additional updates:

- Differential L1-RSRP value of the Q-th best activated TCI state as well as its corresponding TCI codepoint index should always be reported in addition to the N new beams.

- Other activated TCI state(s) in addition to the Q-th best activated TCI state are reported for facilitating the subsequent TCI activation/deactivation.

- Event-1

Basically, L1-RSRP value of the current beam always needs to be reported depending on the definition of Event-1, where a differential L1-RSRP value is sufficient due to the knowledge of the certain threshold that configured by RRC. A corresponding L1-RSRP report format depending on Event-1 by reusing report format for Event-2 can be concluded as in Table 3.

Table 3 L1-RSRP report format depending on Event-1

CRI or SSBRI #1

CRI or SSBRI #2

...

CRI or SSBRI #N

L1-RSRP #1

Differential L1-RSRP #2

...

Differential L1-RSRP #N

Differential L1-RSRP for the Q-th best activated TCI state

Proposal 16: On UE-initiated/event-driven beam reporting depending on Event-1, support the following L1-RSRP report format by reusing the report format for Event-2.

- Differential L1-RSRP #2~#N is determined based on the difference between measured L1-RSRP corresponding to the CRI/SSBRI #2~#N and the measured L1-RSRP corresponding to CRI/SSBRI #1

- L1-RSRP#1 is the largest measured RSRP among reported ones, and an absolute L1-RSRP.

- Differential L1-RSRP value for current beam is determined based on the difference between measured L1-RSRP corresponding to the current beam and the configured certain threshold.

- Beam application latency reduction

In RAN1#119 meeting, the following agreement on beam application latency reduction was endorsed [4].

Agreement Study the following to reduce beam application latency after a UEI/ED beam report is sent - Alt-1: After confirmation/acknowledgement from NW, apply new beam without RRC configuration signaling or MAC-CE signaling - after sending a UE-initiated beam report, the UE could store the QCL properties of the SSB associated with the reference signal reported in the beam report - update TCI state(s) with the reported new beam(s) - activate new beam(s) without additional SSB reception - Alt-2: After receiving a TCI state activation command to activate a TCI state(s), if the new beam(s) associated with the TCI state(s) is reported as synchronized in the UEI/ED beam report, the TCI state(s) becomes applicable for DL reception without additional SSB reception. - Note: A reported new beam is determined as synchronized by UE, if the UE stores the QCL properties associated with the reported new beam(s) after the UEI/ED beam report is sent - FFS: How to inform a reported new beam in a UEI/ED beam report (i.e., introducing one-bit indicator for each reported new beam or all the reported new beam are assumed to be synchronized) - Alt-3: After sending a UE-initiated beam report, the UE could store the QCL properties of the SSB associated with the reference signal reported in the beam report. - In such case, at the reception of a subsequent reception of Unified TCI States Activation/Deactivation MAC CE, the UE activates new beam(s) without additional SSB reception - Other alternatives are not precluded

For the above three alternatives of beam application latency reduction, we have the following elaborations:

  - For Alt-1, due to it aims for UE-initiated beam switching directly, it is a questionable way to reach out beam application latency reduction, especially if the update TCI state derived from the reported new beam(s) cannot be feasible from the perspective of NW scheduling. To address this subsequent issue, confirmation from NW and/or an ACK for the confirmation from the UE are required, which may unfortunately lead to a longer beam application latency. Besides, it is unclear of whether a timeline needs to be specified for when to start applying the TCI state switching after UE-initiated beam reporting, which may not be efficient of beam application latency reduction.

  - For Alt-2, one ambiguity is why UE cannot store the QCL properties of all new beam(s) that reported in the UE-initiated beam reporting. In other words, UE should be capable to store the QCL properties of all reported new beams, which is the same to Alt-3.

  - For Alt-3, it is a good starting point to facilitate beam application latency reduction by allowing UE to activate new beam(s) without additional SSB reception after receiving Unified TCI States Activation/Deactivation MAC CE. On the other hand, it is worth noting that such beam application latency reduction should be fulfilled at least based on Event-7, which is dedicated to beam reporting of activated TCI state with Q-th best quality.

In light of the above, we have the following proposal:

Proposal 17: Regarding beam application latency reduction after transmission of UE-initiated/event-driven beam report, support Alt-3.

- Alt-3: After sending a UE-initiated beam report for Event-7, the UE could store the QCL properties of the SSB associated with the reference signal reported in the beam report.

  - In such case, at the reception of a subsequent reception of Unified TCI States Activation/Deactivation MAC CE, the UE activates new beam(s) without additional SSB reception if the activated TCI state is QCLed with the SSB associated with the reference signal reported in the beam report.

- Other issues

Configuration of Mode-A and Mode-B

Firstly, supporting both modes simultaneously for a single CSI report configuration may introduce additional UE complexity and its benefits are not identified so far. Therefore, either mode-A or mode-B should be enabled by an RRC parameter in a CSI report configuration.

Secondly, CSI report configurations for UE-initiated/event-driven beam reporting for a single UE should be configured with the same mode to prevent potential coexistence issues and to reduce the implementation complexity.

Proposal 18: On beam report transmission procedure for UE-initiated/event-driven beam reporting, support to introduce an RRC parameter in a CSI report configuration to enable either mode-A or mode-B.

- Note: UE expects that the CSI report configurations for UE-initiated/event-driven beam reporting across all CCs are configured with a same mode.

Transmission occasion for UL reserved/configured-grant resource

Regarding beam report transmission procedure for UE-initiated/event-driven beam reporting, for either Mode-A or Mode-B, some UL reserved/configured-grant resources are used as uplink signaling container (e.g., first PUCCH carrying BRI in both modes and Type-1 CG PUSCH in Mode-B). Unfortunately, although we have further extension of dynamic beam (unified TCI) update to UL resource (e.g., PUCCH), transmission occasion (e.g., time-domain offset) still can NOT be updated simultaneously. In NW, considering UE mobility while serving multiple UEs, there is serious misalignment between cell-specific/pre-determined beam switching pattern in time-domain and dynamic TCI update. Therefore, transmission occasion of the pre-configured UL resource should be update simultaneously in such case as shown in Figure 1. Such enhancement offers benefits in achieving higher transmission reliability by addressing beam pattern misalignment and reducing report latency by eliminating the need for reconfiguration of pre-configured UL resources.

Figure 1 Updating transmission occasion (TO) of reserved/configured-grant resource (e.g., first PUCCH carrying BRI) based on the indicated TCI state

Proposal 19: Regarding the UL reserved/configured-grant resources used as uplink signaling container for UE-initiated//event-driven report (e.g., first PUCCH in both modes and Type-1 CG PUSCH in Mode-B), support that transmission occasion for reserved UL resource (as an indispensable part for UL signal medium/container) can be updated simultaneously via indicated TCI state to balance dynamic beam update and NW resource overhead.

- Conclusion

In this contribution, we discuss the potential candidate solutions to support UE-initiated beam management with the following proposals and text proposals.

Proposal 1: On beam report transmission procedure for UE-initiated/event-driven beam reporting in both Mode-A and Mode-B, support to introduce a prohibit-timer for first PUCCH.

- In the case that triggering-event associated with a CSI report configuration is determined, if the prohibit timer is NOT running, the UE can transmit first PUCCH.

  - At the first symbol after the end of the second PUSCH transmission, the UE starts the prohibit timer.

Proposal 2: Regarding the multiplexing/dropping rule(s) of 1-bit first PUCCH for the Case-2 that the 1-bit first PUCCH is collided/overlapped with a PUSCH, support Option-3 that piggyback 1-bit indication of first PUCCH into the PUSCH.

Proposal 3: Regarding the multiplexing/dropping rule(s) of 1-bit first PUCCH for the Case-3 that multiple 1-bit first PUCCHs corresponding to different CSI configuration for UE-initiated/event-driven beam reporting are overlapping in the time domain, support that it is up to UE implementation to transmit only one of the 1-bit first PUCCHs.

Proposal 4: Regarding the multiplexing/dropping rule(s) of 1-bit first PUCCH for the Case-4 that the 1-bit first PUCCH is collided/overlapped with a PUCCH format 0/1 carrying HARQ-ACK information, support that the PUCCH resource of the 1-bit first PUCCH is to carry the multiplexing of the 1-bit indication and the HARQ-ACK information via cyclic shift when using PUCCH format 0, and the PUCCH resource of the 1-bit first PUCCH is to carry the HARQ-ACK information only when using PUCCH format 1.

Proposal 5: Regarding the multiplexing/dropping rule(s) of 1-bit first PUCCH for the Case-5 that the 1-bit first PUCCH is overlapped with a PUCCH format 2/3/4 carrying HARQ-ACK information and/or CSI report(s), support to multiplex the 1-bit indication with the HARQ-ACK information and/or legacy CSI report via a UCI bit sequence.

- The UCI bit sequence generation reuses the legacy rule for SR with the following modification.

  - The UCI bit sequence is generated based on a first ascending order of the values of SR ID associated with the SRs and a second ascending order of the values of PUCCH resource ID associated with the first PUCCHs.

- The UCI bit sequence indicates a positive/negative SR or BRI.

Proposal 6: On beam report transmission procedure for UE-initiated/event-driven beam reporting, in order to clarify that the Type-1 CG PUSCH for carrying the beam report can carry any other UCI in Mode-B, support to down-select one of the following alternatives:

- Alt-1: UE multiplexes the HARQ-ACK information, 1-bit indication in first PUCCH, SR or CSI report on the Type-1 CG PUSCH for carrying the beam report based on collision handling rules.

- Alt-2: UE always can multiplex the HARQ-ACK information, 1-bit indication in first PUCCH, SR and/or CSI report on the Type-1 CG PUSCH for carrying the beam report in case of overlapping occurs.

- Proposal 7: On beam report transmission procedure for UE-initiated/event-driven beam reporting, for the case the pre-configured Type-1 CG PUSCH carry the beam report and other UCI in Mode-B, support to leverage the legacy UCI multiplexing/dropping rules as much as possible, i.e.,

- UE can multiplex HARQ-ACK information and BRI in the pre-configured Type-1 CG PUSCH carrying the beam report.

- UE does not transmit the Type-1 CG-PUSCH carrying UEI beam reports when a PUCCH carrying positive SR overlaps with the Type-1 CG-PUSCH.

- UE does not multiplex CSI reports from the PUCCH in the pre-configured Type-1 CG PUSCH carrying the beam report.

Proposal 8: On beam report transmission procedure for UE-initiated/event-driven beam reporting, in case that one PUCCH resource of first PUCCH can be associated with one or multiple CSI report configurations and multiple UEI beam reports occur, support that only a single UEI beam report with the highest priority is carried in the second PUSCH (Option 2).

- The priority of a UEI beam report is determined based on a formula independent to the formula defined for legacy CSI reports.

Proposal 9: On cross-CC beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Event-2, support to introduce new RRC parameter to indicate the CC in which the indicated TCI state associated with the CSI reporting configuration is applied.

Proposal 10: On cross-CC beam report transmission procedure for UE-initiated/event-driven beam reporting, support that CSI report configurations in a given CC can be associated with same or different trigger event type(s).

Proposal 11: Regarding triggering event determination within a time window for Event 2, on condition(s) of resetting the counting of trigger event instances, support Candidate#7.

- Candidate#7: The RRC parameter(s) associated with the CSI report configuration for UEI beam report is reconfigured.

  - RRC parameter(s) at least comprise RS configuration for new beam and the threshold for event evaluation.

  - FFS: Other RRC parameter(s).

Proposal 12: Regarding time window triggering event determination for Event 2, on the measurement window for initiating the UE-initiated/event-driven beam reporting procedure, support Option-4.

Proposal 13: Regarding additional indication of whether the CRI/SSBRI satisfies the condition of Event-2, support  the 2-bit field of additional indication as follows:

  - The codepoint is ‘11’ if triggering event condition is met for the reported L1-RSRP value of the corresponding RS and event instances of the corresponding RS is greater than the configured counting number.

  - The codepoint is ‘10’ if triggering event condition is met for the reported L1-RSRP value of the corresponding RS and event instances of the corresponding RS is not greater than the configured counting number.

  - The codepoint is ‘01’ if triggering event condition is not met for the reported L1-RSRP value of the corresponding RS and event instances of the corresponding RS is greater than the configured counting number.

  - The codepoint is ‘00’ if triggering event condition is not met for the reported L1-RSRP value of the corresponding RS and event instances of the corresponding RS is not greater than the configured counting number.

Proposal 14: Regarding additional indication of whether the CRI/SSBRI satisfies the condition of Event-2, support the following clarification if the additional indication is 1-bit field:

  - Bit value is 0 if event instances of the corresponding RS is not greater than the configured counting number or triggering event condition is not met for the reported L1-RSRP value of the corresponding RS.

  - Bit value is 1 if event instances of the corresponding RS is greater than or equal to the configured counting number and triggering event condition is met for the reported L1-RSRP value of the corresponding RS.

Proposal 15: On UE-initiated/event-driven beam reporting, regarding UL signaling content(s) of L1-RSRP report depending on Event-7, the option-3 on Event-2 is taken as a starting point with the following additional updates:

- Differential L1-RSRP value of the Q-th best activated TCI state as well as its corresponding TCI codepoint index should always be reported in addition to the N new beams.

- Other activated TCI state(s) in addition to the Q-th best activated TCI state are reported for facilitating the subsequent TCI activation/deactivation.

Proposal 16: On UE-initiated/event-driven beam reporting depending on Event-1, support the following L1-RSRP report format by reusing the report format for Event-2.

- Differential L1-RSRP #2~#N is determined based on the difference between measured L1-RSRP corresponding to the CRI/SSBRI #2~#N and the measured L1-RSRP corresponding to CRI/SSBRI #1

- L1-RSRP#1 is the largest measured RSRP among reported ones, and an absolute L1-RSRP.

- Differential L1-RSRP value for current beam is determined based on the difference between measured L1-RSRP corresponding to the current beam and the configured certain threshold.

Proposal 17: Regarding beam application latency reduction after transmission of UE-initiated/event-driven beam report, support Alt-3.

- Alt-3: After sending a UE-initiated beam report for Event-7, the UE could store the QCL properties of the SSB associated with the reference signal reported in the beam report.

  - In such case, at the reception of a subsequent reception of Unified TCI States Activation/Deactivation MAC CE, the UE activates new beam(s) without additional SSB reception if the activated TCI state is QCLed with the SSB associated with the reference signal reported in the beam report.

Proposal 18: On beam report transmission procedure for UE-initiated/event-driven beam reporting, support to introduce an RRC parameter in a CSI report configuration to enable either mode-A or mode-B.

- Note: UE expects that the CSI report configurations for UE-initiated/event-driven beam reporting across all CCs are configured with a same mode.

Proposal 19: Regarding the UL reserved/configured-grant resources used as uplink signaling container for UE-initiated//event-driven report (e.g., first PUCCH in both modes and Type-1 CG PUSCH in Mode-B), support that transmission occasion for reserved UL resource (as an indispensable part for UL signal medium/container) can be updated simultaneously via indicated TCI state to balance dynamic beam update and NW resource overhead.

Text Proposal 1: To adopt the following changes in section 9.2.5.1 in TS 38.213.

- 9.2.5.1 UE procedure for multiplexing HARQ-ACK or CSI and SR in a PUCCH In the following, a UE is configured to transmit K PUCCHs for respective K SRs in a slot, as determined by a set of schedulingRequestResourceId, a schedulingRequestResourceId associated with schedulingRequestID-BFR-SCell, a schedulingRequestResourceId associated with schedulingRequestID-BFR, a schedulingRequestResourceId associated with schedulingRequestID-BFR2 if the UE provides twoLRRcapability, and a schedulingRequestResourceId associated with schedulingRequestID-LBT-SCell, with SR transmission occasions that would overlap with a transmission of a PUCCH with HARQ-ACK information from the UE in the slot or with a transmission of a PUCCH with CSI report(s) from the UE in the slot. A UE is configured to transmit M PUCCHs for respective M BRIs in a slot, as determined by a set of pucch-ResourceId associated with firstPUCCHResourceConfig-UEIBR, with BRI transmission occasions that would overlap with a transmission of a PUCCH with HARQ-ACK information from the UE in the slot or with a transmission of a PUCCH with CSI report(s) from the UE in the slot. <Irrelevant part is omitted> If a UE would transmit a PUCCH with OACK HARQ-ACK information bits in a resource using PUCCH format 2 or PUCCH format 3 or PUCCH format 4 in a slot, as described in clauses 9.2.1 and 9.2.3, log2K+M+1 bits representing a negative/positive SR or BRI, firstly in ascending order of the values of schedulingRequestResourceId, a schedulingRequestResourceId associated with schedulingRequestID-BFR-SCell, a schedulingRequestResourceId associated with schedulingRequestID-BFR, a schedulingRequestResourceId associated with schedulingRequestID-BFR2 if the UE provides twoLRRcapability, and a schedulingRequestResourceId associated with schedulingRequestID-LBT-SCell, and secondly in ascending order of the values of pucch-ResourceId associated with firstPUCCHResourceConfig-UEIBR, are appended to the HARQ-ACK information bits and the UE transmits the combined OUCI=OACK+log2K+M+1 UCI bits in a PUCCH using a resource with PUCCH format 2 or PUCCH format 3 or PUCCH format 4 that the UE determines as described in clauses 9.2.1 and 9.2.3. If one of the SRs is a positive LRR, the value of the log2K+M+1 bits indicates the positive LRR. If one of the BRIs is a positive BRI and none of the SRs is a positive LRR, the value of the log2K+M+1 bits indicates the positive BRI. An all-zero value for the log2K+M+1 bits represents a negative SR value across all K SRs and a negative BRI across all M BRIs. If a UE would transmit a PUCCH with CSI report bits in a resource using PUCCH format 2 or PUCCH format 3 or PUCCH format 4 in a slot, log2K+M+1 bits representing corresponding negative/positive SR, firstly in ascending order of the values of schedulingRequestResourceId, a schedulingRequestResourceId associated with schedulingRequestID-BFR-SCell, a schedulingRequestResourceId associated with schedulingRequestID-BFR, a schedulingRequestResourceId associated with schedulingRequestID-BFR2 if the UE provides twoLRRcapability, and a schedulingRequestResourceId associated with schedulingRequestID-LBT-SCell, and secondly in ascending order of the values of pucch-ResourceId associated with firstPUCCHResourceConfig-UEIBR, are prepended to the CSI information bits as described in clause 9.2.5.2 and the UE transmits a PUCCH with the combined OUCI=log2K+M+1+OCSI UCI bits in a resource using the PUCCH format 2 or PUCCH format 3 or PUCCH format 4 for CSI reporting. If one of the SRs is a positive LRR, the value of the log2K+M+1 bits indicates the positive LRR. If one of the BRIs is a positive BRI and none of the SRs is a positive LRR, the value of the log2K+M+1 bits indicates the positive BRI. An all-zero value for the log2K+M+1 bits represents a negative SR value across all K SRs and a negative BRI across all M BRIs. <Irrelevant part is omitted>

Text Proposal 2: To adopt the following changes in section 9 in TS 38.213.

- 9 UE procedure for reporting control information <Irrelevant part is omitted> If a UE - would multiplex UCI in a PUCCH transmission that overlaps with a PUSCH transmission, and - the PUSCH and PUCCH transmissions fulfil the conditions in clause 9.2.5 for UCI multiplexing, the UE - multiplexes only HARQ-ACK information and BRI, if any, from the UCI in the PUSCH transmission and does not transmit the PUCCH if the UE multiplexes aperiodic, semi-persistent CSI reports or UEI-CSI reports in the PUSCH; - multiplexes only HARQ-ACK information, BRI and CSI reports, if any, from the UCI in the PUSCH transmission and does not transmit the PUCCH if the UE does not multiplex aperiodic, semi-persistent CSI reports or UEI-CSI reports in the PUSCH. <Irrelevant part is omitted>

Text Proposal 3: To adopt the following changes in section 5.2.5 in TS 38.214.

- 5.2.5 Priority rules for CSI reports <Irrelevant part is omitted> CSI reports are associated with a priority value PriiCSIy,k,c,s=2∙Ncells∙Ms∙y+Ncells∙Ms∙k+Ms∙c+s where - for aperiodic CSI reports to be carried on PUSCH for semi-persistent CSI reports to be carried on PUSCH, for semi-persistent CSI reports to be carried on PUCCH and for periodic CSI reports to be carried on PUCCH; - for CSI reports carrying L1-RSRP or L1-SINR and for CSI reports not carrying L1-RSRP or L1-SINR; - c is the serving cell index and Ncells is the value of the higher layer parameter maxNrofServingCells; - for a CSI report configured with ltm-CSI-ReportConfig, c is the serving cell index value where the report configuration is configured. - s is the reportConfigID and is the value of the higher layer parameter maxNrofCSI-ReportConfigurations. - for a CSI report configured with ltm-CSI-ReportConfig, s is the ltm-CSI-ReportConfigId and Ms is the value of the higher layer parameter maxNrofLTM-CSI-ReportConfigurations UEI-CSI reports are associated with a priority value PriiCSIc,s=Ms∙c+s where - c is the serving cell index where the report configuration is configured. - s is the reportConfigID and is the value of the higher layer parameter maxNrofCSI-ReportConfigurations. <Irrelevant part is omitted>

Text Proposal 4: To adopt the following changes in section 5.2.1.4.3 in CR38.214.

5.2.1.4.3 L1-RSRP Reporting <Irrelevant part is omitted> For L1-RSRP reporting, if the higher layer parameter nrofReportedRS in CSI-ReportConfig is configured to be one, or if the higher layer parameters nrOfReportedCells and nrOfReportedRS-PerCell are both configured to be one, the reported L1-RSRP value is defined by a 7-bit value in the range [-140, -44] dBm with 1dB step size, if the higher layer parameter nrofReportedRS is configured to be larger than one, or if the higher layer parameter groupBasedBeamReporting is configured as 'enabled', or if the higher layer parameter groupBasedBeamReporting-r17 is configured, or if the higher layer parameter groupBasedBeamReporting-r18 is configured, or if any of the higher layer parameters nrOfReportedCells and nrOfReportedRS-PerCell is configured to be larger than one, the UE shall use differential L1-RSRP based reporting, where the largest measured value of L1-RSRP is quantized to a 7-bit value in the range [-140, -44] dBm with 1dB step size, and the differential L1-RSRP is quantized to a 4-bit value. The differential L1-RSRP value is computed with 2 dB step size with a reference to the largest measured L1-RSRP value which is part of the same L1-RSRP reporting instance. The mapping between the reported L1-RSRP value and the measured quantity is described in [11, TS 38.133]. If the higher layer parameter XYZ in CSI-ReportConfig is configured, the UE shall report Condition met indicator for any of the CRI or SSBRI. Condition met indicator is set to 0 if event instances of the corresponding RS is not greater than eventInstanceCount-r19 within eventDetectionTimeWindowLength-r19 or triggering event condition is not met eventThreshold-r19 for the reported L1-RSRP value of the corresponding RS, and Condition met indicator is set to 1 if event instances of the corresponding RS is greater than or equal to eventInstanceCount-r19 within eventDetectionTimeWindowLength-r19 and triggering event condition is met eventThreshold-r19 for the reported L1-RSRP value of the corresponding RS. <Irrelevant part is omitted>

- References

- RP-242394, WID revision: NR MIMO Phase 5, Moderator (Samsung)

- Chair notes RAN1#120 eom0, Feb 17th - 21st, 2025, 3GPP RAN1

- Chair notes RAN1#118bis eom0, Oct 14th - 18th, 2024, 3GPP RAN1

- Chair notes RAN1#119 eom0, Nov 18th - 22nd, 2024, 3GPP RAN1