3GPP TSG RAN WG1 #120bis	R1-2501800

Wuhan, China, April 7th – 11th, 2025

Source:	vivo

Title:	Remaining issues on Rel-19 CSI enhancements

Agenda Item:	9.2.2

Document for:	Discussion and Decision

Introduction

The following two revised objectives related to CSI enhancement are listed in MIMO Evolution WID [1].

- Specify CSI support for up to 128 CSI-RS ports, targeting FR1 - Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks - Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s) - Extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design - SRS port grouping and its association to the two codewords for the 6/8Rx low complexity receiver supporting more than 4 layers, with legacy codebook - No enhancement on codeword-to-layer mapping, DL resource allocation, CSI feedback, and DCI format - Note: Whether to support 6Rx with more than 4 layers is to be decided in RAN4 Rel-19 RF enhancements WI - Specify UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD - Inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH

In this contribution, we discuss CSI enhancement for up to 128 CSI-RS ports and UE reporting enhancement for CJT calibration.

Views on CSI enhancement for up to 128 CSI-RS port

In this chapter, we will focus on the remaining issues related to the support of up to 128 CSI-RS ports.

  - Port indexing

For the Rel-19 Type-I and Type-II codebook refinement for 48, 64, and 128 CSI-RS ports, regarding the mapping from CSI-RS resource index and CSI-RS resource port index per resource to port index for CSI/PMI calculation. The following two methods are adopted in the spec..

Mapping method 1: Sequential ordering/indexing within (1st resource, 1st polarization), then (2nd resource, 1st polarization), …, then (Kth resource, 1st polarization), then (1st resource, 2nd polarization), then (2nd resource, 2nd polarization), …, then (Kth resource, 2nd polarization)

Mapping method 2: Sequential ordering/indexing within (where K*n2 = N2):

for the 1st polarization, (1st n2 ports in 1st resource, 1st polarization), (1st n2 ports in 2nd resource, 1st polarization), …, (1st n2 ports in Kth resource, 1st polarization), then (2nd n2 ports in 1st resource, 1st polarization), (2nd n2 ports in 2nd resource, 1st polarization), …, (2nd n2 ports in Kth resource, 1st polarization), … then (N1th n2 ports in 1st resource, 1st polarization), (N1th n2 ports in 2nd resource, 1st polarization), …, (N1th n2 ports in Kth resource, 1st polarization) ,

and then for the 2nd polarization, (1st n2 ports in 1st resource, 2nd polarization), (1st n2 ports in 2nd resource, 2nd polarization), …, (1st n2 ports in Kth resource, 2nd polarization), then (2nd n2 ports in 1st resource, 2nd polarization), (2nd n2 ports in 2nd resource, 2nd polarization), …, (2nd n2 ports in Kth resource, 2nd polarization), … then (N1th n2 ports in 1st resource, 2nd polarization), (N1th n2 ports in 2nd resource, 2nd polarization), …, (N1th n2 ports in Kth resource, 2nd polarization)

After reviewing the endorsed CRs for TS 38211[2] and TS 38214[3], it was found that both sides captured the same port indexing functionality repeatedly, which would cause the port indexing to operate incorrectly. Therefore, one of the sides had to be removed.

From our point of view, getting 48, 64, or 128 CSI-RS ports is not redesigning a new CSI-RS, but aggregating multiple legacy CSI-RS resources. That is, there are no new CSI-RS port indices except 3000, ..., 3031. Therefore, we do not support modifying the antenna port numbering formula in TS 38211, which will result in some new CSI-RS port indices, such as 3032, ..., 3127.

Additionally, the mapping method is a codebook-specific parameter and it is reasonable to capture it in the codebook definition of TS 38214.

Therefore, we believe that these two mapping methods should be captured in TS 38214 and the related content in TS 38211 should be removed.

Regarding the mapping from CSI-RS resource index and CSI-RS resource port index per resource to port index for CSI/PMI calculation, support capturing these two mapping methods in TS 38214 and removing the related content in TS 38211.

  - 3-bit scaling factor for CQI calculation

The following agreements were reached on the 3-bit scaling factor.

Agreement For the Rel-19 Type-I codebook refinement for 48, 64, and 128 CSI-RS ports, for RI=v=1, support the following: - for each group of X1X2 SD basis vectors, a 3-bit scaling factor can be NW-configured via higher-layer (RRC) signalling, where the scaling factors are defined as scalings on the power control offset configured for the associated CSI-RS resources - The values of X1 and X2 for this feature are separately configured from those for CBSR - Separate configuration (RRC signalling) from CBSR - The candidate values of X1 and X2 are the same as those agreed for CBSR - The codepoints of each of the group-specific 3-bit scaling factors are mapped to values of 1, 1/2, 1/3 ,1/4,1/6,1/8,1/12,1/16 - Note: This feature is a separate UE capability FFS: Whether this can be extended to RI=v>1 as well as Type-II codebook refinement Agreement For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, regarding per-layer scaling factor applied to each of the selected SD basis vectors associated with RI=v=2 for the 3-bit scaling factor(s): - The scaling formula is ρ.si where ρ is a multiplicative factor independent of i - Reuse legacy precoder normalization (per discretion of the spec editor) - FFS (RAN1#119): Whether min(ρ.si , 1) operation is needed - FFS (RAN1#119): Whether ρ other than 1 (baseline) is needed (e.g. v or 1v) - Regarding the configuration of the si value (3-bit indicator per SD basis vector group), decide, by RAN1#119, between the following: - Alt1. RI=1 and RI=2 are separately configured (RI-specific) - Alt2. A same configuration is used for RI=1 and RI=2 (RI-common) Agreement For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, regarding per-layer scaling factor applied to each of the selected SD basis vectors associated with RI=v=2 for the 3-bit scaling factor(s), ρ=1 - Note: In this case, the min(si,1) operation is not needed Agreement For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, regarding per-layer scaling factor applied to each of the selected SD basis vectors for the 3-bit scaling factor(s), the configuration of the si value (3-bit indicator per SD basis vector group) is RI-common (a same configuration is used for RI=1 and RI=2)

First of all, the 3-bit power scaling factor is defined only for Pc adjustment based on the current agreements, which is not part of CBSR. Hence we think the relevant text in the CBSR part shall be removed, and we only keep the text in the power control offset part.

Further, based on the above agreements, we believe that the specification adopts the most direct method of using 3-bit scaling factors, that is, the UE directly uses the associated 3-bit power offset scaling factor on the SD basis vector i selected at the i-th layer, where i = 0, 1. The following formula is the CQI calculation assumption when configuring a 3-bit scaling factor.

yi(3000)(i)⋮yi(3000+P-1)(i)=WiPcNCDMs(0)000…000PcNCDMs(v-1)xi(0)(i)⋮xi(ν-1)(i)

Based on the agreements, for the 3-bit scaling factor, UE shall directly use the 3-bit scaling factor on the SD basis vector i selected by layer i, where i=0,1.

Based on the above observation, there is no relevant agreement on the “not exceeding” text nor “sx1,x22v” text. The following TPs should be adopted.

Adopt the following TP in CR of TS 38214.

##### 5.2.2.2.1a Refined Type I Single-Panel Codebook <omitted text> The bitmap parameter typeI-softScalingRanks1-2-r19 forms the bit sequence b3As-1,…,b1,b0, where b0 is the LSB and b3As-1 is the MSB and the number of bits is given by Bs=3N1O1N2O2/X1(s)X2(s), where the values X1(s),X2(s)∈{2,1, 2,2, 4,1, 4,2, 4,4, (8,1)} are configured by the higher layer parameters valueOfX1-typeI-powerScaling-r19 and valueOfX2-typeI-powerScaling-r19, respectively. Bits b3N2O2X2(s)x1+x2+2b3N2O2X2(s)x1+x2+1b3N2O2X2(s)x1+x2 indicate a scaling factor, sx1,x2, for vector group Gs(x1,x2), x1=0,…,N1O1X1(s)-1, x2=0,…,N2O2X2(s)-1, given by Gsx1,x2=vX1sx1+l,X2sx2+m:l=0,1,…,X1s-1;m=0,1,…,X2s-1 For a scaling factor sx1,x2, a UE can assume that, for rank 1, and for rank 2, if supported by UE capability, the PDSCH signal for each layer mapped to a vector of group Gs(x1,x2) would have the same ratio of EPRE to CSI-RS EPRE for all K CSI-RS resources, not exceeding sx1,x22/υ times the powerControlOffset (in linear scale) of the respective CSI-RS resource. The mapping from bits b3N2O2X2(s)x1+x2+2b3N2O2X2(s)x1+x2+1b3N2O2X2(s)x1+x2 to sx1,x2 is given in Table 5.2.2.2.1a-8. Table 5.2.2.2.1a-8: Configurable scaling factor sx1,x2 for restricted vector group Gs(x1,x2) Bits b3N2O2X2(s)x1+x2+2b3N2O2X2(s)x1+x2+1b3N2O2X2(s)x1+x2; Scaling factor sx1,x2. 000; 14. 001; 123. 010; 122. 011; 16. 100; 12. 101; 13. 110; 12. 111; 1. <omitted text> ##### 5.2.2.5.1 UE assumptions for CQI/PMI/RI calculation <omitted text> - For a UE configured with 48, 64 and 128 antenna ports, obtained by aggregating K CSI-RS resources for channel measurement, for CQI calculation, the UE should assume that PDSCH signals on antenna ports in the set [1000,…,1000+υ-1], for υ layers, would result in signals equivalent to corresponding symbols transmitted on antenna ports [3000+p0,3000+p1,…,3000+p(P-1)], with index p(p')∈0,1,…,P/K-1, of the respective CSI-RS resource [k0,k1,…,k(P-1)] with kp'∈{0,1,…,K-1}, corresponding to the codebook ports p'=0,1,…,P-1, in order of increasing codebook port index, according to the mapping described in Clause 5.2.2.2.1a for codebookType set to 'typeI-SinglePanel-r19', 'eTypeII-r19', 'typeII-FePortSelection-r19' or 'typeII-Doppler-r19' and in Clause 5.2.2.2.2a for codebookType set 'typeI-MultiPanel-r19', such that yk0(3000+p(0))(i)yk1(3000+p(1))(i)⋮ykP-1(3000+p(P-1))(i)=W(i)x0(i)⋯xυ-1(i) where P∈{48,64,128} is the number of CSI-RS ports aggregated across the K CSI-RS resources, xi=x0(i)⋯xυ-1(i)T is a vector of PDSCH symbols from the layer mapping defined in Clause 7.3.1.4 of [4, TS 38.211] and W(i) is the precoding matrix corresponding to the reported PMI applicable to x(i). If a UE is configured with codebookType set to 'typeI-SinglePanel-r19' and with the higher layer parameter typeI-softScalingRank1-2-r19, for υ=1, and υ=2 if supported by UE capability, the UE can assume that the PDSCH signal for each layer mapped to a vector of group Gs(x1,x2) would have the same ratio of EPRE to CSI-RS EPRE for all K CSI-RS resources, equal to the sx1,x22 times the powerControlOffset (in linear scale) of the respective CSI-RS resourcenot exceeding sx1,x22/υ times the powerControlOffset (in linear scale) of the respective CSI-RS resource, where Gs(x1,x2) and sx1,x22 are described in Clause 5.2.2.2.1a; otherwise, the UE can assume that the PDSCH signals for υ layers would have the same ratio of EPRE to CSI-RS EPRE for all K CSI-RS resources, equal to the powerControlOffset of the respective CSI-RS resource. <omitted text>

Antenna port assumption for Rel-19 Type-II Doppler codebooks

In a legacy aperiodic CMR configuration for Rel-18 Type-II Doppler codebook, the UE shall assume that the antenna ports with the same port index of the K aperiodic CSI-RS resources are the same.

However, KDOPP = {4, 8, 12} CSI-RS resource groups are introduced for Type-II Doppler CSI. This means that within a CMR group, there may be multiple CSI-RS ports with the same CSI-RS port index mapped to different antenna ports, e.g., the port 3000s for the different CSI-RS resources in the following figure are associated with the different PMI port indices.

Therefore, the antenna port assumption needs to be further clarified when the UE is configured with KDOPP = {4, 8, 12} CSI-RS resource groups.

The corresponding relationship between CSI-RS ports and PMI ports for Mapping method 1

KDOPP = {4, 8, 12} CSI-RS resource groups are introduced for Type-II Doppler CSI. This means that within a CMR group, there may be multiple CSI-RS ports with the same CSI-RS port index mapped to different antenna ports.

To address this issue, a method that can be adopted is CSI-RS ports mapped to the same PMI row index across the K aperiodic CSI-RS resources share the same antenna port. Here, each PMI row index corresponds to KDOPP CSI-RS ports, with each CSI-RS port representing a CMR group. Therefore, the following TP should be adopted.

Adopt the following TP in CR of TS 38214 for the UE configured with codebookType set to 'typeII-Doppler-r19'.

##### 5.2.1.4.1 Resource Setting configuration <omitted text> A UE configured with a CSI-ReportConfig with the higher layer parameter N4, reportQuantity set to 'cri-RI-PMI-CQI' and codebookType set to 'typeII-Doppler-r19', is expected to be configured with 48, 64 or 128 CSI-RS ports obtained by aggregating 2≤K≤4 CSI-RS resources in a resource set for channel measurement. A UE can be configured with K periodic or semi-persistent CSI-RS resources, or with KDOPP∈{4,8,12} CSI-RS resource groups of K aperiodic CSI-RS resources each. For an aperiodic CSI-RS resource set for channel measurement, the KDOPP ⋅K CSI-RS resources are triggered by the same triggering instance and the separation between the first CSI-RS resources of two consecutive CSI-RS resource groups is m∈{1,2} slots, which is configured by higher layer parameter aperiodicResourceOffset. The KDOPP aperiodic CSI-RS resource groups are transmitted following the order of the CSI-RS resource groups configured in the CSI-RS resource set. The KDOPP ⋅K CSI-RS resources are ordered ascendingly by the CSI-RS resource ID and group index kDOPP=0,1,…,KDOPP-1, such that CSI-RS resources {kDOPP⋅K,kDOPP⋅K+1,…,kDOPP+1K-1} are associated with the kDOPP-th CSI-RS resource group. If interference measurement is performed on CSI-IM, only one resource is configured in the corresponding csi-IM-ResourceSet, where the one CSI-IM resource is associated with all the K CSI-RS resources, or K aperiodic CSI-RS resources in each of the KDOPP groups, in the CSI-RS resource set for channel measurement. If interference measurement is performed on NZP CSI-RS, only one resource is configured in the corresponding NZP-CSI-RS-ResourceSet for interference measurement. A UE shall assume that CSI-RS ports mapped to the same codebook port index, p'=0,1,…,PCSI-RS-1, as described in Clause 5.2.2.2.1a, across the K aperiodic CSI-RS resources share the same antenna port. <omitted text>

Views on UE reporting enhancement for CJT calibration

This section provides our views on the remaining issues on enhancement for CJT calibration (CJTC).

Clarifications on CP and ppm

Prior to RAN1#120bis, the following text of CR of TS 38214[3] was reached on the value of Dd and FO.

<omitted text> If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cjtc-Dd', - the UE selects and reports a reference CSI-RS resource set, nref, from the NTRP configured CSI-RS resource sets for tracking. - The UE reports an indication of the delay offset, Dn,offset, associated with each configured CSI-RS resource set n=0,1,…NTRP-1, relative to the reference CSI-RS resource set, nref, by indicating the interval [δin,δin+1) the delay offset falls into, as described in Clause 5.2.1.4.6. For the reference CSI-RS resource set, nref, the value of Dnref,offset is assumed zero and it is not reported. - The UE reports a one-bit indication, dn, for each configured CSI-RS resource set n=0,1,…,NTRP-1, where dn is equal to '1' if the corresponding delay offset, relative to the reference CSI-RS resource set, nref, plus delay spread, is not larger than the length of the cyclic prefix for symbols other than 0 and 7⋅2μ, or '0' otherwise. For the reference CSI-RS resource set, nref, the value of dnref is assumed one and it is not reported. <omitted text> ##### 5.2.1.4.6 CJTC-Dd reporting of delay offset For a CSI-ReportConfig with higher layer parameter reportQuantity set to 'cjtc-Dd', the interval [δin,δin+1) in which the delay offset, Dn,offset, falls into, for CSI-RS resource set n=0,…,NTRP-1, n≠nref, is indicated by iDO=i0⋯iNTRP-1 in∈{0,1,…,MD-1} where the mapping from in to δin is given by δin=in⋅ADMD-1 and δMD=∞, and the values of AD∈{CP/2, CP}, where CP is the length of the cyclic prefix for symbols other than 0 and 7⋅2μ, and MD∈{32, 64, 128, 256} are configured by the higher layer parameters valueOfAD and valueOfMD, respectively. The index value in=MD-1, corresponding to the interval [AD,∞), represents an out-of-range delay offset Dn,offset. ##### 5.2.1.4.7 CJTC-F reporting of frequency offset For a CSI-ReportConfig with higher layer parameter reportQuantity set to 'cjtc-F', the frequency offset value, FOn , for CSI-RS resource set n=0,…,NTRP-1, n≠nref, is indicated by kFO=k0⋯kNTRP-1 kn∈{0,1,…,MFO-1} where the mapping from kn to FOn is given by FOn=kn⋅AFOMFO-1,kn=0,1,…,MFO-2'invalid',kn=MFO-1 and the values of AFO∈{0.1ppm, 0.2ppm} and MFO∈{16, 32, 256} are configured by the higher layer parameters valueOfAFO and valueOfMFO, respectively. <omitted text>

Regarding CP, one point that may need clarification is that symbols of subframes associated with the NTRP configured CSI-RS resource sets used for tracking are used for CP calculation.

Regarding 0.1ppm or 0.2ppm, the reference frequency in relation to ppm should be clarified. A straightforward method to define the reference frequency in relation to ppm is the frequency of Point A.

Therefore, we recommend the following TP to make the values ​​of Dd and FO clearer.

Adopt the following TP in CR of TS 38214 to make the values of Dd and FO clearer

##### 5.2.1.4.2 Report quantity configurations <omitted text> If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cjtc-Dd', - the UE selects and reports a reference CSI-RS resource set, nref, from the NTRP configured CSI-RS resource sets for tracking. - The UE reports an indication of the delay offset, Dn,offset, associated with each configured CSI-RS resource set n=0,1,…NTRP-1, relative to the reference CSI-RS resource set, nref, by indicating the interval [δin,δin+1) the delay offset falls into, as described in Clause 5.2.1.4.6. For the reference CSI-RS resource set, nref, the value of Dnref,offset is assumed zero and it is not reported. - The UE reports a one-bit indication, dn, for each configured CSI-RS resource set n=0,1,…,NTRP-1, where dn is equal to '1' if the corresponding delay offset, relative to the reference CSI-RS resource set, nref, plus delay spread, is not larger than the length of the cyclic prefix for symbols other than 0 and 7⋅2μ in the subframes associated with the NTRP configured CSI-RS resource sets used for tracking, or '0' otherwise. For the reference CSI-RS resource set, nref, the value of dnref is assumed one and it is not reported. <omitted text> ##### 5.2.1.4.6 CJTC-Dd reporting of delay offset For a CSI-ReportConfig with higher layer parameter reportQuantity set to 'cjtc-Dd', the interval [δin,δin+1) in which the delay offset, Dn,offset, falls into, for CSI-RS resource set n=0,…,NTRP-1, n≠nref, is indicated by iDO=i0⋯iNTRP-1 in∈{0,1,…,MD-1} where the mapping from in to δin is given by δin=in⋅ADMD-1 and δMD=∞, and the values of AD∈{CP/2, CP}, where CP is the length of the cyclic prefix for symbols other than 0 and 7⋅2μ in the subframes associated with the NTRP configured CSI-RS resource sets used for tracking, and MD∈{32, 64, 128, 256} are configured by the higher layer parameters valueOfAD and valueOfMD, respectively. The index value in=MD-1, corresponding to the interval [AD,∞), represents an out-of-range delay offset Dn,offset. ##### 5.2.1.4.7 CJTC-F reporting of frequency offset For a CSI-ReportConfig with higher layer parameter reportQuantity set to 'cjtc-F', the frequency offset value, FOn , for CSI-RS resource set n=0,…,NTRP-1, n≠nref, is indicated by kFO=k0⋯kNTRP-1 kn∈{0,1,…,MFO-1} where the mapping from kn to FOn is given by FOn=kn⋅AFOMFO-1,kn=0,1,…,MFO-2'invalid',kn=MFO-1 and the values of AFO∈{0.1ppm, 0.2ppm} and MFO∈{16, 32, 256} are configured by the higher layer parameters valueOfAFO and valueOfMFO, respectively. The reference frequency in relation to ppm is the frequency of Point A. <omitted text>

  - linking CJTC Dd and Rel-18 eType-II CJT CSI report

Prior to RAN1#120bis, the following text of CR of TS 38214[3] was reached for jointly triggering and separately triggering of linking CJTC Dd and Rel-18 eType-II CJT CSI report.

<omitted text> - Subject to UE capability, the CSI-ReportConfig with reportQuantity set to 'cjtc-Dd' can be linked, by the higher layer parameter linkedCJTCReport, to an aperiodic CSI-ReportConfig with reportQuantity set to 'cri-RI-PMI-CQI' and codebookType set to 'typeII-CJT-r18' and with the corresponding CSI-RS resources for channel measurement that are aperiodic and not QCLed with the same reference signal with respect to {average delay}. The CSI-RS resource sets associated with the CJTC-Dd report are linked to the CSI-RS resources associated with the 'typeII-CJT-r18' report by a fixed correspondence between the resource set IDs and resource IDs, respectively, in sequential order of configuration in their respective Resource Setting. - When the two CSI reports are jointly triggered, the UE is expected to compensate for the delay offset values corresponding to the CSI-RS resources measured for 'typeII-CJT-r18' calculation and reported in the CJTC-Dd report in the same reporting instance. - When the two CSI reports are separately triggered, the higher layer parameter delayOffsetCompensation indicates whether the UE is expected to compensate for the delay offset values corresponding to the CSI-RS resources measured for 'typeII-CJT-r18' calculation and reported in the latest CJTC-Dd report whose reporting instance’s last symbol is before the first symbol of the DCI triggering the 'typeII-CJT-r18' report. The UE expects that the number of CSI-RS resource sets configured in the Resource Setting for the CJTC-Dd report is the same as the number of CSI-RS resources in the Resource Setting associated with the linked 'typeII-CJT-r18' report. If multiple 'typeII-CJT-r18' reports linked to CJTC-Dd report(s) are triggered in the same aperiodic trigger state and delayOffsetCompensation is configured, the UE is expected to apply delay offset compensation for all the linked 'typeII-CJT-r18' reports. <omitted text>

To capture the following agreement, the text above introduces a restriction for separate triggering, i.e., the UE expects the number of CSI-RS resource sets configured for the CJTC-Dd report to be the same as the number of CSI-RS resources for "typeII-CJT-r18" report.

Agreement For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured, regarding linking the CMRs in the two CSI Report Settings, for separate triggering, the UE expects that the number of configured CSI-RS resources associated with the Rel-18 eType-II CJT CSI is always the same as the number of TRS resource sets associated with the Rel-19 CJTC Dd report

However, for joint triggering, this restriction is not available, which means that the number of CSI-RS resource sets for CJTC-Dd report and the number of CSI-RS resources for CJT CSI report can be asymmetric. If the asymmetry exists, it means that the UE needs to determine which TRPs need to be compensated and which TRPs do not need to be compensated, which brings additional complexity and specification work. Therefore, a unified design for separate triggering and joint triggering should be considered, i.e., the number of CSI-RS resource sets for CJTC-Dd report is the same as the number of CSI-RS resources for CJT CSI report, regardless of separate triggering or joint triggering.

Adopt the following TP in CR of TS 38214 to make a unified design for separate triggering and joint triggering.

##### 5.2.1.4.2 Report quantity configurations <omitted text> - Subject to UE capability, the CSI-ReportConfig with reportQuantity set to 'cjtc-Dd' can be linked, by the higher layer parameter linkedCJTCReport, to an aperiodic CSI-ReportConfig with reportQuantity set to 'cri-RI-PMI-CQI' and codebookType set to 'typeII-CJT-r18' and with the corresponding CSI-RS resources for channel measurement that are aperiodic and not QCLed with the same reference signal with respect to {average delay}. The CSI-RS resource sets associated with the CJTC-Dd report are linked to the CSI-RS resources associated with the 'typeII-CJT-r18' report by a fixed correspondence between the resource set IDs and resource IDs, respectively, in sequential order of configuration in their respective Resource Setting. The UE expects that the number of CSI-RS resource sets configured in the Resource Setting for the CJTC-Dd report is the same as the number of CSI-RS resources in the Resource Setting associated with the linked 'typeII-CJT-r18' report. - When the two CSI reports are jointly triggered, the UE is expected to compensate for the delay offset values corresponding to the CSI-RS resources measured for 'typeII-CJT-r18' calculation and reported in the CJTC-Dd report in the same reporting instance. - When the two CSI reports are separately triggered, the higher layer parameter delayOffsetCompensation indicates whether the UE is expected to compensate for the delay offset values corresponding to the CSI-RS resources measured for 'typeII-CJT-r18' calculation and reported in the latest CJTC-Dd report whose reporting instance’s last symbol is before the first symbol of the DCI triggering the 'typeII-CJT-r18' report. The UE expects that the number of CSI-RS resource sets configured in the Resource Setting for the CJTC-Dd report is the same as the number of CSI-RS resources in the Resource Setting associated with the linked 'typeII-CJT-r18' report. If multiple 'typeII-CJT-r18' reports linked to CJTC-Dd report(s) are triggered in the same aperiodic trigger state and delayOffsetCompensation is configured, the UE is expected to apply delay offset compensation for all the linked 'typeII-CJT-r18' reports. <omitted text>

Conclusions

To summarize, we have the following proposals and observations.

Proposal 1: Regarding the mapping from CSI-RS resource index and CSI-RS resource port index per resource to port index for CSI/PMI calculation, support capturing these two mapping methods in TS 38214 and removing the related content in TS 38211.

- Observation 1: Based on the agreements, for the 3-bit scaling factor, UE shall directly use the 3-bit scaling factor on the SD basis vector i selected by layer i, where i=0,1.

- Proposal 2: Adopt the following TP in CR of TS 38214.

##### 5.2.2.2.1a Refined Type I Single-Panel Codebook <omitted text> The bitmap parameter typeI-softScalingRanks1-2-r19 forms the bit sequence b3As-1,…,b1,b0, where b0 is the LSB and b3As-1 is the MSB and the number of bits is given by Bs=3N1O1N2O2/X1(s)X2(s), where the values X1(s),X2(s)∈{2,1, 2,2, 4,1, 4,2, 4,4, (8,1)} are configured by the higher layer parameters valueOfX1-typeI-powerScaling-r19 and valueOfX2-typeI-powerScaling-r19, respectively. Bits b3N2O2X2(s)x1+x2+2b3N2O2X2(s)x1+x2+1b3N2O2X2(s)x1+x2 indicate a scaling factor, sx1,x2, for vector group Gs(x1,x2), x1=0,…,N1O1X1(s)-1, x2=0,…,N2O2X2(s)-1, given by Gsx1,x2=vX1sx1+l,X2sx2+m:l=0,1,…,X1s-1;m=0,1,…,X2s-1 For a scaling factor sx1,x2, a UE can assume that, for rank 1, and for rank 2, if supported by UE capability, the PDSCH signal for each layer mapped to a vector of group Gs(x1,x2) would have the same ratio of EPRE to CSI-RS EPRE for all K CSI-RS resources, not exceeding sx1,x22/υ times the powerControlOffset (in linear scale) of the respective CSI-RS resource. The mapping from bits b3N2O2X2(s)x1+x2+2b3N2O2X2(s)x1+x2+1b3N2O2X2(s)x1+x2 to sx1,x2 is given in Table 5.2.2.2.1a-8. Table 5.2.2.2.1a-8: Configurable scaling factor sx1,x2 for restricted vector group Gs(x1,x2) Bits b3N2O2X2(s)x1+x2+2b3N2O2X2(s)x1+x2+1b3N2O2X2(s)x1+x2; Scaling factor sx1,x2. 000; 14. 001; 123. 010; 122. 011; 16. 100; 12. 101; 13. 110; 12. 111; 1. <omitted text> ##### 5.2.2.5.1 UE assumptions for CQI/PMI/RI calculation <omitted text> - For a UE configured with 48, 64 and 128 antenna ports, obtained by aggregating K CSI-RS resources for channel measurement, for CQI calculation, the UE should assume that PDSCH signals on antenna ports in the set [1000,…,1000+υ-1], for υ layers, would result in signals equivalent to corresponding symbols transmitted on antenna ports [3000+p0,3000+p1,…,3000+p(P-1)], with index p(p')∈0,1,…,P/K-1, of the respective CSI-RS resource [k0,k1,…,k(P-1)] with kp'∈{0,1,…,K-1}, corresponding to the codebook ports p'=0,1,…,P-1, in order of increasing codebook port index, according to the mapping described in Clause 5.2.2.2.1a for codebookType set to 'typeI-SinglePanel-r19', 'eTypeII-r19', 'typeII-FePortSelection-r19' or 'typeII-Doppler-r19' and in Clause 5.2.2.2.2a for codebookType set 'typeI-MultiPanel-r19', such that yk0(3000+p(0))(i)yk1(3000+p(1))(i)⋮ykP-1(3000+p(P-1))(i)=W(i)x0(i)⋯xυ-1(i) where P∈{48,64,128} is the number of CSI-RS ports aggregated across the K CSI-RS resources, xi=x0(i)⋯xυ-1(i)T is a vector of PDSCH symbols from the layer mapping defined in Clause 7.3.1.4 of [4, TS 38.211] and W(i) is the precoding matrix corresponding to the reported PMI applicable to x(i). If a UE is configured with codebookType set to 'typeI-SinglePanel-r19' and with the higher layer parameter typeI-softScalingRank1-2-r19, for υ=1, and υ=2 if supported by UE capability, the UE can assume that the PDSCH signal for each layer mapped to a vector of group Gs(x1,x2) would have the same ratio of EPRE to CSI-RS EPRE for all K CSI-RS resources, equal to the sx1,x22 times the powerControlOffset (in linear scale) of the respective CSI-RS resourcenot exceeding sx1,x22/υ times the powerControlOffset (in linear scale) of the respective CSI-RS resource, where Gs(x1,x2) and sx1,x22 are described in Clause 5.2.2.2.1a; otherwise, the UE can assume that the PDSCH signals for υ layers would have the same ratio of EPRE to CSI-RS EPRE for all K CSI-RS resources, equal to the powerControlOffset of the respective CSI-RS resource. <omitted text>

- Observation 2: KDOPP = {4, 8, 12} CSI-RS resource groups are introduced for Type-II Doppler CSI. This means that within a CMR group, there may be multiple CSI-RS ports with the same CSI-RS port index mapped to different antenna ports.

- Proposal 3: Adopt the following TP in CR of TS 38214 for the UE configured with codebookType set to 'typeII-Doppler-r19'.

##### 5.2.1.4.1 Resource Setting configuration <omitted text> A UE configured with a CSI-ReportConfig with the higher layer parameter N4, reportQuantity set to 'cri-RI-PMI-CQI' and codebookType set to 'typeII-Doppler-r19', is expected to be configured with 48, 64 or 128 CSI-RS ports obtained by aggregating 2≤K≤4 CSI-RS resources in a resource set for channel measurement. A UE can be configured with K periodic or semi-persistent CSI-RS resources, or with KDOPP∈{4,8,12} CSI-RS resource groups of K aperiodic CSI-RS resources each. For an aperiodic CSI-RS resource set for channel measurement, the KDOPP ⋅K CSI-RS resources are triggered by the same triggering instance and the separation between the first CSI-RS resources of two consecutive CSI-RS resource groups is m∈{1,2} slots, which is configured by higher layer parameter aperiodicResourceOffset. The KDOPP aperiodic CSI-RS resource groups are transmitted following the order of the CSI-RS resource groups configured in the CSI-RS resource set. The KDOPP ⋅K CSI-RS resources are ordered ascendingly by the CSI-RS resource ID and group index kDOPP=0,1,…,KDOPP-1, such that CSI-RS resources {kDOPP⋅K,kDOPP⋅K+1,…,kDOPP+1K-1} are associated with the kDOPP-th CSI-RS resource group. If interference measurement is performed on CSI-IM, only one resource is configured in the corresponding csi-IM-ResourceSet, where the one CSI-IM resource is associated with all the K CSI-RS resources, or K aperiodic CSI-RS resources in each of the KDOPP groups, in the CSI-RS resource set for channel measurement. If interference measurement is performed on NZP CSI-RS, only one resource is configured in the corresponding NZP-CSI-RS-ResourceSet for interference measurement. A UE shall assume that CSI-RS ports mapped to the same codebook port index, p'=0,1,…,PCSI-RS-1, as described in Clause 5.2.2.2.1a, across the K aperiodic CSI-RS resources share the same antenna port. <omitted text>

- Proposal 4: Adopt the following TP in CR of TS 38214 to make the values of Dd and FO clearer

##### 5.2.1.4.2 Report quantity configurations <omitted text> If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cjtc-Dd', - the UE selects and reports a reference CSI-RS resource set, nref, from the NTRP configured CSI-RS resource sets for tracking. - The UE reports an indication of the delay offset, Dn,offset, associated with each configured CSI-RS resource set n=0,1,…NTRP-1, relative to the reference CSI-RS resource set, nref, by indicating the interval [δin,δin+1) the delay offset falls into, as described in Clause 5.2.1.4.6. For the reference CSI-RS resource set, nref, the value of Dnref,offset is assumed zero and it is not reported. - The UE reports a one-bit indication, dn, for each configured CSI-RS resource set n=0,1,…,NTRP-1, where dn is equal to '1' if the corresponding delay offset, relative to the reference CSI-RS resource set, nref, plus delay spread, is not larger than the length of the cyclic prefix for symbols other than 0 and 7⋅2μ in the subframes associated with the NTRP configured CSI-RS resource sets used for tracking, or '0' otherwise. For the reference CSI-RS resource set, nref, the value of dnref is assumed one and it is not reported. <omitted text> ##### 5.2.1.4.6 CJTC-Dd reporting of delay offset For a CSI-ReportConfig with higher layer parameter reportQuantity set to 'cjtc-Dd', the interval [δin,δin+1) in which the delay offset, Dn,offset, falls into, for CSI-RS resource set n=0,…,NTRP-1, n≠nref, is indicated by iDO=i0⋯iNTRP-1 in∈{0,1,…,MD-1} where the mapping from in to δin is given by δin=in⋅ADMD-1 and δMD=∞, and the values of AD∈{CP/2, CP}, where CP is the length of the cyclic prefix for symbols other than 0 and 7⋅2μ in the subframes associated with the NTRP configured CSI-RS resource sets used for tracking, and MD∈{32, 64, 128, 256} are configured by the higher layer parameters valueOfAD and valueOfMD, respectively. The index value in=MD-1, corresponding to the interval [AD,∞), represents an out-of-range delay offset Dn,offset. ##### 5.2.1.4.7 CJTC-F reporting of frequency offset For a CSI-ReportConfig with higher layer parameter reportQuantity set to 'cjtc-F', the frequency offset value, FOn , for CSI-RS resource set n=0,…,NTRP-1, n≠nref, is indicated by kFO=k0⋯kNTRP-1 kn∈{0,1,…,MFO-1} where the mapping from kn to FOn is given by FOn=kn⋅AFOMFO-1,kn=0,1,…,MFO-2'invalid',kn=MFO-1 and the values of AFO∈{0.1ppm, 0.2ppm} and MFO∈{16, 32, 256} are configured by the higher layer parameters valueOfAFO and valueOfMFO, respectively. The reference frequency in relation to ppm is the frequency of Point A. <omitted text>

- Proposal 5: Adopt the following TP in CR of TS 38214 to make a unified design for separate triggering and joint triggering.

##### 5.2.1.4.2 Report quantity configurations <omitted text> - Subject to UE capability, the CSI-ReportConfig with reportQuantity set to 'cjtc-Dd' can be linked, by the higher layer parameter linkedCJTCReport, to an aperiodic CSI-ReportConfig with reportQuantity set to 'cri-RI-PMI-CQI' and codebookType set to 'typeII-CJT-r18' and with the corresponding CSI-RS resources for channel measurement that are aperiodic and not QCLed with the same reference signal with respect to {average delay}. The CSI-RS resource sets associated with the CJTC-Dd report are linked to the CSI-RS resources associated with the 'typeII-CJT-r18' report by a fixed correspondence between the resource set IDs and resource IDs, respectively, in sequential order of configuration in their respective Resource Setting. The UE expects that the number of CSI-RS resource sets configured in the Resource Setting for the CJTC-Dd report is the same as the number of CSI-RS resources in the Resource Setting associated with the linked 'typeII-CJT-r18' report. - When the two CSI reports are jointly triggered, the UE is expected to compensate for the delay offset values corresponding to the CSI-RS resources measured for 'typeII-CJT-r18' calculation and reported in the CJTC-Dd report in the same reporting instance. - When the two CSI reports are separately triggered, the higher layer parameter delayOffsetCompensation indicates whether the UE is expected to compensate for the delay offset values corresponding to the CSI-RS resources measured for 'typeII-CJT-r18' calculation and reported in the latest CJTC-Dd report whose reporting instance’s last symbol is before the first symbol of the DCI triggering the 'typeII-CJT-r18' report. The UE expects that the number of CSI-RS resource sets configured in the Resource Setting for the CJTC-Dd report is the same as the number of CSI-RS resources in the Resource Setting associated with the linked 'typeII-CJT-r18' report. If multiple 'typeII-CJT-r18' reports linked to CJTC-Dd report(s) are triggered in the same aperiodic trigger state and delayOffsetCompensation is configured, the UE is expected to apply delay offset compensation for all the linked 'typeII-CJT-r18' reports. <omitted text>

References

RP-242394, New WID: NR MIMO Phase 5, RAN#105.

R1-2501656, Introduction of NR MIMO Phase5.

R1-2501667, Introduction of CSI enhancements for NR MIMO Phase 5.