3GPP TSG RAN WG1 #120bis                                                            R1-2501817

WuHan, China, April 7th – 11st, 2025

Source:	vivo

Title:	Views on Rel-19 ISAC deployment scenarios

Agenda Item:	9.7.1

Document for:	Discussion and Decision

Introduction

A new study item of channel modeling for integrated sensing and communications (ISAC) has been approved in RAN #102 [1], in which the objectives of the study for ISAC are as follows:

- The focus of the study is to define channel modelling aspects to support object detection and/or tracking (as per the SA1 meaning in TS 22.137). The study should aim at a common modelling framework capable of detecting and/or tracking the following example objects and to enable them to be distinguished from unintended objects: - UAVs - Humans indoors and outdoors - Automotive vehicles (at least outdoors) - Automated guided vehicles (e.g. in indoor factories) - Objects creating hazards on roads/railways, with a minimum size dependent on frequency. - All six sensing modes should be considered (i.e. TRP-TRP bistatic, TRP monostatic, TRP-UE bistatic, UE-TRP bistatic, UE-UE bistatic, UE monostatic). - Frequencies from 0.5 to 52.6 GHz are the primary focus, with the assumption that the modelling approach should scale to 100 GHz. (If significant problems are identified with scaling above 52.6 GHz, the range above 52.6 GHz can be deprioritized.) - For the above use cases, sensing modes and frequencies: - Identify details of the deployment scenarios corresponding to the above use cases. - Define channel modelling details for sensing using TR38.901 as a starting point, and taking into account relevant measurements, including: - modelling of sensing targets and background environment, including, for example (if needed by the above use cases), radar cross-section (RCS), mobility and clutter/scattering patterns, - spatial consistency. - It will be discussed at RAN#105 whether to include additional study beyond channel modelling for ISAC.

In RAN1 #120, accordingly, the agreements were made as follows:

Agreement-1 For ISAC channel modelling calibration, RAN1 considers both large-scale and full-scale calibration to include parameters and values for at least the following: - large scale parameters, where fast fading is not included - full-scale calibration parameters, which includes fast fading. - NOTE0: one part of calibration work does not include additional components and does not include spatial consistency - FFS: whether spatial consistency is specified as an additional component for ISAC CM - NOTE1: additional calibrations including spatial consistency can also be considered case by case for different scenarios. - NOTE2: Inclusion of EO in ISAC CM calibrations can also be considered case by case for different scenarios. Agreement-2 Calibration of ISAC CM includes separate calibration of the target channel and of the background channel - FFS: additional calibration for the combined channel (combination of target and background channel). Agreement-3 For the purposes of large scale calibration for UAV sensing targets, the following calibration parameters are proposed below in Table x. Table x. Simulation assumptions for large scale calibration for UAV sensing targets Parameters; Values. Scenario; UMa-AV. Sensing mode; TRP monostatic, TRP-TRP bistatic, TRP-UE bistatic, UE-UE bistatic Note: further down-selection of the sensing modes for UAV sensing is not precluded. Sectorization; 3 sectors per cell site: 30, 150 and 270 degrees. Carrier Frequency; FR1: 6 GHz FR2: 30 GHz. BS antenna configurations; Single dual-pol isotropic antenna. BS Tx power; FR1: 56dBm FR2: 41dBm. Bandwidth; FR1: 100MHz FR2: 400MHz. BS noise figure; FR1: 5dB FR2: 7dB. UT antenna configurations; (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1). UT noise figure; FR1: 9dB FR2: 10dB. Sensing target distribution; 1 target uniformly distributed (across multiple drops) within the center cell. Vertical distribution: Fixed height value of 200 m.. Component A of the RCS for each scattering point; a fixed value of A. Minimum 3D distances between pairs of Tx/Rx and sensing target; 10 m. Wrapping Method; No wrapping method is used if interference is not modelled, otherwise geographical distance based wrapping. Metrics; Coupling loss (based on LOS pathloss) - FFS: how to select sensing Tx and Rx FFS: additional metrics, wideband SIR and SINR based on RSRP if interference is modelled..

In this contribution, we discuss on the remaining issues for the ISAC channel model calibration, and suggest the calibration parameters for human sensing targets scenarios and spatial consistency as well. In addition, some initial calibration results are provided in this contribution.

Discussion

Methodology for calibration

According to [2], some remaining issues with FFS parameters for calibration need to be further discussed, such as sensing Tx-Rx pairs selection, Tx-ST-Rx selection, and concatenation method. Furthermore, we may need to consider whether combined channel should be calibrated or not.

Sensing link selection for calibration

For the communication channel calibration in TR 38.901, the coupling loss to draw calibration CDF curve is the coupling loss between UE and its serving cell, the weak links (e.g., NLOS propagation) between UE and other cells referred as interference links are calibrated by SIR/SINR. Regarding the calibration for ISAC channel, two issues on how to determine the sensing links for calibration need further discussion. One is how to determine the sensing Tx-Rx pair to calculate the Tx-ST-Rx channel for calibration, and the other is how to select Tx-ST-Rx links for calibration.

Regarding the Tx-Rx pair selection for calibration, with each sensing target, the sensing Tx-Rx selection is related to the sensing mode. For TRP/UE mono-static sensing, each TRP/UE can be the sensing Tx/Rx. For TRP-TRP/UE-UE bistatic, each pair of TRP-TRP/UE-UE can be the sensing Tx-Rx pair. For TRP-UE/UE-TRP bistatic, each pair of serving TRP-UE can be the sensing Tx-Rx pair. According to the communication procedure, for each UEs, the serving TRP selection can be based on the RSRP between TRP and UE.

- For TRP-UE/UE-TRP bistatic, each pair of serving TRP-UE can be the sensing Tx-Rx pair, and the serving TRP selection can be based on the RSRP between TRP and UE.

For each sensing target, the sensing channel can be calculated for each Tx-ST-Rx based on the Tx-Rx pair, another issue is how to select the sensing links for calibration. According to the email discussion, one option is that select the best N Tx-ST-Rx based on the coupling loss to draw the CDF curves for calibration. By doing this way, however, some sensing channel conditions, such as NLOS+NLOS, LOS+NLOS, NLOS+LOS, may not be calibrated if only best N Tx-ST-Rx links are calibrated. Therefore, it is better select all the Tx-ST-Rx links to draw the CDF figures for calibration.

Observation 1: Some sensing channel conditions, such as NLOS+NLOS, LOS+NLOS, NLOS+LOS, may not be calibrated if only best N Tx-ST-Rx links are calibrated.

- Select all the Tx-ST-Rx links to draw the CDF figures for calibration.

Deployment scenarios

For each sensing target type, several deployment scenarios are agreed in the previous meetings, in our options, considering the multiple sensing use cases, there is no need to have limitation and priority for each deployment scenario.

- RAN1 does not limit the deployment scenarios for calibration, and it is up to company inputs.

Sensing modes

According to [1], one of the objectives of the study for ISAC channel model is that all six sensing modes should be considered (i.e. TRP-TRP bistatic, TRP monostatic, TRP-UE bistatic, UE-TRP bistatic, UE-UE bistatic, UE monostatic). In our options, considering the multiple sensing use cases, there is no need to have limitation and priority for each sensing mode. The only problem is the lack of channel model for some sensing modes in some deployment scenarios.

- RAN1 does not limit the sensing mode, and it is up to company inputs.

Convolution method

Based on the agreement in the last meeting, two convolution methods are supported in ISAC channel modeling. For the full convolution method, it is clear enough that it can be used as the basic method in channel model calibration. For the one-by-one random coupling convolution method, there are still some implementation details that are unclear, such that whether the selected coupling paths need power normalization before or after one-by-one random coupling, which is not captured in the current agreement. However, this may greatly affect the final calibration results since there is a large gap in the final angular expansion distribution of the target channel under the two conditions with and without power normalization, referring to our companion contribution [3]. Therefore, RAN1 needs to determine whether or how power normalization is taken if using the convolution method of one-by-one random coupling for calibration.

- Full convolution is used as a baseline for ISAC channel calibration.

- Whether or how to perform power normalization should be determined for ISAC channel calibration if ray level 1-by-1 random coupling convolution between Tx-target link and target-Rx link is used.

Calibration for combined channel

The purpose of channel calibration is to allow the companies to make sure the consistency of the channel model by addressing implementation errors from channel modeling development. For the ISAC channel model, although the target channel will be added on top of background channel, yet the overall combined channel should be consistent if the target channel is well calibrated. However, duo to the new definition of background channel for mono-static sensing mode, the calibration on it can be individually performed. This can reduce the calibration stages and result in low calibration workload.

- RAN1 individually calibrates the target channel and background channel if the background channel is newly designed. Otherwise, RAN1 only calibrates the target channel.

Calibration parameters for human

Large scale calibration

      - Human indoor

Table 1: Simulation assumptions for large scale calibration for human indoor.

| Parameters | Values |
| --- | --- |
| Scenario | Indoor office, Indoor room |
| Cell layout | Office size (WxLxH): 120mx50mx3m, ISD=20m Room size (WxLxH): 20mx20mx3m, one TRP |
| Sensing mode | TRP monostatic, TRP-UE bistatic, UE-UE bistatic, TRP-TRP bistatic for office |
| Sensing type | Adult: 0.5m x 0.5m x 1.75m |
| Sectorization | 1 sector per cell site |
| Carrier Frequency | 7 GHz, 30 GHz |
| Bandwidth | 7GHz: 200MHz 30GHZ: 400MHz |
| BS height | 3m (ceiling) |
| BS antenna configurations | Single dual-pol isotropic antenna |
| BS Tx power | 7GHz: 24dBm 30GHZ: 23dBm |
| BS noise figure | 7GHz: 5dB 30GHZ: 7dB |
| UT height | 1m |
| UT antenna configurations | (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1) |
| UT mobility | 3Km/h |
| UT Tx power | 23dBm |
| UT distribution | Uniformly distributed within the indoor horizontal plane. 10 UTs per cell. |
| UT noise figure | 7GHz: 9dB 30GHz: 10dB |
| Polarized antenna model | Model-2 in clause 7.3.2 in TR 38.901 |
| Sensing target distribution | 1 target uniformly distributed (across multiple drops) within one cell. Office: total 12 targets Room: total 1 target |
| Component A of the RCS for each scattering point | a fixed value of A, (A is -1.37dBsm) |
| Sensing target mobility | 3Km/h |
| Scattering point | Single scattering point is in the center of the human. The vertical height for pathloss calculation is 0.875 m. |
| Minimum 3D distances between pairs of Tx/Rx and sensing target | For TRP-ST, 1m For UT-ST, 1m |
| Minimum 3D distances targets | 1m |
| Wrapping Method | No wrapping method is used. |
| Coupling loss definition | Power scaling factor, including pathloss, shadow fading, RCS component. |
| Sensing Tx/Rx selection | For TRP mono-static: each TRP can be the sensing Tx/Rx. For TRP-TRP/UE-UE bistatic, each pair of TRP-TRP/UE-UE can be the sensing Tx/Rx pair. For TRP-UE bistatic, each TRP-serving UE can be the sensing Tx/Rx pair. The TRP-serving UE selection based on the TRP-UE coupling loss. Calculate the coupling loss for each Tx-ST-Rx, select all the Tx-ST-Rx links for calibration. |
| Metrics | Coupling loss. |

- For the purposes of large scale calibration for human indoor sensing scenarios, the parameters in Table 1 are proposed.

      - Human outdoor

Table 2: Simulation assumptions for large scale calibration for human outdoor.

| Parameters | Values |
| --- | --- |
| Scenario | UMa |
| Cell layout | Hexagonal grid, 19 macro sites, 3 sectors per site (ISD = 500m) |
| Sensing mode | TRP monostatic, TRP-TRP bistatic, TRP-UE bistatic, UE-UE bistatic |
| Sensing type | Adult: 0.5m x 0.5m x 1.75m |
| Sectorization | 3 sectors per cell site: 30, 150 and 270 degrees. For single isotropic antenna, single 360-degree sector can be assumed. |
| Carrier Frequency | 7 GHz, 30 GHz |
| Bandwidth | 7GHz: 200MHz 30GHZ: 400MHz |
| BS height | 25m |
| BS antenna configurations | Single dual-pol isotropic antenna |
| BS Tx power | 7GHz: 59dBm 30GHZ: 41dBm |
| BS noise figure | 7GHz: 5dB 30GHZ: 7dB |
| UT height | 1.5m |
| UT antenna configurations | (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1) |
| UT mobility | 3Km/h |
| UT Tx power | 23dBm |
| UT distribution | Uniformly distributed within the horizontal plane. 10 outdoor UTs per cell. |
| UT noise figure | 7GHz: 9dB 30GHz: 10dB |
| Polarized antenna model | Model-2 in clause 7.3.2 in TR 38.901 |
| Sensing target distribution | 1 target uniformly distributed (across multiple drops) within the center site. |
| Component A of the RCS for each scattering point | a fixed value of A, (A is -1.37dBsm) |
| Sensing target mobility | 3Km/h |
| Scattering point | Single scattering point is in the center of the human. The vertical height for pathloss calculation is 0.875 m. |
| Minimum 3D distances between pairs of Tx/Rx and sensing target | For TRP-ST, 25m For UT-ST, 1m |
| Minimum 3D distances targets | 1m |
| Wrapping Method | No wrapping method is used if interference is not modelled, otherwise geographical distance based wrapping. |
| Coupling loss definition | Power scaling factor, including pathloss, shadow fading, RCS component. |
| Sensing Tx/Rx selection | For TRP mono-static: each TRP can be the sensing Tx/Rx. For TRP-TRP/UE-UE bistatic, each pair of TRP-TRP/UE-UE can be the sensing Tx/Rx pair. For TRP-UE bistatic, each TRP-serving UE can be the sensing Tx/Rx pair. The TRP-serving UE selection based on the TRP-UE coupling loss. Calculate the coupling loss for each Tx-ST-Rx, select all the Tx-ST-Rx links for calibration. |
| Metrics | Coupling loss. |

- For the purposes of large scale calibration for human outdoor sensing scenarios, the parameters in Table 2 are proposed.

Full calibration

      - Human indoor

Table 3: Simulation assumptions for full calibration for human indoor.

| Parameters | Values |
| --- | --- |
| Scenario | Indoor office, Indoor room |
| Cell layout | Office size (WxLxH): 120mx50mx3m, ISD=20m Room size (WxLxH): 20mx20mx3m, one TRP |
| Sensing mode | TRP monostatic, TRP-UE bistatic, UE-UE bistatic, TRP-TRP bistatic for office |
| Sensing type | Adult: 0.5m x 0.5m x 1.75m |
| Sectorization | 1 sector per cell site |
| Carrier Frequency | 7 GHz, 30 GHz |
| Bandwidth | 7GHz: 200MHz 30GHZ: 400MHz |
| BS height | 3m (ceiling) |
| BS antenna configurations | Single dual-pol isotropic antenna |
| BS Tx power | 7GHz: 24dBm 30GHZ: 23dBm |
| BS noise figure | 7GHz: 5dB 30GHZ: 7dB |
| UT height | 1m |
| UT antenna configurations | (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1) |
| UT mobility | 3Km/h |
| UT Tx power | 23dBm |
| UT distribution | Uniformly distributed within the indoor horizontal plane. 10 UTs per cell. |
| UT noise figure | 7GHz: 9dB 30GHz: 10dB |
| Polarized antenna model | Model-2 in clause 7.3.2 in TR 38.901 |
| Sensing target distribution | 1 target uniformly distributed (across multiple drops) within one cell. Office: total 12 targets Room: total 1 target |
| Sensing target mobility | 3Km/h |
| Scattering point | Single scattering point is in the center of the human. The vertical height for pathloss calculation is 0.875 m. |
| Component A of the RCS for each scattering point | a fixed value of A, (A is -1.37dBsm) |
| Component B1, B2 of the RCS for each scattering point | According to the agreement of RCS in 9.7.2. |
| Fast fading model | According to 38.901 and 38.858. |
| (u, std) for XPR of target | According to the agreement in 9.7.2. |
| The power threshold for path dropping after concatenation for target channel | -40dB |
| Minimum 3D distances between pairs of Tx/Rx and sensing target | For TRP-ST, 1m For UT-ST, 1m |
| Minimum 3D distances targets | 1m |
| Wrapping Method | No wrapping method is used. |
| Coupling loss definition | By definition, need to consider all direct and indirect paths. The following parameters are included in the calculation: - power scaling factor (pathloss, shadow fading, and RCS component A included) - for small scale RCS B1/B2 and power of rays in Tx-target/target-Rx links (Pn',m',n,mk,p), Tx/Rx antenna pattern, 3 polarization matrixes, i.e., Pn',m',n,mk,pFrx,u,θθrx,n',m',ZOAk,p,ϕrx,n',m',AOAk,pFrx,u,ϕθrx,n',m',ZOAk,p,ϕrx,n',m',AOAk,pTCPMrx,n',m'k,pCPMn',m',n,mk,pCPMtx,n, mk,p ∙Ftx,s,θθtx,n,m,ZODk,p,ϕtx,n,m,AODk,pFtx,s,ϕθtx,n,m,ZODk,p,ϕtx,n,m,AODk,p |
| Sensing Tx/Rx selection | For TRP mono-static: each TRP can be the sensing Tx/Rx. For TRP-TRP/UE-UE bistatic, each pair of TRP-TRP/UE-UE can be the sensing Tx/Rx pair. For TRP-UE bistatic, each TRP-serving UE can be the sensing Tx/Rx pair. The TRP-serving UE selection based on the TRP-UE coupling loss. Select all the Tx-ST-Rx links for calibration. |
| Metrics | Coupling loss. CDF of the Delay Spread and Angle Spread (ASD, ZSD, ASA, ZSA). ASD and ZSD are the departure angle spread of sensing TX, ASA and ZSA are the arrival angle spread of sensing RX. |

- For the purposes of full calibration for human indoor sensing scenarios, the parameters in Table 3 are proposed.

      - Human outdoor

Table 4: Simulation assumptions for full calibration for human outdoor.

| Parameters | Values |
| --- | --- |
| Scenario | UMa |
| Cell layout | Hexagonal grid, 19 macro sites, 3 sectors per site (ISD = 500m) |
| Sensing mode | TRP monostatic, TRP-TRP bistatic, TRP-UE bistatic, UE-UE bistatic |
| Sensing type | Adult: 0.5m x 0.5m x 1.75m |
| Sectorization | 3 sectors per cell site: 30, 150 and 270 degrees. For a single isotropic antenna, single 360-degree sector can be assumed. |
| Carrier Frequency | 7 GHz, 30 GHz |
| Bandwidth | 7GHz: 200MHz 30GHZ: 400MHz |
| BS height | 25m |
| BS antenna configurations | Single dual-pol isotropic antenna |
| BS Tx power | 7GHz: 59dBm 30GHZ: 41dBm |
| BS noise figure | 7GHz: 5dB 30GHZ: 7dB |
| UT height | 1.5m |
| UT antenna configurations | (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1) |
| UT mobility | 3Km/h |
| UT Tx power | 23dBm |
| UT distribution | Uniformly distributed within the horizontal plane. 10 outdoor UTs per cell. |
| UT noise figure | 7GHz: 9dB 30GHz: 10dB |
| Polarized antenna model | Model-2 in clause 7.3.2 in TR 38.901 |
| Sensing target distribution | 1 target uniformly distributed (across multiple drops) within the center site. |
| Sensing target mobility | 3Km/h |
| Scattering point | Single scattering point is in the center of the human. The vertical height for pathloss calculation is 0.875 m. |
| Component A of the RCS for each scattering point | a fixed value of A |
| Component B1, B2 of the RCS for each scattering point | According to the agreement of RCS in 9.7.2. |
| Fast fading model | According to 38.901 and 38.858. |
| (u, std) for XPR of target | According to the agreement in 9.7.2. |
| The power threshold for path dropping after concatenation for target channel | -40dB |
| Minimum 3D distances between pairs of Tx/Rx and sensing target | For TRP-ST, 25m For UT-ST, 1m |
| Minimum 3D distances targets | 1m |
| Wrapping Method | No wrapping method is used if interference is not modelled, otherwise geographical distance based wrapping. |
| Coupling loss definition | By definition, need to consider all direct and indirect paths. The following parameters are included in the calculation: - power scaling factor (pathloss, shadow fading, and RCS component A included) - for small scale RCS B1/B2 and power of rays in Tx-target/target-Rx links (Pn',m',n,mk,p), Tx/Rx antenna pattern, 3 polarization matrixes, i.e., Pn',m',n,mk,pFrx,u,θθrx,n',m',ZOAk,p,ϕrx,n',m',AOAk,pFrx,u,ϕθrx,n',m',ZOAk,p,ϕrx,n',m',AOAk,pTCPMrx,n',m'k,pCPMn',m',n,mk,pCPMtx,n, mk,p ∙Ftx,s,θθtx,n,m,ZODk,p,ϕtx,n,m,AODk,pFtx,s,ϕθtx,n,m,ZODk,p,ϕtx,n,m,AODk,p |
| Sensing Tx/Rx selection | For TRP mono-static: each TRP can be the sensing Tx/Rx. For TRP-TRP/UE-UE bistatic, each pair of TRP-TRP/UE-UE can be the sensing Tx/Rx pair. For TRP-UE bistatic, each TRP-serving UE can be the sensing Tx/Rx pair. The TRP-serving UE selection based on the TRP-UE coupling loss. Select all the Tx-ST-Rx links for calibration. |
| Metrics | Coupling loss. CDF of the Delay Spread and Angle Spread (ASD, ZSD, ASA, ZSA). ASD and ZSD are the departure angle spread of sensing TX, ASA and ZSA are the arrival angle spread of sensing RX. |

- For the purposes of full calibration for human outdoor sensing scenarios, the parameters in Table 4 are proposed.

Calibration parameters for spatial consistency

According to the agreements in [2], the spatial consistency for the following links is needed to be model:

- Case 5: links between same UT and two nodes X/Y, subjected to correlation distance, i.e., link UT1-X and link UT1-Y, where nodes X/Y can be target or UT

- Case 6: links between same target and two nodes X/Y, subjected to correlation distance, i.e., link target1-X and link target1-Y, where nodes X, Y are different UTs

- Case 7: link X1-Y1 and link X2-Y2, subjected to correlation distance, where X1, X2, Y1, Y2 are 4 different nodes

In our views, case 5 and case 6 are the basic cases, and case 7 is a combination of case 5 and case 6. Hence, for spatial consistency calibration of ISAC channel, to further reduce the workload, one of case 5 or case 6 may be sufficient.

According to the current discussion in RAN1, the spatial consistency can rely on either the target-specific method or the UT-specific method. In order to ensure the consistency among the companies, RAN1 needs to determine one of the methods of the spatial consistency.

In the calibration assumptions, the number of UTs per cell is 10, while the number of targets per cell is only 1. Hence, using target-specific method described in [3] to model the correlation between target channels can be much simpler than using UT-specific method.

- For spatial consistency calibration of ISAC channel, select Case 5 or Case 6 for calibration.

- Case 5: links between same UT and two nodes X/Y, subjected to correlation distance, i.e., link UT1-X and link UT1-Y, where nodes X/Y can be target or UT.

- Case 6: links between same target and two nodes X/Y, subjected to correlation distance, i.e., link target1-X and link target1-Y, where nodes X, Y are different UTs.

- RAN1 uses the target-specific method described in [3] to model the spatial consistency of ISAC channel.

Human indoor

Table 5: Simulation assumptions for spatial consistency calibration for human indoor.

| Parameters | Values |
| --- | --- |
| Scenario | Indoor office |
| Cell layout | Office size (WxLxH): 120mx50mx3m, ISD=20m (refer to TR 38.901) |
| Sensing mode | TRP-UE bistatic |
| Sensing type | Adult: 0.5m x 0.5m x 1.75m |
| Sectorization | 1 sector per cell site |
| Carrier Frequency | 7 GHz |
| Bandwidth | 200MHz |
| BS height | 3m (ceiling) |
| BS antenna configurations | Single dual-pol isotropic antenna |
| BS Tx power | 24dBm |
| BS noise figure | 5dB |
| UT height | 1m |
| UT antenna configurations | (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1) |
| UT mobility | 3Km/h |
| UT Tx power | 23dBm |
| UT noise figure | 9dB |
| Polarized antenna model | Model-2 in clause 7.3.2 in TR 38.901 |
| Sensing target mobility | 3Km/h |
| Scattering point | Single scattering point is in the center of the human. The vertical height for pathloss calculation is 0.875 m. |
| Component A of the RCS for each scattering point | a fixed value of A (A is -1.37dBsm) |
| Component B1, B2 of the RCS for each scattering point | According to the agreement of RCS in 9.7.2. |
| Minimum 3D distances between pairs of Tx/Rx and sensing target | For TRP-ST, 1m For UT-ST, 1m |
| Fast fading channel | Fast fading channel is modelled according to 38.901 and 38.858. |
| Minimum 3D distances targets | 1m |
| Wrapping Method | No wrapping method is used. |
| Correlation link | Option 1: For target channel with same Target and different UTs (Case 6) Option 2: For target channel with different Targets and same UT (Case 5) |
| UT and Target distribution and calibration method | Take links between same Target and different UTs as an example (Option 1) - Step1: Random drop a Target in office region of 120m*50m - Step2: Random drop multiple UTs in office 120m*50m, e.g., X UTs - Step3: According to spatial consistency method, generate links from the Target to all UTs - Step4: Determine all permutations of link pairs of Target-UTs, e.g., total X*(X-1)/2 pairs. Then calculate the location distance of two link pairs by the distance between two UTs - Step5: Bin all the link pairs into certain distance groups, e.g., 0m <= the location distance of link pair < 1m 0m group 1m <= the location distance of link pair < 2m 1m group 2m <= the location distance of link pair < 3m 2m group 3m <= the location distance of link pair < 4m 3m group … N m <= the location distance of pair < N+1 m N m group Note: N is at least twice the maximum correlation distance associated with the channel parameters in the scenario - Step6: Calculate the correlation coefficient metric of each distance groups, e.g., first, collect the delay/AOA/Los state value of Target-UT link for each pair belonging to N m group, then calculate the correlation coefficient metric of these collected variables using NOTE1 - Step7: Draw the x-y cure, where x=0, …, N, y= correlation coefficient metric of x m group. |
| Spatial consistency modeling method | Option 1: Global grid, and shift UT/ST locations Option 2: UT specific grid, considering the correlation of grids of different UT, e.g., Cholesky decomposition Option 3: ST specific grid, considering the correlation of grids of different ST, e.g., Cholesky decomposition |
| Correlation coefficient metrics | 1) Cross-correlation coefficient of delay for the third cluster of channel between link pairs 2) Cross-correlation coefficient of AOA (for UT) for the third cluster of channel between link pairs 3) Cross-correlation coefficient of LOS/NLOS status of channel between link pairs 4) Optional: Cross correlation coefficient of the target channel response in Step 11 on the first non-DC subcarrier in an OFDM symbol on antenna port 0 received on the first UT antenna over multiple realizations between link pairs 5) Optional: CDF of coupling loss, defined in full calibration |
| NOTE 1 The cross-correlation coefficient for real number can be written as [E(XY)-E(X)E(Y)] /sqrt([E(X^2)-E(X)^2])/ sqrt([E(Y^2)-E(Y)^2]), the cross-correlation coefficient for complex number can be written as ¦[E(XY*)-E(X)E(Y)*]/sqrt([E(XX*)-E(X)E(X)*])/sqrt([E(YY*)-E(Y)E(Y)*])¦. |  |

- For the purposes of spatial consistency calibration for human indoor sensing scenarios, the parameters in Table 5 can be a starting point.

Vehicle Urban grid

Table 6 Simulation assumptions for spatial consistency calibration for vehicles

| Parameters | Values |
| --- | --- |
| Scenario | Urban grid |
| Cell layout | ISD =500m, the same layout with 37.885 |
| Sensing mode | TRP-UE bistatic |
| Sectorization | 3 sectors per cell site: 0, 120, 240 degrees to align with the 37.885. For a single isotropic antenna, single 360-degree sector can be assumed. |
| Carrier Frequency | 7GHz |
| Bandwidth | 200MHz |
| BS height | 25m |
| BS antenna configurations | Single dual-pol isotropic antenna |
| BS Tx power | 59 dBm |
| BS noise figure | 5dB |
| UT height | 1.5m |
| UT antenna configurations | (M, N, P, Mg, Ng; Mp, Np) = (1, 1, 2, 1, 1; 1, 1) |
| UT array orientation | ΩUT, is the lane direction in horizontal panel, ΩUT,= 90 degree, ΩUT, = 0 degree |
| UT type | Pedestrian UE |
| UT mobility | 3Km/h in all line |
| UT Tx power | 23dBm |
| UT noise figure | 9 dB |
| Polarized antenna model | Model-2 in clause 7.3.2 in TR 38.901 |
| Fast fading channel | Fast fading channel is modelled according to 37.885. |
| min distance between sensing Tx and sensing Rx | For TRP-UT: 25 m For UT-UT: 1m |
| Target type | Type 2 vehicles, 5m*2m*1.6m |
| Scattering point | Single scattering point is in the center of the vehicle. |
| Sensing target mobility | 60 Km/h in all line |
| Component A of the RCS for each scattering point | According to the agreement of RCS in 9.7.2. |
| Component B1, B2 of the RCS for each scattering point | According to the agreement of RCS in 9.7.2. |
| Target orientation | Along the street in horizontal domain |
| Minimum 3D distances between pairs of Tx/Rx and sensing target | For TRP-target: 25m For UT-target: 1m. |
| Minimum 3D distance between sensing targets | 10m between the rear bumper of a vehicle and the front bumper of the following vehicle in the same lane. |
| Wrapping method | No wrapping method is used if interference is not modelled, otherwise geographical distance based wrapping. |
| Correlation link | Down-select one from following two options: Option 1: For target channel with same Target and different UTs (Case 6) Option 2: For target channel with different Targets and same UT (Case 5) |
| UT and Target distribution and calibration method | Take links between same Target and different UTs as an example (Option 1) - Step1: Random drop one or four (each side) Target(s) in center urban grid region - Step2: Random drop multiple UTs in center urban grid region, e.g., X UTs - Step3: According to spatial consistency method, generate links from the Target to all UTs - Step4: Determine all permutations of link pairs of Target-UTs, e.g., total X*(X-1)/2 pairs. Then calculate the location distance of two link pairs by the distance between two UTs - Step5: Bin all the link pairs into certain distance groups, e.g., 0m <= the location distance of link pair < 1m 0m group 1m <= the location distance of link pair < 2m 1m group 2m <= the location distance of link pair < 3m 2m group 3m <= the location distance of link pair < 4m 3m group … N m <= the location distance of pair < N+1 m N m group Note: N is at least twice the maximum correlation distance associated with the channel parameters in the scenario - Step6: Calculate the correlation coefficient metric of each distance groups, e.g., first, collect the delay/AOA/Los state value of Target-UT link for each pair belonging to N m group, then calculate the correlation coefficient metric of these collected variables using NOTE1 Step7: Draw the x-y cure, where x=0, …, N, y= correlation coefficient metric of x m group. |
| Spatial consistency modeling method | Option 1: Global grid, and shift UT/ST locations Option 2: UT specific grid, considering the correlation of grids of different UT, e.g., Cholesky decomposition Option 3: ST specific grid, considering the correlation of grids of different ST, e.g., Cholesky decomposition |
| Correlation coefficient metrics | 1) Cross-correlation coefficient of delay for the third cluster of channel between link pairs 2) Cross-correlation coefficient of AOA (for UT) for the third cluster of channel between link pairs 3) Cross-correlation coefficient of LOS/NLOS status of channel between link pairs 4) Optional: Cross correlation coefficient of the target channel response in Step 11 on the first non-DC subcarrier in an OFDM symbol on antenna port 0 received on the first UE antenna over multiple realizations between link pairs 5) Optional: CDF of coupling loss, defined in full calibration |
| NOTE 1 The cross-correlation coefficient for real number can be written as [E(XY)-E(X)E(Y)] /sqrt([E(X^2)-E(X)^2])/ sqrt([E(Y^2)-E(Y)^2]), the cross-correlation coefficient for complex number can be written as ¦[E(XY*)-E(X)E(Y)*]/sqrt([E(XX*)-E(X)E(X)*])/sqrt([E(YY*)-E(Y)E(Y)*])¦. |  |

- For the purposes of spatial consistency calibration for vehicles sensing scenarios, the parameters in Table 6 can be a starting point.

Initial calibration results

UAV

Large scale calibration

The simulation assumptions are based on what we defined in Agreement-3, as listed in Table A-1 of Section 6, Annex A.

      - In case of 6GHz

Figure 1 shows the CDF of power scaling factor for mono-static and bi-static sensing modes, in case of 6GHz.

Figure 1: CDF of power scaling factor for mono-static and bi-static sensing modes, in case of 6GHz.

      - In case of 30GHz

Figure 2 shows the CDF of power scaling factor for mono-static and bi-static sensing modes, in case of 30GHz.

Figure 2: CDF of power scaling factor for mono-static and bi-static sensing modes, in case of 30GHz.

Human indoor room

Large scale calibration

The simulation assumptions are based on the proposed parameters listed in Table 1.

      - In case of 7GHz

Figure 3 shows the CDF of power scaling factor for mono-static and bi-static sensing modes, in case of 7GHz.

Figure 3: CDF of power scaling factor for mono-static and bi-static sensing modes, in case of 7GHz.

      - In case of 30GHz

Figure 4 shows the CDF of power scaling factor for mono-static and bi-static sensing modes, in case of 30GHz.

Figure 4: CDF of power scaling factor for mono-static and bi-static sensing modes, in case of 30GHz.

Human indoor office

Large scale calibration

The simulation assumptions are based on the proposed parameters listed in Table 1.

      - In case of 7GHz

Figure 5, Figure 6 and Figure 7show the CDF of power scaling factor in case of 7GHz for TRP mono-static mode, TRP-TRP bi-static mode and TRP-UE bi-static mode, respectively.

Figure 5: CDF of power scaling factor for TRP mono-static sensing mode, in case of 7GHz.

Figure 6: CDF of power scaling factor for TRP-TRP bi-static sensing mode, in case of 7GHz.

Figure 7: CDF of power scaling factor for TRP-UE bi-static sensing mode, in case of 7GHz.

      - In case of 30GHz

Figure 8, Figure 9 and Figure 10 show the CDF of power scaling factor in case of 30GHz for TRP mono-static mode, TRP-TRP bi-static mode and TRP-UE bi-static mode, respectively.

Figure 8: CDF of power scaling factor for TRP mono-static sensing mode, in case of 30GHz.

Figure 9: CDF of power scaling factor for TRP-TRP bi-static sensing mode, in case of 30GHz.

Figure 10: CDF of power scaling factor for TRP-UE bi-static sensing mode, in case of 30GHz.

Human outdoor

Large scale calibration

The simulation assumptions are based on the proposed parameters listed in Table 2.

      - In case of 7GHz

Figure 11, Figure 12 and Figure 13 show the CDF of power scaling factor in case of 7GHz for TRP mono-static mode, TRP-TRP bi-static mode and TRP-UE bi-static mode, respectively.

Figure 11: CDF of power scaling factor for TRP mono-static sensing mode, in case of 7GHz.

Figure 12: CDF of power scaling factor for TRP-TRP bi-static sensing mode, in case of 7GHz.

Figure 13: CDF of power scaling factor for TRP-UE bi-static sensing mode, in case of 7GHz.

      - In case of 30GHz

Figure 14,  Figure 15 and Figure 16 show the CDF of power scaling factor in case of 30GHz for TRP mono-static mode, TRP-TRP bi-static mode and TRP-UE bi-static mode, respectively.

Figure 14: CDF of power scaling factor for TRP mono-static sensing mode, in case of 30GHz.

Figure 15: CDF of power scaling factor for TRP-TRP bi-static sensing mode, in case of 30GHz.

Figure 16: CDF of power scaling factor for TRP-UE bi-static sensing mode, in case of 30GHz.

Conclusions

In this contribution, we have discussed the views on ISAC channel calibration. The observations and proposals are summarized as follows.

Observation 1: Some sensing channel conditions, such as NLOS+NLOS, LOS+NLOS, NLOS+LOS, may not be calibrated if only best N Tx-ST-Rx links are calibrated.

- For TRP-UE/UE-TRP bistatic, each pair of serving TRP-UE can be the sensing Tx-Rx pair, and the serving TRP selection can be based on the RSRP between TRP and UE.

- Select all the Tx-ST-Rx links to draw the CDF figures for calibration.

- RAN1 does not limit the deployment scenarios for calibration, and it is up to company inputs.

- RAN1 does not limit the sensing mode, and it is up to company inputs.

- Full convolution is used as a baseline for ISAC channel calibration.

- Whether or how to perform power normalization should be determined for ISAC channel calibration if ray level 1-by-1 random coupling convolution between Tx-target link and target-Rx link is used.

- RAN1 individually calibrates the target channel and background channel if the background channel is newly designed. Otherwise, RAN1 only calibrates the target channel.

- For the purposes of large scale calibration for human indoor sensing scenarios, the parameters in Table 1 are proposed.

- For the purposes of large scale calibration for human outdoor sensing scenarios, the parameters in Table 2 are proposed.

- For the purposes of full calibration for human indoor sensing scenarios, the parameters in Table 3 are proposed.

- For the purposes of full calibration for human outdoor sensing scenarios, the parameters in Table 4 are proposed.

- For spatial consistency calibration of ISAC channel, select Case 5 or Case 6 for calibration.

- Case 5: links between same UT and two nodes X/Y, subjected to correlation distance, i.e., link UT1-X and link UT1-Y, where nodes X/Y can be target or UT.

- Case 6: links between same target and two nodes X/Y, subjected to correlation distance, i.e., link target1-X and link target1-Y, where nodes X, Y are different UTs.

- RAN1 uses the target-specific method described in [3] to model the spatial consistency of ISAC channel.

- For the purposes of spatial consistency calibration for human indoor sensing scenarios, the parameters in Table 5 are proposed.

- For the purposes of spatial consistency calibration for vehicles sensing scenarios, the parameters in Table 6 are proposed.

References

- RP-234069, “New SID: Study on channel modelling for Integrated Sensing and Communication (ISAC) for NR”, Nokia, Nokia Shanghai Bell.

- 3GPP RAN1 #120, Chair notes RAN1#120, Athens, Greece, February 17th – 21nd, 2025.

- R1-2501818, “Views on Rel-19 ISAC channel modelling”, vivo, WuHan, China, April 7th – 11st, 2025.

Annex A. Channel calibration parameters for UAV

Table A-1: Simulation assumptions for large scale calibration for UAV sensing targets

| Parameters | Values |
| --- | --- |
| Scenario | UMa-AV |
| Sensing mode | TRP monostatic, TRP-TRP bistatic Note: further down-selection of the sensing modes for UAV sensing is not precluded |
| Target type | UAV of small size (0.3m x 0.4m x 0.2m) |
| Sectorization | 3 sectors per cell site: 30, 150 and 270 degrees. For a single isotropic antenna, single 360-degree sector can be assumed. |
| Carrier Frequency | FR1: 6 GHz FR2: 30 GHz |
| BS antenna configurations | Single dual-pol isotropic antenna |
| BS Tx power | FR1: 56dBm FR2: 41dBm |
| Bandwidth | FR1: 100MHz FR2: 400MHz |
| BS noise figure | FR1: 5dB FR2: 7dB |
| UT antenna configurations | (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1) |
| UT noise figure | FR1: 9dB FR2: 10dB |
| Sensing target distribution | 1 target uniformly distributed (across multiple drops) within the 3 sectors of the center cell. Vertical distribution: Fixed height value of 200 m. |
| Component A of the RCS for each scattering point | a fixed value of A, (A is -12.81dBsm) |
| Minimum 3D distances between pairs of Tx/Rx and sensing target | 10 m (horizontal) |
| Wrapping Method | No wrapping method is used if interference is not modelled, otherwise geographical distance based wrapping |
| LOS condition, pathloss, SF | Generate the LOS condition, pathloss, SF for each of Tx-target and target-Rx links For monostatic, same LOS condition/pathloss/SF for both Tx-target and target-Rx links |
| Definition of Coupling loss | Power scaling factor (pathloss, shadow fading, and RCS component A included) LTX-SPST-RX=PLdBd1+PLdBd2+10lgc24πf2-10lgσRCS,A+SFdB,1+SFdB,2 |
| Sensing Tx/Rx selection | All the Tx-Rx pairs or best N=[1],4 Tx-Rx pairs to be selected for the target. For TRP mono-static: Based on the Tx-Rx pair with the smallest power scaling factor of the target channel. For TRP-TRP bistatic: Total: C(19,2) =19*18/2 Tx-Rx pairs Based on the Tx-Rx pair with the smallest power scaling factor of the target channel. |
| Metrics | Coupling loss (based on LOS pathloss) |